3GPP TSG RAN WG1 #117                                  R1-2404875

Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024

Source:	OPPO

Title:          Discussion on ISAC deployment scenarios

Agenda Item:   9.7.1

Document for:  Discussion and Decision

# Description

RAN1 #116-bis meeting reaches the following agreements for ISAC deployment scenarios.

Agreement Any TRP and/or UE location in the corresponding communication scenario can be selected as sensing transmitters and receivers locations. FFS: other possible sensing transmitters and receivers locations. Agreement The following table can be used by companies to propose values for each sensing target. - Additional parameters/rows can be added if needed Table x. Evaluation parameter template for sensing scenarios Parameters; Value. Applicable communication scenarios. Sensing transmitters and receivers properties. Supported sensing modes. Sensing target; Outdoor/indoor. 3D mobility. 3D distribution. Orientation. Physical characteristics (e.g., size). [Unintended/Environment objects]; Types. 3D mobility. 3D distribution. Orientation. Physical characteristics (e.g., size). [Sensing area]. Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects].

# Discussion

## Deployment parameters for indoor case

Indoor scenario is considered with human to be the sensing target in indoor office, and with automated guided vehicle to be the sensing target in indoor factory. The sensing entity (i.e., Tx/Rx) can be either TRP or UE. The layout of TRP is same with communication case. The location distributions of sensing target and environment object are similar with the distribution of UE in communication case. The details assumptions for indoor scenarios are given in Table 1. For Indoor office scenario, the environment object could be furniture or small living animal such as pets. Human, metal robot and clutter could be the environment objects in indoor factory scenario.

Table 1 Assumptions for Indoor scenarios

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Indoor office | Indoor factory |
| Sensing transmitters and receivers properties |  | Layout： - 120m*50m*3m Vertical height: - 1.5m (UE), 3m (TRP) Horizontal: - Uniformly random, if height=1.5m - fixed location with ISD=20m, if height=3m | Layout： - 20-160000 m2 Vertical height: - 1.5m (UE), [5~10]m (TRP) Horizontal: - Uniformly random, if height=1.5m - fixed location with ISD=[20m], if height=[5~10]m |
| Supported sensing modes |  | mono-static, bi-static, multi-static | mono-static, bi-static, multi-static |
| Sensing target | Outdoor/indoor | Indoor | Indoor |
|  | 3D mobility | 0, 3km/h | 3km/h |
|  | 3D distribution | Vertical height: 0.8~1m Horizontal: arbitrary | Vertical height: 1m Horizontal: arbitrary |
|  | Orientation | Vertical: 0-180° Horizontal: 0-360° | Vertical: 0-180° Horizontal: 0-360° |
|  | Physical characteristics (e.g., size) | Human: - 1.8m*0.5m*0.5m (adult) - 1m*0.5m*0.2m (child) | vehicle: - 5m*2m*1.6m (passenger vehicle) - 13m*2.6m*3m (truck) |
| Unintended/Environment objects | Types | furniture, pet | human, clutter |
|  | 3D mobility | 0, 3km/h | 0, 3km/h |
|  | 3D distribution | Vertical height: 0.2~1 m Horizontal: arbitrary | Vertical height: 1m Horizontal: arbitrary |
|  | Orientation | Vertical: 0-180° Horizontal: 0-360° | Vertical: 0-180° Horizontal: 0-360° |
|  | Physical characteristics (e.g., size) | Furniture (wood): - 1.8m*1m*0.5m Pet: - 0.5m*0.5m*0.5m | Human/robot: - 1.8m*0.5m*0.2m Clutter (metal): - 1m*1m*1m |
| Sensing area |  | full area of target distribution | full area of target distribution |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] |  | 0m (for Tx-Rx in mono-static sensing only); 1m (for all other pairs) | 0m (for Tx-Rx in mono-static sensing only); 1m (for all other pairs) |

## Deployment parameters for outdoor case

The human, UAV and vehicle could be the sensing targets in outdoor scenarios. The details assumptions for outdoor scenarios with UMi/UMa/RMa are given in Table 2. The environment object could be human, vehicle, bird, or buildings. For the use case of vehicle, the assumptions are given in Table 3 with Urban gird and High-way scenarios.

Table 2 Assumptions for UMi/UMa/RMa scenarios

| Parameters |  | Value | Value | Value |
| --- | --- | --- | --- | --- |
| Applicable communication scenarios |  | UMi/UMi-AV | UMa/UMa-AV | RMa/RMa-AV |
| Sensing transmitters and receivers properties |  | Layout: - Hexagonal cell - ISD=200m(UMi/UMi-AV), 500m(UMa/UMa-AV), 1732m(RMa/RMa-AV) Vertical height: - As a UE (outdoor only): 1.5m - As a TRP: 10m(UMi/UMi-AV), 25m(UMa/UMa-AV), 35m(RMa/RMa-AV) - As a UAV-UE (for UAV scenarios only): same distribution as UAV target Horizontal: - If being a TRP: Fixed location at hexagonal cell centers; - otherwise: Uniformly random per hexagonal cell. |  |  |
| Supported sensing modes |  | mono-static, bi-static, multi-static |  |  |
| Sensing target | Outdoor/ indoor | Outdoor |  |  |
|  | 3D mobility | 0, 3km/h, 160km/h (UAV only) |  |  |
|  | 3D distribution | Vertical height: uniformly random in [1.5m, H], where - H=1.5 for non-UAV scenarios - H=300m for UAV scenarios. Horizontal: arbitrary in hexagonal cell. |  |  |
|  | Orientation | Vertical: 0-180° Horizontal: 0-360° |  |  |
|  | Physical characteristics (e.g., size) | Human: - 1.8m*0.5m*0.5m (adult) - 1m*0.5m*0.2m (child) Vehicle: - 5m*2m*1.6m (passenger vehicle) - 13m*2.6m*3m (truck) UAV: - 0.5m*0.5m*0.5m |  |  |
| Unintended/Environment objects | Types | For UAV scenarios: birds |  |  |
|  | 3D mobility | 30km/h, 120km/h |  |  |
|  | 3D distribution | Vertical height: Uniformly random in [1m, 300m] Horizontal: arbitrary in hexagonal cell. |  |  |
|  | Orientation | Vertical: 0-90° Horizontal: 0-360° |  |  |
|  | Physical characteristics (e.g., size) | Bird(UAV scenarios only): - 0.2m*0.2m*0.2m |  |  |
| Sensing area |  | full area of target distribution |  |  |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] |  | 0m (for Tx-Rx in mono-static sensing only); 10m (for all other pairs) |  |  |

Table 3 Assumptions for Urban grid and High-way scenarios

| Parameters |  | Value |  |
| --- | --- | --- | --- |
| Applicable communication scenarios |  | Urban grid | High-way/Railway |
| Sensing transmitters and receivers properties |  | TRP Layout: - Hexagonal cell - ISD=500m - Road grid=250m*433m Vertical height: - 1.5m(UE, outdoor only) - 25m(TRP) Horizontal: - Uniformly random in area not occupied by rectangular buildings, if height=1.5m - Fixed location at hexagonal cell centers, if height=25m | High-way: HST: TRP Layout: - Hexagonal cell - ISD=1732m Vertical height: - 1.5m(UE, outdoor only) - 35m(TRP) Horizontal: - Uniformly random over straight-line, if height=1.5m - Fixed location as figure shows, if height=35m |
| Supported sensing modes |  | mono-static, bi-static, multi-static | mono-static, bi-static, multi-static |
| Sensing target | Outdoor/indoor | Outdoor | Outdoor |
|  | 3D mobility | 0, 3km/h, 30km/h, 120km/h | 0, 3km/h, 30km/h, 120km/h |
|  | 3D distribution | Vertical height: - 1m Horizontal: - arbitrary over straight-line. | Vertical height: - 1m Horizontal: - arbitrary over straight-line. |
|  | Orientation | Vertical: 0-180° Horizontal: 0-360° | Vertical: 0-180° Horizontal: 0-360° |
|  | Physical characteristics (e.g., size) | Vehicle: - 5m*2m*1.6m (passenger vehicle) - 13m*2.6m*3m (truck) | Vehicle: - 5m*2m*1.6m (passenger vehicle) - 13m*2.6m*3m (truck) |
| Unintended/ Environment objects | Types | Human, animal | Human, animal, rocks |
|  | 3D mobility | 0, 3km/h | 0, 3km/h |
|  | 3D distribution | Vertical height: 1m Horizontal: arbitrary | Vertical height: 1m Horizontal: arbitrary |
|  | Orientation | Vertical: 0-180° Horizontal: 0-360° | Vertical: 0-180° Horizontal: 0-360° |
|  | Physical characteristics (e.g., size) | Human: - 1.8m*0.5m*0.5m (adult, animal) - 1m*0.5m*0.2m (child) | Human: - 1.8m*0.5m*0.5m (adult, animal) - 1m*0.5m*0.2m (child) Rocks - 1m*1m*1m |
| Sensing area |  | full area of target distribution | full area of target distribution |
| Minimum 3D distances between pairs of Tx/Rx/sensing target/[unintended objects] |  | 0m (for Tx-Rx in mono-static sensing only); 10m (for all other pairs) | 0m (for Tx-Rx in mono-static sensing only); 10m (for all other pairs) |

Proposal 1: Table 1~3 in this contribution are adopted in description of ISAC deployment scenarios.

# Conclusion

Proposal 1: Table 1~3 in this contribution are adopted in description of ISAC deployment scenarios.

# Reference

- RP-234069 New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR

- TR 38.901 Study on channel model for frequencies from 0.5 to 100 GHz

- TR 36.777 Study on Enhanced LTE Support for Aerial Vehicles

- TR 37.885 Study on evaluation methodology of new Vehicle-to-Everything (V2X) use cases for LTE and NR

- TR 38.802 Study on New Radio Access Technology Physical Layer Aspects