3GPP TSG-RAN WG1 Meeting #118	R1- 2406495

Maastricht, Netherlands, August 19th – August 23rd, 2024

Agenda Item:	9.1.3.2

Source:	NVIDIA

Title:	Additional study on AI-enabled CSI compression

Document for:	Discussion

# 1	Introduction

The approval of the Rel-19 work package marks the next wave of 5G Advanced evolution [1]. The package includes a work item on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface [2].

The work item consists of multiple objectives, including further study on AI/ML-enabled CSI compression:

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

At previous RAN1 meetings, initial agreements on additional study on AI-enabled CSI compression were agreed (see Appendix).

In this contribution, we discuss the remaining issues of additional study on AI-enabled CSI compression.

# 2	Background

AI/ML techniques, such as autoencoders, can be used in CSI compression and feedback. The basic principle of an autoencoder is to take an input, perform non-linear compression by an encoder to a lower-dimensional latent representation, and decompress the latent representation by a decoder to a defined target. In the context of CSI compression, the UE’s estimated CSI is taken as the input to the encoder. The output of the UE’s encoder (i.e., the latent representation of the channel in a quantized form) is transmitted over the air interface (Uu) to the BS. The BS uses a matching decoder to reconstruct the channel state. Hence, the autoencoder model is divided between the UE and BS, creating an inter-node dependency for model training and model management.

Using autoencoders for CSI compression is in line with the agreement that for the evaluation of the AI/ML based CSI compression sub use cases, a two-sided model is considered as a starting point, including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. At least for inference, the CSI generation part is located at the UE side, and the CSI reconstruction part is located at the gNB side.

The autoencoders can be used to compressed either the raw channel matrix estimated by UE or the eigenvector(s) of the raw channel matrix estimated by UE, etc. Raw channel matrix-based CSI feedback provides gNB with the most flexibility in determining downlink transmission, though it deviates from the existing codebook-based CSI feedback. In contrast, the channel eigenvector-based CSI feedback is more in line with the conventional codebook-based CSI feedback methods, but it does not provide as much flexibility as the raw channel matrix-based CSI feedback. Both types of CSI feedback schemes are of interest and deserve study.

# 3	Performance study

To evaluate AI/ML based algorithms for CSI compression, datasets are needed. Both real data and synthetic data can be used to develop and evaluate AI/ML based algorithms. 3GPP has well established simulation methodology, which can be used to generate synthetic data. CSI enhancement would be most valuable in the scenarios where there is high-capacity demand. Therefore, the evaluation could focus on UMi-street canyon and UMa scenarios.

In general, the statistical models in TR 38.901 can be used as baseline for link and system evaluation of AI/ML based algorithms for CSI feedback enhancement. However, the Rel-18 study shows that using data generated from stochastic channel models is not sufficient to demonstrate the performance gain of CSI compression compared to non-AI/ML based algorithms.

Observation 1: The Rel-18 study shows that using data generated from stochastic channel models is not sufficient to demonstrate the performance gain of CSI compression compared to non-AI/ML based algorithms.

To improve the performance gain, site-specific AI/ML models for CSI compression should be considered. Site-specific optimization refers to the continuous process of fine-tuning network configurations on a per-site basis. The goal is to ensure optimal performance for individual cell-sites in terms of key performance indicators (KPIs) like coverage, capacity, quality-of-service (QoS), quality of experience (QoE) and energy efficiency. Which attributes are tuneable and important for site-specific optimization depends on the RAN deployment architecture, as well as the capabilities of cell-site specific network elements. As cellular networks continue to evolve with the emergence and deployment of innovative technologies beyond 5G, site-specific RAN optimization is poised to become more sophisticated due to the increasing number of frequency bands, demanding KPIs (e.g., higher data rates, ultra-low latency, wider coverage, lower energy consumption, and high-accuracy positioning) and innovation of more advanced antenna technologies.

Proposal 1: Site-specific AI/ML models for CSI compression should be considered to improve performance gain.

It was agreed that for the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

  - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

  - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

It was emphasized that while modelling the spatial correlation, companies should ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel.

Nonetheless, to enable the development and evaluation of site-specific AI/ML models, it is beneficial to define a common reference scenario with site specificity.

There are two options for defining a common reference scenario with site specificity:

- Option 1: Real-scenario map that is a virtual representation of a real area on earth.

- Option 2: Synthetic-scenario map that is artificially constructed to mimic a certain environment such as urban macro, rural macro, indoor office, and indoor factory.

Proposal 2: Define a common reference scenario with site specificity as a basis for further study of AI/ML based CSI compression.

Proposal 3: Select one the following options to define a common reference scenario with site specificity as a basis for further study of AI/ML based CSI compression:

- Option 1: Real-scenario map that is a virtual representation of a real area on earth.

- Option 2: Synthetic-scenario map that is artificially constructed to mimic a certain environment such as urban macro, rural macro, indoor office, or indoor factory.

As a starting point of the discussion, 3GPP may consider the existing scenarios, e.g., those defined by METIS [3]. Specifically, for urban scenarios, the Madrid grid developed by the METIS project can be considered.

Figure 1: Madrid grid for Urban Micro and Urban Macro. Source: METIS D6.1 [3].

Once a scenario is defined, ray tracing can be used to generate data for the development and evaluation of site-specific AI/ML models for CSI compression. Ray tracing is a rendering and simulation technique used in computer graphics, optics, and other fields to simulate the way rays of light or other radiation travel through a virtual environment. In the context of wireless communication and radio wave propagation, ray tracing is often employed to model and simulate the paths that EM waves take as they propagate through various materials and interact with surfaces and obstacles. It provides a deterministic and physics-based modelling approach that simulates the paths of individual rays of EM waves, considering reflections, refractions, diffractions, and other interactions with objects and surfaces. It offers high-resolution simulations, capturing the specific paths of rays and the effects of the surrounding environment, making it valuable for site-specific planning, antenna design, and network optimization in the field of wireless communication.

Proposal 4: With a common reference scenario with site specificity, ray tracing is used to generate channel data for the development and evaluation of site-specific AI/ML models for CSI compression.

# 4	Inter-vendor training collaboration

For autoencoder based CSI feedback, one important question is how to train an autoencoder model for deployment. There are different training options that have various levels of collaboration between UE and gNB:

- Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

- Type 2: Joint training of the two-sided model at network side and UE side, respectively.

- Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

Figure 2 illustrates Type-1 training. In this case, joint training is performed at a single entity in a single training session. This entity can be at the network or UE side. For example, if the training is done at the network side, the network can transfer the encoder model to the UE, and vice versa.

Figure 2: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided.

Figure 3 illustrates Type-2 training. In this case, joint training is performed at two sides in a single training session. The encoder model and decoder model are trained at UE side and at network side, respectively. But the models are jointly trained in the same loop for forward propagation and backward propagation, by exchanging forward activation and backward gradient between UE-side entity and network-side entity.

As an example, Type-2 training procedure may go as follows. For each forward propagation and backward propagation loop (assuming that the dataset between UE side and network side is aligned):

- Step 1: UE side generates the forward propagation results (i.e., CSI feedback) based on the data samples, and sends the forward propagation results to network side.

- Step 2: Network side reconstructs the CSI based on forward propagation results, trains the CSI reconstruction part, and generates the backward propagation information (e.g., gradients), which are then sent to UE side.

- Step 3: UE side trains the CSI generation part based on the backward propagation information from network side.

Figure 3: Joint training of the two-sided model at network side and UE side, respectively.

Figure 4 illustrates Type-3 training. In this case, separate training is performed at two sides in different training sessions. Unlike Type-2 training, there is no collaboration during training in Type-3 training. But it is necessary to have some coordination outside the training process to ensure that the UE-side CSI encoder model and the NW-side CSI decoder are compatible.

As illustrated in Figure 4, we can have separate training starting with UE side (the left part of the figure), or separate training starting with NW side (the right part of the figure).

As an example, Type-3 training procedure starting with UE side training (UE-first training) may go as follows.

- Step 1: UE side trains the UE side CSI generation part and the UE side CSI reconstruction part (which is not used for inference) jointly.

- Step 2: After UE side training is finished, UE side shares network side with a set of information (e.g., a training dataset consisting of (encoder output, target CSI)) that is used by the network side to be able to train the CSI reconstruction part.

- Step3: Network side trains the network side CSI reconstruction part based on the received set of information.

Similarly, as an example, Type-3 training procedure starting with network side training (network-first training) may go as follows.

- Step 1: Network side trains the network side CSI generation part (which is not used for inference) and the network side CSI reconstruction part jointly.

- Step 2: After network side training is finished, network side shares UE side with a set of information (e.g., a training dataset consisting of (Input of CSI encoder, encoder output)) that is used by the UE side to be able to train the UE side CSI generation part.

- Step 3: UE side trains the UE side CSI generation part based on the received set of information.

Figure 4: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively.

Different training approaches offer different advantages and challenges. Among these, training type 1 holds the promise of yielding superior performance, but the deployment and management of the models can be intricate. Take training type 1 at the network side for example. The network side entity shall involve the UE vendor of the targeted UE type in the training so that the trained CSI encoder model is compatible with the UE’s implementation. However, this effort entails the disclosure of proprietary information by the UE vendor to the network side, which can be a significant challenge.

On the other hand, training type 2 retains the confidentiality of proprietary information at both the network and UE sides, as model information remains within their respective domains. Nonetheless, the two respective training entities need to coordinate their training iterations to exchange forward activation and backward propagation results, which can lead to significant coordination effort and overhead.

Training type 3 emerges as an option that not only safeguards proprietary information but also eliminates the necessity for collaboration during the training iterations because coordination can take place outside the training process.

The training complexity inherent in a two-sided AI/ML model for the air interface is further compounded by the necessity for multi-vendor interoperability and compatibility. The CSI decoder located at the gNB needs to be compatible with different CSI encoders at the UEs, and vice versa. In scenarios where a common CSI decoder model is utilized for multiple CSI encoder models, the network-side training entity—under training type 1 or 2—must coordinate with UE vendors for joint training efforts. Notably, the release of a new UE type could potentially trigger retraining across all vendors. Similar challenges exist in training type 1 or 2 when a shared CSI encoder model is used for multiple CSI decoder models.

These issues can be mitigated in training type 3. In particular, if a common CSI decoder model is used for multiple CSI encoder models (or vice versa), the retraining due to the release of a new UE type can be confined to involve only the associated UE vendor and the network vendor due to the separate training nature in training type 3. In summary, for CSI feedback hinging on a two-sided AI/ML model, training type 3 emerges as a more pragmatic and feasible approach in comparison to training types 1 and 2.

To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, several options deserve further study.

- Option 1: Fully standardized reference model (structure + parameters)

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Option 5: Standardized model format + Reference model exchange between NW-side and UE-side

Option 1 eliminates the inter-vendor collaboration complexity as it does not require bilateral collaboration between vendors. However, Option 1 may have limited performance in the field compared to Options 3, 4, and 5. Also, Option 1 may require high specification effort.

Similar to Option 1, Option 2 may have limited performance in the field compared to Options 3, 4, and 5, as well as requiring high specification effort. Option 2 may be deprioritized for inter-vendor training collaboration.

For Option 3, two variants may be considered. In Option 3a, parameters received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. The parameter exchange originates from the NW-side and ends at the UE-side. Parameters exchanged from the NW-side to UE-side is either CSI generation or reconstruction part or both. In Option 3b, parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. The method of exchanging is over the air-interface via model transfer/delivery Case z4. The parameter exchange is from NW to UE. Parameters exchanged from the NW-side to UE-side is CSI generation part.

For Option 4, dataset received at the UE or UE-side goes through offline engineering at the UE- side (e.g., UE-side OTT server), e.g., model training or offline testing. The dataset exchange originates from the NW-side and ends at the UE-side. The dataset exchanged from the NW-side to UE-side may consist of (target CSI, CSI feedback), or (CSI feedback, reconstructed target CSI), or (target CSI, CSI feedback, reconstructed target CSI).

For Option 5, two variants may be considered. In Option 5a, model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. The model exchange originates from the NW-side and ends at the UE-side. Model exchanged from the NW-side to UE-side is either CSI generation or reconstruction part or both. In Option 5b, model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. The method of exchanging is over the air-interface via model transfer/delivery Case z4, assuming that the model structure is aligned based on offline inter-vendor collaboration. The model exchange is from NW to UE. Model exchanged from the NW-side to UE-side is CSI generation part.

For Options 3, 4, and 5, standardized signaling can be used for exchange of parameter, dataset, and model to alleviate the inter-vendor training collaboration complexity. Such exchanges have potential specification impacts, including exchange methods, format, contents, and additional information such as performance target and dataset information. Other potential specification impact areas include model pairing, UE capability, model scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling, quantization of feedback, and model structure details.

Proposal 5: RAN1 to conclude that it is feasible to resolve issues related to inter-vendor training collaboration for AI/ML-based CSI compression.

# 5	Performance monitoring

Traditionally, 3GPP defines requirements for testing equipment in controlled laboratory conditions prior to its field deployment. Generalization verification is an intricate task. In particular, the training of an AI/ML model can be tailored to encompass the entirety of conditions outlined in the standard, thus demonstrating superior performance during testing. However, the AI/ML model may be overfitted to the standardized setups, possibly compromising its robustness in real-world environments that are not aligned with the controlled conditions. Moreover, AI/ML models might need periodic updates even after they are deployed in live networks. These new considerations call for the implementation of performance monitoring mechanisms to detect non-compliance, as well as the formulation of new testing procedures to effectively validate the functionality of AI/ML-based features operating in the field.

For network-side performance monitoring, it may be based on the target CSI reported by the UE via enhanced Type-2 or alike codebook, or leverage SRS-based monitoring.

For UE-side performance monitoring, it may be based on the output of the CSI reconstruction model at the UE, or estimation of monitoring output such as intermediate KPI without reconstructing a target CSI. It may also leverage precoded RS (e.g., CSI-RS, DMRS) transmitted from network based on the output of the CSI reconstruction model. Additionally, the network may indicate to the UE the output of the CSI reconstruction model using enhanced Type-2 or alike codebook.

Proposal 6: RAN1 to study post-deployment performance monitoring mechanisms to detect performance degradation and non-compliance to guarantee satisfactory performance of AI/ML-based CSI compression in the field.

# Conclusion

In the previous sections, we discuss general aspects of AI/ML framework for NR air interface and make the following observations:

Observation 1: The Rel-18 study shows that using data generated from stochastic channel models is not sufficient to demonstrate the performance gain of CSI compression compared to non-AI/ML based algorithms.

Based on the discussion in the previous sections we propose the following:

Proposal 1: Site-specific AI/ML models for CSI compression should be considered to improve performance gain.

Proposal 2: Define a common reference scenario with site specificity as a basis for further study of AI/ML based CSI compression.

Proposal 3: Select one the following options to define a common reference scenario with site specificity as a basis for further study of AI/ML based CSI compression:

- Option 1: Real-scenario map that is a virtual representation of a real area on earth.

- Option 2: Synthetic-scenario map that is artificially constructed to mimic a certain environment such as urban macro, rural macro, indoor office, or indoor factory.

Proposal 4: With a common reference scenario with site specificity, ray tracing is used to generate channel data for the development and evaluation of site-specific AI/ML models for CSI compression.

Proposal 5: RAN1 to conclude that it is feasible to resolve issues related to inter-vendor training collaboration for AI/ML-based CSI compression.

Proposal 6: RAN1 to study post-deployment performance monitoring mechanisms to detect performance degradation and non-compliance to guarantee satisfactory performance of AI/ML-based CSI compression in the field.

# Reference

X. Lin, “The Bridge Toward 6G: 5G-Advanced Evolution in 3GPP Release 19,” arXiv preprint arXiv:2312.15174, December 2023.

RP-234039, “Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface,” 3GPP TSG RAN #102, December 2023.

METIS, “Deliverable D6.1 - Simulation guidelines,” April 2014.

# Appendix A: Evaluation results

In this appendix, we provide some evaluation results on CSI compression using AI/ML based algorithms. The system-level simulation assumption and scenarios are built on the basis of the RAN1 agreements.

We consider CSI compression and feedback with an autoencoder, which is a two-sided model including an AI/ML-based CSI generation part to generate the CSI feedback information and an AI/ML-based CSI reconstruction part which is used to reconstruct the CSI from the received CSI feedback information. The autoencoder consists of mainly convolutional layers and fully connected layers. The input to the autoencoder is the eigenvectors of the raw channel matrix.

We train and test the autoencoder for a range of compression ratios. The compression ratio is defined as the ratio of the number of real-valued channel eigenvector coefficients and the number of real-valued feedback coefficients.

Table 1 presents the performance results of the trained autoencoders in terms of the cosine similarity between true and reconstructed eigenvectors of raw channel matrix. For both cases, the results show that the higher the compression ratio, the higher the cosine similarity value. When the autoencoder is trained and tested on CDL-C, we can see that the cosine similarity is as high as 0.939 for a compression ratio as low as 1/64. When the autoencoder is trained and tested on dense urban scenario, we can see that the cosine similarity is 0.797 for a compression ratio of 1/16. Therefore, it is much easier for the autoencoders to compress CSI in CDL-C than in dense urban scenario. This is not surprising as the link level channel model CDL-C has fixed angle values and represents only a single channel realization while the system level channel in the dense urban scenario is much more sophisticated.

Table 1: Cosine similarity between true and reconstructed eigenvectors of raw channel matrix.

| CDL-C for training and testing | Compression ratio | 1/64 | 1/32 | 1/16 |
| --- | --- | --- | --- | --- |
|  | Cosine similarity | 0.939 | 0.976 | 0.989 |
| Dense urban for training and testing | Compression ratio | 1/16 | 1/8 | 1/4 |
|  | Cosine similarity | 0.797 | 0.845 | 0.891 |

Next, we explore the model generalization performance. Specifically, we use the autoencoders trained for dense urban scenario and test them using the channel data generated by CDL-C. Table 2 presents the performance results. We can see that the autoencoders trained in the sophisticated dense urban scenario perform well in CDL-C. We can see that the cosine similarity is 0.9358 for a compression ratio of 1/16.

Table 2: Generalization performance of CSI autoencoder: Training on dense urban and testing on CDL-C.

| Dense urban for training; CDL-C for testing | Compression ratio | 1/16 | 1/8 | 1/4 |
| --- | --- | --- | --- | --- |
|  | Cosine similarity | 0.9358 | 0.9655 | 0.9661 |

In contrast, we find in our experiments that the autoencoders trained in CDL-C do not perform well in the sophisticated dense urban scenario. This is not surprising as AI/ML models are only as good as the data they are trained on, and the autoencoders trained in CDL-C do not learn enough the channel data pattern in the sophisticated dense urban scenario.

For ease of visualization, we plot the results in Table 1 and Table 2 in the same figure below.

Figure 5: Performance of CSI autoencoder.

# Appendix B

## B.1	RAN1#116 agreements

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study:

| Case | Target CSI slot(s) | Whether the UE uses past CSI information | Whether the network uses past CSI information |
| --- | --- | --- | --- |
| 0 | Present slot | No | No |
| 1 | Present slot | Yes | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |
| 5 | Present slot | No | Yes |

Note 1: For the UE, the past CSI information may include past model inputs and/or any information derived from them. For the network, the past CSI information may include past CSI feedback instances and/or any information derived from them.

Note 2: For case 3 and case 4, the UE may perform prediction as a separate step or jointly with compression. Similarly, the network may perform prediction as a separate step or jointly with reconstruction. Companies to report which option is selected, the number of future slots, and whether the prediction is AI/ML-based or not.

Note 3: “Target CSI slot(s)” refers to the slot(s) to which the CSI feedback in the report corresponds. “Present slot” refers to the slot of the most recent CSI-RS measurement used to generate the CSI report. “Future slot(s)” includes at least one slot after the present slot and may include the present slot as well.

Note 4: Down-selection is not precluded.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following as baseline options for UE distribution:

- Option 1: 80% indoor, 20% outdoor

- Option 2: 100% outdoor

Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 10 km/h, 20 km/h, 30 km/h, 60 km/h, 120 km/h. Assumption on O2I car penetration loss and spatial consistency follow the R18 AI based CSI prediction.

Working Assumption

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following benchmark scheme for performance comparison:

- For cases without prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI compression study.

- For cases with prediction of future CSI, use the same benchmark scheme assumed in R18 AI/ML-based CSI prediction study, with R18 MIMO eType II codebook for compressing the feedback.

Agreement

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, study the following aspects of the performance/complexity trade-off when comparing the localized model with a benchmark model that is not localized:

- Performance of the localized model that has similar or lower complexity as the benchmark model.

- Model complexity of the localized model that achieves similar or better performance as the benchmark model.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following evaluation assumptions:

- CSI-RS configuration

  - Periodic: 5 ms periodicity (baseline), 20 ms periodicity(encouraged)

  - Aperiodic (for cases with prediction): Optional, CSI-RS burst with K resources and time interval m milliseconds (based on R18 MIMO eType-II)

- CSI reporting periodicity: {5, 10, 20} ms; other values are not precluded

- For cases with the use of past CSI information, to report observation window, including number/time distance of historic CSI/channel measurements.

- For cases with prediction, to report prediction window, including number/time distance of predicted CSI/channel.

Agreement

To alleviate / resolve the issues related to inter-vendor training collaboration of AI/ML-based CSI compression using two-sided model, study the following options:

- Option 1: Fully standardized reference model (structure + parameters)

- Option 2: Standardized dataset

- Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Option 5: Standardized model format + Reference model exchange between NW-side and UE-side

Note 1: The above options may not be mutually exclusive and may be used together.

Note 2: Other options are not precluded.

Note 3: The study should consider how different methods of exchanging the parameters / dataset / reference model would affect the feasibility and collaboration complexity of options 3 / 4 / 5 respectively, e.g., over the air-interface, offline delivery, etc.

Note 4: “Dataset” refers to a set of data samples of CSI feedback and associated target CSI.

Agreement

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

Note: While modelling the spatial correlation, strive to ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel. To report methods to generate training and testing dataset.

Agreement

- For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19,

  - adopt the CSI feedback overhead rate as reference, where the CSI feedback overhead rate is the average bit-rate of CSI feedback overhead across time.

Note: The CSI feedback overhead of a single report is calculated as in R18 CSI compression study.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for cases with prediction of future CSI, in which prediction and compression are separated, to optionally evaluate a scheme with ideal prediction as an additional evaluation case for reference.

Note: The ideal prediction scheme should model realistic channel estimation.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for Case 2, Case 4 and Case 5, study the performance impact resulting from non-ideal UCI feedback.

Agreement

For the study of inter-vendor collaboration issues for AI/ML-based CSI compression using a two-sided model, consider at least the following aspects when comparing different options:

- Inter-vendor collaboration complexity, e.g., whether bilateral collaboration is required between vendors.

- Performance.

- Interoperability and RAN4 / testing related aspects.

- Feasibility.

## B.2	RAN1#116bis agreements

Agreement

For the results template used to collect evaluation results for temporal domain compression Case 1/2/5, adopt Table 1 used in Rel-18 as starting point with the following additions:

- Temporal domain CSI setting

  - CSI feedback periodicity

  - CSI-RS periodicity

- Description of model input/output and Case

  - Compression case, e.g., Case 1/2/5

  - Usage of historical CSI at UE/NW side (e.g., number / time distance, eigen-vectors / raw channels, etc)

  - Methods to handle UCI loss (if applicable), e.g., CSI buffer reset, CSI retransmission, etc.

  - Methods to handle rank adaptation (if applicable)

- UE distribution (Option 1 or Option 2) and UE speed

- CSI feedback overhead rate: X/Y/Z bits per normalized time unit

  - Normalized time unit = 5ms and adopt same X/Y/Z values as in Table 1 of Rel-18

- Benchmark scheme

  - Rel-16 eT2 and compression Case 0 (i.e., Rel-18 AI/ML based CSI compression)

- Whether/how spatial consistency is modelled

- Whether/how UCI loss is modelled

  - The same UCI loss model shall be applied to the benchmark for fair comparison.

- Whether/how rank adaptation is modelled

- Modelling of channel estimation error

- Whether/how phase discontinuity is modelled (if applicable)

Agreement

For the results template used to collect evaluation results for temporal domain prediction and compression Case 3/4, adopt Table 1 used in Rel-18 as starting point with the following additions:

- Temporal domain CSI setting

  - CSI feedback periodicity

  - CSI-RS periodicity

- Description of model input/output and use case

  - Compression case, e.g., case 3 / 4

  - Observation window (usage of historical CSI at UE/NW side, e.g., number / time distance, eigen-vectors / raw channels, etc)

  - Prediction window (e.g., time distance between 1st prediction instance and last observation instance, number / time distance of predicted CSI)

  - Methods to handle UCI loss (if applicable)

- UE distribution (Option 1 or Option 2) and UE speed

- CSI feedback overhead rate: X/Y/Z bits per normalized time unit

  - Normalized time unit = 5ms and adopt same X/Y/Z values as in Table 1 of Rel-18

- SGCS values before (if applicable) and after compression

- Assumption on the prediction of future CSI

  - Separate step or jointly with compression

  - If separate, description of the AI or non-AI prediction algorithms: ideal prediction, AI-based prediction, non-AI-based prediction (e.g., nearest historical CSI and its location, learning window size / time correlation matrix size for auto-regression based prediction),

    - Note: the same prediction algorithm to be used for the benchmark scheme.

- Benchmark schemes

  - Description of feedback schemes, i.e., Rel-18 doppler eT2

- Whether/how spatial consistency is modelled

- Whether/how UCI loss is modelled

  - The same UCI loss model shall be applied to the benchmark for fair comparison.

- Modelling of channel estimation error

- Whether/how phase discontinuity is modelled (if applicable) Modelling of phase discontinuity

Agreement

- For Option 3, further define the two sub-options:

  - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- For Option 5, further define the two sub-options:

  - 5a: Model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing.

  - 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations.

- For Option 4, it is clarified that:

  - Dataset received at the UE or UE-side goes through offline engineering at the UE- side (e.g., UE-side OTT server), e.g., model training or offline testing.

- Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavours (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Agreement (amended as shown in red in Wednesday session)

- For Option 3/4/5, focus further discussion on the following assumptions:

  - Option 3a/5a

    - The model(5a)/parameter(3a) exchange originates from the NW-side and ends at the UE-side.

    - Model(5a)/parameters(3a) exchanged from the NW-side to UE-side is either CSI generation or reconstruction part or both.

      - Option 3a-1/5a-1: Model/Parameters exchanged from the NW-side to UE-side is CSI generation part.

      - Option 3a-2/5a-2: Model/Parameters exchanged from the NW-side to UE-side is CSI reconstruction part.

      - Option 3a-3/5a-3: Model/Parameters exchanged from the NW-side to UE-side are both CSI generation part and CSI reconstruction part.

      - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance.

        - Performance target

        - Dataset or information related to collecting dataset

    - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc.

  - Option 3b

    - The method of exchanging is over the air-interface via model transfer/delivery Case z4.

    - The parameter exchange is from NW to UE.

    - Parameters exchanged from the NW-side to UE-side is CSI generation part.

  - Option 5b

    - The method of exchanging is over the air-interface via model transfer/delivery Case z4, assuming that the model structure is aligned based on offline inter-vendor collaboration.

    - The model exchange is from NW to UE.

    - Model exchanged from the NW-side to UE-side is CSI generation part.

  - Option 4:

    - The dataset exchange originates from the NW-side and ends at the UE-side.

    - Option 4-1: Dataset exchanged from the NW-side to UE-side consists of (target CSI,  CSI feedback).

    - Option 4-2: Dataset exchanged from the NW-side to UE-side consists of (CSI feedback, reconstructed target CSI).

    - Option 4-3: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback, reconstructed target CSI).

    - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance.

      - Performance target

    - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc.

  - Note: For each option/sub-option of interest, companies to bring discussion on how inter-vendor collaboration complexity, interoperability, and feasibility may be addressed. Companies to strive to provide solution(s) that can address all the following aspects: inter-vendor collaboration complexity, performance, interoperability, and feasibility.

  - Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavours (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Agreement

For the results template used to collect evaluation results for AI/ML-based CSI compression using localized models, adopt Table 1 used in Rel-18 as starting point, capturing the generalized model result and the localized model result as separate columns, with the following additions for the localized model:

- Dataset description

  - Local region modelling: e.g., Option 1 or Option 2, and further details

  - Temporal modelling: e.g., how temporal variation is modelled in train and test sets

  - Dataset description for generalized model

Conclusion

In Rel-19 study of temporal domain aspects of AI/ML-based CSI compression using two-sided model, CSI prediction that is performed entirely at NW-side is deprioritized.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, for the temporal domain prediction and compression Case 3 and Case 4, adopt the following evaluation assumptions as baseline:

- Observation window (number/distance):

  - For periodic CSI-RS with 5ms periodicity: 12/5ms, 10/5ms, 8/5ms, 5/5ms, 4/5ms, unrestricted observation window

  - For periodic CSI-RS with 20ms periodicity: up to companies (encouraged)

  - For aperiodic CSI-RS: 12/2ms, 8/2ms, 4/2ms

  - Others can be additionally submitted

- Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance):  4/5ms/5ms

  - Others can be additionally submitted, e.g. 4/1ms/5ms, 8/1ms/5ms, 4/5ms/10ms, 1/-/5ms

Conclusion

For multi-vendor results table, adopt Rel-18 Table 4 for joint training and Rel-18 Table 5 for separate training as starting point, with the same additions of above 2 agreements.

Conclusion

For model generalization results table, adopt Rel-18 Table 2 and Generalization Case 1 / 2 / 3 as starting point with same additions above. For generalization aspects, adopt the following

- Various UE speed

- UE distribution

- Various CSI-RS periodicity

Conclusion

For model scalability results table, adopt Rel-18 Table 3 and Generalization Case 1 / 2 / 3 as starting point with same additions above. For generalization aspects, adopt the following

- Various numbers of antenna ports

- Various frequency granularity

- Various payload size

Conclusion:

- Conclude, from RAN1 perspective, that Option 1, if feasible for specification, eliminate the inter-vendor collaboration complexity (e.g., whether bilateral collaboration is required between vendors).

- It is RAN1’s understanding that Option 1 corresponds to RAN4 options, e.g., RAN4-Option3, or RAN4-Option4. Further study and final conclusion on interoperability and RAN4 testing of the RAN4-Option3 and RAN4-Option4 is up to RAN4.

Observation

- Option 1 and 2 may have limited performance in the field compared to Options 3, 4, and 5, further study is needed

- Option 1 and 2 may require high specification effort from RAN1 perspective.

Conclusion

- Deprioritize Option 2 for inter-vendor training collaboration.

  - Note: This deprioritization shall not affect the ongoing discussion in RAN4 on RAN4-Option3 and RAN4-Option4.

Agreement

For the results template used to collect evaluation results for temporal domain prediction and compression Case 4, adopt Table 1 used in Rel-18 as starting point with the following additions:

- Description of model input/output and use case

  - Methods to handle rank adaptation (if applicable)

## B.3	RAN1#117 agreements

Conclusion

Standardized signalling, if feasible and specified, can be used for parameter / model exchange in option 3a/5a and 3b to alleviate/resolve the inter-vendor training collaboration complexity.

- Standardized signalling may be reused for exchanging CSI generation part, CSI reconstruction part, or both, etc, when necessary and feasible.

- Standarized signalling may be over-the-air, or other approaches.

Standardized signalling, if feasible and specified, can be used for dataset exchange in option 4 to alleviate/resolve the inter-vendor training collaboration complexity.

- Standardized signalling may be reused for dataset exchanging, when necessary and feasible.

- Standardized signalling may be over-the-air, or other approaches.

Note: feasibility will be discussed separately.

Agreement

- For option 3a/3b/4/5a and their sub-options, at least the following potential specification impacts have been identified. Further study the necessity, feasibility, their specification impact.

- Exchange

  - Parameter / model exchange methods, format/contents, and related spec impacts (3a/3b/5a)

  - Dataset exchange methods, format/type/contents of data/dataset, and related spec impacts (4)

  - Additional information, if necessary, that may be shared from the NW-side to help UE-side offline engineering and provide performance guidance (3a/5a/4)

    - Performance target (3a/5a/4)

    - Dataset or information related to collecting dataset (3a/5a)

    - Any other additional information

- Model pairing (3a/3b/4/5a)

- UE capability (3a/3b/4/5a)

- Model related aspects, such as scalability (e.g., payload sizes, antenna ports, bandwidth), rank and layer handling (3a/3b/4/5a)

- Quantization of feedback (3a/3b/4/5a)

- Model structure details (3a/3b)

Note: Option 3a/4/5a and option 3b serve two different deployment time scales, UE capabilities, device-side optimizations, and training methods, and therefore may be complementary to each other, with potential specification of both.

- Specification of option 1, if needed from RAN1, can reuse specification of option 3a/3b, with the additional specification of parameters.

Agreement

For option 1 / 3 / 4 / 5 and their sub-options, study mechanisms (e.g., post-deployment performance monitoring) for identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation to guarantee good performance in the field.

Agreement

For temporal domain aspects Case 3/4, change the small / medium / large payload region definition as follows:

Note: X, Y, Z, A, B, and C are feedback overhead rates in bits per time unit of 5ms. Note: For X, Y, and Z, α=[2] for rank=1/2 and α=[4] for rank=4 Note: For A, B, and C, β=[0.5] for rank=1 and β=[0.75] for rank=2/4

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression (Cases 1-5), in addition to FLOPs, also consider FLOPs per normalized time unit. Use 5msec as the normalized time unit.

Agreement

In the results template for capturing the evaluation of temporal domain aspects Case 3/4 of AI/ML based CSI compression, regarding the “upper bound”, capture both of the following:

- upper bound based on ideal CSI prediction and without CSI compression

- upper bound based on benchmark CSI prediction and without CSI compression

Agreement

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, regarding training,

- The k-th local model is trained on region #B_k (the k-th local region), 1<=k<=N.

- The generalized model is trained on Region #A that may be constructed via any of the following methods that is appropriate for the given generalized/local region modeling approach.

  - Region #A is the same as the union of regions #B_1, …, #B_N.

  - Region #A is a proper superset of the union of regions #B_1, …, #B_N.

  - Region #A is generated separately from regions #B_1, …, #B_N.

  - Note: companies to report which method was used.

For the evaluation of AI/ML-based CSI compression using localized models in Release 19, regarding testing,

- The trained generalized model, local model, and the non-AI/ML benchmark are tested on the regions #B_1, …, #B_N.

- In case N>1, when reporting the results, companies may report the performance of the generalized model, the local models, and the non-AI/ML benchmark, by averaging the performance over the regions #B_1,…,B_N. Companies to report the value of N.

Agreement

For collecting evaluation results for temporal domain aspects of AI/ML-based CSI compression using localized models, use the same results template used to collect evaluation results for AI/ML-based CSI compression using localized models

- Adding the same temporal setting that is used for results template used to collect evaluation results for temporal domain compression Case 1/2/5.

| Temporal setting | Temporal domain aspect Case 1-5 |
| --- | --- |
|  | CSI-RS configuration: periodic or aperiodic For periodic: periodicity For aperiodic: # of resources K in the CSI-RS burst / time internal m in msec |
|  | CSI reporting periodicity |
|  | Usage of historical CSI at UE side: number / time distance |
|  | Usage of historical CSI at NW side: number / time distance |
|  | Prediction window: number / time distance between prediction instances / distance from the last observation instance to the 1st prediction instance (Only applicable to Case 3,4) |

Agreement

Further study following monitoring options in Rel-19, including the necessity and feasibility,

- NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability

  - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1)

  - SRS-based monitoring

- UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability

  - Based on the output of the CSI reconstruction model at the UE (Case 2-1)

    - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side.

  - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2)

  - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI

  - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model

  - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook

Regarding monitoring metrics:

- Monitoring accuracy also includes generalization considerations, if applicable.

- Complexity also includes LCM complexity, if applicable.

- Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions.

- Testability of UE reported metrics

Discussion may include the following aspects:

- Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration

- Temporal domain aspects of CSI compression

- How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation

Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches.

Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches.

Agreement

For temporal domain aspects Case 3 and 4, study the impact on LCM aspects of separate prediction and compression, and joint prediction and compression.

Note: Observations of companies results till RAN1#117 are captured in FL summary R1-2405419.