3GPP TSG RAN WG1 #118                                   R1-2406880

Maastricht, the Netherlands, August 19th – 23rd, 2024

Source:	Sharp

Title:	Discussion on downlink and uplink channel/signal aspects

Agenda Item:	9.4.2.3

Document for:	Discussion and Decision

Introduction

A new study item on solutions for Ambient IoT [1] for Rel-19 was approved in RAN#102, with objectives including the following:

- RAN1-led: For the Ambient IoT DL and UL: - [...] - Downlink channel/signal aspects - Uplink channel/signal aspects - [...]

In this document, we share our views on some aspects related to R2D (“downlink”) and D2R (“uplink”) channels/signals.

Discussion

R2D (Reader-to-device)

R2D channels/signals

The following was agreed in RAN1#117,

Agreement For the start-indicator part of the R2D time acquisition signal, study the two options below: - Option 1: ON/OFF pattern i.e. high/low voltage transmission - Option 2: OFF pattern, i.e. low voltage transmission

It seems that Option 2 assumes a high-voltage transmission immediately preceding the start-indicator as a prerequisite (i.e. similarly to the case of UHF RFID). We don’t think the situation in A-IoT is completely the same as in UHF RFID, e.g. when considering presence of energy storage in devices, and harmonized design for all device types, and so we don’t think the high-voltage transmission should be mandated for each and every R2D transmission. Therefore, we propose to go with Option 1.

- For the start-indicator part of the R2D time acquisition signal, support Option 1 (ON/OFF pattern i.e. high/low voltage transmission).

Control information

An R2D transmission can be intended for a single device, a group of devices, or all devices in coverage. A device with ultra-low power consumption in mind should be able to determine, with minimum effort, whether it is intended for by a particular R2D transmission, and only if so, it needs further processing of that R2D transmission. Therefore, it is highly desirable that R2D control information (carrying indication of destination devices, or additionally cast type) and higher layer payload are respectively mapped to two separate blocks of R2D chips.

- R2D control information at least includes device ID(s) or device group ID(s), or additionally cast type.

- R2D control information and higher layer payload are respectively mapped to two separate blocks of R2D chips (e.g. a first block of R2D chips for R2D control information immediately preceding a second block of R2D chips for higher layer payload).

R2D control information should also include information about the block of R2D chips for higher layer data, for example, R2D line coding code rate (e.g. 1 / number of R2D chips per bit), total number of R2D chips without repetition (or TBS), R2D repetition type, and number of R2D repetitions.

And R2D control information should also include information about the scheduled D2R transmission(s), for example, D2R chip duration (which can be indicated as a multiple or a fraction of R2D chip duration), D2R line coding code rate (e.g. 1 / number of D2R chips per bit), number of D2R chips without repetition (or TBS), D2R repetition type, number of D2R repetitions, and D2R frequency domain resource allocation (e.g. “large frequency-shift” for Device 2a, or RBs allocated for Device 2b).

- R2D control information includes the following information about the block of R2D chips for higher layer data: R2D line coding code rate (e.g. 1 / number of R2D chips per bit), total number of R2D chips without repetition (or TBS), R2D repetition type, and number of R2D repetitions.

- R2D control information includes the following information about the scheduled D2R transmission(s): D2R chip duration, D2R line coding code rate (e.g. 1 / number of D2R chips per bit), number of D2R chips without repetition (or TBS), D2R repetition type, number of D2R repetitions, and D2R frequency domain resource allocation (e.g. “large frequency-shift” for Device 2a, or RBs allocated for Device 2b).

TBS

As indicated in TR 38.848, “the design target of maximum message size is approximately 1000 bits to be received by the Ambient IoT device, and approximately 1000 bits to be transmitted from the Ambient IoT device, based on the maximum application layer packet size”, therefore, it makes sense to take a maximum TBS of 1000 bits as starting point for R2D data transmission.

- For a R2D data transmission, the maximum TBS is 1000 bits.

D2R (Device-to-reader)

TBS

With a similar reason for R2D in section 2.1.2, it makes sense to take a maximum TBS of 1000 bits as starting point for D2R data transmission.

- For a D2R data transmission, the maximum TBS is 1000 bits.

However, for some specific messages, e.g. the PDRCH transmission in response to a PRDCH transmission initiating a contention-based access procedure, since it is the first transmission of a device for the procedure, and a lot of devices may respond to the PRDCH transmission (potentially resulting in a collision), it is highly desirable that the payload size of the PDRCH transmission is strictly limited, and it may be desirable that the payload size is a pre-defined value.

- RAN1 to study an appropriate maximum payload size of the PDRCH transmission in response to a PRDCH transmission initiating a contention-based access procedure.

Conclusion

In this contribution, we discuss some aspects related to R2D (“downlink”) and D2R (“uplink”) channels/signals, and make the following proposals.

- For the start-indicator part of the R2D time acquisition signal, support Option 1 (ON/OFF pattern i.e. high/low voltage transmission).

- R2D control information at least includes device ID(s) or device group ID(s), or additionally cast type.

- R2D control information and higher layer payload are respectively mapped to two separate blocks of R2D chips (e.g. a first block of R2D chips for R2D control information immediately preceding a second block of R2D chips for higher layer payload).

- R2D control information includes the following information about the block of R2D chips for higher layer data: R2D line coding code rate (e.g. 1 / number of R2D chips per bit), total number of R2D chips without repetition (or TBS), R2D repetition type, and number of R2D repetitions.

- R2D control information includes the following information about the scheduled D2R transmission(s): D2R chip duration, D2R line coding code rate (e.g. 1 / number of D2R chips per bit), number of D2R chips without repetition (or TBS), D2R repetition type, number of D2R repetitions, and D2R frequency domain resource allocation (e.g. “large frequency-shift” for Device 2a, or RBs allocated for Device 2b).

- For a R2D data transmission, the maximum TBS is 1000 bits.

- For a D2R data transmission, the maximum TBS is 1000 bits.

- RAN1 to study an appropriate maximum payload size of the PDRCH transmission in response to a PRDCH transmission initiating a contention-based access procedure.

References

- RP-234058, “New SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, Huawei (moderator, RAN1 Vice-Chair), RAN#102.

- TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN”.