3GPP TSG RAN WG1 #118	R1-2407005

Maastricht, NL, Auguest 19th – 23rd, 2024

Source:	Sharp

Title:	Enhancement for asymmetric DL sTRP/UL mTRP scenarios

Agenda Item:	9.2.4

Document for:	Discussion and Decision

Background

In RAN#102 meeting, the Rel-19 WID on NR MIMO was approved [1], which includes the following objective corresponding to the asymmetric DL sTRP/UL mTRP scenarios:

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

In asymmetric DL sTRP/UL mTRP deployment scenarios, the UL TRP does not transmit any DL RSs (e.g., CSI-RS), which can result in potential information loss for the UE regarding channel variation and pathloss during UL transmission to the UL TRP. This contribution aims to present our considerations on pathloss offset configuration and pathloss offset for PRACH transmission toward the UL TRP and pathloss offset for PHR calculation.

Discussions

  - Pathloss offset configuration

In RAN1#117 meeting, the following conclusion is made.

Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

We think this conclusion is unclear regarding which joint TCI state is applied to a DL channel when the UE is indicated two joint TCI states. For example, when the UE has two indicated joint TCI states, and the TCI selection field is present in a DCI, either 1st indicated joint TCI state or 2nd indicated joint TCI state can be applied to a DL channel. In other words, the NW can dynamically switch between the 1st indicated joint TCI state and 2nd indicated joint TCI state for a DL transmission by using the TCI selection field even though only one DL TRP exist. This switching intends the dynamic point selection. However, we don’t need the dynamic point selection in this scenario.  Hence, we can easily define that the 1st indicated joint TCI state corresponds to the DL/UL TRP and 2nd indicated joint TCI state corresponds to the UL TRP.

Furthermore, according to the current spec, when the UE determines UL transmission power, a PL RS, which is mandatorily included in each indicated joint TCI state, is applied for each UL transmission. However, the UE does not receive the PL RS from the UL TRP and only one PL RS in either 1st indicated joint TCI state or 2nd joint indicated TCI state is applied for the UL transmission power. As aforementioned, we think the 1st joint indicated TCI state corresponds to the DL/UL TRP and the 2nd indicated joint TCI state corresponds to the UL TRP. Therefore, the PL RS in 1st indicated joint TCI state is applied for the UL transmission towards to both DL/UL TRP and UL TRP on the asymmetric DL sTRP/UL mTRP scenario.

Likewise, only one PL offset value included in either 1st indicated joint TCI state or 2nd indicated joint TCI state is used to determine UL transmission power even when the UE has two indicated joint TCI states. Since the 2nd indicated joint TCI state corresponds to the UL TRP, the PL offset should be configured only in the 2nd indicated joint TCI state.

Moreover, when the UE has two indicated jointTCI states, one or two indicated TCI state(s) can be applied to a DL reception. However, in asymmetric DL sTRP/UL mTRP scenario, only one indicated joint TCI state is applied to the DL reception. In our view, only the 1st indicated joint TCI state is applied to the DL reception on the asymmetric DL sTRP/UL mTRP scenario.

Proposal 1: 1st indicated TCI state should correspond to DL/UL TRP and 2nd indicated TCI state should correspond to UL TRP.

      - When two joint TCI states are indicated, DL PL RS in 1st indicated TCI state is applied for a UL transmission corresponding to both DL/UL TRP and UL TRP.

      - PL offset is configured in only 2nd indicated joint TCI state.

      - Only the 1st indicated joint TCI state is applied to DL reception.

Only the 1st indicated joint TCI state is applied to the DL reception on the asymmetric DL sTRP/UL mTRP scenario. In other words, the 1st indicated joint TCI state is always associated with the DL/UL TRP. However, according to the current spec, TCI selection field can be configured and indicates 1st or 2nd or both indicated TCI state applied to PDSCH reception. Hence, TCI selection field should not be configured on the asymmetric DL sTRP/UL mTRP scenario.

Proposal 2: TCI selection field should not be configured on the asymmetric DL sTRP/UL mTRP scenario.

  - Pathloss offset for PDCCH order

In RAN1#117 meeting, the following agreements are made.

Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1#118 meeting: - Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission - FFS: the details of DCI field design.

For Alt1, based on the current specification (i.e., TS 38.213) without considering two TA cases, the path loss used for a PDCCH-order PRACH transmission can be based on the DL RS with which the DM-RS of the PDCCH is quasi-collocated. After calculating the path loss, if the PRACH transmission is directed toward the UL TRP, the UE will apply the PL offset indicated by the DCI to compensate for the path loss during the RACH procedure. This PL offset is one of the PL offsets configured by the RRC. The size of the DCI field indicating the path loss offset for the PRACH transmission toward the UL TRP will be determined by the number of configured path loss offsets.

For Alt3, a unified path loss offset configuration can be applied for all UL transmissions (e.g., PUSCH, PUCCH, SRS, and PRACH) toward a UL TRP. The main issue with Alt3 is how to inform the UE whether the PDCCH-order PRACH is initiated for a UL TRP and whether the dedicated preamble is transmitted toward the UL TRP. If the PDCCH-order PRACH transmission is not associated with the UL TRP, the path loss can be determined in the legacy way. If the PDCCH-order PRACH transmission is associated with the UL TRP, the path loss can be determined by the PL RS and the path loss offset corresponding to the indicated joint/UL TCI state for the UL TRP within the unified TCI framework. This issue, i.e., identifying whether the PDCCH-order PRACH transmission is associated with the UL TRP, can be easily resolved by using a one-bit DCI field. It should be noted that the reserved bits in the corresponding DCI format are limited. Additionally, we have already agreed that the PL offset can be associated with the UL/joint TCI state, so discussing another way to configure the PL offset for a specific UL transmission (e.g., PDCCH-order PRACH transmission) may not be necessary. Hence, we prefer Alt3.

Proposal 3: We support Alt 3, i.e., For PDCCH-order PRACH transmission, the PL offset should be associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission.

Proposal 4: Introduce a one-bit field in the DCI to indicate whether the corresponding PRACH is transmitted toward UL TRP.

  - Pathloss offset for PHR calculation

In RAN1 #117 meeting, the following agreements are made.

Agreement For the asymmetric DL sTRP/UL mTRP scenarios, study whether/how to consider PL offset in PHR calculation, including Type 1 PHR based on actual PUSCH transmission, Type 1 PHR based on reference PUSCH, Type 3 PHR based on actual SRS and Type 3 PHR based on reference SRS

For Type 1 PHR based on actual PUSCH transmission, whether the PL offset is applied for PHR calculation can be determined by the indicated UL/joint TCI state applied for the actual PUSCH transmission. If the indicated UL/joint TCI state is associated with a PL offset, it is natural to apply the PL offset for PHR calculation; otherwise, the PHR is calculated without the PL offset.

Proposal 5: For Type 1 PHR based on actual PUSCH transmission, apply PL offset for the PHR calculation and the calculation is based on the indicated joint/UL TCI state is associated with a PL offset.

For Type 1 PHR based on reference PUSCH, if ul-powerControl is not provided, the UE applies the default behavior to determine the PL RS used to calculate PHR. That is, the PL RS is obtained using pusch-PathlossReferenceRS-Id = 0 and the PHR can be calculated without a PL offset. However, if ul-powerControl is provided and the PL RS is associated with the indicated joint/UL TCI state, a PL offset is required to apply for PHR calculation when the indicated joint/UL TCI state is associated with a PL offset.

Proposal 6: For Type 1 PHR based on reference PUSCH transmission, if ul-powerControl is not provided, the PHR is calculated without PL offset.

Proposal 7: For Type 1 PHR based on reference PUSCH transmission, if ul-powerControl is provided, whether to apply PL offset for the PHR calculation is based on whether the indicated joint/UL TCI state is associated with a PL offset.

In our view, Type 3 PHR is basically used for DL CA on the TDD operation. However, the NW cannot identify the SRS transmission power precisely because there is a case that a DCI including a TPC command is dropped. For this reason, we think the Type 3 PHR is also useful to adjust a PL offset value without the DL measurement. For Type 3 PHR based on actual SRS transmission, similar to the Type 1 PHR based on actual PUSCH transmission, whether the PL offset is applied for PHR calculation can be determined by the indicated UL/joint TCI state applied for the actual SRS transmission.

Proposal 8: For Type 3 PHR based on actual SRS transmission, whether to apply PL offset for the PHR calculation is based on whether the indicated joint/UL TCI state is associated with a PL offset.

For Type 3 PHR based on reference SRS transmission, the default behavior will be applied to determine the PL RS used to calculate PHR. That is, the PL RS is obtained from the corresponding value in the SRS-ResourceSetId-Id = 0. In this case, applying pathloss offset to calculate PHR may not be necessary.

Proposal 9: For Type 3 PHR based on actual SRS transmission, the PHR is calculated without PL offset

  - Two TA

In RAN1#117 meeting, the following proposal remains:

Proposal 3.1: To fulfil the asymmetric DL sTRP/UL mTRP deployment scenarios, support two TAs for single DCI based multi-TRP/panel and single TRP. - Reuse Rel-18 specification of two TA for multi-DCI based multi-TRP/panel and remove the restriction that coresetPoolIndex needs to be configured.

In our view, supporting of two TAs for single DCI based multi-TRP is not out-of-scope. On the asymmetric DL sTRP/UL mTRP scenario, an additional UL TRP is deployed to compensate for the UL coverage. That is why the PL offset is also introduced on this scenario. Likewise, we think the propagation delay between the UE and UL TRP is highly different from the one between the UE and DL/UL TRP. Therefore, we can support two TAs for the asymmetric DL sTRP/UL mTRP.

Furthermore, we think the Rel-18 specification of two TAs for multi-DCI based multi-TRP can be almost reused because a TAG ID is included in each indicated TCI state.

Proposal 10: Support two TAs for single DCI based multi-TRP and single TRP.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: 1st indicated TCI state should correspond to DL/UL TRP and 2nd indicated TCI state should correspond to UL TRP.

- When two joint TCI states are indicated, DL PL RS in 1st indicated TCI state is applied for a UL transmission corresponding to both DL/UL TRP and UL TRP.

- PL offset is configured in only 2nd indicated joint TCI state.

- Only the 1st indicated joint TCI state is applied to DL reception.

Proposal 2: TCI selection field should not be configured on the asymmetric DL sTRP/UL mTRP scenario.

Proposal 3: For PDCCH-order PRACH transmission, the PL offset should be associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission.

Proposal 4: If we go for Alt3, introduce a one-bit field in the DCI to indicate whether the corresponding PRACH is transmitted toward UL TRP.

Proposal 5: For Type 1 PHR based on actual PUSCH transmission, whether to apply PL offset for the PHR calculation is based on whether the indicated joint/UL TCI state is associated with a PL offset.

Proposal 6: For Type 1 PHR based on reference PUSCH transmission, if ul-powerControl is not provided, the PHR is calculated without PL offset.

Proposal 7: For Type 1 PHR based on reference PUSCH transmission, if ul-powerControl is provided, whether to apply PL offset for the PHR calculation is based on whether the indicated joint/UL TCI state is associated with a PL offset.

Proposal 8: For Type 3 PHR based on actual SRS transmission, whether to apply PL offset for the PHR calculation is based on whether the indicated joint/UL TCI state is associated with a PL offset.

Proposal 9: For Type 3 PHR based on actual SRS transmission, the PHR is calculated without PL offset

Proposal 10: Support two TAs for single DCI based multi-TRP and single TRP.

- References

- [1]	RP-234007, New WID: NR MIMO Phase 5, Dec., 2023.