3GPP TSG RAN WG1 #118		                                                      R1-2407367

Maastricht, NL, August 19th – 23rd, 2024

Source:	Moderator (CATT)

Title:	Summary of rate matching pattern for multicast reception in RRC_INACTIVE state

Agenda Item:	8.1

Document for:	Discussion and Decision

- Introduction

This contribution aims to collect company’s opinions on the draft CR [1] on rate matching pattern for multicast reception in RRC_INACTIVE state.

- Discussion

Reason for change:

When UE is configured to receive broadcast, REs indicated by RateMatchPatternLTE-CRS are not available for broadcast reception. For multicast reception in RRC_INACTIVE_state, the same scheme should be reused. But in current specification, it is not clear whehter REs indicated by RateMatchPatternLTE-CRS are available or not for multicast reception in RRC_INACTIVE_state.

Proposed TP on TS 38.214:

5.1.4.2 PDSCH resource mapping with RE level granularity The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModListDCI-1-2 instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList. The procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 1_3. The procedures for PDSCH scheduled by PDCCH with DCI format 1_0 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_1 and the procedures for PDSCH scheduled by PDCCH with DCI format 1_1 described in this clause equally apply to PDSCH scheduled by PDCCH with DCI format 4_2, by applying the parameters of aperiodicZP-CSI-RS-ResourceSetsToAddModList in pdsch-ConfigMulticast instead of aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config. A UE may be configured with any of the following higher layer parameters: - REs indicated by the 'RateMatchPatternLTE-CRS' in lte-CRS-ToMatchAround in ServingCellConfig or ServingCellConfigCommon configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - REs indicated by 'RateMatchPatternLTE-CRS' in lte-CRS-PatternList1-r16 or lte-CRS-PatternList3-r18 in ServingCellConfig configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for PDSCH. - For the UE for broadcast reception or multicast reception in RRC_INACTIVE_state, REs indicated by 'RateMatchPatternLTE-CRS' in pdsch-ConfigMCCH or pdsch-ConfigMTCH configuring cell-specific RS, in 15 kHz subcarrier spacing applicable only to 15 kHz subcarrier spacing PDSCH, of one LTE carrier in a serving cell are declared as not available for broadcast PDSCH or multicast PDSCH reception in RRC_INACTIVE_state. The total number of RateMatchPatternLTE-CRS for broadcast reception or multicast reception in RRC_INACTIVE_state that a UE can be configured with is the same as for unicast in Rel-15. < Unchanged parts are omitted >

### Discussion

Companies provide your views in the following table:

| Company | Comments |
| --- | --- |
| Moderator | Based on offline discussion, the draft CR can be endorsed as an alignment CR. |

- Conclusion

The draft CR has endorsed on Tuesday online session as follow:

Agreement

The draft CR in R1-2406338 is endorsed for the TS 38.214 editor’s alignment CR for Rel-18.

- References

- R1-2406338 Draft CR on rate matching pattern for multicast reception in RRC_INACTIVE state, CATT, CBN, China broadnet