3GPP TSG-RAN WG1 Meeting #118	R1-2407569

Maastricht, The Netherlands, August 19 - 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.211 | CR | 0139 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alignment of parameter names |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core, NR_feMIMO-Core |  |  |  |  |  | Date: |  |  | 2024-08-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Misalignment of RRC parameter names between 38.211 and 38.331 - For MBS PDSCH DMRS sequence generation, the configured ID used for initializing the sequence generator is the same one for MBS broadcast and multicast, which is not aligned with the agreement. In addition, the parameter connfigured for PDSCH scrambling is not clear whether it is for broadcast or for multicast. (R1-2407265) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Aligning RRC parameter names with 38.331 - Differentiate the ID used for initializing PDSCH DMRS sequence generator for MBS broadcast from that for multicast. Differentiate the ID used for PDSCH scrambling for MBS broadcast from that for multicast. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Inconsistent parameter names across specifications. - gNB may have different interpretations with UEs regarding which ID is used for the multicast or for the broadcast. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.1.4.2, 7.3.1.1, 7.4.1.1.1, 7.4.1.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.4.1.4.2	Sequence generation

The sounding reference signal sequence for an SRS resource shall be generated according to

r(pi)n,l'=ru,v(αi,δ)n

0≤n≤Msc,bSRS-1

l'∈0,1,…,NsymbSRS-1

where Msc,bSRS is given by clause 6.4.1.4.3, ru,vα,δ(n) is given by clause 5.2.2 with δ=log2KTC and the transmission comb number KTC∈2,4,8 is contained in the higher-layer parameter transmissionComb. The cyclic shift αi for antenna port pi is given as

αi=2πnSRScs,inSRScs,max
nSRScs,i=nSRScs+nSRScs,maxpi-10002NapSRS2 mod nSRScs,maxif NapSRS=4 and nSRScs,max=6nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,maxotherwise,

where nSRScs∈0,1,…,nSRScs,max-1 is contained in the higher layer parameter transmissionComb. The maximum number of cyclic shifts nSRScs,max are given by Table 6.4.1.4.2-1.

The sequence group u=fghns,fμ,l'+nIDSRSmod 30 and the sequence number v in clause 5.2.2 depends on the higher-layer parameter groupOrSequenceHopping in the SRS-Resource IE or the SRS-PosResource IE. The SRS sequence identity nIDSRS∈0, 1, …, 65535nIDSRS is given by the higher layer parameter sequenceId in the SRS-Resource IEnIDSRS∈0, 1, …, 1023nIDSRS∈0, 1, …, 65535. The quantity l'∈0,1,…,NsymbSRS-1 is the OFDM symbol number within the SRS resource.

-	if groupOrSequenceHopping equals 'neither', neither group, nor sequence hopping shall be used and

-	if groupOrSequenceHopping equals 'groupHopping', group hopping but not sequence hopping shall be used and

where the pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDSRS at the beginning of each radio frame.

-	if groupOrSequenceHopping equals 'sequenceHopping', sequence hopping but not group hopping shall be used and

where the pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDSRS at the beginning of each radio frame.

Table 6.4.1.4.2-1: Maximum number of cyclic shifts nSRScs,max as a function of KTC.

| KTC | nSRScs,max |
| --- | --- |
| 2 | 8 |
| 4 | 12 |
| 8 | 6 |

#### 7.3.1.1	Scrambling

Up to two codewords  can be transmitted. In case of single-codeword transmission, .

For each codeword , the UE shall assume the block of bits bq0, …, bq(Mbitq-1), where Mbit(q) is the number of bits in codeword  transmitted on the physical channel, are scrambled prior to modulation, resulting in a block of scrambled bits bq0, …, bq(Mbitq-1)according to

b(q)i=b(q)i+c(q)(i) mod 2

where the scrambling sequence cq(i) is given by clause 5.2.1. The scrambling sequence generator shall be initialized with

cinit=nRNTI⋅215+q⋅214+nID

where

-	 equals the higher-layer parameter dataScramblingIdentityPDSCH if configured and the RNTI equals the C-RNTI, MCS-C-RNTI, or CS-RNTI, and the transmission is not scheduled using DCI format 1_0 in a common search space;

-	nID∈0,1,…,1023 equals the higher-layer parameter dataScramblingIdentityPDSCH in pdsch-ConfigMulticast if configured in a common MBS frequency resource for multicast and the RNTI equals the G-RNTI or G-CS-RNTI;

-	nID∈0,1,…,1023 equals the higher-layer parameter dataScramblingIdentityPDSCH in pdsch-ConfigMCCH or pdsch-ConfigMTCH if configured in a common MBS frequency resource for broadcast and the RNTI equals the MCCH-RNTI or G-RNTI, respectively;

-	nID∈0,1,…,1023 equals

-	the higher-layer parameter dataScramblingIdentityPDSCH if the codeword is scheduled using a CORESET with CORESETPoolIndex equal to 0;

-	the higher-layer parameter dataScramblingIdentityPDSCH2 if the codeword is scheduled using a CORESET with CORESETPoolIndex equal to 1;

if the higher-layer parameters dataScramblingIdentityPDSCH and dataScramblingIdentityPDSCH2 are configured together with the higher-layer parameter CORESETPoolIndex containing two different values, and the RNTI equals the C-RNTI, MCS-C-RNTI, or CS-RNTI, and the transmission is not scheduled using DCI format 1_0 in a common search space;

-	nID=NIDcell otherwise

and where  corresponds to the RNTI associated with the PDSCH transmission as described in clause 5.1 of [6, TS 38.214].

##### 7.4.1.1.1	Sequence generation

The UE shall assume the sequence rn is defined by

.

where the pseudo-random sequence ci is defined in clause 5.2.1. The pseudo-random sequence generator shall be initialized with

cinit=217Nsymbslotns,fμ+l+12NIDnSCIDλ+1+217λ2+2NIDnSCIDλ+nSCIDλmod 231

where l is the OFDM symbol number within the slot, ns,fμ  is the slot number within a frame, and

-	NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_1 or 1_2 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI;

-	NID0∈0,1,…,65535 is given by the higher-layer parameter scramblingID0 in the DMRS-DownlinkConfig IE if provided and the PDSCH is scheduled by PDCCH using DCI format 1_0 with the CRC scrambled by C-RNTI, MCS-C-RNTI, or CS-RNTI;

-	NID0,NID1∈0,1,…,65535 are given by the higher-layer parameters scramblingID0 and scramblingID1, respectively, in the DMRS-DownlinkConfig IE in pdsch-ConfigMulticast if provided in a common MBS frequency resource for multicast and the PDSCH is scheduled by PDCCH using DCI format 4_2 with the CRC scrambled by G-RNTI or G-CS-RNTI;

-	NID0∈0,1,…,65535 is given by the higher-layer parameter scramblingID0 in the DMRS-DownlinkConfig IE in pdsch-ConfigMulticast if provided in a common MBS frequency resource for multicast and the PDSCH is scheduled by PDCCH using DCI format 4_1 with the CRC scrambled by G-RNTI or G-CS-RNTI;

-	NID0∈0,1,…,65535 is given by the higher-layer parameter scramblingID0 in pdsch-ConfigMCCH or pdsch-ConfigMTCH if provided in a common MBS frequency resource for broadcast and the PDSCH is scheduled by PDCCH with the CRC scrambled by MCCH-RNTI or G-RNTI, respectively;

-	NIDnSCIDλ=NIDcell  otherwise;

-	nSCIDλ and λ are given by

-	if the higher-layer parameter dmrs-Downlink in the DMRS-DownlinkConfig IE is provided

nSCIDλ=nSCIDλ=0 or λ=21-nSCIDλ=1

λ=λ

where λ is the CDM group defined in clause 7.4.1.1.2.

-	otherwise by

nSCIDλ=nSCID

λ=0

The quantity nSCID∈0, 1 is given by the DM-RS sequence initialization field, if present, in the DCI associated with the PDSCH transmission if DCI format 1_1, 1_2, or 4_2 in [4, TS 38.212] is used, otherwise nSCID=0.

##### 7.4.1.5.3	Mapping to physical resources

For each CSI-RS configured, the UE shall assume the sequence  being mapped to resources elements k,lp,μ according to

when the following conditions are fulfilled:

-	the resource element k,lp,μ is within the resource blocks occupied by the CSI-RS resource for which the UE is configured

The reference point for k=0 is subcarrier 0 in common resource block 0.

The value of ρ is given by the higher-layer parameter density in the CSI-RS-ResourceMapping IE or the CSI-RS-CellMobility IE and the number of ports X is given by the higher-layer parameter nrofPorts. For NZP CSI-RS configured by the TRS-ResourceSet IE, the density ρ=3 and number of ports X=1.

The UE is not expected to receive CSI-RS and DM-RS on the same resource elements.

The UE shall assume  for a non-zero-power CSI-RS where  is selected such that the power offset specified by the higher-layer parameter powerControlOffsetSS in the NZP-CSI-RS-Resource IE or in the TRS-ResourceSet IE, if provided, is fulfilled.

The quantities k', l', , and  are given by Tables 7.4.1.5.3-1 to 7.4.1.5.3-5 where each k,l in a given row of Table 7.4.1.5.3-1 corresponds to a CDM group of size 1 (no CDM) or size 2, 4, or 8. The CDM type is provided by the higher layer parameter cdm-Type in the CSI-RS-ResourceMapping IE. For NZP CSI-RS configured by the TRS-ResourceSet IE, the CDM type is 'noCDM'. The indices k' and l' index resource elements within a CDM group.

The time-domain locations l0∈0,1, …, 13 and l1∈2, 3, …, 12 are provided by the higher-layer parameters firstOFDMSymbolInTimeDomain and firstOFDMSymbolInTimeDomain2, respectively, in the CSI-RS-ResourceMapping IE or the CSI-RS-ResourceConfigMobility IE and defined relative to the start of a slot. For NZP CSI-RS configured by TRS-ResourceSet IE, the time-domain location l0∈0,1, …, 13 is provided by the higher-layer parameter firstOFDMSymbolInTimeDomain or firstOFDMSymbolInTimeDomain+4.

The frequency-domain location is given by a bitmap provided by the higher-layer parameter frequencyDomainAllocation in the CSI-RS-ResourceMapping IE, the CSI-RS-ResourceConfigMobility IE, or the TRS-ResourceSet IE, with the bitmap and value of ki in Table 7.4.1.5.3-1 given by

-	, ki-1=fi for row 1 of Table 7.4.1.5.3-1

-	, ki-1=fi for row 2 of Table 7.4.1.5.3-1

-	, ki-1=4fi for row 4 of Table 7.4.1.5.3-1

-	, ki-1=2fi for all other cases

where  is the bit number of the  bit in the bitmap set to one, repeated across every 1ρ of the resource blocks configured for CSI-RS reception by the UE. The starting position and number of the resource blocks in which the UE shall assume that CSI-RS is transmitted are given by the higher-layer parameters freqBand and density in the CSI-RS-ResourceMapping IE for the bandwidth part given by the higher-layer parameter BWP-Id in the CSI-ResourceConfig IE or given by the higher-layer parameters nrofPRBs in the CSI-RS-CellMobility IE where the the startPRB given by csi-rs-MeasurementBW is relative to common resource block 0. For NZP CSI-RS configured by TRS-ResourceSet IE, the starting position and number of the resource blocks in which the CSI-RS can be transmitted are given by the higher-layer parameters nrofRBs, and startingRB in the TRS-ResourceSet IE, where startingRB is relative to common resource block 0 and the density ρ=3.

The UE shall assume that a CSI-RS is transmitted using antenna ports  numbered according to

where  is the sequence index provided by Tables 7.4.1.5.3-2 to 7.4.1.5.3-5,  is the CDM group size, and  is the number of CSI-RS ports. The CDM group index  given in Table 7.4.1.5.3-1 corresponds to the time/frequency locations  for a given row of the table. The CDM groups are numbered in order of increasing frequency domain allocation first and then increasing time domain allocation.

For a CSI-RS resource configured as periodic or semi-persistent by the higher-layer parameter resourceType, configured by the higher-layer parameter CSI-RS-CellMobility or configured by the higher-layer parameter TRS-ResourceSet, the UE shall assume that the CSI-RS is transmitted in slots satisfying

where the periodicity  (in slots) and slot offset  are obtained from the higher-layer parameter CSI-ResourcePeriodicityAndOffset, slotConfig or periodicityAndOffset. The UE shall assume that CSI-RS is transmitted in a candidate slot as described in clause 11.1 of [5, TS 38.213], clause 10.4B of [5, TS 38.213].

The UE may assume that antenna ports within a CSI-RS resource are quasi co-located with QCL Type A, Type D (when applicable), and average gain.

Table 7.4.1.5.3-1: CSI-RS locations within a slot.

| Row | Ports | Density | cdm-Type |  | CDM group index |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 3 | noCDM | k0,l0, k0+4,l0, k0+8,l0 | 0,0,0 | 0 | 0 |
| 2 | 1 | 1, 0.5 | noCDM | k0,l0, | 0 | 0 | 0 |
| 3 | 2 | 1, 0.5 | fd-CDM2 | k0,l0, | 0 | 0, 1 | 0 |
| 4 | 4 | 1 | fd-CDM2 | k0,l0, k0+2,l0 | 0,1 | 0, 1 | 0 |
| 5 | 4 | 1 | fd-CDM2 | k0,l0, k0,l0+1 | 0,1 | 0, 1 | 0 |
| 6 | 8 | 1 | fd-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0 | 0,1,2,3 | 0, 1 | 0 |
| 7 | 8 | 1 | fd-CDM2 | k0,l0, k1,l0, k0,l0+1, k1,l0+1 | 0,1,2,3 | 0, 1 | 0 |
| 8 | 8 | 1 | cdm4-FD2-TD2 | k0,l0, k1,l0 | 0,1 | 0, 1 | 0, 1 |
| 9 | 12 | 1 | fd-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0, k4,l0, k5,l0 | 0,1,2,3,4,5 | 0, 1 | 0 |
| 10 | 12 | 1 | cdm4-FD2-TD2 | k0,l0, k1,l0, k2,l0 | 0,1,2 | 0, 1 | 0, 1 |
| 11 | 16 | 1, 0.5 | fd-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0, k0,l0+1, k1,l0+1, k2,l0+1, k3,l0+1 | 0,1,2,3, 4,5,6,7 | 0, 1 | 0 |
| 12 | 16 | 1, 0.5 | cdm4-FD2-TD2 | k0,l0, k1,l0, k2,l0, k3,l0 | 0,1,2,3 | 0, 1 | 0, 1 |
| 13 | 24 | 1, 0.5 | fd-CDM2 | k0,l0, k1,l0, k2,l0, k0,l0+1, k1,l0+1, k2,l0+1,k0,l1, k1,l1, k2,l1, k0,l1+1, k1,l1+1, k2,l1+1 | 0,1,2,3,4,5, 6,7,8,9,10,11 | 0, 1 | 0 |
| 14 | 24 | 1, 0.5 | cdm4-FD2-TD2 | k0,l0, k1,l0, k2,l0, k0,l1, k1,l1, k2,l1 | 0,1,2,3,4,5 | 0, 1 | 0, 1 |
| 15 | 24 | 1, 0.5 | cdm8-FD2-TD4 | k0,l0, k1,l0, k2,l0 | 0,1,2 | 0, 1 | 0, 1, 2, 3 |
| 16 | 32 | 1, 0.5 | fd-CDM2 | k0,l0, k1,l0, k2,l0, k3,l0,k0,l0+1, k1,l0+1, k2,l0+1, k3,l0+1, k0,l1, k1,l1, k2,l1, k3,l1, k0,l1+1, k1,l1+1, k2,l1+1, k3,l1+1 | 0,1,2,3, 4,5,6,7, 8,9,10,11, 12,13,14,15 | 0, 1 | 0 |
| 17 | 32 | 1, 0.5 | cdm4-FD2-TD2 | k0,l0, k1,l0, k2,l0, k3,l0, k0,l1, k1,l1, k2,l1, k3,l1 | 0,1,2,3,4,5,6,7 | 0, 1 | 0, 1 |
| 18 | 32 | 1, 0.5 | cdm8-FD2-TD4 | k0,l0, k1,l0, k2,l0, k3,l0 | 0,1,2,3 | 0,1 | 0,1, 2, 3 |

Table 7.4.1.5.3-2: The sequences  and  for cdm-Type equal to 'noCDM'.

| Index | wf0 | wt0 |
| --- | --- | --- |
| 0 | 1 | 1 |

Table 7.4.1.5.3-3: The sequences  and  for cdm-Type equal to 'fd-CDM2'.

| Index | wf0wf1 | wt0 |
| --- | --- | --- |
| 0 |  | 1 |
| 1 |  | 1 |

Table 7.4.1.5.3-4: The sequences  and  for cdm-Type equal to 'cdm4-FD2-TD2'.

| Index | wf0wf1 | wt0wt1 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |

Table 7.4.1.5.3-5: The sequences  and  for cdm-Type equal to 'cdm8-FD2-TD4'.

| Index | wf0wf1 | wt0wt1wt2wt3 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  |  |
| 2 |  |  |
| 3 |  |  |
| 4 |  |  |
| 5 |  |  |
| 6 |  |  |
| 7 |  |  |