3GPP RAN1#118-bis	 R1- 2408308

14-18 Oct 2024, Beijing, Chongqing, Hong Kong, Urumqi

Agenda item:   9.4.2.1

Source:             Lekha Wireless Solutions

Title:                  On General Physical Layer Design Considerations for Ambient IoT (internet of things) Applications

Document for:  Decision, Discussion

1. Introduction

In RAN1#118 [1], general aspects of physical layer design were discussed, and the following agreements were made.

Agreement For D2R FEC study, the LTE convolutional code polynomials are a reference. Other designs can be studied subject to: - Constraint length K = 8 or K = 7 or K = 6 or K = 4 for further study - Mother code-rate R = 1/6, 1/4, 1/3, 1/2 for further study - FFS other details, e.g. final code rate by puncturing, shift-register initialization/termination. Agreement For D2R line codes, the study assumes the following codewords corresponding to an information bit 0 or bit 1, before considering potential small frequency-shifting: - For FM0: - According to Figures 6-8 and 6-9 of UHF RFID standard - For Miller: - According to Figure 6-12 of UHF RFID standard. Agreement - OOK and Binary PSK for baseband modulation are feasible for D2R for all devices. - The variant of Binary FSK to study further for all devices is identified as MSK (and not GMSK) - FFS: whether MSK is feasible for all devices

This contribution presents our views on physical layer design aspects for ambient IoT applications.

2. Discussion

2.1 R2D Line Coding – The below points were agreed upon in RAN1#116-bis [2]:

- Line codes studied are Manchester encoding and pulse-interval encoding (PIE).

- Mapping(s) from bit(s) to line-code codewords.

- Time domain definition of chips and relation to OFDM symbols, resource allocation unit, etc.

- The SI intends to further down select between Manchester encoding and PIE.

| Comparison between Manchester encoding and PIE |  |
| --- | --- |
| Manchester Encoding | Pulse Interval Encoding |
| In telecommunication and data storage, Manchester code (also known as phase encoding, or PE) is a line code in which the encoding of each data bit is either low then high, or high then low, for equal time. | Pulse interval encoding (PIE) is a method of transmitting data to an RFID tag by emitting pulses of energy at varying intervals, to indicate the ones and zeroes of binary code stored on that tag. |
| It is a self-clocking signal with no DC component. | The messages sent are short compared to the repeat time between messages which allows the receiver to relax to the centre of its detection range between messages. |
| Manchester encoding extracts a price for its use because it requires two times the bandwidth of the actual data rate being sent through the communications channel. | It is asymmetric in pulse width for each symbol which is more sensitive to time delay noise, resulting in higher threshold of detection and lower bit rates |
| It is superior for noise immunity meaning it has a much-improved bit error rate (BER) at low SNRs. | It can be incredibly low power and low cost for the Rx which can be passive or use energy stored from the carrier to power a short burst on its TX for RFID response. |

Observation 1 - PIE offers promise of low power and low cost but suffers from false edge detection and higher transmit time, whereas redundant coding like Manchester although it uses more spectrum due to the self-clocking of redundancy of transmitting a full clock cycle per bit, can have better data integrity or BER.

Proposal 1 – Owing to its drawbacks like false edge detection and higher transmit time, pulse interval encoding doesn’t seem to be a good fit for A-IoT R2D line-coding. Hence, Manchester is more preferred in this scenario.

2.2 D2R Line Coding – The below points were agreed upon in RAN1#116-bis [2]:

- Study the following for D2R line coding: Manchester encoding, FM0 encoding, Miller encoding, no line coding.

- Mapping from bit(s) to line code(s) codewords.

- Study must consider- spectrum shape, power consumption and complexity, BER and BLER, resilience to SFO, any relation to CFO.

The features, benefits and limitations of Manchester encoding can be understood from the above table. Below table gives a comparison of Miller and FM0 encoding. The time domain transitions of the line codes are given in Fig 1. The spectral plots are given in Fig 2.

Fig 1: Time domain plots of Miller, Manchester and FM0 code

| Comparison of Miller and FM0 Encoding |  |
| --- | --- |
| Miller Encoding | FM0 Encoding |
| Miller encoding is also known as delay encoding. It can be used for higher operating frequency, and it is similar to Manchester encoding except that the transition occurs in the middle of an interval when the bit is 1. | FM0 is also known as Bi-phase space encoding. A transition is present on every bit and an additional transition may occur in the middle of the bit. Here the data rate is twice. |
| While using the Miller delay, noise interference can be reduced. | Sufficient clock information can be recovered from the data stream so that a separate clock is not needed. Therefore, for transmission, the number of wires is minimized. |

Observation 2 - Manchester codes have better data integrity or BER. They also have minimal DC component when compared to the Miller and FMO codes as observed from the power spectral density plot in Fig 2. Miller tends to have better noise performance, whereas FM0 has higher data-rate with self-clocking capability and minimized cost.

Proposal 2 – For A-IoT D2R line-coding, Manchester coding can be considered owing to its spectral efficiency, resilience to SFO, and its capability to support FDMA.

Fig 2: Spectrum plot of Miller, Manchester and FM0 code

2.3 D2R Modulation – The below points were agreed upon in RAN1#116-bis [2]:

- Study the following for D2R baseband modulation, for potential down-selection: OOK, BPSK, BFSK (only if OOK and BPSK not suited).

- Study must consider- exact baseband signal generation, spectral efficiency comparison, power consumption and complexity feasibility, impacts of phase discontinuity, demodulation methods at D2R receiver, decoding performance at reader, spectrum aliasing impact for BFSK.

Here is a comparison of OOK, BPSK, and BFSK modulation based on the mentioned aspects.

Spectral Efficiency

OOK is more spectrally efficient than frequency-shift keying, but more sensitive to noise when using a regenerative receiver or a poorly implemented superheterodyne receiver. For a given data rate, the bandwidth of a BPSK (Binary Phase Shift keying) signal and the bandwidth of OOK signal are equal.

Power Consumption

The power consumption of a modulation scheme depends on several factors, including the complexity of the modulation technique, the efficiency of the transmission system, and the specific implementation details. OOK modulation is relatively simple, as it only requires turning the carrier signal on and off to represent binary data. This is illustrated in Fig.3 for OOK-1 and Fig.4 for OOK-4. BPSK and BFSK may consume more power due to their increased complexity compared to OOK.

Fig 3: Time domain plot of OOK-1 for 2 symbols

Fig 4. Time domain plot of OOK-4 for 2 symbols (6 chips per symbol, each symbol represents the bit pattern 101001)

Each symbol is represented by 128 samples of data and 8 samples of CP

Phase Discontinuity

Each transition between the on and off states in OOK introduces a phase shift, resulting in phase discontinuities. Likewise, in BPSK, the carrier instantaneously jumps whenever the message bit of the current symbol differs from the message bit of the previous symbol. BFSK modulation does not inherently maintain phase continuity because it modulates the frequency of the carrier signal to represent binary data. However, it is possible to configure BFSK in such a way that phase continuity is maintained across the entire input. Continuous-phase frequency shift keying (CPFSK) is a type of frequency shift keying (FSK) that maintains phase continuity from symbol to symbol.

Demodulation

The simplest demodulation technique for OOK involves using a threshold detector. The received signal is compared against a predetermined threshold. If the received signal's amplitude is above the threshold, it is interpreted as a '1' bit; otherwise, it is interpreted as a '0' bit. Another demodulation method for OOK involves envelope detection, where the amplitude variations of the received signal are extracted to recover the original data. OOK demodulation is straightforward and suitable for low-complexity systems.

Coherent demodulation is commonly used for BPSK, where the receiver must maintain phase coherence with the transmitter. This typically involves recovering a phase reference from the received signal (e.g., using a local oscillator) and comparing it with the received signal to determine the phase shift. Non-coherent demodulation techniques, such as differential BPSK (DBPSK), are also used. DBPSK compares the phase difference between adjacent symbols to determine the transmitted bits, eliminating the need for phase synchronization.

Coherent demodulation in CPFSK involves recovering a frequency reference from the received signal and comparing it with the received signal to detect frequency shifts. Non-coherent demodulation techniques, such as frequency discrimination, can also be used for BFSK.

Fig 5. SNR vs BER curve for OOK-1 vs BPSK in AWGN

Decoding Performance

The BER (Bit Error Rate) performance of BFSK and OOK is worse compared to BPSK modulation. The SNR vs BER curve of OOK-1 vs BPSK is given in Fig 5. It is observed that BPSK has a 3 dB better performance when compared to OOK-1 at 1e-4 BER. The performance curve of the OOK-1 signal in the presence of fading channel and AWGN has been captured in Fig 6. The simulation assumptions for the plots are given in appendix.

Fig 6. Performance Comparison of OOK-1 in AWGN and TDL A Channel

Observation 3 -BPSK offers superior decoding performance. However, there are several advantages for on-off keying which include low power consumption, simple demodulation techniques, and spectral efficiency.

Proposal 3 - For the D2R transmission, OOK modulation scheme can be supported.

2.4 D2R Modulation The below point was agreed upon in RAN1#118 for FFS [1]:

The variant of Binary FSK to study further for all devices is identified as MSK (and not GMSK)

  - FFS: whether MSK is feasible for all devices

Below table gives the comparison of FSK and MSK in terms of spectral efficiency, BER performance and implementation complexity.

| Comparison of FSK and MSK |  |  |
| --- | --- | --- |
| Characteristic | FSK | MSK |
| Spectral Efficiency | FSK is less bandwidth-efficient compared to MSK. This means that for a given bit rate, FSK may require more bandwidth, which could affect the overall design and complexity of the communication system. | MSK is a type of continuous-phase frequency shift keying, which allows it to use bandwidth more efficiently than conventional FSK. Its continuous phase transitions reduce spectral side lobes. |
| BER performance | The BER performance of FSK is more degraded when compared to MSK. | Generally, MSK offers better BER performance at lower SNRs because of its continuous phase modulation, which helps to reduce errors due to noise. |
| Implementation complexity | The hardware for FSK can be simpler when compared to MSK, which might only require oscillators and switches. | Implementing MSK can be more complex than FSK due to the need for continuous-phase modulation and demodulation techniques. |

Fig 7. SNR vs BER curve for FSK vs MSK in AWGN

Fig 7 depicts the SNR vs BER comparison for FSK and MSK. From the figure, we can see that MSK has better performance when compared to FSK.

Fig 8. Spectral plot of FSK vs MSK

Fig 8 illustrates an example of the spectral plot of FSK and MSK. From the figure, we can make out that MSK has better side lobe suppression when compared to FSK.

MSK is also more spectrally efficient than BPSK.  The spectrum of MSK falls off as the fourth power, versus the second power for BPSK.

Observation 4 - MSK provides advantages in bandwidth efficiency and performance at the cost of increased complexity in both modulation and demodulation processes.

Proposal 4 - For the D2R transmission, MSK modulation scheme can also be supported.

2.5 D2R FEC – The below points were agreed upon in RAN1#118 [1]:

For D2R FEC study, the LTE convolutional code polynomials are a reference. Other designs can be studied subject to:

- Constraint length K = 8 or K = 7 or K = 6 or K = 4 for further study

- Mother code-rate R = 1/6, 1/4, 1/3, 1/2 for further study

- FFS other details, e.g. final code rate by puncturing, shift-register initialization/termination.

The LTE Convolutional Encoder block implements the encoding polynomials specified by LTE standard TS 36.212 [4]. The convolutional code has constraint length 7 and is tail biting with coding rate 1/3 and the generator polynomials are G0=133, G1=171 and G2=165 (in octal format).

Figure 9 shows the SNR vs BER comparison of OOK and BPSK with LTE convolutional coding with rate 1/3. It also captures the uncoded scheme and the results with code rate ½.

Fig 9. SNR vs BER for OOK vs BPSK without coding, with CC code rate 1/3, with CC code rate ½

The below table shows the comparison of the Rate ½ and Rate 1/3 coding schemes.

| Comparison of the coding schemes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Sl. No. | Rate | Constraint length | Generator Polynomials (Octal) | Coding gain w.r.t. uncoded | Effective throughput w.r.t uncoded |
| 1. | 1/3 | 7 | G0=133, G1=171 and G2=165 | 6-7 dB | 33.33% |
| 2. | 1/2 | 7 | G0=171, G1=133 | 4-5 dB | 50% |

Observation 5 - LTE convolutional codes with rate 1/3 provides 6-7 dB gain when compared to the uncoded systems. Moreover, the rate 1/3 code can be punctured to rate ½ code which can be utilised in scenarios where prioritising throughput is more critical than the error correction performance.

Proposal 5 - For the D2R transmission of Ambient IoT, LTE convolutional codes with rate 1/3 is supported for D2R FEC.

3. Conclusion

In this document different options for waveform and modulation, line coding, and forward error correction techniques for the Ambient IoT device are discussed and the following proposals are made:

Observation 1 - PIE offers promise of low power and low cost but redundant coding like Manchester which uses more spectrum due to the self-clocking of redundancy of transmitting a full clock cycle per bit, can have better data integrity or BER.

Proposal 1 – Owing to its drawbacks like false edge detection and high transmit time, pulse interval encoding doesn’t seem to be a good fit for A-IoT R2D line-coding. Hence, Manchester is more preferred in this scenario.

Observation 2 - Manchester codes have better data integrity or BER. They also have minimal DC component when compared to the Miller and FMO codes as observed from the power spectral density plot in Fig 2. Miller tends to have better noise performance, whereas FM0 has higher data-rate with self-clocking capability and minimized cost.

Proposal 2 – For A-IoT D2R line-coding, Manchester coding can be considered owing to its spectral efficiency, resilience to SFO, and its capability to support FDMA.

Observation 3 - BPSK offers superior decoding performance. However, there are several advantages of on-off keying which include low power consumption, simple demodulation techniques, and spectral efficiency.

Proposal 3 - For the D2R transmission, OOK modulation scheme can be supported.

Observation 4 - MSK provides advantages in bandwidth efficiency and performance at the cost of increased complexity in both modulation and demodulation processes.

Proposal  4 - For the D2R transmission, MSK modulation scheme can also be supported.

Observation 5 - LTE convolutional codes with rate 1/3 provides 6-7 dB gain when compared to the uncoded systems. Moreover, the rate 1/3 code can be punctured to rate ½ code which can be utilised in scenarios where prioritising throughput is more critical than the error correction performance.

Proposal 5 - For the D2R transmission of Ambient IoT, LTE convolutional codes with rate 1/3 is supported for D2R FEC.

4. References

- R1-2407249, Feature Lead Summary #1 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”, 3GPP TSG RAN-WG1 Meeting #118, Maastricht, Netherlands, 19-23 August, 2024, Moderator (Huawei)

- R1-2403679, Feature Lead Summary #4 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”, RAN1#116-bis, April 2024, Huawei (moderator).

- R1-2405441, Feature Lead Summary #2 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”, RAN1#117, May 2024, Huawei (moderator).

- TS 36.212, Evolved Universal Terrestrial Radio Access (E-UTRA); Multiplexing and channel coding

APPENDIX

| Channel Model | TDL A |
| --- | --- |
| Doppler | 4 Hz (vel 3 km/hr, Fc 1.44 GHz) |
| Subcarrier Spacing | 15 kHz |
| FEC | None |
| Repetition | None |
| Waveform | OOK1 (OFDM with CP) |
| IFFT Size | 128 |
| CP Length | 8 |