3GPP TSG RAN WG1 #118bis			R1-2408331

Hefei, China, October 14th – 18th, 2024

Source:	Sharp

Title:	Discussion on reply LS on applicable functionality reporting for beam management UE-sided model

Agenda Item:	5

Document for:	Discussion and Decision

# Background

The LS from RAN2 [1] provides the following questions. We present our views on the draft response to RAN2.

RAN2 further has made following agreements and signalling procedure (see the attached figure) on applicable functionality reporting for beam management UE-sided model: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig. - 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. - 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: - 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI - 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: - 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). - 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. RAN2 also agreed the applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5. FFS the initial activation state. FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3. FFS on additional L1/L2 signaling for activation/deactivation. FFS if multiple applicable functionalities can be activated at the same time. FFS what is the granularity of functionality. The above agreements were made based on the following assumptions: NW-side additional condition is assumed as associated ID in RAN2 (which is assumed by majority of companies). To further progress life cycle management for beam management UE-sided model, RAN2 has following questions for which RAN2 would like to check RAN1’s understanding: On General - Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others? On NW-side additional condition and configuration - Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID? - Q3: Is NW-side additional condition functionality specific? - Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): - Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition? - Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities? - Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc? - Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities? - Q5: What is the content of applicable functionality reporting in Step 4? - Q6: What is the content of inference configuration in Step 5? On Functionality Activation - Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3? - Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5? - Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated? - Q10: Is L1/L2 signaling for functionality activation/deactivation needed?

# Discussion

On General - Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others?

The following agreement in the study item phase could be the starting point for Q1 [2].

Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM - Note: Applicability of functionality-based LCM and model-ID-based LCM is a separate discussion.

As in the agreement, functionality-based LCM operates based on one configuration of an AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. The former implies that the configuration corresponds to an AI/ML-enabled Feature/FG. The latter implies that an AI/ML-enabled Feature/FG is a superset of multiple specific configurations and the superset implies a use case. Therefore, RAN1 should support both use-case level and sub-use case level granularity.

Proposal 1: Reply the following to Q1 in the RAN2 LS:

Use-case level and sub-use case level granularity should be supported.

On NW-side additional condition and configuration - Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID?

RAN1 agreed the definition of the associated ID, including working assumption as follows [3].

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

The above agreement provides the answer to Q2. Therefore, we propose the following.

Proposal 2: Reply the following to Q2 in the RAN2 LS:

RAN1 had made some agreements about associated ID and NW-side additional conditions as follows.

Associated IDs can be:

Used to ensure consistency of NW-side additional condition across training and inference, i.e., link inference configuration to training data collection configurations.

Ensures the UE assumes the similar properties of a DL Tx beam or beam set/list are applied during inference and training.

Both the UE and the gNB understand the information linked to the associated ID.

For example, a particular associated ID might correspond to a specific beam codebook and the associated index/mapping of Set A and Set B. When this ID is used, both the UE and gNB know that they need to use the beam codebook and indexing/mapping associated with that ID.

On NW-side additional condition and configuration - … - Q3: Is NW-side additional condition functionality specific?

As in the agreement above, the associated ID can be configured within the CSI framework for beam management use cases. For positioning, the associated ID should be configured in different configurations. Therefore, the associated ID should be use-case-specific or functionality-specific. Furthermore, considering that the associated ID provides information about NW-side additional conditions (e.g., the antenna structure of the gNB). Therefore, it can also be gNB-specific.

Proposal 3: Reply the following to Q3 in the RAN2 LS:

It should be per functionality and/or gNB-specific.

On NW-side additional condition and configuration - … - Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): - Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

In our understanding, the associated ID is not mandatory. UEs with generalized AI/ML models may support a functionality irrespective of NW-side additional conditions.

Proposal 4-1: Reply the following to Q4-1 in the RAN2 LS:

It’s not mandatory. It depends on the functionality. Some functionality may not require NW-side additional conditions to evaluate the applicable functionality reporting.

On NW-side additional condition and configuration - … - Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): - … - Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities?

To adapt to the rapid change in UE’s environment, it is straightforward to configure multiple functionalities and to support L1/L2-based activation deactivation mechanisms. Therefore, we propose the following.

Proposal 4-2: Reply the following to Q4-2 in the RAN2 LS:

YES, the network may provide one or more configurations (e.g., inference configurations) to the UE, for a single or multiple functionalities. It is possible to send multiple configurations for 1 functionality and (de) activate it using L1/L2 signaling.

On NW-side additional condition and configuration - … - Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): - … - Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc?

The associated ID conveys the NW-side additional condition, which the UE uses to determine the applicable functionalities in step 4. According to the RAN2 LS, inference configuration can be provided in Step 3 and/or Step 5. It is clear that inference configuration and the NW-side additional conditions have different roles.

Our understanding is that inference configuration is a configuration that enables inference and is associated with CSI reporting configuration. Therefore, NW-side additional conditions are separate from inference configuration. Full or partial inference configuration may be provided for a target functionality. NW-side additional conditions may be provided via the associated ID and the inference configuration in step 3.

Proposal 4-3: Reply the following to Q4-3 in the RAN2 LS:

NW-side additional conditions are separate from inference configuration. Full or partial inference configuration maybe provided for a target functionality. NW side additional conditions are optional, and it may be provided together with the inference configuration if required.

On NW-side additional condition and configuration - … - Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5): - … - Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities?

The following information can be contained in the inference configuration for beam management.

Proposal 4-4: Reply the following to Q4-4 in the RAN2 LS:

The inference configuration may contain for e.g., information such as

- AI/ML Model related information such as:

  - Inference reporting format: e.g., Neural network model to predict future beam quality

  - Inference trigger

  - Prediction Window

  - Thresholds: Events/Conditions (e.g., model (un) availability, thresholds for signal quality or latency, Functionality related KPI thresholds) that trigger the functionality to be deemed applicable/inapplicable.

For specific use cases such as Beam Management, the inference configuration may further contain info. Such as

- CSI-ReportConfig:

  - Periodicity: Moderate frequency (e.g., every 20ms).

  - Resources: Set B includes a set of active beams for measurement; Set A includes a set of candidate beams for inference.

- ResourceConfigId:

  - Set B: Active beams for measurement.

  - Set A: Candidate beams for inference.

- QCL Information:

Assumptions: Detailed spatial correlations between beams in Set A and Set B.

On NW-side additional condition and configuration - … - Q5: What is the content of applicable functionality reporting in Step 4?

The applicable functionality reporting can be used to report the UE’s applicable functionality. Therefore, functionality IDs, applicability status should be included. Furthermore, it is suggested to include UE’s recommendation of e.g., the best functionality among the applicable functionalities in the report.

For example, The UE may indicate one or more applicable functionality(s) of the serving cell. UE may also request a network if it needs to change the configuration or request to fully re-configure the functionality or (de) activate a particular functionality. The UE may request the network to change the configuration of an already applicable and activated functionality. The UE may ask to perform a full functionality reconfiguration. The UE may indicate the preferred configuration (if multiple configurations are provided for a given functionality), or the UE may indicate its preferred applicable functionality if multiple applicable functionalities can be activated.

Proposal 5: Reply the following to Q5 in the RAN2 LS:

Functionality IDs (one or multiple)

Applicability status (applicable/inapplicable)

Recommended actions (e.g., beam selection, resource allocation, request to change   functionality configuration, preferred functionality configuration)

The UE may indicate one or more applicable functionality(s) of the serving cell.

UE may also request network if it needs to change configuration or request to fully re-configure the functionality or (de) activate a particular functionality.

The UE may request the network to change configuration of a functionality that is already applicable and already activated.

The UE may request to perform a full functionality reconfiguration and/or

The UE may indicate preferred configuration (if multiple configurations are provided for given functionality) or the UE may indicate its preferred applicable functionality if multiple applicable functionalities can be activated.

On NW-side additional condition and configuration - … - Q6: What is the content of inference configuration in Step 5?

Inference configuration in Step 3 and 5 should be at least similar or the same.

Proposal 6: Reply the following to Q6 in the RAN2 LS:

Step 5 can have detailed functionality configurations for e.g., in Beam management it can have a default functionality/inference configuration or preferred functionality/inference configuration

  - CSI-ReportConfig:

    - Periodicity: Moderate frequency (e.g., every 20ms).

    - Resources: Set B includes a set of active beams for measurement; Set A includes a set of candidate beams for inference.

  - ResourceConfigId:

    - Set B: Active beams for measurement.

    - Set A: Candidate beams for inference.

  - QCL Information:

Assumptions: Detailed spatial correlations between beams in Set A and Set B.

On Functionality Activation - Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3?

The network should wait for the UE’s applicable functionality reporting before activating one of the configured functionalities in step 3. Therefore, we propose the following.

Proposal 7: Reply the following to Q7 in the RAN2 LS:

No. Inference configuration provided in Step 3 should be used to evaluate applicable functionalities. The activation can be done later using L1/L2 signaling.

On Functionality Activation - … - Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5?

As discussed in the discussion of Q4-2, L1/L2-based activation/deactivation should be supported. Therefore, by default, RRC reconfiguration should not activate a specific functionality immediately especially when multiple functionalities are configured in step 5.

A functionality may be activated immediately if the network indicates a default or preferred functionality/inference configuration to the UE. This is particularly relevant when a functionality is configured with multiple functionality or inference configurations, allowing the network to guide the UE on which configuration to prioritize for activation.

Proposal 8: Reply the following to Q8 in the RAN2 LS:

Functionality can be immediately activated if the network indicates default functionality/inference configuration or preferred functionality/inference configuration (in case one functionality is configured with multiple functionality/inference configurations) to the UE.

Otherwise: The activation can be done using L1/L2 signaling.

On Functionality Activation - … - Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated?

It should be possible if the UE can do so.

Proposal 9: Reply the following to Q9 in the RAN2 LS:

Yes. If the UE internal conditions permit. If the internal conditions of the UE do not allow, the UE may notify the network through the UAI, providing details on any internal issues due to AI/ML functionality that it encounters, which could impact its operation.

On Functionality Activation - … - Q10: Is L1/L2 signaling for functionality activation/deactivation needed?

As discussed in Q4-2, we see the necessity of L1/L2-based activation/deactivation. However, as discussed in P8, the functionality can be activated implicitly without additional L1/L2 signaling if the network indicates to the UE either the default functionality/inference configuration or the preferred functionality/inference configuration (in case one functionality is configured with multiple functionality/inference configurations).

Proposal 10: Reply the following to Q10 in the RAN2 LS:

Yes, it is needed.

However, as discussed in P8, the functionality can be activated implicitly without additional L1/L2 signaling, if the network indicates default functionality/inference configuration or preferred functionality/inference configuration (in case one functionality is configured with multiple functionality/inference configurations) to the UE.

# Conclusion

In this contribution, we have the following proposals:

Proposal 1: Reply the following to Q1 in the RAN2 LS:

Use-case level and sub-use case level granularity should be supported.

Proposal 2: Reply the following to Q2 in the RAN2 LS:

RAN1 had made some agreements about associated ID and NW-side additional condition as following.

Associated IDs can be:

Used to ensure consistency of NW-side additional condition across training and inference, i.e., link inference configuration to training data collection configurations.

Ensures the UE assumes the same similar properties of a DL Tx beam or beam set/list are applied during inference and training.

Both the UE and the gNB understand the information linked to the associated ID.

For example, a particular associated ID might correspond to a specific beam codebook and the associated index/mapping of Set A and Set B. When this ID is used, both the UE and gNB know that they need to use the beam codebook and indexing/mapping associated with that ID.

Proposal 3: Reply the following to Q3 in the RAN2 LS:

It should be per functionality and/or gNB specific.

Proposal 4-1: Reply the following to Q4-1 in the RAN2 LS:

It’s not mandatory. It depends on the functionality. Some functionality may not require NW-side additional condition to evaluate the applicable functionality reporting.

Proposal 4-2: Reply the following to Q4-2 in the RAN2 LS:

YES, the network may provide one or more configurations (e.g., inference configurations) to the UE, for a single or multiple functionalities. It is possible to send multiple configurations for 1 functionality and (de) activate it using L1/L2 signaling.

Proposal 4-3: Reply the following to Q4-3 in the RAN2 LS:

NW-side additional conditions are separate from inference configuration. Full or partial inference configuration maybe provided for a target functionality. NW side additional conditions are optional, and it may be provided together with the inference configuration if required.

Proposal 4-4: Reply the following to Q4-4 in the RAN2 LS:

The inference configuration may contain for e.g., information such as

- AI/ML Model related information such as:

  - Inference reporting format: e.g., Neural network model to predict future beam quality

  - Inference trigger

  - Prediction Window

  - Thresholds: Events/Conditions (e.g., thresholds for signal quality or latency, Functionality related KPI thresholds) that trigger the functionality to be deemed applicable/inapplicable.

For specific use cases such as Beam Management, the inference configuration may further contain info. Such as

- CSI-ReportConfig:

  - Periodicity: Moderate frequency (e.g., every 20ms).

  - Resources: Set B includes a set of active beams for measurement; Set A includes a set of candidate beams for inference.

- ResourceConfigId:

  - Set B: Active beams for measurement.

  - Set A: Candidate beams for inference.

- QCL Information:

Assumptions: Detailed spatial correlations between beams in Set A and Set B.

Proposal 5: Reply the following to Q5 in the RAN2 LS:

Functionality IDs (one or multiple)

Applicability status (applicable/inapplicable)

Recommended actions (e.g., beam selection, resource allocation, request to change   functionality configuration, preferred functionality configuration)

The UE may indicate one or more applicable functionality(s) of the serving cell.

UE may also request network if it needs to change configuration or request to fully re-configure the functionality or (de) activate a particular functionality.

The UE may request the network to change configuration of a functionality that is already applicable and already activated.

The UE may request to perform a full functionality reconfiguration and/or

The UE may indicate preferred configuration (if multiple configurations are provided for given functionality) or the UE may indicate its preferred applicable functionality if multiple applicable functionalities can be activated.

Proposal 6: Reply the following to Q6 in the RAN2 LS:

Step 5 can have detailed functionality configurations for e.g., in Beam management it can have a default functionality/inference configuration or preferred functionality/inference configuration

  - CSI-ReportConfig:

    - Periodicity: Moderate frequency (e.g., every 20ms).

    - Resources: Set B includes a set of active beams for measurement; Set A includes a set of candidate beams for inference.

  - ResourceConfigId:

    - Set B: Active beams for measurement.

    - Set A: Candidate beams for inference.

  - QCL Information:

Assumptions: Detailed spatial correlations between beams in Set A and Set B.

Proposal 7: Reply the following to Q7 in the RAN2 LS:

No. Inference configuration provided in Step 3 should be used to evaluate applicable functionalities. The activation can be done later using L1/L2 signaling.

Proposal 8: Reply the following to Q8 in the RAN2 LS:

Functionality can be immediately activated if the network indicates default functionality/inference configuration or preferred functionality/inference configuration (in case one functionality is configured with multiple functionality/inference configurations) to the UE.

Otherwise: The activation can be done using L1/L2 signaling.

Proposal 9: Reply the following to Q9 in the RAN2 LS:

Yes. If the UE internal conditions permit. If the internal conditions of the UE do not allow, the UE may notify the network through the UAI, providing details on any internal issues due to AI/ML functionality that it encounters, which could impact its operation.

Proposal 10: Reply the following to Q10 in the RAN2 LS:

Yes, it is needed.

However, as discussed in P8, the functionality can be activated implicitly without additional L1/L2 signaling, if the network indicates default functionality/inference configuration or preferred functionality/inference configuration (in case one functionality is configured with multiple functionality/inference configurations) to the UE.

# References

- R1-2407604, LS on applicable functionality reporting for beam management UE-sided model, October 2024.

- RAN1 Chair’s Notes, RAN1#112bis, April 2023

- RAM1 Chair's Notes, RAN1#118, August 2024