3GPP TSG-RAN WG1 #118bis	R1-2408751

Hefei, China, October 14th – 18th , 2024

Source:	Sharp

Title:	Discussion on other aspects of AI/ML model and data

Agenda item:	9.1.4.2

Document for: 	Discussion and Decision

Introduction

In RAN#105 meeting, Rel-19 work item on AI/ML for NR air interface [1] has the following revised objectives for further study on the necessity and details of model identification, data collection for UE side model training and model transfer/delivery:

Study objectives: - Necessity and details of model Identification concept and procedure in the context of LCM for two sided models [RAN2/RAN1] - CN/OAM/OTT collection of UE-sided model training data [RAN2/RAN1]: - For the FS_NR_AIML_Air study use cases, identify the corresponding contents of UE data collection - Analyse the UE data collection mechanisms identified during the FS_NR_AIML_Air (TR 38.843 section 7.2.1.3.2) study along with the implications and limitations of each of the methods - Model transfer/delivery [RAN2/RAN1]: - Determine whether there is a need to consider standardised solutions for transferring/delivering AI/ML model(s) considering at least the solutions identified during the FS_NR_AIML_Air study

In this contribution, we provide our views on model identification.

Discussion

Model identification procedure

According to the revised scope of objectives for the Rel-19 AI/ML study, the model identification concept will continue to be studied in Rel-19. Based on the model identification procedure discussed in the previous meeting, both MI-Option2 and MI-Option3 are applicable to the two-sided model. These two model identification procedures can serve as starting points for further study on model identification for the two-sided model.

Proposal 1: Both MI-Option2 and MI-Option3 are used as starting points for the discussion on the model identification concept for two-sided models.

MI-Option2 (Model identification with data transfer)

In RAN1#117 meeting, the following conclusion is made:

Agreement From RAN1 perspective, for UE part of two-sided model, further study the following example of MI-Option2 (including the feasibility/necessity) - AI-Example2-1 - A: A dataset is transferred from the NW/NW-side to UE/UE-side via standardized signaling. - Note: RAN1 study of Step A only focuses on RAN1 aspect of the dataset transfer from NW to UE. Other solution for dataset exchange is out of RAN1 scope. - B: UE part of two-sided model(s) is(are) developed based on at least the above dataset. - C: UE reports information of its UE part of two-sided model(s) corresponding to the above dataset to the NW. - FFS: How model ID is determined/assigned for each AI/ML model (including relationship between dataset and model ID) - Note: Some step(s) may not be needed for MI-Option2 - Note: The above example is based on the assumption of NW-first training. It is separate discussion for the assumption of UE-first training. - Note: The study should consider the impact on inter-vendor collaboration, at least including complexity, performance, interoperability in RAN4/testing related aspects and feasibility. - FFS: whether/how to consider UE-side additional condition(s) for the dataset

In AI-Example2-1, one of the unresolved issues is how to assign the model ID. In Step A, the dataset transfer from NW to UE can be generated based on an additional NW-side condition, which can be associated with the NW part of the two-sided model. In MI-Option1, we already have a common understanding that an associated ID can be linked to a NW-side additional condition. Therefore, in Step A, the NW can transfer the dataset to the UE with an associated ID that is related to a specific NW part of the two-sided model. Then, in Step B, the UE part of the two-sided model(s) can be developed based on the dataset associated with the NW-side additional condition.

Observation 1: In AI-Example2-1, the associated ID can implicitly represent a NW part of two-sided model.

In Step C, after developing the UE part of the two-sided model(s) based on the dataset associated with the NW-side additional condition, the UE will report information on its part of the two-sided model(s), developed from the dataset transferred from NW to UE. However, Step C may not be necessary in the AI-Example. Similarly, the associated ID can be mapped to the UE part of the two-sided model, meaning that the associated ID can implicitly represent the UE part of the two-sided model(s).

Observations 2: In AI-Example2-1, Step C may not be needed unless NW needs UE side additional conditions  to develop its NW part of two-sided model(s).

Observations 3: In AI-Example2-1, the associated ID can implicitly represent a UE part of two-sided model.

The relationship between the NW part of the two-sided models, the UE part of the two-sided models, and the associated IDs is illustrated in Fig. 1. In Fig. 1, each associated ID can implicitly represent the NW part and the UE part of the two-sided models. For example, associated ID#3 can implicitly represent the NW part of two-sided model#4, the UE part of two-sided model#2, and the UE part of two-sided model#3.

Proposal 2: For MI-Option2, the associated ID can be used as the model ID for either the NW part or the UE part of the two-sided model.

Figure 1: An example of the relationship among NW part/UE part of two-sided model and associated ID

MI-Option3 (Model identification with model transfer)

For MI-Option3, the NW will train UE-side models and transfer them to the UE for inference. More specifically, the NW will first obtain information related to the AI/ML models supported by the UE. These supported AI/ML models will then be developed and stored on the NW side. The NW will transfer the applicable AI/ML model to the UE if the NW needs the UE to perform model inference or if the UE requests to perform model inference. In MI-Option3, the NW needs to develop and control every AI/ML model supported by the UE. For efficiency, a model ID is needed in MI-Option3 for model identification during transfer.

Proposal 3: Model ID is needed for MI-Option3.

Conclusions

In this contribution, we discussed the issues regarding model identification procedure for two-sided model, we have observations and proposals as follows:

- Observations 1: In AI-Example2-1, the associated ID can implicitly   represent a NW part of  two-sided model.

- Observations 2: In AI-Example2-1, Step C may not be needed unless NW needs UE side.   additional conditions  to develop its NW part of two-sided model(s).

- Observations 3: In AI-Example2-1, the associated ID can implicitly   represent a UE part of  two-sided model.

Both MI-Option2 and MI-Option3 are used as starting points for the discussion on the model identification concept for two-sided models.

For MI-Option2, the associated ID can be used as the model ID for either the NW part or the UE part of the two-sided model.

Model ID is needed for MI-Option3.

References

- Qualcomm, RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#105, Sep.9-12, 2024.