3GPP TSG RAN WG1 #118-bis			R1-2408898

Hefei, China, October 14th – 18th, 2024

Source:	WILUS Inc.

Title:	Discussion on SBFD TX/RX/measurement procedures

Agenda item:	9.3.1

Document for:	Discussion

Introduction

From the RAN1#116 and RAN1#118 meeting [1]-[4], RAN1 agreed under the agenda of SBFD TX/RX/measurement procedures for many technical issues. In this contribution, we provide our views on issues such as semi-static configuration of SBFD subbands, UE transmission/reception/measurement behavior, procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE for SBFD operation and DL/UL collision handling for a SBFD aware UE.

Discussion on subband non-overlapping full duplex

Semi-static configuration of SBFD subbands

At the RAN1#117 meeting, the following was agreed as cell specific indication of SBFD subband frequency location for semi-static frequency indication of SBFD subband. And it was captured at RAN1#118 meeting as working assumption for cell-specific indication of frequency locations of SBFD subbands.

Agreement at RAN1#117 For cell-specific indication of SBFD subband frequency location, adopt the following option. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). Agreement at RAN1#118 For configuration of SBFD symbols within a TDD-UL-DL pattern period, the following parameters are supported - A starting slot index - A starting symbol index within the starting slot - An ending slot index - An ending symbol index within the ending slot Working Assumption at RAN1#118 For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured

Signaling aspects such as signaling container and confirmation of working assumption for cell-specific indication of frequency locations of SBFD subbands should be further concluded for cell-specific configuration of SBFD subbands. Firstly, for the signaling container, due to the fact that SBFD operation is supported for UEs in RRC_IDLE/INACTIVE mode and duplicate configurations would not be considered for semi-static indication of time and frequency domain location of SBFD subbands to UEs in RRC_CONNECTED and in RRC_IDLE/INACTIVE mode, it seems reasonable to carry the same configuration via at least SIB-1 for cell-specific indication of time and frequency domain location of SBFD subbands to both UEs in RRC_CONNECTED and UEs in RRC_IDLE/INACTIVE modes.

- Proposal 1: It seems reasonable to carry by at least SIB-1 semi-static indication of time and frequency domain location of SBFD subbands for both UEs in RRC_CONNECTED and UEs in RRC_IDLE/INACTIVE mode.

Cell-specific or UE-specific configuration on frequency location of SBFD subbands

For semi-static indication of frequency locations of SBFD subbands, the following agreement were made in RAN1#116. The remaining issues are to confirm working assumption of cell-specific configuration on frequency location of SBFD subbands and whether to additionally support UE-specific configuration on frequency locations of SBFD subbands.

Agreement at RAN1#116 For RRC connected mode UEs, at least cell-specific configuration on time and frequency(working assumption) location of SBFD subbands is supported within a TDD carrier. - FFS: Additional support of UE-specific configuration on time and/or frequency locations of SBFD subbands

One of the motivations for introducing UE-specific configuration on the frequency position of the SBFD subbands is to reduce the impact of the UE-to-UE CLI by accommodating the UE-specific guardbands. However, as one of the ways to reduce the impact of the UE-to-UE CLI without the UE-specific configuration of the SBFD subband, a gNB can consider not implicitly scheduling some RBs for creating guardbands between UEs if the UE-to-UE CLI between UEs is severely measured and the gNB can avoid strong the UE-to-UE CLI. Therefore, we propose to confirm the working assumption that cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier without UE-specific configuration on frequency locations of SBFD subbands.

- Proposal 2: We propose to confirm the working assumption that cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier.

  - UE-specific configuration on frequency locations of SBFD subbands is not supported.

SBFD operation in DL and/or flexible symbols via TDD-UL-DL-ConfigCommon

The main motivation to allow DL receptions outside DL subband(s) in a symbol configured as DL and/or flexible symbols via TDD-UL-DL-ConfigCommon is to reflect dynamic traffics for DL transmission and also to measure PRBs for CLI measurement outside DL subband(s). It was agreed at the TSG-RAN#102 meeting that DL reception is confined within only DL subband(s) for semi-static SBFD operation since UL/DL subband(s) cannot be changed once semi-statically configured for the SBFD-aware UE. However, it was FFS for CLI measurement by the UE outside of the DL subband(s). Regarding that the target of SBFD operation is to further enhance UL performance (coverage, latency, and capacity) by allocating more UL resources in DL and flexible symbols, only limited resources can be used for DL reception by semi-static SBFD configuration. Even considering this, for CLI measurement as inter-subband interference between DL/UL subbands, it may be desirable to perform DL measurement outside the DL subband. Therefore, for CLI measurement, it should be further considered to allow measurement at least in the guardband(s).

- Proposal 3: DL receptions outside DL subband(s) should be allowed for CLI measurement in SBFD operation in a symbol configured as DL or flexible in TDD-UL-DL-ConfigCommon.

At the RAN1#116 meeting, the following was agreed for discussion purpose.

Agreement at RAN1#116 For discussion purpose, UL subband frequency resources within active UL BWP are called UL usable PRBs and DL subband(s) frequency resources within active DL BWP are called DL usable PRBs. For determining UL/DL usable PRBs, consider the following options. - Option 1: UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols. - Option 2: UL/DL usable PRBs are explicitly configured within active UL/DL BWP in SBFD symbols.

In determining the usable UL/DL PRBs (Physical Resource Blocks), Option 1, which is based on active BWP information and UL/DL subband information that the UE already knows, may be more preferable than using additional explicit signaling while considering additional signaling overhead like in Option 2. This approach reduces unnecessary explicit signaling and enables efficient resource management. Such an approach can be particularly helpful in increasing flexibility and efficiency in dynamic network environments. Option 1 has the following advantages:

- Reduced signaling overhead: No need for additional signaling, thus reducing network traffic and processing load.

- Efficient resource utilization: Using information the terminal already knows for resource allocation increases resource efficiency.

- Faster response time: Provides quick resource allocation and response time without waiting for explicit signaling.

- Proposal 4: We propose to support Option 1(i.e., UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols) to determine the usable UL/DL PRBs within UL/DL subband respectively.

Interaction with legacy SFI in DCI format 2_0

Agreement at RAN1#116 For SBFD-aware UE transmission and reception in the SBFD symbols configured in DL and/or flexible in TDD-UL-DL-ConfigCommon, - UL transmissions within UL usable PRBs are allowed - FFS SSB symbols - DL receptions within DL usable PRBs are allowed - UL transmissions outside UL usable PRBs are not allowed - DL receptions outside DL usable PRBs are not allowed - This restriction is not applicable for CLI measurement CLI measurement behaviours for SBFD-aware UE are discussed in agenda item 9.3.3. RAN1 to discuss SBFD aware UE behaviors in SBFD symbols with interaction with legacy TDD slot configuration indications via TDD-UL-DL-ConfigDedicated and SFI in DCI format 2_0 - DCI format 2_0 cannot be used to revert SBFD symbol to non-SBFD symbol

The following three alternatives were considered in the RAN1#117 meeting for interaction with dynamic SFI in DCI format 2_0 which was FFS at the RAN1#116 meeting above.

- Alt. 1: An SBFD aware UE does not expect to be configured to monitor DCI format 2_0 for the serving cell.

- Alt. 2: SFI in DCI format 2_0 is not applicable to SBFD symbols. An SBFD aware UE interprets SFI in DCI format 2_0 as legacy for non-SBFD symbols.

- Alt. 3: SFI in DCI format 2_0 is applicable to both non-SBFD and SBFD symbols. An SBFD aware UE interprets SFI in DCI format 2_0 as legacy for non-SBFD symbols, and interprets SFI in DCI format 2_0 as link direction for SBFD symbols, i.e. whether to transmit within UL usable PRBs or to receive within DL usable PRBs.

A gNB can configure a UE to set the SBFD symbols through the cell specific SBFD configuration on DL symbols and flexible symbols configured through TDD-UL-DL-ConfigCommon. Depending on the scheduling of the gNB, the gNB may not configure SBFD symbols on all or some of flexible symbols for a given cell. For the UE behavior related to DL reception and UL transmission in non-SBFD symbols (i.e., not configured with SBFD symbols through cell specific SBFD configuration) configured with flexible symbols through TDD-UL-DL-ConfigCommon, it may be desirable from the viewpoint of coexistence with the legacy non-SBFD UEs in a non-SBFD symbols to maintain the same operation as in the legacy non-SBFD UEs. In terms of scheduling flexibility at the gNB, it may be advantageous to apply the legacy operation to the UE’s transmission and reception operation in non-SBFD symbols by using legacy SFI, which can dynamically change the setting to DL or UL for flexible symbols. However, for SBFD symbols via the cell specific SBFD configuration, the SFI in DCI format 2_0 should not be applicable to SBFD symbols according to agreement at the RAN1#116 meeting that the SFI in DCI format 2_0 cannot be used to revert SBFD symbols to non-SBFD symbols.

- Proposal 5: We support that SFI in DCI format 2_0 is not applicable to SBFD symbols and an SBFD aware UE interprets SFI in DCI format 2_0 as legacy for non-SBFD symbols.

SBFD operation in SSB symbols

In the RAN1#117 meeting, RAN1 agreed to prioritize SSB reception over configured UL transmission and dynamically scheduled UL transmission in SBFD symbols. It should be further concluded for the FFS related SSB symbols configured with SBFD subbands.

Agreement RAN1#117 meeting Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

For a slot consisting of an SSB symbol cell-specifically configured based on a given cell, SSB is prioritized over configured UL transmission or dynamically scheduled UL transmission in any UL subband, so resources set to the UL subband may essentially be wasted. At least, if it is configured with DL symbols via TDD-UL-DL-ConfigCommon and is configured with SBFD symbols according to SBFD subband configuration, it may be further considered to be regarded the slot consisting of the corresponding SSB symbols as a DL slot due to the fact that SSB reception is prioritized on a slot consisting of the corresponding SBFD symbols and all types of UL transmissions are deprioritized to SSB reception. From the perspective of the SBFD aware UE, a waste of resources set in the UL subband can be prevented by considering the slot constituting the SSB symbol as DL slot. Additionally, the legacy non-SBFD UE considers the slot as a DL slot according to TDD-UL-DL-ConfigCommon, so there is no problem with coexistence with the legacy non-SBFD UEs. However, only slots that have SSBs configured cell-specifically as slots constituting the relevant SSB symbols can be considered as slots that can be regarded as DL slots. If slots with SSBs configured UE-specifically are also handled the same, inter-UE CLI may occur between SBFD aware UEs. Hence, for slots with SSBs configured UE-specifically, it makes it possible to perform only DL reception within the SBFD DL subband or DL ​​usable PRBs. Even though conversion from SBFD symbols to non-SBFD symbols through SFI in DCI format 2_0, which is dynamic signaling, is not allowed according to the agreement at the RAN1#116 meeting, considering that the Cell specific SSB configuration is set to RRC signaling, its use as a DL slot may be desirable in terms of efficient use of resources.

- Proposal 6: We propose that a slot consisting of SSB symbols can be further considered as a full DL slot if the SSB symbols are cell-specifically configured by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon, otherwise, SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

Enhancements for UL transmissions and DL receptions

At the TSG-RAN#102 meeting, it was agreed to specify enhancement on resource allocation in frequency domain in SBFD symbols and enhancements to physical channels/signals and procedure for UE transmissions, receptions and measurement across SBFD symbols and non-SBFD symbols in different slots. We provide our views on enhancements to UE transmissions, receptions and measurement for SBFD operation.

    - Unaligned boundaries between RBG/PRG/CSI reporting subband and SBFD subband

RBG for PDSCH FDRA Type 0

At the RAN1#116-bis meeting, the following was agreed for frequency resource allocation Type 0 for PDSCH or PUSCH. We provide our views for PDSCH reception on part of the DL RBG inside the DL subband.

Agreement at RAN1#116-bis For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. - SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs.

Figure 1. PDSCH FDRA type 0 with {D-U-D} subband frequency pattern.

In Figure 1, a UE is configured with DL BWP size as 275 PRBs (100MHz with 30kHz SCS) and allocated with DL subbands as 110 PRBs (40MHz) and UL subband as 55 PRBs (20MHz). In this case, the UE determines P = 16 regardless of RBG-Size configured from gNB. Then, RBG 6-10, which overlap with UL subband cannot be used for PDSCH FDRA for SBFD-aware UE. Especially, PRB 96-109 of RBG 6 and PRB 165-175 of RBG 10 cannot be used even though such PRBs are included in one of DL subbands.

That part of the DL RBG inside the DL subband, i.e., PRB 96-109 of RBG 6 and PRB 165-175 of RBG 10 can be used for PDSCH reception by reusing PDSCH rate matching mechanism, i.e., a UE shall assume that PRBs in UL subband (and guard band, if specified) are not available for PDSCH reception. In other words, a SBFD-aware UE shall assume that PDSCH is rate matched around the PRBs in UL subband. Since the frequency location of UL subband and DL subband(s) is known to a SBFD-aware UE, there is no ambiguity on PRBs in UL subband between gNB and the SBFD-aware UE. Therefore, the part of the DL RBG inside DL subband can be used without enhancement on FDRA type 0, i.e., reusing configuration granularity of RBG for PDSCH FDRA Type 0. This approach has less specification impact and it’s worth noting that FDRA type 1 can be also reused to allocate PDSCH across non-contiguous DL subbands via PDSCH rate matching mechanism.

- Proposal 7: We propose to reuse PDSCH rate matching mechanism to address unaligned boundaries of DL/UL subband and configuration granularity of RBG for PDSCH FDRA (i.e., FDRA type 0) with less specification impact.

    - UL transmission/DL reception across SBFD symbols and non-SBFD symbols in different slots

Regarding whether to restrict or not UL transmission and DL reception across SBFD symbols and non-SBFD symbols, if frequency domain resource allocation of UL transmission and DL reception is available for both SBFD symbols and non-SBFD symbols, it would not be necessary to restrict UL transmission and DL reception across SBFD symbols and non-SBFD symbols. Thus, the UL transmission or DL reception can be in SBFD symbols and non-SBFD symbols. Additional restriction in addition to current specification is not needed, and it’s up to gNB scheduling.

- Proposal 8: If a FDRA of UL transmission or DL reception is available for both SBFD symbols and non-SBFD symbols, the UL transmission or DL reception can be across SBFD symbols and non-SBFD symbols.

PDSCH with slot aggregation

Regarding frequency resource allocation options for PDSCH receptions across SBFD symbols and non-SBFD symbols in different slots for a SBFD-aware UE, it was captured as working assumption at the previous RAN1#118 meeting as below.

Working Assumption at RAN1#117 meeting For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid. - For SPS PDSCH, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only - For PDSCH repetitions, FFS whether the number of PRBs for TBS determination is based on assigned PRBs, or based on assigned PRBs within DL usable PRBs only, or based on assigned PRBs within DL usable PRBs of the first repetition. - For multi-PDSCH scheduled by a single DCI, FFS whether the number of PRBs for TBS determination is based on assigned PRBs or assigned PRBs within DL usable PRBs only.

The SBFD-aware UE can be configured or indicated to receive PDSCH of the same TB across multiple slots (i.e., slot aggregation). In this case, multiple slots can include both non-SBFD symbols and SBFD symbols. Then, TBS of PDSCH on non-SBFD symbols can be differently determined from that of PDSCH on SBFD symbols. However, TBs with different TBSs would be hard to be softly combined as bit-level. Therefore, we support to determine the number of PRBs for TBS determination based on assigned PRB within DL usable PRBs only as shown in Figure 2 to maximize the combining gain at least for PDSCH repetitions. For the other cases such as SPS PDSCH and multi-PDSCH scheduled by a single DCI, we prefer to determine the number of PRBs for TBS determination based on assigned PRB within DL usable PRBs only.

Figure 2. PDSCH receptions with slot aggregation across non-SBFD symbols and SBFD symbols.

- Proposal 9:

- At least for PDSCH repetitions, we support to determine the number of PRBs for TBS determination based on assigned PRB within DL usable PRBs only.

- For the other cases such as SPS PDSCH and multi-PDSCH scheduled by a single DCI, we prefer to determine the number of PRBs for TBS determination based on assigned PRB within DL usable PRBs only.

Available slot determination for PUSCH/PUCCH repetition

In Rel-17 coverage enhancement WI, enhancement on PUSCH repetition Type A was introduced. Like inter-slot PUCCH repetition case, PUSCH repetition Type A supports repetitions counted based on available slots in 2-step procedure: in the 1st step, the UE determines available slots for K PUSCH repetitions; in the 2nd step the UE determines whether to drop each of the K PUSCH repetition or not according to PUSCH dropping rules, where the PUSCH repetition is still counted in the K repetitions even if it is dropped.

In the 1st step (i.e., available slot determination), a UE determines available slots based on the symbol-level availability. For unpaired spectrum, the slot is determined as not available for PUSCH or PUCCH transmission if at least one of the symbols for PUSCH or PUCCH transmission overlaps with a DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. In other words, the slot is determined as available for PUSCH or PUCCH transmission starting in an UL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or in a flexible symbol that is not SS/PBCH block symbol indicated by ssb-PositionsInBurst in consecutive UL symbols indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or in flexible symbols that are not SS/PBCH block symbols indicated by ssb-PositionsInBurst.

Recall the SBFD operation, UL subband can be allocated in a symbol configured as DL or flexible indicated by TDD-UL-DL-ConfigCommon. In this case, a SBFD-aware UE may not determine the slot with symbols within UL subband since those symbols are configured as DL and flexible indicated by TDD-UL-DL-ConfigCommon even the set of symbols for PUSCH or PUCCH are available for transmission.

To address this, availability in frequency domain can be further considered in addition to availability in time domain. Thus, the SBFD-aware UE can determine the slot as available for PUSCH or PUCCH transmission in the set of symbols within UL subband. Even a symbol for PUSCH or PUCCH transmission overlaps with a DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, the slot can be determined as available if PUSCH or PUCCH transmission are scheduled within UL subband in the symbols.

- Proposal 10: In addition to availability in time domain, availability in frequency domain can be further considered to determine available slot of PUSCH/PUCCH repetition for a SBFD-aware UE.

Actual TDW determination for DMRS bundling

In Rel-17 coverage enhancement WI, DMRS bundling to enable improved channel estimation is introduced. It can be enabled for PUSCH repetition Type A scheduled by DCI format 0_1 or 0_2, for PUSCH repetition Type A with configured grant, for PUSCH repetition Type B, for TB processing over multiple slots PUSCH and for PUCCH repetitions of PUCCH formats 1, 3, and 4.

One or multiple nominal TDWs (time domain windows) which is not longer than the maximum duration determined by UE’s capability, can be configured with duration by gNB. Then, a nominal TDW can be segmented into one or multiple actual TDWs according to events, which result in the loss of power consistency and phase continuity across PUSCH or PUCCH transmissions, such as a downlink slot or downlink reception or downlink monitoring based on TDD-UL-DL-ConfigCommon and TDD-UL-DL-ConfigDedicated, the gap between any two consecutive PUSCH or PUCCH transmissions that exceeds 13 symbols for normal CP and 11 symbols for extended CP, and frequency hopping. Since the downlink slot by TDD-UL-DL-ConfigCommon is one of the events to segment from nominal TDW to actual TDW, joint channel estimation via DMRS bundling is not performed at gNB if there is at least one DL slot in-between PUSCH or PUCCH transmissions.

Recall the SBFD operation, UL subband can be allocated in a symbol configured as DL or flexible indicated by TDD-UL-DL-ConfigCommon. In this case, a SBFD-aware UE may not determine PUSCH or PUCCH transmissions across UL subbands in a slot configured as DL by TDD-UL-DL-ConfigCommon as an actual TDW as shown in Figure 3.

Figure 3. Actual TDW determination across PUSCH transmissions in UL subband.

In Figure 3, a SBFD-aware UE can be scheduled to transmit PUSCH over 4 slots with repetition Type A and configured to enable DMRS bundling with nominal TDW duration as 4 slots. Then, 4 actual TDWs are determined since slot#1, slot#2, and slot#3 are event, which result in the loss of power consistency and phase continuity. As a result, any of PUSCH repetitions can be jointly channel estimated at gNB even the SBFD UE is configured to enable DMRS bundling.

To address this, RBs within UL subband should not be regarded as an event in a slot configured as DL by TDD-UL-DL-ConfigCommon. Thus, PUSCH or PUCCH transmissions in UL subband across DL slots can be determined as an actual TDW, and joint channel estimation can be performed at gNB for UL coverage enhancement.

- Proposal 11: RBs within UL subband in symbols configured as downlink by TDD-UL-DL-ConfigCommon should not be an event of actual TDW determination for a SBFD-aware UE.

At the RAN1#117 meeting, it was agreed to support separate UL power control for PUSCH/PUCCH/SRS transmission in SBFD symbols and non-SBFD symbols. When a UE is configured with DMRS bundling as enabled, power consistency of PUSCH or PUCCH transmissions cannot be maintained across SBFD and non-SBFD symbols if separate UL power control parameters are applied across SBFD and non-SBFD symbols. Therefore, separate UL power control parameters across SBFD and non-SBFD symbols can be considered as an event, and different actual TDWs can be made on PUSCH or PUCCH transmissions across SBFD and non-SBFD symbols if DMRS bundling is configured as enabled for the UE as shown in Figure 4.

Figure 4. Actual TDW determination on PUSCH transmissions across SBFD and non-SBFD symbols with separate UL power control parameters.

In Figure 4, a SBFD UE can be scheduled to transmit PUSCH over 4 slots with repetition Type A, where P0,SBFD denotes P0 value of PUSCH transmission in SBFD symbols, while P0,normal denotes the P0 value in non-SBFD symbols. If the UE is configured to enable DMRS bundling with duration of a nominal TDW as 4 slots, two actual TDWs (actual TDW#1 including PUSCH#1~#3 and actual TDW#2 including PUSCH#4) are made with respect to an event between PUSCH#3 and PUSCH#4. Considering that the DMRS bundling feature was introduced to enhance UL coverage, it seems beneficial to determine PUSCH#1~PUSCH#3 and PUSCH#4 as the same actual TDW by maintaining power consistency although separate UL power control parameters are configured/indicated. In other words, DMRS bundling gain can be prioritized over separate UL power control across SBFD symbols and non-SBFD symbols when DMRS bundling is configured as enabled for the UE. Therefore, we propose to investigate how to maintain power consistency of PUSCH or PUCCH transmissions across SBFD symbols and non-SBFD symbols if DMRS bundling is configured as enabled for the SBFD-aware UE.

- Proposal 12: Although separate UL power control parameters are configured/indicated for PUSCH transmissions or PUCCH transmissions across SBFD symbols and non-SBFD symbols in different slots, we propose to investigate how to maintain power consistency if DMRS bundling is configured as enabled for a SBFD-aware UE.

HPN determination for multi-slot PUSCH

In Rel-17 60GHz WI, multi-slot PUSCH scheduling with a single DCI was specified, where time domain allocations of multiple PUSCH can be discontinuous. Those PUSCH transmissions in multi-slot have different TBs, thus different HPNs (HARQ process numbers) are determined for each PUSCH transmission. Because a single HPN value is contained in a DCI, HPN determination rule was specified as follows.

When the UE is scheduled with multiple PUSCHs by a DCI, HPN indicated by DCI applies to the first PUSCH not overlapping with a DL symbol configured by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. HPN is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation. HPN is not incremented if at least one of the symbols for PUSCH or PUCCH transmission overlaps with a DL symbol configured by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

Similar with available slot determination as we discussed above, enhancement on HPN determination of multi-slot PUSCH scheduled with a single DCI within UL subband is necessary as shown in Figure 5.

Figure 5. HPN determination for multi-slot PUSCH across SBFD symbols and non-SBFD symbols.

In Figure 5, a SBFD-aware UE can be scheduled to transmit multi-slot PUSCH across slot#1 and slot#2 with a single DCI, where slot#1 is consisted of non-SBFD symbols and slot#2 is consisted of SBFD symbols. Even the SBFD-aware UE can transmit PUSCH#2 in the slot#2, the HPN of PUSCH#2 would not be incremented according to the current rule. In this case, ambiguity due to the same HPN between the PUSCHs with different TBs can occur, thus it can affect PUSCH HARQ process.

To address this, availability in frequency domain can be considered in addition to availability in time domain. Thus, the SBFD-aware UE increments HPN for PUSCH transmission in the set of symbols within UL subband. Even if a symbol for PUSCH transmission overlaps with a DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, the HPN can be incremented if PUSCH transmission in the symbols are scheduled within UL subband.

- Proposal 13: In addition to availability in time domain, availability in frequency domain can be further considered to determine HPN of multi-slot PUSCH scheduled by a single DCI for a SBFD-aware UE.

    - PUSCH/PUCCH/SRS

PUSCH/PUCCH frequency hopping

At the RAN1#117 meeting, the following was agreed for PUSCH frequency hopping.

Agreement at RAN1#117 Support separate FH offsets for PUSCH transmissions in SBFD symbols and non-SBFD symbols respectively. - FFS: How to indicate/determine the FH offsets for PUSCH transmissions in SBFD and non-SBFD symbols, respectively. - FFS: Whether/how to update FH calculation to only consider the UL usable PRBs

We discuss whether/how to have separate resources, FH parameters, UL power control parameters, and beam/spatial relation for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots. We provide our views on separate FH parameters for PUCCH and PUSCH on SBFD symbols and non-SBFD symbols in different slots.

If a UE is configured or indicated to transmit PUSCH with frequency hopping (FH), the UE determines FH pattern, i.e., starting RB of 1st hop and 2nd hop respectively. In the current specification, FH pattern can be determined according to the texts from TS 38.214 V18.3.0 [4] as follows.

### 6.3 UE PUSCH frequency hopping procedure ### 6.3.1 Frequency hopping for PUSCH repetition Type A … In case of intra-slot frequency hopping, the starting RB in each hop is given by: , where i=0 and i=1 are the first hop and the second hop respectively, and is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) or as calculated from the resource assignment for MsgA PUSCH (described in [6, TS 38.213]) and is the frequency offset in RBs between the two frequency hops. The number of symbols in the first hop is given by , the number of symbols in the second hop is given by , where NsymbPUSCH,s is the length of the PUSCH transmission in OFDM symbols in one slot. In case of inter-slot frequency hopping and when PUSCH-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot is given by: , where is the current slot number within a system radio frame, where a multi-slot PUSCH transmission can take place, is the starting RB within the UL BWP, as calculated from the resource block assignment information of resource allocation type 1 (described in Clause 6.1.2.2.2) and is the frequency offset in RBs between the two frequency hops.

According to the current specification, the UE determines FH pattern based on the size of UL BWP. If the UL subband is included in the UL BWP, the UE may determine 2nd hop RBs of UL transmission outside the UL subband.

To handle this issue, current modulo operation can be modified without new configuration (i.e., without separate FH parameters). For this purpose, the size of UL BWP (i.e., NBWPsize) can be modified as the size of UL subband with additional parameters such as starting RB index of the UL subband to align the different RB starting between non-SBFD symbols and SBFD symbols as shown in Figure 6.

Figure 6. PUSCH transmission with frequency hopping across non-SBFD symbols and SBFD symbols.

If a UE is configured to transmit PUCCH with frequency hopping (FH), the UE determines FH pattern, i.e., the starting RB of the 1st hop and 2nd, hop respectively. In the current specification, the FH pattern can be determined based on the configured values. That is, the UE transmits the PUCCH starting from a first PRB, provided by startingPRB and starting from a second PRB, provided by secondHopPRB. Different with PUSCH, both starting RBs of the 1st hop and 2nd hop are configured by a gNB. Therefore, a new configuration of the first PRB and second PRB can be introduced for SBFD slots.

Based on the above discussion, we propose enhancement on frequency hopping (both intra-slot and inter-slot) for PUSCH and PUCCH in SBFD operation. Additionally, different frequency hopping patterns can be determined across non-SBFD symbols and SBFD symbols. In non-SBFD symbols, legacy FH pattern should be determined for SBFD-aware UE to guarantee frequency diversity as legacy UE.

- Proposal 14: Separate FH patterns can be determined for PUSCH and PUCCH transmission(s) across SBFD symbols and non-SBFD symbols.

  - For PUSCH, separate FH patterns can be determined by adjusting UL BWP size to UL subband size in current modulo operation without separate FH parameters configuration.

  - For PUCCH, separate FH patterns can be determined by adjusting UL BWP size to UL subband size with separate FH parameters configuration as legacy PUCCH configuration.

PUSCH rate-matching in RE-level

In Rel-15, a RE-level rate-matching for PDSCH was specified around REs for LTE CRS (cell-specific reference signal) to guarantee LTE UE’s CRS detection for DL channel estimation in case of spectrum sharing between NR UE and LTE UE. In addition to LTE CRS, PDSCH rate-matching on REs for ZP (zero power) was specified to guarantee other NR UE’s ZP CSI-RS detection for beam management. A UE can be configured with a RE-level resource pattern for LTE CRS or NR ZP CSI-RS, and then the UE assumes that those REs are not available for PDSCH reception. If the UE is indicated to receive PDSCH in REs that overlap with configured RE-level resource pattern, the UE receives PDSCH by rate-matching REs for PDSCH around the REs for LTE CRS or NR ZP CSI-RS. Note that LTE CRS and NR ZP CSI-RS are likely to be configured in the DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated.

For SBFD-aware UEs, UL transmission can be scheduled within UL subband in a symbol configured as a DL symbol indicated by TDD-UL-DL-ConfigCommon or TDD-UL-DL-ConfigDedicated. Then, resources of PUSCH scheduled to the SBFD-aware UE within the UL subband can overlap with REs of LTE CRS or NR ZP CSI-RS. This can be a more critical issue than the Rel-15 PDSCH case due to CLI between DL reception and UL transmission. For this, a RE-level rate matching for PUSCH can be further investigated, and the Rel-15’s RE-level PDSCH rate matching can be further considered as a starting point.

- Proposal 15: For a SBFD-aware UE, RE-level PUSCH scheduled within UL subband in a symbol configured as DL via TDD-UL-DL-ConfigCommon

  - can be rate-matched around REs of LTE CRS configured for LTE UE in case of spectrum sharing between SBFD-aware UE and LTE UE.

  - can be rate-matched around REs of ZP CSI-RS configured for other NR UE.

Uplink cancellation indication

In Rel-16 URLLC WI, UL CI (cancellation indication) was introduced for inter-UE multiplexing in a cell. To reduce latency of UL transmission of higher priority (i.e., URLLC traffic), a UE can be configured to monitor PDCCH including DCI format 2_4 with CRC scrambled by CI-RNTI. An indication of UL CI for a serving cell is applicable to a PUSCH transmission or a SRS transmission in the cell. For the actual UL cancellation, a UE determines reference time-frequency resource region where UL CI is applied. In time domain, DL symbols configured by TDD-UL-DL-ConfigCommon and symbols configured for SSB reception are excluded from the reference time-frequency resource region of UL CI. In other words, UL symbols and symbols not configured for SSB reception can be included as reference time-frequency resource region of UL CI. This is to guarantee finer signaling granularity under the limited payload size of DCI format 2_4 by excluding time domain resources not used for UL transmission.

UL CI feature can be considered for SBFD operation in consideration that UL latency reduction is one of the key objectives of Rel-19 duplex evolution WI. For example, UL transmission with lower priority (i.e., eMBB traffic) of a SBFD-aware UE within an UL subband can be indicated to cancel by UL CI for another SBFD-aware UE’s UL transmission within the UL subband with higher priority (i.e., URLLC traffic). Since an UL subband can be allocated in symbols configured as DL configured by TDD-UL-DL-ConfigCommon or symbols configured for SSB reception, the SBFD-aware UE excludes those symbols as reference time-frequency resource region that UL CI is applied. However, it seems reasonable that an UL subband in symbols configured as DL by TDD-UL-DL-ConfigCommon and symbols configured for SSB reception should be included as reference time-frequency resource region for UL CI for a SBFD-aware UE in addition to UL symbols and symbols not configured for SSB reception.

- Proposal 16: For a SBFD-aware UE, an UL subband in symbols configured as DL by TDD-UL-DL-ConfigCommon and symbols configured for SSB reception should be included as reference time-frequency resource region for UL CI in addition to UL symbols and flexible symbols not configured for SSB reception.

UE collision handling

At the TSG-RAN#102 meeting, it was agreed to specify UE transmission, reception and measurement especially for collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol. We provide our views on cases of time domain conflict and potential UE behavior to avoid/handle collision cases.

Valid RO vs. DL signal/channel

In legacy TDD operation, a UE can determine valid RO (RACH occasion) within the UL symbol or the flexible symbol (i.e., Ngap symbol after DL and SS/PBCH symbol). For SBFD-aware UE, this can be extended to determine valid RO within the UL symbol in the UL subband. Under the agenda 9.3.2 of SBFD random access, the validity check of ROs was agreed according to the Ngap between non-SBFD symbol or SSB and RO

Furthermore, it might be the case of collisions between valid RO in UL subband and DL signal/channel in DL subband on SBFD symbols, as depicted in Figure 7.

Figure 7. Collision between valid RO and PDSCH.

In Figure 7, symbols for valid RO and PDSCH are overlapped in time domain. To handle this, new UE behavior for SBFD-aware UE can be further considered since the above collision case is not identified in legacy TDD operation. Also, different UE behaviors can be further investigated for the cases whether PDSCH is semi-statically scheduled via RRC configuration or PDSCH is dynamically indicated via DCI.

If PDSCH is semi-statically scheduled, PDSCH can be received for the case when PRACH is not transmitted in valid RO. PDSCH is not received for the case when PRACH is transmitted. Alternatively, PDSCH can be rate-matched even when PRACH is transmitted. If PDSCH is dynamically scheduled, PDSCH can be received and PRACH is not transmitted even in valid RO.

- Proposal 17: For a SBFD-aware UE collision handling on Valid RO vs. DL signal/channel:

  - If PDSCH is dynamically scheduled by a gNB intentionally, PDSCH can be received and PRACH is not transmitted even in valid RO.

  - If PDSCH is semi-statically scheduled, PDSCH can be received for the case when PRACH is not transmitted in valid RO. PDSCH is not received for the case when PRACH is transmitted.

Conclusion

In this contribution, we discussed issues such as semi-static configuration of SBFD subbands, UE transmission/reception/measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE for SBFD operation and summarize our views as the followings:

- Proposal 1: It seems reasonable to carry by at least SIB-1 semi-static indication of time and frequency domain location of SBFD subbands for both UEs in RRC_CONNECTED and UEs in RRC_IDLE/INACTIVE mode.

- Proposal 2: We propose to confirm the working assumption that cell-specific configuration on frequency location of SBFD subbands is supported within a TDD carrier.

  - UE-specific configuration on frequency locations of SBFD subbands is not supported.

- Proposal 3: DL receptions outside DL subband(s) should be allowed for CLI measurement in SBFD operation in a symbol configured as DL or flexible in TDD-UL-DL-ConfigCommon.

- Proposal 4: We propose to support Option 1(i.e., UL usable PRBs are determined as intersection between cell-specific UL subband and active UL BWP in SBFD symbols. DL usable PRBs are determined as intersection between cell-specific DL subband(s) and active DL BWP in SBFD symbols) to determine the usable UL/DL PRBs within UL/DL subband respectively.

- Proposal 5: We support that SFI in DCI format 2_0 is not applicable to SBFD symbols and an SBFD aware UE interprets SFI in DCI format 2_0 as legacy for non-SBFD symbols.

- Proposal 6: We propose that a slot consisting of SSB symbols can be further considered as a full DL slot if the SSB symbols are cell-specifically configured by ssb-PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon, otherwise, SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

- Proposal 7: We propose to reuse PDSCH rate matching mechanism to address unaligned boundaries of DL/UL subband and configuration granularity of RBG for PDSCH FDRA (i.e., FDRA type 0) with less specification impact.

- Proposal 8: If a FDRA of UL transmission or DL reception is available for both SBFD symbols and non-SBFD symbols, the UL transmission or DL reception can be across SBFD symbols and non-SBFD symbols.

- Proposal 9:

- At least for PDSCH repetitions, we support to determine the number of PRBs for TBS determination based on assigned PRB within DL usable PRBs only.

- For the other cases such as SPS PDSCH and multi-PDSCH scheduled by a single DCI, we prefer to determine the number of PRBs for TBS determination based on assigned PRB within DL usable PRBs only.

- Proposal 10: In addition to availability in time domain, availability in frequency domain can be further considered to determine available slot of PUSCH/PUCCH repetition for a SBFD-aware UE.

- Proposal 11: RBs within UL subband in symbols configured as downlink by TDD-UL-DL-ConfigCommon should not be an event of actual TDW determination for a SBFD-aware UE.

- Proposal 12: Although separate UL power control parameters are configured/indicated for PUSCH transmissions or PUCCH transmissions across SBFD symbols and non-SBFD symbols in different slots, we propose to investigate how to maintain power consistency if DMRS bundling is configured as enabled for a SBFD-aware UE.

- Proposal 13: In addition to availability in time domain, availability in frequency domain can be further considered to determine HPN of multi-slot PUSCH scheduled by a single DCI for a SBFD-aware UE.

- Proposal 14: Separate FH patterns can be determined for PUSCH and PUCCH transmission(s) across SBFD symbols and non-SBFD symbols.

  - For PUSCH, separate FH patterns can be determined by adjusting UL BWP size to UL subband size in current modulo operation without separate FH parameters configuration.

  - For PUCCH, separate FH patterns can be determined by adjusting UL BWP size to UL subband size with separate FH parameters configuration as legacy PUCCH configuration.

- Proposal 15: For a SBFD-aware UE, RE-level PUSCH scheduled within UL subband in a symbol configured as DL via TDD-UL-DL-ConfigCommon

  - can be rate-matched around REs of LTE CRS configured for LTE UE in case of spectrum sharing between SBFD-aware UE and LTE UE.

  - can be rate-matched around REs of ZP CSI-RS configured for other NR UE.

- Proposal 16: For a SBFD-aware UE, an UL subband in symbols configured as DL by TDD-UL-DL-ConfigCommon and symbols configured for SSB reception should be included as reference time-frequency resource region for UL CI in addition to UL symbols and flexible symbols not configured for SSB reception.

- Proposal 17: For a SBFD-aware UE collision handling on Valid RO vs. DL signal/channel:

  - If PDSCH is dynamically scheduled by a gNB intentionally, PDSCH can be received and PRACH is not transmitted even in valid RO.

  - If PDSCH is semi-statically scheduled, PDSCH can be received for the case when PRACH is not transmitted in valid RO. PDSCH is not received for the case when PRACH is transmitted.

References

- Draft Report of 3GPP TSG RAN WG1 #118 meeting v0.1.0

- Final Report of 3GPP TSG RAN WG1 #117 meeting v1.0.0

- Final Report of 3GPP TSG RAN WG1 #116bis meeting v1.0.0

- Final Report of 3GPP TSG RAN WG1 #116 meeting v1.0.0

- 3GPP TS 38.214 V18.3.0 (2024-06)