Source: 	MCC Support

Title:	Final Report of 3GPP TSG RAN WG1 #119 v1.0.0

(Orlando, USA, November 18th – 22nd, 2024)

Document for:	Approval

Table of contents

1	Opening of the meeting	5

1.1	Call for IPR	5

1.2	Competition law statement	5

1.3	Network usage conditions	5

1.4	Check-in for Registered Delegates	6

1.5	Recording of RAN1 Meeting	6

2	Approval of Agenda	6

3	Highlights from RAN plenary	6

4	Approval of Minutes from previous meeting	6

5	Incoming Liaison Statements	6

6	Pre-Rel-18 E-UTRA Maintenance	10

7	Pre-Rel-18 NR Maintenance	10

8	Maintenance on Release 18	13

8.1	Maintenance on Rel-18 work items	13

NES	13

MIMO	13

Others	16

Multi-Carrier Enhancement	16

Coverage Enhancement	17

Mobility Enhancement	17

Positioning Enhancement	19

Sidelink	22

NR-NTN	22

IoT-NTN	23

8.2	Rel-18 UE Features	23

9	Release 19	25

9.1	Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface	25

9.1.1	Specification support for beam management	26

9.1.2	Specification support for positioning accuracy enhancement	29

9.1.3	Specification support for CSI prediction	31

9.1.4	Additional study on AI/ML for NR air interface	32

9.1.4.1	CSI compression	32

9.1.4.2	Other aspects of AI/ML model and data	35

9.2	NR MIMO Phase 5	36

9.2.1	Enhancements for UE-initiated/event-driven beam management	36

9.2.2	CSI enhancements	40

9.2.3	Support for 3-antenna-port codebook-based transmissions	43

9.2.4	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	44

9.3	Evolution of NR duplex operation: Sub-band full duplex (SBFD)	45

9.3.1	SBFD TX/RX/measurement procedures	46

9.3.2	SBFD random access operation	50

9.3.3	CLI handling	53

9.4	Study on solutions for Ambient IoT (Internet of Things) in NR	56

9.4.1	Evaluation on Ambient IoT in NR	58

9.4.1.1	Evaluation assumptions and results	58

9.4.1.2	Ambient IoT device architectures	62

9.4.2	Physical layer design for Ambient IoT	64

9.4.2.1	General aspects of physical layer design	64

9.4.2.2	Frame structure and timing aspects	74

9.4.2.3	Downlink and uplink channel/signal aspects	81

9.4.2.4	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	86

9.5	Enhancements of network energy savings for NR	88

9.5.1	On-demand SSB SCell operation	88

9.5.2	On-demand SIB1 for idle/inactive mode UEs	90

9.5.3	Adaptation of common signal/channel transmissions	92

9.6	Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	94

9.6.1	LP-WUS and LP-SS design	94

9.6.2	LP-WUS operation in IDLE/INACTIVE modes	97

9.6.3	LP-WUS operation in CONNECTED modes	99

9.7	Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR	101

9.7.1	ISAC deployment scenarios	102

9.7.2	ISAC channel modelling	106

9.8	Study on channel modelling enhancements for 7-24GHz for NR	109

9.8.1	Channel model validation of TR38.901 for 7-24GHz	109

9.8.2	Channel model adaptation/extension of TR38.901 for 7-24GHz	115

9.9	NR mobility enhancements Phase 4	117

9.9.1	Measurements related enhancements for LTM	118

9.10	XR (eXtended Reality) for NR Phase 3	119

9.10.1	Enabling TX/RX for XR during RRM measurements	119

9.11	Non-Terrestrial Networks (NTN) for NR Phase 3, Internet of Things (IoT) Phase 3, and IoT-NTN TDD mode	120

9.11.1	NR-NTN downlink coverage enhancement	120

9.11.2	Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	122

9.11.3	NR-NTN uplink capacity/throughput enhancement	123

9.11.4	IoT-NTN uplink capacity/throughput enhancement	124

9.11.5	IoT-NTN TDD mode	125

9.12	Multi-Carrier Enhancements for NR Phase 2	128

9.12.1	Multi-cell PUSCH/PDSCH scheduling with a single DCI	128

9.13	TEI	130

9.14	Rel-19 UE Features	131

9.14.1	UE features for AI/ML for NR Air Interface	131

9.14.2	UE features for NR MIMO Phase 5	131

9.14.3	UE features for evolution of NR duplex operation: SBFD	131

9.14.4	UE features for enhancements of network energy savings for NR	131

9.14.5	UE features for LP-WUS/WUR for NR	131

9.14.6	UE features for NR mobility enhancements Phase 4	131

9.14.7	UE features for XR for NR Phase 3	131

9.14.8	UE features for NTN for NR Phase 3	131

9.14.9	UE features for NTN for Internet of Things (IoT) Phase 3	132

9.14.10	UE features for IoT-NTN TDD mode	132

9.14.11	UE features for MCE for NR Phase 2	132

9.14.12	Others	132

10	Closing of the meeting	132

Annex A-1:	List of Tdocs at RAN1 #119	133

Annex A-2:	RAN1#119 Online Session Schedule	134

Annex B:	List of CRs agreed at RAN1 #119	135

Annex C-1:	List of Outgoing LSs from RAN1 #119	137

Annex C-2:	List of Incoming LSs from RAN1 #119	138

Annex D:	List of Approved updated WIDs	139

Annex E:	List of draft TSs/TRs agreed at RAN1 #119	140

Annex F:	List of actions	141

Annex G:	List of participants at RAN1 #119	143

Annex H:	TSG RAN WG1 meetings in 2025 – 2026	144

Main facts summary

The 3GPP TSG WG RAN1#119 meeting, hosted by ATIS, was held at the Hilton Lake Buena Vista, Orlando, USA.

The meeting started at 9:00 on Monday 18th November and finished at 13:35 (local time) on Friday 22nd November 2024.

The number of attending delegates, having confirmed their participation through the electronic check-in application, was 631.

Note that the number of registered (F2F) was 756, and the number of registered (online participation) was 299.

As RAN1#119 was a Face-to-Face meeting with two-way remote participation, GoToWebinar (GTW - for the 3 online sessions) conference calls were set-up for the full duration of the meeting.

Note that to comply with MHPG decision, namely that remote access is to be provided only for the sessions that make formal decisions, there were no access (NO MS TEAMS conf calls) to the offline sessions in Orlando.

The schedule of the RAN1#119 Online Sessions is detailed in Annex A-2.

The list of action points that require RAN1 post meeting follow-up is listed in Annex F (end of document).

The number of contributions for this meeting was 1591.

Note: The amount of documents includes those discussed during the email discussion session post meeting.

|  | Contribution is agreed/approved |
| --- | --- |
|  | Contribution is not pursued (no consensus) |
|  | Further discussion is required – come-back needed |
|  | Working assumption |
|  | Specific action needed from MCC |
|  | Contribution is for email discussion/approval |

# Opening of the meeting

Mr Younsun Kim (RAN1 Chair) welcomed the participants of the RAN WG1 #119 meeting and opened the meeting at 09:00.

Mr. Jerome Vogedes from AT&T on behalf of ATIS and the hosting companies/sponsors, AT&T, Blackberry, Cisco, Comtech, DISH Network, Dolby Labs, Ericsson, 459 Consulting, FUTUREWEI Technologies, Intel, InterDigital Communications, Ligado Networks, Motorola Solutions, NextNav, Nokia, ONEMedia, Pivotal Commware, Qualcomm, Rogers Communications, Southern Linc, T-Mobile USA, Union Telephone Company, and Verizon Wireless, welcomed the delegates, and detailed the domestic arrangements for the full week.

## Call for IPR

The attention of the delegates to the meeting of this Technical Specification Group was drawn to the fact that 3GPP Individual Members have the obligation under the IPR Policies of their respective Organizational Partners to inform their respective Organizational Partners of Essential IPRs they become aware of.

The delegates take note that they are hereby invited:

-	to investigate whether their organization or any other organization owns IPRs which were, or were likely to become Essential in respect of the work of 3GPP.

-	to notify their respective Organizational Partners of all potential IPRs, e.g., for ETSI, by means of the IPR Information Statement and the Licensing declaration forms (e.g. see the ETSI IPR forms https://www.etsi.org/intellectual-property-rights/ipr ).

More information is available via the "Legal" page on the web site: https://www.3gpp.org/about-3gpp/legal-matters .

## Competition law statement

The Chair also drew Member’s attention to the fact that 3GPP activities are subject to antitrust and competition laws and that compliance with said laws is therefore required of any participant of this WG meeting including the Chair and Vice Chairs. In case of question, please contact your legal counsel.

The present meeting will be conducted with strict impartiality and in the interests of 3GPP.

Furthermore, the Chair reminded Members that timely submission of work items/contributions in advance of WG meetings is important to allow for full and fair consideration of such matters.

## Network usage conditions

The PCG has laid down the following network usage conditions:

Users shall not use the network to engage in illegal activities. This includes activities such as copyright violation, hacking, espionage or any other activity that may be prohibited by local laws. Users shall not engage in non-work related activities that consume excessive bandwidth or cause significant degradation of the performance of the network. Since the network is a shared resource, users should exercise some basic etiquette when using the 3GPP network at a meeting. It is understood that high bandwidth applications such as downloading large files or video streaming might be required for business purposes, but delegates should be strongly discouraged in performing these activities for personal use. Downloading a movie or doing something in an interactive environment for personal use essentially wastes bandwidth that others need to make the meeting effective. The meeting Chair should remind end users that the network is a shared resource; the more one user grabs, the less there is for another. Email and its attachments already take up significant bandwidth (certain email programs are not very bandwidth efficient). In case of need the chair can ask the delegates to restrict IT usage to things that are essential for the meeting itself. 1. DON’T place your WiFi device in ad-hoc mode 2. DON’T set up a personal hotspot in the meeting room 3. DO try 802.11a if your WiFi device supports it 4. DON’T manually allocate an IP address 5. DON’T be a bandwidth hog by streaming video, playing online games, or downloading huge files 6. DON’T use packet probing software which clogs the local network (e.g., packet sniffers or port scanners)

## Check-in for Registered Delegates

The attention of the delegates to this meeting was drawn to the fact that it is not permitted to check in other delegates on their behalf. In the event of technical difficulties preventing check-in, delegates should present themselves in person to the Secretary.

## Recording of RAN1 Meeting

Recording of the GoToWebinar sessions of the present meeting is strictly prohibited. No individual or entity - including the speakers and/or the authors - may electronically record any portion of the meeting without prior written consent of the Chair and all the RAN1 meeting participants.

# Approval of Agenda

R1-2409340	Draft Agenda of RAN1#119 meeting	RAN1 Chair

Decision: No comments, the agenda is approved.

In addition to the agenda, the following additional guidance is provided.

R1-2409342	RAN1#119 Meeting Timelines, Scope, Process	RAN1 Chair, ETSI MCC

Decision: The document is noted for information.

# Highlights from RAN plenary

N.A.

# Approval of Minutes from previous meeting

R1-2409341	Report of RAN1#118bis meeting	ETSI MCC

The document was presented by Patrick Merias - MCC (ETSI Mobile Competence Center) and provides the report from RAN1#118bis meeting in Hefei.

Decision: No comments, the report is approved.

# Incoming Liaison Statements

[116bis-Incoming_LSs] – RAN1 Chair

Email discussion on handling of incoming LSs

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

Rel-18 Positioning

R1-2409344	LS on Additional Measurements for SL-TDOA and SL-TOA	RAN2, Qualcomm

Relevant tdocs:

R1-2409663	Draft reply LS on additional Measurements for SL-TDOA and SL-TOA	vivo

R1-2409914	Discussion on additional measurements for SL-TDOA and SL-TOA	CATT, CICTCI

R1-2409915	Draft reply LS on additional measurements for SL-TDOA and SL-TOA	CATT, CICTCI

R1-2410067	Discussion on LS about Additional Measurements for SL-TDOA and SL-TOA	OPPO

R1-2410068	Draft reply LS to RAN2 on Additional Measurements for SL-TDOA and SL-TOA	OPPO

R1-2410144	Draft Reply LS on Additional Measurements for SL-TDOA and SL-TOA	Intel Corporation

R1-2410327	Discussion on LS to RAN1 on additional measurements in SL positioning	Huawei, HiSilicon

R1-2410449	Discussion on additional measurements for SL-TDOA and SL-TOA	ZTE Corporation, Sanechips

R1-2410450	Draft reply LS on additional measurements for SL-TDOA and SL-TOA	ZTE Corporation, Sanechips

R1-2410461	Draft Reply for the LS on Additional Measurements for SL-TDOA and SL-TOA	Qualcomm Incorporated

R1-2410613	Discussion on LS on Additional Measurements for SL-TDOA and SL-TOA	Ericsson

R1-2410615	Draft reply LS Additional Measurements for SL-TDOA and SL-TOA	Ericsson

Decision: RAN2 response related to R1-2407387 from RAN1#118bis. RAN2 is asking for details on requested RAN2 action. To be handled as part of Rel-18 UE features discussion under agenda item 8.2.

R1-2409345	LS to RAN1 on CBR range	RAN2, CATT

Relevant tdocs:

R1-2409400	Discussion on LS to RAN1 on CBR range	Huawei, HiSilicon

R1-2409916	Discussion on CBR range	CATT, CICTCI

R1-2409917	Draft reply LS on CBR range	CATT, CICTCI

R1-2410069	Discussion on LS about CBR range	OPPO

R1-2410070	Draft reply LS to RAN2 on CBR range	OPPO

R1-2410145	Draft Reply LS on on CBR range for SL Positioning	Intel Corporation

R1-2410451	Discussion on CBR range for SL positioning	ZTE Corporation, Sanechips

R1-2410452	Draft reply LS on CBR range for SL positioning	ZTE Corporation, Sanechips

R1-2410462	Draft reply for the LS to RAN1 on CBR range	Qualcomm Incorporated

R1-2410614	Discussion on LS to RAN1 on CBR range	Ericsson

R1-2410616	Draft reply LS on LS to RAN1 on CBR range	Ericsson

Decision: RAN2 requires RAN1 input on current RRC specification not being fully aligned with the RAN1’s parameter list. To be handled under agenda item 8.1 (Positioning) – Ren Da (CATT).

Rel-18 Mobility Enh.

R1-2409346	LS on UE capabilities for inter-frequency L1 measurements for LTM	RAN2, Huawei

Relevant tdocs:

R1-2409924	Discussion on UE capabilities for inter-frequency L1 measurements for LTM	CATT

R1-2410566	Discussion on RAN2 LS on UE capabilities for inter-frequency L1 measurements for LTM	Ericsson

Decision: RAN2 is requesting RAN1 to check RAN2’s understanding on interFreqL1-MeasConfig-r18 and inform them if there is any issue. To be handled as part of Rel-18 UE features discussion under agenda item 8.2.

Rel-18 Less than 5MHz

R1-2409347	LS on carrierBandwidth configuration for less-than-5MHz carriers	RAN2, Huawei

Relevant tdocs:

R1-2410308	Discussion LS on carrier bandwidth configuration for LessThan5MHzFR1	Ericsson

R1-2410331	Draft reply LS on carrierBandwidth configuration for less-than-5MHz carriers	Huawei, HiSilicon

R1-2410457	Discussion on the carrierBandwidth configuration for less-than-5MHz carriers	Qualcomm Incorporated

R1-2410605	Discussion on carrierBandwidth configuration for less-than-5MHz carriers	Huawei, HiSilicon

Decision: RAN2 is requesting RAN4 input on carrierBandwidth configuration in SIB1. Check on Wednesday if RAN1 response is necessary (under agenda item 8.1).

Rel-19 IoT NTN

R1-2409343	LS on OCC for CB-msg3 NPUSCH	RAN2, vivo

Relevant tdocs:

R1-2409498	Discussion on LS on OCC for CB-msg3 NPUSCH	CMCC

R1-2409572	Draft reply RAN2 LS on OCC for CB-msg3 NPUSCH	Samsung

R1-2409622	Discussion on LS on OCC for CB-msg3 NPUSCH	Spreadtrum, UNISOC

R1-2409661	Draft reply LS on OCC for CB-msg3 NPUSCH	vivo

R1-2409662	Discussion on LS on OCC for CB-msg3 NPUSCH	vivo

R1-2409723	Discussion on LS on OCC for CB-msg3 NPUSCH	ZTE Corporation, Sanechips

R1-2409785	Draft Reply LS to RAN2 on OCC for CB-msg3 NPUSCH	Apple

R1-2409830	Discussion on LS on OCC for CB-msg3 NPUSCH	LG Electronics

R1-2409850	Discussion on LS from RAN2 on OCC for CB-msg3 NPUSCH	Nokia, Nokia Shanghai Bell

R1-2409869	Discussion on RAN2 LS on OCC for CB-msg3 NPUSCH	NEC

R1-2409983	Discussion on LS reply on OCC for CB-msg3 NPUSCH	CATT

R1-2410084	Discussion on OCC for CB-msg3 NPUSCH	OPPO

R1-2410085	Draft reply LS on OCC for CB-msg3 NPUSCH	OPPO

R1-2410309	Discussion LS on OCC for CB-msg3 NPUSCH	Ericsson

R1-2410328	Discussion on the reply of LS on OCC for CB-msg3 NPUSCH	Huawei, HiSilicon

R1-2410329	Draft reply LS on OCC for CB-msg3 NPUSCH	Huawei, HiSilicon

R1-2410459	Discussion on OCC for CB-msg3 NPUSCH	Qualcomm Incorporated

R1-2410507	Discussion on reply LS on OCC for CB-msg3 NPUSCH in NB-IoT NTN	MediaTek Inc.

Decision: RAN2 is requesting RAN1 input on the applicability of OCC solution for CB-msg3 NPUSCH. Response is needed. To be handled in agenda item 9.11 ) – Martin (Sony).

R1-2409351	Reply LS to RAN1 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2	RAN4, ZTE Corporation

Decision: RAN4 response to R1-2407548 from RAN1#118bis. To be taken into account as part of discussions on Rel-18 IoT-NTN maintenance.

Rel-19 NES

R1-2409350	LS on SSB relation in On-demand SSB and SSB adaptation on Scell	RAN4, Ericsson

Relevant tdocs:

R1-2409433	Discussion on RAN4 LS on SSB relation in NES	Ericsson

R1-2409495	Discussion on LS on SSB relation in on-demand SSB and SSB adaptation on SCell	CMCC

R1-2409548	LS reply on SSB relation in On-demand SSB and SSB adaptation	ZTE Corporation, Sanechips

R1-2409549	Discussion on LS on SSB relation in On-demand SSB and SSB adaptation	ZTE Corporation, Sanechips

R1-2409571	Draft reply LS on SSB relation in on-demand SSB and SSB adaptation on Scell	Samsung

R1-2409664	Draft reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell	vivo

R1-2409918	Discussion on Reply LS for SSB relation in On-demand SSB and SSB adaptation on Scell	CATT

R1-2410038	Discussion on LS on SSB relation in on-demand SSB and SSB adaptation on SCell	Nokia, Nokia Shanghai Bell

R1-2410039	Draft reply LS on SSB relation in on-demand SSB and SSB adaptation on Scell	Nokia, Nokia Shanghai Bell

R1-2410077	Discussion on SSB relation in On-demand SSB and SSB adaptation operation	OPPO

R1-2410078	Draft reply LS on SSB relation in On-demand SSB and SSB adaptation operation	OPPO

R1-2410147	Draft Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell	Google

R1-2410284	Discussion on RAN4 LS on SSB relation in On-demand SSB and SSB adaptation on Scell	LG Electronics

R1-2410349	Discussion on LS on SSB relation in On-demand SSB and SSB adaptation on SCell	Fujitsu Limited

R1-2410541	Draft reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell	CATT

R1-2410609	Discussion on RAN4 LS on SSB relation in On-demand SSB and SSB adaptation on SCell	Huawei, HiSilicon

Decision: RAN4 is requesting RAN1 input on OD-SSB and SSB adaptation details. Response needed. To be handled in agenda item 9.5 – Ajit (Ericsson).

Rel-19 Duplex Evolution

R1-2409352	LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation	RAN4, Apple

Relevant tdocs:

R1-2409493	Discussion on RAN4 LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation	CMCC

R1-2409537	Draft reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation	ZTE Corporation, Sanechips

R1-2409623	Discussion on LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation	Spreadtrum, UNISOC

R1-2409657	Discussion on the scenarios of UE-to-UE CLI measurement report in SBFD operation	vivo

R1-2409658	Draft reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation	vivo

R1-2409913	Draft reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation	CATT

R1-2410089	Discussion of RAN4 LS on UE-to-UE CLI scenario	OPPO

R1-2410143	Discussion on LS from RAN4 on the scenarios of UE-to-UE CLI measurement report in SBFD operation	Ericsson

R1-2410460	Draft Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation	Qualcomm Incorporated

R1-2410611	Discussion on RAN4 LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation	Huawei, HiSilicon

Decision: RAN4 is requesting RAN1 input on applicable scenarios for UE-to-UE CLI measurement. Response needed. To be handled in agenda item 9.3 – Xinghua (Huawei).

Rel-19 MIMO

R1-2409353	LS on UE RF issues for Rel-19 MIMO enhancement	RAN4, Huawei

Relevant tdocs:

R1-2409383	Draft reply LS to RAN4 on UE RF issues for Rel-19 MIMO enhancement	ZTE Corporation, Sanechips

R1-2409496	Discussion on LS on UE RF issues for Rel-19 MIMO enhancement	CMCC

R1-2409497	Draft reply LS on UE RF issues for Rel-19 MIMO enhancement	CMCC

R1-2409570	Draft reply LS on UE RF issues for Rel-19 MIMO enhancement	Samsung

R1-2409656	Draft LS Reply on UE RF issues for Rel-19 MIMO enhancement	vivo

R1-2409770	Discussion on RAN4 LS on UE RF issues for Rel-19 MIMO enhancement	Ericsson

R1-2409876	Discussion on RAN4 LS on UE RF issues for Rel-19 MIMO enhancement	Xiaomi

R1-2409912	Discussion on RAN4 LS on UE RF issues for Rel-19 MIMO enhancement	CATT

R1-2409974	Draft Reply LS on UE RF issues for Rel-19 MIMO enhancement	Lenovo

R1-2410023	Draft LS reply on UE RF issues for Rel-19 MIMO	Nokia

R1-2410113	Discussion on LS on UE RF issues for Rel-19 MIMO enhancement	OPPO

R1-2410114	Draft reply LS on UE RF issues for Rel-19 MIMO enhancement	OPPO

R1-2410146	Draft Reply LS on UE RF issues for Rel-19 MIMO enhancement	Google

R1-2410458	Discussion on RAN 4 LS on UE RF issues for Rel-19 MIMO enhancement	Qualcomm Incorporated

R1-2410610	Discussion on UE RF issues for Rel-19 MIMO enhancement	Huawei, HiSilicon

Decision: RAN4 is requesting RAN1 input on asymmetric DL sTRP/UL mTRP deployment scenarios and 3TX. Response needed. To be handled in agenda item 9.2. To be moderated by Afshin (InterDigital) and Li (OPPO) under 9.2.3 and 9.2.4, respectively.

Rel-19 AI/ML

R1-2409356	LS on AI/ML Model transfer/delivery to UE	SA5, NEC

Relevant tdocs:

R1-2410283	Discussion of SA5 LS on AI/ML Model Transfer/Delivery to UE	Ericsson

Decision: SA5 is providing updated SA5 status on AI/ML model transfer/delivery to UE. To be taken into account as part of discussions in agenda item 9.1. A few companies considered the text SA5 is referring to belongs to RAN2 part of the TR. RAN2 should be in a position to handle the response to SA5.

Continuation of discussion on RAN2 LS in R1-2407829 from RAN1#118bis

R1-2409478	Discussion and reply LS on applicable functionality reporting for beam management UE-sided model	ZTE Corporation, Sanechips

R1-2409494	Discussion on LS on applicable functionality reporting for beam management UE-sided model	CMCC

R1-2409659	Draft reply LS on applicable functionality reporting for beam management UE-sided model	vivo

R1-2409660	Discussion on LS on applicable functionality reporting for beam management UE-sided model	vivo

R1-2409786	Discussion on RAN2 LS on applicable functionality reporting for beam management UE-sided model	Apple

R1-2409859	Discussion on RAN2 LS on applicable functionality reporting for beam management UE-sided model	NEC

R1-2409911	Discussion on reply LS on applicable functionality reporting for beam management	CATT

R1-2410106	Discussion on LS on applicable functionality reporting for beam management UE-sided model	OPPO

R1-2410107	Draft reply on LS on applicable functionality reporting for beam management UE-sided model	OPPO

R1-2410326	Discussion on the LS reply to RAN2 on functionality in AI/ML	Huawei, HiSilicon

R1-2410355	Discussion of RAN2 LS on applicable functionality reporting for beam management UE-sided model	Ericsson

R1-2410374	Discussion on applicable functionality reporting for beam management UE-sided model	NTT DOCOMO, INC.

R1-2410463	Discussion for LS reply on applicable functionality reporting	Qualcomm Incorporated

Decision: To be handled in agenda item 9.2 – Feifei (Samsung).

For the following LSs, RAN1 was cc-ed.

R1-2409348	Reply LS on synchronization source change at the transmitting anchor UE in SL positioning	RAN2, Ericsson

R1-2409349	Reply LS on Early TA acquisition for the inter-DU scenario	RAN2, Google

R1-2409354	LS on SSB adaptation	RAN4, Apple

R1-2409355	Reply LS on common TA in a regenerative payload scenario	RAN4, CMCC

R1-2409357	Reply LS on AIML Data Collection	SA5, Nokia, ZTE

Following was received during the meeting week:

Rel-19 Ambient IoT

R1-2410923	LS of RAN4 input on Ambient-IoT to TR 38.769	RAN4, CMCC

Decision: RAN4 informs RAN1 that the study is completed from their perspective and provided RAN4 endorsed TP on Ambient IOT to TR 38.769 in R4-2420372. To be considered by SI rapporteur.

R1-2410928	LS on RAN3 outcome of Ambient IoT study	RAN3, Huawei

Decision: RAN3 asks RAN1 to capture the RAN3 endorsed TP on Ambient IOT to TR 38.769 in R3-247924. To be considered by SI rapporteur.

R1-2410930	LS on RAN2 outcome of Ambient IoT study	RAN2, Huawei

Decision: RAN2 asks RAN1 to capture the RAN2 endorsed TP on Ambient IOT to TR 38.769 in R2-2411221. To be considered by SI rapporteur.

# Pre-Rel-18 E-UTRA Maintenance

No contributions submitted to RAN1#119.

# Pre-Rel-18 NR Maintenance

[119-Pre-R18-NR] – RAN1 Chair

To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

Rel-15 New RAT

R1-2409624	Discussion on PDCCH candidate larger AL than the REGs in CORESET	Spreadtrum, UNISOC

Decision: Comeback on Tuesday – Huan (Spreadtrum)

R1-2410740	Moderator’s summary for PDCCH with larger AL	Moderator (Spreadtrum)

From Tuesday session

Conclusion

When the PDCCH candidates with an aggregation level are larger than the REGs in CORESET, UE is not required to monitor these candidates and PDCCH candidates are not counted.

Rel-16 MIMO

Follow up discussion on multi-DCI multi-TRP for PDSCH from RAN1#118bis

R1-2410603	Correction on the configuration of multi-DCI multi-TRP	Huawei, HiSilicon

To be moderated by Yubo (Huawei).

Decision: The issue has already been discussed in previous meetings. The contribution addresses inconsistency between 38.213 and 38.214 regarding the condition of multi-DCI multi-TRP. Comeback on Tuesday.

R1-2410800	Summary on discussion for mDCI mTRP PDSCH	Moderator (Huawei)

Tuesday decision: No further discussion in future meetings.

Rel-17 MIMO

R1-2409575	Draft CR on candidate beams configuration for beam failure recovery	Samsung

Monday decision

Agreement

The change in R1-2409575 is agreed as alignment CR for TS38.213 (Rel-17).

-------------------

Rel-17 MIMO

R1-2409919	Discussion on UE capability for SRS antenna switching	CATT

R1-2410726	Summary on UE capability of SRS antenna switching	Moderator (CATT)

Monday decision: Comeback on Tuesday and  check if proposal 1 can be accepted.

Proposal 1:

On UE indicating supported downgrading configuration of SRS Tx port switching patterns for SRS antenna switching beyond 4Rx, only the SRS Tx port switching patterns with x no more than the largest one associated with the largest y can be indicated.

R1-2410798	Summary#2 on UE capability of SRS antenna switching	Moderator (CATT)

Tuesday decision: No further discussion in future meetings.

-------------------

Rel-17 MIMO

R1-2410121	Discussion on UL TX spatial filter for PUSCH in Rel-17 unified TCI	ZTE Corporation, Sanechips

R1-2410695	Summary on UL Tx spatial filter for PUSCH transmission in R17 UTCI	Moderator (ZTE)

Decision: Most of companies believe the clarification is not needed. Proposal is not pursued.

-------------------

Rel-17 MIMO

Follow up discussion on spatial filter provided by UL TCI state from RAN1#118bis

R1-2409385	Discussion on determining spatial filter provided by UL TCI state in TS38.214	ZTE Corporation, Sanechips

R1-2409386	Draft CR on determining spatial filter provided by UL TCI state in TS38.214	ZTE Corporation, Sanechips, Google, Samsung, Spreadtrum Communications

R1-2410686	Summary of determining spatial filter provided by UL TCI state in TS38.214	Moderator (ZTE)

Monday decision: Comeback on Tuesday – Yang (ZTE).

R1-2410795	Summary#2 of determining spatial filter provided by UL TCI state in TS38.214	Moderator (ZTE)

From Tuesday session

Conclusion

It is RAN1’s understanding that the UE may ignore the reference signal configured in UL TCI state for determining UL TX spatial filter for operation in FR1.

Rel-17 MIMO

Follow up discussion on PDSCH reception scheduled by CORESET not following the unified TCI state from RAN1#118bis

R1-2409368	Discussion on PDSCH reception scheduled by CORESET not following the unified TCI state	MediaTek Inc.

R1-2409369	Draft CR on PDSCH reception scheduled by CORESET not following the unified TCI state	MediaTek. Inc, NTT DOCOMO. INC, Ericsson, Huawei, Hisilicon

R1-2409573	Discussion on TCI state determination for PDSCH reception scheduled by CORESET not following the indicated unified TCI state	Samsung

R1-2409574	Draft CR on TCI state determination for PDSCH reception scheduled by CORESET not following the indicated unified TCI state	Samsung

R1-2409968	Discussion on PDSCH reception scheduled by CORESET not following the unified TCI state	vivo

R1-2410120	Discussion on QCL assumption for PDSCH scheduled by PDCCH in CORESET not following unified TCI	ZTE Corporation, Sanechips

To be moderated by Darcy (MediaTek).

R1-2410652	Summary on PDSCH reception scheduled by CORESET not following the unified TCI state	Moderator (MediaTek Inc.)

From Monday session

Agreement

For the case where scheduling offset >= timeDurationforQCL, clarify the behavior for PDSCH reception scheduled by CORESET NOT following the unified TCI state in Rel-17 spec (TS 38.214 Clause 5.1.5) by adopting Alt2

- Alt2: Regardless of the TCI field present/absent in the DL DCI, the UE applies the unified (indicated) TCI state to the corresponding PDSCH reception.

R1-2410766	Draft Final CR on PDSCH reception scheduled by CORESET not following the unified TCI state	Moderator (MediaTek Inc.)

From Tuesday session

Agreement

- The TP (Rel-17, 38.214, NR_FeMIMO-Core, Cat F) in R1-2410766 is endorsed for Rel-17 TS38.214.

Final CRs in:

R1-2410810	CR on PDSCH reception scheduled by CORESET not following the unified TCI state	Moderator (MediaTek. Inc), ZTE Corporation, Sanechips, Samsung, Ericsson, NTT DOCOMO, OPPO, Google

(Rel-17, 38.214, NR_FeMIMO-Core, CR0647, Cat F) is agreed.

R1-2410814	CR on PDSCH reception scheduled by CORESET not following the unified TCI state	Moderator (MediaTek. Inc), ZTE Corporation, Sanechips, Samsung, Ericsson, NTT DOCOMO, OPPO, Google

(Rel-18, 38.214, NR_FeMIMO-Core, CR0648, Cat A) is agreed.

Maintenance issues on Rel-16 URLLC, Rel-16 2-Step RACH, Rel-17 UE power saving, Rel-17 1024QAM, Rel-17 New RAT, TEI, and LTE NR DC/CA discussed in RAN1 session chaired by Xiaodong.

R1-2410839	Session notes for 7 (Pre-Rel-18 NR maintenance)	Ad-Hoc Chair (CMCC)

Friday decision: The session notes are endorsed and contents reflected below.

Rel-16 URLLC

R1-2410604	Correction on parameters for maximum number of layers in 38.212	Huawei, HiSilicon

R1-2410750	FL summary on Correction on parameters for maximum number of layers in 38.212	Moderator (Huawei)

Presented in Monday session

Decision: The draft CR is not pursued.

Rel-16 2-Step RACH

R1-2410606	Discussion on p0-nominal	Huawei, HiSilicon

R1-2410607	Draft CR for p0-nominal	Huawei, HiSilicon

R1-2410824	Summary#1 of discussion on power control for 2-step RACH	Moderator (Huawei)

R1-2410829	Summary#2 of discussion on power control for 2-step RACH	Moderator (Huawei)

Presented in Wednesday session

No consensus regarding the proposed solutions. Discussion to continue next meeting.

Rel-16 LTE NR DC/CA

R1-2410608	Correction on SCell dormancy indication case 2 in case of BWP switching	Huawei, HiSilicon

R1-2410685	Summary for discussion on SCell dormancy indication case 2	Moderator (Huawei)

From Monday session

Conclusion

The change proposed in R1-2410608 is not necessary.

Final summary in R1-2410770.

Rel-17 NR-NTN

R1-2409707	Draft CR on correction on preamble sequence mapping for NTN	vivo

R1-2410688	Summary on discussion on correction on preamble sequence mapping	Moderator (vivo)

From Monday session

Agreement

Draft CR R1-2409707 is endorsed in principle as alignment CR for 38.211.

Rel-17 UE Power Saving

R1-2410530	Clarification on SSSG timer reset after UE finished DCI-triggered BWP switch	MediaTek Inc.

R1-2410682	Moderator summary 1 of SSSG timer reset after UE finished DCI-triggered BWP switch	Moderator (MediaTek)

Presented in Monday session

R1-2410683	Moderator summary 2 of SSSG timer reset after UE finished DCI-triggered BWP switch	Moderator (MediaTek)

From Thursday session

Conclusion

Update the conclusion made in RAN1 #118-bis as below:

It is common understanding in RAN1 that

- When there is a DCI-based BWP switch for UE configured with R17 search space group switching (searchSpaceGroupIdList-r17), UE switches to the SSSG designated by the DCI on the new BWP, and the SSSG timer is restarted at the first slot boundary after UE finished BWP switching the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

- The above understanding is aligned with current spec.

Final summary in R1-2410871.

Rel-17 1024QAM

R1-2410620	Soft buffer size determination for 1024QAM rank restricted UE	Nokia

Some companies requested more time to check. Discussion to continue next meeting.

Rel-17 New RAT, TEI

R1-2410601	Discussion on skipping uplink transmission in case of BWP switching	Huawei, HiSilicon

R1-2410602	Correction on skipping uplink transmission in case of BWP switching	Huawei, HiSilicon

R1-2410831	Summary of discussion on skipping uplink transmission in case of BWP switching	Moderator (Huawei)

Presented in Wednesday session.

Companies requested more time whether/how the issue should be solved. Discussion to continue next meeting.

# Maintenance on Release 18

## Maintenance on Rel-18 work items

[119-Pre-R18-NR] – RAN1 Chair

To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### NES

R1-2410256	Draft CR on RRC parameter alignment of cellDTX DRX indication in TS38.212	ETRI

From Monday session

Agreement

The following TP is agreed as part of the alignment CR for TS38.212 subclause 7.3.1.3.10.

- If higher layer parameter cellDTX-DRX-L1activation is configured

  - 2 bits as defined in Clause 11.5 of [5, TS38.213] if cellDTX-DRX-ConfigTypeis configured to dtxdrx for the associated serving cell of the block, with the MSB corresponding to cell DTX configuration and the LSB corresponding to cell DRX configuration;

  - 1 bit as defined in Clause 11.5 of [5, TS38.213] if cellDTX-DRX-ConfigTypeis configured to either dtx or drx for the associated serving cell of the block;

R1-2410440	Draft CR on clarification of activation and deactivation time for cell DRX and DTX	Ericsson

From Monday session

Agreement

The TP in R1-2410440 is agreed for TS38.213 alignment CR.

### MIMO

R1-2409387	Discussion on precoding and number of layers indication for Type-1 CG-PUSCH in UL 8-Tx	ZTE Corporation, Sanechips

R1-2410148	Discussion on Precoder and Number of Layers Indication for 8Tx CG-PUSCH	Google

To be moderated by Yushu (Google).

R1-2410738	Summary on Precoder Indication for 8Tx CG-PUSCH	Moderator (Google)

From Monday session

Agreement (confirmed in Tuesday session without the sub-bullets in red)

Send an LS to RAN2 to ask RAN2 to reflect the following RRC parameter:

- precodingAndNumberOfLayers-r18 for precoder and number of layers indication for CG-PUSCH based on 8 antenna ports for codebook based transmission, and the range of its value is from 0 to 1023.

  - The maximum number of layers for the indication of the precoders and number of layers by this RRC parameter is min(4, maxRank)

- srs-ResourceIndicator-r18 for SRI indication for CG-PUSCH based on 8 antenna ports for non-codebook based transmission, and the range of its value is from 0 to 255.

  - When non-codebook based transmission scheme is configured, the maximum number of layers for the SRI indication by this RRC parameter is min(4, maxMIMO-Layers)

LS to RAN2 in:

R1-2410836	LS on Precoder Indication for 8Tx CG-PUSCH	RAN1, Google

Thursday decision: The LS to RAN2 is approved.

R1-2410805	Summary#2 on Precoder Indication for 8Tx CG-PUSCH	Moderator (Google)

From Tuesday session

Agreement

The RRC parameter maxRank and maxMIMO-Layers are applied to both 8TX CG-PUSCH and 8TX DG-PUSCH (same as legacy)

- For determination of TPMI: For 8TX CG-PUSCH, when maxRank is configured to be more than 4, the value of maxRank applied to the 8TX CG-PUSCH is 4

- For determination of SRI: For 8TX CG-PUSCH, when maxMIMO-Layers is configured to be more than 4, the value of maxMIMO-Layers applied to the 8TX CG-PUSCH is 4

Comeback to TP for RAN1 spec

R1-2410876	Draft CR on Precoder Indication for 8Tx CG-PUSCH	Moderator (Google)

From Thursday session

Agreement

- The TP in x10876 is agreed for 38.214.

Final CR in:

R1-2410882	CR on Precoder Indication for 8Tx CG-PUSCH	Moderator (Google), Google, ZTE, Sanechips, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Samsung

(Rel-18, 38.214, NR_MIMO_evo_DL_UL-Core, CR0653, Cat F) is agreed.

R1-2410873	Summary #4 on Precoder Indication for 8Tx CG-PUSCH	Moderator (Google)

Presented in Thursday session.

R1-2409384	Draft CR on implicit BFD-RS determination for S-DCI based MTRP in TS 38.213	ZTE Corporation, Sanechips

From Monday session

R1-2410698	Moderator summary on implicit BFD-RS determination for S-DCI based MTRP	Moderator (ZTE)

Decision: Not an essential correction, draft CR is not pursued.

R1-2409577	Draft CR on applying apply-IndicatedTCIState to PDCCH repetition	Samsung

Monday decision: Comeback on Tuesday.

R1-2410803	Moderator summary on supporting PDCCH repetition under eUTCI	Moderator (Samsung)

From Tuesday session

R1-2410833	Moderator summary on supporting PDCCH repetition under eUTCI round3	Moderator (Samsung)

From Thursday session

Agreement

Under Rel-18 unified TCI framework extension, regarding TCI state(s) indication via higher layer parameter applyIndicated-TCIState for PDCCH reception:

- Clarify that applyIndicated-TCIState setting to ‘both’ is applicable to a CORESET only if sfnSchemePDCCH is configured in a serving cell associated to the CORESET.

Send an LS to RAN2 to request an update to TS38.331.

R1-2410924	[Draft] LS on condition of applying both indicated TCI states for PDCCH reception	Samsung

Friday decision: The draft LS to RAN2 is endorsed and final LS is approved in R1-2410916.

R1-2409578	Draft CR on cell-specific beam resetting for MDCI MTRP under eUTCI	Samsung

Monday decision: Comeback on Tuesday.

R1-2410802	Moderator summary on cell specific beam resetting for MDCI MTRP under eUTCI	Moderator (Samsung)

From Tuesday session

Conclusion

Cell-specific BFR supported in Rel-18 unified TCI framework extension with two indicated unified TCI states is applicable to MDCI based MTRP.

R1-2409920	Correction on TPMI determination for UL transmissions in 38.214	CATT

R1-2410728	Summary on STxMP TPMI determination	Moderator (CATT)

Monday decision: Comeback on Tuesday.

R1-2410799	Summary#2 on STxMP TPMI determination	Moderator (CATT)

From Tuesday session/ confirmed in Thursday session

R1-2410852	Summary#3 on STxMP TPMI determination	Moderator (CATT)

Agreement

Adopt the following TP to 38.214.

6.1.1.1 Codebook based UL transmission <Unrelated parts are omitted> When the higher layer parameter multipanelScheme is set to 'SDMScheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', two SRI(s), and two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator, srs-ResourceIndicator2, precodingAndNumberOfLayers, and precodingAndNumberOfLayers2 in configuredGrantConfig: - When codepoint "10" of SRS Resource Set indicator is indicated or when srs-ResourceIndicator2 and precodingAndNumberOfLayers2 are provided, the first TPMI is used to indicate the precoder to be applied over layers {0…v1-1}, where v1 is the number of layers indicated by the first TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, and the second TPMI is used to indicate the precoder to be applied over layers {v1…. v2+v1-1}, where v2 is the number of layers indicated by the second TPMI, that corresponds to the SRS resource selected by the corresponding SRI when multiple SRS resources are configured for the applicable SRS resource set or if single SRS resource is configured for the applicable SRS resource set, v1 ≤ maxRankSDM or maxRankSDM-DCI-0-2 and v2 ≤ maxRankSDM or maxRankSDM-DCI-0-2 and maxRankSDM or maxRankSDM-DCI-0-2 is defining the maximum number of layers applied over the first and the second SRS resource sets, separately. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI and second TPMI are reserved, the first TPMI is used to indicate the precoder to be applied over layers {0…v-1}, where v ≤ maxRank or maxRankDCI-0-2, where maxRank or maxRankDCI-0-2 is defining the maximum number of layers applied over the first SRS resource set or the second SRS resource set. - Codepoint "11" of SRS Resource Set indicator is reserved. - For one or two TPMI(s), the transmission precoder is selected from the uplink codebook that has a number of antenna ports equal to the higher layer parameter nrofSRS-Ports in SRS-Config for the indicated SRI(s), as defined in Clause 6.3.1.5 of [4, TS 38.211]. When two TPMIs are indicated, the UE shall expect that the precoder indicated by the first TPMI and the precoder indicated by the second TPMI are mapped to different PUSCH antenna ports. - When two SRIs are indicated, the UE shall expect that the number of SRS antenna ports associated with two indicated SRIs would be the same. When the UE is configured with the higher layer parameter txConfig set to 'codebook', the UE is configured with at least one SRS resource. Each of the indicated one or two SRI(s) in slot n is associated with the most recent transmission of SRS resource of associated SRS resource set identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI. When two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', the UE is not expected to be configured with different number of SRS resources in the two SRS resource sets. When higher layer parameter multipanelScheme set to 'SFNscheme' and two SRS resource sets are configured in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with higher layer parameter usage in SRS-ResourceSet set to 'codebook', two SRI(s), and two TPMI(s) are given by the DCI fields of two SRS resource indicator and two Precoding information and number of layers in clause 7.3.1.1.2 and 7.3.1.1.3 of [5, TS 38.212] for DCI format 0_1 and 0_2 or given by srs-ResourceIndicator, srs-ResourceIndicator2, precodingAndNumberOfLayers, and precodingAndNumberOfLayers2 in configuredGrantConfig: - When codepoint "10" of SRS Resource Set indicator is indicated or when srs-ResourceIndicator2 and precodingAndNumberOfLayers2 are provided, the first TPMI is used to indicate precoder to be applied over layers {0…v-1} and the second TPMI is used to indicate the precoder to be applied over layers {0…v-1}, where v ≤ maxRankSFN or maxRankSFN-DCI-0-2 and maxRankSFN or maxRankSFN-DCI-0-2 is defining the maximum number of layers applied over the first SRS resource set and over the second SRS resource set separately. - When codepoint "00" or "01" of SRS Resource Set indicator is indicated, the second SRI and second TPMI are reserved, the first TPMI is used to indicate precoder to be applied over layers {0…v-1}, where v ≤ maxRank or maxRankDCI-0-2, where maxRank or maxRankDCI-0-2 is defining the maximum number of layers applied over the first SRS resource set or the second SRS resource set. <Unrelated parts are omitted>

Final CR in:

R1-2410853	Correction on TPMI determination for UL transmissions in 38.214	Moderator (CATT), New H3C, Ericsson, ZTE, OPPO

(Rel-18, 38.214, NR_MIMO_evo_DL_UL-Core, CR0649, Cat F) is agreed.

R1-2409921	Correction on SRS for DL CSI acquisition in 38.214	CATT

R1-2410727	Summary on SRS antenna switching on 8Tx	Moderator (CATT)

From Monday session

Agreement

Adopt the following as the alignment CR for 38.214:

6.2.1.2 UE sounding procedure for DL CSI acquisition When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R) or the UE may be configured with the following configurations depending on the indicated UE capability srs-AntennaSwitching8T8R ('t1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r2' for 1T2R, 't4r4' for 4T=4R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't2r6' for 2T6R, 't1r6' for 1T6R, 't4r8' for 4T8R, 't2r8' for 2T8R, 't1r8' for 1T8R, ‘noTdm’ or ‘tdmAndNoTdm’ for 8T8R): <Unrelated parts are omitted>

R1-2410625	Discussion on STxMP with DMRS Bundling	Ericsson

Monday decision: Comeback on Tuesday.

R1-2410670	Summary of NR Rel-18 Maintenance Discussion on STxMP with DMRS Bundling	Moderator (Ericsson)

Presented in Tuesday session.

R1-2410860	Summary#3 of NR Rel-18 Maintenance Discussion on STxMP with DMRS Bundling	Moderator (Ericsson)

From Thursday session

Conclusion

- Although RAN1 specifications do not explicitly preclude where pusch-DMRS-Bundling and/or pucch-DMRS-Bundling is configured with multipanelSchemeSDM, multipanelSchemeSFN, or sTx-2Panel, in Rel-18, RAN1 and RAN4 did not develop STxMP targeting the support of DMRS bundling with STxMP PUSCH/PUCCH transmission, and RAN4 input would be needed on the conditions under which DMRS bundling can be used with STxMP PUSCH/PUCCH transmission as well as to develop any potential UE features for the combination of STxMP PUSCH/PUCCH transmission and DMRS bundling.  Note that:

  - DMRS bundling can only be used when the UE is scheduled a one layer PUSCH using PUSCH repetition

  - DMRS bundling can only be used for PUCCH using PUCCH repetition

- Alt 2: RAN1 leaves further discussion of the need for any such UE features to RAN4’s discretion.

Final summary in R1-2410877.

### Others

R1-2410464	Clarifications for 2 TB SPS PDSCH	Qualcomm Incorporated

R1-2410709	FL summary #1 on clarifications for 2 TB SPS-PDSCH and CG-PUSCH	Moderator (Qualcomm Incorporated)

From Thursday session

Conclusion

2 TBs SPS-PDSCH is not supported in NR specification. In a DL BWP configured with 2 TBs, the same mechanism used to disable a TB for DG-PDSCH is reused to disable one of the two TBs for SPS-PDSCH.

Conclusion

NR specification does not support a SPS-PDSCH/CG-PUSCH activation DCI activates SPS-PDSCH/CG-PUSCH new transmission for one TB while schedules SPS-PDSCH/CG-PUSCH retransmission for another TB.

From AI 5

R1-2409347	LS on carrierBandwidth configuration for less-than-5MHz carriers	RAN2, Huawei

Thursday decision: No RAN1 response is necessary.

Maintenance issues on Rel-18 MCE, Coverage Enh, and Mobility Enh discussed in RAN1 session chaired by Xiaodong.

### Multi-Carrier Enhancement

R1-2410842	Session notes for 8.1 (Maintenance on Rel-18 work items – Multi-Carrier)	Ad-Hoc Chair (CMCC)

Friday decision: The session notes are endorsed and contents reflected below.

R1-2409923	Discussion on last DCI determination of DCI format 1_3 for HARQ-ACK codebook	CATT

R1-2409984	Draft CR on last DCI determination of DCI format 1_3 for HARQ-ACK codebook	CATT

R1-2410761	Summary#1 of last DCI determination of DCI format 1_3 for HARQ-ACK codebook	Moderator (CATT)

Presented in Monday session.

R1-2410896	Summary#2 of last DCI determination of DCI format 1_3 for HARQ-ACK codebook	Moderator (CATT)

From Thursday session

No additional specification impact.

R1-2409922	Draft CR on Type-3 HARQ-ACK codebook triggered by DCI format 1_3	CATT

R1-2410623	Draft CR on Type-3 HARQ-ACK codebook triggered by DCI format 1_3	Nokia

R1-2410760	Summary#1 of Type-3 HARQ-ACK codebook triggered by DCI format 1_3	Moderator (CATT)

Presented in Monday session.

R1-2410897	Draft CR on Type-3 HARQ-ACK codebook triggered by DCI format 1_3	Moderator (CATT), Nokia

From Thursday session

Agreement

Draft CR R1-2410897 to TS 38.213 is endorsed in principle as alignment CR.

Final summary in R1-2410897.

R1-2410597	Maintenance of Rel-18 Multicarrier Enhancements	Huawei, HiSilicon

R1-2410612	Corrections on determination of DCI format 0_3 for unpaired spectrum operation	Huawei, HiSilicon

R1-2410622	Ambiguity in field sizes of DCI format 0_3 of a scheduled cell that is dormant.	Nokia

R1-2410705	[119-R18-Maintenance] DCI format 0_3 field size determination	Moderator (Nokia)

From Tuesday session

R1-2410868	Ambiguity in field sizes of DCI format 0_3 of a scheduled cell that is dormant	Moderator (Nokia), Nokia, Huawei, HiSilicon, Samsung

No consensus. Not pursued.

R1-2410597	Maintenance of Rel-18 Multicarrier Enhancements	Huawei, HiSilicon

R1-2410598	Corrections on bit-width determination of Type-1B DCI field for multi-cell scheduling	Huawei, HiSilicon

R1-2410809	Clarification on DCI field value of Type 1-B DCI fields in DCI format 0_3/1_3 in case of a DCI field size of 0 in TS 38.212	Moderator(Huawei), Nokia	(rev of R1-2410621)

R1-2410684	On DCI field value of Type 1-B in DCI format 0_3/1_3	Moderator (Huawei)

From Tuesday session

Agreement

Draft CR R1-2410598 to TS 38.212 is endorsed in principle as alignment CR.

Final summary in R1-2410867.

R1-2410190	Draft CR on Type-1 and Type-2 HARQ-ACK codebook	ZTE Corporation, Sanechips

R1-2410774	Summary of the discussion on Type-1 and Type-2 HARQ-ACK codebook

From Tuesday session

Agreement

Draft CR R1-2410190 to TS 38.213 is endorsed in principle as alignment CR.

R1-2409665	Draft CR on a bandwidth part indicator by a DCI format 0_3/1_3	vivo

From Tuesday session

Agreement

Draft CR R1-2409665 to TS 38.213 is endorsed in principle as alignment CR.

### Coverage Enhancement

R1-2410840	Session notes for 8.1 (Maintenance on Rel-18 work items - Coverage enhancements )	Ad-Hoc Chair (CMCC)

Friday decision: The session notes are endorsed and contents reflected below.

R1-2409580	Discussion on the time period determination for multiple PRACH transmission	Samsung

Not to be discussed in this meeting.

### Mobility Enhancement

R1-2410841	Session notes for 8.1 (Maintenance on Rel-18 work items - Mobility Enhancement)	Ad-Hoc Chair (CMCC)

Friday decision: The session notes are endorsed and contents reflected below.

R1-2410243	Draft CR on LTM TA command application time	MediaTek Inc.

R1-2410744	Summary on LTM TA command application time	Moderator (MediaTek)

Presented in Monday session.

From Thursday session

R1-2410812	Summary#2 on LTM TA command application time	Moderator (MediaTek)

R1-2410903	Summary#3 on LTM TA command application time	Moderator (MediaTek)

R1-2410904	Draft CR on LTM TA command application time	Moderator (MediaTek)

Agreement

Draft CR R1-2410904 to Section 4.2, TS 38.213 is endorsed in principle.

Final CR in:

R1-2410920	CR on LTM TA command application time	Moderator (MediaTek), NTT Docomo, ZTE, Nokia

(Rel-18, 38.213, NR_Mob_enh2-Core, CR0692, Cat F) is agreed.

Final summary in R1-2410919.

R1-2410542	Correction on handling collision of SSB and RA occasion for LTM	Google

R1-2410759	Moderator summary on handling collision of SSB and RA occasion for LTM

Presented in Monday session.

R1-2410816	Moderator summary 2 on handling collision of SSB and RA occasion for LTM	Moderator (Google)

From Thursday session

R1-2410901	Draft LS on collision between SSB and RA occasion for LTM	Google

Decision: The draft LS R1-2410901 is endorsed in principle. Final LS is approved in R1-2410911.

R1-2410123	Draft CR on correction of parameter name for early UL sync in TS 38.211	ZTE Corporation, Sanechips

R1-2410697	Summary on correction of parameter name for early UL sync in TS 38.211	Moderator (ZTE)

From Monday session

Agreement

Draft CR R1-2410123 to TS 38.211 is endorsed in principle for alignment CR.

R1-2410122	Draft CR on correction of parameter name for early UL sync in TS 38.213	ZTE Corporation, Sanechips

R1-2410696	Summary on correction of parameter name for early UL sync in TS 38.213 	Moderator (ZTE)

From Monday session

Agreement

Draft CR R1-2410122 to TS 38.213 is endorsed in principle for alignment CR.

R1-2410242	Draft CR on LTM PRACH and serving cell UL transmission in a same band	MediaTek Inc.

R1-2410743	Summary on LTM PRACH and serving cell UL transmission	Moderator (MediaTek)

Presented in Monday session.

R1-2410811	Summary#2 on LTM PRACH and serving cell UL transmission	Moderator (MediaTek)

R1-2410902	Summary#3 on LTM PRACH and serving cell UL transmission	Moderator (MediaTek)

From Thursday session

Discussion is postponed till the next meeting.

Final summary in R1-2410918.

R1-2410600	Corrections to PRACH transmission for LTM in TS38.211	Huawei, HiSilicon

R1-2410771	Discussion summary of Corrections to PRACH transmission for LTM in TS38.211	Moderator (Huawei)

From Tuesday session

Agreement

The TP in section 3 of R1-2410771 for TS38.211 is endorsed in principle.

Final CR in:

R1-2410879	Corrections to PRACH transmission for LTM	Huawei, HiSilicon, Nokia, Google, Samsung, Fujitsu, Ericsson, NTT DOCOMO, ZTE

(Rel-18, 38.211, NR_Mob_enh2-Core, CR0147, Cat F) is agreed.

R1-2410599	Corrections to SSB-RO association for LTM in TS38.213	Huawei, HiSilicon

R1-2410772	Discussion summary of Corrections to SSB-RO association for LTM in TS38.213	Moderator (Huawei)

From Tuesday session

Agreement

The TP in section 3 of R1-2410772 for TS38.213 is endorsed in principle.

Final CR in:

R1-2410878	Corrections to SSB-RO association for LTM	Huawei, HiSilicon, Nokia, Google, Samsung, Fujitsu, Ericsson, NTT DOCOMO, ZTE

(Rel-18, 38.213, NR_Mob_enh2-Core, CR0691, Cat F) is agreed.

R1-2410565	Draft CR for 38.213 on LTM power control	Ericsson

R1-2410749	Discussion summary of corrections to LTM power control	Moderator (Ericsson)

From Tuesday session

No consensus.

Maintenance issues on Rel-18 Positioning Enh, Sidelink, and NR/IoT-NTN discussed in RAN1 session chaired by David.

R1-2410843	Session notes for 8.1 (Maintenance on Rel-18 work items - IoT NTN/NR NTN, Sidelink, Positioning Enhancements)	Ad-Hoc Chair (Huawei)

Friday decision: The session notes are endorsed and contents reflected below.

### Positioning Enhancement

Follow up discussion from RAN1#118bis

R1-2409666	Discussion on active semi-persistent SRS resource configuration and transmission of SRS	vivo

R1-2409667	Draft CR on active semi-persistent SRS resource configuration and transmission of SRS	vivo

R1-2410453	Discussion on active semi-persistent SRS resource	ZTE Corporation, Sanechips

R1-2410617	Draft CR for correction to semi-persistent configuration of UL SRS in 38.214	Ericsson

R1-2410618	Discussion on correction to semi-persistent configuration of UL SRS in 38.214	Ericsson

To be moderated by Yuan (vivo)

R1-2410787	Summary#1 on semi-persistent configuration of UL SRS and transmission of SRS	Moderator (vivo)

From Wednesday session

Agreement

The TP in section 4 of R1-2410787 is endorsed for Rel-18 for clause 6.2.1 and 6.2.1.4 of TS 38.214.

The first change in the TP is also endorsed for Rel-17.

Comeback for final CRs (Yuanyuan)

R1-2410864	CR on active semi-persistent SRS resource configuration and transmission of SRS	Moderator(vivo), Ericsson, Intel

(Rel-18, 38.214, NR_pos_enh2-Core, CR0650, Cat F) is agreed.

R1-2410865	CR on active semi-persistent SRS resource configuration	Moderator(vivo), Ericsson, Intel

(Rel-17, 38.214, NR_pos_enh-Core, CR0651, Cat F) is agreed.

MCC: To remove “draft” from title before submission to plenary.

R1-2410454	Corrections on positioning in 38.214	ZTE Corporation, Sanechips

To be moderated by Cong (ZTE)

R1-2410815	Summary on Corrections for positioning in 38.214	Moderator (ZTE)

From Wednesday session

Agreement

The TP below is endorsed for the Rel-18 alignment CR for TS 38.214

| Reason for change: | IE name phaseQualityValue is not aligned with the latest version of 37.355: – NR-PhaseQuality The IE NR-PhaseQuality defines the quality of the RSCP/RSCPD measurement. -- ASN1START NR-PhaseQuality-r18 ::= SEQUENCE { phaseQualityIndex-r18 INTEGER (0..179), phaseQualityResolution-r18 ENUMERATED {mdot1, m1,...}, ... } -- ASN1STOP NR-PhaseQuality field descriptions. phaseQualityIndex This field provides an index value for an estimate of the uncertainty of the reported phase for which the IE NR-PhaseQuality is provided in units of degrees.. phaseQualityResolution This field provides the resolution used in the phaseQualityIndex field. Enumerated values mdot1 and m1 correspond to 0.1 and 1 degrees respectively.. Furthermore, the filed description should also be updated: - phaseQualityIndex: provides the index value for the uncertainty of the measurement. - PhaseQualityResolution: the word ‘levels’ should be removed for the following two reasons: 1- To align with field description in 37.355: This field provides the resolution used in the phaseQualityIndex field. 2- When UE reports a resolution, ONLY ONE resolution could be reported, where the reported value is chosen from the candidate values. The reported resolution cannot be a plural description. |
| --- | --- |
| Summary of change: | Correct IE name and corresponding field description related to phase quality measurement in 38.214. |
| Consequences if not approved: | IE name and corresponding field descriptions for phase quality report in Rel-18 positioning are not correct. |

------ Start of TP ------

5.1.6.5.2	PRS for carrier phase positioning

<omitted text>

The UE may be configured to report quality metrics NR-PhaseQuality corresponding to the DL RSCP and RSCPD measurements which include the following fields [17, TS 37.355]:

-	phaseQualityIndex which provides the uncertainty of the measurement

-	phaseQualityResolution which specifies the resolution levels used in the phaseQualityIndex field.

------ End of TP ------

R1-2410455	Draft CR on timeline of BW aggregation SRS for positioning	ZTE Corporation, Sanechips

To be moderated by Chuangxin (ZTE)

R1-2410801	Summary on timeline of BW aggregation SRS for positioning	Moderator (ZTE)

From Wednesday session

Agreement

The TP below is endorsed for TS38.214.

| Reason for change: | In RAN1#118bis meeting, the timeline was agreed for positioning SRS bandwidth aggregation when there is collision between PUSCH/PUCCH and the positioning SRS. It specified the time interval between the last symbol of PDCCH and the SRS. For an SRS transmission for bandwidth aggregation starting in symbol of carrier and a conflicting transmission in any aggregated carrier starting in symbol, the UE shall apply the prioritization / dropping rules in this clause taking into account: - DCI(s) for which the time interval between the last symbol of PDCCH and is at leastsymbols and an additional guard period when applicable, and the time interval between the last symbol of PDCCH and is at least symbols; where the time interval unit of OFDM symbol is counted based on the subcarrier spacing of the aggregated SRS transmission.. However, the conflicting transmission may also be semi-persistent CSI report or semi-persistent MIMO SRS activated by a MAC CE. In such case, the activation should not be too closed to the positioning SRS, otherwise, UE may not have sufficient to drop the positioning SRS. The corresponding description has been captured for carrier switching SRS and positioning SRS with frequency hopping for Redcap UE. |
| --- | --- |
| Summary of change: | Defines the activation used to active CSI reports or MIMO SRS for which the time interval between the activation and the colliding positioning SRS |
| Consequences if not approved: | UE behavior is missing and ambiguous for the case when semi-persistent CSI report or semi-persistent MIMO SRS collides with bandwidth aggregation SRS for positioning |

------ Start of TP ------

6.2.1.4.2	SRS bandwidth aggregation for positioning measurements

The UE is expected to be configured with linkage information SRS-PosResourceSetLinkedForAggBWList on SRS resource sets for positioning across two or three CCs which are linked for bandwidth aggregation. For the linked SRS resource sets, the UE is expected to be configured with the same values of startPosition, nrofSymbols, periodicityAndOffset, slotOffset, alpha, p0, spatialRelationInfoPos, resourceType, subcarrierSpacing, cyclicPrefix, and transmissionComb, and the UE is expected to maintain phase continuity for the SRS transmission on the same symbol(s). The UE assumes that SRS resources across the linked SRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, otherwise, the UE does not assume that SRS resources of the linked SRS resource sets are linked for bandwidth aggregation.

If the UE is configured with dci-TriggeringPosResourceSetLink, and if the UE receives a DCI 0_1, 0_2, 1_1, or 1_2 triggering an aperiodic SRS resource set for positioning linked for bandwidth aggregation in a CC, subject to UE capability, UE transmits SRS of the linked SRS resource sets across all CCs.

A UE in RRC_INACTIVE mode is expected to be configured with frequency information via freqInfo in SRS-PosResourceSetLinkedForAggBW for additional component carrier(s) with respective SRS configuration(s) for bandwidth aggregation.

When an SRS resource configured in a CC without PUSCH or PUCCH is linked for bandwidth aggregation with an SRS resource configured in an active UL BWP of another CC in the same band, there is a guard period during which the UE is not expected to transmit or receive other signals or channels in this band, or any other affected band(s), subject to UE capability.

For the linked SRS resource sets for bandwidth aggregation across CCs in RRC_CONNECTED state, if an SRS configured by the higher layer parameter SRS-PosResource, along with the guard period when applicable, collides with other signals or channels on a symbol and if the SRS in that symbol is dropped, SRS transmission of the linked SRS resource sets across all CCs is dropped on that symbol.

For the linked SRS resource sets for bandwidth aggregation in RRC_INACTIVE state, if an SRS configured by the higher layer parameter SRS-PosResource, along with the guard period when applicable, collides with other signals or channels on a symbol, SRS transmission of all linked SRS resource sets is dropped on that symbol.

If the UE receives an activation or deactivation command of semi-persistent SRS resource set(s) for positioning in up to  three aggregated carriers or SRS resource set(s) for positioning in up to two aggregated carriers as specified in [10, TS 38.321] and when the UE would transmit a PUCCH with HARQ-ACK information in slot n corresponding to the PDSCH carrying the activation or deactivation command, the corresponding actions in [10, TS 38.321] and the UE assumptions on SRS transmission or cessation corresponding to the SRS resource set(s) shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ where µ is the SCS configuration for the PUCCH.

For positioning SRS resources on multiple carriers linked for aggregation, the channel over which a symbol on one carrier for SRS transmission is conveyed can be inferred from the channel over which the same symbol of another carrier or the aggregated carrier is conveyed.

For an SRS transmission for bandwidth aggregation starting in symbol  of carrier  and a conflicting transmission in any aggregated carrier starting in symbol, the UE shall apply the prioritization / dropping rules in this clause taking into account:

-	DCI(s) for which the time interval between the last symbol of PDCCH and  is at leastsymbols and an additional guard period when applicable, and the time interval between the last symbol of PDCCH and  is at least  symbols;

For an SRS transmission for bandwidth aggregation starting in symbol  of carrier  and a conflicting transmission in any aggregated carrier starting in symbol, the UE shall apply the prioritization / dropping rules in this clause taking into account:

-	DCI(s) for which the time interval between the last symbol of PDCCH and  is at leastsymbols and an additional guard period when applicable, and the time interval between the last symbol of PDCCH and  is at least  symbols;

-	semi-persistent CSI reports or SRS configured by the higher layer parameter SRS-Resource considered active at least N2 symbols and an additional guard period before Nc1, and considered active at least N2 symbols before NS.

where the time interval unit of OFDM symbol is counted based on the subcarrier spacing of the aggregated SRS transmission.

<omitted text>

------ End of TP ------

Comeback for final CR (Chuangxin)

R1-2410869	CR on timeline of BW aggregation SRS for positioning	Moderator (ZTE), ZTE Corporation, Sanechips

(Rel-18, 38.214, NR_pos_enh2-Core, CR0652, Cat F) is agreed.

R1-2410596	Draft CR on the prioritization of SL-PRS	Huawei, HiSilicon

To be moderated by Jinhuan (Huawei)

R1-2410796	Feature Lead summary on discussion for prioritization of SL-PRS	Moderator (Huawei)

From Thursday session

Conclusion

For the prioritizations for sidelink and uplink transmissions/receptions, it is RAN1 understanding that SL transmission/receptions is referred to as NR sidelink transmission/receptions in dedicated SL PRS resource pool or as NR sidelink in communication resource pool, respectively.

- No spec change.

From AI 5

R1-2409345	LS to RAN1 on CBR range	RAN2, CATT

Decision: To be handled under agenda item 8.1 (Positioning) – Ren Da (CATT).

R1-2410706	Summary of discussion of reply LS on CBR Range	Moderator (CATT)

From Wednesday session

Agreement

Provide the following response to RAN2’s LS on CBR:

- RAN1 prefers that the maximum values for the following parameters in ASN.1 are specified as proposed by RAN1, i.e.,

  - The maximum number of CBR ranges for SL positioning is set to 8 in RRC IE sl-CBR-RangeDedicatedSL-PRS-RP-List-r18,

  - The maximum number of CBR levels is set to 16 in RRC IE SL-CBR-LevelsDedicatedSL-PRS-RP,

  - The maximum number of SL PRS transmission Configuration index in sl-PRS-TxConfigIndexList-r18 is set to 16.

- However, although it is not preferred, it is acceptable that the CBR range, CBR level and SL-PRS-TxConfigIndexList parameters are not extended, as RAN2 agreed.

- RAN1 respectfully requests RAN2 to identify whether there exists any other ASN.1 issue, if the CBR range, CBR level and SL-PRS-TxConfigIndexList parameters are not extended.

  - E.g. SL-CBR-LevelsConfig-Dedicated-SL-PRS-RP is used but not defined, The maxNrofSL-PRS-TxConfig-r18 should be 63 instead of 64

  - E.g. the index range used in SL-PriorityTxConfigIndexDedicatedSL-PRS-RP should not exceed the range defined in SL-CBR-CommonTxDedicatedSL-PRS-RP-List

Comeback for draft LS on Thursday (Ren)

R1-2410707	Draft Reply LS on CBR Range	Moderator (CATT)

Thursday decision: The draft Reply LS on CBR Range in R1-2410707 is endorsed. Final LS is approved in R1-2410708.

### Sidelink

R1-2410375	Draft CR on usage of the 2nd starting symbol	NTT DOCOMO, INC.

To be moderated by Shohei (NTT DOCOMO)

From Monday session

R1-2410745	Summary of discussion for R1-2410375	Moderator (NTT DOCOMO, INC.)

Agreement

Endorse the TP below for TS37.213

----- Start of TP -----

4.5	Sidelink channel access procedures

<Unchanged parts omitted>

For SL transmission(s) with multiple starting positions in a slot, the following are applicable:

-	If a UE intends to transmit PSCCH/PSSCH in sidelink resource allocation mode 1 or mode 2 using a Type 1 SL channel access procedure, and if the UE cannot access the channel for the transmission from the 1st starting symbol of a slot, the UE shall attempt to transmit PSCCH/PSSCH from the 2nd starting symbol in the same slot according to Type 1 SL channel access procedure. There is no limit on the number of attempts the UE can make using Type 1 SL channel access procedure.

-	If a UE intends to transmit PSCCH/PSSCH in sidelink resource allocation mode 1 or mode 2 using a Type 2 SL channel access procedure, and if the UE cannot access the channel for the transmission from the 1st starting symbol of a slot, the UE shall attempt to transmit PSCCH/PSSCH from the 2nd starting symbol in the same slot and according to Type 2A SL channel access procedure.

----- End of TP -----

Comeback for the draft CR (Shohei)

R1-2410857	Draft CR on transmission attempt from the 2nd starting symbol	Moderator (NTT DOCOMO, INC.), OPPO, Huawei, HiSilicon

From Thursday session

Agreement

The draft CR in R1-2410857 is endorsed with removing the impact to Radio Access Network.

Final CR (Rel-18, 37.213, NR_SL_enh2-Core, CR0063, Cat F) is agreed in R1-2410858.

Final summary in R1-2410856.

R1-2410456	Correction on the application condition of two candidate starting symbols in TS 38.213	ZTE Corporation, Sanechips

To be moderated by Mengzhen (ZTE)

R1-2410746	FLS#1 on 2 candidate starting symbols in TS 38.213	Moderator (ZTE)

Monday decision: Comeback on Thursday (Mengzhen)

R1-2410783	FLS#2 on 2 candidate starting symbols in TS 38.213	Moderator (ZTE)

From Thursday session

Conclusion

Continue to discuss the following issues in RAN1#120:

- Issue 1: misalignment between 38.213 and 38.214 on UE behavior on using sl-StartingSymbolFirst and/or sl-StartingSymbolSecond

- Issue 2: UE behavior at least when only sl-StartingSymbolSecond is (pre-)configured and sl-StartingSymbolFirst is not (pre-)configured

### NR-NTN

R1-2409706	Discussion on NG-RAN measurement in NTN	vivo

To be moderated by Siqi (vivo)

R1-2410689	Moderator summary#1 on NR-RAN measurement in NTN	Moderator (vivo)

From Monday session

For gNB Rx – Tx time difference, UL SRS-RSRP and UL SRS-RSRPP

Conclusion

Draft CR#1 for gNB Rx – Tx time difference in section 4.1 of R1-2409706 for 38.215 is not pursued.

Comeback for proposal 3 in R1-2410689 on Thursday (Siqi)

Agreement

- Adopt draft CR#3 for UL SRS-RSRP and UL SRS-RSRPP in section 4.3 of R1-2409706 for 38.215.

From Thursday session, final CR in:

R1-2410794	Correction to reference point for NG-RAN measurement for UL SRS-RSRP and UL SRS-RSRPP	Moderator (vivo)

(Rel-18, 38.215, NR_NTN_enh-Core, CR0070, Cat F) is agreed.

R1-2410791	Moderator summary#2 on NR-RAN measurement in NTN	Moderator (vivo)

From Thursday session

Conclusion:

RAN1 specification changes (proposal 3 in R1-2410689) to support UL Angle of Arrival (UL AoA) for R18 NR NTN are not pursued.

R1-2409708	Draft CR on correction on support of open loop timing advance calculation for ATG	vivo

To be moderated by Siqi (vivo)

R1-2410690	Summary#1 on discussion on correction on open loop timing advance calculation for ATG	Moderator (vivo)

From Monday session

Agreement

Adopt the following TP for 38.213

----- Start of TP -----

4.2 Transmission timing adjustments <Unchanged parts are omitted > Using higher-layer ephemeris parameters for a serving satellite or atg-gNB-Location for ATG, if provided, a UE pre-compensates the two-way transmission delay on the service link or on the link between gNB and UE for ATG based on NTA,adjUE that the UE determines using the serving satellite position or gNB location for ATG and its own position. To pre-compensate the two-way transmission delay between the uplink time synchronization reference point and the serving satellite, the UE determines NTA,adjcommon [4, TS 38.211] based on one-way propagation delay Delaycommont that the UE determines as: Delaycommont= TACommon2+ TACommonDrift2×t-tepoch+TACommonDriftVariant2×t-tepoch2 where TACommon, TACommonDrift, and TACommonDriftVariant are respectively provided by ta-Common, ta-CommonDrift, and ta-CommonDriftVariant and tepoch is provided by epochTime which is the epoch time of ta-Common, ta-CommonDrift, and ta-CommonDriftVariant [12, TS 38.331]. Delaycommon(t) provides a distance at time t between the serving satellite and the uplink time synchronization reference point divided by the speed of light. The uplink time synchronization reference point is the point where DL and UL are frame aligned with an offset given by NTA,offset. <Unchanged parts are omitted >

----- End of TP -----

Comeback for draft CR, where the purple part agreed at the last meeting is removed (Siqi).

R1-2410792	Summary#2 on discussion on correction on open loop timing advance calculation for ATG	Moderator (vivo)

From Thursday session

Agreement

- Adopt draft CR for the support of open loop timing advance calculation for ATG in section 5 of R1-2410792 for 38.213. Final CR (revision of CR agreed in RAN1#118bis) in:

R1-2410797	Correction on the support of open loop timing advance calculation for ATG	Moderators (Huawei, vivo), HiSilicon, Samsung, Ericsson

(Rel-18, 38.213, NR_NTN_enh-Core, CR0685rev2, Cat F) is agreed.

### IoT-NTN

R1-2409705	Discussion on E-UTRAN measurement in IoT NTN	vivo

To be moderated by Siqi (vivo)

R1-2410691	Moderator summary#1 on E-UTRAN measurement in IoT NTN	Moderator (vivo)

Monday decision: Comeback on Thursday (Siqi)

R1-2410793	Moderator summary#2 on E-UTRAN measurement in IoT NTN	Moderator (vivo)

From Thursday session

Conclusion

Discussion can continue at RAN1#120.

## Rel-18 UE Features

R1-2410663	Session Notes of AI 8.2	Ad-Hoc Chair (NTT DOCOMO, INC.)

Friday decision: The session notes are endorsed and contents reflected below.

[119-R18-UE_features] – Hiroki (NTT DOCOMO)

Email discussion on Rel-18 UE features

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2409392	Rel-18 UE Features	Huawei, HiSilicon

R1-2409579	UE features for Rel-18 work items	Samsung

R1-2410340	Rel-18 UE Features	Ericsson

R1-2410661	Summary of discussion on Rel-18 NR UE features	Moderator (NTT DOCOMO, INC.)

From Tuesday session

NR_MIMO_evo_DL_UL

Agreement:

Introduce following FG for UE STxMP processing capability for codebook.

- FG name

  - UE STxMP processing capability for codebook

- Component

  - 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

  - Note. This FG can be applied for CG+DG also if UE can support those FG.

- Prerequisite

  - 40-6-3a

- Consequence if not supported

  - UE should process multiple TBs within legacy timeline

- Type

  - Per FSPC

- Need of FR1/FR2 differentiation

  - FR2 only

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2,4}

  - For 30kHz SCS: {1,2,4,8}

  - For 60kHz SCS: {2,4,8,16}

  - For 120kHz SCS: {4,8,16,32}

  - For 480kHz SCS: {16,32,64,128}

  - For 960kHz SCS: {32,64,128,256}

  - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH.

- Mandatory or optional

  - Optional with capability signaling

Agreement:

Introduce following FG for UE STxMP processing capability for non-codebook.

- FG name

  - UE STxMP processing capability for non-codebook

- Component

  - 1. Require additional timeline to process multiple TBs for noncodebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG

  - Note. This FG can be applied for CG+DG also if UE can support those FG.

- Prerequisite

  - 40-6-3b

- Consequence if not supported

  - UE should process multiple TBs within legacy timeline

- Type

  - Per FSPC

- Need of FR1/FR2 differentiation

  - FR2 only

- Note

  - candidate values:

  - UE reports candidate value independently for each SCS in unit of symbols

  - For 15kHz SCS: {1,2,4}

  - For 30kHz SCS: {1,2,4,8}

  - For 60kHz SCS: {2,4,8,16}

  - For 120kHz SCS: {4,8,16,32}

  - For 480kHz SCS: {16,32,64,128}

  - For 960kHz SCS: {32,64,128,256}

  - Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH.

- Mandatory or optional

  - Optional with capability signaling

NR_Mob_enh2

Agreement:

Add following note for FG45-1a.

- Note: the SSBs of candidate cell(s) to be measured can be on any band where the UE supports LTM

NR_pos_enh2

Agreement:

Reply to RAN2 as below.

- From RAN1 perspective, to make the additional SL-TDOA/SL-TOA measurement feature work,

  - following changes are recommended. It is up to RAN2 for final decision.

    - SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TOA-MeasElement

    - SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TDOA-MeasElement

  - The signaling of requesting additional measurement is also needed to support above recommendation.

FFS: other reply LS contents

R1-2410662	Summary#2 of discussion on Rel-18 NR UE features	Moderator (NTT DOCOMO, INC.)

From Thursday session

Agreement:

Following is captured in LS to RAN2 on Rel-18 NR UE features list.

---

RAN1 would like to thank RAN2 for the LS in R1-2409344/R2-2409256. Regarding Question 1 and Question 2 in the LS, RAN1 would like to inform RAN2 about the following RAN1 recommendation.

Agreement: Reply to RAN2 as below. - From RAN1 perspective, to make the additional SL-TDOA/SL-TOA measurement feature work, - following changes are recommended. It is up to RAN2 for final decision. - SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TOA-MeasElement - SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..416)) OF SL-TDOA-MeasElement - The signaling of requesting additional measurement is also needed to support above recommendation.

Regarding Question 3, it is RAN1’s understanding based on above agreement that there is no need to perform relative encoding of the additional SL-RSTD/SL-RTOA, SL-PRS RSRP/RSRPP measurements.

---

From Friday session

R1-2410666	Updated RAN1 UE features list for Rel-18 NR after RAN1 #119	Moderators (AT&T, NTT DOCOMO, INC.)

Decision: The  updated list after RAN1#119 in R1-2410666 is agreed.

R1-2410664	Draft LS on updated Rel-18 RAN1 UE features lists for NR after RAN1#119	Moderators (AT&T, NTT DOCOMO, INC.)

Decision: The draft LS to RAN2 (cc: RAN4) on the updated RAN1 UE features list for NR after RAN1#119 is endorsed. Final LS is approved in R1-2410665.

Final summary in R1-2410900.

# Release 19

## Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

Please refer to RP-242399 for detailed scope of the WI. Additional RAN guidance on Rel-18 AI/ML for NR Air Interface can be found in RP-242387.

R1-2410844	Session notes for 9.1 (AI/ML for NR Air Interface)	Ad-Hoc Chair (CMCC)

Friday decision: The session notes are endorsed and contents reflected below.

[119-R19-AI/ML] – Taesang (Qualcomm)

Email discussion on Rel-19 AI/ML

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### Specification support for beam management

R1-2409395	Discussion on AI/ML for beam management	Huawei, HiSilicon

R1-2409447	Discussion on AI/ML for Beam Management	Quectel

R1-2409455	Discussion on AI/ML for beam management	InterDigital, Inc.

R1-2409479	Discussion on AI/ML-based beam management	ZTE Corporation, Sanechips

R1-2409499	Discussion on specification support for beam management	CMCC

R1-2409569	Specification Support for AI/ML for Beam Management	Kyocera

R1-2409581	Discussion for supporting AI/ML based beam management	Samsung

R1-2409625	Discussion on AIML for beam management	Spreadtrum, UNISOC

R1-2409668	Specification support for beam management	vivo

R1-2409741	Specification support for beam management	Intel Corporation

R1-2409749	Specification support for beam management	Tejas Networks Limited

R1-2409780	Specification support for AI-enabled beam management	NVIDIA

R1-2409787	Discussion on AI/ML beam management	Apple

R1-2409840	Discussion on specification support for beam management	Ruijie Networks Co. Ltd

R1-2409855	Discussion on specification support for beam management	NEC

R1-2409877	Discussion on AI/ML for beam management	Xiaomi

R1-2409925	Specification support for AI/ML-based beam management	CATT, CBN

R1-2409957	Discussion on specification support for beam management	Panasonic

R1-2409960	Discussion on specification support for AI/ML beam management	Transsion Holdings

R1-2409985	AI/ML for Beam Management	Nokia

R1-2409994	Discussion on AI/ML for beam management	China Telecom

R1-2410018	AI/ML specification support for beam management	Lenovo

R1-2410029	Discussion on specification support for AI/ML-based beam management	FUTUREWEI

R1-2410048	Discussion on specification support on AI/ML for beam management	Fujitsu

R1-2410101	On specification for AI/ML-based beam management	OPPO

R1-2410149	AI/ML based Beam Management	Google

R1-2410174	Discussion on AI/ML for beam management	HONOR

R1-2410185	FL plan for mobility enhancements in RAN1#119	Moderator (Fujitsu)

R1-2410193	Discussions on AI/ML for beam management	LG Electronics

R1-2410204	Discussion on AIML beam management	TCL	Late submission

R1-2410216	Discussion on specification support for beam management	Sony

R1-2410255	Specification support for beam management	Fraunhofer HHI, Fraunhofer IIS

R1-2410257	Discussion on specification support for beam management	ETRI

R1-2410344	Discussion on AI/ML based beam management	KT Corp.

R1-2410347	AI/ML for Beam Management	Meta

R1-2410354	AI/ML for beam management	Ericsson

R1-2410359	Discussions on specification support for beam management	Sharp

R1-2410367	Discussions on AI/ML for beam management	CAICT

R1-2410373	Discussion on Specification Support of AI/ML for Beam Management	Indian Institute of Tech (M), IIT Kanpur

R1-2410376	Discussion on AI/ML for beam management	NTT DOCOMO, INC.

R1-2410466	Specification support for AI-ML-based beam management	Qualcomm Incorporated

R1-2410504	Specification support for beam management	KDDI Corporation

R1-2410519	Discussion on specification support for AIML-based beam management	MediaTek Inc.

R1-2410547	Specification support for AI/ML beam management	ITL

R1-2410587	On Associated ID for Beam Management Use Case	NTU

R1-2410733	FL summary #0 for AI/ML in beam management	Moderator (Samsung)

From Monday session

Agreement

For UE-sided model, at least for BM-Case 1, the beam information in inference result report is CRI/SSBRI of resource in Set A.

R1-2410734	FL summary #1 for AI/ML in beam management	Moderator (Samsung)

From Tuesday session

Conclusion

For BM-Case 2 of UE-side model, only fixed Set B across different time instance is supported for single CSI report.

Agreement

For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration,

- two CSI-ResourceConfigId s are configured for Set A and Set B separately.

R1-2410735	FL summary #2 for AI/ML in beam management	Moderator (Samsung)

Presented in Wednesday session.

R1-2410736	FL summary #3 for AI/ML in beam management	Moderator (Samsung)

From Thursday session

Agreement

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - The applicability report is based on A) and/or B)

    - It is up to RAN 2 to design the container

    - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied)

      - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

    - B) One set or multiple sets of inference related parameters for applicability report only (not for inference)

      - It is up to RAN2 to design the container.

      - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g.,

        - the associated ID

          - Note: this doesn’t imply the associated ID is mandatory

        - Set A related information

        - Set B related information

        - Report content related information

        - For BM-Case 2,

          - Time instances related information for measurements

          - Time instances related information for prediction

- In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters

  - FFS on whether/what other information along with the applicability is needed

  - If A) is configured in Step 3,

    - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported.

    - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied.

  - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Agreement

At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

- Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

  - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams

  - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report

  - FFS on the timing related issues

  - UE measures the dedicated resource set(s) for monitoring.

R1-2410737	FL summary #4 for AI/ML in beam management	Moderator (Samsung)

From Friday session

Conclusion

For the CSI-ReportConfig for inference configuration provided in Step 5,

- aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete.

- periodic CSI Report is considered as activated after RRCReconfigurationComplete.

- Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig

  - Any specification impact is a separate discussion

Agreement

Send LS to RAN2 with below information.

RAN1 thanks RAN2 for the LS on applicable functionality reporting for beam management UE-sided model.

In RAN1’s discussion of RAN 2 terminologies on beam management,

- The concept/terminology “functionality” of Supported functionalities may refer to UE-capability information/parameters i.e., Rel-19 AI/ML-enabled Features/FGs

- The concept/terminology “ functionality”  of Applicable functionalities may refer to CSI-ReportConfig for inference configuration or a set of inference related parameters

- The Activated functionalities may be enabled based on CSI framework.

Therefore, the meaning and the granularity of “functionality” for Applicable functionalities, Activated functionalities and Supported functionalities may or may not be the same.

RAN 1 made the following agreements related to the Questions from RAN 2:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

RAN1 would like to provide replies on the following questions from RAN2 in R2-2407848:

Q1: In Step 2, what is the granularity of functionality? For example, whether it is a use case (e.g. beam management), whether it is a sub-use case (e.g. beam management Case 1), or others?

Answer to Q1: In Step 2, RAN1 expects that UE reports its UE-capability information/parameters, i.e., Rel-19 AI/ML-enabled Features/FGs (including components and corresponding value ranges). These UE capability information/parameters will depend on how FGs are defined including the granularity, that will be discussed in RAN1 later in the WI.

Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID?

Answer to Q2: RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Q3: Is NW-side additional condition functionality specific?

Answer to Q3: Please also refer to the answer to Q2 to understand the ongoing discussion about the associated ID for NW-side additional condition. And please refer to the agreements related to the Questions from RAN 2.

Q4: RAN2 wonders what information is needed in Step 3 for UE to decide whether a functionality is applicable before Step 4. More specifically, RAN2 would like to ask the following questions (Q4-1 to Q4-5):

Answer to Q4: And please refer to the agreements related to the Questions from RAN 2.

Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

Answer to Q4-1: There is no consensus yet on whether it is mandatory or optional. There is no conclusion yet on whether it is feasible or not for UE to decide the applicability without NW-side additional condition, and RAN 1 is discussing the related issues.

Q4-2: In RAN2, it is FFS whether configuration (e.g. inference configuration) other than NW-side additional condition can be included in Step 3. RAN2 would like to understand whether it is feasible and required for gNB to provide configuration (e.g. inference configuration) other than NW-side additional condition in Step 3 for UE to determine applicable functionalities?

Answer to Q4-2: Please refer to the agreements related to the Questions from RAN 2.

Q4-3: For UE evaluating applicable functionality reporting, if the answer to Q4-2 is Yes, what is the relationship between NW-side additional condition and configuration (e.g. inference configuration)? For example, is NW-side additional condition part of inference configuration, or is inference configuration part of NW-side additional condition, or is NW-side additional condition separate from inference configuration, etc?

Answer to Q4-3: Please refer to the agreements related to the Questions from RAN 2.

Q4-4: If the answer to Q4-2 is Yes, what is the content of configuration (e.g. inference configuration) for UE to determine applicable functionalities?

Answer to Q4-4: Please refer to the agreements related to the Questions from RAN 2.

Q5: What is the content of applicable functionality reporting in Step 4?

Answer to Q5: Please refer to the agreements related to the Questions from RAN 2.

Q6: What is the content of inference configuration in Step 5?

Answer to Q6: Please refer to the agreements related to the Questions from RAN 2. The content of inference configuration as CSI-ReportConfig is to be designed later in RAN1.

Q7: If inference configuration is provided in Step 3, does it activate the functionality immediately upon receiving Step 3?

Answer to Q7: Please refer to the agreements related to the Questions from RAN 2.

Q8: If inference configuration is not provided in Step 3, does configuration in Step 5 activate the functionality immediately upon receiving Step 5?

Answer to Q8: Please refer to the agreements/conclusion related to the Questions from RAN 2.

Q9: If more than one functionality are configured in Step 3 or Step 5, whether multiple/all applicable functionalities can be activated?

Answer to Q9: Please refer to the agreements related to the Questions from RAN 2.

Q10: Is L1/L2 signaling for functionality activation/deactivation needed?

Answer to Q10:  Please refer to the agreements related to the Questions from RAN 2. With that, RAN1 understands that L1 and MAC signalling can be used for aperiodic CSI Report and semi-persistent CSI report.

R1-2410893	[DRAFT] Reply LS on applicable functionality reporting for beam management UE-sided model	Samsung

Decision: The draft LS is endorsed. Final reply LS is approved in R1-2410898.

Final summary in R1-2410892.

### Specification support for positioning accuracy enhancement

R1-2409396	Discussion on  AI/ML for positioning accuracy enhancement	Huawei, HiSilicon

R1-2409443	AI/ML for Positioning Accuracy Enhancement	Ericsson

R1-2409480	Discussion on AI/ML-based positioning enhancement	ZTE Corporation, Pengcheng Laboratory

R1-2409500	Discussion on specification support for positioning accuracy enhancement	CMCC

R1-2409543	Discussion on AI/ML for positioning accuracy enhancement	New H3C Technologies Co., Ltd.

R1-2409582	Discussion for supporting AI/ML based positioning accuracy enhancement	Samsung

R1-2409669	Specification support for positioning accuracy enhancement	vivo

R1-2409742	Specification support for positioning accuracy enhancement	Intel Corporation

R1-2409750	Specification support for positioning accuracy enhancement	Tejas Networks Limited

R1-2409781	Specification support for AI-enabled positioning	NVIDIA

R1-2409788	Discussion on Specification Support for AI/ML-based positioning	Apple

R1-2409841	Discussion on specification support for positioning accuracy enhancement	Ruijie Networks Co. Ltd

R1-2409845	Discussion on support for AIML positioning	InterDigital, Inc.

R1-2409852	Discussion on specification support for AIML based positioning accuracy enhancement	NEC

R1-2409878	Discussion on AI/ML-based positioning accuracy enhancement	Xiaomi

R1-2409926	Specification support for AI/ML-based positioning	CATT, CICTCI

R1-2409986	AI/ML for Positioning Accuracy Enhancement	Nokia

R1-2410019	Specification impacts for AI/ML positioning	Lenovo

R1-2410049	Discussion on specification support for AIML-based positioning accuracy enhancement	Fujitsu

R1-2410102	On specification for AI/ML-based positioning accuracy enhancements	OPPO

R1-2410150	AI/ML based Positioning	Google

R1-2410205	AI/ML positioning accuracy enhancement	Fraunhofer IIS, Fraunhofer HHI

R1-2410215	Discussion on specification support for positioning accuracy enhancement	TCL

R1-2410217	Support for AI/ML for positioning accuracy enhancement	Sony

R1-2410258	Discussion on specification support for positioning accuracy enhancement	ETRI

R1-2410360	Discussion on specification support for AI/ML based positioning accuracy enhancements	Sharp

R1-2410377	Discussion on AI/ML for positioning accuracy enhancement	NTT DOCOMO, INC.

R1-2410414	Discussion on specification support for AI-ML based positioning accuracy enhancement	Baicells

R1-2410424	Specification Support of AI/ML for Positioning Accuracy Enhancement	Indian Institute of Tech (M), IIT Kanpur

R1-2410467	Specification support for AI-ML-based positioning accuracy enhancement	Qualcomm Incorporated

R1-2410531	Design for AI/ML based positioning	MediaTek Korea Inc.

R1-2410571	Discussion on specification support for AI/ML Positioning Accuracy enhancement	CEWiT

R1-2410588	Discussions on specification support for positioning accuracy enhancement for AI/ML	ITL

R1-2410714	Summary #1 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

From Monday session

Conclusion

For measurement report of AI/ML assisted positioning Case 3a, regarding the report of LOS/NLOS indicator,

- LOS/NLOS indicator can’t be reported independently from other measurements

R1-2410715	Summary #2 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

From Tuesday session

Agreement

For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows.

- starting time = first detected path rounded down with timing granularity T.

Note: UE-side measurement is a separate discussion.

R1-2410716	Summary #3 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

From Wednesday session

Agreement

For model performance monitoring of AI/ML positioning Case 1, support at least:

- Option A. The target UE side performs monitoring metric calculation.

  - The target UE may signal the monitoring outcome to the LMF.

  - FFS: content of monitoring outcome

- FFS: Option B

R1-2410717	Summary #4 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

Presented in Thursday session.

R1-2410718	Summary #5 of specification support for positioning accuracy enhancement	Moderator (Ericsson)

From Friday session

Agreement

For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement,

- The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T.

- The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR.

  - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values.

- The timing information is defined relative to a reference time, same as the path-based measurement.

- The Nt’ selected time domain channel measurement values are expected to be those with the highest power.

- The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T.

- LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values:

  - Nt'<=24. FFS: Nt' values.

  - Nt = {32, 64, 128}

  - FFS: k

  - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement

- FFS: whether transmit offset from gNB to LMF

Note: measurement by UE is a separate discussion.

Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

Agreement

For AI/ML based positioning Case 1, all assistance information from legacy UE-based DL-TDOA, other than info #7, can be provided from LMF to UE. For info #7, RAN1 study, if necessary, choose one alternative from the following:

- Alternative 1. Info #7 is provided implicitly via associated ID.

  - Associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 2. Info #7 can be provided either implicitly or explicitly by LMF. Note: no UE capability is introduced on whether info #7 is provided implicitly or explicitly, and the UE can request info #7 to be provided explicitly or implicitly.

  - If provided implicitly, associated ID is signaled by LMF to indicate whether info #7 is consistent between training and inference.

- Alternative 3.  Info #7 is not be provided from LMF to UE.

  - If info #7 is not provided, UE may assume info #7 is consistent between training and inference.

- Alternative 4. Info #7 is provided explicitly from LMF to UE.

| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |
| --- | --- |

Final summary in R1-2410921.

### Specification support for CSI prediction

Discussions on RAN1#118bis and RAN1#119 will be limited to the study on consistency of training/inference.

R1-2410654	Discussion on AI/ML for  CSI prediction	Huawei, HiSilicon	(rev of R1-2409397)

R1-2409449	AI/ML for CSI prediction	Ericsson

R1-2409481	Discussion on specification support for AI CSI prediction	ZTE Corporation, Sanechips

R1-2409501	Discussion on AI/ML for CSI prediction	CMCC

R1-2409583	Views on AI/ML based CSI prediction	Samsung

R1-2409626	Discussion on AIML for CSI prediction	Spreadtrum, UNISOC

R1-2410673	Study on consistency issue for CSI prediction	vivo	(rev of R1-2409670)

R1-2409751	Discussion on study for AI/ML CSI prediction	Tejas Networks Limited

R1-2409782	Specification support for AI-enabled CSI prediction	NVIDIA

R1-2409789	Discussion on AI/ML-based CSI prediciton	Apple

R1-2409853	Discussion on specification support for CSI prediction	NEC

R1-2409879	Discussion on AI/ML model based CSI prediction	Xiaomi

R1-2409927	Specification support for AI/ML-based CSI prediction	CATT

R1-2409987	AI/ML for CSI Prediction	Nokia

R1-2409995	Discussion on AI/ML for CSI prediction	China Telecom

R1-2410020	On AI/ML for CSI prediction	Lenovo

R1-2410042	On AI/ML-based CSI prediction	InterDigital, Inc.

R1-2410050	Discussion on specification support for CSI prediction	Fujitsu

R1-2410103	On specification for AI/ML-based CSI prediction	OPPO

R1-2410151	AI/ML based CSI Prediction	Google

R1-2410194	Discussions on CSI prediction	LG Electronics

R1-2410218	Further views on consistency issues in CSI prediction	Sony

R1-2410248	Discussion on consistency of training / inference for AI/ML-based CSI prediction	Panasonic

R1-2410259	Discussion on specification support for CSI prediction	ETRI

R1-2410336	Discussion on AI/ML for CSI prediction	AT&T

R1-2410378	Discussion on AI/ML for CSI prediction	NTT DOCOMO, INC.

R1-2410468	Specification support for CSI prediction	Qualcomm Incorporated

R1-2410537	AI/ML - Specification support for CSI Prediction	MediaTek Inc.

R1-2410564	AI/ML for CSI prediction	Mavenir

R1-2410817	Summary #1 of CSI prediction	Moderator (LG Electronics)

Presented in Wednesday session.

R1-2410818	Summary #2 of CSI prediction	Moderator (LG Electronics)

Presented in Thursday session.

Final summary in R1-2410899.

### Additional study on AI/ML for NR air interface

#### CSI compression

R1-2409398	Discussion on AI/ML for CSI compression	Huawei, HiSilicon

R1-2409450	AI/ML for CSI compression	Ericsson

R1-2409482	Discussion on study for AI/ML CSI compression	ZTE Corporation, Sanechips

R1-2409502	Discussion on AI/ML for CSI compression	CMCC

R1-2409584	Views on additional study for AI/ML based CSI compression	Samsung

R1-2409627	Discussion on AIML for CSI compression	Spreadtrum, UNISOC

R1-2409671	Discussion on CSI compression	vivo

R1-2409743	AI/ML for CSI compression	Intel Corporation

R1-2409752	Discussion on AI/ML for CSI Compression	Tejas Networks Limited

R1-2409783	Additional study on AI-enabled CSI compression	NVIDIA

R1-2409790	Discussion on AI based CSI compression	Apple

R1-2409854	Discussion on CSI compression	NEC

R1-2409880	Views on AI/ML model based CSI compression	Xiaomi

R1-2409928	Additional study on AI/ML-based CSI compression	CATT

R1-2409988	AI/ML for CSI Compression	Nokia

R1-2410021	On AI/ML for CSI compression	Lenovo

R1-2410030	Discussion of CSI compression on AI/ML for NR air interface	FUTUREWEI

R1-2410043	On AI/ML-based CSI compression	InterDigital, Inc.

R1-2410051	Discussion on CSI compression with AI/ML	Fujitsu

R1-2410104	Additional study on AI/ML-based CSI compression	OPPO

R1-2410152	AI/ML based CSI Compression	Google

R1-2410195	Study on CSI compression	LG Electronics

R1-2410201	Discussion on AI/ML for CSI compression	KAIST

R1-2410202	Discussion on AIML CSI compression	TCL	Late submission

R1-2410249	Discussion on AI/ML for CSI compression	Panasonic

R1-2410260	Discussion on AI/ML for CSI compression	ETRI

R1-2410379	Discussion on AI/ML for CSI compression	NTT DOCOMO, INC.

R1-2410425	Discussion and evaluation results on AI/ML for CSI Compression	Indian Institute of Tech (M), IIT Kanpur

R1-2410469	Additional study on CSI compression	Qualcomm Incorporated

R1-2410508	Additional study on AI/ML for NR air interface - CSI compression	MediaTek Inc.

R1-2410572	Discussion on AI/ML for CSI compression	CEWiT

R1-2410589	Discussion an AI/ML based CSI Compression	IIT Kanpur, Indian Institute of Tech (M)

R1-2410719	Summary#1 of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

From Tuesday session

Conclusion

- For Direction A 4-1 and 3a-1, send LS to RAN2 for the study of feasibility of parameter / dataset exchange

Wednesday further clarification regarding above LS, additionally CC SA2, SA3, SA5.

Note: Samsung denies CC.

R1-2410720	Summary#2 of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

From Tuesday session

Agreement

For Direction A Option 3a-1 and Direction C, study the feasibility of scalable model structure specification over numbers of Tx ports, CSI feedback payload sizes, and bandwidths, number of slots.

- Feasibility of Direction A Option 3a-1 is contingent on the feasibility of the scalable model structure specification.

- Feasibility of Direction C is contingent on the feasibility of the scalable model structure specification and model parameters specification.

- Note: Angular/Delay/Doppler domain conversion and quantization are considered to be part of feasibility study

Agreement

For studying the standardized model structure,

- For temporal domain Case 0,

  - In case of spatial-frequency domain input, adopt Transformer as the backbone structure.

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 2,

  - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation with additional layers

      - Conv-LSTM

      - LSTM

    - Option 2: Latent adaptation on top of Case 0 model structure

      - Reusing Case 0 structure and changing quantization operation and/or feedback payload over different slots

    - Note: other options are not precluded

  - Further study angular-frequency and angular-delay domain input

- For temporal domain Case 3,

  - In case of spatial-frequency domain input, take into account the model structure of Case 0 with an adaptation. Study the following options for the adaptation:

    - Option 1: Input/output adaptation

    - Option 2: Latent adaptation

  - Further study angular-frequency, angular-delay, and angular-delay-Doppler domain input.

- Take precoding matrix in the spatial-frequency domain in a subband granularity as the input, and the reconstructed precoding matrix in the spatial-frequency as the output, for the study of standardized model structure.

  - Note: Processing steps at the UE to derive angular, delay, and/or Doppler domain basis, and corresponding reconstruction steps at the gNB are considered.

  - Note: scalar/vector quantization and dequantization are considered. Quantization-aware training with jointly updated quantization method/parameters (Case 2-2) is used.

R1-2410721	Summary#3 of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

Presented in Wednesday session.

R1-2410722	Summary#4 of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

From Thursday session

Agreement

For NW to collect data for training, study following spec impacts

- Data format: codebook-based Rel-16 eType2 or Rel-18 eType2 for PMI prediction.

  - FFS if enhancement is needed

  - FFS number of samples in the report.

  - FFS whether channel or precoder is needed for temporal Cases 3

- Configuration of rank/layer, number of subbands

- Mechanism for ground-truth reporting

- FFS: Report additional information regarding the samples, e.g., data quality, FFS the definition of data quality and corresponding parameters.

- FFS if enhancements in CSI-RS and SRS configuration is needed.

- FFS: Report associated information that captures UE side additional condition

- FFS: Configuration / reporting of temporal aspects for temporal Case 2 and Case 3, e.g., association between input and output CSI

- FFS: details of CSI measurement

For UE to collect data for training, study following spec impacts

- NW configuration or UE request, e.g., RS configuration/transmission for data collection

- Whether enhancements in CSI-RS configuration is needed.

- Configuration of temporal aspects for temporal case 2/3, e.g., association between input and output CSI

- FFS: Need of configuration of ID, and configuration of ID.

R1-2410723	Summary#5 of Additional study on AI/ML for NR air interface: CSI compression	Moderator (Qualcomm)

From Friday session

R1-2410725	Updated summary of Evaluation Results for AI/ML CSI compression	Moderator (Qualcomm)

Agreement

To capture all observations in section 2 of R1-2410725 to TR38.843.

R1-2410915	[Draft] LS on signalling feasibility of dataset and parameter sharing	Qualcomm

Agreement

Draft LS R1-2410915 with the following updates:

- Revising “The size becomes 600K * (2000 bits) / (8bits/Byte) = 150 MB+ 11.6 MB = 161.6MB if we assume N1=N2.” To “The size becomes N2 * (2000 bits) / (8bits/Byte) + 11.6 MB” , and

- Revising

Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

to

Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

- Deleting

“RAN1 respectfully requests RAN2’s feedback on the feasibility of standardized signaling, considering the size of dataset / encoder parameters provided above.”

- Deleting

“The size is based on Case 0 of two-sided CSI compression; the size may or may not be larger for Case 2 and Case 3.” from “For Option 4-1: sharing {target CSI, CSI feedback} dataset:”

Final LS is approved in

R1-2410922	LS on signalling feasibility of dataset and parameter sharing	RAN1, Qualcomm

Agreement

For temporal domain aspects Case 3, study LCM aspects and specification impacts,

consider the following options for training data collection

- Option 1: The target CSI for training is derived based on the predicted CSI of the future slot(s).

- Option 2: The target CSI for training is derived based on the measured CSI of the future slot(s).

- Note: During inference, the input to the CSI generation part is derived based on the predicted CSI.

consider following options for the monitoring labels

- Option 1: The monitoring label is derived based on the predicted CSI of the future slot(s).

  - CSI prediction output is used as input to CSI generation part.

  - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately.

- Option 2: The monitoring label is derived based on the measured CSI of the future slot(s)

  - Option 2a: CSI prediction output is used as input to CSI generation part.

    - Note: This corresponds to end-to-end monitoring of CSI prediction and compression.

  - Option 2b: Measured CSI of the future slot(s) is used as input to CSI generation part for monitoring purpose.

    - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately.

Study how the functionality/model control (activation, deactivation, switching, and fallback) for CSI prediction and CSI compression interacts.

Final summary in R1-2410724.

#### Other aspects of AI/ML model and data

Including model identification/procedure for two-sided model, collection of UE-sided model training data, and model transfer/delivery

R1-2409399	Discussion on other aspects of the additional study for AI/ML	Huawei, HiSilicon

R1-2409483	Discussion on other aspects of AI/ML model and data	ZTE Corporation, Sanechips

R1-2409503	Discussion on other aspects of AI/ML model and data	CMCC

R1-2409585	Views on additional study for other aspects of AI/ML model and data	Samsung

R1-2409628	Discussion on other aspects of AI/ML model and data	Spreadtrum, UNISOC

R1-2409672	Other aspects of AI/ML model and data	vivo

R1-2409731	Discussion on other aspects of AI/ML	Ericsson

R1-2409744	Other aspects of AI/ML model and data	Intel Corporation

R1-2409753	Discussion on Other aspects of  AI/ML model and data	Tejas Networks Limited

R1-2409784	Additional study on other aspects of AI model and data	NVIDIA

R1-2409791	Discussion on other aspects of AI/ML model and data	Apple

R1-2409856	Discussion on other aspects of AI/ML model and data	NEC

R1-2409881	Further study on AI/ML model and data	Xiaomi

R1-2409929	Additional study on AI/ML for other aspects	CATT, CICTCI

R1-2409989	Other aspects of AI/ML for two-sided model	Nokia

R1-2409996	Discussion on other aspects of AI ML model and data	China Telecom

R1-2410022	Discussion on other aspects of AI/ML model and data	Lenovo

R1-2410031	Discussion on other aspects of AI/ML model and data on AI/ML for NR air-interface	FUTUREWEI

R1-2410044	On other aspects of AI/ML model and data	InterDigital, Inc.

R1-2410052	Discussion on other aspects of AI/ML model and data	Fujitsu

R1-2410105	Additional study on other aspects of AI/ML model and data	OPPO

R1-2410153	AI/ML Model and Data	Google

R1-2410173	Discussion on other aspects for AI/ML for air interface	Panasonic

R1-2410192	Other aspects of AI/ML model and data	Continental Automotive

R1-2410196	Discussion on other aspects of AI/ML model and data	LG Electronics

R1-2410203	Discussions on other aspects of AlML In NR air interface	TCL	Late submission

R1-2410261	Discussion on other aspects of AI/ML model and data	ETRI

R1-2410335	Other Aspects of AI/ML framework	AT&T

R1-2410380	Discussion on other aspects of AI/ML model and data	NTT DOCOMO, INC.

R1-2410426	Discussion on other aspects of AI/ML model and data	Sharp

R1-2410470	Other aspects of AI/ML model and data	Qualcomm Incorporated

R1-2410775	Summary #1 for other aspects of AI/ML model and data	Moderator (OPPO)

From Wednesday session

Agreement

For study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model,

- ID-X can be used for pairing the UE-part and the NW-part of a two-sided model

- FFS: other information needed for pairing

Agreement

Regarding the relationship of model ID, first indication, and second indication for model transfer/delivery Case z4, further study the following options:

- Opt.1: model ID consists of the information of the first indication and the second indication

  - E.g., model ID is a combination of the first and second indications

- Opt.2: The second indication is assumed as the model ID

- Opt.3: Model ID is assigned by network and is separated from the first indication and the second indication

Conclusion

Regarding MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, from RAN1 perspective, how to construct the dataset, including whether a dataset constructed from one cell or from multiple cells is up to network implementation.

R1-2410776	Summary #2 for other aspects of AI/ML model and data	Moderator (OPPO)

From Thursday session

Conclusion

For the study of model delivery/transfer Case z4, if the model delivery/transfer is directly used for inference, the following options are identified as the candidate solutions to determine the readiness of AI model with the transferred parameters for inference (either or combination of the following options)

- Option 1: UE sends signaling to network to notify that the AI model with the transferred parameters is ready for activation

- Option 2: The AI model with the transferred parameters can be assumed ready for activation from a minimum applicable time after the completion of model delivery/transfer

R1-2410777	Summary #3 for other aspects of AI/ML model and data	Moderator (OPPO)

Final summary in R1-2410778.

## NR MIMO Phase 5

Please refer to RP-242394 for detailed scope of the WI.

[119-R19-MIMO] – Eko (Samsung)

Email discussion on Rel-19 MIMO Phase 5

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2409592	List of RRC and MAC CE impact for Rel-19 MIMO Ph5	Samsung (Moderator)

From Thursday session

R1-2410757	DRAFT LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5	Moderator (Samsung)

Decision: The LS to RAN2 on RRC and MAC CE impacts is endorsed. Final LS is approved in R1-2410758.

### Enhancements for UE-initiated/event-driven beam management

R1-2409370	Enhancements for UE-initiated/event-driven beam management	MediaTek Inc.

R1-2409377	Discussion on enhancements for UE-initiated/event-driven beam management	ZTE Corporation, Sanechips

R1-2409427	On UE initiated/event-driven beam management	Huawei, HiSilicon

R1-2409459	Further Details on Rel-19 UE/Event-Driven Beam Management	InterDigital, Inc.

R1-2409504	Discussion on  enhancements for UE-initiated/event-driven beam management	CMCC

R1-2409563	Remaining issues for UE-initiated beam management	Ericsson

R1-2409586	Views on Rel-19 UE-initiated/event-driven beam management	Samsung

R1-2409629	Discussion on UE-initiated/event-driven beam management	Spreadtrum, UNISOC

R1-2409673	Remaining issues on UE-initiated/event-driven beam management	vivo

R1-2409710	Discussion on UE-initiated/event-driven beam management	TCL

R1-2409748	Enhancements for Event-driven Beam Management	Intel Corporation

R1-2409792	Enhancements for UE-initiated beam management	Apple

R1-2409842	Discussion on enhancements for UE-initiated/event-driven beam management	Ruijie Networks Co. Ltd

R1-2409857	Discussion on enhancements for UE-initiated or event-driven beam management	NEC

R1-2409888	Discussion on enhancements for UE-initiated/event-driven beam management	Xiaomi

R1-2409933	Views on Rel-19 UE-initiated/event-driven beam management	CATT

R1-2409961	Enhancements for UE-initiated/event-driven beam management	Transsion Holdings

R1-2409969	Enhancements for UE-initiated/event-driven beam management	Lenovo

R1-2410035	Enhancements for UE-initiated/event-driven beam management	FUTUREWEI

R1-2410053	Discussion on enhancements for UE-initiated/event-driven beam management	Fujitsu

R1-2410108	Discussions on UE-initiated/event-driven beam management	OPPO

R1-2410116	Offline summary for Rel-19 MIMO UEIBM (9.2.1) pre-RAN1#119	Moderator (ZTE)

R1-2410164	Discussion on enhancements for UE-initiated or event-driven beam management	Google

R1-2410171	Discussion on enhancements for UE-initiated/event-driven beam management	Hyundai Motor Company

R1-2410175	Discussion on the UE-initiated/event-driven beam management	HONOR

R1-2410197	UE-initiated beam management	LG Electronics

R1-2410219	Enhancements for UE-initiated/event-driven beam management	Sony

R1-2410262	Enhancements for UE-initiated/event-driven beam management	ETRI

R1-2410315	Enhancements to facilitate UE-initiated/event-driven beam management	Nokia

R1-2410346	Enhancements for UE Initiated Beam Management	Meta

R1-2410381	Discussion on enhancements for UE-initiated/event-driven beam management	NTT DOCOMO, INC.

R1-2410427	Discussion on UE-initiated/event-driven beam management	ITRI

R1-2410435	Enhancements for UE-initiated/event-driven beam management	Sharp

R1-2410471	UE-initiated/event-driven beam management	Qualcomm Incorporated

R1-2410533	Enhancements for UE-initiated/Event-Driven Beam Management 	Panasonic

R1-2410538	Discussion on UE initiated beam report	ASUSTeK

R1-2410540	Discussions on enhancements for UE-initiated/event-driven beam management	KDDI Corporation

R1-2410573	Enhancements for UEiBM	CEWiT

R1-2410585	Discussion on enhancements for UE-initiated/event-driven beam management	NICT

From Monday session

R1-2410117	Moderator Summary #1 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH does NOT carry the beam report, for the second UL channel in Mode-B, Option-2 is supported:

- Option-2: Type-1 CG PUSCH can NOT be transmitted

Agreement

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the following is supported

- the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from the same PUCCH group.

- (working assumption) the first PUCCH and the second PUSCH associated for UE-initiated/event-driven beam reporting are from different PUCCH groups

  - Subject to separate UE capability

Conclusion

On cross-CC beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Event-2, the case that the first PUCCH and the second PUSCH are from the different CG is NOT supported.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, down-select one of Alt-1 and Alt-2 for both mode-A and mode-B.

- Alt-1 (dedicated SR): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR/reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

  - FFS: whether/how to enhance legacy SR to support notification of beam report transmission in Mode-B

- Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

- Reuse multiplexing/dropping rule(s) of SR as baseline

  - FFS: overlapping with SR/LRR or PUSCH

Note: Further details on first PUCCH retransmission for mode A and mode B will be separately discussed.

R1-2410118	Moderator Summary #2 on UE-initiated/event-driven beam management	Moderator (ZTE)

From Tuesday session

Agreement

On UE-initiated/event-driven beam reporting, for Event 7, the scheme-1 and scheme-2 for deriving ‘RS for current beam’ on Event-2 is reused with the following further interpretation:

- Scheme-1: ‘RS for current beam’ is the QCL RS in the activated TCI state with the Q-th best quality.

- Scheme-2: ‘RS for current beam’ is the SSB which is QCLed with the QCL RS in the activated TCI state with the Q-th best quality.

- Basic feature of the triggering event determination for Event-7: Once quality of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality, UE initiated beam report occurs

Note: For Event-2, we have the following definition for scheme-1 and scheme-2

- Scheme-1: RS for current beam is the QCL RS in the indicated TCI state

- Scheme-2: RS for current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state.

Agreement

On UE-initiated/event-driven beam reporting, for Event 1, down-select at least one among the following options for RS measurement

- Option-1: RS resource set for new beam is NOT configured in the CSI reporting configuration, and an explicit RRC selection for scheme-1 and scheme-2 is introduced.

- Option-2: RS resource set for new beam is configured in the CSI reporting configuration, and the following implicit manner for enabling one of either scheme-1 or scheme-2 is used:

  - If the RS(s) for new beam are CSI-RS configured in a CSI-RS resource set configured with repetition, Scheme-1 is enabled; otherwise, Scheme-2 is enabled.

R1-2410119	Moderator Summary #3 on UE-initiated/event-driven beam management	Moderator (ZTE)

From Wednesday session

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type

- Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration.

  - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId.

  - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

- Reuse multiplexing/dropping rule(s) of SR as baseline

  - FFS: overlapping with SR/LRR or PUSCH

Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

Agreement

Confirm the following working assumption in RAN1#117:

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

Agreement

On UE-initiated/event-driven beam reporting, at least one of the following is supported as an extension on L1-RSRP report format depending on Event-2.

- Option-1: For each of reported CRI/SSBRI, to introduce additional indication of whether the CRI/SSBRI satisfies the condition of Event-2.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - The presence of this field is only for the case that N > 1 and the time window and M are configured

- Option-2: For each of reported CRI/SSBRI, to introduce additional indication of the number of Event-2 instances for the CRI/SSBRI(s) within a time window.

  - The presence of this field is enabled by RRC with subjective to UE capability.

  - The presence of this field is only for the case that N > 1 and the time window and M are configured.

- Option-3: No further enhancement.

Note: As agreed in RAN1#117, at least one of the reported CRI/SSBRI(s) should satisfy the condition of Event-2

FFS: Whether/how to handle the case if UE has not sent any first PUCCH associated for UE-initiated/event-driven beam reporting, for Mode-A.

R1-2410872	Moderator Summary #4 on UE-initiated/event-driven beam management	Moderator (ZTE)

From Thursday session

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A, down-select one of the following options in RAN1#120

- Option-1: A CSI trigger state is dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration.

- Option-2: A CSI trigger state can be associated with

  - only UE-initiated/event-driven beam reporting

  - or only legacy AP-CSI configuration

  - or UE-initiated/event-driven beam reporting and legacy AP-CSI configuration

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, further study at least the following cases:

- Case-1: The 1-bit first PUCCH is collided/overlapped with a PUCCH carrying normal SR and/or a PUCCH with normal LRR

- Case-2: The 1-bit first PUCCH is collided/overlapped with a PUSCH

- Case-3: The 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain.

Agreement

Study the following to reduce beam application latency after a UEI/ED beam report is sent

- Alt-1: After confirmation/acknowledgement from NW, apply new beam without RRC configuration signaling or MAC-CE signaling

  - after sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report

  - update TCI state(s) with the reported new beam(s)

  - activate new beam(s) without additional SSB reception

- Alt-2: After receiving a TCI state activation command to activate a TCI state(s), if the new beam(s) associated with the TCI state(s) is reported as synchronized in the UEI/ED beam report, the TCI state(s) becomes applicable for DL reception without additional SSB reception.

  - Note: A reported new beam is determined as synchronized by UE, if the UE stores the QCL properties associated with the reported new beam(s) after the UEI/ED beam report is sent

  - FFS: How to inform a reported new beam in a UEI/ED beam report (i.e., introducing one-bit indicator for each reported new beam or all the reported new beam are assumed to be synchronized)

- Alt-3: After sending a UE-initiated beam report, the UE could store the QCL properties of the SSB associated with the reference signal reported in the beam report.

  - In such case, at the reception of a subsequent reception of Unified TCI States Activation/Deactivation MAC CE, the UE activates new beam(s) without additional SSB reception.

- Other alternatives are not precluded.

Agreement

On UE-initiated/event-driven beam reporting, regarding trigger events, the following working assumption in RAN1#118bis is confirmed:

On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported.

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality.

  - Q is RRC configured with subjective to UE capability signalling

    - UE may only indicate a single candidate value or not support Event-7.

- The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise

- The additionally supported events will be lower priority compared to event 2.

Agreement

Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC).

- Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s).

  - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s).

- Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s)

  - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS.

  - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS.

  - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s).

  - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}.

  - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s).

Note: There is the same periodicity for the new beam RS(s).

FFS: Down-selection among above Alt(s) for cross-CC beam reporting.

Strong concerns were expressed by Huawei and CATT that if only Alt-1 is supported in the end, the feature will not be practical.

Send an LS to RAN4.

From Friday session

R1-2410913	[DRAFT] LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management	Moderator (ZTE)

Decision: Draft LS is endorsed and final version is approved in R1-2410914.

### CSI enhancements

Including support for up to 128 CSI-RS ports on FR1 and UE reporting enhancement for CJT and SRS port grouping for TDD low-complexity 6/8RX receiver.

R1-2409371	CSI enhancements	MediaTek Inc.

R1-2409378	Discussion on CSI enhancements	ZTE Corporation, Sanechips

R1-2409428	On 128 CSI-RS ports and UE reporting enhancement	Huawei, HiSilicon

R1-2409432	CSI enhancements for Rel. 19 MIMO	Fraunhofer IIS, Fraunhofer HHI

R1-2409460	Further Details  on Rel-19 Enhancements of CSI	InterDigital, Inc.

R1-2409505	Discussion on CSI enhancements	CMCC

R1-2409589	Views on Rel-19 CSI enhancements	Samsung

R1-2409630	Discussion on CSI enhancements	Spreadtrum, UNISOC

R1-2409674	Remaining issues on Rel-19 CSI enhancements	vivo

R1-2409747	CSI enhancements for MIMO	Intel Corporation

R1-2409761	CSI enhancements 	Tejas Networks Limited

R1-2409793	Views on R19 MIMO CSI enhancement	Apple

R1-2409851	Discussion on CSI enhancements	NEC

R1-2409889	Further discussion on Rel-19 MIMO CSI enhancements	Xiaomi

R1-2410657	Views on NR MIMO CSI enhancements Phase 5	CATT	(rev of R1-2409934)

R1-2409970	Discussion on CSI enhancements	Lenovo

R1-2410040	CSI enhancements	TCL

R1-2410054	Discussion on Rel-19 CSI enhancements	Fujitsu

R1-2410109	CSI enhancements for Rel-19 MIMO	OPPO

R1-2410154	CSI Enhancement for NR MIMO	Google

R1-2410176	Discussion on CSI enhancements	HONOR

R1-2410220	Further views on CSI enhancements	Sony

R1-2410303	Discussion on Open Issues of CSI Enhancement	Rakuten Mobile, Inc

R1-2410667	CSI enhancement for NR MIMO Phase 5	Nokia	(rev of R1-2410316)

R1-2410353	Remaining issues on CSI enhancements for large antenna arrays and CJT	Ericsson

R1-2410382	Discussion on CSI enhancements	NTT DOCOMO, INC., NTT CORPORATION

R1-2410436	CSI enhancements	Sharp

R1-2410472	CSI enhancements for >32 ports and UE-assisted CJT	Qualcomm Incorporated

R1-2410549	Discussion on CSI enhancements for NR MIMO Phase 5	KDDI Corporation

R1-2410586	Discussion on CSI enhancements	NICT

R1-2409587	Moderator summary for OFFLINE discussion on Rel-19 CSI enhancements	Samsung (Moderator)

R1-2409588	Moderator summary on Rel-19 CSI enhancements	Samsung (Moderator)

From Monday session

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, the 1-bit indicator per CSI trigger state applies to all the NTRP configured CSI-RS resources.

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, the timeline (Z/Z’) is determined as Z/Z’ associated with the Rel-18 eType-II CJT, plus Drelax

- The value of Drelax is a UE capability, taken from {0, drelax}

  - FFS: The value of drelax (>0), including whether it depends on SCS

- For linking CJTC Dd and Rel-18 eType-II CJT CSI, joint triggering is a separate UE feature group from separate triggering

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, if the UCI associated with each of the M CRIs comprises two parts and is multiplexed on PUCCH, the UE determines the PUCCH resource, the number of PRBs for the PUCCH resource, assuming that the CSI associated with each of the M reported CRIs corresponds to rank 1.

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured, regarding linking the CMRs in the two CSI Report Settings, for separate triggering, the UE expects that the number of configured CSI-RS resources associated with the Rel-18 eType-II CJT CSI is always the same as the number of TRS resource sets associated with the Rel-19 CJTC Dd report.

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI reported in CSI part-1, if resource-specific RI restriction is configured, zero padding bits are introduced in CSI part 1 for each of the (M-MR) reported CRIs:

- For a k-th CRI from the (M-MR) reported CRIs (where k=1, 2, …, (M-MR)), OP,k zero padding bits are inserted where:

  - OP,k=Nmax-Nreported,k

    - Nmax=maxi∈S(Ni), where Ni is the size of RI field corresponding to the i-th CRI, and S is the set of CRIs corresponding to the (Ks-MR) resources

    - Nreported,k is the size of RI field corresponding to the k-th CRI

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with two separate triggers, to indicate whether or not the UE should perform delay offset (DO) compensation based on the latest linked CJTC Dd report when calculating the Rel-18 Type-II CJT CSI, when a single CSI trigger state associated with >1 Rel-18 Type-II CJT CSI reports, the 1-bit indicator per CSI trigger state applies to all the Rel-18 Type-II CJT CSI reports linked with the CJTC Dd report(s).

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s):

- The scaling formula is ρ.si where ρ is a multiplicative factor independent of i

  - Reuse legacy precoder normalization (per discretion of the spec editor)

  - FFS (RAN1#119): Whether min(ρ.si , 1) operation is needed

  - FFS (RAN1#119): Whether ρ other than 1 (baseline) is needed (e.g. v or 1v)

- Regarding the configuration of the si value (3-bit indicator per SD basis vector group), decide, by RAN1#119, between the following:

  - Alt1. RI=1 and RI=2 are separately configured (RI-specific)

  - Alt2. A same configuration is used for RI=1 and RI=2 (RI-common)

Conclusion

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding packing order and priority levels of NRep CSI reports with M CRIs, the UCI packing order and the consecutive priority levels (higher to lower) follows the legacy specifications.

R1-2410754	Moderator Summary for 1st offline on Rel-19 CSI enhancements	Moderator (Samsung)

R1-2410755	Moderator Summary for 2nd offline on Rel-19 CSI enhancements	Moderator (Samsung)

R1-2410751	Moderator Summary#2 on Rel-19 CSI enhancements: Round 2	Moderator (Samsung)

From Wednesday session

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding wideband P/SP-CSI reported using one-part CSI, if resource-specific RI restriction is configured, the zero padding bits for each of the M reported CRI are determined as follows:

- For a k-th CRI from the M reported CRIs, OP,k=Nmax-Nreported,k, where:

  - Nmax= maxj∈Q  (Nmax,j) , where Q is the set of CRIs corresponding to Ks resources and Nmax,j is the maximum payload size of associated CSI fields for a j-th CRI, and Nmax,j=NRI(j)+maxrj∈Srank,jB(rj), where Srank,j is the set of rank values rj that are allowed to be reported for the j-th CRI;

  - Nreported,k= NRI(k)+B(Rk) , where Rk is the reported rank for k-th CRI;

  - Note: NRI(k) is the size of RI field corresponding to k-th CRI.

  - Note: The definition of the operator B(∙) is as legacy (as defined in 38.212).

Note: Here k, j=1, 2, …, KS

Note: How this is captured in the spec (including exact formulation) is up to the editor(s).

Agreement

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding aperiodic CSI-RS resource configuration, an RRC-configured resource-level slot offset (relative to the resource-set-level slot offset, using the same design as Rel-19 Type-I/II codebook refinement for 48, 64, and 128 ports) is supported for aperiodic CSI-RS resource set.

- FFS: The number of bits for indicating the resource-level slot offset (relative to the resource-set-level slot offset) for KS resources, including the value(s) of the slot offset.

- FFS: Whether, in addition, configuring an available slot offset for each CSI-RS resource within the aperiodic CSI-RS resource set.

  - Note: “Available slot offset” is analogous to the Rel-17 SRS triggering offset enhancement.

Agreement

For the Rel-19 Type-I SP codebook refinement for P=48, 64, 128 CSI-RS ports, when the UE reports or multiplexes the CSI that include Part 2 CSI reports on PUCCH, the PUCCH resource, the number of PRBs for the PUCCH resource, and/or the number of Part 2 CSI reports are determined based on the RI value that results in the largest UCI payload.

- For Scheme-A, the RI value is:

  - 8 when the RI value(s) allowed by the configured RI restriction per CSI reporting configuration include 5, 6, 7, and/or 8,

  - 1 otherwise.

- For Scheme-B, the RI value is 4.

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint triggering carried on a same PUSCH (hence on a same slot), the UCI associated with the CJTC Dd report is multiplexed in CSI Part 1.

- The previously agreed UCI design and mapping order for CJTC Dd report are reused.

- The legacy UCI design, UCI mapping order, and UCI omission for the Rel-18 eType-II CJT CSI are reused.

Note: The above proposal reuses the legacy UCI design principles, where the UCI associated with the CJTC Dd is placed in the part of UCI as TS 38212 Table 6.3.1.1.2-13; the CSI part 1 of Rel-18 eType-II CJT CSI is placed in the part of UCI as TS 38.212 Table 6.3.1.1.2-13 and the CSI part 2 of Rel-18 eType-II CJT CSI is placed in the part of UCI as TS 38.212 Table 6.3.1.1.2-14.

Agreement

For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), the UE assumes that the NTRP CSI-RS resources are transmitted without DL/UL switching in between the NTRP resources.

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors associated with RI=v=2 for the 3-bit scaling factor(s), ρ=1.

- Note: In this case, the min(si,1) operation is not needed.

Agreement

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, regarding per-layer scaling factor applied to each of the selected SD basis vectors for the 3-bit scaling factor(s), the configuration of the si value (3-bit indicator per SD basis vector group) is RI-common (a same configuration is used for RI=1 and RI=2).

Conclusion

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports with the Rel-15 Type-I SP codebook, joint operation with the Rel-18 NES framework is not supported.

R1-2410752	Moderator Summary#3 on Rel-19 CSI enhancements: Round 3	Moderator (Samsung)

R1-2410756	Moderator Summary for 3rd offline on Rel-19 CSI enhancements	Moderator (Samsung)

### Support for 3-antenna-port codebook-based transmissions

Including support 3T6R SRS antenna switching.

R1-2409372	Enhancements for 3-antenna-port transmissions	MediaTek Inc.

R1-2409379	Discussion on 3-antenna-port codebook-based transmissions	ZTE Corporation, Sanechips

R1-2409429	Discussion on 3-antenna-port UL transmission	Huawei, HiSilicon

R1-2409461	Further Details on Rel-19 CB-based UL for 3TX UE	InterDigital, Inc.

R1-2409506	Discussion on support for 3-antenna-port codebook-based transmissions	CMCC

R1-2409590	Views on Rel-19 3-antenna-port codebook-based transmissions	Samsung

R1-2409675	Remaining issues on 3-antenna-port codebook-based uplink transmissions	vivo

R1-2409794	Remaining issues on R19 MIMO 3Tx transmission	Apple

R1-2409890	Discussion on the support of 3-antenna-port CB based transmissions	Xiaomi

R1-2409935	Views on 3-antenna-port uplink transmissions	CATT

R1-2409971	Support for 3-antenna-port codebook-based transmissions	Lenovo

R1-2410055	Discussion on uplink enhancement for UE with 3Tx	Fujitsu

R1-2410110	Discussion on 3-antenna-port uplink transmissions	OPPO

R1-2410155	Uplink 3 Port Codebook based Transmission	Google

R1-2410317	On the support for 3-antenna-port codebook-based transmissions	Nokia

R1-2410341	Remaining issues for 3 Tx UL transmissions	Ericsson

R1-2410383	Discussion on support for 3-antenna-port codebook-based transmissions	NTT DOCOMO, INC.

R1-2410437	Support for 3-antenna-port transmission	Sharp

R1-2409463	FL Summary Support for 3TX CB-based Uplink; First Round	Moderator (InterDigital, Inc.)

From Monday session

Conclusion

To support 3T6R antenna switching for a 3TX UE,

- Reuse the power control parameters configuration principles of 4T8R for 3T6R, i.e., when two aperiodic SRS resource sets are configured,

  - if a UE is provided TCI-State in dl-OrJointTCI-StateList or TCI-UL-State, the UE shall expect that the two SRS resource sets are associated with the same values of the higher layer parameters p0AlphaSetforSRS and pathlossReferenceRS-Id

  - Otherwise, the UE shall expect that the two SRS resource sets are configured with the same values of the higher layer parameters alpha, p0, pathlossReferenceRS, and srs-PowerControlAdjustmentStates in SRS-ResourceSet.

Agreement

The following capabilities are introduced for Rel-19

- 't3r3' for 3T3R.

- 't3r6' for 3T6R.

From AI 5

R1-2409353	LS on UE RF issues for Rel-19 MIMO enhancement	RAN4, Huawei

From Monday session

Agreement

Support the following replies to Questions 2 and 3 in RAN4 LS,

Question 2: RAN4 would like to check with RAN1 whether a 3Tx UE supporting UL full power transmission mode 0 needs to achieve the same maximum output power with both 1Layer and 2Layers. Question 3: RAN4 would like to check with RAN1 whether 3Tx UE can support UL full power transmission mode 0 with 2Layer 3-port codebooks but not support 1Layer 3-port codebooks.

Reply to Q2: Yes, a 3Tx UE supporting UL full power transmission mode 0, delivers the same maximum output power with both 1- and 2-layer precoders.

Reply to Q3: No, a 3Tx UE supporting UL full power transmission mode 0, shall support full power with both 1- and 2-layer 3-port codebook.

R1-2409464	FL Summary Support for 3TX CB-based Uplink; Second Round	Moderator (InterDigital, Inc.)

From Wednesday session

R1-2410762	Draft Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement	InterDigital, OPPO

Decision: The draft LS is endorsed and final version is approved in R1-2410870.

Agreement

For a UE operating with 3TX,

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as codebook.

- the RRC parameter for enabling a configured 4-port SRS resource with port 1003 disabled, is applied for all SRS resource set(s) for SRS usage set as antenna switching.

- Note: The RRC parameter mentioned in the above two bullets is configured per set

A UE does not expect an SRS resource to be configured in both codebook and antenna switching resource sets with conflicting (disabled state) configuration for the port 1003.

Final summary in R1-2409465.

### Enhancement for asymmetric DL sTRP/UL mTRP scenarios

Including support for 2TA without CoresetPoolIdx association for asymmetric DL sTRP/UL mTRP.

R1-2409373	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	MediaTek Inc.

R1-2409380	Discussion on enhancements for asymmetric DL sTRP/UL mTRP scenarios	ZTE Corporation, Sanechips, China Telecom

R1-2409430	Enhancements for asymmetric DL sTRP/UL mTRP scenarios	Huawei, HiSilicon

R1-2409462	Further Details  on Rel-19 Asymmetric mTRP Operation	InterDigital, Inc.

R1-2409507	Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios	CMCC

R1-2409591	Views on Rel-19 asymmetric DL sTRP/UL mTRP scenarios	Samsung

R1-2409631	Enhancements for asymmetric DL sTRP/UL mTRP scenarios	Spreadtrum, UNISOC

R1-2409676	Remaining issues on asymmetric DL sTRP/UL mTRP scenarios	vivo

R1-2409709	Discussion on asymmetric DL sTRP/UL mTRP scenarios	TCL

R1-2409746	Enhancements for asymmetric DL/UL scenarios	Intel Corporation

R1-2409762	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Tejas Networks Limited

R1-2409769	Enhancement for asymmetric DL sTRP UL mTRP scenarios	Ericsson

R1-2409795	Enhancements for asymmetric DL sTRP/UL mTRP	Apple

R1-2409858	Discussion on enhancements for asymmetric DL sTRP and UL mTRP scenarios	NEC

R1-2409891	Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios	Xiaomi

R1-2409936	Views on Rel-19  asymmetric DL sTRP/UL mTRP scenarios	CATT

R1-2409962	Discussion on enhancements for asymmetric DL sTRP/UL mTRP scenarios	Transsion Holdings

R1-2409972	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Lenovo

R1-2409997	Discussion on enhancements for asymmetric DL sTRP/UL mTRP scenarios	China Telecom, ZTE

R1-2410056	Discussion on UL-only mTRP operation	Fujitsu

R1-2410111	Enhancements on asymmetric DL sTRP/UL mTRP scenarios	OPPO

R1-2410165	Discussion on enhancement for asymmetric DL sTRP and UL mTRP scenarios	Google

R1-2410198	Discussions on asymmetric DL sTRP/UL mTRP scenarios	LG Electronics

R1-2410221	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Sony

R1-2410263	Discussion on DL single TRP and UL mTRP operation	ETRI

R1-2410318	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Nokia

R1-2410384	Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios	NTT DOCOMO, INC.

R1-2410438	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Sharp

R1-2410473	Enhancement for asymmetric DL sTRP and UL mTRP deployment scenarios	Qualcomm Incorporated

R1-2410534	"Enhancement for Asymmetric DL sTRP/UL mTRP Scenarios"	Panasonic

R1-2410539	Discussion on asymmetric DL sTRP and UL mTRP	ASUSTeK

R1-2410649	Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

Presented in Monday session

R1-2410650	Summary #2 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

From Tuesday session

Agreement

Support to apply PL offset on PDCCH-order PRACH in FR2

- The design for applying PL offset on PDCCH-order PRACH in FR1 is fully reused here.

Note: there is no extra enhancement for Tx beam determination for PDCCH-order PRACH in FR2 in Rel-19.

Agreement

Support DCI format 1_1 to indicate TPC command for SRS CLPC adjustment state(s) separate from PUSCH:

- (Working Assumption) Introduce a 2-bit TPC command field to indicate TPC command for SRS associated with separate SRS CLPC adjustment state where:

  - The 2-bit TPC command field is present if UE reports supporting a dedicated UE capability, and a corresponding RRC parameter is configured (which is a new RRC to enable this).

- (Working Assumption) Introduce a 1-bit SRS close-loop indicator to indicate one of the two separate SRS CLPC adjustment states for the TPC command

  - The 1-bit SRS close-loop indicator is present if UE reports supporting another dedicated UE capability and a corresponding RRC parameter is configured (which is a new RRC to enable this) and two separate SRS CLPC adjustment states are configured.

Agreement

For a UE supporting the UE capability of “Overlapping UL transmission reduction” of Rel-19 2TA:

- The overlapping duration of the latter one of two overlapping UL transmissions is reduced, where the two overlapped UL transmissions are associated with two TAGs.

Agreement

For an SRS resource set not configured with followUnifiedTCI-StateSRS, regarding how to determine the PL offset and the CLPC adjustment state, down-select from the following Alts:

- Alt1: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17) and no extra enhancement.

- Alt2: The PL offset and CLPC adjustment state indicated by the TCI state configured to the SRS resource with lowest ID (i.e., reusing the rule specified in Rel-17), the SRS with lowest ID is always configured with one joint/UL TCI state, the “referenceSignal” in a UL TCI state is optional.

- Alt3:

  - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17)

  - Otherwise, one of the two separate SRS CLPC adjustment states and one PL offset are configured in the SRS resource set, and they are applied on the SRS resources.

- Alt4:

  - When the SRS resource with lowest ID is configured with a TCI state, The PL offset and CLPC adjustment state indicated by that TCI state are applied to those SRS resources (i.e., reusing the rule specified in Rel-17)

  - Otherwise, one PL offset and one CLPC adjustment state are applied on the SRS resources.

    - FFS how to define the PL offset and CLPC adjustment state

From AI 5

R1-2409353	LS on UE RF issues for Rel-19 MIMO enhancement	RAN4, Huawei

From Tuesday session

(Question 1: RAN4 would like to inquire if the UL TRP(s) can transmit SSBs per the scope of the WID.)

Agreement

The answer to the Question 1 in LS R1-2409353 is:

- From the perspective of UE: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s), else, UE may expect to receive SSB from UL TRP(s).

See final reply LS in R1-2410870.

Final summary in R1-2410651.

## Evolution of NR duplex operation: Sub-band full duplex (SBFD)

Please refer to RP-241614 for detailed scope of the WI.

[119-R19-Duplex] – Xinghua (Huawei)

Email discussion on Rel-19 SBFD

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### SBFD TX/RX/measurement procedures

Including semi-static indication of SBFD resources.

R1-2409374	Discussion for SBFD TX/RX/ measurement procedures	New H3C Technologies Co., Ltd.

R1-2409410	On subband full duplex design	Huawei, HiSilicon

R1-2409452	Discussion on SBFD TX/RX/measurement procedures	MediaTek Inc.

R1-2409474	Discussion on SBFD TX/RX/measurement procedures	TCL

R1-2409488	Discussion on SBFD TX/RX/measurement procedures	LG Electronics

R1-2409508	Discussion on SBFD TX/RX/measurement procedures	CMCC

R1-2409538	Discussion on transmission, reception and measurement procedures for SBFD operation	ZTE Corporation, Sanechips

R1-2409566	Discussion on SBFD TX_RX_measurement procedures	InterDigital, Inc.

R1-2409593	SBFD operation and procedures	Samsung

R1-2409632	Discussion on SBFD TX/RX/measurement procedures	Spreadtrum, UNISOC

R1-2409677	Discussion on Rel-19 SBFD operation	vivo

R1-2409763	Discussion for SBFD TX/RX/ measurement procedures	Tejas Networks Limited

R1-2409796	Discussion on SBFD TX/RX/measurement procedures	Apple

R1-2409843	Discussion on SBFD TX/RX/measurement procedures	Ruijie Networks Co. Ltd

R1-2409892	Discussion on reception and transmission procedure for SBFD operation	Xiaomi

R1-2409937	Discussion on SBFD TX/RX/measurement procedures	CATT

R1-2409963	Discussion on SBFD operation	Transsion Holdings

R1-2409998	Discussion on SBFD TX/RX/measurement procedures	China Telecom

R1-2410045	Discussion on SBFD TX/RX/measurement procedures	Panasonic

R1-2410057	Discussion on SBFD Tx/Rx/measurement procedures	Fujitsu

R1-2410086	Discussion on SBFD Tx/Rx/measurement procedures	OPPO

R1-2410137	On SBFD TX/RX/measurement procedures	Nokia, Nokia Shanghai Bell

R1-2410140	SBFD TX/RX/measurement procedures	Ericsson

R1-2410177	Discussion on SBFD TX/RX/measurement procedures	HONOR

R1-2410191	Views on SBFD TX/RX Measurement Procedures	Lenovo

R1-2410206	Discussion on subband non-overlapping full duplex Tx-Rx and measurement operations	NEC

R1-2410222	SBFD Procedures	Sony

R1-2410241	Discussion on SBFD Tx/Rx/measurement procedures 	Fraunhofer HHI, Fraunhofer IIS

R1-2410264	Discussion on SBFD TX/RX/measurement procedures	ETRI

R1-2410385	Discussion on SBFD TX/RX/measurement procedures	NTT DOCOMO, INC.

R1-2410410	SBFD Tx/Rx/measurement aspects	Sharp

R1-2410428	Discussion on SBFD TX/RX/measurement procedures	ITRI

R1-2410474	SBFD Transmission, Reception and Measurement Procedures	Qualcomm Incorporated

R1-2410545	Discussion on SBFD TX/RX/measurement procedures	Google Ireland Limited

R1-2410548	Partial PRG handling for SBFD	ASUSTeK

R1-2410558	Discussion on SBFD TX/RX/measurement procedures	WILUS Inc.

R1-2410574	Discussion on SBFD TX/RX/ measurement procedures	CEWiT

R1-2410692	Summary #1 of SBFD TX/RX/measurement procedures	Moderator (CATT)

From Tuesday session

Agreement

For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, an SBFD aware UE does not transmit or receive the physical channel/signal within the slot., except

- PRACH transmission in a valid RO across SBFD symbols and non-SBFD symbols is subject to the agreements in AI 9.3.2.

- For PUSCH repetition type B, down-select from the following options.

  - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

  - Option 2: A nominal repetition is segmented into actual repetitions as in legacy operation. An actual repetition if it is mapped to both SBFD and non-SBFD symbols is dropped.

R1-2410693	Summary #2 of SBFD TX/RX/measurement procedures	Moderator (CATT)

From Wednesday session

Agreement

Confirm the following working assumption made in RAN1#118.

Working Assumption

For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275.

- One or two SBFD DL subbands can be configured

Agreement

For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SP-CSI on PUSCH is SBFD symbols,

- When an assigned RBG overlaps with the subband boundary, only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission.

- SBFD aware UE does not expect to be assigned with a RBG for PUSCH which is fully outside UL usable PRBs.

Agreement

Update the following agreement made in RAN1#118 meeting:

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) with Configuration 1,

- For PUSCH repetition type A with available slot counting, A-SRS with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type.

- For CG PUSCH with neither TBoMS nor PUSCH repetition type A with available slot counting,and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped.

Agreement

Update the following agreement made in RAN1#118bis meeting in cyan:

Agreement

For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by:

RBstart=RBstart, i=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, i=1

For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by:

RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart[-RBUL SB start]+RBoffset)modNUL SBsize, nsμmod2=1

Where

- RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP.

- NUL SBsize is the number of UL usable PRBs.

- RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. For PUSCH transmissions with Configuration 2, RBstart is the starting PRB index with reference to the start of UL active BWP after applying RB offset between non-SBFD symbols and SBFD symbols.

- RB offset is the frequency hopping offset for PUSCH in SBFD symbols

- Note: Definition of nsμ is unchanged from existing specifications

Decide in RAN1#119, one of the following options:

- Alt1: Remove -RBUL SB start in the above equations

- Alt2: Remove square brackets from [-RBUL SB start] in the above equations

Agreement

For dynamically scheduled PDSCH repetitions with Configuration 2, for TBS determination.

- Option 1:

  - If the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs.

  - If the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs.

Agreement

Red text is working assumption.

For separate SRS configurations for SBFD and non-SBFD symbols, support Option 1.

Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage.

- SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols.

  - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped.

  - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission.

    - FFS impact on availableSlotOffsetList

  - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols.

  - FFS: whether there is impact on frequency hopping counter

- SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols.

  - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped.

  - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission.

    - FFS impact on availableSlotOffsetList

  - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols.

  - FFS: whether there is impact on frequency hopping counter

- The number of SRS resources in a SRS-ResourceSet for SBFD symbols is the same as the number of SRS resources in a SRS-ResourceSet for non-SBFD symbols for the same usage.

- For SRS-ResourceSets with usage set to 'nonCodebook', only one SRS port for each SRS resource is configured as legacy.

- For SRS-ResourceSets with usage set to 'Codebook',

  - Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the numbers of SRS ports are the same for all the SRS resources in the two SRS-ResourceSets

  - FFS when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2'

- Maintain single TRP behaviour with respect to DCI fields and RRC parameters in rrc-ConfiguredUplinkGrant

  - SRS resource set indicator (SRSI) field is absent from DCI. srs-ResourceSetId is absent in rrc-ConfiguredUplinkGrant.

  - If txConfig is set to 'codebook', second precoding information (TPMI) field is absent from DCI and precodingAndNumberOfLayers2 is absent from rrc-ConfiguredUplinkGrant.

  - Second SRS resource indicator (SRI) field is absent from DCI

    - For DG PUSCH without repetition within a slot, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as that of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI.

      - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as that of PUSCH transmission

    - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 1, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI.

      - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource  identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission

    - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 2, the SRS resource indicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. For PUSCH transmissions in non-SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI.

      - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols.

    - If only a single SRS resource is configured in both SRS resource sets, the SRI field is absent from DCI. In this case “SRS resource identified by the SRI” in the above bullets corresponds to the single SRS resource in each SRS resource set.

  - srs-ResourceIndicator2 is absent from rrc-ConfiguredUplinkGrant.

    - For Type 1 CG PUSCH with Configuration 1, srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission.

      - If txConfig is set to 'codebook', precodingAndNumberOfLayers corresponds to the SRS resource  identified by the SRI in the SRS-ResourceSet corresponding to  the same symbol type as the valid symbol type of PUSCH transmission.

    - For Type 1 CG PUSCH with Configuration 2, srs-ResourceIndicator  is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols. For PUSCH transmissions in non-SBFD symbols, the srs-ResourceIndicator   is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols.

      - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in  the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in  the SRS-ResourceSet corresponding to non-SBFD symbols.

  - If txConfig is set to 'nonCodebook', each SRS resource set is associated with an associated CSI-RS.

- Applicable to usage set to 'codebook' or 'nonCodebook'

- FFS usage set to  'beamManagement'

- Not applicable to usage set to 'antennaSwitching’

Conclusion

There is no RAN1 consensus to support mTRP scenario for SBFD in Rel-19.

Agreement

The maximum number of power control loops is the same as legacy (i.e. no more than 2) for SBFD operation in Rel-19.

Agreement

For a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, SFI in DCI format 2_0 is only applicable to non-SBFD symbols but not applicable to SBFD symbols

- Note: no additional specification efforts are expected to support joint operation of SBFD and dynamic SFI.

R1-2410694	Summary #3 of SBFD TX/RX/measurement procedures	Moderator (CATT)

From Thursday session

Agreement:

For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only,

- For type 2 CG PUSCH, SPS PDSCH, down-select from the following options:

  - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI.

  - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI.

Agreement

At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for determining starting PRB for PUSCH transmissions in SBFD symbols,

- Equation 1-C2: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize

  - If RBoffsetSBFD is not configured, it is zero

Agreement

For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework,

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols

  - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE,

- Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability.

- SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis.

  - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP.

  - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP.

    - For SRS, only Configuration 1 is applicable.

Agreement

It is clarified that ‘the first transmission/reception’ in the following agreement in RAN1#118 refers to the first transmission/reception occasion indicated by scheduling DCI.

- It is not expected that the first transmission/reception occasion indicated by scheduling DCI is mapped to both SBFD and non-SBFD symbols.

Agreement

For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following.

<Unrelated part omitted>

- For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception.

<Unrelated part omitted>

### SBFD random access operation

R1-2409375	Discussion for SBFD random access operation	New H3C Technologies Co., Ltd.

R1-2409411	On subband full duplex random access operation	Huawei, HiSilicon

R1-2409453	Discussion on SBFD Random Access Operation	MediaTek Inc.

R1-2409475	Discussion on SBFD random access operation 	TCL

R1-2409489	Discussion on SBFD random access operation	LG Electronics

R1-2409509	Discussion on SBFD random access operation	CMCC

R1-2409539	Discussion on SBFD random access operation	ZTE Corporation, Sanechips

R1-2409567	On SBFD random access operation	InterDigital, Inc.

R1-2409594	Random access on SBFD resources	Samsung

R1-2409633	Discussion on SBFD random access operation	Spreadtrum, UNISOC

R1-2409678	Discussion on random access for Rel-19 SBFD	vivo

R1-2409764	Discussion on SBFD Random Access Operation	Tejas Networks Limited

R1-2409797	Views on SBFD random access operation	Apple

R1-2409844	Discussion on SBFD random access operation	Ruijie Networks Co. Ltd

R1-2409873	Discussion on random access for SBFD	NEC

R1-2409893	Discussion on SBFD random access operation	Xiaomi

R1-2409910	Discussion on SBFD random access operation	Korea Testing Laboratory

R1-2409938	Discussion on SBFD random access operation	CATT

R1-2409964	Discussion on SBFD random access operation	Transsion Holdings

R1-2409999	Discussion on SBFD random access operation	China Telecom

R1-2410046	Discussion on SBFD random access operation	Panasonic

R1-2410058	Discussion on SBFD random access operation	Fujitsu

R1-2410087	Discussion on SBFD random access operation	OPPO

R1-2410138	SBFD random access operation	Nokia, Nokia Shanghai Bell

R1-2410141	SBFD Random access operation	Ericsson

R1-2410172	Discussion on SBFD random access operation	Hyundai Motor Company

R1-2410223	SBFD RACH Operations	Sony

R1-2410251	SBFD random access operation	Lenovo

R1-2410265	SBFD random access operation	ETRI

R1-2410324	On SBFD random access operation	Google Ireland Limited

R1-2410345	Discussion on SBFD Random Access operation	KT Corp.

R1-2410386	Discussion on SBFD random access operation	NTT DOCOMO, INC.

R1-2410412	SBFD random access aspects	Sharp

R1-2410429	Discussion on SBFD random access operation for SBFD aware UEs	ITRI

R1-2410475	SBFD Random Access Operation	Qualcomm Incorporated

R1-2410544	Discussion on SBFD random access operation	ASUSTEK COMPUTER (SHANGHAI)

R1-2410559	Discussion on SBFD random access operation	WILUS Inc.

R1-2410575	Discussion on SBFD random access operation	CEWiT

R1-2410674	Summary#1 on SBFD random access operation	Moderator (CMCC)

From Tuesday session

Agreement

Introduce explicit indications (1-bit SIB1 indication and/or dedicated signalling to convey cell-specific configuration) to indicate whether RACH configuration Option 1 for SBFD random access operation is enabled or not from network side.

Agreement

For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs.

- FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority

Agreement

For RACH configuration Option 2, the association period and association pattern period of additional-ROs are determined separately from legacy-ROs.

- The legacy rules for determining association period and association pattern period are reused in principle for additional-ROs.

Agreement

Confirm the following working assumption

Working Assumption

For a PRACH transmission with Npreamblerep preamble repetition within additional-ROs, support reusing current specification rules for the determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs.

- The time period of the set(s) of Npreamblerep valid additional-ROs is determined separately from the time period for legacy-ROs.

Agreement

It is up to RAN2 to determine the detailed specified/configured conditions/prioritizations for RO selection for initial PRACH transmission attempt in one random access procedure.

Agreement

For CFRA triggered by PDCCH order for SBFD aware-UEs, SBFD-aware UE is explicitly indicated to use additional-RO or legacy-RO via a 1-bit field in the DCI of PDCCH order.

R1-2410675	Summary#2 on SBFD random access operation	Moderator (CMCC)

From Wednesday session

Agreement

For RACH configuration Option 2, and for interpretation of the parameter prach-ConfigurationIndex provided by the additional RACH configuration, down-select from the following options:

- Alt-1: introduce a subframe/slot offset (similar as prach-ConfigurationSOffset-IAB) to determine the subframe/slot number for ROs in SBFD symbols for FR1/FR2.

  - FFS: details of the subframe/slot offset.

- Alt-2: introduce a new parameter indicating from an entry from a pre-defined/configured table to derive additional subframe/slot numbers.

  - Details are FFS.

- Alt-3: introduce a bitmap indicating or puncturing subframe(s)/slot(s) for additional ROs in SBFD symbols.

  - Details are FFS.

- Alt-4: no enhancement.

Agreement

For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

- msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs.

Agreement

For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

- Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

Agreement

For SBFD aware UEs, for Msg3 PUSCH transmission,

- the number of NUL,hop hopping bits is based on the number of PRBs in initial UL BWP as in legacy specification.

- the frequency offset for the 2nd hop is reserved for the case when the value of NUL,hop= 11 as in legacy specification.

Agreement

For an SBFD aware UE, if addition RO is selected for Msg1 transmission, only Configuration 1 is supported for Msg3 repetition.

Agreement

For determining PUCCH resource sets in SBFD symbols before dedicated PUCCH resource configuration, reuse the Table 9.2.1-1 in TS 38.213, with the following modifications:

- use the size of UL usable PRBs to replace the size of initial UL BWP to determine the PRB offset in row index 15.

Agreement

For RACH configuration Option 1 with Alt 1-1, for determination of lowest RO of additional-ROs in SBFD symbols, Alt 2-1 is supported.

- Alt 2-1: The parameter msg1-FrequencyStart in rach-ConfigCommon is reinterpreted as the frequency offset of lowest RO in frequency domain with respective to the lowest PRB of UL usable PRBs.

- ROs that are outside of UL usable PRBs are invalid.

Agreement

For SBFD aware UEs, update the following equations to determine the lowest PRB indices of the PUCCH transmission in the first hop and second hop in SBFD symbols before dedicated PUCCH resource configuration:

- if rPUCCH/8=0

  - RBUL SB start+RBBWPoffset∙NRB+rPUCCH/NCS∙NRB for the first hop

  - RBUL SB start+NUL SBsize-RBBWPoffset∙NRB-(1+rPUCCH/NCS) ∙NRB for the second hop

- if rPUCCH/8=1

  - RBUL SB start+NUL SBsize-RBBWPoffset∙NRB-(1+(rPUCCH-8)/NCS) ∙NRB for the first hop

  - RBUL SB start+RBBWPoffset∙NRB+(rPUCCH-8)/NCS∙NRB for the second hop

Where

- RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP.

- NUL SBsize is the number of UL usable PRBs.

- RBBWPoffset is the RB offset with reference to the lowest PRB of UL usable PRBs.

Note: Definitions of NRB, rPUCCH and NCS are unchanged from existing specifications.

R1-2410676	Summary#3 on SBFD random access operation	Moderator (CMCC)

From Thursday session

Agreement

For RACH configuration Option 2, for the case that additional ROs overlap with legacy ROs, down-select from the following alternatives:

- Alt-1: do not optimize this case.

- Alt-2: the additional-ROs are considered invalid when overlapping with legacy-ROs.

Agreement

For Msg3 PUSCH repetition Configuration 1, the valid symbol type is determined based on the symbol type of the first transmission occasion.

- UE does not expect the first transmission occasion includes different symbol types (i.e., SBFD symbols and non-SBFD symbols).

- FFS: Available slot counting on SBFD symbols.

Agreement

Study how to determine the Msg3 PUSCH power control in SBFD symbols and non-SBFD symbols based on at least the following parameters.

- preambleReceivedTargetPower, msg3-DeltaPreamble/deltaPreamble, TPC Command in RAR.

### CLI handling

R1-2409376	Discussion on CLI handling	New H3C Technologies Co., Ltd.

R1-2409412	On cross-link interference handling for subband full duplex	Huawei, HiSilicon

R1-2409454	Discussion on CLI Handling in SBFD System	MediaTek Inc.

R1-2409490	Discussion on CLI handling	LG Electronics

R1-2409510	Discussion on CLI handling	CMCC

R1-2409540	Discussion on CLI handling for Rel-19 duplex operation	ZTE Corporation, Sanechips

R1-2409568	On CLI handling for SBFD operation	InterDigital, Inc.

R1-2409595	Cross-link interference handling for SBFD	Samsung

R1-2409634	Discussion on CLI handling	Spreadtrum, UNISOC

R1-2409679	Discussion on CLI handling for Rel-19 NR duplex	vivo

R1-2409765	Discussion on gNB-gNB CLI handling	Tejas Networks Limited

R1-2409798	Discussion on CLI handling	Apple

R1-2409874	CLI handling for NR duplex operations	NEC

R1-2409894	Discussion on CLI handling for SBFD operation	Xiaomi

R1-2409939	Discussion on CLI handling for NR duplex evolution	CATT

R1-2410047	Discussion on CLI handling	Panasonic

R1-2410088	CLI handling in NR duplex operation	OPPO

R1-2410139	Cross-link interference handling for duplex evolution	Nokia, Nokia Shanghai Bell

R1-2410142	CLI handling	Ericsson

R1-2410224	SBFD CLI Handling	Sony

R1-2410266	Discussion on CLI handling for SBFD operation	ETRI

R1-2410325	On the gNB-to-gNB CLI and the UE-to-UE CLI handling	Google Ireland Limited

R1-2410387	Discussion on CLI handling	NTT DOCOMO, INC.

R1-2410433	Views on uplink resource muting	SHARP Corporation

R1-2410476	Enhancements for CLI handling	Qualcomm Incorporated

R1-2410576	Discussion on CLI handling	CEWiT

R1-2410788	Summary #1 of CLI handling	Moderator (Huawei)

From Tuesday session

Agreement

Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement

- Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis)

- FFS: Other configurations

Define SRS-RSRP-MeasurementResource with the following parameters:

- Legacy SRS Resource IE

- FFS: Other parameters

Agreement

Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement.

- Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis)

Define CLI-RSSI-MeasurementResource with the following parameters:

- CLI-RSSI measurement resource ID

- Starting PRB index

- Number of PRBs

- Starting symbol of the CLI-RSSI resource within a slot

- Number of symbols of the CLI-RSSI resource within a slot

- Periodicity and slot offset for the CLI-RSSI resource

- FFS: Other parameters

Agreement

To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH

- Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI.

R1-2410789	Summary #2 of CLI handling	Moderator (Huawei)

From Wednesday session

Agreement

For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols.

Working assumption

For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM

- A comb offset {0, 1} is configured for the up to two UL muting symbols.

Agreement

If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol.

Agreement

If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS.

Agreement

Confirm the following working assumption

Working Assumption

UL resource muting is allowed on PUSCH symbols that carry UCI, i.e., UCI is not mapped on muted REs

Note: As mentioned in the WID, no specification changes are allowed on data and control multiplexing in section 6.2.7, TS 38.212.

Agreement

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter

- For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD.

- If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE

- TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none.

- If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

Agreement

For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported

- Method #3: UE measures RSSI within UL subband

- A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

Conclusion

There is no RAN1 consensus on the following:

- Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19.

R1-2410790	Summary #3 of CLI handling	Moderator (Huawei)

From Thursday session

Agreement

3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting.

- FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Include the above in the LS to RAN4. In addition, mention that RAN1 is still discussing the boosting in symbols containing PTRS.

Agreement

Send an LS to RAN4 with the following contents:

On UL resource muting, RAN1 has made the following agreements

Agreement If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol. Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI Agreement For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Working assumption For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols Agreement If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS. Agreement 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting. - FFS: PUSCH and PT-RS EPRE determination for the PUSCH symbol containing PT-RS with UL resource muting when transform precoding is not enabled.

Note that RAN1 is still discussing the boosting in symbols containing PTRS. RAN1 would like to ask RAN4 to check impact on transmit signal quality/MPR, if any.

Final LS is in R1-2410889.

Agreement

Send reply LS to RAN4 with following information:

When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios.

For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations.

However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID

  - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs

From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

Final LS is in R1-2410890.

Conclusion

For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported.

Conclusion

There is no RAN1 consensus to support aperiodic CLI reporting on PUCCH in Rel-19.

Agreement

For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported

- The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4}

- The M CLI measurement resource ID(s) are reported

- The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported

- When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference

- FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP

- FFS: whether/how to report differential value in case the largest measurement is out of range

Agreement

For PUSCH repetition Type A in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) is applied for all PUSCH repetitions.

For PUSCH repetition Type B in SBFD symbols with Configuration 1, the same time location of none, one or two UL muting symbol(s) per slot is applied for all the actual PUSCH repetitions.

Agreement

If transform precoding is enabled for a PUSCH, study further the following two options:

- Option 1: The DFT size is modified as MscPUSCH2 for symbol(s) with UL resource muting

- Option 2: The DFT size is MscPUSCH for symbol(s) with UL resource muting

Note: MscPUSCH is the total number of subcarriers of the PUSCH.

FFS: Whether any of option 1, option 2 needs to be reflect into RAN1 specifications.

## Study on solutions for Ambient IoT (Internet of Things) in NR

Please refer to RP-240826 for detailed scope of the SI. For additional clarification on the work scope, please refer to section 8 of RP-240854 and section 3 of RP-242360.

R1-2410845	Session notes for 9.4 (Study on solutions for Ambient IoT in NR)	Ad-Hoc Chair (Huawei)

Friday decision: The session notes are endorsed and contents reflected below.

[119-R19-A-IoT] – Xiaodong (CMCC)

Email discussion on Rel-19 Ambient IoT

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2409993	TR 38.769 v1.1.0 “Study on Solutions for Ambient IoT (Internet of Things)”	Huawei (TR editor)

Post RAN1#118bis decision: As per email decision posted on Nov 6th, the TR update is endorsed as v1.1.0 of TR 38.769 (in RAN1#119 Tdoc# R1-2409993), version to be used as baseline for any further updates.

R1-2410330	TP for Conclusions in TR 38.769	Huawei, HiSilicon

R1-2410703	Feature Lead Summary #1 for 9.4: “Ambient IoT – TR Conclusions”	Moderator (Huawei)

From Thursday session

Agreement

TP#1 to TP#5 (and sub-TPs) below are endorsed for the conclusion section of TR38.769.

TP#1

It is feasible to support a harmonized air interface design, with minimized differences (where necessary) for Ambient IoT, to enable the devices described in Clause 3 for addressing the objectives stated in [2]. The extent to which mMaximum distance targets, differentiated per device and deployment/topology scenario, as described in Clause 4.1, can be met under different sets of evaluation assumptions is reported in Clause 7.1. and aA latency target for inventory of a single device in TR 38.848, can be met under different evaluation assumptions according to evaluations reported in Clause 7.2.1. OtherThe addressed subset of design targets described in TR 38.848 can be met or taken into account in feasible designs, and the design targets left to WGs in Clause 5 of TR 38.848 have been refined. Evaluations have also reported the inventory completion time for multiple devices. Corresponding evaluation scenarios, assumptions, and methodologies have been developed.

TP#2

The basic blocks of A-IoT device architectures based on RF-ED receiver for device 1, 2a, and 2b have been identified, and also for receiver architectures based on IF-ED or ZIF for device 2b. For device 2a, the characteristics of blocks representing reflection amplifier(s) and larger frequency shifter have been studied. Further aspects impacting Aspects to consider further regarding whether the large frequency shifter block is feasible, the feasibility, and its necessity or effectiveness of the large frequency shifter block have also been identified.

Purposes for which clocks(s) may be used by the different A-IoT devices 1/2a/2b have been identified, together with descriptions of the corresponding power consumption, and clock accuracy characteristics.

Sub-TP #3.1 v2

Regarding the potential impact of device (un)availability due to energy harvesting, two 'Directions' were found, depending on whether the reader does not, or can, provide information to a device based on which the device may become available/unavailable. Company solutions are reported in Clause 6.2, grouped according to the Direction they apply to.

Sub-TP #3.2 v2

The physical channels studied are PRDCH in R2D, and PDRCH in D2R, for both of which the generation process steps have has been described at high level. The steps include as examples, possible CRC attachment, FEC in D2R, line-coding/square wave multiplication, potential repetition(s) in D2R, small frequency shift in D2R, and modulation or waveform generation. Feasible candidates for the methods involved in the processes each step have also been documented, together with evaluations and pros/cons reported by companies. Information related to R2D reception and D2R scheduling control information have been listed together with ways they could be conveyedtransmitted, are studied may be transmitted to be contained in PRDCH or PDRCH, or in other ways.

The physical signals studied have potential purposes involving start indication, timing acquisition/tracking, calibration/frequency synchronization at the device and the reader, channel or interference estimation at the reader, and indicating the end of a R2D/D2R transmission. Some of those purposes could alternatively be provided by information in a PRDCH/PDRCH.

Sub-TP #3.3 v2

Pros and cons of multiplexing/multiple access options for R2D and D2R links have been studied by company reports. Options and relationships among the resource allocations for messages of the random access procedures have been identified. The needed timing relationships between corresponding transmissions have been captured, for example the minimum time between a R2D transmission and the corresponding D2R transmission following it; and others.

TP #4

Several cCarrier-wave waveforms and their characteristics have been identified, together with cases for how CW transmission could be deployed in terms of the CW node and the spectrum used. Two CW waveforms were studied: a single-tone unmodulated sinusoid; or two unmodulated single-tone sinusoids. Their characteristics were studied and compared in terms of D2R backscattering reception performance and CW interference suppression at the reader, spectrum utilization of the backscattered signal and guard-bands, and relative complexity of CW generation. The effects of frequency hopping and antenna hopping were included.

TP#5

For Topology 2, RAN1 studied resource allocation and/or control of the intermediate UE via the Uu interface via higher-layer signaling, or L1 and higher-layer signaling.

The harmonized air interface design focuses on DT and DO-DTT use cases. The study also assessed whether the harmonized air interface design can address the DO-A use case, and concluded that it cannot. The part(s) which is/are not sufficient for the DO-A use case are summarized in Clause 6.10.

Agreement

The TP below is endorsed for the conclusion of TR38.769.

----- Start of TP -----

RAN1 recommends that in a normative phase, the design should start from the descriptions in this TR.

(1) The following techniques are recommended in the first normative phase:

- PRDCH and PDRCH, as respectively the single physical channels for R2D and D2R

- Modulations of OOK-1 and OOK-4 in R2D

- Multiplexing in time domain for R2D

- Convolutional FEC in D2R and no FEC in R2D

- R2D and D2R signal(s)

(2) At least the following aspects may be considered for inclusion or non-inclusion in a given first or subsequent Release:

- Device 1 and/or Device 2a (with or without large frequency shift) and/or Device 2b (with RF-ED and/or IF-ED and/or ZIF receiver architecture)

- D1T1 and/or D2T2

  - Scenario A1 and/or A2 and/or B and/or C

- Line code(s) Manchester/PIE for R2D, and Manchester / Miller / no line code for D2R

- Multiple access by TDMA and/or FDMA and/or CDMA for D2R; FDMA for device 2b (IF-ED/ZIF receiver) in R2D

- OOK and/or BPSK and/or MSK modulation for D2R

- Carrier-wave waveform 1 and/or 2, and transmission case(s)

- Device (un)availability direction 1 and/or 2

- Proximity determination solution 1 and/or 2

----- End of TP -----

Final summary in R1-2410704.

[Post-119-R19-A-IoT] – Matthew (Huawei)

Email discussion for endorsement of TR 38.769 for submission to RAN#106, from December 2 to 4.

### Evaluation on Ambient IoT in NR

#### Evaluation assumptions and results

Including assumptions on coverage and coexistence evaluations, link budget calculations, and remaining design targets of TR 38.848

R1-2409362	Evaluation assumptions and results for Ambient IoT	Nokia

R1-2409390	Evaluation assumptions and results for Ambient IoT	Ericsson

R1-2409416	Evaluation methodology and assumptions for Ambient IoT	Huawei, HiSilicon

R1-2409511	Discussion on evaluation assumptions and results for Ambient IoT	CMCC

R1-2410629	Discussion on Ambient IoT evaluations	ZTE Corporation, Sanechips	(rev of R1-2409550)

R1-2410630	Considerations for evaluation assumptions and results	Samsung	(rev of R1-2409596)

R1-2409635	Discussion on evaluation assumptions and results for Ambient IoT	Spreadtrum, UNISOC

R1-2409680	Evaluation methodologies assumptions and results for Ambient IoT	vivo

R1-2409757	Discussion on Link Level simulation of A-IoT	Tejas Networks Limited

R1-2409799	On remaining evaluation assumptions and results for AIoT	Apple

R1-2409895	Evaluation assumptions and results for Ambient IoT	Xiaomi

R1-2409940	The evaluation methodology and preliminary results of Ambient IoT	CATT

R1-2410000	Discussion on evaluation assumptions and results for Ambient IoT	China Telecom

R1-2410024	Discussion on evaluation assumptions and results for Ambient IoT devices	FUTUREWEI

R1-2410090	Discussion on evaluation assumptions and results for A-IoT	OPPO

R1-2410178	Discussion on evaluation assumptions and results for Ambient IoT	HONOR

R1-2410207	Discussion on ambient IoT evaluation framework	NEC

R1-2410285	Discussion on Ambient IoT evaluation	LG Electronics

R1-2410312	Evaluation results for Ambient IoT	InterDigital, Inc.

R1-2410351	Link level simulation for Ambient IoT R2D	Panasonic

R1-2410388	Study on evaluation assumptions and results for Ambient IoT	NTT DOCOMO, INC.

R1-2410422	Discussion on evaluation assumptions and results for Ambient IoT	Indian Institute of Tech (M)

R1-2410477	Evaluation Assumptions and Results	Qualcomm Incorporated

R1-2410513	Evaluation assumptions and results for A-IoT	MediaTek Inc.

R1-2410551	Discussion on the evaluation assumptions for Ambient IoT devices	Lenovo

R1-2410658	Discussion on Evaluation assumptions and results	CEWiT	(rev of R1-2410577)

R1-2410590	Evaluation assumption and results for AIoT	IIT Kanpur, Indian Institute of Tech (M)

R1-2410637	FL summary #1 for Ambient IoT evaluation	Moderator (CMCC)

From Tuesday session

Agreement

For Rel-19 Ambient IoT, the target value(s) for applicable maximum distance are refined as follows,

- For device 1 in D1T1:

  - 15m

- For device 2a in D1T1:

  - 25m

- For device 2b in D1T1:

  - 50m

- For device 2a in D2T2:

  - 15m

- For device 2b in D2T2:

  - 40m

R1-2410638	FL summary #2 for Ambient IoT evaluation	Moderator (CMCC)

From Wednesday session

R1-2410827	TP for Clause 7.2.1 of TR38.769 for single device latency	Moderator (CMCC)

Agreement

Capture the TP for single device latency in R1-2410827 for updating section 7.2.1 of TR 38.769.

- Replace company names with ‘[ref]’

- Update the last observation as shown below:

  - 5 sources provide results not including Step D D2R transmission

- Assuming Msg3 size (Device ID) of 96 bits and Step C R2D message size small than or equal to 400 bits,

- For data rate of 1 kbps, 5 sources [R1-9411-10], [R1-9411-11], Samsung, NTT DOCOMO, Spreadtrum provide results, and the single device latency is [196.28 ms ~ 612 ms].

- For data rate of 7 kbps, 5 sources [R1-9411-5], [R1-9411-10], [R1-9411-11], Samsung, NTT DOCOMO provide results, the single device latency is [28.66 ms ~ 87.90 ms].

R1-2410828	TP for Clause 7.2.2 of TR38.769 for multi-device inventory completion time	Moderator (CMCC)

Agreement

Capture the TP for multiple device inventory completion time in R1-2410828 for updating section 7.2.2 of TR 38.769.

- Replace company names with ‘[ref]’

R1-2410826	TP for Clause 7.1.1-7.1.4 of TR38.769 for coverage results	Moderator (CMCC)

Agreement

For coverage evaluation, the text proposal in R1-2410826 is endorsed to update section 7.1.1, 7.1.2, 7.1.3 and 7.1.4 of TR38.769 with the following changes to be made

- Replace company names with ‘[ref]’

- In the observation subsection, change the CW cancellation capability range for D1T1 into (-inf, 130dB), [130-140), [140-150), [150, +inf) dB

- Update Figure 7.1.1.2.1.2-1 to make it clear to show the companies’ s name

- Update Figures in non-coherent D2R LLS (section 7.1.3) to make it clear to show the companies’ name.

- In Tables for link budget for section 7.1.1 and 7.1.2, change Note 15 into “Row indices in the spreadsheet (row 1 corresponds to row 6 in spreadsheet)”.

R1-2410854	TP for Annex Z of TR38.769 for receiver sensitivity	Moderator (CMCC)	(rev of R1-2410825)

Agreement

Capture the TP for receiver sensitivity in R1-2410854 for updating annex A of TR 38.769.

- Replace company names with ‘[ref]’

- Replace company names in the first column in each table with the reference to the RAN1#119 Tdoc

Agreement

Update the [2D] in link budget table as follows,

| [2D] | Receiver Noise Figure (dB) | For RF-ED receiver - 20dB, Device 2 - FFS other values For IF/ZIF receiver - 15dB, Device 2 Other values can be optionally used and reported by companies | For BS as reader - 5dB For intermediate UE as reader - 7dB |
| --- | --- | --- | --- |

Update the note D in link budget table as follows,

Note D:	For device 2 with an RF ED-based receiver on the R2D link, if the receiver sensitivity derived from Budget-Alt2, assuming a noise figure of [X dB]  assuming a noise figure in [2D] from the table, exceeds the receiver sensitivity based on Budget-Alt1, then Budget-Alt2 is applied.

Agreement

Update the [1E3] in link budget table as follows,

| [1E3] | CW2D distance (m) | N/A | For scenarios ‘B’ D1T1-B: - 5m, - 10m, - 20m - CW2D distance is derived assuming CW node is located with the same position as ‘R1’ in ‘A1’ scenario. (See note 1) D2T2-B: - 5m, - 10m, FFS other values For scenarios ‘A1’ and ‘A2’: Calculated (see note 1), (i.e., CW2D distance is calculated by assuming CW2D pathloss = D2R pathloss) Note 1: Only applicable for device 1/2a. Note 2: Companies to report which value(s) are evaluated. |
| --- | --- | --- | --- |

Agreement

Update the [1M] in link budget table as follows,

| [1M] | EIRP (dBm) | Calculated (see Note 1) FFS: any limitation of the EIRP subject to future discussion | Calculated (see Note 1) |
| --- | --- | --- | --- |

Agreement

Update the [2K] in link budget table as follows,

| [2K] | CW cancellation (dB) | N/A | Companies to report for scenario A2/A1/B for BS and intermediate UE. Notes: - Only applicable for device 1/2a - The value provided is for the unmodulated single-tone CW. The impact of a multi-tone CW, e.g., assuming an [X] dB difference, is FFS The value for other cases (e.g., multi-tone CW) can be reported by companies. |
| --- | --- | --- | --- |

Agreement

Update the [3C] in link budget table as follows,

| [3C] | BS selection/macro-diversity gain (dB) | 0 dB FFS: other values are not precluded Note: only applicable for D1T1 | 0 dB FFS: other values are not precluded Note: only applicable for D1T1 |
| --- | --- | --- | --- |

Agreement

Update the [0q] in link budget table as follows,

| [0q] | Sampling frequency | Companies to report the sampling frequency (e.g., 1.92Msps or other feasible values if any) Initial SFO (Sampling Frequency Offset) (Fe): - (M) Randomly select a value from the range of [0.1 ~ 1] *10^4 ppm for device 2, - (M) Randomly select a value from the range of [0.1 ~ 1] * 10^5 ppm for device 1, - (O) Randomly select a value from the range of [0.1 ~ 1] *10^5 ppm for device 2, - FFS: Optionally evaluate a fixed value SFO for device 1 and 2 - Note: For random selection, the value is randomly selected per simulation drop, according to a uniform distribution - Note: Above values are only for sampling purpose. - FFS other values - Note: Above assumptions are only for LLS evaluation purpose only for R2D and D2R. The timing drift ΔT over a time T is modelled as ΔT = ±Fe * T. - Note: Accuracy can be improved after clock calibration for at least device 2. FFS applicable for device 1 - Note: SFO after clock calibration can be applied to Fe. - FFS other models CFO for device 2b: 100 ppm (M) 200 ppm (O) 1000 ppm (O, only as initial CFO) Drift rate of 0.1 or 1 ppm/s. Companies to report which value they used. Note: Above assumptions are for LLS evaluation purpose only |
| --- | --- | --- |

Agreement

Update the [0q] in link budget table as follows,

| [2a1] | Transmission bandwidth | [2a1]-Alt1 (M): DSB X kHz is considered for D2R transmission bandwidth. The value is for two sidebands, i.e., the total transmission bandwidth for DSB is X kHz [2a1]-Alt2: SSB X kHz is considered for D2R transmission bandwidth. The value is for one sideband, i.e., the total transmission bandwidth for SSB is X kHz. Not applicable for at least device 1 For device 2b only, FFS for device 2a. or device 2a with large frequency shift. X = {[15 (M)], [180 (O)]}, other values are not precluded and reported by companies |
| --- | --- | --- |

Agreement

Update Table 4.2.2-2 in TR 38.769 as follows,

Table 4.2.2-2: Assumptions on layout for D1T1 and D2T2

| Parameter | Assumptions for D1T1 | Assumptions for D2T2 |  |
| --- | --- | --- | --- |
| Scenario | InF-DH | InH-office | InF-DL |
| Hall size | 120x60 m | 120 x50 m | 300x150 m |
| Room height | 10 m | 3m | 10 m |
| Sectorization | None |  |  |
| BS deployment / Intermediate UE dropping | 18 BSs on a square lattice with spacing D, located D/2 from the walls. L=120m x W=60m; D=20m BS height = 8 m | L=120m x W=50m; Intermediate UE height = 1.5 m FFS: Intermediate UE dropping Option 1: - Intermediate UEs are assumed to be deployed in a grid with D = e.g. 10 m / 20 m. Option 2: - Intermediate UEs are assumed to be uniformly distributed over the horizontal area. Number of UE is either 1 or 2. | L=300m x W=150m; Intermediate UE height = 1.5 m FFS: Intermediate UE dropping Option 1: - The intermediate UEs are assumed to be deployed in a grid with D = e.g. 10 m / 20 m. Option 2: - Intermediate UEs are assumed to be uniformly distributed over the horizontal area. Number of UE is either 1 or 2. |
| Device distribution | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area | Device Height= 1.5 m A-IoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations Option 1: The devices within the maximum distance, which is obtained by the coverage evaluation, from each intermediate UE are involved in the evaluations. Option 2: The devices within the applicable maximum distance target are involved in the evaluations. | Device Height= 1.5m A-IoT devices drop uniformly distributed over the horizontal area FFS: which devices are involved in the evaluations Option 1: The devices within the maximum distance, which is obtained by the coverage evaluation, from each intermediate UE are involved in the evaluations. Option 2: The devices within the applicable maximum distance target are involved in the evaluations. |
| Device mobility (horizontal plane only) | 3 kph | 3 kph | 3 kph |
| Note: The CW distribution in D1T1-B or D2T2-B is reported by companies |  |  |  |

R1-2410639	FL summary #3 for Ambient IoT evaluation	Moderator (CMCC)

From Thursday session

R1-2410855	TP for Clause 7.1.1-7.1.4 of TR38.769 for coverage results	Moderator (CMCC)	(rev of R1-2410826)

Agreement

For coverage evaluation, the text proposal in R1-2410855 is endorsed to update section 7.1.1, 7.1.2, 7.1.3 and 7.1.4 of TR38.769.

Note: this is an update of the earlier agreement copied below

Agreement

For coverage evaluation, the text proposal in R1-2410826 is endorsed to update section 7.1.1, 7.1.2, 7.1.3 and 7.1.4 of TR38.769 with the following changes to be made

- Replace company names with ‘[ref]’

- In the observation subsection, change the CW cancellation capability range for D1T1 into (-inf, 130dB), [130-140), [140-150), [150, +inf) dB

- Update Figure 7.1.1.2.1.2-1 to make it clear to show the companie’s name

- Update Figures in non-coherent D2R LLS (section 7.1.3) to make it clear to show the companies’ name.

- In Tables for link budget for section 7.1.1 and 7.1.2, change Note 15 into “Row indices in the spreadsheet (row 1 corresponds to row 6 in spreadsheet)”.

R1-2410875	TP for coverage results spreadsheet for TR38.769	Moderator (CMCC)

Agreement

Include the spreadsheet of the coverage evaluation results (R1-2410875) as an attachment of the TR38.769.

Agreement

Capture the following as part of the conclusion sub-section 7.1.

RAN1 has evaluated the coverage for InF-DH NLOS scenarios for D1T1 and InH-Office LOS/InF-DL NLOS scenarios for D2T2, across all devices 1/2a/2b. Coverage evaluation results comparing various R2D Tx power/EIRP value, R2D Rx sensitivity, BLER 1%/10% target, data rate ~1kbps/~5-7kbps, D2R coherent receiver/non-coherent receiver, R2D budget-Alt1 and Alt2, CW cancellation, backscatter loss and modulation schemes, D2R with/without channel coding schemes, etc, have been provided. The observations for coverage evaluation results can be found in Clause 7.1.1.1.2/7.1.1.2.2/7.1.1.3.2/7.1.2.1.2/7.1.2.2.2/7.1.2.3.2.

#### Ambient IoT device architectures

R1-2409358	Discussion on ambient IoT device architectures	TCL

R1-2409363	Ambient IoT device architectures	Nokia

R1-2409391	Ambient IoT device architectures	Ericsson

R1-2409417	Ultra low power device architectures for Ambient IoT	Huawei, HiSilicon

R1-2409512	Discussion on Ambient IoT device architectures	CMCC

R1-2409551	Discussion on Ambient IoT device architectures	ZTE Corporation, Sanechips

R1-2409597	Remaining issues for Ambient-IoT device architectures	Samsung

R1-2409636	Discussion on Ambient IoT device architectures	Spreadtrum, UNISOC, SGITG

R1-2409681	Discussion on Ambient IoT Device architectures	vivo

R1-2409758	Discussion on receiver architecture of A-IoT	Tejas Networks Limited

R1-2409800	On remaining details of device architecture for AIoT	Apple

R1-2409896	Discussion on Ambient IoT device architectures	Xiaomi

R1-2409941	Study of the Ambient IoT devices architecture	CATT

R1-2410025	On remaining open issues in Rel-19 Ambient IoT device architecture	FUTUREWEI

R1-2410091	Discussion on device architecture for A-IoT device	OPPO

R1-2410208	Device architecture requirements for ambient IoT	NEC

R1-2410286	Discussion on Ambient IoT device architectures	LG Electronics

R1-2410313	Device architectures for Ambient IoT	InterDigital, Inc.

R1-2410389	Study on device architectures for Ambient IoT	NTT DOCOMO, INC.

R1-2410423	Discussion on Ambient IoT Device Architectures	Indian Institute of Tech (M), IIT Kanpur

R1-2410478	Ambient IoT Device Architecture	Qualcomm Incorporated

R1-2410502	Analysis for CFO calibration for device 2b	Wiliot Ltd.

R1-2410514	Ambient IoT device architectures	MediaTek Inc.

R1-2410710	FL Summary #1 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

From Wednesday session

Agreement

Update clock purpose #3 as follows.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #3 | Time counting (if supported) | e.g. tens kHz | 1 (if applicable), 2a, 2b | <0.1uW | [10^4 - 10^5] | Calibration may not always be feasible for this clock purpose |

Agreement

Update clock purpose #4 as follows.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #4 | Large frequency shift (if supported for device 2a) | 10s MHz (e.g., 50MHz) | 2a | [10s] uW | [10^3-10^4] | 10^3 (Note 2). Whether better accuracy can be achieved has not been determined. |
| Note2: No drastic temperature change is assumed during calibration. |  |  |  |  |  |  |

Agreement

Adopt following table to update clock purpose #5.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #5 | LO for carrier frequency (for up/down conversion) | According to carrier frequency e.g., 900 MHz | 2b | 10s ~ 100s uW | [10^3 ~ 10^4] | Option 4: within 10s – 200 (Note 2), there is no need to determine a single value for the study |
| Note 2: No drastic temperature change is assumed during calibration. |  |  |  |  |  |  |

R1-2410711	FL Summary #2 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

From Thursday session

Agreement

For clock purpose #1, adopt following table.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #1 | Sampling | a few MHz | 1 (Note 4) | < 1uW |  | Option 1: 10^5 (Note 2) |
|  |  |  |  |  | [10^4 ~ 10^5] | Option 2: 10^4 (Note 2) the study did not determine whether or not this accuracy can be achieved with < 1uW power consumption at least for device 1 |
|  |  |  | 2a, 2b | < 1uW | 10^4 ~ 10^5 | 10^4 (Note 2) |
|  |  |  |  | < 10uW | 10^3 ~ 10^4 | 10^3 (Note 2) |
| Note 2: Calibration is based on signal from reader. No drastic temperature change is assumed during calibration. Note 3: Accuracy after clock sync / calibration is better than initial clock accuracy. Note 4: Device 2a, 2b could also use similar implementation |  |  |  |  |  |  |

Observation

Several sources [Samsung, Docomo, Oppo, InterDigital, Qualcomm, ZTE] think that for clock purpose #1 (sampling), accuracy after clock sync / calibration at device side of 104 ppm is achievable with < 1uW power consumption for device 1, and that 105 ppm may not be sufficient for certain designs.

R1-2410712	FL Summary #3 for 9.4.1.2 Ambient IoT Device Architecture	Moderator (Qualcomm)

From Thursday session

Agreement

Update the earlier observation as below

Observation

Several sources [Samsung R1-2409597, Docomo R1-2410389, Oppo R1-2410091, InterDigital R1-2410313, Qualcomm R1-2410478, ZTE R1-2409551] think that for clock purpose #1 (sampling), accuracy after clock sync / calibration at device side of 104 ppm is achievable with < 1uW power consumption for device 1, and that 105 ppm may not be sufficient for certain designs.

- Source [Samsung] R1-2409597 mentioned that device 1 can support basic calibration operation within a reasonable power consumption or complexity, achieving accuracy of 10^4 PPM.

- Source [DCM] R1-2410389 mentioned that w/ clock calibration based on CAP design, 10^4 could be achieved after clock calibration.

- Source [Oppo] R1-2410091 reported that clock purpose #1 could be applied to all device types. But, considering complexity and power, device 1 and 2 could have different power, accuracy. For Device1, the power consumption should be < 1 uW; the timing accuracy should be 10^4 ~ 10^5; the calibration can reach to 10^4.

- Source [InterDigital] R1-2410313 proposed device 1 with accuracy of 10^4 after clock sync/calibration.

- Source [QC] R1-2410478] reported that using simple clock counting based calibration method, 1% could be achieved with power consumption <1uW. It was reported that 1% is the sweet spot of target accuracy considering performance, complexity, signal design, etc.

- Source [ZTE] R1-2409551 mentioned that device can use recommended clock calibration method to get high accuracy of 103~104 ppm for device 1 and 102~103 ppm for device 2.

Agreement

For clock purpose #2, adopt following table.

| # | Purpose | Clock speed | Applicable Device types | Power consumption | Initial clock Accuracy (ppm) | Accuracy after clock sync / calibration at device side (ppm) |
| --- | --- | --- | --- | --- | --- | --- |
| #2 | Small frequency shift | a few MHz | 1 (Note 4) | < 1uW |  | Option 1: 10^5 (Note 2) |
|  |  |  |  |  |  | Option 2: 10^4 (Note 2) the study did not determine whether or not this accuracy can be achieved with < 1uW power consumption at least for device 1 |
|  |  |  | 2a, 2b (if applicable) | < 10uW |  | 10^3 (Note 2) |
| Note 2: Calibration is based on signal from reader. No drastic temperature change is assumed during calibration. Note 4: Device 2a, 2b (if applicable) could also use similar implementation |  |  |  |  |  |  |

Observation

For A-IoT device clock, 9 sources [TCL] R1-2409358, [ZTE] R1-2409551, [HW] R1-2409417, [QC] R1-2410478, [Samsung] R1-2409597, [CATT] R1-2409941, [Vivo] R1-2409681, [DCM] R1-2410389, [MTK] R1-2410514 have reported that clock calibration based on at least clock counting for a known duration is feasible for A-IoT devices.

Final summary in R1-2410713.

### Physical layer design for Ambient IoT

#### General aspects of physical layer design

Including numerologies, bandwidths, multiple access, waveform, modulation, and coding

R1-2409359	Discussion on general aspects of physical layer design for Ambient IoT	TCL

R1-2409364	General aspects of physical layer design for Ambient IoT	Nokia

R1-2409388	General aspects of physical layer design for Ambient IoT	Ericsson

R1-2409418	On general aspects of physical layer design for Ambient IoT	Huawei, HiSilicon

R1-2409513	Discussion on general aspects of A-IoT physical layer design	CMCC

R1-2409535	Discussion on Physical Layer Design for Ambient-IoT	EURECOM

R1-2409552	Discussion on general aspects of physical layer design for Ambient IoT	ZTE Corporation, Sanechips

R1-2409598	Considerations for general aspects of Ambient IoT	Samsung

R1-2409637	Discussion on general aspects of physical layer design for Ambient IoT	Spreadtrum, UNISOC

R1-2409682	Discussion on General Aspects of Physical Layer Design	vivo

R1-2409801	On remaining general physical layer design aspects for AIoT	Apple

R1-2409864	Discussion on general aspects of ambient IoT physical layer design	NEC

R1-2409897	Discussion on physical layer design of Ambient IoT	Xiaomi

R1-2409942	Discussion on general aspects of physical layer design	CATT

R1-2409976	On General Physical Layer Design Considerations for Ambient IoT Applications	Lekha Wireless Solutions

R1-2410001	Discussion on general aspects of physical layer design for Ambient IoT	China Telecom

R1-2410026	On remaining open issues in Rel-19 Ambient IoT physical layer design	FUTUREWEI

R1-2410059	Discussions on FEC/repetition in R2D and D2R	Fujitsu

R1-2410092	Discussion on general aspects of physical layer design of A-IoT communication	OPPO

R1-2410225	Physical layer design of Ambient IoT	Sony

R1-2410267	Discussion on general aspects of physical layer design	ETRI

R1-2410287	General aspects of Ambient IoT physical layer design	LG Electronics

R1-2410311	Discussion on physical layer design for Ambient IoT	InterDigital, Inc.

R1-2410352	General aspects of physical layer design for Ambient IoT	Panasonic

R1-2410372	Discussion on A-IoT physical layer design	ASUSTeK

R1-2410390	Study on general aspects of physical layer design for Ambient IoT	NTT DOCOMO, INC.

R1-2410416	Discussion on general aspects of physical layer design	Sharp

R1-2410479	General aspects of physical layer design	Qualcomm Incorporated

R1-2410515	General aspects of physical layer design	MediaTek Inc.

R1-2410552	Discussion on the physical layer design aspects for Ambient IoT devices	Lenovo

R1-2410591	Discussion on General aspects of physical layer design of AIoT	IIT Kanpur, Indian Institute of Tech (M)

R1-2410700	Feature Lead Summary #1 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

From Tuesday session

Agreement

Capture in the TR clause on “D2R waveform and modulation”:

- Device 2b internally generates carrier wave with only a single tone. Device 2b does not use a ‘2 single tone’ carrier wave.

Agreement

For device 2b, FDMA in D2R can be achieved by direct modulation of its internally generated carrier wave at the desired frequency.

Agreement

Capture the following TP update into TR 38.769, where source IITM will be added where relevant.

…(unchanged parts omitted)… For Alt M1-1, the potential impacts are discussed as follows: - Some sources [R1-9421-1], [R1-9421-5], [R1-9421-6], [R1-9421-8], [R1-9421-25], [R1-9421-28], [IITM] report that CP handling of Alt M1-1 can be used for both small and large M values for OOK-4, while [R1-9421-8] reports that for large M values Alt M1-1 is used in combination with Alt M1-2. - Some sources [R1-9421-3], [R1-9421-32] report that CP handling of Alt M1-1 is challenging to be used for large M values for OOK-4 considering large SFO and [R1-9421-8], [R1-9421-18] report that CP handling of Alt M1-1 may not completely remove CP samples due SFO impact. - Among of them, [R1-9421-5] show that the performance loss of PRDCH carrying 20 bits due CP handling is negligible at 10% BLER even for large M values (e.g. M=24) under large SFO (e.g. 104-105 ppm). Sources [R1-9421-8], [R1-9421-32] show some performance loss due CP handling for both small (M=4) and large M values (M=24) under large SFO (e.g. 104-105 ppm ) while [R1-9421-32] shows [1~2 dB] loss compare to no CP case for M<24, and an error floor at BLER=10% for M=24. - Some sources [R1-9421-9], [R1-9421-18] report that the device needs additional complexity to handle CP, while other sources [R1-9421-5], [R1-9421-25] reports that it is feasible in terms of implementation complexity based on transition edge detection. - One source [CATT] report that the device might remove the wrong portion of the CP part of the OFDM symbol due to timing error, which could introduce the false rising/falling edge for the subsequent OOK demodulation. For Alt M1-2, the potential impacts are discussed as follows: - Some sources [R1-9421-1], [R1-9421-4], [R1-9421-9], [R1-9421-3], [R1-9421-5], [R1-9421-6], [R1-9421-8], [R1-9421-32], [R1-9421-18], [R1-9421-25], [R1-9421-27], [CATT] report that CP handling of Alt M1-2 cannot be used for large M values, e.g. M>8, while [R1-9421-8] reports that for large M values Alt M1-2 is used in combination with Alt M1-1. - One source [R1-9421-22] report that CP handling of Alt M1-2 can be used for both small and large M values (e.g. M>8) if with the knowledge of OFDM symbol boundaries. - Among of them, [R1-9421-8] show that the performance of Alt M1-2 is not applicable for large M values (e.g. M=24) under large SFO (e.g. 104 ppm). For Method Type 2, two approaches regarding subcarrier orthogonality are studied: - Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol. - Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same. - Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. - Alt M2-2: Method Type 2 does not retain subcarrier orthogonality. For Method Type 2, depending on the design, the chip duration generation of OOK-4 for M-chip per OFDM symbol transmission could possibly be determined by: - M, and the length of OFDM symbol with CP - M, and the length of OFDM symbol without CP - Depending on detailed solutions, chip duration may or may not be constant. - One Some sources [R1-9421-28][Huawei] report that non-constant OOK chip duration may impact performance, while some other source [R1-9421-32] report that non-constant OOK chip duration does not impact performance. For Alt M2-1, the potential impacts are discussed as follows, - Some sources [R1-9421-5], [R1-9421-9], [R1-9421-8], [R1-9421-33], [R1-9421-21], [R1-9421-11], [R1-9421-18], [R1-9421-3], [Sony] report that CP handling of Alt M2-1 cannot be used for large M values (e.g. M>8). Source [Ericsson] report that for M>8, the CP size becomes comparable to that of the normal OOK chip, and hence it would be challenging to identify the invalid transition caused by CP. Sources [CATT] report that if chip duration is comparable to CP duration, CP could not be identified as the invalid chip by the A-IoT device, e.g., M>8. - Some sources [R1-9421-1], [R1-9421-6], [R1-9421-28], [R1-9421-32] report that CP handling of Alt M2-1 can be used for both small and large M values. - Among of them, some sources [R1-9421-6], [R1-9421-32] show the performance of Alt M2-1 for small (M=4) and large M values (M=24) under large SFO (e.g. 105 ppm). - Some sources [R1-9421-28], [R1-9421-9], [R1-9421-32] report that CP handling of Alt M2-1 may result in non-constant OOK chip duration around CP. Source [Huawei] report that due non-constant OOK chip duration around CP, Alt M2-1 has ~1dB worse performance than Alt M1-1 at BLER 10% and BLER 1% when it used for small M value (e.g., M = 6). - Some sources [R1-9421-3], [R1-9421-5], [R1-9421-11], [R1-9421-32], [R1-9421-22], [R1-9421-4], [R1-9421-27] report that CP handling of Alt M2-1-1 would increase the overhead and reduce spectral efficiency. - Some sources [R1-9421-25], [R1-9421-1], [R1-9421-9] report that CP handling of Alt M2-1-1 may not be completely transparent to the device thus add additional complexity. - Source [CATT] report that if chip duration is significantly different from CP length, M2-1-2 would be complicated to be used for removing false transition edge occurring at the end of the CP. And M2-1-2 would require high complexity of A-IoT device implementation if it is used for the R2D preamble. For Alt M2-2, the solutions and potential impacts are discussed as follows, - [R1-9421-8], [R1-9421-12], [R1-9421-11], [R1-9421-21], [Panasonic] report solutions for Alt M2-2 (e.g. CP is copied from the start of OFDM symbol or do not insert CP to OFDM symbol). - [R1-9421-3], [R1-9421-5], [R1-9421-6], [R1-9421-10], [R1-9421-32], [R1-9421-25], [R1-9421-28], [R1-9421-4], [R1-9421-9], [R1-9421-22], [R1-9421-27] report that CP handling of Alt M2-2 would cause interference to NR, while [R1-9421-8] reports single PRB guard band would be sufficient to handle interference. [Panasonic] reports the guard band would anyway be needed when SCS is different between R2D and other NR signal. - Sources [R1-9421-5], [R1-9421-25], [R1-9421-28], [R1-9421-31], [R1-9421-9], [R1-9421-22], [R1-9421-27] report that CP handling of Alt M2-2 would increase the transmitter complexity. …(unchanged parts omitted)…

Agreement

Capture the following TP update into TR 38.769

…(unchanged parts omitted)… Table 6.1.1.x-1 is a starting point for study of M values and the associated minimum Btx,R2D value. The reader can use any transmission bandwidth greater than or equal to the minimum Btx,R2D value. Note: Depending on further study, the maximum value of M may be less than 32. Note: The performance can be better when transmission bandwidth greater than the minimum Btx,R2D, depending on device processing and transmit power constraint. Table 6.1.1.x-1: Starting point for M values and the associated minimum Btx,R2D value M; Minimum Btx,R2D # of PRBs. 1; 1. 2; 1. 4; 1. 6; 1. 8; 2. 12; 2. 16; 2. 24; 23. 32; 34. …(unchanged parts omitted)…

Agreement

For R2D FEC, adopt the TP below in Section 6.1.1.x.1 of TR 38.769:

***unchanged parts omitted*** 6.1.1.x.1 Channel coding and CRC PRDCH without FEC is studied as the baseline, with evaluations performed by comparison to this baseline. The study assumes PRDCH can attach a CRC, where the baseline design is using a 6-bit or 16-bit CRC with polynomials as per TS 38.212 [R1-3]. A baseline of no CRC attachment is also included. For the study of CRC designs, see Clause 6.1.0.2. Sources [Huawei], [TCL], [Vivo], [ZTE], [Samsung], [Futurewei][CMCC][Xiaomi][Fujitsu] and [Apple] provide justifications for not having R2D FEC beyond the baseline, with the following observations: - Sources [Huawei], [ZTE], [Futurewei], [Xiaomi] and [Fujitsu] state that FEC decoders require complicated arithmetic or logical operations which are too complicated to be implemented in device 1. - Sources [ZTE] and [Samsung] state that it would be difficult for a device to implement a FEC decoder due to its low power consumption. - Source [Huawei] and [Fujitsu] state that FEC decoder procedures such as the de-interleaving operation or route metric caching require volatile memory of a certain size with a certain reading/writing throughput, which cannot be supported by device 1. - They also mention that the received signal power at the device can be relatively high (e.g., >-60 dBm), making the receiver sensitivity not the bottleneck of the link budget for target coverage, even for device 2b, thus questioning the necessity of R2D FEC. Sources [Ericsson] and [Qualcomm] provide the following justifications for using FEC in R2D for device 2b: - Source [Ericsson] claims that CC with small constraint lengths (e.g., 3 or less) offer a substantial performance gain over uncoded transmission, especially in a fading environment, with reasonable complexity. CC with explicit tail-biting transmission to aid decoding may be suitable for R2D. - Source [QC] claim that even simple block code (e.g., Golay, RM) with hard decisions can significantly reduce the required SNR for achieving a target BLER e.g., 1%. ***unchanged parts omitted***

Agreement

For R2D repetitions, adopt the TP below in Section 6.1.1.x.2 of TR 38.769:

***unchanged parts omitted*** 6.1.1.x.2 Repetition Regarding R2D repetitions, it is reported by sources [R1-9421-11] (only for R2D control, if supported), [R1-9421-12], [R1-9421-32], [R1-9421-13], [R1-9421-21], [R1-9421-19], [R1-9421-28] and [R1-9421-30] that R2D repetitions should be supported. The following are observations regarding the different types of repetition that should be supported. … … ***unchanged parts omitted*** On the other hand, it is reported by sources [Nokia], [Ericsson], [Huawei], [CMCC], [TCL] and [Vivo] that R2D repetitions should not be supported, giving justifications: - Source [Nokia] mention that the transmission power of a R2D transmission is typically much greater than its corresponding D2R transmissions, and if the R2D transmission has coverage issues, then the corresponding D2R transmission would not reach the reader. Hence it should be considered for D2R transmissions alone. - Source [Ericsson], [TCL] say that not supporting R2D repetition can be the baseline. - Source [CMCC], [LG] and [Xiaomi] include that the decision to support R2D repetitions can be based on whether the activation threshold is a bottleneck according to the coverage evaluations. - Source [CMCC][Xiaomi] and [Huawei] also comment that from a device perspective, especially device 1 with low complexity and memory storage, it is not possible to combine multiple repetitions. ***unchanged parts omitted***

Agreement

For R2D repetitions, adopt the TP below in Section 6.1.1.x.2 of TR 38.769:

***unchanged parts omitted*** Bit-level repetition Positive observations: - Source [R1-9421-3] state that bit level repetition can be studied if coverage enhancement of the R2D link is required. - Source [R1-9421-12] state that bit level repetition where every input bit repeated for 8 times before Manchester coding could have ~4dB gain when compared with no repetition. They claim using Manchester codes with repetitions require a simple structure and consumes extremely low power. - Source [R1-9421-28] state that bit level repetitions with scrambling is required since the former would improve the link budget and the latter would add extra randomness to the information bits, providing gain by suppressing the interference. They also claim that repetitions can be used in devices that cannot soft combine the repetitions, and majority-based detection would offer gain for these devices. Negative observations: - Source [R1-9421-9] state that since envelope detection is used for R2D reception, bit level repetition may not provide expected gain for the reception. - Source [R1-9421-8] state that though it may be feasible, it increases the device’s processing complexity for reception, e.g., combination, repetition parameters determination. - Source [Fujitsu] state that repetition gain of a bit-level repetition, which requires additional standardization effort to define necessary control information, mainly comes from the energy accumulation of the signal, and should be similar with the achievable gain by directly lowering the chip rate/reducing the M value, which does not require this additional effort. Block-level repetition Positive observations: - Source [R1-9421-32] state that at least for large TBs, repeatedly transmitting the TB multiple times consecutively provides time diversity gain and increases the probability that at least one of the repetitions can be successfully decoded. - Source [ZTE] further state that the device can perform the block-wise detection without chase combination of the repeated blocks so that block-level repetition may not need additional buffer and increase the complexity and cost. - Source [Fujitsu] state that block-level repetition can obtain a bigger repetition gain than that achieved by bit- or chip-level repetition, and can enjoy both the time diversity gain and the gain of energy accumulation. Negative observations - Source [R1-9421-8] state that considering limited capability and cost for an A-IoT device, block level repetition for R2D should be excluded. - Source [Fujitsu] state that block-level repetition additionally requires a very large volatile memory to store all received repetitions of one block. Chip-level repetition Positive observations: - Source [R1-9421-9] state that it may be useful for R2D transmission coverage and can be considered to generate a lower data rate than 7kbps. - Source [R1-9421-30] state that chip-level repetition increases the chip duration, improving the edge detection at the receiver, thereby having a ~2dB performance increase when compared to bit level repetitions. Negative observations: - Sources [R1-9421-3], [R1-9421-8] and [R1-9421-11] state that chip-level repetition is equivalent to long chip transmission, i.e., by using a smaller modulation index, and therefore, there is no need to support this option. - Source [Fujitsu] state that repetition gain of a chip-level repetition, which requires additional standardization effort to define necessary control information, mainly comes from the energy accumulation of the signal, and should be similar with the achievable gain by directly lowering the chip rate/reducing the M value, which does not require this additional effort. ***unchanged parts omitted***

Agreement

Add the following TPs to the TR:

6.1.1.x R2D multiplexing For R2D, time-domain multiplexing is the baseline. Code-domain multiplexing is not considered for device 1/2a/2b. Frequency-domain multiplexing is not considered for the devices with an RD-ED receiver (see Clause 5). For device 2b with IF-ED or ZIF receivers, the study considered the following technical aspects: Table 6.1.1.x-1: Observations on the feasibility and necessity of FDM for Device 2b Aspects to be considered for feasibility/benefit; Observations. Inventory completion time; Sources [R1-9421-3], [R1-9421-22], [R1-9421-4], [R1-9421-12] state that FDM is beneficial to reduce the inventory completion time, especially considering more devices per reader due to the larger maximum distance for Device 2b. Source [vivo] state that inventory latency reduction would be limited, due to difficulty of allocating frequency resources efficiently for Msg2 by reader with uncertainty of number of successful Msg 1, and difficulty of informing an A-IoT device R2D frequency location other than Msg2.. Device implementation; Sources [R1-9421-18], [R1-9421-27], [R1-9421-3], [R1-9421-22], [R1-9421-12] state that channel selection may be performed by a narrowband filter (IF filter or BB filter) after the mixer for Device 2b, if the LO accuracy is sufficiently good. Source [Panasonic] states that narrowband RF filtering at device side to realize R2D FDM would be challenging considering reception performance and complexity, while such filtering would also limit the deployment scenario supported by device. Sources [R1-9421-34], [R1-9421-24], [R1-9421-6], [R1-9421-2], [R1-9421-5] state that it would be challenging for a device using an RF-ED receiver architecture to distinguish the different incoming signal fall into the RF BW without narrowband RF filtering which may cause increasing device implementation complexity and power consumption. Source [R1-9421-3] states that the larger R2D responses are harder for the devices to process in the case of TDM+FDM/TDM only for D2R/R2D, respectively.. Spectrum utilization; Source [R1-9421-34] state that the spectral efficiency may be impacted by the guard band across the FDMed R2D transmissions to multiple devices. Source [Ericsson] state that the spectrum utilization can still be higher for non-RF-ED based devices if FDM is used, despite guard bands. Source [vivo] state that spectrum efficiency improvement would be limited, due to difficulty of allocating frequency resources efficiently for Msg2 by reader with uncertainty of number of successful Msg 1, and difficulty of informing an A-IoT device R2D frequency location other than Msg2.. Coverage (in the case of single reader); Source [R1-9421-5] states the R2D link budget of a reader is decreased due to the power splitting between the parallel R2D channels. Source [R1-9421-3] states the coverage target of Device 2b is still larger than that of Device 1 (with RF-ED architecture).. Reader implementation (in the case of single reader); Source [R1-9421-5] states that additional interference suppression may be needed to deal with the intermodulation between the parallel R2D transmissions.. Harmonized design for all devices; Sources [R1-9421-26], [R1-9421-9], [R1-9421-19], [Spreadtrum] state that it is not appropriate to include FDM only for Device 2b, while Device 1 and 2a cannot support it. Source [vivo] state that non-harmonized resource allocation for different device types complicates the system design. Source [Ericsson] state that a deployment supporting a combination of RF-ED devices and non-RF ED devices can be harmonized by TDMA’ing different types of R2D time slots, where some time slots can support only a single frequency occasion while other time slots support multiple frequency occasions..

Agreement

In R2D, a chip corresponds to one OOK symbol.

Conclusion

Since R2D chip duration is a consequence of CP handling design, it is not studied further in RAN1, and is left to a later phase.

Agreement

Capture the following TP update into TR38.769

***unchanged parts omitted*** For all devices, the following D2R baseband modulations are studied: - OOK - Binary PSK - Binary FSK, as MSK (and not GMSK) OOK and BPSK for baseband modulation are feasible for D2R for all devices. - Sources [R1-9421-3], [R1-9421-11], [R1-9421-28], [R1-9421-16] report that MSK is feasible in some way: - [R1-9421-3], [R1-9421-11] say it is feasible for all devices, for example when it is implemented with multiple impedances switching - [R1-9421-28] say that it would be implemented as square-wave MSK for devices 1 and 2a, and sine-wave MSK for device 2b - For device 1 and 2a this type of MSK does not have continuous phase - [R1-9421-3] say that benefits include lower sidelobes than OOK and BPSK, and lower BER than OOK and same BER as BPSK - Sources [R1-9421-5], [R1-9421-2], [R1-9421-9], [R1-9421-7], [R1-9421-8], [R1-9421-10], [R1-9421-23] report that MSK is either infeasible or should be deprioritized for all devices. - [R1-9421-5], [R1-9421-9], [R1-9421-7], [R1-9421-8], [R1-9421-2], [R1-9421-10], [R1-9421-23] say that MSK is less spectrally efficient than OOK and BPSK because there are issues due to poor phase accuracy in the device - [R1-9421-5], [R1-9421-7], [R1-9421-2], [R1-9421-8], [R1-9421-10] say that MSK would increase reader and device complexity - [R1-9421-8] say that MSK performance for device 2b would materially degrade due to CFO - [TCL] say that it is difficult to modulate MSK signal using impedance switching due to the implementation complexity, including frequency mapping and phase continuation. [Xiaomi] say that if multiple impedances switching are applied to maintain the phase continuity, it violates the principle of low device cost. ***unchanged parts omitted***

Agreement

For D2R line codes, FM0 is deprioritized.

Agreement

For small frequency shifts in D2R, adopt the TP below in Section 6.1.2.x.1 of TR 38.769:

6.1.2.x.1 Small frequency shifts ***unchanged parts omitted*** Sources [R1-9421-1], [R1-9421-8], [Huawei] and [R1-9421-28] state that Manchester codeword repetitions within the same time duration corresponding to an information bit is equivalent to bit-level repetitions within the same duration prior to Manchester encoding. Sources [CMCC] and [Vivo] state that option 1 has a more concentrated spectrum, and requires lesser bandwidth as compared to Option 2. Source [Vivo] further states that while Option 1 and option 2 show similar BLER performance for single device case, Option 1 outperforms Option 2 with FDMA, especially with presence of 105 ppm SFO. Option 1 can achieve additional gain for coverage evaluation due to lower effective noise power. Sources [R1-9421-8], [R1-9421-32] and [R1-9421-13] state that the output waveform for Manchester line codes by Option 2 introduces a phase reversal of the output waveform in the middle of the time duration corresponding to an information bit as compared to Option 1. ***unchanged parts omitted***

Agreement

For small frequency shifts in D2R using Manchester line codes by multiplying the Manchester codeword with a square wave corresponding to the small frequency-shift, the time duration Tb corresponding to each information bit includes Rs number of square wave periods, where Rs = Tb/(2 * chip length), such that the amount of small frequency shift in Hz is Rs/Tb = 1/(2 * chip length).

Agreement

For small frequency shifts in D2R using a square-wave corresponding to the small frequency-shift and no line codes, the time duration Tb corresponding to each information bit includes R number of square wave periods generated by 2R OOK chips [0, 1, 0, 1 …]/[1, 0, 1, 0 ..] or BPSK chips [-1, +1, -1, +1, …]/[+1, -1, +1, -1, …], such that the amount of small frequency shift in Hz is R/Tb.

Agreement

For D2R block level repetitions, adopt the TP below in Section 6.1.2.x.3 of TR 38.769:

***unchanged parts omitted*** 6.1.2.x.3 Repetition For definitions of repetition types, see Clause 6.1.0. For D2R, at least block-level and bit-level repetition type 1 and type 2 are studied. Block-level repetition ***unchanged parts omitted*** Performance comparisons - Source [R1-9421-5] state that block level repetition yields ~2.5 dB performance gain compared with bit level type 2 due to the additional time diversity gain for the combination of decoding. - Source [R1-9421-10] state that block level repetition provides ~4dB performance gain @1% BLER compared with bit level type 1. - Source [R1-9421-32] state that block level repetition provides ~6dB performance gain @10% BLER compared with no repetitions and the performance between block level repetition and bit level repetition type 2 is the same. - Source [R1-9421-8] state that the performance difference between block level repetition and bit level repetition without CW hopping is minor, while block level repetition outperforms bit level repetition with CW hopping. - Sources [R1-9421-11] and [R1-9421-32] state that bit level repetition and block level repetition have similar performance in the AWGN channel but block level repetition could achieve more time diversity gain than that of bit level type 2 in a fading channel. - Source [Xiaomi] state that for the no FEC case, with 3 times repetition, block level repetition provides ~5dB gain at 1% BLER when compared with bit level type 1 repetition. ***unchanged parts omitted***

Agreement

For D2R FEC, update Table 6.1.2.x.1-1 of Section 6.1.2.x.1 of TR 38.769 as follows:

***unchanged parts omitted*** Table 6.1.2.x.1-1: Summary of study on D2R FEC Option #; CC Design; Pros; Cons. Baseline; Constraint length 7 Code rate 1/3; [R1-9421-8] Decoding performance is increased by ~3dB@10% BLER, when compared to no CC but with repetitions. [R1-9421-8] Decoding performance is increased by ~7dB@10% BLER, when compared to no CC or repetitions. [R1-9421-27] Decoding performance is increased by 6.23dB@10% BLER with 2RX, when compared to no CC or repetitions. [R1-9421-27] Decoding performance is increased by 6.42dB@10% BLER with 4RX, when compared to no CC or repetitions. [R1-9421-11] Decoding performance is increased by ~2dB@10% BLER, when compared to LTE CC-TBCC with code rate 1/2. [R1-9421-16] Decoding performance is increased by ~2.5dB@1% BER, when compared to code rate 1/2. [R1-9421-10] Decoding performance is increased by ~1.5dB@1% BLER, when compared to constraint length 4, code rate 1/3 [R1-9421-10] Decoding performance is increased by ~2.5dB@1% BLER, when compared to constraint length 6, code rate 1/3 [Nokia] Decoding performance is increased by 3 dB@ 10% BLER with 2 RX, when compared to no CC or repetitions. 1; Constraint length 4 Code rate 1/2 – 1/4; [R1-9421-3] Code rate 1/2: Detection performance is increased by 3dB@10% BLER, when compared to no CC or line codes.; [R1-9421-9] Code rate 1/2: Decoding performance is decreased by ~0.86dB@10% BLER, when compared to constraint length 7, code rate 1/2. [R1-9421-32] Code rate 1/2: Decoding performance is decreased by ~1dB@10% BLER, when compared to constraint length 7, code rate 1/2. [R1-9421-32] Code rate 1/4: Decoding performance is decreased by ~1.4dB@10% BLER, when compared to constraint length 7, code rate 1/4. [CATT] Code rate 1/2, 1/3, TBCC: Decoding performance is decreased by ~1dB@10% BLER, when compared to baseline with TBCC.. ***unchanged parts omitted***

Agreement

Update section 6.1.2.x.1 of the TR on D2R multiple access:

D2R multiple access 6.1.2.x.1 Multiple access schemes Time-domain multiple access, and frequency domain multiple access at least by using a small frequency shift in baseband are studied. Whether code-domain multiple access is feasible and necessary for all devices is FFS. Time-domain multiple access is the baseline. Sources [R1-9421-9], [R1-9421-11], [R1-9421-3], [R1-9421-1], [Nokia], state that the benefit of TDMA is the low implementation complexity for both device and reader, while the inventory efficiency may be relatively low for TDMA only, and sources [R1-9421-27], [R1-9421-22], [R1-9421-24], [CATT], [Nokia], [Qualcomm], state that the guard interval, if supported, between consecutive D2R transmissions from different devices depends on the SFO after clock calibration. According to sources [R1-9421-3], [R1-9421-9], [R1-9421-11], [R1-9421-35], [R1-9421-27], [R1-9421-1], [R1-9421-5], [R1-9421-25], [Nokia], the potential benefit of frequency-domain multiple access is to increase the transmission efficiency and reduce collisions, while the cons include more complicated frequency resource management and reception processing at reader according to source [R1-9421-9], and potentially increased power consumption for devices according to sources [R1-9421-11], [R1-9421-17], [R1-9421-12], [Nokia]. It is observed that the performance of FDMA may be impacted by the following aspects. - Large SFO of device - Sources [R1-9421-28], [R1-9421-32] state that large SFO (e.g. up to 105 ppm) produces higher BLER degradation due to inter-device interference than a smaller (e.g. up to 104 ppm) or the ideal case of zero SFO. Source [Qualcomm] states that under the case of the large SFO (e.g. up to 105 ppm), two among four devices using small frequency shifts have BLER floor and cannot achieve BLER 1%. Source [R1-9421-32] state that under the case of the large SFO (e.g. up to 105 ppm), two FDMA-ed devices induce about 0.6~1dB performance loss compared to single device. - Source [R1-9421-5] state that the large SFO (e.g. up to 105 ppm) has little impact (e.g., ≤1dB) on the performance of FDMA between multiple devices. - Sources [R1-9421-8] think that sufficient gap between D2R transmissions should be reserved to accommodate frequency error caused by SFO/CFO - Sources [R1-9421-17] think that the required guard band size increases for higher switching frequencies for passive devices. - Source [CMCC] observed that the performance gap among 10% SFO and 1% SFO (residual) is similar compared FDMA with no FDMA, e.g., about 0.5dB @ 10% BLER, i.e., the performance gap among 10% SFO and 1% SFO (residual) is irrelevant to whether FDMA scheme is used or not. - Source [Huawei] state that large SFO (e.g. up to 105 ppm) has little impact on the link performance of FDMA, as it is observed that the performance gap between 10% and 1% SFO is negligible in the case of FDMA among 4 devices. - Timing offset between devices - Sources [R1-9421-25] state that timing offset results in a degradation of up to ~4 dB and the loss varies for different devices depending on the level of experienced interference - Maximum range of small frequency shift - Sources [R1-9421-24] think that the frequency gap among devices will impact the interference, which is highly depends on the small frequency shift capability, i.e., how large the frequency shift can be via small frequency shift. - Harmonics in the backscattered signal - Sources [R1-9421-8] state that FDMA is feasible by proper frequency resource allocation not using odd harmonic frequency of FDMed D2R transmissions. - Potential intermodulation spectral leakage in the backscattered signal - Frequency resource collision - Source [Sony] thinks that if the guard band size between D2R transmissions is fixed, allocating passive devices with large SFO to frequency shifts closer to the A-IoT carrier frequency and either (1) passive devices with smaller SFO or (2) active devices to frequency shifts further from the A-IoT carrier frequency reduces frequency resource collision. - Number of multiplexed devices - Source [R1-9421-32] reports that performance loss increases with the increase of device number. Besides, for FDMA detection at reader side, there is about 1.5 - 3dB performance loss from 6 FDMA-ed devices compared to single device. - Source [Sony] thinks that the potential number of multiplexed devices depends in the maximum rate of frequency switching. - Source [Sony] thinks that if the guard band size depends on the SFO capability of the device, the number of multiplexed devices can be increased if passive devices with large SFO are allocated frequency shifts closer to the A-IoT carrier frequency and passive / active devices with smaller SFO are allocated frequency shifts further from the A-IoT carrier frequency. According to sources [R1-9421-11], [IIT], [R1-9421-12], [R1-9421-32], [Nokia], CDMA can improve the resource utilization efficiency without increasing the device complexity significantly. Sources [R1-9421-27] thinks CDMA would help the multiplexing among readers and it can alleviate the cross-link interference. Source [R1-9421-12] thinks CDMA is also beneficial for the latency reduction and success rate improvement of access procedure. Source [CATT] thinks that the CDMA scheme is mostly used for the signals without carrying information. However, sources [R1-9421-18], [R1-9421-26], [R1-9421-3], [R1-9421-1], [R1-9421-5], [R1-9421-6], [R1-9421-8], [Nokia] show concerns on the necessity and feasibility of CDMA, especially considering the limited capability (e.g., large SFO/CFO) of Ambient IoT devices and the cost (additional memories to store a set of codewords at device) versus benefits. In detail, the observations are as follows. Impact of SFO on the performance of CDMA In the case of large SFO (e.g., 105 ppm): - Source [R1-9421-6] think that the orthogonality between different codes/sequences will be severely disrupted, as the large SFO will accumulate an additional sampling error of 10 points of 100 points. - Source [R1-9421-8] think that the increased inter-device interference would materially degrade D2R performance, e.g., increased false alarm rate and miss detection probability, which in turn reduce spectrum efficiency even lower than the case of simple TDMA. - Source [R1-9421-1] think that the accurate timing and power control required by CDMA are far-fetched for Ambient IoT devices, referring to the IS-95 CDMA system. - Source [R1-9421-32] state that CDM-ed MA has comparable or better performance than FDM-ed MA under different SFO assumptions (i.e., 0/1E3~1E4/1E4~1E5) and device numbers (i.e., 1/2/3). Source [R1-9421-32] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 4-length orthogonal code is feasible for D2R transmission carrying 20 information bits. - Source [ZTE] state that convolutional codes and BPSK modulation for the CDMA improve the D2R transmission performance with multiple multiplexing devices. Source [ZTE] state that for D2R transmission with TBS16+CRC0, the performance difference between the CDMA scheme using 4-length orthogonal code for 4 devices and the repetition scheme for single device at 1%BLER is less than 0.5dB, and the CDMA scheme has 1~2dB SNR gain at 1%BLER compared with FDMA scheme at the same multiplexing device numbers. While for D2R transmission with TBS96+CRC16, the performance difference between the CDMA scheme with 4-length orthogonal code for 4 devices and the repetition scheme for single device at 1%BLER is also less than 0.5dB, and the CDMA scheme has 1~2.5dB SNR gain at 10%BLER compared with FDMA scheme at the same multiplexing device numbers. - Source [Qualcomm R1-9421-28] state that CDMA for msg-1 enables multiplexing large number of msg-1 sequences even when power variation is [-9dB, +9dB], while quite large number of SFO hypotheses is necessary at reader to achieve reasonable false-alarm and miss detection probabilities with the SFO of [0.1 – 1] * 105 ppm. - Source [R1-9421-5] state that correlation properties of sequences are severely damaged with the SFO of 105 ppm. Besides, D2R receiver fails to estimate the SFO of each of the parallel D2R transmissions for the SFO of 105 ppm. - Source [R1-9421-12] state that CDM of RACH preambles using either m-sequences or Gold sequences of length 63 is feasible and preambles from multiple devices can be clearly detected by the reader, even in challenging conditions (SFO = 5%, SNR = 0dB). For 1% missed-detection rate, simulation results showed that m-sequences and Gold sequences are able to achieve this performance level when SNR is about -24dB and -23dB, respectively. In the case of relatively smaller SFO (e.g., 104 ppm), - Source [Qualcomm R1-9421-28] state that CDMA for msg-1 enables multiplexing large number of msg-1 sequences even when power variation is [-12dB, +12dB] when power variation is within [-3, +3] dB for the SFO of [0.1 – 1] * 104 ppm with reasonable number of SFO hypotheses at reader. Impact of CFO on the performance of CDMA for Device 2b - Source [R1-9421-5] state that codeword detection at D2R receiver fails considering non-coherent demodulation has to be used due to the quick phase rotation caused by the residual CFO of e.g. 10s or 100s of Hz after CFO estimation and correction. Impact of timing offset between CDMed D2R transmissions on the performance of CDMA for Device 2b - Source [R1-9421-31] state that binary modulated orthogonal sequence such as Golay sequence can tolerate timing error by selecting a suitable cyclic shift spacing. - Source [R1-9421-12] state that it is possible to detect multiple transmitters with timing difference and power difference among devices. - Source [R1-9421-24] think that poor synchronization performance in time and frequency domain of device would degrade the code orthogonality and thus results in a bad cross-correlation performance. - Source [R1-9421-21] think that the different propagation delays from devices may also degrades decoding performance. - Source [R1-9421-32] state that the negative impact of asynchronization can be mitigated with some enhancements, e.g. enhanced synchronization sequence and enhanced detection method at reader/BS side, e.g., sliding window based detection and setting constraints on the start of D2R transmission. The impact of power variation between devices on the performance of CDMA is studied as follows. - Source [Qualcomm R1-9421-28] state that the larger power variation of at least up to [-9dB, +9dB] can be addressed by reader receiver and CDMA can achieve significant throughput gain for msg-1 severely degrades the capacity of CDMA for msg-1. - Source [R1-9421-32] state that the greater the disparity in received power among multiple devices, the better performance will be obtained by SIC receiver with CDM-based multiple access scheme. Except the impact of SFO/CFO of devices, whether CDMA provides benefit is also studied as depending on the length of the orthogonal or pseudo-orthogonal code and the number of available codes for parallel D2R transmissions: - Source [R1-9421-3] think that using spreading sequence can lead to transmitting a larger number of bits which can be extremely inefficient considering that the devices are extremely power inefficient. - Source [R1-9421-3] think that CDMA might be too complex to implement in A-IoT devices, which might involve complexities with generating orthogonal sequences. - Source [R1-9421-24] think that a large device density (e.g., 150 devices per 100 m2 for indoor scenarios per TR) requires a long code sequence, which is challenging for the device with limited buffer size. - Source [R1-9421-2] think that CDMA leads to higher power consumption and lower data rate. - Source [R1-9421-8] think that the usable number of binary sequences would be much smaller due to impairment such as timing/frequency error and interference. - Source [R1-9421-12] state that in comparison to RN16, when Msg. 1 is transmitted using RACH preamble m-sequences or Gold sequences, the number of usable binary sequences that can be used is large since the base sequence design from LTE and NR can be reused. - Source [R1-9421-12] state RN16 cannot tolerate collision for any one of its bits. Once collided, the bit sequence is changed and became non-detectable. On the other hand, m-sequences and Gold sequences are able to tolerate transmission overlap. - Source [R1-9421-32] state that CDMA by mapping Manchester encoded bit or convolutional encoded bit with 16-length or 64-length orthogonal code improve the D2R transmission performance and multiplexing capacity compared with using 4-length orthogonal code for mapping. Source [ZTE] state that for D2R transmission with TBS16+CRC0 under the assumption of SFO being 1E4~1E5, the performance difference between the CDMA scheme with 64-length orthogonal code for 4 devices and the repetition scheme for single device at 1%BLER is about 1dB, and the CDMA scheme has about 2.5dB SNR gain at 1%BLER compared with FDMA scheme at the same multiplexing device numbers. - Source [R1-9421-32] state that BPSK modulation and convolutional code for the CDMA further improve the D2R transmission performance with multiple multiplexing devices and multiplexing capacity compared with OOK based modulation. Source [ZTE] state that for D2R transmission with TBS16+CRC0 under the assumption of SFO being 1E4~1E5 and using convolutional codes and BPSK modulation, the performance difference between the CDMA scheme using 16-length or 64-length orthogonal code for 6 devices and the repetition scheme for single device at 1%BLER is less than 1dB, and the CDMA scheme has about 9dB SNR gain at 1%BLER compared with FDMA scheme at the same multiplexing device numbers.

Conclusion

No further discussion is needed in the SI for the FFS on “the definition of D2R chip duration for MSK” from RAN1#118bis.

Agreement

Capture in the TR:

- “Since device 2b has internal carrier-wave generation, the 2SB and/or 1SB Btx,D2R and Bocc,D2R can be affected by how it performs D2R modulation and baseband waveform generation.”

Agreement

For CRC, adopt the TP below in Section 6.1.1.x.1 of TR 38.769:

6.1.0.2 CRC ***unchanged parts omitted*** For the CRC generator polynomials: Sources [R1-9421-7], [R1-9421-12], [R1-9421-10], [R1-9421-11], [R1-9421-21] recommend that the same polynomials from TS 38.212 are reused, giving justifications: - Sources [R1-9421-11], [R1-9421-12] state that the polynomials from TS 38.212 were already carefully and thoroughly evaluated and ensured in the NR channel coding design, so there is no need of considering other polynomials. - Source [R1-9421-10] states that the device complexity is increased when different polynomials are introduced for the D2R transmission. - Source [R1-9421-7] states that link performance is significantly impacted by the CRC lengths, and not the CRC polynomials, hence the polynomials from TS 38.212 should be used. On the other hand, sources [R1-9421-32], [R1-9421-28], [TCL] and [Panasonic] recommend that the same polynomial for CRC-6 gCRC6D=D6+D5+1 from TS 38.212 is reused, but a new polynomial for CRC-16 is introduced, gCRC16D=D16+D11+D6+D5+1, incorporating the CRC-6 polynomial, giving justifications: ***unchanged parts omitted***

Agreement

For CRC, adopt the TP below in Table 6.1.0.2-1 of Section 6.1.1.x.1 of TR 38.769:

***unchanged parts omitted*** Table 6.1.0.2-1: CRC evaluations CRC-6; CRC-16. TBS; Source; CRC overhead; PFA or Pud; Source; CRC overhead; PFA or Pud. 8 bits; [R1-9421-5] [QUALCOMM119]; 43%; ~10^(-8) Pud @10-3 BER; [R1-9421-5] [QUALCOMM119]; 67%; ~10^(-11) Pud @10-3 BER. 12 bits; [R1-9421-3] [QUALCOMM119]; 33% 33%; ~10^(-2) PFA ~10^(-8) Pud @10-3 BER; [R1-9421-3] [QUALCOMM119]; 57% 57%; ~10^(-5) PFA ~10^(-11) Pud @10-3 BLER. 16 bits; [R1-9421-5]; 27%; [R1-9421-5]; 50%. [R1-9421-32] [QUALCOMM119]; 27% 27%; ~10^(-7) Pud @10-3 BER; [R1-9421-32] [QUALCOMM119]; 50% 50%; ~10^(-10) Pud @10-3 BER. 24 bits; [R1-9421-5]; 20%; ~10^(-6) Pud @ 10% BLER and 2.7dB SNR; [R1-9421-5]; 40%; ~10^(-9) Pud @10% BLER and 2.8dB SNR. [R1-9421-3] [R1-9421-32] [QUALCOMM119]; 20% 20% 20%; ~10^(-2) PFA ~10^(-7) Pud @10-3 BER; [R1-9421-3] [QUALCOMM119]; 40% 40%; ~10^(-5) PFA ~10^(-10) Pud @10-3 BLER. 32 bits; [R1-9421-5] [R1-9421-32] [QUALCOMM119]; 16% 16% 16%; ~10^(-5) Pud @10-3 BLER; [R1-9421-5] [R1-9421-32] [QUALCOMM119]; 33% 33% 33%; ~10^(-10) Pud @10-3 BLER. 40 bits; [R1-9421-5] [R1-9421-32] [QUALCOMM119]; 13% 13% 13%; ~10^(-5) PFA @10% BLER ~10^(-4) Pud @10-3 BLER; [R1-9421-5] [R1-9421-32] [QUALCOMM119]; 29% 29% 29%; ~10^(-10) Pud @10-3 BLER. 48 bits; [R1-9421-3] [R1-9421-32] [QUALCOMM119]; 11% 11% 11%; ~10^(-2) PFA ~10^(-5) PFA @10% BLER ~10^(-4) Pud @10-3 BLER; [R1-9421-3] [R1-9421-32] [QUALCOMM119]; 25% 25% 25%; ~10^(-5) PFA ~10^(-10) Pud @10-3 BLER. 96 bits; [R1-9421-5] [R1-9421-3] [R1-9421-32] [QUALCOMM119]; 6% 5% 6% 6%; ~10^(-2) PFA ~10^(-4) Pud @10-3 BLER; [R1-9421-5] [R1-9421-3] [R1-9421-32] [QUALCOMM119]; 14% 14% 14% 14%; ~10^(-5) PFA ~10^(-9) Pud @10-3 BLER. 128 bits; [R1-9421-32] [QUALCOMM119]; 4% 4%; ~10^(-3) Pud @10-3 BLER; [R1-9421-32] [QUALCOMM119]; 11% 11%; ~10^(-9) Pud @10-3 BLER. 256 bits; [R1-9421-5] [QUALCOMM119]; 2% 2%; ~10^(-3) Pud @10-3 BLER; [R1-9421-5] [QUALCOMM119]; 6% 6%; ~10^(-9) Pud @10-3 BLER. ***unchanged parts omitted***

R1-2410701	Feature Lead Summary #2 for 9.4.2.1: “Ambient IoT – General aspects of physical layer design”	Moderator (Huawei)

From Wednesday session

Agreement

Capture the following TP update into TR 38.769

OOK and BPSK for baseband modulation are feasible for D2R for all devices. - Sources [R1-9421-3], [R1-9421-11], [R1-9421-28], [R1-9421-16] report that MSK is feasible in some way: - [R1-9421-3], [R1-9421-11], [Wiliot R1-2402720] say it is feasible for all devices, for example when it is implemented with multiple impedances switching. - [R1-9421-28] say that it would be implemented as square-wave MSK for devices 1 and 2a, and sine-wave MSK for device 2b - For device 1 and 2a this type of MSK does not have continuous phase - [R1-9421-3] say that benefits include lower sidelobes than OOK and BPSK, and lower BER than OOK and same BER as BPSK - Sources [R1-9421-5], [R1-9421-2], [R1-9421-9], [R1-9421-7], [R1-9421-8], [R1-9421-10], [R1-9421-23] report that MSK is either infeasible or should be deprioritized for all devices. - [R1-9421-5], [R1-9421-9], [R1-9421-7], [R1-9421-8], [R1-9421-2], [R1-9421-10], [R1-9421-23] say that MSK is less spectrally efficient than OOK and BPSK because there are issues due to poor phase accuracy in the device - [R1-9421-5], [R1-9421-7], [R1-9421-2], [R1-9421-8], [R1-9421-10] say that MSK would increase reader and device complexity - [R1-9421-8] say that MSK performance for device 2b would materially degrade due to CFO - Source [MTK] reports that BPSK performance with coherent receiver highly depends on the CFO value and D2R transmission duration after a D2R amble. 2SB modulation is feasible for D2R transmission for all devices. Feasibility and necessity of 1SB modulation would depend on impacts due to issues including: device-side filtering requirements (i.e. image suppression), RF resource usage / spectral efficiency, etc.

Agreement

For small frequency shifts in D2R, update the previously-agreed TP, shown in red text, with the additional text shown in green.

6.1.2.x.1 Small frequency shifts ***unchanged parts omitted*** Sources [R1-9421-1], [R1-9421-8], [Huawei] and [R1-9421-28] state that Manchester codeword repetitions within the same time duration corresponding to an information bit is equivalent to bit-level repetitions within the same duration prior to Manchester encoding. Sources [CMCC] and [Vivo] state that option 1 has a more concentrated spectrum, and requires lesser bandwidth as compared to Option 2. Source [Vivo] further states that while Option 1 and option 2 show similar BLER performance for single device case, Option 1 outperforms Option 2 with FDMA, especially with presence of 105 ppm SFO. Option 1 can achieve additional gain for coverage evaluation due to lower effective noise power. Source [InterDigital] observed that while Option 1 has a narrower main lobe, it has higher sidelobes than Option 2. Sources [R1-9421-8], [R1-9421-32] and [R1-9421-13] state that the output waveform for Manchester line codes by Option 2 introduces a phase reversal of the output waveform in the middle of the time duration corresponding to an information bit as compared to Option 1. ***unchanged parts omitted***

Final summary in R1-2410702.

#### Frame structure and timing aspects

Including synchronization and timing, random access, scheduling and timing relationships

R1-2409365	Frame structure and timing aspects for Ambient IoT	Nokia

R1-2409389	Frame structure and timing aspects for Ambient IoT	Ericsson

R1-2409419	On frame structure and timing aspects of Ambient IoT	Huawei, HiSilicon, CBN, China Broadnet

R1-2409485	Discussion on frame structure and physical layer procedures for Ambient IoT	Lenovo

R1-2409514	Discussion on frame structure and timing aspects for A-IoT	CMCC

R1-2409553	Discussion on frame structure and physical layer procedure for Ambient IoT	ZTE Corporation, Sanechips

R1-2409599	Considerations for frame structure and timing aspects	Samsung

R1-2409638	Discussion on frame structure and timing aspects for Ambient IoT	Spreadtrum, UNISOC

R1-2409683	Discussion on Frame structure, random access, scheduling and timing aspects for Ambient IoT	vivo

R1-2409733	Discussion on frame structure and timing aspects	InterDigital, Inc.

R1-2409759	Resource allocation and frame structure of A-IoT	Tejas Networks Limited

R1-2409802	On remaining frame structure and timing aspects for AIoT	Apple

R1-2409865	Discussion on frame structure and timing for ambient IoT	NEC

R1-2409898	Discussion on frame structure and timing aspects for Ambient IoT	Xiaomi

R1-2409943	Study of Frame structure and timing aspects for Ambient IoT	CATT

R1-2410647	Discussion on frame structure and timing aspects for Ambient IoT	China Telecom	(rev of R1-2410002)

R1-2410027	Frame Structure and Timing Aspects for Ambient IoT	FUTUREWEI

R1-2410060	Discussion on frame structure and timing aspects	Fujitsu

R1-2410063	Discussion on A-IoT Frame Structure and Timing Aspects	Panasonic

R1-2410093	Discussion on frame structure and timing aspects of A-IoT communication	OPPO

R1-2410166	Discussion on frame structure and timing aspects	Google

R1-2410179	Discussion on frame structure and timing aspects for Ambient IoT	HONOR

R1-2410226	Frame structure and timing aspects of Ambient IoT	Sony

R1-2410268	Discussion on frame structure and timing aspects	ETRI

R1-2410288	Frame structure and timing aspects for Ambient IoT	LG Electronics

R1-2410356	Discussion on frame structure and timing aspects for Ambient IoT	TCL

R1-2410391	Study on frame structure and timing aspects for Ambient IoT	NTT DOCOMO, INC.

R1-2410409	Discussion on frame structure and timing aspect	ASUSTeK

R1-2410417	Discussion on frame structure and timing aspects	Sharp

R1-2410480	Frame structure and timing aspects	Qualcomm Incorporated

R1-2410516	Frame structure and timing aspects	MediaTek Inc.

R1-2410560	Discussion on frame structure and timing aspects for Ambient IoT	WILUS Inc.

R1-2410578	Discussion on Frame structure and timing aspects	CEWiT

R1-2410592	Frame structure and timing aspects of AIoT	IIT Kanpur, Indian Institute of Tech (M)

R1-2410643	FL summary #1 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

From Monday session

Agreement

Capture following observations in the TR 38.769, where CFO is assumed to be zero or negligible.

For coherent detection of PDRCH with a payload of 16 bits or 20 bits with 6-bit or 16-bit CRC, using 1/2 Manchester coding and 1/3 or 1/2 convolutional code:

- Source [6, ZTE] observed that with up to 10% SFO, ~1kbps data rate, when the same amble(s) overhead is maintained, Option 3 provides comparable performance results to Option 1.

- Source [7, Samsung] observed that with up to 10% SFO, ~5kbps data rate, for device 1 and with up to 1% SFO for device 2, the decoding performance with/without midamble are similar

- Source [9, vivo] observed that Option 1 is sufficient to achieve 10% and 1% BLER, with no more than 8 SFO hypotheses tested at the reader side.

  - With up to 10% SFO, ~ 5kbps data rate, the SNR needed to achieve 10% and 1% BLER is similar (~ -2dB and 4 dB) for Option 1, Option 2 of D2R preamble+1midamble and Option 3.

  - With up to 1% SFO, ~ 5kbps data rate, the SNR needed to achieve 10% and 1% BLER is similar (~ -2.8dB and 3.3dB) for Option 1, Option 2 of D2R preamble+1 midamble and Option 3.

- Source [20, OPPO] observed that with perfect SFO estimation, 1kbps data rate and OOK modulation, there is no noticeable performance gain from using midamble(s) and/or postamble for PDRCH transmission, regardless of the payload size.

- Source [30, Qualcomm] observed that the required SFO estimation accuracy to achieve 1% and 10% BLER is <5*10^3 ppm

  - With up to 10% SFO, achieving the required accuracy necessitates more than 20 SFO hypotheses at the reader side for Option 1 and 10 SFO hypotheses are sufficient for Option 3 of D2R preamble + postamble. But for Option 3 reader has to store the received samples and wait for the postamble that is after the end of PDRCH for any of SFO/channel/interference estimation, demodulation, decoding, etc.

  - With up to 1% SFO, 4 SFO hypotheses are sufficient for Option 1 to achieve the required accuracy.

For coherent detection of PDRCH with a payload of 96bits with 16-bit CRC (or 6-bit CRC [14, Xiaomi]), using 1/2 Manchester coding and 1/3 or 1/2 convolutional code,

- Sources [3, Huawei], [5, CMCC] and [14, xiaomi] observed that Option 1 cannot achieve 10% BLER.

- Sources [6, ZTE], [7, Samsung], [9, vivo], [20, OPPO] and [30, QC] observed that Option 1 can achieve 10% BLER.

- Sources [3, Huawei], [5, CMCC], [6, ZTE], [7, Samsung], [9, vivo], [14, xiaomi], [16, China Telecom] observed that adding additional amble improves the performance.

  - Source [3, Huawei] observed that with up to 10% SFO,

    - Option 2 of D2R preamble+ 1 midamble achieves 10% BLER at SNR around -3dB, but cannot achieve 1% BLER.

    - Option 3 of D2R preamble+ postamble achieves 10% BLER at SNR around -4dB, and can achieve 1% BLER at SNR around 4dB.

  - Source [5, CMCC] observed that with up to 10% SFO, Option 3 allows reader to precisely search and detect the SFO with 0.03% residual SFO at -3dB SNR TDL-A channel, achieving 10% BLER -2.44dB SNR for ~1 kbps data rate and -2.17 dB for ~6 kbps data rate. Source [5, CMCC] further observed that when the reader adopts same number of SFO hypothesis based on preamble, with 1% SFO, Option 3 can achieve 10% BLER at -4.27 dB SNR for ~1 kbps and at -4.29 dB SNR for ~6 kbps, which provides 1~2 dB performance gain when compared to 10% SFO.

  - Source [14, xiaomi] observed that with up to 10% SFO, Option 2 of D2R preamble + 1 midamble, option 3, and option 4 of D2R preamble + 1 midamble+postamble achieve basically the same performance, the SNR for 10% BLER is 5dB for 1.25 kbps data rate.

  - Source [6, ZTE] observed that with up to 10% SFO, ~1kbps data rate, and the same amble(s) overhead, Option 3 can provide 1~2 dB, 5dB performance gain for 10% BLER, 1% BLER, respectively, compared to Option 1. Additionally, Option 3 can provide ~1dB, 2dB performance gain for 10% BLER, 1% BLER, respectively, compared to Option 2 of D2R preamble +1 midamble.

  - Source [7, Samsung] observed that for ~5kbps data rate, compared to option 1,

    - For device 1 with up to 10% SFO, Option 2 of D2R preamble + 1 midamble provides ~0.5 dB SNR gain at 10% BLER with TDL-A channel and ~0.9 dB SNR gain with TDL-D channel.

    - For device 2 with up to 1% SFO, Option 2 of D2R pramble + 1midamble provides ~1 dB SNR gain at 10% BLER with TDL-A channel and ~1.4 dB SNR gain with TDL-D channel.

  - Source [9, vivo] observed that,

    - With up to 10% SFO, ~5.5kbps data rate, Option 1 achieves 10% and 1% BLER at SNR ~0.7dB and 10dB, respectively; Additionally, maintaining the same amble overhead, Option 2 (D2R preamble + 1 midamble) and Option 3 demonstrate similar performance, achieving 10% and 1% BLER at SNR around -1.7dB and 5.2dB, respectively.

    - With up to 1% SFO, ~5.5kbps data rate, Option 1 achieves 10% and 1% BLER at SNR ~ -1.3dB and 11dB, respectively. Additionally, with the same amble overhead, the SNR difference between Option 2 (D2R preamble+1midamble) and Option 3 for 10% and 1% BLER is less than 1dB, with SNRs ~ -3.1dB to -2.5dB for 10% BLER and ~3.6dB to 4.5dB for 1% BLER.

  - Source [16, China Telecom] observed that with up to 10% SFO, ~7.5kbps data rate, there is ~6~7dB performance gap at 10% BLER and ~10.5~11.5dB performance gap at 1% BLER between option 2 of D2R preamble+111 midambles and option 1. Note that Source [16, China Telecom] does not use any convolutional code.

- Source [20, OPPO] observed that with perfect SFO estimation, 1kbps data rate and OOK modulation, there is no noticeable performance gain from using midamble(s) and/or postamble for PDRCH transmission, regardless of the payload size.

- Source [30, Qualcomm] observed that the required SFO estimation accuracy to achieve 1% and 10% BLER is <10^3 ppm. To achieve the required accuracy,

  - For Option 1, more than 50 SFO hypotheses at reader side are necessary for device with up to 10% SFO and 6 SFO hypotheses are sufficient at reader side for device with up to 1% SFO.

  - For Option 3, 10 SFO hypotheses are sufficient for device with up to 10% SFO, but reader has to store the received samples and wait for the postamble that is after the end of PDRCH for any of SFO/channel/interference estimation, demodulation, decoding, etc.

For coherent detection of PDRCH with a payload of 400bits with 16-bit CRC, using 1/2 Manchester coding and 1/3 or 1/2 convolutional code,

- For option 1 of D2R preamble only,

  - Sources [3, Huawei], [5, CMCC], [6, ZTE], [8, Spreadtrum], [9, vivo], [14, xiaomi] observed that with up to 10% SFO, 10% BLER cannot be achieved.

  - Source [20, OPPO] observed that with perfect SFO estimation, 1kbps data rate and OOK modulation, there is no noticeable performance gain from using midamble(s) and/or postamble for PDRCH transmission.

- For other amble options,

  - Source [3, Huawei] observed that

    - With accurate SFO estimation, Option 2 of D2R preamble + 4 midambles can achieve 10% BLER at SNR ~ 2.7dB but cannot achieve 1% BLER.

    - With up to 10% SFO, Option 3 cannot achieve 10% BLER.

    - With up to 10% SFO, Option 4 of D2R preamble+2 midambles+postamble achieves 10% BLER at SNR of ~0.25dB; But it cannot achieve 1% BLER. Option 4 of D2R preamble+3 or 4 midambles+postamble, achieves a 10% BLER at an SNR of around -0.2 dB, and achieves 1% BLER at SNR around 9dB or 8dB, respectively.

  - Source [5, CMCC] observed that with up to 10% SFO, Option 4 of D2R preamble combined with 1 to 4 midambles + postamble, achieves 10% BLER at SNR of 2.5 dB, 1 dB, 0.8 dB, or 0.5 dB, respectively, for a data rate of around 1 kbps.

  - Source [6, ZTE] observed that with up to 10% SFO,

    - Option 3 can provide ~5.5 dB performance gain compared to option 2 of D2R preamble+1midamble for 10% BLER, with the same amble(s) overhead for ~1kbps data rate.

    - Option 2 of D2R preamble+1midamble cannot achieve 1% BLER for ~1kbps data rate.

    - Option 4 of the D2R preamble+1 or 2 midamble(s)+postamble, has similar performance, it can achieves a 10% BLER at SNR of -1dB and achieves a 1% BLER at SNR of 6dB and 5dB respectively for ~1kbps data rate.

  - Source [8, Spreadtrum] observed that with up to 10% SFO,

    - Option 3 of D2R preamble+ postamble cannot achieve 10% BLER for ~7kpbs.

    - Option 4 of D2R preamble + 1 midamble + postamble can achieve 10% BLER and 1% BLER at SNR around -6dB and 0 dB, respectively for ~7kpbs data rate.

  - Source [9, vivo] observed that

    - With up to 10% SFO, maintaining the same amble overhead, both Option 2 of D2R preamble+5 midambles and Option 4 of D2R preamble+4 midambles+1 postamble demonstrate similar performance, achieving 10% BLER at SNR ~0.2dB. While for 1% BLER, the SNR for Option 2 and Option 4 is ~ 9.2dB and 12.8dB, respectively for ~5.5kpbs data rate.

    - With up to 1% SFO, maintaining the same amble overhead, both Option 2 of D2R preamble+5 midambles and Option 4 of D2R preamble+4 midambles+1postamble demonstrate similar performance, achieving 10% BLER at SNR around -1.2dB. While for 1% BLER, the SNR for Option 2 and Option 4 is ~7.8dB and 9.1dB, respectively for ~5.5kpbs data rate.

  - Source [14, xiaomi] observed that with up to 10% SFO, Option 2 of D2R preamble+3 midambles and Option 4 of D2R preamble+3 midambles+postamble can achieve 10% BLER when the SNR is within the range of [15, 25] dB for 1.25 kbps data rate.

  - Source [30, Qualcomm] observed that the required SFO estimation accuracy to achieve 1% and 10% BLER is much smaller than 10^3 ppm. To achieve the required accuracy,

    - For Option 2 of D2R preamble+X midamble(s) where midamble inserted per every certain number of PDRCH bits (e.g., 192 bits),

      - For SFO estimation using each amble for the subsequent PDRCH bits (e.g., 192 bits), with up to 10% SFO, more than 50 SFO hypotheses are necessary at the reader side and with up to 1% SFO, 6 SFO hypotheses are sufficient at the reader side.

      - For SFO estimation based on the time gap between preamble and midamble, with up to 10% SFO, 10 SFO hypotheses are used, but reader has to store the received samples and wait for the midamble to start SFO/channel/interference estimation, demodulation, decoding, etc.

    - For Option 3 of D2R preamble+postamble, SFO estimation is based on the time gap between preamble and postamble, with up to 10% device SFO, 10 SFO hypotheses are used for reader, but reader has to store the received samples and wait for the postamble that is after the end of PDRCH for any of SFO/channel/interference estimation, demodulation, decoding, etc

For the synchronization and timing tracking of D2R transmission,

- Source [5, CMCC] report that with up to 10% SFO, option 1 is not sufficient for D2R reception since the residual SFO at reader side is larger than 1%. While with option 3, the reader can precisely search and detect the SFO with a residual SFO of 0.03% at -3dB SNR TDL-A channel.

- Source [14, xiaomi] report that

  - For packet size of 96bits, when the SNR is increased from -4dB to 20dB, the ratio of device residual SFO over 100ppm decreases to 6% for Option 2, 3 and 4, but remains at 95% for Option 1.

  - For packet size of 400bits, when the SNR is increased from -4dB to 20dB, the ratio of device residual SFO larger than 10ppm decreases to 5% for Option 2, 3, and 4, but is still 99.6% for Option 1.

- Sources [9, vivo], [15, CATT] report that SFO estimation based on D2R preamble can achieve accurate estimation without additional ambles (midamble or postamble).

  - Source [9, vivo][7 Samsung] observed that for non-coherent detection of PDRCH, the number of SFO hypotheses and the SNR needed for 10% and 1% BLER cannot significantly be reduced for option 2, 3 and 4 compared to the option 1. Moreover, the additional ambles i.e., midamble(s) and/or postamble introduces additional overhead and prevents pipelined processing of the reception.

  - Source [15, CATT] observed that

    - The coarse estimation of SFO based on the D2R preamble indicates that the SFO estimation error is less than 1% with a probability of 99.3%, and less than 0.1% with a probability of 49.9%.

    - The fine estimation of SFO based on the D2R preamble shows that the SFO estimation error is less than 1% with a probability of 99.5%, and less than 0.1% with a probability of 90.8%.

    - Reader/gNB can achieve a probability of not less than 99.5% for SFO estimation error below 1%, and 90.8% for SFO estimation error below 0.1% by receiving D2R preamble signals.

- Source [30, Qualcomm] report that for D2R with coherent demodulation at reader, the reader needs to estimate the device clock frequency with the accuracy of 0.5% (5 * 10^3 ppm) or lower for a short message (e.g., 72 bits after CRC/coding) and 0.1% (10^3 ppm) or lower for a long message (e.g., 224 bits after CRC/coding). The source further reports that design of D2R amble(s) (e.g., overhead) and the correspondingly required number of SFO hypothesis for the estimation depend on the sampling clock accuracy that the device uses for D2R.

Note: in the observations above where coherent detection is used, sources that evaluated option 3 and option 4 assumed that the postamble is used at least for time/frequency tracking and for channel estimation.

Agreement

For the CFO calibration signal, which is required only for device 2b to reduce the frequency offset range and the guard-bandwidth of D2R transmission, the following observations are captured in TR 38.769:

- Source [3, Huawei] report that a single-tone RF signal is used as the CFO calibration signal, it is not a part of time acquisition signal and can be transmitted as an optional R2D signal after the PRDCH transmission.

- Sources [2, Ericsson], [19, Panasonic] and [20, OPPO] report that additional synchronization signal is needed.

  - [OPPO] state the R2D timing acquisition signal may not be sufficient or may not be usable for CFO calibration since a reference frequency is needed when separate LOs are used for Tx and Rx in device 2b.

- Sources [7, Samsung], [9, vivo], [30, Qualcomm], [36, Apple] report that additional synchronization signal is needed if the synchronization for carrier frequency using R2D signal/channel does not provide required functionalities for device 2b.

- Source [5, CMCC][31, MTK] report that it may not be possible to achieve enough frequency accuracy (0.01 ppm) even after CFO calibration based on R2D time acquisition signals for coherent detection at reader especially when the D2R data rate is low.

Agreement

For device 2b, a signal for CFO calibration should be provided to synchronize / calibrate the device clock for LO for carrier frequency (Clock purpose #5) to achieve the accuracy after clock sync / calibration at device side captured in Table 5.2.3-1.

- Frequency calibration at device 2b is beneficial at least to reduce the guard-bandwidth of D2R transmission.

Agreement

Adopt the updates documented in R1-2410653 for section 6.2 of the TR38.769.

R1-2410653	TP updates on section 6.2 Device (un)availability for TR 38.769	Moderator (vivo)

R1-2410644	FL summary #2 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

From Tuesday session

Agreement

Adopt following update to the TP agreed on Monday

Capture following observations in the TR 38.769, where CFO is assumed to be zero or negligible.

[omit unchanged part]

For coherent detection of PDRCH with a payload of 96bits with 16-bit CRC (or 6-bit CRC [14, Xiaomi]), using 1/2 Manchester coding and 1/3 or 1/2 convolutional code,

- Sources [3, Huawei], [5, CMCC] and [14, xiaomi] observed that Option 1 cannot achieve 10% BLER.

- Sources [6, ZTE], [7, Samsung], [9, vivo], [20, OPPO] and [30, QC] observed that Option 1 can achieve 10% BLER.

- Sources [3, Huawei], [5, CMCC], [6, ZTE], [7, Samsung], [9, vivo], [14, xiaomi], [16, China Telecom] observed that adding additional amble improves the performance.

  - Source [3, Huawei] observed that with up to 10% SFO,

    - Option 2 of D2R preamble+ 1 midamble achieves 10% BLER at SNR around -3dB, but cannot achieve 1% BLER.

    - Option 3 of D2R preamble+ postamble achieves 10% BLER at SNR around -4dB, and can achieve 1% BLER at SNR around 4dB.

  - Source [5, CMCC] observed that with up to 10% SFO, Option 3 allows reader to precisely search and detect the SFO with 0.03% residual SFO at -3dB SNR TDL-A channel, achieving 10% BLER -2.44dB SNR for ~1 kbps data rate and -2.17 dB for ~6 kbps data rate. Source [5, CMCC] further observed that when the reader adopts same number of SFO hypothesis based on preamble, with 1% SFO, Option 3 can achieve 10% BLER at -4.27 dB SNR for ~1 kbps and at -4.29 dB SNR for ~6 kbps, which provides 1~2 dB performance gain when compared to 10% SFO.

  - Source [14, xiaomi] observed that with up to 10% SFO, Option 2 of D2R preamble + 1 midamble, option 3, and option 4 of D2R preamble + 1 midamble+postamble achieve basically the same performance, the SNR for 10% BLER is 5dB for 1.25 kbps data rate.

  - Source [6, ZTE] observed that with up to 10% SFO, ~1kbps data rate, and the same amble(s) overhead, Option 3 can provide 1~2 dB, 5dB performance gain for 10% BLER, 1% BLER, respectively, compared to Option 1. Additionally, Option 3 can provide ~1dB, 2dB performance gain for 10% BLER, 1% BLER, respectively, compared to Option 2 of D2R preamble +1 midamble.

  - Source [7, Samsung] observed that for ~5kbps data rate, compared to option 1,

    - For device 1 with up to 10% SFO, Option 2 of D2R preamble + 1 midamble provides ~0.5 dB SNR gain at 10% BLER with TDL-A channel and ~0.9 dB SNR gain with TDL-D channel.

    - For device 2 with up to 1% SFO, Option 2 of D2R pramble + 1midamble provides ~1 dB SNR gain at 10% BLER with TDL-A channel and ~1.4 dB SNR gain with TDL-D channel.

    - Moreover, the additional ambles i.e., midamble(s) and/or postamble introduces additional overhead and postamble may prevent pipelined processing of the reception.

  - Source [9, vivo] observed that,

    - With up to 10% SFO, ~5.5kbps data rate, Option 1 achieves 10% and 1% BLER at SNR ~0.7dB and 10dB, respectively; Additionally, maintaining the same amble overhead, Option 2 (D2R preamble + 1 midamble) and Option 3 demonstrate similar performance, achieving 10% and 1% BLER at SNR around -1.7dB and 5.2dB, respectively.

    - With up to 1% SFO, ~5.5kbps data rate, Option 1 achieves 10% and 1% BLER at SNR ~ -1.3dB and 11dB, respectively. Additionally, with the same amble overhead, the SNR difference between Option 2 (D2R preamble+1midamble) and Option 3 for 10% and 1% BLER is less than 1dB, with SNRs ~ -3.1dB to -2.5dB for 10% BLER and ~3.6dB to 4.5dB for 1% BLER.

  - Source [16, China Telecom] observed that with up to 10% SFO, ~7.5kbps data rate, there is ~6~7dB performance gap at 10% BLER and ~10.5~11.5dB performance gap at 1% BLER between option 2 of D2R preamble+111 midambles and option 1. Note that Source [16, China Telecom] does not use any convolutional code.

- Source [20, OPPO] observed that with perfect SFO estimation, 1kbps data rate and OOK modulation, there is no noticeable performance gain from using midamble(s) and/or postamble for PDRCH transmission, regardless of the payload size.

- Source [30, Qualcomm] observed that the required SFO estimation accuracy to achieve 1% and 10% BLER is <10^3 ppm. To achieve the required accuracy,

  - For Option 1, more than 50 SFO hypotheses at reader side are necessary for device with up to 10% SFO and 6 SFO hypotheses are sufficient at reader side for device with up to 1% SFO.

  - For Option 3, 10 SFO hypotheses are sufficient for device with up to 10% SFO, but reader has to store the received samples and wait for the postamble that is after the end of PDRCH for any of SFO/channel/interference estimation, demodulation, decoding, etc.

For coherent detection of PDRCH with a payload of 400bits with 16-bit CRC, using 1/2 Manchester coding and 1/3 or 1/2 convolutional code,

- For option 1 of D2R preamble only,

  - Sources [3, Huawei], [5, CMCC], [6, ZTE], [8, Spreadtrum], [9, vivo], [14, xiaomi] observed that with up to 10% SFO, 10% BLER cannot be achieved.

  - Source [20, OPPO] observed that with perfect SFO estimation, 1kbps data rate and OOK modulation, there is no noticeable performance gain from using midamble(s) and/or postamble for PDRCH transmission.

- For other amble options,

  - Source [3, Huawei] observed that

    - With accurate SFO estimation, Option 2 of D2R preamble + 4 midambles can achieve 10% BLER at SNR ~ 2.7dB but cannot achieve 1% BLER.

    - With up to 10% SFO, Option 3 cannot achieve 10% BLER.

    - With up to 10% SFO, Option 4 of D2R preamble+2 midambles+postamble achieves 10% BLER at SNR of ~0.25dB; But it cannot achieve 1% BLER. Option 4 of D2R preamble+3 or 4 midambles+postamble, achieves a 10% BLER at an SNR of around -0.2 dB, and achieves 1% BLER at SNR around 9dB or 8dB, respectively.

  - Source [5, CMCC] observed that with up to 10% SFO, Option 4 of D2R preamble combined with 1 to 4 midambles + postamble, achieves 10% BLER at SNR of 2.5 dB, 1 dB, 0.8 dB, or 0.5 dB, respectively, for a data rate of around 1 kbps.

  - Source [6, ZTE] observed that with up to 10% SFO,

    - Option 3 can provide ~5.5 dB performance gain compared to option 2 of D2R preamble+1midamble for 10% BLER, with the same amble(s) overhead for ~1kbps data rate.

    - Option 2 of D2R preamble+1midamble cannot achieve 1% BLER for ~1kbps data rate.

    - Option 4 of the D2R preamble+1 or 2 midamble(s)+postamble, has similar performance, it can achieve a 10% BLER at SNR of -1dB and achieves a 1% BLER at SNR of 6dB and 5dB respectively for ~1kbps data rate.

  - Source [8, Spreadtrum] observed that with up to 10% SFO,

    - Option 3 of D2R preamble+ postamble cannot achieve 10% BLER for ~7kpbs.

    - Option 4 of D2R preamble + 1 midamble + postamble can achieve 10% BLER and 1% BLER at SNR around -6dB and 0 dB, respectively for ~7kpbs data rate.

  - Source [9, vivo] observed that

    - With up to 10% SFO, maintaining the same amble overhead, both Option 2 of D2R preamble+5 midambles and Option 4 of D2R preamble+4 midambles+1 postamble demonstrate similar performance, achieving 10% BLER at SNR ~0.2dB. While for 1% BLER, the SNR for Option 2 and Option 4 is ~ 9.2dB and 12.8dB, respectively for ~5.5kpbs data rate.

    - With up to 1% SFO, maintaining the same amble overhead, both Option 2 of D2R preamble+5 midambles and Option 4 of D2R preamble+4 midambles+1postamble demonstrate similar performance, achieving 10% BLER at SNR around -1.2dB. While for 1% BLER, the SNR for Option 2 and Option 4 is ~7.8dB and 9.1dB, respectively for ~5.5kpbs data rate.

  - Source [14, xiaomi] observed that with up to 10% SFO, Option 2 of D2R preamble+3 midambles and Option 4 of D2R preamble+3 midambles+postamble can achieve 10% BLER when the SNR is within the range of [15, 25] dB for 1.25 kbps data rate.

  - Source [30, Qualcomm] observed that the required SFO estimation accuracy to achieve 1% and 10% BLER is much smaller than 10^3 ppm. To achieve the required accuracy,

    - For Option 2 of D2R preamble+X midamble(s) where midamble inserted per every certain number of PDRCH bits (e.g., 192 bits),

      - For SFO estimation using each amble for the subsequent PDRCH bits (e.g., 192 bits), with up to 10% SFO, more than 50 SFO hypotheses are necessary at the reader side and with up to 1% SFO, 6 SFO hypotheses are sufficient at the reader side.

      - For SFO estimation based on the time gap between preamble and midamble, with up to 10% SFO, 10 SFO hypotheses are used, but reader has to store the received samples and wait for the midamble to start SFO/channel/interference estimation, demodulation, decoding, etc.

    - For Option 3 of D2R preamble+postamble, SFO estimation is based on the time gap between preamble and postamble, with up to 10% device SFO, 10 SFO hypotheses are used for reader, but reader has to store the received samples and wait for the postamble that is after the end of PDRCH for any of SFO/channel/interference estimation, demodulation, decoding, etc

  - Source [7, Samsung] observes that the additional ambles i.e., midamble(s) and/or postamble introduces additional overhead and postamble may prevent pipelined processing of the reception.

For the synchronization and timing tracking of D2R transmission,

- Source [5, CMCC] report that with up to 10% SFO, option 1 is not sufficient for D2R reception since the residual SFO at reader side is larger than 1%. While with option 3, the reader can precisely search and detect the SFO with a residual SFO of 0.03% at -3dB SNR TDL-A channel.

- Source [14, xiaomi] report that

  - For packet size of 96bits, when the SNR is increased from -4dB to 20dB, the ratio of device residual SFO over 100ppm decreases to 6% for Option 2, 3 and 4, but remains at 95% for Option 1.

  - For packet size of 400bits, when the SNR is increased from -4dB to 20dB, the ratio of device residual SFO larger than 10ppm decreases to 5% for Option 2, 3, and 4, but is still 99.6% for Option 1.

- Sources [9, vivo], [15, CATT] report that SFO estimation based on D2R preamble can achieve accurate estimation without additional ambles (midamble or postamble).

  - Source [9, vivo][7 Samsung] observed that for non-coherent detection of PDRCH, the number of SFO hypotheses and the SNR needed for 10% and 1% BLER cannot significantly be reduced for option 2, 3 and 4 compared to the option 1. Moreover, the additional ambles i.e., midamble(s) and/or postamble introduces additional overhead and postamble may prevents pipelined processing of the reception.

  - Source [15, CATT] observed that

    - The coarse estimation of SFO based on the D2R preamble indicates that the SFO estimation error is less than 1% with a probability of 99.3%, and less than 0.1% with a probability of 49.9%.

    - The fine estimation of SFO based on the D2R preamble shows that the SFO estimation error is less than 1% with a probability of 99.5%, and less than 0.1% with a probability of 90.8%.

    - Reader/gNB can achieve a probability of not less than 99.5% for SFO estimation error below 1%, and 90.8% for SFO estimation error below 0.1% by receiving D2R preamble signals.

- Source [30, Qualcomm] report that for D2R with coherent demodulation at reader, the reader needs to estimate the device clock frequency with the accuracy of 0.5% (5 * 10^3 ppm) or lower for a short message (e.g., 72 bits after CRC/coding) and 0.1% (10^3 ppm) or lower for a long message (e.g., 224 bits after CRC/coding). The source further reports that design of D2R amble(s) (e.g., overhead) and the correspondingly required number of SFO hypothesis for the estimation depend on the sampling clock accuracy that the device uses for D2R.

- Source [37, MediaTek] reports that transmitting 96-bit packet size with 16-bit CRC requires residue SFO after reader compensation to be 1000 ppm, and transmitting 1000-bit packet size with 16-bit CRC requires residue SFO after reader compensation to be 100 ppm.

Note: in the observations above where coherent detection is used, sources that evaluated option 3 and option 4 assumed that the postamble is used at least for time/frequency tracking and for channel estimation.

Agreement

For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies the following for the starting time for Msg2 monitoring

- For Option 1, where a PRDCH for Msg2 transmission corresponds to an A-IoT Msg1 received from one device,

  - Option a: the starting time for Msg2 monitoring is separate for each Msg1 resource.

  - Option b: the starting time for Msg2 monitoring is common for multiple Msg1 resources.

- For Option 2, where a PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices, the starting time for Msg2 monitoring is common for multiple Msg1 resources.

R1-2410645	FL summary #3 on frame structure and timing aspects for Rel-19 Ambient IoT	Moderator (vivo)

From Wednesday session

Agreement

For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options to determine the time duration for Msg2 monitoring

- Option 1: Predefined

- Option 2: Indicated in the R2D transmission triggering random access

- Note: Option 1 and Option 2 may not be mutually exclusive.

From Thursday session

Agreement

For Msg2 transmission in response to multiple Msg1 transmissions, which is initiated by a R2D transmission triggering random access, RAN1 identifies at least the following options for a device to determine the reference time for the starting time for Msg2 monitoring

  - Option 1: End of time domain resource where the device transmitted the Msg1

  - Option 2: End of the last of X time domain resource(s) determined for Msg1 transmission(s)

  - Option 3: End of the R2D transmission triggering random access

  - Option 4: End of a R2D transmission carrying Msg2

  - Note the reference time is used to determine the starting time for Msg2 monitoring, it does not mean that the starting time always equals the reference time.

Final FL summary in R1-2410866.

#### Downlink and uplink channel/signal aspects

Including detailed physical layer design aspects such as information payload, time/frequency domain resource, feasibility and required functionalities for proximity determination, etc.

R1-2409360	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	TCL

R1-2409366	R2D and D2R channel/signal aspects for Ambient IoT	Nokia

R1-2409420	Physical channels and signals for Ambient IoT	Huawei, HiSilicon, CBN, China Broadnet

R1-2409486	Discussion on channel/signal aspects for Ambient IoT	Lenovo

R1-2409515	Discussion on downlink and uplink channel/signal aspects	CMCC

R1-2409554	Discussion on channel and signal for Ambient IoT	ZTE Corporation, Sanechips

R1-2409600	Considerations for downlink and uplink channel/signal aspect	Samsung

R1-2409639	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	Spreadtrum, UNISOC

R1-2409684	Discussion on Downlink and uplink channel/signal aspects	vivo

R1-2409734	Discussion on downlink and uplink channel/signal aspects	InterDigital, Inc.

R1-2409760	Study the downlink and uplink signal aspects of A-IoT	Tejas Networks Limited

R1-2409803	On remaining details of physical channels/signals for AIoT	Apple

R1-2409866	Discussion on downlink and uplink channel for ambient IoT	NEC

R1-2409899	Discussion on downlink and uplink channel and signal aspects for Ambient IoT	Xiaomi

R1-2409944	DL and UL Physical Channels/signals design in support of Ambient IoT devices	CATT

R1-2410003	Discussion on downlink and uplink channel/signal aspects for Ambient IoT	China Telecom

R1-2410028	D2R and R2D Channel/Signal Aspects for Ambient IoT	FUTUREWEI

R1-2410061	Discussion on downlink and uplink channel/signal aspects	Fujitsu

R1-2410094	Discussion on downlink and uplink channel/signal aspects for A-IoT	OPPO

R1-2410127	Downlink and uplink channel/signal aspects for Ambient IoT	Ericsson

R1-2410167	Discussion on downlink and uplink transmission aspects	Google

R1-2410199	Discussion on downlink and uplink channels and signals for A-IoT  	Panasonic

R1-2410227	Downlink and uplink channel aspects of Ambient IoT	Sony

R1-2410269	Discussion on downlink and uplink channel/signal aspects for A-IoT	ETRI

R1-2410289	Downlink and uplink channel/signal aspects for Ambient IoT	LG Electronics

R1-2410392	Study on downlink and uplink channel/signal aspects for Ambient IoT	NTT DOCOMO, INC.

R1-2410411	Discussion on downlink and uplink channel/signal aspect	ASUSTeK

R1-2410418	Discussion on downlink and uplink channel/signal aspects	Sharp

R1-2410439	Considerations for downlink and uplink channel/signal aspects	Semtech Neuchatel SA

R1-2410481	Downlink and uplink channel/signal aspects	Qualcomm Incorporated

R1-2410517	Downlink and uplink channel/signal aspects	MediaTek Inc.

R1-2410579	Discussion on Downlink and Uplink channel/signal aspects	CEWiT

R1-2410593	Discussion on Downlink and uplink channel/signal aspects	IIT Kanpur, Indian Institute of Tech (M)

R1-2409805	FL summary#1 on downlink and uplink channel/signal aspects	Apple

From Tuesday session

Agreement

Following observations on R2D clock-acquisition part are captured in TR 38.769:

- On impact/restriction of M values for the clock-acquisition part

  - 9 sources [TCL, Nokia, Huawei, CMCC, ZTE, Apple, CATT, Mediatek, Qualcomm] provided observations on the impact/restriction of M values for the clock-acquisition part design requirements:

    - 1 source [Nokia] observed that increasing value of M, while retaining the same transmission duration, improves the auto-/cross- correlation properties of the sequence due to increase in sequence length and use of  M≥4 provides better timing estimation accuracy even in the presence of SFO as the sequence length spans only over a shorter duration.

    - 2 sources [TCL, Huawei] observed for option 1 of the clock-acquisition part design that no restriction is required to be placed on the M values. Furthermore, 1 source [Huawei] observed that the same 2 ON-OFF voltage (with the same duration) satisfies the FDR performance metric of less than 1% for different M values, e.g., M = 2, 6 and 24, where FDR is the False detection ratio (FDR), i.e. incorrectly calculating M, is the performance metric.

    - 1 source [CMCC] observed that pattern of the clock-acquisition part is related to M chips per OFDM symbol and when M is small, the clock-acquisition part may cross multiple OFDM symbols, and the CP insertion may degrade the timing acquisition performance.

    - 1 source [ZTE] observed that with option 2, the duration of the clock-acquisition part remains consistent across all M values, at least three OFDM symbols maybe required for clock-acquisition part and it maybe not as efficient as option1

    - 1 source [Apple] observed that among the two options studied for the clock-acquisition part, option 2 provides increased robustness, especially in case of large value of M, when compared to option 1 and potentially increase the detection performance of the clock-acquisition part.

    - 1 source [CATT] observed that if the chip duration is variable based on the M value used for OOK-4 waveform, the detection performance would be limited by the received SINR of the CAP with clear transition of the rising and falling edges.

    - 1 source [Qualcomm] observed that the option 1 with M>1 has shorter duration of clock acquisition part than M=1 and worse timing acquisition accuracy. At least part of PRDCH following the clock acquisition part may need to be used to improve the timing acquisition. Furthermore, the larger M (e.g., M>4) with small chip duration is more sensitive to the SFO accuracy and the restriction of M for the clock acquisition part may be needed.

    - 1 source [Mediatek] further observed that different M values may impact the chip accuracy obtained by the clock acquisition part.

- On impact of CP insertion/handling on the clock-acquisition part

  - 10 sources [TCL, CMCC, ZTE, Samsung, Vivo, CATT, NTT Docomo, Qualcomm, Mediatek, Spreadtrum] observed that the CP insertion/handling may impact the design requirements of the clock-acquisition part:

    - 1 source [CMCC] further observed that when the clock-acquisition part occupies more than one OFDM symbol, ON-OFF state transition around CP can avoid the error rising or falling edges due to the CP insertion.

    - 1 source [ZTE] further observed that to mitigate the impact of the CP in the clock-acquisition part for large M values, it can reuse the CP handling method for PRDCH

    - 1 source [Samsung] further observed that CP insertion/handling on the clock-acquisition part can cause false rising/falling transition and, therefore, the clock acquisition part should be designed such that it does not incur a false rising or falling edges due to CP insertion when CP-OFDM is used for OOK signal generation.

    - 1 source [vivo] further observed that CP insertion/handling on the clock acquisition part will impact the chip duration estimation accuracy. It is further observed that for CP handling, device may not be able to count the clock and estimate OFDM symbol duration accurately until the clock acquisition part if the start indicator only includes a single ON-OFF transmission.

    - 1 source [CATT] further observed that the SER will be degraded due to uneven chip interval when the CP is inserted within an OFDM symbol, where SER refers to the number of samples which is mismatched for comparing to the total number of samples in a chip.

    - 1 source [NTT Docomo] further observed if CP insertion would cause false rising/falling edges, accuracy of timing acquisition may be impacted.

    - 1 source [Mediatek] further observed that the issues of chip extension, false raising/falling transition, and additional raising/falling transition caused by CP insertion/handling considering different M values will impact the chip accuracy obtained by the clock acquisition part.

    - 1 source [Spreadtrum] further observed that the design of clock acquisition part should consider that CP insertion does not cause a false rising or falling edges and does not cause different length of multiple high / low voltages within the clock acquisition part when the clock acquisition spans multiple OFDM symbols.

  - 1 source [Huawei] observed CP insertion/handling may not impact the design requirements of the clock-acquisition part

Agreement

For the D2R preamble design, following aspects have been studied and can be captured in the TR 38.769:

- Autocorrelation Property

  - 10 sources [Nokia, Huawei, CMCC, Xiaomi, CATT, Oppo, Ericsson, NTT Docomo, Qualcomm, ZTE] observed that the signal should have good autocorrelation properties for accurate peak detection based on the signal correlation at the reader

- Cross-correlation Property

  - 7 sources [Nokia, CMCC, Oppo, Ericsson, Qualcomm, ZTE, CATT] observed that the signal should have good cross-correlation properties if multiple D2R preamble sequences are considered (e.g. for multiple access schemes (if supported) for D2R transmissions).

- Line coding

  - 1 source [Nokia] observed that line coding may impact the autocorrelation property of the sequence.

  - 1 source [Huawei] observed that for D2R preamble, to apply backscattering, line coding can help improve the detection performance based on shifting the D2R signal’s frequency location away from the carrier wave

- Sequence Types (not limited to below types only)

  - M-sequence

    - 3 sources [Nokia, Vivo, Xiaomi] observed that m-sequence can be considered for D2R preamble mainly owing to good correlation properties.

  - Golay sequence

    - 4 sources [CMCC, Vivo, Xiaomi, Samsung] observed that Golay sequence can be considered for D2R preamble mainly owing to good correlation properties and availability of large number of distinct sequences and complementary pairs.

  - Walsh sequence

    - 1 source [Oppo] observed that Walsh sequence can be considered as a candidate  for D2R preamble thanks to its good auto/cross-correlation property and flexible length

- General Observations

  - 1 source [Huawei] observed can achieve 0.97% residual SFO with 98% probability under -2.5dB SNR and 0.1% MDR with [-1/8, 1/8] chip timing error with 99.05% probability under -2.5dB SNR with D2R preamble including 2-parts with clock-like sampling frequency signal and timing-acquisition signal, having 32-length ‘1’ sequence (encoded to 64-chip Manchester code) and 32-length sequence (encoded to 64-chip Manchester code), respectively.

  - 4 sources [TCL, CMCC, ZTE, Vivo] observed that for D2R preamble with binary signal, the timing synchronization performance is highly related to the sequence length of the preamble. Furthermore, 1 source [CMCC] observed that to achieve a BLER performance at 10%, the timing synchronization error should be less than 10%. Furthermore, 1 source [ZTE] observed that the channel estimation performance is also highly related to sequence length. 1 source [ZTE] observed that using a 32 bits preamble provides ~8 dB, ~5 dB performance gain than using 8 bits, 16 bits preamble, respectively. And using a 64 bits preamble provides ~2.5dB performance gain than using a 32 bits preamble.

  - 1 source [Ericsson] observed that for D2R preamble with binary signal, normalized SFO estimation error of less than 10% can be achieved with a training sequence length 64 or longer. The simulated D2R preamble consisting of a Golay complementary pair can tolerate SFO up to 1% (AWGN) with up to 1 dB loss in performance for a sufficiently long preamble sequence length (32 or greater).

Agreement

For determining the end of PRDCH at the device, following two options are studied and captured in the TR 38.769:

- Option 1: TBS information (via implicit/explicit L1 R2D control information)

- Option 2: Postamble (at the end of PRDCH)

14 sources [Nokia, Huawei, ZTE, CMCC, Samsung, Ericsson, Oppo, LGE, Qualcomm, Spreadtrum, Mediatek, Cewit, Ericsson, vivo] provided following observations on the above two options for determining the end of PRDCH:

- 3 sources [Nokia, Huawei, ZTE] observed that option 2 provide two benefits, namely, the variable payload length and to provide timing acquisition before the subsequent transmission of either PDRCH or PRDCH, thus improving the detectability at both reader and the device, respectively. Furthermore, 1 source [Huawei] observed that R2D postamble indicates the TBS with high efficiency for small packets by avoiding a large padding overhead, unlike option 1, which may require devices to perform blind detection of different PRDCH formats (if supported) and the overhead caused by the inclusion of a R2D postamble does not exceed 20% for even the smallest of message sizes and may be less than the signaling overhead caused by using a dedicated TBS indicator

- 1 source [CMCC] observed for option 2, that for small payload size with only a few bits, the presence of long postamble generates large resource overhead, while for large payload size with more bits, the resource overhead of postamble is smaller.

- 1 source [vivo] observed for option 2, that for small payload size with only a few bits, the presence of long postamble generates large resource overhead.

- 1 source [Samsung] observed option 2 is not strictly required, however, given the possible clock drift at a device, it may be still beneficial to also attach postamble at least for the determination of the end of PRDCH at a device.

- 3 sources [Oppo, Spreadtrum, CEWiT] observed that with option 2, the false detection may be higher for shorter postamble. Source [OPPO[ observed that in contrast to option 2, it is more reliable and efficient to indicate TBS with control information in option 1

- 2 sources [LGE, vivo] observed that if a message type or a command ID is included in L1 control information and implicitly indicates a known size of a fixed TB, then there is no need for either option 1 or option 2

- 2 sources [Qualcomm, vivo] observed that option 1 has the advantages of avoiding blind detection of postamble and providing the power saving for non-target devices to skip the R2D detection.

- 1 source [MediaTek] observed that option 1 is feasible for the device to avoid the unnecessary reception of a TB with a specific size and thus enable power saving, e.g., when the TB has a size exceeding the allowance of the device remaining power.

- 1 source [Ericsson] observed option 2 is not strictly required if the end of PRDCH can be explicitly indicated by R2D control information, and it is subject to the miss-detection rate. It may be beneficial if a PRDCH postamble can serve as an additional timing acquisition signal prior to a PDRCH transmission.

Agreement

For transmission of R2D control information for R2D reception and D2R scheduling, following two options are studied and captured in the TR 38.769:

- Option 1: L1-control signaling

- Option 2: Higher-layer signaling

12 sources [Huawei, CMCC, ZTE, Samsung, Oppo, Qualcomm, Futurewei, Spreadtrum, NTT Docomo, Cewit, Ericsson, vivo] provided following observations on the above two options for signaling R2D control information:

- 1 source [Huawei] observed that option 2 avoids physical layer design, while at the same time offering flexibility in terms of the size of the grant. On the other hand, option 1 may require  the device to perform blind detection of different PRDCH formats (if supported), while at the same time, reducing the flexibility and resource utilization efficiency. Option 1 would also require a separate CRC, which would result in breaking the pipelining of the CRC decoding due to the added latency and increases overhead.

- 3 sources [CMCC, ZTE, Futurewei] observed that for D2R scheduling information, as the PDRCH transmission only occurs when the device can successfully decode the preceding PRDCH transmission, therefore no benefit is observed for option 1 compared to option 2. Furthermore, one source [CMCC] also observed that for R2D reception, if TBS of PRDCH is indicated by R2D control information, such R2D control information should be carried via L1 signaling as the device needs to understand the packet size of PRDCH before decoding.

- 1 source [Samsung] observed that for D2R scheduling information indication, option 1 has an advantage as the decoding of D2R scheduling information is decoupled from that of R2D payload and the device can early start the preparation of the PDRCH. For R2D scheduling information indication, Option 1 has an advantage as the device can early detect whether the corresponding R2D transmission is addressed to the device or not, which also saves device’s power consumption.

- 1 source [Oppo] observed that D2R scheduling information can be transmitted as physical layer signalling or higher layer signalling as there is no fundamental difference between the 2 options.

- 1 source [Qualcomm] observed that option 1 allows the decoding of R2D and D2R scheduling information to occur before the decoding of the R2D payload. The D2R scheduling in L1 control enables the device to provide feedback on the (un)successful detection of the R2D payload, e.g., due to insufficient energy/power or channel/interference variation. Additionally, 6 sources [ZTE, Qualcomm, CMCC, Samsung, OPPO, vivo] observed that the R2D scheduling information in L1 control enables early identification of target devices and power saving for non-target devices by allowing them to skip the following R2D detection.

- 1 source [Spreadtrum] observed that device can decode L1 control information first which can allow device to obtain the scheduling information (e.g. TBS rated information) before data part reception in option 1 and if device knows from control information that the following data is not for it, the data reception can be skipped.

- 1 source [NTT Docomo] observed that option 1 allows the decoding of R2D control information separately from the subsequent R2D reception, which enables devices to feedback when subsequent R2D reception is failed and also enables identification of target devices before the subsequent R2D reception.

- 1 source [CEWiT] observed that Option 1 enables the separate decoding of R2D control information before R2D data reception, which supports feedback if R2D reception fails. Additionally, L1 control can carry D2R scheduling info, allowing devices to avoid the non-intended R2D reception.

- 1 source [Ericsson] observed that with Option 1, L1 R2D control information may include important information needed to decode the data portion of the PRDCH signal. This requires L1 R2D control information part to have a fixed payload size and self-decodable with separate CRC.

Agreement

For D2R scheduling, midamble (if supported) related information can be explicitly/implicitly indicated via corresponding PRDCH.

Agreement

Following observations related to the agreed options on resource allocation and/or controlling of intermediate UE in topology 2 are additionally captured in TR 38.769:

  - 1 source [Vivo] observed that if UE determines the resource used for A-IoT transmission within A-IoT resources configured by the network, option 1 provides better resource efficiency and option 2 enables faster A-IoT resource adaptation compared to option 1. And it is further observed that if the resource allocation from NW is for each R2D and/or D2R transmission and is indicated by a L1 signaling of option2, option 2 may affect the A-IoT communication timing relations and causes substantial overhead between NW and UE reader and may lead to resource inefficiency as NW lacks direct information of channel condition between A-IoT device and UE reader.

  - 1 source [Xiaomi] observed that option 1 can save signaling overhead b/w gNB and intermediate UE compared to option 2 but may cause resource waste issue. For resource allocation of intermediate UE, Option 2 can provide better resource efficiency compared to Option 1 but requires more signaling overhead b/w gNB and intermediate UE.

  - 1 source [CATT] compared option 1 and option 2 in terms of signaling overhead, signaling latency, flexibility and resource utilization. It is observed that signaling overhead is higher with option 2, but latency is higher with option 1. Option 2 may provide higher flexibility and better resource utilization compared to option 1.

  - 1 source [Fujitsu] observed that option 1 only requires the standardization workload for adding new higher layer signaling and does not impact the current DCI design for UEs, but the resource utilization efficiency of option 1 may be lower than that of option 2. Option 2 can provide better resource usage efficiency than that of option 1, but Option 2 requires standardization effort on renewing DCI design for intermedia nodes.

  - 1 source [Qualcomm] observed that option 2 is too costly in terms of overhead and latency if using DCI-based scheme to trigger each R2D or D2R, but it is beneficial for flexible resource sharing between AIoT communications as well as the coexistence with the legacy transmission.

  - 1 source [Mediatek] observed that Option 2 would increase the latency of A-IoT transmission. Furthermore, it is observed that the two options are not necessarily exclusive.

  - 1 source [Spreadtrum] observed that the signaling overhead of option 2 is higher and the spec impact in RAN1 is lager. If each R2D or D2R transmission resource is scheduled by DCI, option 2 will bring high transmission latency and low transmission efficiency.

  - 1 source [ ZTE] observed that Option 2 has larger standardization impacts, for example the DCI design. And Option 2 may affect the A-IoT timing definitions.

R1-2409806	FL summary#2 on downlink and uplink channel/signal aspects	Apple

From Wednesday session

Agreement

For D2R control information, following is captured in TR 38.769:

14 sources [Huawei, Samsung, Spreadtrum, Fujitsu, Oppo, Ericsson, NTT Docomo, Qualcomm, Futurewei, ZTE, Cewit, Xiaomi, CATT, ETRI] studied feedback information regarding the (un)successful reception of the R2D command at the device and provided following observations:

- 5 sources [Huawei, Futurewei, ZTE, Xiaomi, ETRI] observed that L1-based ACK/NACK is unnecessary or infeasible for ambient IoT, since all D2R transmission are controlled and activated by the reader. Additionally, 1 source [ZTE] observed that it is infeasible for devices to report NACK since it cannot distinguish whether the R2D transmission is dedicated for it and the resource allocation for D2R NACK if the R2D transmission is not successfully detected. And ACK can be transmitted in higher layer signaling.

- 11 sources [Samsung, Spreadtrum, Oppo, Ericsson, NTT Docomo, Qualcomm, Nokia, Mediatek, Cewit, Xiaomi, Apple] observed that ACK/NACK feedback improves transmission reliability, when the BS or intermediate node sends R2D transmission to the device in the command procedure and if the device does not respond ACK/NACK, the BS or intermediate node cannot determine whether the transmission is successful. 1 source [Fujitsu] observed that in the use cases of command where some device actions are required and no D2R message needs to be fed back, only D2R ACK feedback in response to the PRDCH is beneficial for the reader to know that the reception of the PRDCH is successful and the required actions have been done by the device.

Agreement

Following observations on R2D clock-acquisition part are additionally captured in TR 38.769:

- On purpose of SFO estimation/correction based on the clock-acquisition part

  - 3 sources [Nokia, CATT, Qualcomm] provided observations on the applicability of clock-acquisition part for frequency synchronization:

    - 1 source [Nokia] observed that the length of preamble sequence may need to consider also the robustness against SFO

    - 1 source [CATT] observed that device 2a/2b may require higher synchronization accuracy for signal transmission or backscattering and therefore, the design of CAP may be required to accommodate the requirement of additional frequency synchronization and clock calibration for Device 2a/2b.

    - 1 source [Qualcomm] further observed for Option 1, as the CAP duration with high M is decreased, only CAP may not be sufficient for SFO correction and for Option 2, as the CAP duration is fixed and independent from M, the CAP with long enough duration can support SFO correction.

- On purpose of CFO estimation/correction based on the clock-acquisition part

  - 2 sources [Ericsson, Qualcomm] provided observations on the applicability of clock-acquisition part for frequency synchronization:

    - 1 source [Ericsson] observes that the clock-acquisition part can be utilized to solve the frequency synchronization problem without impacting the time-domain sequence, for example by transmitting in some frequency resources and it can be a harmonized solution for both chip duration indication and device frequency synchronization. However, it is further observed that if the time interval between an R2D transmission and the corresponding D2R transmission and if the device loses the timing obtained from the R2D timing acquisition signal due to timing drift at the time for the D2R transmission, then an additional synchronization signal is needed

    - 1 source [Qualcomm] further observed for Option 1, as the CAP duration with high M is decreased, only CAP may not be sufficient for CFO correction and for Option 2, as the CAP duration is fixed and independent from M, the CAP with long enough duration can support CFO correction.

Final summary in R1-2409807.

#### Waveform characteristics of carrier-wave provided externally to the Ambient IoT device

Including interference handling at Ambient IoT UL receiver and at NR base station

R1-2409361	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	TCL

R1-2409367	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Nokia

R1-2409421	On external carrier wave for backscattering based Ambient IoT device	Huawei, HiSilicon

R1-2409487	Discussion on external carrier wave for Ambient IoT	Lenovo

R1-2409516	Discussion on waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CMCC

R1-2409555	Discussion on carrier wave for Ambient IoT	ZTE Corporation, Sanechips

R1-2409601	Considerations for Waveform characteristics of carrier-wave	Samsung

R1-2409640	Discussion on waveform characteristics of external carrier-wave for Ambient IoT	Spreadtrum, UNISOC

R1-2409685	Discussion on CW waveform and interference handling at AIoT UL receiver	vivo

R1-2409804	On remaining details of carrier waveform and interference handling for AIoT	Apple

R1-2409867	Discussion on the waveform of carrier-wave for the Ambient IoT	NEC

R1-2409900	Discussion on waveform characteristics of carrier-wave	Xiaomi

R1-2409945	Discussion on the waveform characteristics of carrier-wave for the Ambient IoT device	CATT

R1-2410095	Discussion on Waveform characteristics of carrier-wave provided externally to the A-IoT device	OPPO

R1-2410128	Waveform characteristics of carrier wave provided externally to the Ambient IoT device	Ericsson

R1-2410270	Discussion on carrier-wave for Ambient IoT	ETRI

R1-2410290	Considerations on carrier-wave transmission for Ambient IoT	LG Electronics

R1-2410314	Carrier-wave for Ambient IoT	InterDigital, Inc.

R1-2410393	Study on waveform characteristics of carrier-wave for Ambient IoT	NTT DOCOMO, INC.

R1-2410482	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	Qualcomm Incorporated

R1-2410518	Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	MediaTek Inc.

R1-2410580	Discussion on Waveform characteristics of carrier-wave provided externally to the Ambient IoT device	CEWiT

R1-2410773	FL summary#1 on CW waveform characteristics for A-IoT	Moderator (Spreadtrum)

From Tuesday session

Agreement

For CW transmission with large frequency shift, capture the following observation in TR 38.769.

If large frequency shift is supported by device 2a, the D2R backscattering can be transmitted in a different carrier as CW for D2R backscattering (e.g., CW in DL spectrum and D2R in UL spectrum, or CW in UL spectrum and D2R in DL spectrum), and the followings are observed. - In-band full-duplex capability is not needed for CW transmission and D2R reception at D2R receiver side, if the D2R receiver only supports device 2a with large frequency shift capability. - No interference from CW to the corresponding D2R reception at D2R receiver side. - For CW in DL spectrum and D2R in UL spectrum, higher CW transmission power can be assumed in the DL spectrum than that of in the UL spectrum.

Agreement

Adopt the following TP for the gap between two frequency hops in section 6.8.2 of TR 38.769:

<Unchanged parts are omitted>

For the gap between two tones or two frequency hops of a single tone to be able to leverage frequency diversity gain, bandwidth and spectrum characteristics of the D2R transmission, and coherence bandwidth, should be taken into account.

<Unchanged parts are omitted>

Agreement

Capture the D2R reception performance comparison as the TP below in section 6.8.2 of TR 38.769:

<Unchanged parts are omitted>

Table 6.8.2-1: Observations and/or comparisons of CW waveform candidates

| CW waveform characteristics | Waveform 1 compared to Waveform 2 NOTE 1: Waveform 1 without frequency hopping NOTE 2: Waveform 2 with both tones from the same CW node | Waveform 1 with frequency hopping (2-hops) compared to waveform 1 without frequency hopping |
| --- | --- | --- |
| D2R reception performance | <Unchanged parts are omitted> | Observations on frequency diversity: - 6 sources observed that waveform 1 with frequency hopping should be used together with bit-level repetition or TB-level repetition. - 3 sources further observed that the unaligned timing (due to SFO at A-IoT device) between reader and devices makes it hard to align the boundary of each hop of the external carrier-wave with a corresponding bit or block repetition of the D2R transmission. - 2 sources observed that for milli-second level CW duration for each frequency hop/block repetition, the diversity gain loss caused by miss-alignment of a few micro-seconds level is marginal, TB level repetition can tolerate some time mis-alignment. - 1 source [R1-2408851] observed that waveform 1 with frequency hopping achieves frequency diversity gain by using polynomial sweeping-based CC encoding (proposed by [R1-2408851]) instead of repetitions When bit-level repetition, TB-level repetition or polynomial sweeping-based CC encoding is not used, - Waveform 1 with frequency hopping provides [0, 0.5] dB frequency diversity gain compared to waveform 1 without frequency hopping at 1% or 10% BLER in a fading channel for a 1Rx receiver and a 1Tx CW transmitter. - In a TDL-A fading channel with 30ns delay spread - For 10MHz gap between two hops, 1 source observed 0 dB@10%BLER frequency diversity gain. - In a TDL-A fading channel with 150ns delay spread - For 1.65MHz gap between two hops, 1 source observed 0.5 dB@10%BLER and 0 dB@1%BLER frequency diversity gain. When bit-level repetition, TB-level repetition is used, - Waveform 1 with frequency hopping provides [0.5, 8] dB frequency diversity gain compared to waveform 1 without frequency hopping at 1% or 10% BLER in a fading channel for a 1Rx receiver and a 1Tx CW transmitter, at least depending on the gap between the two hops and the channel's coherence bandwidth. - In a TDL-A fading channel with 30ns delay spread - For the gap of 2.16MHz, 1 source observed 4dB@1% BLER and 2dB@10% BLER frequency diversity gains. - For the gap between [5MHz, 10MHz], the frequency diversity gains at 1% BLER target observed by 2 sources are within [5.8, 8] dB, and the frequency diversity gains at 10% BLER target observed by 3 sources are within [1.2, 6] dB. The 8 dB gain is achieved under the assumption of ideal channel estimation. - In a TDL-D fading channel with 30ns delay spread - For 10MHz gap, 1 source observed 1 dB@1%BLER and 0.5dB@10%BLER frequency diversity gain - In a TDL-A fading channel with 150ns delay spread - For the gap between [1.65MHz, 2.16MHz], the frequency diversity gains at 1% BLER target observed by 2 sources are within [5.5, 8] dB, and the frequency diversity gain at 10% BLER target observed by 2 sources is 5 dB. The 8 dB gain is achieved under the assumption of ideal channel estimation. When polynomial sweeping-based CC encoding [R1-2408851] is used or assumed - In a TDL-A fading channel with 30ns delay spread, and for 10MHz gap between two hops, 1 source [R1-2408851] observed that waveform 1 with frequency hopping provides 4 dB frequency diversity gain compared to waveform 1 without frequency hopping at 1% BLER for a 1Rx receiver and a 1Tx CW transmitter. The following observations were made by one source ([R1-2408854]): - Waveform 1 with frequency hopping and antenna hopping provides [1, 9] dB spatial diversity and frequency diversity compared to Waveform 1 with no frequency hopping and no antenna hopping at 1% or 10% BLER in a fading channel for a 1Rx receiver and a 2Tx CW transmitter, at least depending on the gap between the two tones and the channel's coherence bandwidth and assuming the ideal channel estimation. - In a TDL-D fading channel with 30ns delay spread, for the gap of 10MHz, the gain is 2.6 dB at 1% BLER and 1 dB at 10% BLER. - In a TDL-A fading channel with 30ns delay spread, for the gap of 2.16MHz, the gain is 7dB at 1% BLER target and 3.5dB at 10% BLER target. For the gap of 10MHz, the gain is 9dB at 1% BLER target and 5dB at 10% BLER target. Note: The total transmission power is assumed the same for both waveforms. Note: The above evaluations assume the same time domain resources overhead for waveform 1 with or without frequency hopping. |
| Spectrum utilization of backscattered signal corresponding to the CW waveforms | <Unchanged parts are omitted> | <Unchanged parts are omitted> |
| CW interference suppression at D2R receiver | <Unchanged parts are omitted> | <Unchanged parts are omitted> |
| Relative complexity of CW generation | <Unchanged parts are omitted> | <Unchanged parts are omitted> |

<Unchanged parts are omitted>

Final FL summary on CW waveform characteristics for A-IoT in R1-2410927.

## Enhancements of network energy savings for NR

Please refer to RP-242354 for detailed scope of the WI.

[119-R19-NES] – Ajit (Ericsson)

Email discussion on Rel-19 NES enhancement

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

From Friday session

R1-2410886	Draft reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell	Moderator (Ericsson)

Decision: The draft LS is endorsed. Final LS is approved in R1-2410917.

### On-demand SSB SCell operation

R1-2409413	On-demand SSB SCell operation for eNES	Huawei, HiSilicon

R1-2409448	Discussion on on-demand SSB SCell operation	Quectel

R1-2409517	Discussion on on-demand SSB SCell operation	CMCC

R1-2409544	Discussion on on-demand SSB SCell operation	Panasonic

R1-2409556	Discussion on on-demand SSB for NES	ZTE Corporation, Sanechips

R1-2409602	On-demand SSB SCell operation	Samsung

R1-2409641	Discussion on on-demand SSB SCell operation	Spreadtrum, UNISOC

R1-2409686	Discussions on on-demand SSB Scell operation	vivo

R1-2409712	On-demand SSB SCell Operation	Nokia, Nokia Shanghai Bell

R1-2409735	Discussion on on-demand SSB SCell operation	Intel Corporation

R1-2409754	On demand SSB	Tejas Networks Limited

R1-2409772	Discussion on on-demand SSB SCell operation	InterDigital, Inc.

R1-2409808	On-demand SSB SCell Operation	Apple

R1-2409901	Discussion on on-demand SSB SCell operation	Xiaomi

R1-2409946	Discussion on on-demand SSB SCell operation	CATT

R1-2409965	Discussion on On-Demand SSB SCell operation	Transsion Holdings

R1-2410004	Discussion on on-demand SSB operation for SCell	China Telecom

R1-2410032	Discussion of on-demand SSB Scell operation	FUTUREWEI

R1-2410062	Discussion on on-demand SSB SCell operation	Fujitsu

R1-2410074	Discussion on the enhancement to support on-demand SSB SCell operation	OPPO

R1-2410156	On-demand SSB SCell Operation	Google

R1-2410209	Discussion on on-demand SSB for SCell operation	NEC

R1-2410228	On-demand SSB SCell operation	Sony

R1-2410252	On-demand SSB SCell operation	Lenovo

R1-2410271	Discussion on On-demand SSB SCell operation	ETRI

R1-2410291	On-demand SSB SCell operation	LG Electronics

R1-2410394	Discussion on on-demand SSB SCell operation	NTT DOCOMO, INC.

R1-2410430	Discussion on on-demand SSB SCell operation	ITRI

R1-2410441	On-demand SSB SCell operation	Ericsson

R1-2410483	On-demand SSB operation for Scell	Qualcomm Incorporated

R1-2410505	Discussion on details of on-demand SSB operation on SCell	Sharp

R1-2410521	On-demand SSB SCell operation	MediaTek Inc.

R1-2410550	DCI based signaling for on-demand SSB 	ASUSTeK

R1-2410563	Discussion of On-demand SSB SCell operation	Mavenir

R1-2410581	Discussion on on-demand SSB Scell operation	CEWiT

From AI 5

R1-2409350	LS on SSB relation in On-demand SSB and SSB adaptation on Scell	RAN4, Ericsson

Decision: Response needed – Ajit (Ericsson).

R1-2410779	Summary #1 of on-demand SSB for NES	Moderator (LG Electronics)

From Tuesday session

Agreement

Response to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?) of Obj.1:

- The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. RAN1 is discussing whether to introduce other periodicity value(s) for on-demand SSB.

- The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB.

- RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB.

Further update to be made based on RAN1#119 progress.

Agreement

Response to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?) of Obj.1:

- The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB.

R1-2410780	Summary #2 of on-demand SSB for NES	Moderator (LG Electronics)

From Wednesday session

Agreement

Response to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?) of Obj.1:

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications

  - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same

- At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

R1-2410781	Summary #3 of on-demand SSB for NES	Moderator (LG Electronics)

From Thursday session

Agreement

- For a cell supporting on-demand SSB SCell operation, support to configure time domain location of on-demand SSB per on-demand SSB periodicity by RRC for both Case #1 and Case #2.

  - For Case #1 (i.e., No always-on SSB on the cell),

    - Based on two parameters, where one is to indicate SFN offset from a reference point and the other is to indicate half frame index

      - The reference point is SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0

      - If SFN offset parameter is NOT configured, UE assumes SFN offset set to 0.

      - If half frame index parameter is NOT configured, UE assumes half frame index set to 0.

      - The value range of SFN offset is 0 to 15 unless longer periodicity for on-demand SSB than 160 ms is introduced.

      - The value range of half frame index is 0 or 1.

  - For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), down-select one of the following alternatives.

    - Alt A: Same as for Case #1

    - Alt B: Based on a single parameter which is to indicate the time offset between always-on SSB and on-demand SSB (e.g., similar to ssb-TimeOffset)

Agreement

New periodicity value for on-demand SSB other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms) is NOT introduced in Rel-19.

Agreement

Down-select at least one of the following alternatives.

- Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB.

- Alt 2: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt 3: Do not support the case where always-on SSB is CD-SSB on a synchronization raster.

Down-select at least one of the following alternatives.

- Alt A: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB can be same or different from the frequency location of always-on SSB, subject to its configuration.

- Alt B: If always-on SSB is CD-SSB and not on a synchronization raster, the frequency location of on-demand SSB is the same as the frequency location of always-on SSB

- Alt C: Do not support the case where always-on SSB is CD-SSB and not on a synchronization raster.

R1-2410782	Summary #4 of on-demand SSB for NES	Moderator (LG Electronics)

From Friday session

Agreement

Response to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?) of Obj.1:

- RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst

- RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB

Agreement

For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective.

- Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication

  - Deactivation by RRC is up to RAN2

  - FFS: Which scenario Option 1 is used

- Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated

  - FFS: Whether Option 4, 4a is needed in addition to Option 2

  - FFS: Whether the value of N can be implicitly determined using a timer

Final summary in R1-2410912.

### On-demand SIB1 for idle/inactive mode UEs

R1-2409414	Discussion on on-demand SIB1 for eNES	Huawei, HiSilicon

R1-2409518	Discussion on on-demand SIB1 for UEs in idle/inactive mode	CMCC

R1-2409545	Discussion on on-demand SIB1 for idle/inactive mode UEs	Panasonic

R1-2409557	Discussion on on-demand SIB1 for NES	ZTE Corporation, Sanechips

R1-2409603	On-demand SIB1 for idle/inactive mode UEs	Samsung

R1-2409642	Discussion on on-demand SIB1 for idle/inactive mode UEs	Spreadtrum, UNISOC

R1-2409687	Discussions on on-demand SIB1 for idle/inactive mode UEs	vivo

R1-2409711	Discussion on on-demand SIB1 for idle/inactive mode UEs	KT Corp.

R1-2409713	On-demand SIB1 for Idle/Inactive mode UEs	Nokia, Nokia Shanghai Bell

R1-2409736	Discussion on-demand SIB1 for idle/inactive mode UEs	Intel Corporation

R1-2409755	On demand SIB1	Tejas Networks Limited

R1-2409773	Discussion on on-demand SIB1 for idle/inactive mode UEs	InterDigital, Inc.

R1-2409809	On On-demand SIB1 for IDLE/INACTIVE mode UEs	Apple

R1-2409902	Discussion on on-demand SIB1 for idle/inactive mode UEs	Xiaomi

R1-2409947	Discussion on on-demand SIB1	CATT

R1-2409966	Discussion on on-demand SIB1 transmission for idle/inactive mode UEs	Transsion Holdings

R1-2410033	Discussion of on-demand SIB1 for idle/inactive mode UEs	FUTUREWEI

R1-2410075	Discussion on the enhancement to support on demand SIB1 for idle/inactive mode UE	OPPO

R1-2410132	Discussion on on-demand SIB1 transmission for network energy savings	Fujitsu

R1-2410157	On-demand SIB1 for Idle/Inactive Mode UE	Google

R1-2410210	Discussion on on-demand SIB1 for UEs in idle/inactive mode	NEC

R1-2410229	On-demand SIB1 for idle/inactive mode UEs	Sony

R1-2410253	On-demand SIB1 for idle/inactive mode UEs	Lenovo

R1-2410272	On-demand SIB1 for idle/inactive mode UEs	ETRI

R1-2410292	On-demand SIB1 for idle/inactive mode UEs	LG Electronics

R1-2410301	Views on On-demand SIB1 operation for idle/inactive UEs	Vodafone, Deutsche Telekom

R1-2410350	Discussion on on-demand SIB1 for idle/inactive mode UEs	DENSO CORPORATION

R1-2410361	Discussion on on-demand SIB1 transmission for idle UEs	Sharp

R1-2410368	Discussion on on-demand SIB1 in idle/inactive mode	CAICT

R1-2410395	Discussion on on-demand SIB1 for idle/inactive mode UEs	NTT DOCOMO, INC.

R1-2410442	On-demand SIB1 for UEs in idle/inactive mode for NES	Ericsson

R1-2410484	On-demand SIB1 procedure	Qualcomm Incorporated

R1-2410522	On-demand SIB1 for idle or inactive mode UEs	MediaTek Inc.

R1-2410561	On-demand SIB1 for NES	Fraunhofer IIS, Fraunhofer HHI

R1-2410582	Discussion on on-demand SIB1	CEWiT

R1-2410677	FL summary 1 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

From Tuesday session

Agreement

For parameters agreed for UL WUS, whether it is mandatory or optional is for further discussion.

Agreement

At least for RO validation determination for WUS transmission, the parameter “ssb-PeriodicityServingCell” is included in the UL-WUS configuration at least for TDD system.

- FFS: FDD system

Agreement

Confirm the working assumption below from RAN1 #118-bis to include it in the UL-WUS configuration.

- (working assumption) ULSubCarrierSpacing

Agreement

CORESET0 of NES cell is used for RAR CORESET of that NES cell.

Agreement

At least for the case where SSB is transmitted on sync-raster, the indication of the quantity K_SSB (defined in TS 38.211 as subcarrier offset from subcarrier 0 in common resource block NCRBSSB to the lowest-numbered subcarrier of the SS/PBCH block) is included in the UL-WUS configuration for FDD and TDD NES cell.

R1-2410678	FL summary 2 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

From Wednesday session

Agreement

The parameter “n-TimingAdvanceOffset” is included in the UL-WUS configuration.

Agreement

The reference time point to determine the window starting time for on-demand SIB1 is based on the RAR window for UL WUS wherein UE successfully received a RAR.

- FFS: Use starting or ending slot of the RAR window as reference time

Agreement

The duration of the time window for on-demand SIB1 is indicated in the UL WUS configuration.

- Unit of the duration is in slot

- FFS: Starting time

R1-2410679	FL summary 3 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

From Thursday session

Agreement

On how to use PDCCH-ConfigSIB1 for R19 NES-capable UE in MIB of NES Cell when K_SSB = 30 in FR1 or K_SSB = 14 in FR2 on NES cell if SSB of NES cell is on the sync raster, down-select from the following options:

- Option 1: Use it to indicate frequency assistance information to search SSB for Cell A

  - FFS: Details on the range/granularity of the frequency assistance information

  - FFS: Potential impact to RAN3

- Option 2: Use it to indicate searchSpaceZero and controlResourceSetZero of OD-SIB1

- Option 3: Above feature is not supported.

R1-2410680	FL summary 4 for on-demand SIB1 in idle/inactive mode	Moderator (MediaTek)

From Friday session

Agreement

At least the contents of RAR in response to SI request are included in RAR in response to UL WUS.

Conclusion

There is no RAN1 consensus to support UL WUS repetition in Rel-19.

Agreement

The mapping rule between ra-PreambleStartIndex for OD-SIB1 and SSB follows the mapping rule between ra-PreambleStartIndex for OSI request and SSB as in legacy specification.

Final summary in R1-2410681.

### Adaptation of common signal/channel transmissions

R1-2409415	On common channel/signal adaptation for eNES	Huawei, HiSilicon

R1-2409470	Adaptation of common signal/channel transmissions	Quectel

R1-2409519	Discussion on adaptation of common signal/channel transmissions	CMCC

R1-2409558	Discussion on common signal/channel for NES	ZTE Corporation, Sanechips

R1-2409604	Adaptation of common signal/channel transmissions	Samsung

R1-2409643	Discussion on adaptation of common signal/channel transmissions	Spreadtrum, UNISOC

R1-2409688	Discussions on adaptation of common signal/channel transmissions	vivo

R1-2409714	Adaptation of common signal/channel transmissions	Nokia, Nokia Shanghai Bell

R1-2409737	Discussion on adaptation of common signal/channel transmission	Intel Corporation

R1-2409756	Adaptation of common signals/channels	Tejas Networks Limited

R1-2409774	Discussion on adaptation of common signal/channel transmissions	InterDigital, Inc.

R1-2409810	On adaptation of common signal/channel for NES enhancements	Apple

R1-2409903	Discussion on adaptation of common signal and channel transmissions	Xiaomi

R1-2409948	Discussion on adaptation of common signal/channel transmissions	CATT

R1-2409967	Discussion on adaptive transmission of common signal or common channel	Transsion Holdings

R1-2410005	Discussion on common signal/channel adaptation	China Telecom

R1-2410034	Discussion of the adaptation of common signal/channel transmissions	FUTUREWEI

R1-2410076	Discussion on adaptation of common signal/channel transmission	OPPO

R1-2410134	Discussion on adaptation of common signal / channel transmissions	Fujitsu

R1-2410158	Adaptation of Common Signals	Google

R1-2410180	Discussion on adaptation of common signal channel transmissions	HONOR

R1-2410211	Discussion on adaptation of common signal/channel transmissions	NEC

R1-2410230	Adaptation of common signal/channel transmissions	Sony

R1-2410247	Discussion on adaptation of common signal/channel transmission	Panasonic

R1-2410273	Adaptation of common signal/channel transmissions	ETRI

R1-2410293	Adaptation of common signal/channel transmissions	LG Electronics

R1-2410302	Adaptation of common signals and channels	Lenovo

R1-2410362	Discussion on adaptation of common signal/channel transmissions	Sharp

R1-2410396	Discussion on adaptation of common signal/channel transmissions	NTT DOCOMO, INC.

R1-2410443	Adaptation of common signal/channel transmissions for NES	Ericsson

R1-2410485	Adaptation of common channel transmissions	Qualcomm Incorporated

R1-2410523	Adaptation of common signal/channel transmissions	MediaTek Inc.

R1-2410583	Discussion on adaptation of common signal and channel transmissions	CEWiT

From AI 5

R1-2409350	LS on SSB relation in On-demand SSB and SSB adaptation on Scell	RAN4, Ericsson

Decision: Response needed – Ajit (Ericsson).

R1-2410784	Summary#1 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

From Tuesday session

Agreement

Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?):

- RAN1 agreed that at least SSB burst periodicity is adapted.

- There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options.

Agreement

Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?):

- The frequency location is same before and after SSB adaptation.

Agreement

Reply to Q3(What is the spatial relation before and after SSB adaptation?):

- There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation.

- At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

Further update to be made based on RAN1#119 progress if any.

Agreement

At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16]

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

    - Alt 2-6: Network configured time period

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

R1-2410785	Summary#2 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

From Wednesday session

Conclusion

There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode).

Working Assumption

For DCI-based adaptation for additional PRACH resources,

- Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI

  - Option 1: Use reserved bits in the DCI format

    - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format

  - Option 2: Use Bits 5-8 within the Short Message (from upper layers)

    - Note: Availability should be confirmed by checking with RAN2.

  - Option 3: Use bits available for both Option 1 and Option 2

FFS: Payload size for adaptation for additional PRACH resources

R1-2410786	Summary#3 of AI 9.5.3 for R19 NES	Moderator (Ericsson)

From Thursday session

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - [DCI payload size = 1 bit]

  - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources

  - FFS: details

R1-2410887	Summary#4 of AI 9.5.3 for R19 NES	Moderator(Ericsson)

From Friday session

Agreement

Confirm the following working assumption from RAN1#118bis.

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

Final summary in R1-2410888.

## Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

Please refer to RP-241824 for detailed scope of the WI.

[119-R19-LP_WUS] – Xin (vivo)

Email discussion on Rel-19 LP-WUS/WUR

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2410885	Summary of RAN1 agreements on LP-WUS/WUR for NR	Rapporteur (vivo)

For information, not treated.

### LP-WUS and LP-SS design

R1-2409423	Signal Design of LP-WUS and LP-SS	Huawei, HiSilicon

R1-2409444	LP-WUS and LP-SS design	Nokia

R1-2409456	Discussion on LP-WUS and LP-SS design for low power WUS	InterDigital, Inc.

R1-2409476	Discussion on LP-WUS and LP-SS Design 	TCL

R1-2409520	Discussion on LP-WUS and LP-SS design	CMCC

R1-2409534	Discussion on LP-WUS and LP-SS Design	EURECOM

R1-2409546	Discussion on the LP-WUS and LP-SS design	Panasonic

R1-2409559	Discussion on LP-WUS design	ZTE Corporation, Sanechips

R1-2409605	Discussion on LP-WUS and LP-SS design	Samsung

R1-2409644	Discussion on LP-WUS and LP-SS design	Spreadtrum, UNISOC

R1-2409689	Discussion on LP-WUS and LP-SS design	vivo

R1-2409811	LP-WUS and LP-SS design	Apple

R1-2409860	Discussion on LP-WUS and LP-SS design	NEC

R1-2409904	Discussion on LP-WUS and LP-SS design	Xiaomi

R1-2409949	Design of LP-WUS and LP-SS	CATT

R1-2410036	Discussion on LP-WUS and LP-SS Design	FUTUREWEI

R1-2410646	Signal design for LP-WUS and LP-SS	OPPO	(rev of R1-2410071)

R1-2410135	Discussion on LP-WUS and LP-SS design	Fujitsu

R1-2410181	Discussion on LP-WUS and LP-SS design	HONOR

R1-2410231	LP-WUS and LP-SS design	Sony

R1-2410294	Discussion on LP-WUS and LP-SS design	LG Electronics

R1-2410310	LP-SS Code Sets and Comparison	Everactive

R1-2410397	Discussion on LP-WUS and LP-SS design	NTT DOCOMO, INC.

R1-2410419	Discussion on LP-WUS and LP-SS design	Sharp

R1-2410444	LP-WUS and LP-SS design	Ericsson

R1-2410486	LP-WUS and LP-SS design	Qualcomm Incorporated

R1-2410510	LP-WUS and LP-SS design	MediaTek Inc.

R1-2410556	Discussion on LP-WUS and LP-SS design	Lenovo

R1-2410569	On LP-WUS and LP-SS design	Nordic Semiconductor ASA

R1-2410747	Summary #1 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

From Monday session

Agreement

Update the existing agreement as below

In case of overlaid OFDM sequence carrying information, support option 2:

- Option 2: One sequence is selected from multiple candidates overlaid OFDM sequences on each OOK ‘ON’ symbol, and OFDM-based LP-WUR obtain LP-WUS information at least by overlaid OFDM sequence(s). Consider the following two sub-options for potential down-selection.

  - Option 2-1: The overlaid OFDM sequence(s) carry part of information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by OFDM sequence(s) and location of the OFDM sequence(s)/OOK ‘ON’ symbols.

  - Option 2-2: The overlaid OFDM sequence(s) carry all information bits of LP-WUS. OFDM-based LP-WUR can obtain the whole information bits by the overlaid OFDM sequence(s)

    - FFS the how the information bits are carried by the overlaid OFDM sequence(s)

Note: the overlaid OFDM sequence in each OOK ‘ON’ symbol can be different according to information bits to be carried by the overlaid OFDM sequence within the LP-WUS.

R1-2410748	Summary #2 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

From Tuesday session

Agreement

For the overlaid OFDM sequence in time domain, the sequence is generated based on

,

- M=1, is given by the largest prime number such that,is the overlaid OFDM sequence length.

  - The base overlaid sequence is generated by extension of

,

  - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s)

,,

- M=2, 4, is down-selected from the following:

  - Alt1: is given by the largest prime number such that,is the overlaid OFDM sequence length.

    - The base overlaid sequenceis generated by extension of

,

    - With CS(s) applied to the base overlaid OFDM sequence if any:denotes the potential cyclic shift (s)

,

    - Note it doesn’t preclude any pulse shaping scheme if any.

  - Alt2:is given by the largest prime number such that,  is the overlaid OFDM sequence length.

    - The base overlaid sequenceis generated by inserting zeros before and/or after . The total number of zeros is.

For example,,

    - With CS(s) applied to the base overlaid OFDM sequence if any: denotes the potential cyclic shift (s)

For example,,

where,

    - FFS the value of G, with G>0.

- FFS value(s) of root q and/or CS  for generating the candidate sequences if applied

- FFS on whether changes are needed to handle intra-/inter-cell interference

- Note: the overlaid OFDM sequence in time domain is based on potential modification to ZC sequence as listed above.

  - Above overrides any previous RAN1 agreement / working assumption

Agreement

At least for M>1, for the overlaid OFDM sequence in time domain, down-select the sequence length from the following:

- Alt1:, with

  - FFS how to map the generated overlaid OFDM sequence to the frequency domain

- Alt2:

- Note: X is the number of RBs of LP-WUS/LP-SS bandwidth (blanked guard RBs are not included)

Agreement

CS(s) and/or root(s) used for overlaid OFDM sequence in the time domain are derived from RRC signalling:

- FFS: the set of values of CS and root for configuration

- FFS: details of the RRC signaling

Agreement

For the length L of LP-SS binary sequence, limit the selection to the following:

- M=1, L= {4, 6, 8}

- M=2, L= {8, 12, 16}

- M=4, L= {16, 32}

Agreement

To compare LP-SS binary sequence performance, cross-correlation is deprioritized over sync accuracy, i.e., cross-correlation within a set of 4 sequences is further compared for different sets of sequences with similar sync accuracy.

R1-2410837	Summary #3 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

From Wednesday session

Agreement

For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120:

- Alt1: The M values for LP-WUS and LP-SS are always same.

- Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS.

- Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

R1-2410838	Summary #4 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

From Thursday session

Agreement

Regarding the maximum number of candidates overlaid sequences to carry LP-WUS information per OOK ON chip for one cell:

- support maximum 4 candidates overlaid sequences for M=4

For future meetings:

Companies are encouraged to consider [H][FL4] Proposal 3.3-1 in R1-2410838.

Agreement

Regarding whether to support additional sync signal to LP-SS, down-selection from the following:

- Option 1A: No additional sync signal, LP-SS periodicity =320ms

  - Additionally support at least one of [80ms,160ms]

    - FFS: whether different values can be applicable for different M values

- Option 1B: No additional sync signal, LP-SS periodicity =320ms

  - No additional support of other periodicities

- Option 2: With additional sync signal, LP-SS periodicity =320ms

  - FFS additionally support other LP-SS periodicities

  - FFS details on additional sync signal

  - FFS additional sync signal is configurable and/or conditionally present

R1-2410883	Summary #5 of discussions on LP-WUS and LP-SS design	Moderator (vivo)

From Friday session

For future meetings:

Companies are encouraged to consider the following for future discussions

Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs,

- Option 1: A bitmap with each bit corresponding to [one or more] UEs

  - 1A No CRC

    - Per bit coding, e.g., one bit is mapped to one of 2 codeword/sequences

    - The maximum number of codewords targeting per bit position in the bitmap for a UE is 1

    - FFS coded bit length

    - Single MO

  - 1B No CRC

    - Bitmap-level coding, e.g., one bitmap of L bits is mapped to one of 2L codeword/sequences

    - The maximum number of codewords/sequences targeting for a UE is 2L-1

    - Single MO

  - 1C With CRC

    - FFS CRC length

    - Single MO

- Option 2: A codepoint value corresponding to [one or more] UEs

  - 2A only 1-1 mapping from codepoint to UE

    - The maximum number of codepoints targeting per MO for a UE is 1

    - FFS the maximum number of MOs to be detected by a UE

  - 2B 1-to-1, 1-to-all mapping from codepoint to UEs

    - The maximum number of codepoints targeting per MO for a UE is up to 2

    - FFS the maximum number of MOs to be detected by a UE

  - 2C 1-to-X mapping from codepoint to UEs,

    - The maximum number of codepoints targeting per MO for a UE is up to

    - FFS the maximum number of MOs to be detected by a UE

- Note: L is the number of UEs associated monitoring the same WUS

Agreement

For RRC idle/inactive state, for the case where associated CD-SSB and initial DL BWP have the same SCS at least when the associated CD-SSB and LP-WUS are on the same carrier

- The single SCS for LP-WUS/LP-SS is same as the associated CD-SSB

- FFS: Case where associated CD-SSB and initial DL BWP have different SCSs

- FFS: Which initial BWP for RedCap

R1-2410925	Template for collecting LP-SS binary sequence results	Moderator (vivo)

Agreement

For collecting LP-SS binary sequence results

- Endorse the template for collecting LP-SS binary sequence results in R1-2410925

- Email discussion for collecting LP-SS binary sequences by 10th Jan.

- Please include your cross-checking results (e.g., a separate excel sheet) in the t-doc for #120.

R1-2410884	Final summary of discussions on LP-WUS and LP-SS design	Moderator (vivo)

### LP-WUS operation in IDLE/INACTIVE modes

R1-2409424	Procedures and functionalities of LP-WUS in IDLE/INACTIVE mode	Huawei, HiSilicon

R1-2409445	LP-WUS operation in IDLE/Inactive mode	Nokia

R1-2409457	Discussion on LP-WUS operation in IDLE/INACTIVE modes	InterDigital, Inc.

R1-2409477	Discussion on LP-WUS Operation in IDLE/INACTIVE modes	TCL

R1-2409521	Discussion on LP-WUS operation in IDLE/INACTIVE modes	CMCC

R1-2409547	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Panasonic

R1-2409560	Discussion on LP-WUS operation in IDLE/INACTIVE mode	ZTE Corporation, Sanechips

R1-2409606	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Samsung

R1-2409645	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Spreadtrum, UNISOC

R1-2409690	Discussion on LP-WUS operation in IDLE/INACTIVE modes	vivo

R1-2409812	LP-WUS operation in IDLE/INACTIVE modes	Apple

R1-2409861	Discussion on LP-WUS operation in RRC IDLE/INACTIVE mode	NEC

R1-2409905	Discussion on LP-WUS operation in Idle/Inactive modes	Xiaomi

R1-2409950	System design and procedure of LP-WUS operation for UE in IDLE/Inactive Modes	CATT

R1-2410037	Discussion on LP-WUS Operation in IDLE/INACTIVE Modes	FUTUREWEI

R1-2410072	Further consideration on LP-WUS operation in RRC_IDLE/INACTIVE modes	OPPO

R1-2410232	LP-WUS operation in IDLE / INACTIVE modes	Sony

R1-2410274	Discussion on LP-WUS operation in IDLE/INACTIVE modes	ETRI

R1-2410295	Discussion on LP-WUS operation in IDLE/INACTIVE modes	LG Electronics

R1-2410300	On LR and MR operating frequencies	Vodafone, Deutsche Telekom

R1-2410398	Discussion on LP-WUS operation in IDLE/INACTIVE modes	NTT DOCOMO, INC.

R1-2410420	Discussion on LP-WUS operation in IDLE/INACTIVE modes	Sharp

R1-2410445	LP-WUS operation in IDLE and INACTIVE modes	Ericsson

R1-2410487	LP-WUR operation in idle and inactive modes	Qualcomm Incorporated

R1-2410511	LP-WUS operation in IDLE/INACTIVE modes	MediaTek Inc.

R1-2410557	Discussion on LP-WUS operation in Idle/Inactive modes	Lenovo

R1-2410568	On LP-WUS operation in IDLE/Inactive	Nordic Semiconductor ASA

R1-2409814	Summary #1 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

From Monday session

Agreement

Confirm the following working assumption with the addition of the note at the bottom:

Working Assumption

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode,

- The UE may start LP-WUS monitoring if

  - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

- The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

  - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

  - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied

- FFS the serving cell measurement metrics

- The entry/exit thresholds can be configured separately for different types of LR

- It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB.

- Note: This may be revisited based on the RAN2/RAN4 discussion.

Note: this does not intend to impact any agreements and working assumptions made in RAN1/RAN2 after the working assumption.

Agreement

At least support the case that the number of beams for LP-WUS/LP-SS is the same as the number of SSB beams.

Conclusion

Do not support one-to-multiple mapping between the LP-WUS/LP-SS beams and the SSB beams.

Conclusion

No additional RAN1 specification impact specific to support multiple-to-one mapping between the LP-WUS/LP-SS beams and the SSB beams.

R1-2409815	Summary #2 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

From Tuesday session

Agreement

For the offset value(s) between an LO and a reference PO/PF, down-select between

- Option 1-1

- Configurability between Option 1-1 and Option 1-2

- Option 2B-1

Agreement

At least for the 1:1 LO to PO mapping,

- The maximum value of M for Option A or Option B with G=1 (if either of them is supported) is 4.

Conclusion

There is no RAN1 consensus to support additional codepoints indicating wake-up at least for 1:1 LO to PO mapping.

R1-2409816	Summary #3 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

From Wednesday session

For future meetings:

Consider the following alternatives for UE capability report on the wake-up delay:

- Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}.

  - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization.

  - FFS: translation of wake-up delay for different SSB periodicities

- Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}.

- Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time.

  - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]}

  - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively.

- Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

Agreement

Each LP-WUS or LP-SS is QCLed with an SSB with [select one from ‘typeA’, ‘typeC’], and when applicable, ‘typeD’ with the same SSB.

R1-2410861	Summary #4 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

From Thursday session

Working Assumption

If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration.

Conclusion

From RAN1 perspective, there is no consensus on the following proposal:

UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if	the UE has not received LP-WUS for a long time.

R1-2410862	Summary #5 on LP-WUS operation in IDLE/INACTIVE mode	Moderator (Apple)

From Friday session

For future meetings:

Companies are encouraged to consider section 3.5 of R1-2410862 for further discussions in future meetings.

Final summary in R1-2410863.

### LP-WUS operation in CONNECTED modes

R1-2409425	Procedures and functionalities of LP-WUS in CONNECTED mode	Huawei, HiSilicon

R1-2409446	LP-WUS operation in CONNECTED mode	Nokia

R1-2409458	Discussion on RRC CONNECTED mode LP-WUS monitoring	InterDigital, Inc.

R1-2409522	Discussion on LP-WUS operation in CONNECTED mode	CMCC

R1-2409561	Discussion on LP-WUS operation in CONNECTED mode	ZTE Corporation, Sanechips

R1-2409607	Discussion on LP-WUS operation in CONNECTED modes	Samsung

R1-2409646	Discussion on LP-WUS operation in CONNECTED modes	Spreadtrum, UNISOC

R1-2409691	Discussion on LP-WUS operation in CONNECTED modes	vivo

R1-2409813	LP-WUS operation in CONNECTED modes	Apple

R1-2409862	Discussion on LP-WUS operation in RRC CONNECTED mode	NEC

R1-2409875	Discussion on LP-WUS operation in CONNECTED mode	Panasonic

R1-2409906	Discussion on LP-WUS operation in Connected mode	Xiaomi

R1-2409951	System design and procedure of LP-WUS operation for UE in CONNECTED Modes	CATT

R1-2410073	Further consideration on LP-WUS operation in connected mode	OPPO

R1-2410233	LP-WUS operation in CONNECTED mode	Sony

R1-2410275	Discussion on LP-WUS operation in CONNECTED modes	ETRI

R1-2410296	Discussion on LP-WUS operation in CONNECTED modes	LG Electronics

R1-2410358	Discussion on LP-WUS procedures in Connected mode	TCL

R1-2410363	Discussion on LP-WUS operation in CONNECTED modes	Lenovo

R1-2410399	Discussion on LP-WUS operation in CONNECTED mode	NTT DOCOMO, INC.

R1-2410421	Discussion on LP-WUS operation in CONNECTED modes	Sharp

R1-2410446	LP-WUS operation in CONNECTED mode	Ericsson

R1-2410488	LP-WUR operation in connected mode	Qualcomm Incorporated

R1-2410512	LP-WUS operation in CONNECTED modes	MediaTek Inc.

R1-2410699	FL summary #1 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

From Monday session

Agreement

For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS QCL type(s) is {Type A or Type C – for down-selection} and Type D, when applicable.

Agreement

LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication.

- Note: LP-WUS indication does not impact SCell dormancy behavior.

R1-2410804	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

From Wednesday session

Agreement

For LP-WUS CONNECTED mode operation, the following case is not supported:

- Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell.

R1-2410859	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

From Thursday session

Agreement

- For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer.

  - Approach 2:

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle.

- For LP-WUS MOs in connected mode for Option 1-2, following is considered further

  - Approach 1:

    - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s).

    - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset.

  - Approach 2:

    - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters.

    - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start.

  - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time]

    - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start;

    - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start.

R1-2410906	FL summary #4 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

From Friday session

Agreement

For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution

- Use existing DRX Command MAC CE and Long DRX Command MAC CE

- When UE is not in C-DRX active time, UE monitors LP-WUS

Send LS to RAN2 to ask the feasibility of the above potential solution.

Conclusion

For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group

- When UE is not in C-DRX active time, UE monitors LP-WUS

Send LS to RAN2 to ask whether RAN2 has same understanding or not.

R1-2410908	Draft LS on LP-WUS operation in CONNECTED modes	Moderator (NTT DOCOMO)

Decision: The draft LS is endorsed. Final LS on LP-WUS operation in CONNECTED mode is approved in R1-2410909.

Agreement

For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively:

- If the parameter is NOT configured:

  - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported.

  - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

- If the parameter is configured:

  - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time.

  - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time.

- FFS: Whether RAN1 spec impact is required.

For future meetings:

Companies are encouraged to consider [Fri]Combined Proposal 3.2-1/3.2-2c of R1-2410906 for further discussions in future meetings.

Final summary in R1-2410907.

## Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR

Please refer to RP-242348 for detailed scope of the SI.

R1-2410846	Session notes for 9.7 (Study on channel modelling for Integrated Sensing And Communication for NR)	Ad-Hoc Chair (Huawei)

Friday decision: The session notes are endorsed and contents reflected below.

[119-R19-ISAC] – Yingyang (xiaomi)

Email discussion on Rel-19 ISAC channel model

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### ISAC deployment scenarios

R1-2409393	Deployment scenarios for ISAC channel model	Huawei, HiSilicon

R1-2409471	Discussion on ISAC deployment scenarios and requirements	EURECOM

R1-2409523	Discussion on ISAC deployment scenarios	CMCC, China Southern Power Grid

R1-2409608	Discussion on  ISAC deployment scenarios	Samsung

R1-2409692	Views on Rel-19 ISAC deployment scenarios	vivo

R1-2409717	Discussion on ISAC deployment scenarios	TOYOTA InfoTechnology Center

R1-2409766	Discussion on ISAC Deployment Scenarios	Nokia, Nokia Shanghai Bell

R1-2409776	Deployment scenarios for integrated sensing and communication with NR	NVIDIA

R1-2409817	Discussion on ISAC deployment scenarios	Apple

R1-2409836	Discussion on ISAC deployment scenarios	LG Electronics

R1-2409846	Discussion on ISAC deployment scenarios	InterDigital, Inc.

R1-2409907	Deployment scenarios and evaluation assumptions for ISAC channel model	Xiaomi

R1-2409952	Discussion on ISAC deployment scenarios	CATT, CICTCI

R1-2410006	Discussion on ISAC deployment scenarios	China Telecom

R1-2410097	Discussion on ISAC deployment scenarios	OPPO

R1-2410125	Discussion on ISAC Deployment Scenarios	Ericsson

R1-2410162	ISAC deployment scenarios	Tejas Networks Limited

R1-2410234	Considerations on ISAC deployment scenarios	Sony

R1-2410322	Discussion on ISAC deployment scenarios 	Lenovo

R1-2410332	ISAC channel model calibration and scenario parameters	AT&T, FirstNet

R1-2410369	Considerations on ISCA deployment scenarios	CAICT

R1-2410400	Study on deployment scenarios for ISAC channel modelling	NTT DOCOMO, INC.

R1-2410447	Discussion on ISAC deployment scenarios	ZTE Corporation, Sanechips

R1-2410489	Discussion on ISAC deployment scenarios	Qualcomm Incorporated

R1-2410524	Discussion on ISAC deployment scenario	MediaTek Inc.

R1-2410626	Discussion on ISAC deployment scenarios	Tiami Networks

R1-2410627	Discussion on ISAC channel modeling	Tiami Networks

R1-2410337	FL Summary #1 on ISAC Deployment Scenarios	Moderator (AT&T)

From Tuesday session

Guidance for further work

1. Rapporteurs are encouraged to start providing a draft CR for both agendas to RAN1#120.

2. Jerome to provide an initial proposal for calibrations discussions by the end of RAN1#119.

3. RAN1 agenda will clarify that input on calibrations discussions is to be provided to agenda 9.7.1 starting at RAN1#120.

Agreement

For UAV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118 as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-UAV

Details on ISAC-UAV scenarios are listed in Table x.

Table x. Evaluation parameters for UAV sensing scenarios

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios |  | UMi, UMa, RMa [38.901] UMi-AV, UMa-AV, RMa-AV |
| Sensing transmitters and receivers properties | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs locations in the corresponding communication scenarios. NOTE1: This may include aerial UEs for UMi-AV, UMa-AV, RMa-AV communication scenarios. In this case, other Rx/Tx properties (e.g. mobility) are also taken from the corresponding communication scenario. |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: uniform distribution between 0 and 180km/h, if horizontal velocity is not fixed to 0. Vertical velocity: 0km/h, optional {20, 40} km/h NOTE2: 3D mobility can be horizontal only or vertical only or a combination for each sensing target FFS: time-varying velocity. NOTE 3: time-varying velocity may be considered for future evaluations. |
|  | 3D distribution | Horizontal plane: Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. FFS: Value of N, defined area, and other distributions N = {1, 2, 3, 4, 5} NOTE4: N=0 may be considered for the evaluation of false alarm Vertical plane: Option A: Uniform between 1.5m and 300m. Option B: Fixed height value chosen from {25, 50, 100, 200, 300} m assuming vertical velocity is equal to 0. FFS Other options are not precluded. NOTE5: target(s) are outside the minimum distance to the Tx/Rx |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics (e.g., size) | Size: - Option 1: 1.6m x 1.5m x 0.7m - Option 2: 0.3m x 0.4m x 0.2m FFS: Material(s), Structure, Other size(s) |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option B: Min distances based on min. TRP/UE distances defined in TR36.777 as a starting point. NOTE5: the sensing target is assumed in the far field of sensing Tx/Rx Option C: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a target Option 2: 10 meters |
| [Unintended/Environment objects, e.g., types, characteristics, mobility, distribution, etc.] |  | FFS |

NOTE:	A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

R1-2410338	FL Summary #2 on ISAC Deployment Scenarios	Moderator (AT&T)

From Thursday session

Agreement

For Automotive sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Automotive

Details on ISAC-Automotive scenarios are listed in Table x.

Table x. Evaluation parameters for Automotive sensing scenarios

| Parameters |  | Values |
| --- | --- | --- |
| Applicable communication scenarios |  | Highway, Urban Grid. NOTE1 |
| Sensing transmitters and receivers properties |  | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenario. NOTE2 FFS: Additional option: ISD between TRPs of Urban Grid is 250m |
| Sensing target | LOS/NLOS | LOS and NLOS (including NLOSv) |
|  | Outdoor/indoor | Outdoor |
|  | Mobility (horizontal plane only) | Based on TR37.885 mobility for urban grid or highway scenario |
|  | Distribution (horizontal) | Based on dropping in TR37.885 per urban grid or highway communication scenario |
|  | Orientation | Lane direction in horizontal plane |
|  | Physical characteristics (e.g., size) | Type 1/2 (passenger vehicle) Type 3 (truck/bus) Vehicle type distribution per TR 37.885 as a starting point FFS: Other sizes, additional distributions, and vehicle types, e.g. one new type of e-scooter/motorcycle/bike |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min distances based on min. TRP/UE distances defined in TR37.885 as a starting point. Option 2: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc. |

NOTE1:	Calibration for UMi, Uma, RMa is not performed for the automotive scenario, but UMi, Uma, RMa can be considered for future evaluations of the automotive sensing target scenarios. Calibration for UMi, Uma, RMa is expected to be performed for another sensing scenario.

NOTE2:	A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For Human (indoor and outdoor) sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Human

Details on ISAC-Human scenarios are listed in Table x.

Table x. Evaluation parameters for Human (indoor and outdoor) sensing scenarios

| Parameters |  | Indoor Values | Outdoor Values |
| --- | --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Indoor office, indoor factory [TR38.901] Indoor room [TR38.808] | UMi, Uma, RMa [TR38.901] |
| Sensing transmitters and receivers properties | Rx/Tx Locations NOTE 2 | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario | Rx/Tx locations are selected among the TRPs and UE locations in the corresponding communication scenario |
|  | Rx/Tx Mobility for UEs | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr |
| Sensing target | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor | Outdoor |
|  | 3D mobility | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 3km/hr (horizontal plane with random direction straight-line trajectory) | Option 1: 0km/h Option 2: 3km/h Option 3: Uniform distribution between 0km/h and 10km/hr (horizontal plane with random direction straight-line trajectory) |
|  | 3D distribution | N targets uniformly distributed over the horizontal area of the convex hull of the TRP deployment FFS: Value of N NOTE1: N=0 may be considered for the evaluation of false alarm | Option A: N targets uniformly distributed within one cell. Option B: N targets uniformly distributed per cell. Option C: N targets uniformly distributed within an area not necessarily determined by cell boundaries. Uniform in horizontal plane NOTE1: N=0 may be considered for the evaluation of false alarm |
|  | Orientation | Random over the horizontal area | Random over the horizontal area |
|  | Physical characteristics (e.g., size) | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m | Size (Length x Width x Height): - Child: 0.2m x 0.3m x 1m - Adult Pedestrian: 0.5m x 0.5m x 1.75m |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859as a starting point NOTE2: the sensing target is assumed in the far field of sensing Tx/Rx | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx Option 2: Min distances defined in TR 38.901 and TR36.843 and TR38.859 as a starting point NOTE3: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [x] m. value of x is FFS |
| Environment Objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS, based on outcome for AI 9.7.2 | FFS, based on outcome for AI 9.7.2 |

NOTE1:	For the human (indoor and outdoor) sensing targets, additional communication scenarios can be considered for future evaluations. Channel model calibration for Urban Grid with outdoor humans is expected to be performed from Objects creating hazards on the road/railway sensing scenarios.

NOTE2:	A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

Agreement

For AGV sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-AGV

Details on ISAC-AGV are listed in Table x.

Table x. Evaluation parameters for Automated Guided Vehicles

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | InF (TR38.901 including Table 7.8-7) |
| Sensing transmitters and receivers properties NOTE2 |  | Rx/Tx location are selected among the TRPs and UEs location in the corresponding communication scenario Rx/Tx Mobility for UEs - Option 1: 0 km/h - Option 2: 3km/h - Option 3: Uniform distribution between 0km/h and 3km/h |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Indoor |
|  | 3D mobility | Horizontal velocity with random straight-line trajectory - Option 1: Uniform distribution in the range of up to 30 km/h - Option 2: Fixed velocities [3, 10] km/h |
|  | 3D distribution | Option A: Uniformly distributed in the convex hull of the horizontal BS deployment Option B: Uniformly distributed in horizontal plane |
|  | Orientation | Horizontal plane only |
|  | Physical characteristics (e.g., size) | Size (L x W x H) - Option 1: 0.5m x 1.0m x 0.5m - Option 2: 1.5 m x 3.0m x 1.5 m - FFS: Material, Additional sizes, and AGV size distribution |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: Min distances based on min. TRP/UE distances defined in TR38.901 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option A: At least larger than the physical size of a target Option B: Fixed value, [x] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | FFS |

NOTE1:	For the AGV sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2:	A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

NOTE3:	RAN1 can further discuss narrowing down the number of sub-scenarios of InF

Agreement

For Objects creating hazards sensing target scenarios, the following table is agreed for deployment scenario parameters/values using the agreements from RAN1#118-bis as a baseline:

The detailed scenario description in this clause can be used for channel model calibration.

ISAC-Hazards

Details on ISAC-Hazards are listed in Table x.

Table x. Evaluation parameters for objects creating hazards

| Parameters |  | Value |
| --- | --- | --- |
| Applicable communication scenarios NOTE1 |  | Highway, Urban grid, HST (High Speed Train) |
| Sensing transmitters and receivers properties NOTE2 | Rx/Tx Locations | Rx/Tx locations are selected among the TRPs and UEs (e.g., VRU, vehicle, RSU-type UEs) locations in the corresponding communication scenarios. FFS: Option 2 Additional option ISD between TRPs of Urban Grid is 250 m |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: up to [10] km/h for humans and animals FFS: Additional velocities, trajectory |
|  | 3D distribution | Uniformly distributed in horizontal plane |
|  | Orientation | Random distribution in horizontal plane |
|  | Physical characteristics (e.g., size) | For human/pedestrians: Child: 0.2m x 0.3m x 1m Adult: 0.5m x 0.5m x 1.75m For animals: Size: 1.5m x 0.5m x 1 m FFS: other types/sizes of targets may be considered for future evaluations |
| Minimum 3D distances between pairs of Tx/Rx and sensing target |  | Option 1: Min. distance is larger than the min. far-field distance of the sensing Tx/Rx from the sensing target Option 2: based on min TRP/UE distances defined in TR37.885 and TR38.802 and TR36.843 and TR38.859 NOTE: the sensing target is assumed in the far field of sensing Tx/Rx |
| Minimum 3D distance between sensing targets |  | Option 1: At least larger than the physical size of a sensing target Option 2: Fixed value, [10] m. value of x is FFS |
| Environment objects, e.g., types, characteristics, mobility, distribution, etc. |  | EO Type 2 for Urban Grid - FFS: details, e.g. up to 4 walls modelled as EO type 2, per building of size [413m x 230m x 20m]. FFS: number of buildings, how many walls are modelled, additional building sizes, etc. |

NOTE1:	For the objects creating hazards sensing targets, additional communication scenarios can be considered for future evaluations.

NOTE2:	A percentage of TRPs/UEs that have sensing capabilities may be considered for future evaluations.

[Post-119-ISAC-01] – Jerome (AT&T)

Email discussion on simulation assumptions for channel model calibration, from January 8th-17th:

- Companies are encouraged to provide initial channel model calibration assumptions, parameters, and values

- Jerome to collect inputs, provide potential consolidated tables, proposals for consideration by January 17th

Decision: The discussions are to be carried over to RAN1#120.

### ISAC channel modelling

R1-2409394	Channel modelling for ISAC	Huawei, HiSilicon

R1-2409472	Discussion on ISAC channel modeling	EURECOM

R1-2409524	Discussion on channel modeling methodology for ISAC	CMCC,BUPT,SEU, PML

R1-2409609	Discussion on ISAC channel modelling	Samsung

R1-2409647	Discussion on ISAC channel modeling	Spreadtrum, UNISOC

R1-2409693	Views on Rel-19 ISAC channel modelling	vivo, BUPT

R1-2409718	Discussion on ISAC channel modelling	TOYOTA InfoTechnology Center

R1-2410832	Discussion on ISAC channel modeling	Intel Corporation	(rev of R1-2410671, rev of R1-2409740)

R1-2409767	Discussion on ISAC channel modeling	Nokia, Nokia Shanghai Bell

R1-2409777	Channel modeling for integrated sensing and communication with NR	NVIDIA

R1-2410739	Discussion on ISAC channel modelling	Apple	(rev of R1-2409818)

R1-2409837	Discussion on ISAC channel modelling	LG Electronics

R1-2409847	Discussion on ISAC channel modeling	InterDigital, Inc.

R1-2410668	Discussion on ISAC channel model	Xiaomi, BJTU, BUPT	(rev of R1-2409908)

R1-2409953	Discussion on ISAC channel modelling	CATT, CICTCI

R1-2409977	Discussions on ISAC Channel Modelling	Lekha Wireless Solutions

R1-2410659	ISAC Channel Modeling and Measurement Validation	BUPT, CMCC, VIVO	(rev of R1-2409992)

R1-2410007	Discussion on ISAC channel modelling	China Telecom

R1-2410098	Study on ISAC channel modelling	OPPO

R1-2410126	Discussion on ISAC Channel Modelling	Ericsson

R1-2410136	Discussion on ISAC channel modeling	NIST

R1-2410163	ISAC channel modelling	Tejas Networks Limited

R1-2410235	Views on Channel Modelling for ISAC	Sony

R1-2410321	Discussion on Channel Modelling for ISAC	Lenovo

R1-2410333	Discussions on ISAC Channel Modeling	AT&T

R1-2410370	Considerations on ISAC channel modelling	CAICT

R1-2410401	Discussion on ISAC channel modeling	NTT DOCOMO, INC.

R1-2410648	Discussion on channel modelling for ISAC	ZTE Corporation, Sanechips	(rev of R1-2410448)

R1-2410660	Discussion on ISAC channel modelling	Qualcomm Incorporated	(rev of R1-2410490)

R1-2410525	Discussion on ISAC channel modelling	MediaTek Inc.

R1-2410011	Summary #1 on ISAC channel modelling	Moderator (Xiaomi)

From Tuesday session

Agreement

Bistatic RCS values for a scattering point of a target are obtained by fixing an incident direction in LCS of target and varying the scattered directions in LCS of target; then changing to other incident direction.

R1-2410012	Summary #2 on ISAC channel modelling	Moderator (Xiaomi)

From Wednesday session

Agreement

- To generate indirect paths of NLOS ray + NLOS ray in the target channel

  - Option 0 is recommended, i.e., ray level full convolution between Tx-target link and target-Rx link for radio propagation Case 1/2/3/4

  - Option 3 to generate a reduced number of indirect paths of NLOS ray + NLOS ray is recommended, i.e., ray level 1-by-1 random coupling between Tx-target link and target-Rx link is supported for radio propagation Case 1/2/3/4

    - If number of rays in the two links are different, e.g., M1, M2 respectively for link 1 and link 2,

      - If M1<M2, randomly M1 rays are selected in link 2, otherwise randomly M2 rays are selected in link 1

  - Other methods are up to company choice for complexity reduction

  - Both option 0 and 3 will be calibrated independently. Company should report which option is used in calibration

- The power threshold for path dropping is X=[-25] dB

  - X is relative to the strongest indirect path in the target channel

- FFS: further power normalization of target channel is performed after path dropping,

- Note: power normalization when target channel and background channel are combined can be discussed separately

- FFS The set of remaining indirect paths can be updated during movement of Tx, target or Rx

Agreement

The following RCS models are supported when human is modelled with single scattering point for monostatic, where different RCS values and/or models can be supported for human due to different size, shape, frequency, etc.

- Model 1

  - B1=0 dB

  - A is mean RCS value

  - B2 is modelled using log-normal distribution

- Model 2

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

- FFS: conditions for using which model

Agreement

The following RCS model is supported when vehicle is modelled with single scattering point for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

- B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

  - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

  - Alt 2: a function

  - Alt 3: Lookup table

- B2 is modelled using log-normal distribution

- FFS RCS component A

Agreement

When vehicle is modelled with multiple scattering points for monostatic, where different RCS values can be supported for vehicle due to different size, shape, frequency, etc.

- the recommended five scattering points are located in front, left, back, right and roof side of the vehicle

- the following RCS model is supported for each scattering point

  - B1 have dependency on incident/scattered angles, with further down-selection among the alternatives below:

    - Alt 1: formulated similar as the antenna radiation power pattern in 38.901

    - Alt 2: a function

    - Alt 3: Lookup table

  - B2 is modelled using log-normal distribution

  - FFS RCS component A

Agreement

EO type-1 (when modelled) is modelled in the same way as a sensing target in the ISAC channel model.

Agreement

- If blockage/forward scattering between sensing targets is not considered, a propagation path from Tx to Rx interacting with more than one sensing targets is not modelled.

- FFS whether/how blockage/forward scattering can be modelled in the target channel.

Agreement

- Doppler for a target including both macro-Doppler and micro-Doppler can be modeled using a unified formula,

rrx,n',m'Tt.vrxt+rp,n',m'Tt∙vspt+rtx,n,mTt.vtxt+rp,n,mTt∙vsptλ0+ft

Where,

    - rrx,n',m'T is the spherical unit vector at receiver for the link from Rx to the scattering point

    - rtx,n,mT  is the spherical unit vector at transmitter for the link from Tx to the scattering point

    - rp,n',m'T is the spherical unit vector at the scattering point for the link from the scattering point to Rx

    - rp,n,mT is the spherical unit vector at the scattering point for the link from the scattering point to Tx

  - Dual mobility model in 7.6.10, TR 38.901 is used as start point to model Doppler effect ft due to movement of stochastic clusters, i.e., 2αn,mDn,mλ0

    - ft is only applicable for indirect path

    - Support one term of 2αn,mDn,mλ0 for indirect path of LOS ray+NLOS ray, NLOS ray+LOS ray

    - Support two terms of  2αn,mDn,mλ0 for indirect path of NLOS ray+NLOS ray

  - Doppler is separately determined for each of the multiple scattering points of a target

  - vspt can include macro-Doppler and/or micro-Doppler motion, 
vspt=vmacrot+vmicro,pt

  - FFS: maximum speed of moving scatterers

  - FFS: ratio of moving scatterers among all scatterers

R1-2410013	Summary #3 on ISAC channel modelling	Moderator (Xiaomi)

From Thursday session

Agreement

- The following options are supported to generate the combined ISAC channel

  - Option 1: The ISAC channel of a pair of sensing Tx/Rx is obtained by summing the target channel(s) and background channel, i.e., power normalization is not performed.

  - Option 2: As an additional modelling component, power normalization is performed when summing the target channel(s) and background channel, to keep the same/similar channel power as the background channel without target. Down select between

    - Alt 1: Power normalization on both target channel and background channel

    - Alt 2: Power normalization on background channel only

    - Alt 3: the target channel of a target will replace one cluster in the background channel

- FFS Blockage is modelled for the background channel due to sensing target and/or EO type-2

- FFS condition to select option, e.g. depending on scenario, sensing mode, number of target/EO type-2

Agreement

To model the polarization matrix of a direct/indirect path at a scattering point of an object other than EO type-2, the polarization matrix of the scattering point, i.e., CPMsp,i is modelled by αi,1,αi,2,βi,1,βi,2, and initial random phases Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ, i.e., CPMsp,i=αi,1expjΦsp,iθθβi,1expjΦsp,iθϕβi,2expjΦsp,iϕθαi,2expjΦsp,iϕϕ

  - The initial random phase Φsp,iθθ,Φsp,iθϕ,Φsp,iϕθ,Φsp,iϕϕ is [uniformly distributed within -π,π]

  - FFS correlation between αi,1,αi,2,βi,1,βi,2

  - FFS specular reflection

  - FFS: CPM normalization

The following options are considered for further study, down select one option from the following

- Option 1: αi,1=αi,2=1, βi,1=βi,2=κsp,i-1 is generated for path i, where κsp,i is XPR ratio

  - κsp,i is randomly generated by log-normal distribution. FFS mean/variance of the distribution

- Option 2: αi,1=1, αi,2,βi,1,βi,2 are variables generated for path i

- Option 3: αi,1,αi,2,βi,1,βi,2 are variables generated for path i

  - CPMsp,i defined in LCS

- Option 4: αi,1=αi,2=1, βi,1=βi,2=0 is generated for path i

Agreement

The finite size of the EO type-2 affects identification of specular reflection point. In the target channel, EO type-2 is modelled only if the specular reflection point is in the area of the EO type-2.

Agreement

Component B2 of RCS is upper bounded by kσ dB for the log-normal distribution, where σ is the standard deviation of B2 in dB. FFS the value of k.

Agreement

When the EO type-2 is modelled in the target channel, down select between the following options to determine the LOS condition of the Tx-target link and target-Rx link

- Option A: If type-2 EO is in the LOS ray of one link, the link is determined as NLOS condition, and otherwise use the LOS probability equation to determine the LOS/NLOS condition

  - FFS changes to the LOS probability defined in existing TRs

  - FFS details on blockage by EO type-2

- Option B: Use the LOS probability equation to determine the LOS/NLOS condition of one link, and then the impacts of type-2 EO is modeled by a blockage model.

Final summary in R1-2410014.

## Study on channel modelling enhancements for 7-24GHz for NR

Please refer to RP-234018 for detailed scope of the WI.

R1-2410847	Session notes for 9.8 (Study on channel modelling enhancements for 7-24GHz for NR)	Ad-Hoc Chair (CMCC)

Friday decision: The session notes are endorsed and contents reflected below.

[119-R19-7-24GHz] – Daewon (Intel)

Email discussion on channel modelling for 7-24GHz

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2410170	Draft CR/TP to incorporate the agreement for Rel-19 7-24GHz Channel model	ZTE Corporation,Intel Corporation

R1-2410631	Data source descriptions for 7 – 24 GHz SI	Moderator (Intel Corporation)

From Thursday session

Note:

To check and review the following results and measurement data provided in RAN1 #119 for further discussion in next RAN1 meeting. R1-2410631 contains the list of data sources for the results and measurements provided in RAN1 #119, #118-bis, #118, #117, and #116-bis.

### Channel model validation of TR38.901 for 7-24GHz

R1-2409402	Considerations on the 7-24GHz channel model validation	Huawei, HiSilicon

R1-2409441	Discussion on validation of channel model	Ericsson

R1-2409467	Further Discussion on Angle Scaling for MIMO CDL Channel	InterDigital, Inc.

R1-2409610	Discussion on channel model validation of TR38.901 for 7 - 24 GHz	Samsung

R1-2409694	Views on channel model validation of TR38.901 for 7-24GHz	vivo, BUPT

R1-2409724	Discussion on the channel model validation	ZTE Corporation, Sanechips

R1-2409738	Discussion on channel modeling verification for 7-24 GHz	Intel Corporation

R1-2409778	Channel model validation of TR 38901 for 7-24 GHz	NVIDIA

R1-2409819	Validation of Channel Model	Apple

R1-2409954	Discussion on channel model validation for 7-24GHz	CATT

R1-2410015	Channel model validation for 7-24 GHz	Lenovo

R1-2410213	Discussion on channel model validation of TR38.901 for 7-24GHz	BUPT

R1-2410236	Further Discussion of channel model validation of TR38.901 for 7–24GHz	Sony

R1-2410254	Instantaneous directional channel measurements and parameter estimation at 14 GHz	ROHDE & SCHWARZ

R1-2410299	Measurements of the angular spreads in a urban and suburban macrocells	Vodafone, Ericsson

R1-2410319	Channel model validation of TR38.901 for 7-24 GHz	Sharp, Nokia

R1-2410655	Discussion on Validation of the Channel Model in 38901	AT&T	(rev of R1-2410334)

R1-2410402	Discussion on channel model validation for 7-24 GHz	NTT DOCOMO, INC.

R1-2410491	Channel Model Validation of TR38.901 for 7-24 GHz	Qualcomm Incorporated

R1-2410594	Discussion on Channel model validation of TR38.901 for 7-24GHz	Nokia

R1-2410632	Summary of issues for Rel-19 7-24 GHz Channel Modeling Validation	Moderator (Intel Corporation)

R1-2410633	Summary #1 of discussions for Rel-19 7-24 GHz Channel Modeling Validation	Moderator (Intel Corporation)

From Tuesday session

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- Building type ratio: 90% residential, 10% commercial

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT Vertical Distribution Option 2: uniform distribution across all floors for a building type

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT mobility in horizontal plane only: Indoor UTs: 3 km/h, outdoor UTs: 40 km/h

Agreement

For handheld devices adopt the following device dimensions for UE antenna modeling:

- 15 cm x 7 cm x 0 cm

Conclusion

For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable). For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19.

- Note: If deemed important, it should be still possible to discuss how specific channel model parameters need to be updated.

R1-2410634	Summary #2 of discussions for Rel-19 7-24 GHz Channel Modeling Validation	Moderator (Intel Corporation)

From Wednesday session

Conclusion

Based on measurement data provided, RAN1 concludes pathloss models in TR38.901 for the following scenarios are validated and no updates to TR are made.

- InH-Office LOS

- InH-Office NLOS

- RMa LOS

- RMa NLOS

- InF LOS

- InF NLOS

Observation

For the following scenarios, some sources have observed some differences in delay spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay spread consistent with the model in the TR.

- InF LOS

- InF NLOS

Conclusion

For the following scenarios, there is no consensus to update delay models due to lack of consistent and significant observed difference between model and measurements.

- InF LOS

- InF NLOS

Agreement

Study further on the need for introduction of intra-cluster power profile.

- Potential options for introduction of intra-cluster power profile:

  - Introduce unequal power per ray within a cluster and change the unequal angle offset to equal angle offset among rays within a cluster to maintain the same PAS characteristic as in TR 38.901 (example in R1-2409724)

  - Power scaling within a cluster based on DS associated with the cluster (example in R1-2410213)

R1-2410635	Summary #3 of discussions for Rel-19 7-24 GHz Channel Modeling Validation	Moderator (Intel Corporation)

From Thursday session

Agreement

Further study penetration loss modeling, including the following aspects:

- Need for change and details of the updates to the material penetration loss, for example, if any

|  | Concrete | Wood |
| --- | --- | --- |
| Existing TR | Lconcrete=5+4f | Lwood=4.85+0.12f |
| New Updates | Lconcrete=α∙(5+4f) α=Tconcrete23cm | Lwood=α∙(4.85+0.12f) α=Twood6cm |
| Tconcrete and Twood represent reference (default) thickness of the material. For concrete and wood, the reference thickness are 23 cm and 6 cm, respectively. |  |  |

  - Whether penetration loss can be scaled linearly with thickness for each material

  - For standard multi-pane glass, FFS whether and how to reflect the penetration loss resonation effect, which may relate to thickness of the glass.

  - FFS the impact of updates to material penetration loss to total build penetration loss

- Need for change and details of total building penetration loss, e.g. low-loss and high-loss model, if any

- Impact of material composition on material penetration loss

Agreement

For the following scenarios, further study path loss model parameters for at least 6 – 24 GHz frequency range.

- UMi LOS

- UMi NLOS

- UMa LOS

- UMa NLOS

Agreement

For the following scenarios, further study delay spread parameters for at least 6 – 24 GHz frequency range.

- UMi LOS

- UMi NLOS

- InH-Office LOS

- InH-Office NLOS

- UMa LOS

- UMa NLOS

Agreement

Based on measurement data provided, RAN1 concludes angular spread parameters in TR38.901 that following scenarios are validated and no updates to TR are made.

- UMa LOS: ZSD

- UMa NLOS: ZSD

Observation

For the following scenarios, some sources have observed some differences in angular spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed angular spread consistent with the model in the TR.

- InH LOS and NLOS: ZSA, ZSD

- UMi LOS: ZSA, ZSD

- UMi NLOS: ZSD

- UMa LOS: ZSA

Conclusion

For the following scenarios, there is no consensus to update angular spread models due to lack of consistent and significant observed difference between model and measurements.

- InH LOS and NLOS: ZSA, ZSD

- UMi LOS: ZSA, ZSD

- UMi NLOS: ZSD

- UMa LOS: ZSA

Agreement

For the following scenarios, further study angular spread parameters for at least 6 – 24 GHz frequency range.

- InH LOS/NLOS: ASA, ASD

- UMi LOS: ASA, ASD

- UMi NLOS: ASA, ASD, ZSA

- UMa LOS: ASD, ASA

- UMa NLOS: ASA, ASD, ZSA

- InF LOS and NLOS: ASD, ASA, ZSA, ZSD

Agreement

Further study number of clusters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

- Consistency/continuity across frequencies below 6GHz and above 24 GHz, including whether to update values for other frequencies based on measurements

- Whether changes to number of clusters result in meaningful impact to system performances and evaluation of features

Conclusion

Based on measurement data provided, RAN1 concludes shadow fading parameters in TR38.901for the following scenarios are validated and no updates to TR are made.

- InH LOS

- InH NLOS

- UMi NLOS

Observation

For the following scenarios, some sources have observed some differences in shadow fading in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed shadow fading with the model in the TR.

- UMi LOS

Conclusion

For the following scenarios, there is no consensus to update shadow fading due to lack of consistent and significant observed difference between model and measurements.

- UMi LOS

Agreement

Further study the need for updating the K-factor parameters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

- Consistency/continuity across frequencies below 6 GHz and above 24 GHz, including whether to update values for other frequencies based on measurements.

- Whether changes to K-Factor result in meaningful impact to system performances and evaluation of features.

Working Assumption

- Adopt the following absolute delay for UMi and UMa scenarios.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | UMi | UMa |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.4 |
|  | σlgΔτ | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 15 | 50 |

Agreement

- Introduce absolute delay for InH scenarios.

  - Absolute delay is enabled by re-using the absolute time of arrival for InF scenarios.

  - FFS details of the parameter for InH. Consider the following for further study.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | Option 1) -7.9 Option 2) -8.6 |
|  | σlgΔτ | Option 1) 0.3 Option 2) 0.1 |
| Correlation distance in the horizontal plane [m] |  | 10 |

Agreement

- To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles.

  - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,

  - μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A.

  - WrapTo180(⋅) is a function which wraps an azimuth angle to the half-open interval (-180, 180].

- Down-select among options.

  - Option 1)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - where s is a scale factor chosen to change the distribution of the angles (up to each company to provide information, e.g. s=ASdesiredASmodel).

  - Option 2a)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s=minx≥0{x: ASx=ASdesired}.

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 2b)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s is chosen such that AS(ϕn,intermediate) = AS_desired

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 3)

    - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model,

    - Note: The resulting scaled ray angle may or may not achieve the desired angle mean and the desired rms angle spread.

- FFS whether and how to enable specular rays of CDL-D and CDL-E to be in the direction of the desired vector.

Agreement

For suburban scenario, further consider following options and down-select a value for BS height assumption for calibration purposes:

- Option 1) BS height = 25m

- Option 2) BS height = 35m

Note: It should be understood that BS height selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios may be applicable for a range of BS heights [25 – 35]m.

Agreement

For suburban scenario, further consider following options and down-select a value for ISD assumption for calibration purposes:

- Option 1) ISD = 1299 m

- Option 2) ISD = 1732 m

Note: It should be understood that ISD selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios is applicable for a range of ISD [1200 – 1800]m.

Agreement

For suburban scenario, further study the LOS probability equation.

- Example of LOS probability equation for study:

  - PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ, where dm, α, β, γ, δ are modeling parameters

- Study includes whether distinction of LOS probability should be made for UEs in residential and commercial buildings, which have different building heights.

- Study includes whether or not vegetation should be explicitly considered and modeled for determining LOS probability, if determined to be explicitly modeled, how to model vegetation impact, e.g. reuse of InF clutter to change the LOS probability.

Agreement

Further study the need of additional low loss outdoor-to-indoor (O2I) building penetration loss model for suburban scenario.

- Potential example of the new low loss O2I building penetration loss:

PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

Agreement

- UMa can also be used for other ISDs between 200m and 500m, including the ISD proposed in ITU-R M.2412 -0

- One ISD value will be selected for calibration with ISD = 500m

- No change to any other parameters due to other ISD values

- FFS: How to reflect it in TR38.901

Agreement

Capture channel modeling updates regarding suburban macrocell scenario as follows:

- Define a new scenario, e.g., SMa

- The key differentiating aspects of SMa (compared to UMa and RMa) in high level description will be highlighted in Section “6.2 Scenario of Interest” of the TR.

Agreement

Further study device dimension and antenna placement applicability for customer premise equipment (CPE) devices, include the need for defining CPE device type dimensions.

Agreement

Further refine previous agreement  (made in RAN1#118bis)  on UE antenna radiation patternmodeling as follows:

- For UE antenna radiation pattern modeling, consider the following options for further study:

  - Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB |
| Combining method for 3D antenna element pattern (dB) | FFS A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | FFS dBi |
| Note: max direction gain and half power beam widths should be selected such that total antenna efficiency should be equal or less than 0dBi. |  |

  - Option 2) half-wavelength dipole.

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis

      - FFS: range of θ

  - Option 2b) half-wavelength dipole

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: sin(π/2*cos(θ))

    - range of θ = -90° to +90°

  - Option 3) omni-directional antenna pattern

  - Other options

Agreement

For UE antenna modeling of handheld devices,

- Further study on default antenna placement locations for calibration purpose for 2, 3, 4, 6, and 8 antenna elements at least for FR1 and 6-24 GHz, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2.

  - FFS on handling of antenna array panels

Agreement

For handheld UE device type antenna radiation pattern, further study details of antenna field pattern orientation with respect to the direction of antenna for each antenna candidate locations.

- For example, maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location.

R1-2410636	Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation	Moderator (Intel Corporation)

Final summary in R1-2410910.

### Channel model adaptation/extension of TR38.901 for 7-24GHz

Including near-field propagation and spatial non-stationarity

R1-2409403	Considerations on the 7-24GHz channel model extension	Huawei, HiSilicon

R1-2409442	Discussion on adaptation and extension of channel model	Ericsson

R1-2409468	Further Discussion on Channel Model Extension for FR3	InterDigital, Inc.

R1-2409491	Discussion on channel modelling adaptation/extension for 7-24GHz	LG Electronics

R1-2409611	Discussion on channel model adaptation/extension of TR38.901 for 7 - 24 GHz	Samsung

R1-2409695	Views on channel model adaptation/extension of TR38.901 for 7-24GHz	vivo, BUPT

R1-2409725	Discussion on the channel model adaptation and extension	ZTE Corporation, Sanechips

R1-2410656	Discussion on channel model adaptation/extension	Intel Corporation	(R1-2409739)

R1-2409779	Channel model adaptation of TR 38901 for 7-24 GHz	NVIDIA

R1-2409821	Channel Model Adaptation and Extension of TR38.901 for 7-24 GHz	Apple

R1-2409955	Discussion on channel model adaptation/extension for 7-24GHz	CATT

R1-2410016	Channel model extension for 7-24 GHz	Lenovo

R1-2410099	Channel model adaptation and extension for 7-24GHz	OPPO

R1-2410131	Discussion on channel model adaptation/extension for 7-24 GHz	Fujitsu

R1-2410214	Discussion on modeling near-field propagation and spatial non-stationarity in TR38.901 for 7-24GHz	BUPT, CMCC

R1-2410237	Further discussion of channel model adaptation/extension of TR38.901 for 7–24GHz	Sony

R1-2410320	Views on channel modelling adaptation/extension for 7-24GHz	Sharp, NYU WIRELESS

R1-2410492	Channel Model Adaptation/Extension of TR38.901 for 7-24GHz	Qualcomm Incorporated

R1-2410520	Discussion on channel modelling enhancements for 7-24GHz for NR	MediaTek Inc.

R1-2410595	Discussion on Channel model adaptation/extension of TR38.901 for 7-24GHz	Nokia

R1-2410628	Channel model adaptation/extension of TR38.901 for 7-24 GHz	CEWiT	Late submission

R1-2410767	Summary#1 of channel model adaptation and extension	Moderator (ZTE Corporation)

From Tuesday session

Agreement

For near-field channel, the following formula is adopted to model the angular domain parameters of direct path between TRP and UE as antenna element-wise channel parameter:

Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

Where θLOS,ZOA,u,s, ϕLOS,AOA,u,s, θLOS,ZOD,u,s, ϕLOS,AOD,u,s are the respective antenna element-wise elevation arrival angles, azimuth arrival angles, elevation departure angles and azimuth departure angles of LoS path between the transmit antenna element s and receive antenna element u.

R1-2410768	Summary#2 of channel model adaptation and extension	Moderator (ZTE Corporation)

From Wednesday session

Working Assumption

- For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the nearest K blockers from BS are selected.

- For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, at least for blockage model B, the following parameters to define the building edge are introduced in the Table 7.6.4.2-5 in TR 38.901:

|  | Typical set of blockers | Blocker dimensions | Mobility pattern |
| --- | --- | --- | --- |
| Outdoor | Building edge | Cartesian: w=50 m; h=20 m | Stationary |

  - In the blockage model B, the following equation is used to calculate the attenuation caused by the building edge:

    - LdB=20log100.5-F, where F represents one of Fh1, Fh2, Fv1, and Fv2 defined in equation 7.6-29 in section 7.6.4.2 of TR 38.901.

Agreement

For the modelling of spatial non-stationarity at BS side, if stochastic based approach, at least for unified visible probability (VP) and visibility region (VR), is adopted, the following steps are considered:

- Step 1: Determine whether a cluster is impacted by SNS

- Step 2: Generate the visibility region

- Step 3: Calculate the power attenuation factor

FFS: the details of each step

Note: Merge of some of the steps is not precluded

Working Assumption

For the modelling of spatial non-stationarity at UE side mainly due to close proximity, if blocker-based approach is adopted, the Option-3 is supported with following details:

- Three typical cases are introduced: one hand grip, dual hand grip, head and one hand grip.

  - FFS: probability of each case in the simulation.

- For each case, to model the attenuation for each of antenna in the candidate antenna location defined in the UE antenna model for handheld device with one of following alternatives:

  - Alt 1: Introduce a fixed value for each candidate antenna position per case.

  - Alt 2: Introduce a distribution for candidate antenna position.

    - FFS: distribution per antenna position(s) and per case.

R1-2410769	Summary#3 of channel model adaptation and extension	Moderator (ZTE Corporation)

From Thursday session

Agreement

For near-field channel, to generate the channel parameter, the reference point is defined as:

- The physical center of the antenna array/center of the device.

Conclusion

For near-field channel, no changes are expected on following parameter for direct path between TRP and UE:

- Doppler shift

Conclusion

For near-field channel, no changes are expected on following parameter for non-direct path between TRP and UE:

- Doppler shift

Agreement

For near-field channel, the following formula is adopted to model the angular domain parameters of non-direct path between TRP and UE as antenna element-wise channel parameter:

Frx,u,θθn,m,ZOA,u,ϕn,m,AOA,uFrx,u,ϕθn,m,ZOA,u,ϕn,m,AOA,uTexpjΦn,mθθkn,m-1expjΦn,mθϕkn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,s,ϕn,m,AOD,sFtx,s,ϕθn,m,ZOD,s,ϕn,m,AOD,s

where θn,m,ZOA,u,ϕn,m,AOA,u, θn,m,ZOD,s,ϕn,m,AOD,s are the ray-wise angular domain parameters of ray m cluster n between the transmit antenna element s and receive antenna element u.

Agreement

For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the rotation and power variation calculation are conducted in ray level.

Agreement

For near-field channel, for the non-direct paths, the distance d1 between the BS and the 1st point associated with cluster is generated with one of following options that to be selected in RAN1#120:

- Option-1: The distance d1 is generated directly following a specific distribution.

  - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster

  - For other k2 clusters, the distance  d1 is equal or less than the speed of light times the absolute delay of the cluster and generated following a specific distribution

    - FFS: The specific distribution, e.g., log-normal distribution for UMi

    - FFS: The values of k1 and k2

  - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901

    - FFS: The value for UMi, UMa, InH-Office.

- Option-2: The distance d1 is generated by a scaling factor multiplied by the absolute distance corresponding to the absolute delay of the cluster, where the scaling factor is generated from a specific distribution.

  - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster (i.e., the scaling factor is 1)

  - For other k2 clusters, the distance  d1 is equal or less than the speed of light times the absolute delay of the cluster and the scaling factor generated following a specific distribution (i.e., the scaling factor is equal or less than 1)

    - FFS: Distribution of scaling factor, e.g., Beta distribution for UMa

    - FFS: The values of k1 and k2

  - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901

    - FFS: The value for UMi, UMa, InH-Office.

Note 1: If d2 is agreed to be considered, the generation of d2 can follow the same principle with the generation of d1.

Working Assumption

For near-field channel, for the non-direct paths, the impact of spherical wavefront is optionally considered at UE side.

R1-2410880	Summary#4 of channel model adaptation and extension	Moderator (ZTE Corporation)

Agreement

For near-field channel, in principle, one of following options is considered to calculate the antenna element-wise delay of direct path between TRP and UE:

- Option-1: The following formula is used to calculate the antenna element-wise delay of direct path:

τ1,u,s=τ1-d3D-|ru,s|c

where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side.

  - The above formula is applied with the same principle as existing model in Clause 7.6.2 of TR 38.901, and the channel response of LOS ray between Rx antenna u and Tx antenna s at delay τ at time t is given by:

Hu,s,1LOSt;τ=Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

exp-j2πd3Dλfexp-j2πru,s-d3Dλfexpj2πrrx,LOST∙vλ0tδτ-τ1,u,s

- Option-2: The antenna element-wise delay of direct path is directly modeled based on the distance of direct path between antenna elements at TRP and UE side.

  - Following formula is adopted to model the antenna element-wise delay in the channel coefficient as:

δτ-τ1,u,s,  τ1,u,s=τ1-d3D-|ru,s|c

where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side.

- Option-3: Confirm the following working assumption made in RAN1#118BIS:

  - “The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.”

- Note: τ1 is same as that is defined in 7.5-30 in TR38.901

Final summary in R1-2410881.

## NR mobility enhancements Phase 4

Please refer to RP-242356 for detailed scope of the WI.

[119-R19-Mobility] – Yosuke (Fujitsu)

Email discussion on channel modelling for NR mobility enhancement

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### Measurements related enhancements for LTM

R1-2409381	Discussion on measurements related enhancements for LTM	ZTE Corporation, Sanechips

R1-2409422	Measurements related enhancements for LTM	Huawei, HiSilicon

R1-2409492	Discussion on measurements related enhancements for LTM	LG Electronics

R1-2409525	Discussion on measurements related enhancements for LTM	CMCC

R1-2409612	Views on Rel-19 measurement related enhancements for LTM	Samsung

R1-2409648	Discussion on measurements related enhancements for LTM	Spreadtrum, UNISOC

R1-2409696	Discussion on measurements related enhancements for LTM	vivo

R1-2409771	Measurement related enhancements for LTM	Nokia

R1-2409820	Measurement related enhancements for LTM	Apple

R1-2409909	Discussion on measurements related enhancements for LTM	Xiaomi

R1-2409956	Discussion on measurements related enhancements for LTM	CATT

R1-2409973	Measurements related enhancements for LTM	Lenovo

R1-2409975	Measurement related enhancements for LTM	Ericsson

R1-2409991	Discussion on measurement related enhancements for LTM	Lekha Wireless Solutions

R1-2410017	Measurements related enhancements for LTM	InterDigital, Inc.

R1-2410041	Measurements enhancements for LTM	TCL

R1-2410115	Discussions on measurement enhancement for LTM	OPPO

R1-2410124	Discussion on measurement related enhancements for LTM	Fujitsu Limited

R1-2410168	Discussion on measurements related enhancements for LTM	Google

R1-2410212	Discussion on measurements related enhancements for LTM	NEC

R1-2410238	Measurements related enhancements for LTM	Sony

R1-2410244	LTM measurements related enhancements	MediaTek Inc.

R1-2410276	Measurements related enhancements for LTM	ETRI

R1-2410348	Measurement Enhancements for LTM	Meta

R1-2410403	Discussion on measurement related enhancements for LTM	NTT DOCOMO, INC.

R1-2410432	Discussion on measurements related enhancements for LTM	KDDI Corporation

R1-2410493	Measurements related enhancement for LTM	Qualcomm Incorporated

R1-2410543	Discussion on measurements related enhancements for LTM	Sharp

R1-2410186	FL summary 1 of Measurements related enhancements for LTM	Moderator (Fujitsu)

From Monday session

Agreement

For the identification of the serving cell RS for event evaluation,

If the RS(s) for candidate cell(s) are CSI-RS configured in a CSI-RS resource set configured with repetition, QCL RS of the indicated TCI-state is used for the serving cell; otherwise, SSB QCLed with QCL RS of the indicated TCI-state is used for the serving cell.

- UE does not expect the following configuration:

  - CSI-RS resource in the indicated TCI state of serving cell is NOT configured in a CSI-RS resource set configured with repetition, and

  - CSI-RS is configured as measurement resource for the candidate cell(s).

Agreement

CSI-RS for BM as the referenceSignal with QCL-TypeD is supported for an LTM TCI state, where TRS is configured as referenceSignal with QCL-TypeA

R1-2410187	FL summary 2 of Measurements related enhancements for LTM	Moderator (Fujitsu)

From Wednesday session (declared as WA in Thursday session with the modified red text)

Working Assumption

As baseline, CSI-RS measurement and CSI reporting operations are performed after reception of LTM CSC MAC CE.

- The report is sent directly to target cell

- Introduce UE capability for CSI-RS measurement can start before reception of LTM CSC MAC CE

  - Other than UE capability, there is no strive for no additional spec impact compared to the baseline (only one triggering mechanism will be specified)

R1-2410188	FL summary 3 of Measurements related enhancements for LTM	Moderator (Fujitsu)

From Thursday session

Conclusion

- No consensus to support CSI-RS for mobility for L1 measurement in Rel-19 LTM.

- Note: From the actual gNB transmission viewpoint, CSI-RS for mobility and CSI-RS for BM may be the same.

Final summary in R1-2410189.

## XR (eXtended Reality) for NR Phase 3

Please refer to RP-241771 for detailed scope of the WI.

[119-R19-XR] – Margarita (Nokia)

Email discussion on Rel-19 XR Phase 3

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### Enabling TX/RX for XR during RRM measurements

R1-2409426	Discussions on scheduling enhancements considering RRM measurements for XR	Huawei, HiSilicon

R1-2409526	Discussion on enabling TX/RX for XR during RRM measurements	CMCC

R1-2409536	Enhancements to enable TX/RX for XR during RRM measurements	Fraunhofer IIS, Fraunhofer HHI

R1-2409562	Discussion on measurement gap for XR	ZTE Corporation, Sanechips

R1-2409613	Remaining topics on enabling TX/RX during RRM measurements for XR	Samsung

R1-2409649	Discussion on enabling TXRX for XR during RRM measurements	Spreadtrum, UNISOC

R1-2409697	Discussion on enabling data transmissions for XR during RRM measurements	vivo

R1-2409715	Enabling TX/RX for XR during RRM measurements	Nokia

R1-2409732	Enabling TX/RX for XR during RRM measurements	Lenovo

R1-2409775	Discussion on enabling TX/RX for XR during RRM measurements	InterDigital, Inc.

R1-2409822	Enhancements for XR	Apple

R1-2409863	Discussion on enabling TX/RX for XR during RRM measurements	NEC

R1-2409882	Discussion on enabling TX/RX for XR during RRM measurements	Xiaomi

R1-2409930	Signaling control of scheduling restriction during measurement gap in support of XR services	CATT

R1-2410096	Enhancements to enable TX/RX for XR during RRM measurements	OPPO

R1-2410159	Discussion on Enabling TX/RX for XR During RRM Measurements	Meta

R1-2410200	Discussion on enabling TX/RX for XR during RRM measurements	Panasonic

R1-2410239	On Enabling TX/RX for XR during RRM measurements	Sony

R1-2410245	Enabling TX RX for XR during RRM measurements	MediaTek Inc.

R1-2410297	Discussion on XR during RRM measurements	LG Electronics

R1-2410323	On enabling Tx/Rx for XR during RRM measurements 	Google Ireland Limited

R1-2410404	Discussion on enabling TX/RX for XR during RRM	NTT DOCOMO, INC.

R1-2410494	Enabling Tx/Rx for XR during RRM measurements	Qualcomm Incorporated

R1-2410535	Enabling TX/RX for XR traffic during RRM measurement gaps	Ericsson

R1-2410741	Moderator summary #1 - Enabling TX/RX for XR during RRM measurements	Moderator (Nokia)

From Tuesday session

Agreement

For explicit indication by DCI, bit value equal to “1” indicates the corresponding gap/restriction occasion is to be “skipped”.

- FFS: Interpretation of bit value equal to “0”.

Proposal 2.1.5.3a-v2:

For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI.

- Supported by: Qualcomm, Spreadtrum, vivo, OPPO, MediaTek, ZTE, Apple

- Strong concerns: Samsung, CATT

Proposal 2.1.5.3b-v2:

For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

- Supported by: Samsung, LGE, Huawei, InterDigital, CATT, Meta

- Strong concerns: vivo, Apple, ZTE, Qualcomm

R1-2410742	Moderator summary #2 - Enabling TX/RX for XR during RRM measurements	Moderator (Nokia)

From Friday session

Agreement

A component of an FG can have Option 1 as a first value and Option 2 as a second value:

- Option 1: For explicit indication by DCI, bit value equal to “0” means UE ignores the indication of this field in the DCI.

- Option 2: For explicit indication by DCI, bit value equal to “0” means UE behaviour for the corresponding gap/restriction occasion is as per legacy behaviour.

Note: UE indicates only one value of this component.

Agreement

For explicit indication by DCI to skip a particular gap/restriction, in addition to DCI formats 0_1/1_1 and 0_2/1_2, one bit indication can be included as a part of DCI formats 0_3/1_3.

- Explicit indication can be configured for each DCI format individually by higher layer.

- The skipping indication corresponds to the first gap/restriction occasion among co-scheduled cells by the DCI (Type-1C field)

Above applies only for intra-band CA cases (same SCS for cells in the set of co-scheduled cells).

## Non-Terrestrial Networks (NTN) for NR Phase 3, Internet of Things (IoT) Phase 3, and IoT-NTN TDD mode

Please refer to RP-241789 for detailed scope of the WI for NR-NTN Phase 3. Please refer to RP-242397 for detailed scope of the WI for IoT-NTN Phase 3. Please refer to RP-242415 for detailed scope of the WI for Introduction of IoT-NTN TDD mode.

R1-2410848	Session notes for 9.11 (Non-Terrestrial Networks for NR Phase 3 and Internet of Things Phase 3)	Ad-Hoc Chair (Huawei)

Friday decision: The session notes are endorsed and contents reflected below.

[119-R19-NTN] – Mohamed (Thales)

Email discussion on Rel-19 NTN enhancement

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2409439	Work plan for Rel-19 NR_NTN_Ph3	THALES, CATT	Late submission

R1-2409564	Work plan for WID: introduction of IoT-NTN TDD mode	Iridium Satellite LLC

From AI 5

R1-2409343	LS on OCC for CB-msg3 NPUSCH	RAN2, vivo

Decision: Response is needed – Martin (Sony).

From Wednesday session

Agreement

The following response is sent in reply to RAN2 LS R1-2409343:

Response to RAN2: The OCC solution that RAN1 is working on may be applied to CB-Msg3 NPUSCH format 1 single tone at least for OCC length 2, for the subcarrier spacing(s) for which CB-Msg3 is supported, when the power imbalance is small. The design details for the OCC solution are ongoing.

Action to RAN2: RAN1 respectfully asks RAN2 to take the above information into account.

Comeback for draft LS (Martin)

From Thursday session

R1-2410894	Draft Reply LS on OCC for CB-msg3 NPUSCH	Moderator (Sony)

Agreement

The draft LS reply to RAN2 is endorsed in R1-2410894. Final LS is approved in R1-2410895.

### NR-NTN downlink coverage enhancement

R1-2409405	Discussion on downlink coverage enhancements for NR NTN	Huawei, HiSilicon

R1-2409434	NR NTN Downlink coverage enhancements	THALES

R1-2409451	Discussions on downlink coverage enhancement for NR NTN	Fraunhofer IIS, Fraunhofer HHI

R1-2409473	Discussion on NTN System Level Downlink Coverage Enhancement	EchoStar

R1-2409527	Discussion on NR-NTN DL coverage enhancement	CMCC

R1-2409614	Discussion on downlink coverage enhancement for NR-NTN	Samsung

R1-2409650	Discussion on NR-NTN downlink coverage enhancement	Spreadtrum, UNISOC

R1-2409698	Discussion on NR-NTN downlink coverage enhancement	vivo

R1-2409719	NR-NTN downlink coverage enhancement	InterDigital, Inc.

R1-2409726	Discussion on DL coverage enhancement for NR NTN	ZTE Corporation, Sanechips

R1-2409823	On NR-NTN Downlink Coverage Enhancement	Apple

R1-2409831	Discussion on NR-NTN downlink coverage enhancement	LG Electronics

R1-2409870	NR-NTN downlink coverage enhancement	NEC

R1-2409883	Discussion on NR-NTN downlink coverage enhancement	Xiaomi

R1-2409958	Discussion on DL coverage enhancements for NR-NTN	NICT

R1-2409978	Further consideration on downlink coverage enhancement for NR NTN	CATT

R1-2410064	Discussion on NR-NTN downlink coverage enhancement	TCL

R1-2410079	Discussion on NR-NTN downlink coverage enhancement	OPPO

R1-2410129	Discussion on downlink coverage enhancements	Fujitsu

R1-2410160	NR-NTN downlink coverage enhancement techniques	Sharp

R1-2410169	On NR-NTN downlink coverage enhancement	Ericsson

R1-2410182	Discussion on NR-NTN downlink coverage enhancement	HONOR

R1-2410277	Discussion on NR-NTN downlink coverage enhancement	ETRI

R1-2410282	NR-NTN Downlink Coverage Enhancement	Panasonic

R1-2410364	Discussion on downlink coverage enhancement for NR NTN	Lenovo

R1-2410405	Discussion on DL coverage enhancement for NR-NTN	NTT DOCOMO, INC.

R1-2410415	Discussion on downlink coverage enhancement for NR NTN	Baicells

R1-2410495	Downlink coverage enhancement for NR NTN	Qualcomm Incorporated

R1-2410526	NR-NTN downlink coverage enhancement	MediaTek Inc.

R1-2410546	Downlink Coverage Enhancement for NR NTN	Google Ireland Limited

R1-2410553	Further discussion on downlink coverage enhancements for NR over NTN	Nokia, Nokia Shanghai Bell

R1-2410567	Discussion on NR-NTN DL coverage enhancement	KT Corp.

R1-2410584	Discussion on Downlink Coverage Enhancements for NR NTN	CEWiT

R1-2410619	Operator views on DL Coverage Enhancements for NR NTN	Inmarsat, Viasat

R1-2409435	FL Summary #1: NR-NTN downlink coverage enhancements	THALES

From Tuesday session

Agreement

For PDSCH with Msg4 Link level enhancement:

- Support PDSCH repetition

  - FFS: signalling design including number of repetitions

  - FFS: impact on UE capability

- Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB

- Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

R1-2409436	FL Summary #2: NR-NTN downlink coverage enhancements	THALES

From Thursday session

Observation

Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan.

Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

Agreement

For link level enhancement of PDSCH with SIB1:

- Support PDSCH repetitions within 20 ms duration

  - The number of repetitions is fixed to 2 repetitions

  - Further discuss the specification impact for at least the following:

    - Procedure and signaling (enabling repetitions, associated time resource determination, etc.)

- Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size.

- Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

- Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern.

Agreement

For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN.

- FFS: intra-slot and/or inter-slot

Final summary in R1-2409438.

### Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

R1-2409406	Discussion on HD-FDD RedCap UEs and eRedCap UEs for FR1-NTN	Huawei, HiSilicon

R1-2409528	Discussion on the collision issues of HD-FDD Redcap UE in FR1-NTN	CMCC

R1-2409615	Discussion on support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	Samsung

R1-2409651	Discussion on support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	Spreadtrum, UNISOC

R1-2409699	Discussion on support of RedCap and eRedCap UEs with NR-NTN	vivo

R1-2409720	Discussion on half-duplex RedCap issues for NTN FR1 operation	InterDigital, Inc.

R1-2409727	Discussion on support of RedCap/eRedCap UEs for NR NTN	ZTE Corporation, Sanechips

R1-2409824	Discussion on support of RedCap UEs with NR NTN operation	Apple

R1-2409832	Discussion on support of (e)RedCap UEs with NR-NTN operating in FR1-NTN bands	LG Electronics

R1-2409884	Discussion on the support of Redcap and eRedcap UEs in NR NTN	Xiaomi

R1-2409979	Discussion on the enhancement of RedCap and eRedCap UEs In NTN	CATT

R1-2410008	Discussion on Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	China Telecom

R1-2410065	Discussion on HD-FDD Redcap UEs and eRedcap UEs for FR1-NTN	TCL

R1-2410080	Discussion on supporting of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	OPPO

R1-2410183	Discussion on support of (e)RedCap UEs in NR NTN	HONOR

R1-2410278	Discussion on HD UEs with NR NTN	ETRI

R1-2410305	On HD-FDD Redcap UEs for NTN	Ericsson

R1-2410371	Discussion on support of RedCap/eRedCap UEs in NTN	CAICT

R1-2410406	Discussion on support of RedCap and eRedCap UEs in FR1-NTN	NTT DOCOMO, INC.

R1-2410434	Views on the support of (e)RedCap UEs with NR NTN	SHARP Corporation

R1-2410496	Support of Redcap and eRedcap UEs in NR NTN	Qualcomm Incorporated

R1-2410527	Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	MediaTek Inc.

R1-2410532	Support of RedCap and eRedCap UEs  in NR NTN	Nordic Semiconductor ASA

R1-2410554	Additional considerations on (e)RedCap operation in NR over NTN	Nokia, Nokia Shanghai Bell

R1-2410806	Summary #1 for 9.11.2 Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	Moderator (CATT)

Presented in Tuesday session.

R1-2410807	Summary #2 for 9.11.2 Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands	Moderator (CATT)

From Thursday session

Agreement

For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3,

- Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note.

- For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all the other use cases above.

    - This is signaled by RRC configuration.

Note: UE shall comply to the following existing procedure in 38.331:

UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

Agreement

The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode.

- FFS whether to prioritize UL or prioritize DL.

Final summary in R1-2410808.

### NR-NTN uplink capacity/throughput enhancement

R1-2409407	Discussion on uplink capacity/throughput enhancement for FR1-NTN	Huawei, HiSilicon

R1-2409529	Discussion on the NR-NTN uplink capacity/throughput enhancements	CMCC

R1-2409616	Discussion on uplink capacity/throughput enhancement for NR-NTN	Samsung

R1-2409652	Discussion on NR-NTN uplink capacity/throughput enhancement	Spreadtrum, UNISOC, SGITG

R1-2409700	Discussion on NR-NTN uplink capacity enhancement	vivo

R1-2409721	NR-NTN uplink capacity/throughput enhancement	InterDigital, Inc.

R1-2409728	Discussion on UL capacity enhancement for NR NTN	ZTE Corporation, Sanechips

R1-2409825	On NR-NTN Uplink Capacity Enhancement	Apple

R1-2409833	Discussion on NR-NTN uplink capacity/throughput enhancement	LG Electronics

R1-2409838	Discussion on the NR-NTN uplink capacity/throughput enhancements	TCL

R1-2409871	NR-NTN uplink capacity/throughput enhancement	NEC

R1-2409885	Discussion on NR-NTN PUSCH capacity enhancement	Xiaomi

R1-2409959	Discussion on NR-NTN uplink capacity/throughput enhancement	NICT

R1-2409980	Discussion on UL capacity enhancement for NR NTN	CATT

R1-2410009	Discussion on NR-NTN uplink enhancement	China Telecom

R1-2410081	Discussion on NR-NTN uplink capacity/throughput enhancement	OPPO

R1-2410130	Discussion on uplink capacity/cell throughput enhancement for FR1-NTN	Fujitsu

R1-2410184	Discussion on NR-NTN UL capacity & throughput enhancement	HONOR

R1-2410279	Discussion on NR-NTN uplink capacity/throughput enhancement	ETRI

R1-2410304	Discussion on NR-NTN Uplink Capacity/Throughput Enhancement 	Lenovo

R1-2410343	Views on KPIs for uplink capacity enhancement for NR NTN 	ViaSat Satellite Holdings Ltd

R1-2410357	Uplink capacity/throughput enhancement for NR-NTN	Panasonic

R1-2410669	Discussion on NR-NTN uplink capacity/throughput enhancement	NTT DOCOMO, INC.	(rev of R1-2410407)

R1-2410413	NR-NTN uplink capacity enhancement	Sharp

R1-2410497	NR-NTN uplink capacity / throughput enhancement	Qualcomm Incorporated

R1-2410528	NR-NTN uplink capacity and throughput enhancements	MediaTek Inc.

R1-2410555	Further discussion on uplink capacity enhancements for NR over NTN	Nokia, Nokia Shanghai Bell

R1-2410562	On uplink capacity/cell throughput enhancement for NR NTN	Ericsson

R1-2410640	Feature lead summary #1 of AI 9.11.3 on NR-NTN uplink capacity and throughput enhancements	Moderator (MediaTek)

From Monday session

Agreement

RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows:

- Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs.

- Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences

- RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs.

- RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs.

Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4.

Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact.

- RAN1 assumes no specification impact for CFO grouping

- RAN1 does not pursue closed-loop frequency adjustment commands.

- RAN1 assumes that RAN4 does not define new UE requirements for CFO.

Observation:

Option 1 Inter-slot OCC with OCC length 4 to multiplex up to 4 UEs with 2 PRBs can meet VoIP 2% BLER within 1 dB of single UE baseline for 8 slots or larger with UE grouping with similar CFO (e.g. maximum differential CFO of 200 Hz).

Observation:

Option 2 Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs with TBoMS can meet VoIP 2% BLER target within 1 dB of single UE baseline with 2 PRBs and 4 repetitions or larger; and with 1 PRB and 8 repetitions or larger, without need for CFO grouping.

R1-2410641	Feature lead summary #2 of AI 9.11.3 on NR-NTN uplink capacity and throughput enhancements	Moderator (MediaTek)

From Wednesday session

Agreement

For RV cycling for OCC with DG-PUSCH, the following are considered:

- Option 1: RV cycling is used across OCC groups

  - Note 1: RV cycling is applied when the number of repetitions is greater than the OCC length

- Option 2: Fixed RV is used across OCC groups

- Option 3: For OCC length 2, fixed RV is used across two OCC groups, RV cycling is used across groups of two OCC groups

Agreement

If PUCCH without repetition overlaps with inter-slot OCC with any PUSCH repetitions in an OCC group, the following options to be considered:

- Option 1: UCI is dropped

  - FFS: whether all UCI is dropped

- Option 2: UCI is transmitted on PUCCH, and all PUSCH repetitions within the OCC group are dropped.

- Option 3: UCI is multiplexed on PUSCH with inter-slot OCC

  - Option 3-a: UCI is multiplexed on all PUSCH repetitions within an OCC group with inter-slot OCC

    - FFS: which OCC group

  - Option 3-b: UCI is multiplexed on PUSCH and OCC is not applied within the OCC group

  - Option 3-c: UCI is multiplexed on PUSCH and OCC is not applied within the PUSCH repetitions

Note: combination of the above can be considered

FFS Details timeline of PUCCH and PUSCH

FFS: handling of PUCCH with repetition

FFS: handling of different UCI types

Final summary in R1-2410642.

### IoT-NTN uplink capacity/throughput enhancement

R1-2409408	Discussion on UL capacity enhancements for IoT NTN	Huawei, HiSilicon

R1-2409530	Discussion on the IoT -NTN uplink capacity/throughput enhancements	CMCC

R1-2409617	Discussion on uplink capacity/throughput enhancement for IoT-NTN	Samsung

R1-2409653	Discussion on IoT-NTN uplink capacity/throughput enhancement	Spreadtrum, UNISOC

R1-2409701	Discussion on IoT-NTN uplink capacity enhancement	vivo

R1-2409722	IoT-NTN uplink capacity/throughput enhancement	InterDigital, Inc.

R1-2409729	Discussion on UL capacity enhancement for IoT NTN	ZTE Corporation, Sanechips

R1-2409826	Discussion on IoT-NTN Uplink Capacity Enhancement	Apple

R1-2409834	Discussion on IoT-NTN uplink capacity/throughput enhancement	LG Electronics

R1-2409839	Discussion on the IoT-NTN uplink capacity/throughput enhancements	TCL

R1-2409848	IoT-NTN uplink capacity enhancement	Nokia, Nokia Shanghai Bell

R1-2409872	IoT-NTN uplink capacity/throughput enhancement	NEC

R1-2409886	Discussion on IoT-NTN uplink capacity enhancement	Xiaomi

R1-2409981	Discussion on UL capacity enhancement for IoT NTN	CATT

R1-2410082	Discussion on IoT-NTN uplink capacity/throughput enhancement	OPPO

R1-2410161	IoT NTN OCC methods and signaling for NPUSCH and NPRACH	Sharp

R1-2410240	On IoT-NTN uplink capacity enhancements	Sony

R1-2410280	Discussion on uplink capacity/throughput enhancement for IoT NTN	ETRI

R1-2410306	On uplink capacity enhancements for IoT-NTN	Ericsson

R1-2410365	Discussion on uplink capacity enhancement for IoT NTN	Lenovo

R1-2410498	IOT-NTN uplink capacity/throughput enhancement	Qualcomm Incorporated

R1-2410503	Operator views on KPIs for uplink capacity enhancement for IOT- NTN 	ViaSat Satellite Holdings Ltd

R1-2410506	IoT-NTN uplink capacity/throughput enhancement	Nordic Semiconductor ASA

R1-2410529	IoT-NTN uplink capacity and throughput enhancement	MediaTek Inc.

R1-2410763	FL Summary #1 for IoT-NTN	Moderator (Sony)

From Monday session

Agreement

For 3.75kHz SCS OCC for NPUSCH format 1, the maximum OCC length is 2 for connected mode.

Agreement

For single tone 15kHz SCS Slot-level OCC for NPUSCH format 1, OCC length larger than 2 is not supported.

R1-2410764	FL Summary #2 for IoT-NTN	Moderator (Sony)

From Wednesday session

Agreement

For NPUSCH Format 1 single-tone 15kHz SCS, RAN1 studies at least the following options for CDM DMRS with legacy pattern for down-selection:

- Option 1: DMRS symbols are spread before the OCC is applied, e.g. according to the formula:

ru,OCCn=ru(nM)q(n mod M)

Where: M is the OCC length, q is the assigned OCC codeword for the UE and run is the reference signal sequence defined in TS36.211 section 10.1.4.1.1

- Option 2: DMRS symbols are not spread before the OCC is applied.

  - Option 2_1: OCC is applied to the legacy complex-valued DMRS symbol used in slot 1 and slot 2, e.g. according to the formula: ru,OCCn=ru(n)q(n mod M)

  - Option 2_2: OCC is applied to the complex-valued DMRS symbol used in slot 1 and slot 2. Depending on the OCC codeword, different DMRS sequence is used.

- Option 3: DMRS symbols are not spread and OCC is not applied.

  - Legacy complex-valued DMRS symbol is used in slots corresponding to an OCC codeword of NPUSCH. Different DMRS sequences are used for multiplexed UEs.

Agreement

For NPUSCH Format 1 single-tone 15kHz SCS, the slot-level scheme for non-DMRS symbols is that spreading is performed in the unit of one slot.

- Note: whether RU length is extended or not after applying OCC is a separate discussion.

Agreement

For support of single-tone OCC for NPUSCH format 1 for connected mode, the parameters that need to be signalled are:

- OCC sequence index

- Enabling of OCC feature

- FFS: whether signaling is explicit or implicit

Final summary in R1-2410765.

### IoT-NTN TDD mode

Prioritize discussion on study phase objective in RAN1#119.

R1-2409409	Discussion on IoT-NTN TDD mode	Huawei, HiSilicon

R1-2409440	Discussion on IoT-NTN TDD mode	THALES

R1-2409531	Discussion on the new NB-IoT NTN TDD mode	CMCC

R1-2409565	Analysis - loT-NTN TDD mode DL synchronization and SIB scheduling	Iridium Satellite LLC

R1-2409618	Discussion on IoT-NTN TDD mode	Samsung

R1-2409654	Discussion on IoT-NTN TDD mode	Spreadtrum, UNISOC

R1-2410672	Discussion on IoT-NTN TDD mode	vivo	(rev of R1-2409702)

R1-2409730	Discussion on the IoT-NTN TDD mode	ZTE Corporation, Sanechips

R1-2409827	Discussion on IoT-NTN TDD mode	Apple

R1-2409835	Discussion on IoT-NTN TDD mode	LG Electronics

R1-2409849	Discussion on IoT-NTN TDD mode	Nokia, Nokia Shanghai Bell

R1-2409887	Discussion on IoT-NTN TDD mode	Xiaomi

R1-2409982	Evaluation on IoT-NTN TDD mode	CATT

R1-2410083	Discussion on IoT-NTN TDD mode	OPPO

R1-2410307	On TDD mode for IoT-NTN	Ericsson

R1-2410366	Discussion on IoT-NTN TDD mode	Lenovo

R1-2410499	IOT-NTN TDD mode	Qualcomm Incorporated

R1-2410570	On R19 TDD IoT-NTN	Nordic Semiconductor ASA

R1-2410687	Feature lead summary #1 on IoT-NTN TDD mode	Moderator (Qualcomm Incorporated)

From Wednesday session

Observation

RAN1 makes the following observations on downlink synchronization:

Case 1: For N=9, D=8:

- For NPSS:

  - For one-shot detection (90ms delay), 9 sources provided simulation results, with a median required SNR of 1.30dB. The link budget margin is (median margin):

    - 4.21 dB for LEO-1200 (0dBi antenna gain)

    - -1.89 dB for LEO-600 (-5.5dBi antenna gain).

  - For two-shot detection (180ms delay), 3 sources provided simulation results, with a median required SNR of -1.20dB. The link budget margin is (median margin):

    - 6.71dB for LEO 1200 (0dBi antenna gain)

    - 0.61dB for LEO-600 (-5.5dBi antenna gain).

  - For four-shot detection (360ms delay), 3 sources provided simulation results, with a median required SNR of -3.80dB. The link budget margin is (median margin):

    - 9.31dB for LEO-1200 (0dBi antenna gain)

    - 3.21dB for LEO-600 (-5.5dBi antenna gain).

  - NOTE: The assumptions for combining for 2-shot and 3-shot detection are different across companies.

- For NSSS:

  - For one-shot detection (180ms delay), 5 sources provided simulation results, with a median required SNR of -2.50dB. The link budget margin is (median margin):

    - 8.01dB for LEO-1200 (0dBi antenna gain)

    - 1.91dB for LEO-600 (-5.5dBi antenna gain)

- For NPBCH:

  - For 90ms combining, 4 sources provided simulation results, with a median required SNR of 3.48dB. The link budget margin is (median margin):

    - 2.03dB for LEO-1200 (0dBi antenna gain)

    - -4.07dB for LEO-600 (-5.5dBi antenna gain)

  - For 640ms combining, 6 sources provided results, with a median required SNR of  -4.25dB. The link budget margin is (median margin):

    - 9.76dB for LEO-1200 (0dBi antenna gain)

    - 3.66dB for LEO-600 (-5.5dBi antenna gain)

Case 2: For N=9, D=20:

- For NPSS:

  - For a combining of 2 consecutive NPSS (80ms delay), 1 source provided simulation results, with a median required SNR of -0.8dB. The link budget margin is (median margin):

    - 6.31dB for LEO-1200 (0dBi antenna gain)

    - 0.21dB for LEO-600 (-5.5dBi antenna gain)

- For NPBCH:

  - For 640ms combining, 1 source provided simulation results, with a median required SNR of -5.10dB. The link budget margin is (median margin):

    - 10.61dB for LEO-1200 (0dBi antenna gain)

    - 4.51dB for LEO-600 (-5.5dBi antenna gain)

Case 3: For N=9, D=30:

- For NPSS:

  - For combining of 3 consecutive NPSS (90ms delay), 1 source provided simulation results, with a median required SNR of -2.4 dB. The link budget margin is (median margin):

    - 7.91dB for LEO-1200 (0dBi antenna gain)

    - 1.81dB for LEO-600 (-5.5dBi antenna gain)

- For NPBCH:

  - For 640ms combining, 1 source provided simulation results, with a median required SNR of
 -7.10dB the link budget margin is (median margin):

    - 12.61dB for LEO-1200 (0dBi antenna gain)

    - 6.51dB for LEO-600 (-5.5dBi antenna gain)

NOTE: Other evaluated scenarios (LEO-600 with 0dBi antenna gain, LEO-1200 with -5.5dBi antenna gain, LEO-800 with 0dBi antenna gain) have margins in between LEO-1200 (0dBi antenna gain) and LEO-600 (-5.5dBi antenna gain)

NOTE: Different companies provided input for different cases. For the companies that evaluated multiple cases, the performance (in terms of acquisition delay) of Case 3 and Case 2 is better than the performance of Case 1.

Observation

Based on simulations submitted to this meeting, in terms of downlink synchronization (NPSS/NSSS/NPBCH), all the following cases meet the link budget requirements for IOT-NTN TDD and are feasible (from the downlink synchronization point of view) from RAN1 perspective:

- Case 1: N=9, D=8

- Case 2: N=9, D=20

- Case 3: N=9, D=30

Observation

In Rel-19, RAN1 concludes that at least N=9 is feasible from the following points of view:

- DL synchronization, based on the link level evaluations submitted at RAN#119.

- Coexistence with the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band.

Agreement

Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints:

- At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods.

  - The same downlink slot is used in all the 90ms periods.

- At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods.

  - The same uplink slot is used in all the 90ms periods.

- The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4).

- NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

Observation

RAN1 concludes that at least D=8 and U=8 is feasible with N=9 from point of view of the design constraints (from earlier agreement) imposed by the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band.

R1-2410874	Feature lead summary #2 on IoT-NTN TDD mode	Moderator (Qualcomm Incorporated)

From Thursday session

Conclusion

For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for some SIB1-NB TBS values, at least when the configured number of repetitions is 16 and when SIB1-NB TBS value <=X bits.

- When considering only NPSS/NSSS/NPBCH/SIB1-NB, and if the number of SIB1-NB repetitions is 16:

  - If additionalTransmissionSIB1 is set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 43.75% in the anchor carrier.

  - If additionalTransmissionSIB1 is not set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 37.5% in the anchor carrier.

  - NOTE: In the above calculations, overhead due to transmission of other SIBs has not been included.

- The value of X may be further studied during the normative phase. Note: X is a TBS value supported by the specifications for SIB1-NB.

Agreement

The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options:

- Option 1: Always the same across H-SFNs.

  - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9.

  - Option 1-2: H-SFN duration is unchanged

- Option 2: Different across some H-SFNs.

  - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB)

  - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

Agreement

RAN1 to further discuss how to handle the overlap of NPUSCH with non-U NB-IoT subframes and the overlap of NPRACH (including NPRACH occasions) with non-U NB-IoT subframes, including studying at least the following:

- New periodicities to align with the TDD structure.

- Postponement of a channel when it overlaps with non-U NB-IoT subframes.

- Restriction of a channel to be fully confined within a single set of U NB-IoT subframes.

- Dropping (full or partial) of channels when they overlap with non-U NB-IoT subframes.

- Impact on segmented pre-compensation.

Agreement

RAN1 to further discuss how to handle the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes, including studying at least the following:

- New periodicities to align with the TDD structure.

- Postponement of a channel when it overlaps with non-D NB-IoT subframes.

- Restriction of a channel to be fully confined within a single set of D NB-IoT subframes.

- Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes.

- The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications

Agreement

The downlink subframes within D=8 are to be down-selected among the following:

- Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames)

- Option 2: [4 5 6 7 8 9 0 1] (across two consecutive radio frames)

- Option 3: [8 9 0 1 2 3 4 5] (across two consecutive radio frames)

- Option 4: [9 0 1 2 3 4 5 6] (across two consecutive radio frames)

## Multi-Carrier Enhancements for NR Phase 2

Please refer to RP-242408 for detailed scope of the WI for Multi-Carrier Enhancements for NR Phase 2.

R1-2410849	Session notes for 9.12 (Multi-Carrier Enhancements for NR Phase 2)	Ad-Hoc Chair (CMCC)

Friday decision: The session notes are endorsed and contents reflected below.

[119-R19-MCE] – Haipeng (Lenovo)

Email discussion on Rel-19 multi-carrier enhancement

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### Multi-cell PUSCH/PDSCH scheduling with a single DCI

Including scheduling of different SCS/carrier types and (one or) multiple PUSCHs/PDSCHs per scheduled cell.

R1-2409404	Discussion on Rel-19 Multi-carrier enhancements	Huawei, HiSilicon

R1-2409484	Discussion on multi-cell scheduling with a single DCI	Lenovo

R1-2409532	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	CMCC

R1-2409541	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	ZTE Corporation, Sanechips

R1-2409619	Enhancements for multi-cell PUSCH/PDSCH scheduling	Samsung

R1-2409655	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	Spreadtrum, UNISOC

R1-2409703	Discussion on enhancement of multi-cell PUSCH/PDSCH scheduling with a single DCI	vivo

R1-2409716	On Rel-19 Multi-carrier enhancements for NR Phase 2	Nokia

R1-2409828	On multi-cell PUSCH/PDSCH scheduling with single DCI	Apple

R1-2409868	Discussion on multi-cell scheduling with a single DCI	NEC

R1-2409931	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	CATT

R1-2410010	Discussion on multi-carrier enhancements for NR phase 2	China Telecom

R1-2410066	Discussion on  Multi-cell PUSCH/PDSCH scheduling with a single DCI	TCL

R1-2410100	Discussion of multi-cell scheduling with a single DCI	OPPO

R1-2410250	Discussion on multi-carrier enhancements for NR Phase 2	Panasonic

R1-2410281	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	ETRI

R1-2410298	Discussion on single DCI based multi-cell scheduling for Rel-19	LG Electronics

R1-2410408	Discussion on multi-cell PUSCH/PDSCH scheduling with a single DCI	NTT DOCOMO, INC.

R1-2410500	Multi-cell PUSCH/PDSCH scheduling with a single DCI	Qualcomm Incorporated

R1-2410509	Multi-cell PUSCH/PDSCH scheduling with a single DCI	MediaTek Inc.

R1-2410536	Multi-cell PxSCH scheduling with a single DCI	Ericsson

R1-2410729	Feature lead summary#1 on multi-cell scheduling with a single DCI	Moderator (Lenovo)

From Monday session

Agreement

- Time-domain HARQ-ACK bundling is configured per cell as Rel-17.

R1-2410730	Feature lead summary#2 on multi-cell scheduling with a single DCI	Moderator (Lenovo)

From Tuesday session

Agreement

- Specification supports the maximum number of PUSCHs/PDSCHs for a scheduled cell by a DCI format 0_3/1_3 is 8.

- Payload size of a DCI format 0_3/1_3 exceeding 140 is not supported in Rel-19.

R1-2410731	Feature lead summary#3 on multi-cell scheduling with a single DCI	Moderator (Lenovo)

From Wednesday session

Agreement

Type-2 HARQ-ACK codebook is generated by concatenating a first sub-codebook and a second sub-codebook.

- The first sub-codebook comprises HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single PDSCH, or each scheduling a single cell with multiple PDSCHs on it and nrofHARQ-BundlingGroups configured as 1, and HARQ-ACK information bit(s) for DCI(s) having associated HARQ-ACK information without scheduling PDSCH reception.

- The second sub-codebook comprises HARQ-ACK information bits for PDSCHs scheduled by DCI(s) with each scheduling more than one cell, or each scheduling a single cell with multiple PDSCHs on it without nrofHARQ-BundlingGroups or nrofHARQ-BundlingGroups configured larger than 1.

- Separate DAI counting is applied for DCI(s) associated with the first sub-codebook and DCI(s) associated with the second sub-codebook as Rel-18.

- Note: For providing HARQ-ACK information corresponding to SCell dormancy indication, the UE assumes that the UE receives a PDSCH on the serving cell associated with fields in DCI format 1_3 used for SCell dormancy indication as Rel-18.

R1-2410732	Feature lead summary#4 on multi-cell scheduling with a single DCI	Moderator (Lenovo)

From Thursday session

Agreement

- For the second sub-codebook, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M, to select one of the following options.

  - Option 1: M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. M is implicitly derived based on RRC configuration.

  - Option 2: M is explicitly configured by RRC parameter for the PUCCH group.

  - Other options are not precluded.

Agreement

- For NDI indication in DCI format 0_3/1_3 for TB1, further study

  - Option 1: For each block of NDI field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3.

    - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

  - Option 2a: Total number of bits of the NDI field is equal to the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

    - The number of bits of a block of NDI field corresponding to a scheduled cell is equal to the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell.

    - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- Other options are not precluded.

- Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of NDI field for TB1, same as in Rel-18.

Agreement

- For RV indication in DCI format 0_3/1_3 for TB1, further study

  - Option 1: For each block of RV field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the cell.

    - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell.

  - Option 2a: Total number of bits of the RV field is determined based on the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

    - The number of bits of a block of RV field corresponding to a scheduled cell is determined based on the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell and number of bits for RV configured for the cell.

    - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- Other options are not precluded.

- Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of RV field for TB1, same as in Rel-18.

Final summary in R1-2410905.

## TEI

R1-2410850	Session notes for 9.13 (TEI19)	Ad-Hoc Chair (CMCC)

Friday decision: The session notes are endorsed and contents reflected below.

[119-R19-TEI] – Haruhi (NTT DOCOMO)

Email discussion on Rel-19 TEI proposals

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2409401	Enhancement of SRS beamforming for FR2 positioning	Huawei, HiSilicon, China Unicom, ZTE, Sanechips, CATT

R1-2409542	On Rel-19 TEI proposals	ZTE Corporation, Sanechips

R1-2409990	TEI-19 proposal on link adaptation enhancement	Ericsson

R1-2410246	CSI Acquisition Improvement for Type-II codebook	Orange, ZTE, BT

R1-2410624	TEI19: Counting of active NZP-CSI-RS resources	Nokia, Apple, Ericsson, MediaTek, NTT DOCOMO, Spreadtrum

R1-2410813	FL Summary #1 on Rel-19 TEIs	Moderator (NTT DOCOMO, INC.)

From Tuesday session

Agreement

For up to 32 HARQ process numbers for TN in FR1 and FR2-1,

- The value ranges of the new RRC parameters are defined as follows

  - harq-ProcessNumberSizeDCI-0-1-Ext-r19 with value range of ‘INTEGER (5)’,

  - harq-ProcessNumberSizeDCI-1-1-Ext-r19 with value range of ‘INTEGER (5)’,

  - harq-ProcessNumberSizeDCI-0-2-Ext-r19 with value range of ‘INTEGER (0..5)’,

  - harq-ProcessNumberSizeDCI-1-2-Ext-r19 with value range of ‘INTEGER (0..5)’,

  - harq-ProcessNumberSizeDCI-0-3-Ext-r19 with value range of ‘INTEGER (0..5)’,

  - harq-ProcessNumberSizeDCI-1-3-Ext-r19 with value range of ‘INTEGER (0..5)’.

R1-2410830	FL Summary #2 on Rel-19 TEIs	Moderator (NTT DOCOMO, INC.)

Presented in Wednesday session.

The other proposals require further work. To be checked in future meetings.

R1-2410891	FL Summary #3 on Rel-19 TEIs	Moderator (NTT DOCOMO, INC.)

## Rel-19 UE Features

### UE features for AI/ML for NR Air Interface

Placeholder only. No contributions in RAN1#119.

### UE features for NR MIMO Phase 5

For RAN1#119, in addition to individual company/organization/university tdocs, the rapporteurs are to provide initial input on UE features for Rel-19 MIMO.

R1-2410820	Session Notes of AI 9.14.2	Ad-Hoc Chair (AT&T)

Friday decision: The session notes are endorsed.

[119-R19-UE_features] – Ralf (AT&T)

Email discussion on Rel-19 UE features

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2409382	Discussion on UE features for NR MIMO Phase 5	ZTE Corporation, Sanechips

R1-2409431	UE features for NR MIMO Phase 5	Huawei, HiSilicon

R1-2409533	"Discussion on 	UE features for NR MIMO Phase 5"	CMCC

R1-2409620	UE features for NR MIMO Phase 5	Samsung

R1-2409621	Initial list of UE features for Rel-19 MIMO Ph5	Samsung (Moderator)

R1-2409704	Discussion on Rel-19 MIMO UE features	vivo

R1-2409768	Initial views on UE features NR MIMO Phase 5 WI	Nokia

R1-2409829	Views on UE features for NR MIMO Phase 5	Apple

R1-2409932	On UE features for NR MIMO Phase 5	CATT

R1-2410112	UE features for NR MIMO Phase 5	OPPO

R1-2410342	UE features for NR MIMO Phase 5	Ericsson

R1-2410501	UE features for NR MIMO phase 5	Qualcomm Incorporated

R1-2410819	Summary of UE features for NR MIMO Phase 5	Moderator (AT&T)

Summary of email discussion [119-R19-UE_features] during RAN1 #119. The agreements reached during RAN1 #119 are summarized in R1-2410820.

R1-2410821	Initial RAN1 UE features list for Rel-19 NR MIMO Phase 5	Moderators (AT&T, NTT DOCOMO, INC.)

### UE features for evolution of NR duplex operation: SBFD

Placeholder only. No contributions in RAN1#119.

### UE features for enhancements of network energy savings for NR

Placeholder only. No contributions in RAN1#119.

### UE features for LP-WUS/WUR for NR

Placeholder only. No contributions in RAN1#119.

### UE features for NR mobility enhancements Phase 4

Placeholder only. No contributions in RAN1#119.

### UE features for XR for NR Phase 3

Placeholder only. No contributions in RAN1#119.

### UE features for NTN for NR Phase 3

Placeholder only. No contributions in RAN1#119.

### UE features for NTN for Internet of Things (IoT) Phase 3

Placeholder only. No contributions in RAN1#119.

### UE features for IoT-NTN TDD mode

Placeholder only. No contributions in RAN1#119.

### UE features for MCE for NR Phase 2

Placeholder only. No contributions in RAN1#119.

### Others

For discussions on UE features for Rel-19 TEI and other relevant issues. Placeholder only. No contributions in RAN1#119.

# Closing of the meeting

Meeting was closed on November 22nd 18:35 UTC (excluding post email discussions - see Annex F).

# Annex A-1:	List of Tdocs at RAN1 #119

Please see excel file attached to this report

RAN1 Brk 2

RAN1 Brk 1

RAN1 Main

Annex A-2:	RAN1#119 Online Session Schedule

|  | Monday (start at 9:00AM) |  |  | Tuesday (start at 8:30AM) |  |  | Wednesday (start at 8:30AM) |  |  | Thursday (start at 8:30AM) |  |  | Friday (start at 8AM) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 08:30 ~ 10:30 (120 min) | RAN1#119 commences at 09:00 on Monday Agenda items 1, 2, 3, 4, 5 Agenda items 7 |  |  | Duplex (120) | 7–24GHz (90) R19 UE feat. (30) | A-IoT (120) | NES (120) | 7–24GHz (100) AI 7 (20) | A-IoT (120) | NES (90) AI 8.1 (30) | 7–24GHz (90) R19 UE feat. (30) | A-IoT (120) | NES (90) XR (30) LP-WUS (30) | AI/ML (150) | A-IoT (150) |
| Morning coffee break: 10:30 ~ 11:00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11:00 ~ 13:00 (120 min) | Agenda item 8.1 (60) . MIMO, NES, Others MIMO (60) |  |  | NES (120) | AI/ML (120) | ISAC (90) R18 UE feat. (30) | LP-WUS (100) AI 8.1 (20) | TEI (30) MCE (50) R19 UE feat. (40) | ISAC (90) AI 8.1 (30) | LP-WUS (90) Mobility (30) | AI/ML (120) | ISAC (90) AI 8.1 (30) | LP-WUS (60) |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Lunch break: 12:15 ~ 13:15 |  |  |
| Lunch break: 13:00 ~ 14:30 |  |  |  |  |  |  |  |  |  |  |  |  | Any other issues Session reports RAN1#119 expected to close at 15:30 |  |  |
| 14:30 ~ 16:30 (120 min) | MIMO (80) Mobility (40) | AI 7 (30) AI 8.1(60) MCE (30) | A-IoT (120) | AI 7, 8.1 (60) XR (30) MIMO (30) | AI/ML (120) | A-IoT (120) | Duplex (120) | AI/ML (120) | A-IoT (120) | Duplex (120) | AI/ML (120) | A-IoT (120) |  |  |  |
| Afternoon coffee break: 16:30 ~ 17:00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17:00 ~ 19:00 (120 min) | LP-WUS (120) | AI/ML (120) | AI 8.1 (30) NTN (90) | MIMO (30) LP-WUS (90) | TEI (50) AI 8.1(40) MCE (30) | NTN (120) | MIMO (90) Mobility (30) | AI/ML (120) | NTN (120) | Duplex (30) MIMO (90) | Start from 5:30 R18 UE feat. (20) AI 7,8.1 (30) MCE (40) | NTN (120) |  |  |  |
| All sessions end at 19:15 – no exceptions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Annex B:	List of CRs agreed at RAN1 #119

| TD# | Title | Source to WG | Rel | TS | Vsn | Work Item | CR | Rev | Cat |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2410794 | Correction to reference point for NG-RAN measurement for UL SRS-RSRP and UL SRS-RSRPP | Moderator (vivo) | Rel-18 | 38.215 | 18.3.0 | NR_NTN_enh-Core | 0070 |  | F |
| R1-2410797 | Correction on the support of open loop timing advance calculation for ATG | Moderators (Huawei, vivo), HiSilicon, Samsung | Rel-18 | 38.213 | 18.4.0 | NR_NTN_enh-Core | 0685 | 2 | F |
| R1-2410810 | CR on PDSCH reception scheduled by CORESET not following the unified TCI state | Moderator (MediaTek. Inc), ZTE Corporation, Sanechips, Samsung, Ericsson, NTT DOCOMO, OPPO, Google | Rel-17 | 38.214 | 17.11.0 | NR_FeMIMO-Core | 0647 |  | F |
| R1-2410814 | CR on PDSCH reception scheduled by CORESET not following the unified TCI state | Moderator (MediaTek. Inc), ZTE Corporation, Sanechips, Samsung, Ericsson, NTT DOCOMO, OPPO, Google | Rel-18 | 38.214 | 18.4.0 | NR_FeMIMO-Core | 0648 |  | A |
| R1-2410853 | Correction on TPMI determination for UL transmissions in 38.214 | Moderator (CATT), New H3C, Ericsson, ZTE, OPPO | Rel-18 | 38.214 | 18.4.0 | NR_MIMO_evo_DL_UL-Core | 0649 |  | F |
| R1-2410858 | CR on transmission attempt from the 2nd starting symbol | Moderator (NTT DOCOMO, INC.), OPPO, Huawei, HiSilicon | Rel-18 | 37.213 | 18.4.0 | NR_SL_enh2-Core | 0063 |  | F |
| R1-2410864 | CR on active semi-persistent SRS resource configuration and transmission of SRS | Moderator(vivo), Ericsson, Intel | Rel-18 | 38.214 | 18.4.0 | NR_pos_enh2-Core | 0650 |  | F |
| R1-2410865 | Draft CR on active semi-persistent SRS resource configuration | Moderator(vivo), Ericsson, Intel | Rel-17 | 38.214 | 17.11.0 | NR_pos_enh-Core | 0651 |  | F |
| R1-2410869 | CR on timeline of BW aggregation SRS for positioning | Moderator (ZTE), ZTE Corporation, Sanechips | Rel-18 | 38.214 | 18.4.0 | NR_pos_enh2-Core | 0652 |  | F |
| R1-2410878 | Corrections to SSB-RO association for LTM | Huawei, HiSilicon, Nokia, Google, Samsung, Fujitsu, Ericsson, NTT DOCOMO, ZTE | Rel-18 | 38.213 | 18.4.0 | NR_Mob_enh2-Core | 0691 |  | F |
| R1-2410879 | Corrections to PRACH transmission for LTM | Huawei, HiSilicon, Nokia, Google, Samsung, Fujitsu, Ericsson, NTT DOCOMO, ZTE | Rel-18 | 38.211 | 18.4.0 | NR_Mob_enh2-Core | 0147 |  | F |
| R1-2410882 | CR on Precoder Indication for 8 port CG-PUSCH | Moderator (Google), Google, ZTE, Sanechips, Ericsson, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Samsung | Rel-18 | 38.214 | 18.4.0 | NR_MIMO_evo_DL_UL-Core | 0653 |  | F |
| R1-2410920 | CR on LTM TA command application time | Moderator (MediaTek), NTT Docomo, ZTE, Nokia | Rel-18 | 38.213 | 18.4.0 | NR_Mob_enh2-Core | 0692 |  | F |
| R1-2410931 | Rel-18 editorial corrections for TS 38.212 | Huawei | Rel-18 | 38.212 | 18.4.0 | NR_MC_enh-Core, NR_cov_enh2-Core, NR_MIMO_evo_DL_UL-Core, NR_SL_enh2-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, Netw_Energy_NR-Core, NR_pos_enh2-Core | 0209 | 1 | F |
| R1-2410932 | Alignment of parameter names | Ericsson | Rel-17 | 38.211 | 17.9.0 | NR_newRAT-Core, TEI17, NR_SL_enh-Core, NR_NTN_solutions-Core | 0144 | 1 | F |
| R1-2410933 | Alignment of parameter names | Ericsson | Rel-18 | 38.211 | 18.4.0 | NR_newRAT-Core, TEI17, NR_SL_enh-Core, NR_NTN_solutions-Core | 0145 | 1 | A |
| R1-2410934 | Alignment of parameter names | Ericsson | Rel-18 | 38.211 | 18.4.0 | NR_MIMO_evo_DL_UL-Core, NR_Mob_enh2-Core | 0146 | 1 | F |
| R1-2410935 | Rel-17 editorial corrections for TS 38.213 | Samsung | Rel-17 | 38.213 | 17.11.0 | NR_newRAT-Core, TEI17, NR_FeMIMO-Core | 0687 | 1 | F |
| R1-2410936 | Rel-17 editorial corrections for TS 38.213 (mirrored to Rel-18) | Samsung | Rel-18 | 38.213 | 18.4.0 | NR_newRAT-Core, TEI17, NR_FeMIMO-Core | 0689 | 1 | A |
| R1-2410937 | Rel-18 editorial corrections for TS 38.213 | Samsung | Rel-18 | 38.213 | 18.4.0 | NR_newRAT-Core, TEI18, NR_MC_enh-Core, NR_MIMO_evo_DL_UL-Core, NR_SL_enh2-Core, NR_NTN_enh-Core, NR_pos_enh2-Core, NR_Mob_enh2-Core, Netw_Energy_NR-Core | 0688 | 1 | F |
| R1-2410938 | Rel-18 higher-layers parameter name corrections for TS 38.214 | Nokia | Rel-18 | 38.214 | 18.4.0 | NR_MIMO_evo_DL_UL-Core, NR_SL_enh2-Core, NR_MBS_enh-Core, Netw_Energy_NR-Core, NR_pos_enh2-Core, NR_Mob_enh2-Core | 0646 | 1 | F |

# Annex C-1:	List of Outgoing LSs from RAN1 #119

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2410665 | LS on updated Rel-18 RAN1 UE features lists for NR after RAN1#120 | RAN1, AT&T, NTT DOCOMO, INC. | Rel-18 | NR_MIMO_evo_DL_UL, NR_pos_enh2, Netw_Energy_NR, NR_netcon_repeater, NR_NTN_enh, NR_Mob_enh2, NR_SL_enh2, NR_redcap_enh, NR_MC_enh, NR_XR_enh, NR_FR1_lessthan_5MHz_BW, NR_DSS_enh, NR_BWP_wor, NR_cov_enh2, TEI18 |  | RAN2 | RAN4 |  |  |
| R1-2410708 | Reply LS on CBR Range | RAN1, CATT | Rel-18 | NR_pos_enh2-Core | R2-2409258 | RAN2 |  |  |  |
| R1-2410758 | LS to RAN2 on RRC and MAC CE impacts for Rel-19 NR MIMO Ph5 | RAN1, Samsung | Rel-19 | NR_MIMO_Ph5-Core |  | RAN2 |  |  |  |
| R1-2410836 | LS on Precoder Indication for 8-Port CG-PUSCH | RAN1, Google | Rel-18 | NR_MIMO_evo_DL_UL-Core |  | RAN2 |  |  |  |
| R1-2410870 | Reply to RAN4 LS on UE RF issues for Rel-19 MIMO Enhancement | RAN1, InterDigital, OPPO | Rel-19 | NR_MIMO_Ph5-Core | R4-2417117 | RAN4 |  |  |  |
| R1-2410889 | LS on impact on transmit signal quality/MPR requirement | RAN1, Huawei | Rel-19 | NR_duplex_evo-Core |  | RAN4 |  |  |  |
| R1-2410890 | Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation | RAN1, Huawei | Rel-19 | NR_duplex_evo-Core | R4-2416919 | RAN4 |  |  |  |
| R1-2410895 | Reply LS on OCC for CB-msg3 NPUSCH | RAN1, Sony | Rel-19 | IoT_NTN_Ph3-Core | R2-2409242 | RAN2 |  |  |  |
| R1-2410898 | Reply LS on applicable functionality reporting for beam management UE-sided model | RAN1, Samsung | Rel-19 | NR_AIML_air-Core | R2-2407848 | RAN2 |  |  |  |
| R1-2410909 | LS on LP-WUS operation in CONNECTED mode | RAN1, NTT DOCOMO | Rel-19 | NR_LPWUS-Core |  | RAN2 |  |  |  |
| R1-2410911 | LS on collision between SSB and RA occasion for LTM | RAN1, Google | Rel-18 | NR_Mob_enh2-Core |  | RAN4 |  |  |  |
| R1-2410914 | LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management | RAN1, ZTE | Rel-19 | NR_MIMO_Ph5-Core |  | RAN4 |  |  |  |
| R1-2410916 | LS on condition of applying both indicated TCI states for PDCCH reception | RAN1, Samsung | Rel-18 | NR_MIMO_evo_DL_UL-Core |  | RAN2 |  |  |  |
| R1-2410917 | Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell | RAN1, Ericsson | Rel-19 | Netw_Energy_NR_enh-Core | R4-2416913 | RAN4 | RAN2 |  |  |
| R1-2410922 | LS on signalling feasibility of dataset and parameter sharing | RAN1, Qualcomm | Rel-19 | NR_AIML_air-Core |  | RAN2 | SA2, SA3, SA5 |  |  |

# Annex C-2:	List of Incoming LSs from RAN1 #119

| TDoc # | Title | Source | Release | Related WIs | Reply to | To | Cc | Original LS | Reply in |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R1-2409343 | LS on OCC for CB-msg3 NPUSCH | RAN2, vivo | Rel-19 | IoT_NTN_Ph3-Core |  | RAN1 |  | R2-2409242 |  |
| R1-2409344 | LS on Additional Measurements for SL-TDOA and SL-TOA | RAN2, Qualcomm | Rel-18 | NR_pos_enh2-Core |  | RAN1 | RAN4 | R2-2409256 |  |
| R1-2409345 | LS to RAN1 on CBR range | RAN2, CATT | Rel-18 | NR_pos_enh2-Core |  | RAN1 |  | R2-2409258 |  |
| R1-2409346 | LS on UE capabilities for inter-frequency L1 measurements for LTM | RAN2, Huawei | Rel-18 | NR_Mob_enh2-Core |  | RAN1, RAN4 |  | R2-2409360 |  |
| R1-2409347 | LS on carrierBandwidth configuration for less-than-5MHz carriers | RAN2, Huawei | Rel-18 | NR_FR1_lessthan_5MHz_BW-Core |  | RAN1, RAN4 |  | R2-2409409 |  |
| R1-2409348 | Reply LS on synchronization source change at the transmitting anchor UE in SL positioning | RAN2, Ericsson | Rel-18 | NR_pos_enh2-Core | R4-2410352 | RAN4 | RAN1 | R2-2409267 |  |
| R1-2409349 | Reply LS on Early TA acquisition for the inter-DU scenario | RAN2, Google | Rel-18 | NR_Mob_enh2-Core | R3-244793 | RAN3 | RAN1 | R2-2409376 |  |
| R1-2409350 | LS on SSB relation in On-demand SSB and SSB adaptation on Scell | RAN4, Ericsson | Rel-19 | Netw_Energy_NR_enh-Core |  | RAN1 | RAN2 | R4-2416913 |  |
| R1-2409351 | Reply LS to RAN1 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2 | RAN4, ZTE Corporation | Rel-19 | IoT_NTN_Ph3-Core | R1-2407548 | RAN1 |  | R4-2416918 |  |
| R1-2409352 | LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation | RAN4, Apple | Rel-19 | NR_duplex_evo-Core |  | RAN1 |  | R4-2416919 |  |
| R1-2409353 | LS on UE RF issues for Rel-19 MIMO enhancement | RAN4, Huawei | Rel-19 | NR_MIMO_Ph5-Core |  | RAN1 |  | R4-2417117 |  |
| R1-2409354 | LS on SSB adaptation | RAN4, Apple | Rel-19 | Netw_Energy_NR_enh-Core |  | RAN2 | RAN1 | R4-2416911 |  |
| R1-2409355 | Reply LS on common TA in a regenerative payload scenario | RAN4, CMCC | Rel-19 | NR_NTN_Ph3-Core | R2-2407623 | RAN2 | RAN1 | R4-2416920 |  |
| R1-2409356 | LS on AI/ML Model transfer/delivery to UE | SA5, NEC | Rel-19 | FS_AIML_MGT_Ph2 |  | RAN2, RAN1 | RAN, SA, RAN3 | S5-246292 |  |
| R1-2409357 | Reply LS on AIML Data Collection | SA5, Nokia, ZTE | Rel-19 | FS_AIML_MGT_Ph2 | RP-242389 | RAN2 | RAN, RAN1, RAN3, SA, SA2, SA3 | S5-246299 |  |
| R1-2410923 | LS of RAN4 input on Ambient-IoT to TR 38.769 | RAN4, CMCC | Rel-19 | FS_Ambient_IoT_solutions |  | RAN1 | RAN | R4-2420373 |  |
| R1-2410928 | LS on RAN3 outcome of Ambient IoT study | RAN3, Huawei | Rel-19 | FS_Ambient_IoT_solutions |  | RAN1 | RAN2, SA2 | R3-247923 |  |

# Annex D:	List of Approved updated WIDs

None

# Annex E:	List of draft TSs/TRs agreed at RAN1 #119

| Tdoc Number | Title | Source | Conclusion/Decision |
| --- | --- | --- | --- |
| R1-2410929 | TR 38.769 v2.0.0 “Study on Solutions for Ambient IoT (Internet of Things)” | Huawei (TR editor) | Endorsed v2.0.0 for approval by RAN#106. |

# Annex F:	List of actions

- Outgoing LS

None.

- CR approval

[Post-119-AlignmentCR-Spec##-Rel##] – Spec editors

- Editors, please share drafts before Dec 2. Review to be completed by Dec 3.

- Editors, please replace Spec## and Rel## with the correct specification number and release number.

Rel-17

R1-2410932	Alignment of parameter names	Ericsson

(Rel-17, 38.211, NR_newRAT-Core, TEI17, NR_SL_enh-Core, NR_NTN_solutions-Core, CR0144rev1, Cat F, rev of R1-2409324) is agreed.

R1-2410933	Alignment of parameter names	Ericsson

(Rel-18, 38.211, NR_newRAT-Core, TEI17, NR_SL_enh-Core, NR_NTN_solutions-Core, CR0145rev1, Cat A, rev of R1-2409325) is agreed.

R1-2410935	Rel-17 editorial corrections for TS 38.213	Samsung

(Rel-17, 38.213, NR_newRAT-Core, TEI17, NR_FeMIMO-Core, CR0687rev1, Cat F, rev of R1-2409328) is agreed.

R1-2410936	Rel-17 editorial corrections for TS 38.213 (mirrored to Rel-18)	Samsung

(Rel-18, 38.213, NR_newRAT-Core, TEI17, NR_FeMIMO-Core, CR0689rev1, Cat A, rev of R1-2409330) is agreed.

MCC: To correct CR category in cover page before submission to plenary.

Rel-18

R1-2410934	Alignment of parameter names	Ericsson

(Rel-18, 38.211, NR_MIMO_evo_DL_UL-Core, NR_Mob_enh2-Core, CR0146rev1, Cat F, rev of R1-2409326) is agreed.

R1-2410931	Rel-18 editorial corrections for TS 38.212	Huawei

(Rel-18, 38.212, NR_MC_enh-Core, NR_cov_enh2-Core, NR_MIMO_evo_DL_UL-Core, NR_SL_enh2-Core, NR_Mob_enh2-Core, NR_NTN_enh-Core, Netw_Energy_NR-Core, NR_pos_enh2-Core, CR0209rev1, Cat F, rev of R1-2409327) is agreed.

R1-2410937	Rel-18 editorial corrections for TS 38.213	Samsung

(Rel-18, 38.213, NR_newRAT-Core, TEI18, NR_MIMO_evo_DL_UL-Core, NR_pos_enh2-Core, NR_MC_enh-Core, NR_NTN_enh-Core, NR_Mob_enh2-Core, NR_SL_enh2-Core, Netw_Energy_NR-Core, CR0688rev1, Cat F, rev of R1-2409329) is agreed.

R1-2410938	Rel-18 higher-layers parameter name corrections for TS 38.214	Nokia

(Rel-18, 38.214, NR_Mob_enh2-Core, NR_MIMO_evo_DL_UL-Core, NR_SL_enh2-Core, NR_MBS_enh-Core, Netw_Energy_NR-Core, NR_pos_enh2-Core, CR0646rev1, Cat F, rev of R1-2409334) is agreed.

- TR updates

[Post-119-R19-A-IoT] – Matthew (Huawei)

Email discussion for endorsement of TR 38.769 for submission to RAN#106, from December 2 to 4.

R1-2410929	TR 38.769 v2.0.0 “Study on Solutions for Ambient IoT (Internet of Things)”	Huawei (TR editor)

Decision: As per email decision posted on Dec 4th, (Rel-19, 38.769, FS_Ambient_IoT_solutions) is endorsed as v2.0.0 for approval by RAN plenary.

- Miscellaneous

[Post-119-ISAC-01] – Jerome (AT&T)

Email discussion on simulation assumptions for channel model calibration, from January 8th-17th:

- Companies are encouraged to provide initial channel model calibration assumptions, parameters, and values

- Jerome to collect inputs, provide potential consolidated tables, proposals for consideration by January 17th

Decision: As per email decision posted on Jan 18th, email discussion summary on ISAC CM calibration assumptions is noted in R1-2500059. For further discussions in RAN1#120.

[Post-119-R19-LP_WUS] – Xin (vivo)

Email discussion for collecting LP-SS binary sequences, from Jan 8 to Jan 10

R1-2410939	Collection of LP-SS binary sequences	Moderator (vivo)

Decision: As per email posted on Jan 14th, email discussion summary on LP-SS binary sequences is noted. For further discussions in RAN1#120.

# Annex G:	List of participants at RAN1 #119

Please see excel file attached to this report

# Annex H:	TSG RAN WG1 meetings in 2025 – 2026

| TITLE | TYPE | DATES | LOCATION | CTRY |
| --- | --- | --- | --- | --- |
| 3GPPRAN1#120 | WG | 17 – 21 Feb 2025 | Athens | Greece |
| 3GPPRAN1#120bis | WG | 07 – 11 Apr 2025 | TBD | China |
| 3GPPRAN1#121 | WG | 19 – 23 May 2025 | St Julian’s | Malta |
| 3GPPRAN1#122 | WG | 25 – 29 Aug 2025 | TBD | India |
| 3GPPRAN1#122bis | WG | 13 – 17 Oct 2025 | Prague | CZ |
| 3GPPRAN1#123 | WG | 17 – 21 Nov 2025 | Dallas | US |
| 3GPPRAN1#124 | WG | 09 – 13 Feb 2026 | TBD | EU |
| 3GPPRAN1#124bis | WG | 13 – 17 Apr 2026 | TBD | EU |
| 3GPPRAN1#125 | WG | 18 – 22 May 2026 | TBD | China |
| 3GPPRAN1#126 | WG | 24 – 28 Aug 2026 | TBD | EU |
| 3GPPRAN1#126bis | WG | 12 – 16 Oct 2026 | TBD | Korea |
| 3GPPRAN1#127 | WG | 16 – 20 Nov 2026 | TBD | US |

| MEETING TYPES |  |  |
| --- | --- | --- |
| AH = Ad Hoc | ST = Startup Meeting | RG = Rapporteurs Group |
| JM = Joint | WG = Working Group | SG = Steering Group |
| PM = Preparatory Meeting | CM = Chairmen's meeting | TG = Task Group |
| RM = Resolution Meeting | OR = Ordinary | XO = Extraordinary |

End of document