3GPP TSG RAN WG1 #120			R1-2500262

Athens, Greece, February 17th-21st, 2025

Agenda item:		9.4.4

Source:	China Telecom

Title:	Discussion on other aspects for Ambient IoT

Document for:		Discussion

Introduction

Based on the decision of the RAN plenary #106 meeting [1], the study item of ambient IoT has been finished and a work item will be continued in the next six months. The WI will focus on device 1 with a D1T1 scenario in a simple energy harvesting or proximity determination scheme, and CW will only consider scenario B under a single tone waveform without frequency hopping, the RAN1 scope has been scaled as follows:

- PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

This section will provide our views on several aspects based on the scheduling information, multiple access procedure and timing relationship for ambient IoT.

Scheduling information

During the SI stage, the scheduling information was heatedly discussed. Several scheduling information, such as TDRA, FDRA, and MCS-like information, will be contained in the PRDCH to indicate the R2D reception or D2R transmission [2]. However, whether they will be transmitted by L1 or higher-layer control information is still undetermined and shall be confirmed in the WI stage.

Considering the scheduling information as the higher-layer control information will simplify the PRDCH channel design. Since specific differences between the scheduling information and other higher-layer information are unnecessary, no special PRDCH channel design is needed, and the PRDCH channel design can be regarded as completed. However, such a design will not distinguish the scheduling information and other higher-layer information. Suppose that a transmission scheme is utilized for the PRDCH channel design. In that case, the whole PRDCH transmission will be regarded as wrong transmission regardless of whether the error detection takes place in the scheduling part or the higher-layer data part, and for a PRDCH that only part of the higher-layer data information is error-detected, the device cannot demodulate the other part of data information and complete the corresponding device behaviours, which results in low communication efficiency. Therefore, decoupling the scheduling information and the higher layer data information is an efficient design to avoid a part of the error message affecting the whole demodulation process, it is better to consider the scheduling information as L1 control information to guarantee reliability and avoid an overall demodulation error.

Proposal 1: Support the scheduling information as L1 control information.

A general design to distinguish the L1 control information and higher-layer information is to create two fields for them: a control information field and a higher-layer data field. Still, such a design is too rough for L1 control information. If all control information mixes in the control information field, higher power consumption, and implementation complexity may be required for device 1 to judge the length and content of each L1 control information and parse their meaning. Meanwhile, moving the parsing work up to the higher layer of device 1 will add the processing delay, causing a higher timing drift for the D2R transmission. Therefore, a fine-grained design is needed for the L1 control information field.

In the RFID system, each control information has a corresponding field to indicate its functionality for the tag, and the pre-defined control information field helps the tag to understand the indication in the downlink transmission. Take the ‘Query’ command as an example, 22bits are divided into nine fields, the tag identifies the corresponding field that can understand the reader’s indication easily, and the higher layer only needs to parse them instead of judging and identifying whole control information to improve the communication efficiency, hence the RFID command design can be as a reference.

Figure 1. An example of the RFID ‘Query’ command

Therefore, a potential device PRDCH channel design can be regarded as two parts containing the L1 control information part and a higher-layer data part. In the L1 control information part, several fields are defined for the corresponding L1 control information, such as time domain resource, frequency domain resource, and MCS-like information. A separate part, which can also be regarded as a separate field, is reserved for the higher-layer data information.

Figure 2 illustration of a potential PRDCH channel

Proposal 2: A potential PRDCH channel can be designed as follows:

- The PRDCH channel consists of 2 parts including L1 control information and higher-layer data.

- The L1 control information includes multiple control information fields with a specific size, such as TBS-related, time domain resource, frequency domain resource, ID-related, and other necessary fields.

- The higher-layer data information can be regarded as the data field of the PRDCH channel.

Multiple access

As agreed in the SI stage, multiple access will be performed in the R2D and D2R transmission. In the R2D transmission, since the device cannot distinguish the OOK envelope from different frequency positions, only TDMA is supported in the PRDCH channel. For D2R transmission, considering the network side can demodulate the OOK or BPSK signal from different frequency positions, both FDMA and TDMA are supported in the PDRCH channel. Since multiple access methods have been confirmed, the detailed access process should be designed. Based on the RAN2 agreement, “3-step” CBRA processes are defined, and each communication message is also named MSG1, MSG2, and MSG3. However, the transmission or the monitor resource is not determined, so the entire access process is incomplete. The resource allocation and other necessary processes will be specified during the WI stage.

MSG0 PRDCH

As described in the WID scope, device 1 must acquire the time domain and/or frequency domain resource to access the network by TDMA or FDMA, thus a PRDCH before MSG1 information is necessary to trigger the access process and carry the allocated resource information. However, considering energy harvesting direction 1 will be utilized, the device may not receive the corresponding resource allocation PRDCH due to the unknown device stage for the reader. Hence, a multiple PRDCH transmission scheme may be needed to guarantee that the device will obtain the resource allocation information of these PRDCH transmissions. The max attempt for the multiple PRDCH transmission may be discussed, and the deactivation conditions may also be specified, such as deactivating the multiple PRDCH transmission when receiving the corresponding MSG1 transmission of the device or reaching the max attempt amount.

Figure 3 illustration of a blind PRDCH transmission in the random access

Observation 1: A multiple PRDCH transmission scheme will be performed before the random access procedure of D2R transmission under energy harvesting direction 1.

MSG1 transmission resource

Based on TR 38.769, an initial PRDCH may carry at least a time domain resource at least for MSG1 transmission, and the size of that time domain resource shall be defined. There may be a question about whether a time domain resource is only used for one device or can be utilized for multiple devices and multiplexed in the time domain, which will affect the available device amount in one time domain resource and needs to be clarified.

Figure 4 illustration of different time domain resource understanding

If a time domain resource can only be used for one device, like the left figure in Fig 4, determining only one or two time domain resources may not be enough, and collision probability will rise if multiple devices select the same time domain resource in the TDM method. Even if the FDM method can be considered for devices to select a frequency domain resource and avoid the collision, since the repetition line coding number is limited, the total access resource is still insufficient. Moreover, considering the inaccurate device clock, large time domain resources may require a longer time counting value, causing large timing drift for the last several time domain resources, hence the amount of time domain resources shall be configured neither too small nor too large. However, if a time domain resource can access more devices, like the right figure in Fig 4, the collision problem will be relieved. In that case, fewer time domain resources are enough for devices accessing the network. Moreover, if FDM can also be utilized for devices, configuring one time domain resource may be enough. Therefore, the appropriate time domain resource configuration depends on the number of devices a time domain resource can access.

Proposal 3: The appropriate time domain resource configuration at least for MSG1 depends on the number of devices that can be accessed by one time domain resource.

- If a time domain resource can only be used for one device, the amount of time domain resources shall be neither too small nor too large, e.g., 5, 10.

- If a time domain resource can access more devices, less time domain resource configuration is expected, e.g., 1, 2.

MSG3 transmission resource

In addition, whether the initial PRDCH should carry some resource configuration information for MSG3 shall be discussed. Based on the RAN2 agreement, the MSG0 PRDCH will trigger the access process, and it naturally contains the configuration information for MSG1 and MSG3. However, the MSG3 time domain resource indicated by the MSG0 information may be unreliable. In one aspect, the time interval between the MSG0 reception occasion and the MSG3 transmission occasion is too long, which is a challenge for poor-timing counting performance devices to identify the accurate time domain resources. Therefore, the reader has to perform a lot of time duration redundancy design to prevent the device from not receiving MSG2 messages or not being able to send MSG3 messages due to the miss of time domain resource at the device side, which is an inefficient design for the reader's time domain resources configuration for sending MSG2 and receiving MSG3. Besides, since the reader doesn’t know how many devices can access the network and respond to MSG2 information, the configuration from MSG0 will also have some redundancy resource configuration for MSG3 transmission and reception, which may also be inefficient. Since RAN2 considers that the main information in MSG2 responding to the devices is ACK and the successful ID information and further information is not precluded, the resource configuration of MSG3 configured by MSG2 can be considered for improved communication efficiency when performing the inventory process.

Figure 5 illustration of MSG3 resource configured by initial PRDCH or MSG2 PRDCH

Proposal 4: Support configuring the MSG3 resource by the MSG2 PRDCH transmission if needed.

MSG2 monitoring

Besides, the design of MSG2 is also discussed in the SI stage, the MSG2 can respond to MSG1 information from one device, or multiple MSG1 information from different devices, hence the starting point, reference point, and monitoring duration may have specific designs. As described in TR 38.769, the starting time for MSG2 monitoring can be separate for different MSG1 resources, or common for the same MSG1 resource. The same starting time will have a simple indication design since the starting occasion is the same for those devices. No separate time interval is needed for different time domain resources. However, a potential disadvantage is that the time interval between the first time domain resource and the monitoring occasion will be large, which may affect the accuracy of the indicated monitoring occasion. If the number of configured time domain resources is small, or the interval between the first time domain resource and the MSG2 monitoring starting occasion is not long, the common starting occasion for MSG2 monitoring shall be supported, otherwise the separate starting occasion is more appropriate. Since the reference time occasion will depend on the starting time occasion, the reference occasion scheme should be further confirmed after determining the starting time occasion scheme.

Proposal 5: Determining the starting occasion of MSG2 monitoring shall depend on the total number of time domain resources or the interval between the first time domain resource and the starting occasion.

- A common starting occasion is supported if a short interval or small resource amount is configured.

- Otherwise, a separate starting occasion will be more appropriate.

Timing relationship

As discussed in the SI stage, the time intervals between different R2D and D2R transmissions have been discussed, and four minimum time intervals and two maximum time intervals have been defined, which can be illustrated as follows:

Figure 6 illustration of the time domain interval defined in the SI stage

The detailed time interval definition shall be specified, including the duration size and the basic time interval in the WI stage.

Time unit definition

In the R2D transmission, the information in the PRDCH channel is mapped based on OFDM symbols. However, the devices cannot understand the duration of ‘an OFDM symbol’ since only the envelope can be identified by it instead of the OFDM waveform, thus reusing OFDM symbols as the basic time unit and indicating it to devices is not realistic. Considering the identified envelope is the OOK chip within the OFDM symbols, the time unit granularity of R2D transmission can be regarded as an amount of ‘OOK chip’ for the time interval duration, and the time unit length is defined as the OOK chip length.

For the D2R transmission, the chip length can be regarded as the time unit as well based on the same reason. However, considering the modulation method in the D2R transmission will not be limited by OOK, the time unit granularity of D2R transmission can be named ‘chip’ for the indication of time interval duration, and the time unit length is also regarded as chip length.

Proposal 6: The time unit granularity of R2D transmission or D2R transmission can be defined as ‘chip’, and the time unit length can be the chip length.

Time interval definition

Since we confirm the above-mentioned time intervals, we should define and describe their specific durations. However, suppose we adopt the time unit based on the above definition to characterize the duration of the interval metric. In that case, we will find that the definition of this metric is not uniform because the length of the chip is different in different transmissions, so it isn't easy to define the metric using several chips. From this point of view, using a unified definition of the time duration is more appropriate so that the size of each matric can be expressed uniformly, for example, xxμs. Then, the reader will explicitly transmit the chip size information and the chip amount information to the device to indicate the duration of the metric interval or only provide the chip size information and let the device convert the metric's duration into chip amounts.

Proposal 7: Using microseconds to define the duration of each interval.

Proposal 8: Support explicitly or implicitly indicating the interval duration to the devices.

Conclusions

In this contribution, we have the following proposals:

Observation 1: A multiple PRDCH transmission scheme will be performed before the random access procedure of D2R transmission under energy harvesting direction 1.

Proposal 1: Support the scheduling information as L1 control information.

Proposal 2: A potential PRDCH channel can be designed as follows:

- The PRDCH channel consists of 2 parts including L1 control information and higher-layer data.

- The L1 control information includes multiple control information fields with a specific size, such as TBS-related, time domain resource, frequency domain resource, ID-related, and other necessary fields.

- The higher-layer data information can be regarded as the data field of the PRDCH channel.

Proposal 3: The appropriate time domain resource configuration at least for MSG1 depends on the number of devices that can be accessed by one time domain resource.

- If a time domain resource can only be used for one device, the amount of time domain resources shall be neither too small nor too large, e.g., 5, 10.

- If a time domain resource can access more devices, less time domain resource configuration is expected, e.g., 1, 2.

Proposal 4: Support configuring the MSG3 resource by the MSG2 PRDCH transmission if needed.

Proposal 5: Determining the starting occasion of MSG2 monitoring shall depend on the total number of time domain resources or the interval between the first time domain resource and the starting occasion.

- A common starting occasion is supported if a short interval or small resource amount is configured.

- Otherwise, a separate starting occasion will be more appropriate.

Proposal 6: The time unit granularity of R2D transmission or D2R transmission can be defined as ‘chip’, and the time unit length can be the chip length.

Proposal 7: Using microseconds to define the duration of each interval.

Proposal 8: Support explicitly or implicitly indicating the interval duration to the devices.

- References

- RP-243326, 3GPP RAN#106 meeting, “New WID: Study on solutions for Ambient IoT (Internet of Things) in NR”, Madrid, Spain, December, 2024.

- 3GPP TR 38.769 V2.0.1, Study on solutions for ambient IoT (Internet of Things) (R19).