3GPP TSG RAN WG1 #120			R1-2500271

Athens, Greece, February 17th – 21st, 2025

Agenda item:		9.12.1

Source:	China Telecom

Title:	Discussion on multi-carrier enhancements for NR phase 3

Document for:		Discussion

Introduction

The new WID on Rel-19 multi-carrier enhancements aims enhancement for multi-cell PUSCH/PDSCH scheduling with a single DCI. There are two objectives for the enhancement, including the support of different SCS/carrier type among co-scheduled cells by the single DCI, and one or multiple PUSCHs/PDSCHs per scheduled cell by the single DCI [1].

The objectives were discussed in previous RAN1 #118bis and #119 meetings. This contribution further provides our consideration on specifying the Rel-19 enhancement of multi-cell PUSCH/PDSCH scheduling with a single DCI.

On support of different SCS/carrier type among co-scheduled cells by single DCI

Relaxing the same SCS/carrier type restriction among co-scheduled cells by a single DCI in Rel-19, one aspect discussed in last meeting is HARQ-ACK feedback timing determination considering different SCS among co-scheduled cells.

For determining the timing of a PUCCH carrying HARQ-ACK information, it is specified in the current TS 38.213:

The following apply to the PCell if the UE is provided pucch-sSCellPattern; otherwise, the following apply to the serving cell of the PUCCH transmission. If the UE is provided subslotLengthForPUCCH, n is the last UL slot for PUCCH transmission that overlaps with a PDSCH reception or with a PDCCH reception providing a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception; otherwise, n is the last UL slot for PUCCH transmission that overlaps with the DL slot nD for the PDSCH reception or with the DL slot nD for the PDCCH reception in case of a DCI format that triggers a HARQ-ACK information report and does not schedule a PDSCH reception. … If the UE detects a DCI format scheduling a number of PDSCH receptions ending in DL slot nD or if the UE detects a DCI format generating a HARQ-ACK information bit and does not schedule a PDSCH reception through a PDCCH reception ending in DL slot nD, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot n+k, where k is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, dl-DataToUL-ACK-r16, or dl-DataToUL-ACK-DCI-1-2, or dl-DataToUL-ACK-r17, or dl-DataToUL-ACK-DCI-1-2-r17, or dl-DataToUL-ACK-v1700.

For Rel-18 multi-cell scheduling with same SCS among co-scheduled cells, it was agreed the reference PDSCH for HARQ-ACK timing determination is the PDSCH ending last as indicated in the DCI format 1_3 among the set of co-scheduled PDSCHs.

Agreement When UE detects a DCI format 1_X scheduling a set of PDSCHs, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot , where is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format and is the last UL slot overlapping with the DL slot for the reference PDSCH reception for slot-based PUCCH or an UL slot overlapping with the end of the reference PDSCH reception in DL slot for sub-slot based PUCCH. - FFS details of reference PDSCH Agreement For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH is the PDSCH ending last as indicated in the DCI format 1_X among the set of co-scheduled PDSCHs.

Following Rel-18 operation when the co-scheduled cells have different SCS in Rel-19, for sub-slot based PUCCH, n is the last UL slot that overlaps with the end of the PDSCH ending last as indicated in the DCI. Even if more than one PDSCHs with different SCS end last among the set of co-scheduled PDSCHs, the same last UL slot n is determined and current specification is workable.

For slot-based PUCCH, the understanding of how to determine the last UL slot n is divergent among companies based on the discussion in the last meeting [2]. One understanding is the DL slot nD is the slot containing the PDSCH ending last from the actual PDSCH ending time/ending symbol perspective. Then specification needs to consider the following cases as shown in the figure. The left figure shows the case that DL slot nD for the PDSCH ending last is not the DL slot ending last for PDSCHs in all the co-scheduled cells. As shown in figure 1(a), cell1 with 30KHz and cell2 with 15KHz are co-scheduled cells with PDSCH allocation. PDSCH1 is the PDSCH ending last and slot n of cell1 is DL slot nD which ends earlier than slot m of cell2 with PDSCH2. UL slot overlapping with slot n is regarded as the base slot for k to plus on to determine the HARQ-ACK feedback slot. It may lead insufficient processing time for PDSCH2. Another case is ambiguous on DL slot nD when multiple PDSCHs have the same last ending in different slots with different SCS. As shown in figure 1(b), PDSCH1 on cell1 with 30KHz and PDSCH2 on cell2 with 15KHz are co-scheduled with the same ending time, whether slot n on cell1 or slot m on cell2 is taken as DL slot nD needs to be clarified.

(a) DL slot nD for the PDSCH ending last is not the slot ending last    (b) Ambiguous on DL slot nD

Figure 1 DL slot nD to determine the last UL slot n for slot-based PUCCH

Another understanding is the DL slot nD is the DL slot ending last among the slots containing set of co-scheduled PDSCHs. Thus in both figure 1(a) and (b), slot m in cell2 has the last ending considering PDSCH1 and PDSCH2, which is the DL slot nD. This way to determine the DL slot nD results in the most the relaxed timing for PUCCH and is preferred as in the newest FL proposal [2].

Proposal 3-1 rev3: - For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3, follow Rel-18 operation. - If more than one PDSCH ends last with different SCS among the set of co-scheduled PDSCHs, - If the UE is not provided subslotLengthForPUCCH, the DL slot nD is the DL slot ending last, amongst the DL slots where the more than one PDSCH are scheduled by the DCI format 1_3. - FFS: RAN1 spec impact - If the UE is provided subslotLengthForPUCCH, no spec change is necessary. - Note: Specification of this feature shall not impact the existing UE processing PDSCH timeline requirement for any individual PDSCH, as specified in 5.3.1 of TS38.214.

Regarding the FL proposal, we think the sub-bullet “If more than one PDSCH ends last with different SCS among the set of co-scheduled PDSCHs,” can be removed to also cover the case as shown in figure 1(a). For the RAN1 specification impact, some clarification would be beneficial to avoid the misunderstanding.

Proposal 1: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3, follow Rel-18 operation.

- If the UE is not provided subslotLengthForPUCCH, the DL slot nD is the DL slot ending last, amongst the DL slots where the more than one PDSCH are scheduled by the DCI format 1_3.

  - FFS: RAN1 spec impact

- If the UE is provided subslotLengthForPUCCH, no spec change is necessary.

- Note: Specification of this feature shall not impact the existing UE processing PDSCH timeline requirement for any individual PDSCH, as specified in 5.3.1 of TS38.214.

On support of multiple PUSCHs/PDSCHs per scheduled cell by single DCI

Enhancement on DCI field

To support scheduling of multiple PUSCHs/PDSCHs per scheduled cell by DCI format 0_3/1_3, modification of DCI field is needed referring the field design of DCI format 0_1/1_1 to support the scheduling of multiple PUSCHs/PDSCHs. Base on the discussion in previous meetings, options for bit number of the NDI and RV fields need to be further studied for DCI format 0_3/1_3 [2].

Agreement - For NDI indication in DCI format 0_3/1_3 for TB1, further study - Option 1: For each block of NDI field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3. - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell. - Option 2a: Total number of bits of the NDI field is equal to the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells. - The number of bits of a block of NDI field corresponding to a scheduled cell is equal to the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell. - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells. - Other options are not precluded. - Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of NDI field for TB1, same as in Rel-18. Agreement - For RV indication in DCI format 0_3/1_3 for TB1, further study - Option 1: For each block of RV field, the number of bits is equal to the maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell by the DCI format 0_3/1_3 and number of bits for RV configured for the cell. - Maximum number of schedulable PUSCHs/PDSCHs on the corresponding cell is determined by TDRA table for the cell. - Option 2a: Total number of bits of the RV field is determined based on the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells. - The number of bits of a block of RV field corresponding to a scheduled cell is determined based on the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell and number of bits for RV configured for the cell. - Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells. - Other options are not precluded. - Note: For Option 1 and 2a, DCI format 0_3/1_3 has NcellUL or NcellDL blocks of RV field for TB1, same as in Rel-18.

The same design principle should be considered for the bit construction of NDI field and RV field. For option 1, the number of bits for each block of the field is depended on the RRC configuration, i.e. the configured TDRA table, parameter numberOfBitsForRV-DCI-0-3/1-3 configured for the cell. For FDRA based scheduled cell indication or scheduledCellComboListDCI-0-3/1-3 configured with only one entry, the number of NcellUL or NcellDL blocks is also depended on RRC configuration, thus the NDI or RV for each scheduled PUSCH/PDSCH of each scheduled cell has RRC determined bit position in the field which is easier for UE interpretation. However, for the case scheduled cell is indicated by scheduled cells indicator, the bit position of a block corresponding to a scheduled cell is dynamic based on the interpretation of scheduled cells indicator, as well as the bit position of NDI or RV for each scheduled PUSCH/PDSCH, and the UE interpretation complexity is similar to option 2a. For option 2a, the bit position of a block is also dynamic. UE needs to firstly interpret TDRA field to know the actual number of the scheduled PUSCHs/PDSCHs on each scheduled cell and then gets the bit number/position of each block. On the other hand, comparing the two options from DCI size perspective, if the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells is configured less than the sum of maximum number of schedulable PUSCHs/PDSCHs of each schedulable cell, option 2a would have a smaller NDI and RV field size. It is possible for both the FDRA based scheduled cell indication and scheduled cells indicator based indication for an example shown in Table 1, where all the two schedulable cells are scheduled based on FDRA or scheduled cells indicator. When option 1 of TB1 NDI indication is applied for the example, there are 2 blocks in the TB1 NDI field with 8 bits each, the total TB1 NDI field size is 16bits. When option 2a is applied for the example, total number of bits of the TB1 NDI field is equal to 10 bits.

Table 1 example of TDRA table configuration for 2 scheduled cells

|  | Number of PDSCH scheduled for Cell 1 | Number of PDSCH scheduled for Cell 2 |
| --- | --- | --- |
| Row index 1 | 8 | 2 |
| Row index 2 | 2 | 8 |

Based on the discussion, option 2a is the proposal.

Proposal 2: For NDI indication in DCI format 0_3/1_3 for TB1, total number of bits of the NDI field is equal to the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- The number of bits of a block of NDI field corresponding to a scheduled cell is equal to the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell.

- Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

Proposal 3: For RV indication in DCI format 0_3/1_3 for TB1, total number of bits of the RV field is determined based on the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- The number of bits of a block of RV field corresponding to a scheduled cell is determined based on the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell and number of bits for RV configured for the cell.

- Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

Then for NDI indication in DCI format 1_3 for TB2, a second NDI field is applied to cells configured with maxNrofCodeWordsScheduledByDCI = 2. The total number of bits of the second NDI field is equal to the maximum number of schedulable PDSCHs by the DCI format 1_3 across all the co-schedulable cells configured with maxNrofCodeWordsScheduledByDCI = 2. There are NcellDL,3 blocks of NDI field for TB2 same as in Rel-18. The number of bits for a block of the second NDI field corresponding to a scheduled cell with maxNrofCodeWordsScheduledByDCI = 2 is equal to the actual number of the scheduled PDSCHs by the DCI format 1_3 on the cell. Similar mechanism can be used for RV indication in DCI format 1_3 for TB2.

Proposal 4: For NDI indication in DCI format 1_3 for TB2, total number of bits of the second NDI field is equal to the maximum number of schedulable PDSCHs by the DCI format 1_3 across all the co-schedulable cells configured with maxNrofCodeWordsScheduledByDCI = 2.

- The number of bits of a block of the second NDI field corresponding to a scheduled cell configured with maxNrofCodeWordsScheduledByDCI = 2 is equal to the actual number of the scheduled PDSCHs by the DCI format 1_3 on the cell.

- Some reserved bits are needed when the actual number of the scheduled PDSCHs by the DCI format 1_3 on scheduled cells configured with maxNrofCodeWordsScheduledByDCI = 2 is smaller than the maximum number of schedulable PDSCHs by the DCI format 1_3 across all the co-schedulable cells configured with maxNrofCodeWordsScheduledByDCI = 2.

- DCI format 1_3 has NcellDL,3 blocks of the second NDI field for TB2, same as in Rel-18.

Proposal 5: For RV indication in DCI format 1_3 for TB2, total number of bits of the second RV field is determined based on the maximum number of schedulable PDSCHs by the DCI format 1_3 across all the co-schedulable cells configured with maxNrofCodeWordsScheduledByDCI = 2.

- The number of bits of a block of the second RV field corresponding to a scheduled cell configured with maxNrofCodeWordsScheduledByDCI = 2 is determined based on the actual number of the scheduled PDSCHs by the DCI format 1_3 on the cell and number of bits for RV configured for the cell.

- Some reserved bits are needed when the actual number of the scheduled PDSCHs by the DCI format 1_3 on scheduled cells configured with maxNrofCodeWordsScheduledByDCI = 2 is smaller than the maximum number of schedulable PDSCHs by the DCI format 1_3 across all the co-schedulable cells configured with maxNrofCodeWordsScheduledByDCI = 2.

- DCI format 1_3 has NcellDL,3 blocks of the second RV field for TB2, same as in Rel-18.

Enhancement on Type-2 HARQ-ACK codebook

For HARQ-ACK feedback of the co-scheduled PDSCHs by DCI format 1_3, two sub-codebooks are concatenated for the construction of Type-2 HARQ-ACK codebook in Rel-18. For Rel-19 multi-cell scheduling with one or multiple PUSCHs/PDSCHs per scheduled cell, the WID requires no increasing of the maximum number of sub-codebooks for Type-2 HARQ-ACK codebook. Agreements made for the design of Type-2 HARQ-ACK codebook for Rel-19 in the last meeting include [2]:

Agreement - Type-2 HARQ-ACK codebook is generated by concatenating a first sub-codebook and a second sub-codebook. - The first sub-codebook comprises HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single PDSCH, or each scheduling a single cell with multiple PDSCHs on it and nrofHARQ-BundlingGroups configured as 1, and HARQ-ACK information bit(s) for DCI(s) having associated HARQ-ACK information without scheduling PDSCH reception. - The second sub-codebook comprises HARQ-ACK information bits for PDSCHs scheduled by DCI(s) with each scheduling more than one cell, or each scheduling a single cell with multiple PDSCHs on it without nrofHARQ-BundlingGroups or nrofHARQ-BundlingGroups configured larger than 1. - Separate DAI counting is applied for DCI(s) associated with the first sub-codebook and DCI(s) associated with the second sub-codebook as Rel-18. - Note: For providing HARQ-ACK information corresponding to SCell dormancy indication, the UE assumes that the UE receives a PDSCH on the serving cell associated with fields in DCI format 1_3 used for SCell dormancy indication as Rel-18. Agreement - For the second sub-codebook, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M, to select one of the following options. - Option 1: M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. M is implicitly derived based on RRC configuration. - Option 2: M is explicitly configured by RRC parameter for the PUCCH group. - Other options are not precluded.

One of the issues to be decided is the option to determine M value. Following Rel-17 multi-PDSCH scheduling and Rel-18 multi-cell scheduling principle, the number of HARQ-ACK bits generated each DCI format 1_3 for the second sub-codebook is defined as the maximum number of required HARQ-ACK bits for all co-scheduled cell combinations in the PUCCH group. The details regarding how to derive M implicitly based on RRC configuration need also decision.

Proposal 6: For the second sub-codebook of Rel-19 type-2 HARQ-ACK, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M, M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. For serving cell c in the co-scheduled cell combinations,

- If nrofHARQ-BundlingGroups is configured with value NHARQ-ACKTBG,max>1 and harq-ACK-SpatialBundlingPUCCH is not configured, the UE generates NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK bits;

- If nrofHARQ-BundlingGroups is configured with value NHARQ-ACKTBG,max>1 and harq-ACK-SpatialBundlingPUCCH is configured, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK bits;

- If multi-PDSCH scheduling is configured and neither nrofHARQ-BundlingGroups nor harq-ACK-SpatialBundlingPUCCH is configured, the UE generates NTB,cDL∙NPDSCH,cmax HARQ-ACK bits;

- If multi-PDSCH scheduling and harq-ACK-SpatialBundlingPUCCH are configured, and nrofHARQ-BundlingGroups is not configured, the UE generates NPDSCH,cmax HARQ-ACK bits;

- Where NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c and NPDSCH,cmax is determined by the maximum number of SLIVs corresponding to the cell amongst all rows of the joint TDRA table.

Another proposal discussed in the last meeting is about the HARQ-ACK information bit ordering for a DCI format 1_3 associated with the second sub-codebook. According to the discussions, there are two options for the bit ordering:

- Option 1: For the second sub-codebook, the HARQ-ACK information bits for a DCI format 1_3 are ordered firstly according to ascending order of PDSCH reception starting time on a same serving cell, secondly according to ascending order of codeword index of PDSCH receptions on a same serving cell, when applicable, then according to ascending order of associated serving cell indexes.

- Option 2: For the second sub-codebook, the HARQ-ACK information bits for a DCI format 1_3 are ordered firstly according to ascending order of codeword index of a PDSCH reception on a same serving cell, when applicable, secondly according to ascending order of PDSCH reception starting time on a same serving cell, then according to ascending order of associated serving cell indexes.

Based on the pseudo-code in TS38.213, it is observed HARQ-ACK information bits for co-scheduled PDSCHs on one cell in Rel-17 is ordered firstly on the codeword index, and then in order of the PDSCH reception starting time.

if harq-ACK-SpatialBundlingPUCCH is not provided and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for at least one configured DL BWP of at least one serving cell, o2⋅TD⋅j+2VC-DAI,c,mDL-1ACK = HARQ-ACK information bit corresponding to the first transport block of this cell o2⋅TD⋅j+2VC-DAI,c,mDL-1+1ACK = HARQ-ACK information bit corresponding to the second transport block of this cell Vs=Vs∪2⋅TD⋅j+2VC-DAI,c,mDL-1, 2⋅TD⋅j+2VC-DAI,c,mDL-1+1

Combining the HARQ-ACK information bits ordering in Rel-17 multi-PDSCHs scheduling and Rel-18 multi-cell scheduling, option 2 is proposed.

Proposal 7: For Rel-19 multi-cell multi-PDSCH scheduling by a DCI format 1_3, the HARQ-ACK information bits for the second sub-codebook are ordered firstly according to ascending order of codeword index of a PDSCH reception on a same serving cell, when applicable, secondly according to ascending order of PDSCH reception starting time on a same serving cell, then according to ascending order of associated serving cell indexes.

Conclusion

Based on the discussion in this contribution, we have following proposals:

Proposal 1: For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_3, follow Rel-18 operation.

- If the UE is not provided subslotLengthForPUCCH, the DL slot nD is the DL slot ending last, amongst the DL slots where the more than one PDSCH are scheduled by the DCI format 1_3.

  - FFS: RAN1 spec impact

- If the UE is provided subslotLengthForPUCCH, no spec change is necessary.

- Note: Specification of this feature shall not impact the existing UE processing PDSCH timeline requirement for any individual PDSCH, as specified in 5.3.1 of TS38.214.

Proposal 2: For NDI indication in DCI format 0_3/1_3 for TB1, total number of bits of the NDI field is equal to the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- The number of bits of a block of NDI field corresponding to a scheduled cell is equal to the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell.

- Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

Proposal 3: For RV indication in DCI format 0_3/1_3 for TB1, total number of bits of the RV field is determined based on the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

- The number of bits of a block of RV field corresponding to a scheduled cell is determined based on the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 on the cell and number of bits for RV configured for the cell.

- Some reserved bits are needed when the actual number of the scheduled PUSCHs/PDSCHs by the DCI format 0_3/1_3 is smaller than the maximum number of schedulable PUSCHs/PDSCHs by the DCI format 0_3/1_3 across all the co-schedulable cells.

Proposal 4: For NDI indication in DCI format 1_3 for TB2, total number of bits of the second NDI field is equal to the maximum number of schedulable PDSCHs by the DCI format 1_3 across all the co-schedulable cells configured with maxNrofCodeWordsScheduledByDCI = 2.

- The number of bits of a block of the second NDI field corresponding to a scheduled cell configured with maxNrofCodeWordsScheduledByDCI = 2 is equal to the actual number of the scheduled PDSCHs by the DCI format 1_3 on the cell.

- Some reserved bits are needed when the actual number of the scheduled PDSCHs by the DCI format 1_3 on scheduled cells configured with maxNrofCodeWordsScheduledByDCI = 2 is smaller than the maximum number of schedulable PDSCHs by the DCI format 1_3 across all the co-schedulable cells configured with maxNrofCodeWordsScheduledByDCI = 2.

- DCI format 1_3 has NcellDL,3 blocks of the second NDI field for TB2, same as in Rel-18.

Proposal 5: For RV indication in DCI format 1_3 for TB2, total number of bits of the second RV field is determined based on the maximum number of schedulable PDSCHs by the DCI format 1_3 across all the co-schedulable cells configured with maxNrofCodeWordsScheduledByDCI = 2.

- The number of bits of a block of the second RV field corresponding to a scheduled cell configured with maxNrofCodeWordsScheduledByDCI = 2 is determined based on the actual number of the scheduled PDSCHs by the DCI format 1_3 on the cell and number of bits for RV configured for the cell.

- Some reserved bits are needed when the actual number of the scheduled PDSCHs by the DCI format 1_3 on scheduled cells configured with maxNrofCodeWordsScheduledByDCI = 2 is smaller than the maximum number of schedulable PDSCHs by the DCI format 1_3 across all the co-schedulable cells configured with maxNrofCodeWordsScheduledByDCI = 2.

- DCI format 1_3 has NcellDL,3 blocks of the second RV field for TB2, same as in Rel-18.

Proposal 6: For the second sub-codebook of Rel-19 type-2 HARQ-ACK, the number of HARQ-ACK information bits for each DCI format 1_3 is equal to M, M is the maximum number of HARQ-ACK information bits which can be generated for a DCI format 1_3 across all the configured cell set(s) in the PUCCH group for the UE. For serving cell c in the co-scheduled cell combinations,

- If nrofHARQ-BundlingGroups is configured with value NHARQ-ACKTBG,max>1 and harq-ACK-SpatialBundlingPUCCH is not configured, the UE generates NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK bits;

- If nrofHARQ-BundlingGroups is configured with value NHARQ-ACKTBG,max>1 and harq-ACK-SpatialBundlingPUCCH is configured, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK bits;

- If multi-PDSCH scheduling is configured and neither nrofHARQ-BundlingGroups nor harq-ACK-SpatialBundlingPUCCH is configured, the UE generates NTB,cDL∙NPDSCH,cmax HARQ-ACK bits;

- If multi-PDSCH scheduling and harq-ACK-SpatialBundlingPUCCH are configured, and nrofHARQ-BundlingGroups is not configured, the UE generates NPDSCH,cmax HARQ-ACK bits;

- Where NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c and NPDSCH,cmax is determined by the maximum number of SLIVs corresponding to the cell amongst all rows of the joint TDRA table.

Proposal 7: For Rel-19 multi-cell multi-PDSCH scheduling by a DCI format 1_3, the HARQ-ACK information bits for the second sub-codebook are ordered firstly according to ascending order of codeword index of a PDSCH reception on a same serving cell, when applicable, secondly according to ascending order of PDSCH reception starting time on a same serving cell, then according to ascending order of associated serving cell indexes.

References

- RP-242904, Revised WID: Multi-carrier enhancements for NR Phase 3, Lenovo (Rapporteur), 3GPP TSG RAN #106, Madrid, Spain, December 9-12, 2024

- R1-2410905, Feature lead summary #5 on multi-cell scheduling with a single DCI, Moderator (Lenovo), 3GPP TSG RAN WG1 #119, Orlando, US, November 18 – 22, 2024