3GPP TSG RAN WG1 #120               						R1- 2500484

Athens, Greece, Feb 17th – Feb 21st, 2025

Agenda item:	      9.2.2

Source:	      Tejas Networks Ltd

Title:		      CSI enhancement for NR MIMO Phase 5

Document for:     Discussion and Decision

Introduction

The Rel-19 WID on NR MIMO Phase 5 [1][2] specifies the following objectives on CSI enhancements.

- Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design

In this contribution, we provide our views on the key issues that are prioritized for RAN1 #120 meeting. Based on the agreement in RAN1 #119 meeting [3] and earlier meetings, a few items related to Rel 19 CSI enhancements for up to 128 CSI-RS ports, CRI based CSI reporting, and UE reporting enhancement for CJT deployments, are suggested for discussion and decision. This contribution addresses a few of these study aspects. It also considers certain other issues that are relevant for further discussions.

CSI Refinements for up to 128 ports

## SRS Port Grouping

Based on RAN #105 discussion and the revised WID RP-242394, the proposals on SRS port grouping from RAN1 #117 was re-considered for discussion. Accordingly, the following agreement and conclusion were made in RAN1 #118bis.

Agreement For a UE configured with a total of PSRS=6 or 8 ports across ≥1 SRS resources for antenna switching intended for xT6R or xT8R, respectively, support the following fixed SRS port grouping where (with the PSRS ports indexed in an ascending order according to SRS resource ID and port number within each SRS resource): - SRS port group 0, corresponding to CW0, comprises the even PSRS/2 out of PSRS ports; and - SRS port group 1, corresponding to CW1, comprises the odd PSRS/2 out of PSRS ports The above feature is applicable only for reportQuantity = ‘cri-RI-CQI’ No other spec enhancement is introduced on new CW-to-layer mapping, DL resource allocation, CSI feedback, and DCI format Note: The above grouping assumption is to align NW and UE on the association between SRS ports and reported CQIs for the two CWs when reportQuantity = ‘cri-RI-CQI’. Note: different SRS ports are associated with different UE antenna ports. Note: if one single CW is scheduled, both SRS port groups can correspond to the same CW, i.e. no enhancement is needed for the single-CW case Note: This feature is a separate UE capability and, for UEs supporting this capability, configured via RRC (FFS details on the extend of RRC configuration) Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI For SRS antenna switching with multiple aperiodic SRS resource sets, PSRS ports indexed in an ascending order according to SRS resource set ID and SRS resource ID in a set and port number within each SRS resource Conclusion For a UE configured with a total of PSRS=6 or 8 ports across ≥1 SRS resources for antenna switching intended for xT6R or xT8R, respectively, when SRS port grouping is configured, regarding the mapping between CSI-RS ports and SRS port groups, no additional specification support is introduced - It is up to gNB implementation to ensure adequate operation of this feature

To accommodate changes arising from Rel-19 SRS port grouping feature, certain modifications to the existing Rel-18 specification are inevitable to redefine the SRS resource and resource set configurations and definitions. In general, the modifications are to be applied to xTyR UE antenna configurations for x > 2 and y = 6 and 8 ports, i.e., 2T6R, 2T8R and 4T8R to be specific. Consequently, the following excerpts in TS 38.214 may be one part involved in the specification change, while specifying SRS resources and resource sets, to accommodate SRS port grouping feature.

## 6.2.1.2 UE sounding procedure for DL CSI acquisition < Unchanged parts are omitted > - For 2T6R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'periodic', where in the case of one resource set has three SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair, where each UE antenna port in a UE antenna port pair is associated with a different UE SRS port group than the SRS port group of the other antenna port, and the UE antenna port pair follow consecutive port numbering if the UE is configured with SRS port grouping, and < Identically changed parts are omitted > - For 2T8R, zero or one SRS resource sets configured with resourceType in SRS-ResourceSet set to 'periodic', where in the case of one resource set has four SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a two SRS ports, and the SRS port pair of the resource in the set is associated with a different UE antenna port pair, where each UE antenna port in a UE antenna port pair is associated with a different UE SRS port group than the SRS port group of the other antenna port, and the UE antenna port pair follow consecutive port numbering if the UE is configured with SRS port grouping, and < Identically changed parts are omitted > - For 4T8R, zero or one or two SRS resource sets configured with a different value of resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' if the UE is not indicating srs-AntennaSwitching2SP-1Periodic, or up to two SRS resource sets configured with 'semi-persistent' and up to one SRS resource set configured with 'periodic' if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where the two SRS resource sets configured with 'semi-persistent' are not activated at the same time. Each SRS resource set has two SRS resources transmitted in different symbols, each SRS resource in a given set consisting of a four SRS ports, and the SRS ports of the resource in the set are associated with a different UE antenna ports, where two UE antenna ports among the four different UE antenna ports are associated with a different UE SRS port group than the SRS port group of the other two antenna ports, and the four different UE antenna ports follow consecutive port numbering if the UE is configured with SRS port grouping, or < Unchanged and identically changed parts are omitted >

For the Rel-19 SRS port grouping feature, support the following enhancements to UE sounding procedure for DL CSI acquisition, generalized as;

- For xTyR, SRS resource set(s) configured with resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' 'aperiodic', with y/x or a total of y/x resources, the x SRS ports of each resource in the set(s) are associated with different UE antenna ports, where x/2 UE antenna ports among the x different UE antenna ports are associated with a different UE SRS port group than the SRS port group of the other x/2 antenna ports and the x different antenna ports follow consecutive port numbering if the UE is configured with SRS port grouping.

## Extending Scheme A & Scheme B for ≤ 32 ports

Proposal 1.B: For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, extend the agreed Scheme-A and Scheme-B to 16, 24, and 32 CSI-RS ports, for all applicable RI values with K=1 only, and without any further modification/enhancement of the sub-features pertinent to the Rel-19 Type-I SP design (including, e.g. the Rel-19 Type-I SP CBSR, soft scaling). - For the Rel-19 Type-I SP codebook, the support for 16, 24, and 32ports are 3 separate UE capabilities from the support for the previously agreed number of ports (48, 64, 128 ports) - The Rel-18 SD NES schemes applicable to Rel-15 Type-I SP codebooks are also applicable to the extension of the Rel-19 Type-I SP codebook to 16, 24, and 32 ports - FFS: whether to adopt the extended orthogonal set for the 2nd SD basis for Scheme-A, RI=2-4 and 16, 24, and 32 CSI-RS ports

The above proposal was initiated in the offline discussions of RAN1 #119 and further discussed during the online meetings. It is primarily to exploit the similarity between legacy Type 1 codebook and Rel-19 Scheme-A for RI =1-2. Another aspect is the similarity between the codebook mechanisms for Rel-19 Scheme A and Rel-15 Type-I codebook for PCSI-RS < 16, for the case of RI =3-4. We agree with the proposal in principle as it helps to unify the UE measurement procedure as well as the reporting of Rel 19 UEs for all possible ports.

However, the adoption of extended orthogonal set for the 2nd SD basis vector will further complicate the codebook design, as compared to legacy and Rel 19 agreed codebooks. Hence, in our view Scheme A in its default form can be extended for RI=2-4 and 16, 24, and 32 CSI-RS ports.

Accordingly, we provide the following observation and proposal on the open FFS.

Adoption of extended orthogonal set for the 2nd SD basis vector will further complicate the codebook design, as compared to legacy and Rel 19 agreed codebooks.

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, do not support extended orthogonal set for the 2nd SD basis for Scheme-A, RI=2-4 and 16, 24, and 32 CSI-RS ports.

## CBSR

As the codebook progresses from legacy Type-I design for < 32 ports to Rel 19 design, the legacy CBSR constructions can significantly increase the signalling overhead. Hence, there were discussions and decisions directed toward the refinement of CBSR to incorporate group-based CBSRs.  The following agreements were achieved in RAN1 #117 for this topic.

Agreement (RAN1#117) For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, the following (X1, X2) values are supported: (1,1), (2,1), (2,2), (4,1), (4,2), (4,4),

Agreement (RAN1#117) For the Rel-19 Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, - N1⋅N2X1⋅X2-bit group-based CBSR - the following (X1, X2) values are supported: P; (N1, N2); (X1, X2). 48; (8,3); (1,1), (2,1), (4,1). (6,4); (1,1), (2,1), (2,2),. 64; (16,2); (1,1), (2,1), (2,2), (4,1), (4,2). (8,4); (1,1), (2,1), (2,2), (4,1), (4,2). 128; (16,4); (1,1), (2,1), (2,2), (4,1), (4,2). (8,8); (1,1), (2,1), (2,2), (4,1), (4,2). - FFS: (1,2), (1,4), (2,4), when applicable

Based on the existing realistic panel configurations and the ones defined in the standard, N1≥ N2. Since, the practical network deployments predominantly employ UPAs/NUPAs with N1> N2, the horizontal beams are narrower than vertical beams. And, the CBSR granularity for horizontal beams could be larger than that for vertical beams.

The chair’s notes of RAN1#117 meetings also specifies an agreement, provided below, to establish a dependence between (X1, X2) value on the configured (N1, N2) value. The dependence for Type II codebook was formalized in RAN1 #117 agreement, yet the dependence for Type I is an open item.

Agreement (RAN1#117) For the Rel-19 Type-I SP and Type-II codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR, the value of X1 and X2) are separately NW-configured via higher-layer (RRC) signalling from {1, 2, 4, [8, 16]} - FFS: Dependence on each supported (X1, X2) value on the configured (N1, N2) value

For the study item on the dependence on each supported (X1, X2) value on the configured (N1, N2) value for Type I codebook, we express our views as follows.

On each supported (X1, X2) values on the configured (N1, N2) value, we prefer supporting a single (X1, X2) combination to be associated with a specific (N1, N2) combination. And, to come up with the single combination, we suggest a simple method for maintaining the complexity of CBSR equal to or lesser than that of the legacy 32 port CBSR construction, even after Rel 19 enhancements to 128 CSIRS ports. The suggested CBSR group formation method and dependence of (X1, X2) value on the configured (N1, N2) value is illustrated in Table 1. The group indication X1, X2 are derived from x=roundup_to_next_even( PCSIRS/32) and associated as a function of (N1, N2).

Table 1: CBSR group formation method and Dependence on (X1, X2) value on the configured (N1, N2) value

| Sl. No | X1 | X2 | Dependence on N1, N2 |
| --- | --- | --- | --- |
| 1 | x | 1 | N1>N2≤4 |
| 2 | x2 | 2 | N1>N2≥4 |
| 3 | max⁡(2 , x2) | max⁡(2 , x2) | N1 - N2≤4 |
| wherein, x=roundup_to_next_even( PCSIRS/32) |  |  |  |

By using this method, CBSR grouping can be performed as a function of (N1, N2). The method is applicable to any values of (N1, N2)  and the CBSR bit construction is never larger than 256, which is the existing Type 1 CBSR construction for 32 ports.

A detailed illustration of the values taken up for different values of N1, N2 is provided is shown in Table 2.

Table 2: Illustration of the CBSR bit complexity reduction with the suggested method for various configured (N1, N2) value

The key advantage of this implementation is that the network need not explicitly instruct the UE to perform group based CBSR, as it can be implicitly derived based on the value of PCSIRS. Optionally, the decision table can be restricted to be used in the Rel 19 CBSR implementation and completely discarded by the network for legacy CBSR implementation with PCSIRS ≤ 32.

Type I being a popularly exercised codebook, such an implicit signaling can help the network to have a simpler implementation for Type I codebook realizations.

A simple method of supporting a single (X1, X2) combination to be associated with a specific (N1, N2) combination can be employed for maintaining the complexity of CBSR equal to or lesser than that of the legacy 32 port CBSR construction. The key advantage of this implementation is that the network need not explicitly instruct the UE to perform group based CBSR, as it can be implicitly derived based on the value of PCSIRS.

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR reduction, consider the suggested method to define the dependence between supported (X1, X2) value and the configured (N1, N2) value,

- wherein, x=roundup_to_next_even( PCSIRS/32).

| Sl. No | X1 | X2 | Dependence on N1, N2 |
| --- | --- | --- | --- |
| 1 | x | 1 | N1>N2≤4 |
| 2 | x2 | 2 | N1>N2≥4 |
| 3 | max⁡(2 , x2) | max⁡(2 , x2) | N1 - N2≤4 |

Type I being a popularly exercised codebook, the suggested implicit signaling can help the network to have a simpler implementation for Type I codebook realizations.

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR reduction, consider the suggested implicit mapping between the supported (X1, X2) value and the configured (N1, N2) value to reduce the RRC overhead with explicit signaling.

## CRI based CSI Refinement

Regarding CRI based CSI enhancements, most of the FFS items identified in the initial agreement (referring to the following agreement of RAN1 #116) and quite a lot of related issues have been resolved and agreed. Going forward, very few aspects need discussion and decision. One such aspect is related to the CSI omission rule, and is discussed in detail in this section.

Agreement (RAN1#116) For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, in accordance to the WID, extend the Rel-15 CRI-based CSI reporting as follows: - A UE is configured to measure KS>1 NZP CSI-RS resources with equal number of ports, with up to 32 ports per NZP CSI-RS resource - Note: The maximum number of ports per NZP CSI-RS resource for a given value of KS will be discussed separately - Containing the information of M “quadruplets” {(CRIn, RIn, PMIn, CQIn), n=0, …, M–1} in one CSI reporting instance where the value range of M (≤KS) is {1, …, min(X, KS)} - FFS (by RAN1# 116bis): The supported value(s) of X (candidates are 2, 4, 6, KS) - FFS (by RAN1# 116bis): Whether the value of M is NW-configured via higher-layer (RRC) signalling, or UE-selected (as a part of CSI report), or a combination of the two - A same legacy codebook (with up to 32 ports) is configured for (associated with) all M “quadruplets” FFS: detailed UCI design/optimization (e.g., overhead reduction) FFS: Whether solution to allow CSI reporting for larger number of CSI-RS resources across multiple CSI reports is supported FFS: whether further restriction(s) on CMR configuration is needed, including relation with IMR FFS: the packing order of the information of M “quadruplets”, CSI omission rule FFS: Whether all the K CSI-RS resources are associated with a same CSI-RS resource set or not FFS: Whether KS, maximum # ports per resource, and X depend on codebook type

### Other UCI Design Issues

The following agreements were achieved in RAN1 #118 and RAN1 #119 meetings on the UCI packing order and the 2M consecutive priority levels for a CSI report configured with M CRIs. The priority levels for NRep CSI reports, wherein 1≤ n ≤ NRep were finalized as well.

Agreement (RAN1#118) For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI omission for the UCI reported in CSI part-1, CSI part-1 is either reported entirely or dropped entirely, with the following UCI packing order: - CSI part-1 associated with the first configured CMR among the non-reported MR CRIs - … - CSI part-1 associated with the last configured CMR among the non-reported MR CRIs - CSI part-1 of the 1st reported CRI, - … - CSI part-1 of the (M-MR)th reported CRI, Agreement (RAN1#118) For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports based on Rel-16 eType-II, regarding UCI omission for the UCI reported in CSI part-2, other than priority 0 (G0), the UCI packing order and the 2M consecutive priority levels are defined w.r.t the M CRIs (including the non-reported MR CRIs) as follows: - Consecutive priority levels (higher to lower): - G1 PMI components associated with the first configured CMR among the non-reported MR CRIs, - G2 PMI components associated with the first configured CMR among the non-reported MR CRIs, - …, - G1 PMI components associated with the last configured CMR among the non-reported MR CRIs, - G2 PMI components associated with the last configured CMR among the non-reported MR CRIs, - G1 PMI components of the 1st reported CRI, - G2 PMI components of the 1st reported CRI, - …, - G1 PMI components of the (M-MR)th reported CRI, - G2 PMI components of the (M-MR)th reported CRI. Note: For UCI fields in wideband (G0), G1 PMI components (based on Rel-16 eType-II CB in Section 6.3.2.1.2, TS 38.212), and G2 PMI components, legacy design is fully reused. Agreement (RAN1#118) At least for type 1 single panel, for the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI omission for the UCI reported in CSI part-2, other than priority 0 (G0), the UCI packing order and the 2M consecutive priority levels are defined w.r.t the M CRIs (including the non-reported MR CRIs) as follows: - Consecutive priority levels (higher to lower): - Even sub-bands (G1) associated with the first configured CMR among the non-reported MR CRIs, - Odd sub-bands (G2) associated with the first configured CMR among the non-reported MR CRIs, - …, - Even sub-bands (G1) associated with the last configured CMR among the non-reported MR CRIs, - Odd sub-bands (G2) associated with the last configured CMR among the non-reported MR CRIs, - Even sub-bands (G1) of the 1st reported CRI, - Odd sub-bands (G2) of the 1st reported CRI, - …, - Even sub-bands (G1) of the (M-MR)th reported CRI, - Odd sub-bands (G2) of the (M-MR)th reported CRI. Note: For UCI fields in wideband (G0), even sub-bands (G1), and odd sub-bands (G2), legacy design is fully reused. Conclusion (RAN1#119) For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding packing order and priority levels of NRep CSI reports with M CRIs, the UCI packing order and the consecutive priority levels (higher to lower) follows the legacy specifications.

UCI omission in NR is primarily designed for emergency purposes when the uplink resource allocation is insufficient to carry the UCI. In our view, Rel-19 multi-CRI reporting increases the likelihood of UCI omission. Hence, it would be beneficial to modify the priority rules for CSI reports (M CRI-based) as a function of M. This discussion is taken up in the next section.

      - Priority Rules for CSI Reports

The UCI packing efficiency in Rel 19 multi-CRI reporting is well improved by modifying the Priority Rules for CSI Reports. The legacy priority rule, which states how the priority order for CSI reports is determined, should be modified for M CRI-based CSI reporting by deliberating the value of M to decide the priority rules.

For CRI reporting of M quadruplets, the network can configure MR beams to the UE, wherein the value of MR and the MR selected resources are higher layer (RRC) configured. In our view, when M CRI based CSI reporting is introduced (with MR configured or not configured), the priority criteria associated with the nth report itself shall be redefined to effectively accommodate this inclusion. Hence, we propose to enhance the priority rules for CSI reports with M CRIs by accounting for the multi-CRI aspect in the decision criteria and priority level handling action.

To improve UCI emission in Rel 19, we propose following changes to the priority rule. It may be noted that the modifications will not alter the priority of legacy CSI reports up to Rel-18 and the rule is therefore backward compatible. In summary, the new rule ensures the following;

- For reports with non-M CRI based CSI reports (Legacy CSI reports till Rel-18) should have higher priority over M CRIs based CSI reports.

- For reports configured with MR resources should have higher priority over reports with non-MR based CSI reports.

- For reports with lower M values should have higher priority over higher M CRIs based CSI reports.

As per the proposed priority rule, each report is indexed by y, k, c, s, m, M and MR, and associated with a priority value PriiCSIy,k,c,s,m,M, MR,

PriiCSIy,k,c,s,m,M, MR=2∙Ncells∙Ms∙Mm∙y+Ncells∙Ms∙Mm∙k+Ms∙Mm∙c+s+Ms∙(M-MR)∙m

where the additional terms are defined as,

<Unchanged terms are omitted>

- m = 0 for non-M CRI based CSI reports. Mm = 1 (legacy CSI reports up to Rel-18).

- m = 1 for M CRI based CSI reports. Mm is the maximum number of CRIs configured for multi-CRI CSI reports not carrying L1-RSRP or L1-SINR.

- MR is the number of non-reported CRIs configured in a CSI report;

If Mm, the maximum number of CRIs configured, is established as 4, the priority rule reduces to,

PriiCSIy,k,c,s,m,M, MR=2∙4∙Ncells∙Ms∙y+4∙Ncells∙Ms∙k+4∙Ms∙c+s+Ms∙(M-MR)∙m

To distinguish between the legacy and newly suggested priority rules and to appreciate the benefits of latter, we provide an illustrative table with example assignment of values.

Table 3: Illustration of the benefits of new priority rule in enhancing packing efficiency priority assignment with CSI Omissions

As depicted in the illustrative example, the priority levels of non-M CRI CSI reports (Legacy CSI reports till Rel-18) are unaltered by the new rule and has a higher priority over reports with M CRIs based CSI reports. Similarly, the priority rules followed for cell ids is preserved. However, the priority levels of M CRI CSI reports are redefined and improved for CSI reports with smaller payload size due to smaller M and MR values. This helps UE to prioritize critical smaller reports to gNB from being not blocked by a larger beam refinement/enhancement report with larger M values, merely because its report config id is larger, due to even a latter CSI request.

Hence, in presence of resource contention and UCI omission, weighing the report priority values as a function of the number of reported beams in a single CSI reporting instance helps to manage CSI omission more efficiently as compared to legacy. This in turn improves the overall beam management performance of the network.

It may be reasoned that the network can still manage UCI omission of Rel 19 M CRI CSI reports and prioritize smaller reports by setting a lower reportConfigID for them. However, the following points must be noted;

- The network taking care of prioritizing could create an overhead on gNBs with heavy traffic load and dynamic channel learning requirements.

- Also, the network taking care of prioritizing could have been employed in the legacy systems as well for various cases. i,e., for AP, SP and P CSI reports’ reportconfigid assignment, for primary and secondary cells' reportconfigid assignment or for L1-RSRP, L1-SINR and non- L1-RSRP, L1-SINR reports' reportconfigid assignment. Yet, the current priority rule helps to handle these aspects very efficiently, irrespective of the reportconfigids (with minimal network intervention).

When M CRI based CSI reporting is introduced (with MR configured or not configured) in CSI multi-CRI reporting, the priority criteria associated with the nth report should be redefined as a function of the number of reported beams in a single CSI reporting instance.  In presence of resource contention and UCI omission, this weighing helps to manage CSI omission more efficiently as compared to legacy and improves the overall beam management performance of the network.

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding packing order of NRep CSI reports,

- For reports with non-M CRI based CSI reports (Legacy CSI reports till Rel-18) should have higher priority over M CRIs based CSI reports.

- For reports configured with MR resources should have higher priority over reports with non-MR based CSI reports.

- For reports with lower M values should have higher priority over higher M CRIs based CSI reports.

- PriiCSIy,k,c,s,m,M, MR=2∙Ncells∙Ms∙Mm∙y+Ncells∙Ms∙Mm∙k+Ms∙Mm∙c+s+Ms∙(M-MR)∙m

- where

- m = 0 for non-M CRI based CSI reports. Mm= 1 (legacy CSI reports up to Rel-18),

- m = 1 for M CRI based CSI reports. Mm is the maximum number of CRIs configured for multi-CRI CSI reports not carrying L1-RSRP or L1-SINR.

- Note: It is up to the editor how this is captured in the specification

In our view, it is also useful to have a criterion for ordering the first to last CRIs within the reported M-MR CRIs. The proposal is to decide and explicitly specify whether the reported CRIs are ordered as per the resource config id or based on the beam quality of the configured CRI. Hence, for the completeness of explanation, we suggest that the following proposal is beneficial.

Ordering the first to last CRIs within the reported M-MR CRIs is beneficial during UCI packing and omission. And specifying the ordering of M CRIs including MR non-reported CRIs and M-MR reported CRIs is useful for spec completeness.

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI omission for the UCI reported in CSI part-2, support the following method for assigning priority levels (from higher to lower) to multiple CRIs in a single CSI instance.

- For the non-reported MR CRIs, priority order is assigned based on the RRC configured order of MR.

- For the reported M-MR CRIs (or M CRIs if MR is not configured), priority order is assigned based on the beam quality measure and not based on the RRC configured order of resource config ids.

- Note: It is up to the editor how this is captured in the specification

CSI enhancement for CJT calibration

## Linkage between CJT calibration report and CJT eType-II report

Regarding timeline relaxation when the linkage between CJTC report and CJT eType II CSI report is configured by joint triggering, the following agreements were reached in RAN1 #119 meeting.

Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint trigger, the timeline (Z/Z’) is determined as Z/Z’ associated with the Rel-18 eType-II CJT, plus Drelax - The value of Drelax is a UE capability, taken from {0, drelax} - FFS: The value of drelax (>0), including whether it depends on SCS - For linking CJTC Dd and Rel-18 eType-II CJT CSI, joint triggering is a separate UE feature group from separate triggering Agreement For the Rel-19 aperiodic standalone CJT calibration (CJTC) reporting, when linking CJTC Dd and Rel-18 eType-II CJT CSI reports is configured with a joint triggering carried on a same PUSCH (hence on a same slot), the UCI associated with the CJTC Dd report is multiplexed in CSI Part 1 - The previously agreed UCI design and mapping order for CJTC Dd report are reused - The legacy UCI design, UCI mapping order, and UCI omission for the Rel-18 eType-II CJT CSI are reused Note: The above proposal reuses the legacy UCI design principles, where the UCI associated with the CJTC Dd is placed in the part of UCI as TS 38212 Table 6.3.1.1.2-13; the CSI part 1 of Rel-18 eType-II CJT CSI is placed in the part of UCI as TS 38.212 Table 6.3.1.1.2-13 and the CSI part 2 of Rel-18 eType-II CJT CSI is placed in the part of UCI as TS 38.212 Table 6.3.1.1.2-14

Our views on the remaining FFS is provided in this section.

In joint triggering, the UE is required to process the ‘cjt-Dd’ report before it processes CJT eType-II CSI report. As the CPU occupation is reused, CSI computation timeline needs to be extended. In our view a nominal relaxation will be adequate considering the extended timeline capability in Rel 18 eType II CJT CSI. Moreover, this nominal relaxation will not allow channel ageing to affect the joint triggering. Moreover, UE can do some pre-calculation of the delays from periodic TRS.

Hence, a small relaxation should be added to the CJT CSI values for Z/Z’ and this can be realized by setting drelax = 0.5(Z2/Z2). Accordingly, the legacy CSI computation delay requirement that depends on SCS can be used as the value of drelax to extend the timeline for DO compensation.

Regarding timeline relaxation when the linkage between CJTC report and CJT eType II CSI report is configured by joint triggering, a small relaxation should be added to the CJT CSI values for Z/Z’ and the value of drelax can be defined from legacy CSI computation delay requirement.

For timeline relaxation when the linkage between CJTC report and CJT eType II CSI report is configured by joint triggering, support defining drelax = 0.5(Z2/Z2).

Conclusions

In this section we provide a summary of our observations and proposals.

Adoption of extended orthogonal set for the 2nd SD basis vector will further complicate the codebook design, as compared to legacy and Rel 19 agreed codebooks.

A simple method of supporting a single (X1, X2) combination to be associated with a specific (N1, N2) combination can be employed for maintaining the complexity of CBSR equal to or lesser than that of the legacy 32 port CBSR construction. The key advantage of this implementation is that the network need not explicitly instruct the UE to perform group based CBSR, as it can be implicitly derived based on the value of PCSIRS.

Type I being a popularly exercised codebook, the suggested implicit signaling can help the network to have a simpler implementation for Type I codebook realizations.

When M CRI based CSI reporting is introduced (with MR configured or not configured) in CSI multi-CRI reporting, the priority criteria associated with the nth report should be redefined as a function of the number of reported beams in a single CSI reporting instance.  In presence of resource contention and UCI omission, this weighing helps to manage CSI omission more efficiently as compared to legacy and improves the overall beam management performance of the network.

Ordering the first to last CRIs within the reported M-MR CRIs is beneficial during UCI packing and omission. And specifying the ordering of M CRIs including MR non-reported CRIs and M-MR reported CRIs is useful for spec completeness.

Regarding timeline relaxation when the linkage between CJTC report and CJT eType II CSI report is configured by joint triggering, a small relaxation should be added to the CJT CSI values for Z/Z’ and the value of drelax can be defined from legacy CSI computation delay requirement.

For the Rel-19 SRS port grouping feature, support the following enhancements to UE sounding procedure for DL CSI acquisition, generalized as;

- For xTyR, SRS resource set(s) configured with resourceType in SRS-ResourceSet set to 'periodic' or 'semi-persistent' 'aperiodic', with y/x or a total of y/x resources, the x SRS ports of each resource in the set(s) are associated with different UE antenna ports, where x/2 UE antenna ports among the x different UE antenna ports are associated with a different UE SRS port group than the SRS port group of the other x/2 antenna ports and the x different antenna ports follow consecutive port numbering if the UE is configured with SRS port grouping.

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, do not support the extended orthogonal set for the 2nd SD basis for Scheme-A, RI=2-4 and 16, 24, and 32 CSI-RS ports.

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR reduction, consider the suggested method to define the dependence between supported (X1, X2) value and the configured (N1, N2) value,

- wherein, x=roundup_to_next_even( PCSIRS/32).

| Sl. No | X1 | X2 | Dependence on N1, N2 |
| --- | --- | --- | --- |
| 1 | x | 1 | N1>N2≤4 |
| 2 | x2 | 2 | N1>N2≥4 |
| 3 | max⁡(2 , x2) | max⁡(2 , x2) | N1 - N2≤4 |

For the Rel-19 Type-I SP codebook refinement for 48, 64, and 128 CSI-RS ports, on CBSR reduction, consider the suggested implicit mapping between the supported (X1, X2) value and the configured (N1, N2) value to reduce the RRC overhead with explicit signaling.

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding packing order of NRep CSI reports,

- For reports with non-M CRI based CSI reports (Legacy CSI reports till Rel-18) should have higher priority over M CRIs based CSI reports.

- For reports configured with MR resources should have higher priority over reports with non-MR based CSI reports.

- For reports with lower M values should have higher priority over higher M CRIs based CSI reports.

- PriiCSIy,k,c,s,m,M, MR=2∙Ncells∙Ms∙Mm∙y+Ncells∙Ms∙Mm∙k+                                                                   Ms∙Mm∙c+s+Ms∙(M-MR)∙m

- where

- m = 0 for non-M CRI based CSI reports. Mm= 1 (legacy CSI reports up to Rel-18),

- m = 1 for M CRI based CSI reports. Mm is the maximum number of CRIs configured for multi-CRI CSI reports not carrying L1-RSRP or L1-SINR.

- Note: It is up to the editor how this is captured in the specification

For the Rel-19 CRI-based CSI refinement for up to 128 CSI-RS ports, regarding UCI omission for the UCI reported in CSI part-2, support the following method for assigning priority levels (from higher to lower) to multiple CRIs in a single CSI instance.

- For the non-reported MR CRIs, priority order is assigned based on the RRC configured order of MR.

- For the reported M-MR CRIs (or M CRIs if MR is not configured), priority order is assigned based on the beam quality measure and not based on the RRC configured order of resource config ids.

- Note: It is up to the editor how this is captured in the specification

For timeline relaxation when the linkage between CJTC report and CJT eType II CSI report is configured by joint triggering, support defining drelax = 0.5(Z2/Z2).

References

- RP-234007, “New WID: NR MIMO Phase 5”, Samsung (Moderator), 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023.

- RP-244087, “WID revision: NR MIMO Phase 5”, Samsung (Moderator), 3GPP TSG RAN Meeting #104, Maastricht, Netherlands, March 18-21, 2024.

- Chairman’s Notes, RAN1#119 v20, Orlando, US, November 18th – 22nd, 2024

- RP-242394, “WID revision: NR MIMO Phase 5”, Samsung (Moderator), 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024