3GPP TSG RAN WG1 #120			R1-2500518

Athens, Greece, February 17th – 21st, 2025

| Agenda item: | 9.1.2 |
| --- | --- |
| Source: | Baicells |
| Title: | Discussion on specification support for AI/ML-based positioning accuracy enhancement |
| Document for: | Discussion and Decision |

Introduction

Accurate positioning is an essential and integral component of many industrial use cases and verticals. In R18, a SI Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface investigated the benefits of using AI/ML to further enhance positioning accuracy as one of the target use cases. Spanning the entire duration of the release, the SI also studied the 3GPP general framework for AI/ML for air interface, evaluated the AI/ML model performance and assessed the potential specification impact considering use cases including CSI, beam management and positioning.

Under this background, a new WI [1] for R19 on AI/ML for NR Air Interface has been approved at RAN #102. This WI aims to provide normative support for general framework for AI/ML for air interface and use cases in the preceding R18 study. In addition, some study objectives will be continued to deal with outstanding issues identified in R18 study. The objective of the new WI for RAN1-lead AI/ML based positioning includes the following:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signaling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signaling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In this contribution, we present our views on positioning sub-use cases and potential specification impact for positioning accuracy enhancement.

Priorities of Positioning sub-use cases

In RAN#102, the five sub-use cases have been assigned with different priorities as shown in Figure A-1. In RAN#105, the group came to conclusion that: For second priority issues of positioning, RAN1 will not discuss any proposals targeting second priority issues in Q4, 2024.

Although companies can discuss second priority issues in 2025, we suggest RAN1 to keep working on the three sub-use cases with highest priority given that there are only three RAN1 meetings left before the end of work item.

Discussion

Model input

Performance evaluation of Sample-based and path-based measurement for model input

In this section, we first analyze sample-based and path-based measurements based on companies’ evaluation results, then a simulation on our platform is conducted regarding sample-based and path-based measurements under different settings to validate our view that sample-based measurement shows better positioning accuracy than path-based measurement.

On one hand, sample-based measurements as model input, has been widely evaluated during the R18 study phase which can significantly increase the positioning accuracy under the evaluation scenario compared with traditional positioning methods, which can be seen in TR 38.843. This obvious gain in positioning accuracy is also the main reason positioning use case is now being discussed as normative phase agenda instead of study phase agenda like CSI use case or model transfer.

On the other hand, path-based measurements as model input are only evaluated by several companies by now, with different assumptions. Given that companies often have different preferences on path detection algorithms, the evaluation of path-based measurement can be quite time-consuming and the assumptions and results of the evaluation between companies would be difficult to align with. For example, there are 7 options/methods to choose additional paths other than the first path being discussed during R17, and the discussion of these options may come back during the path-based measurement evaluation, which can further tighten the time budget and negatively affect the normative phase progress. In addition, the ambiguity issue of path-based measurement can’t be neglected as the model performance of path-based measurements may suffer compared to that of sample-based measurements. Specifically, the data collected from different UEs/TRPs to train the AI model can be quite inconsistent given that this large number of path-based measurements from a large number of UEs/TRPs of different vendors are generated using different algorithms. This ambiguity issue can lead to a degradation in positioning performance at certain scenarios, even failing to meet the required positioning accuracy.

Simulation was conducted to compare the performance of sample-based and path-based measurements as shown in Table 2-1. The InF DH{0.6, 6, 2} scenario was chosen with different number of TRPs and different system bandwidth configurations. PDP is used for sample-based measurement while for path-based measurement, 9 strongest paths are selected using our path detection method. Observations are as follows.

Firstly, when the TRP number is different (18 and 6) and the other settings remain the same, AI/ML model-based method can generally improve the positioning accuracy as the model is able to extract more features from the increased number of channel observations, or increased number of TRPs in our simulation scenario.

Following the same logic, when the information provided at the model-input side is relatively insufficient, sample-based measurement with proper configuration usually contains more information than path-based measurement since the path information is only part of the sample-based measurement of a channel. Therefore, the positioning accuracy of sample-based measurement is always superior to that of path-based measurement when bandwidth and TRP number configurations are the same.

Thirdly, as the bandwidth decreases from 100MHz to 20MHz, the positioning accuracy of path-based positioning is more affected and has a more obvious degradation compared to sample-based positioning. In our understanding, with a smaller bandwidth, paths close to each other may be hard to distinguish and this leads to poorer performance of positioning accuracy.

Table 2-1 Comparison of Sample-based and Path-based measurements

| Scenario | Measurement type | TRP number | 20 MHz CDF @90% | 100 MHz CDF @90% | Description |
| --- | --- | --- | --- | --- | --- |
| InF DH {0.6, 6, 2} | Sample-based | 18 | 2.44m | 0.65m | Power delay profile |
|  |  | 6 | 4.81m | 0.86m |  |
|  | Path-based | 18 | 3.44m | 0.93m | Power and delay, up to 9 paths |
|  |  | 6 | 5.91m | 1.42m |  |

Based on the above observation and analysis, we support sample-based measurements as model input since it can provide better positioning accuracy than path-based measurements with different settings of TRP numbers and system bandwidth.

Support sample-based measurements as model input at least for Case 3b.

*Value range of Nt’, k

In RAN1#119, the following agreements were achieved.

Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. - The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement - FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified

In the last meeting, companies reached a general agreement which merges their perspectives towards sample-based measurements and path-based measurements after intensive debate. However, the value ranges of Nt’, k as different measurement parameters need further discussion to support AI/ML positioning.

During the previous evaluation at SI phase, the most popular values among companies for Nt are 32, 64, 128, 256. To ensure consistency with the simulation parameters in the SI stage and to take into account positioning performance, the suggested values for Nt are {32, 64, 128, 256}. For Nt', the number of reported samples should be larger than the number of reported paths, which can provide more channel feature information to better locate the UE. Thus candidate values for Nt' can be {16, 32, 64, 128}.

Regarding k, the candidate values (0…5) since Rel-16 can be used.

For the value ranges of Nt’, k:

Nt’ set includes 16,32,64,128;

k set includes 0…5.

*Starting time of sample-based measurement at UE side

During the last meeting, the starting time for gNB/TRP measurement is agreed as follow.

Agreement For the definition of sample-based measurement, for gNB/TRP measurement of an estimated channel response between a pair of UE and TRP, the starting time of the list of Nt consecutive samples is determined as follows. - starting time = first detected path rounded down with timing granularity T. Note: UE-side measurement is a separate discussion

For UE-side measurement, companies can discuss the options of starting time of the list of Nt consecutive samples using previous meeting proposal in RAN1#118 as baseline.

Proposal 2.1.1.4-1 For the definition of sample-based measurement, for an estimated channel response between a pair of UE and TRP, RAN1 study the following options for down-selection for determining the starting time of the list of Nt consecutive samples: - Option A. the starting time of the list of Nt samples is the timing of the first detected path (with timing granularity T). - Option B. the starting time of the list of Nt consecutive samples is the timing of the earliest sample that the first path power is detectable. - Option C. the starting point of the Nt consecutive samples is determined by a fixed offset (with timing granularity T) relative to the reference time. - Proponent companies provide details on the value of the fixed offset. - Option D. the starting point of the Nt consecutive samples is determined by a configured offset (with timing granularity T) relative to the reference time. - Proponent companies provide details on the value range and signaling details of the configured offset.

For the above options, we prefer Option D, a reference time with a configured offset. This option is more aligned with the traditional positioning methods such as RTOA measurement, and with more flexibility given a configured offset.

Option A and Option B are based on the first path detected, which may lead to performance degradation of the model due to the fact that different UEs/gNBs have different path detection algorithms. Given that these two options were also supported by several companies in previous meetings, the group may consider down-selection and support multiple options, such as Option B and Option D.

There are also other possible mechanisms to better support UE-side measurement. For example, using configuration signaling based on UE capabilities. Moreover, UE can support the offset configuration by the NW-side if there is an offset configuration field included in the configuration signaling. In the absence of such a field, UE will use the first detected path as the starting time as default.

Regarding starting time for model input measurement for UE-side measurement, RAN1 to consider supporting multiple options, such as Option B and Option D.

*Reference time

The agreement below was reached in RAN1 #116 and RAN1 #116bis.

Agreement For AI/ML based positioning Case 3b, for gNB channel measurements reported to LMF, the timing information is represented relative to the existing UL RTOA reference time T0+tSRS as defined in TS 38.215. FFS: whether it is applicable when Case 3b is used to support multi-RTT Agreement For AI/ML assisted positioning Case 3a, at least LOS/NLOS indicator and/or timing information are supported for reporting. - If LOS/NLOS indicator is reported, the indicator can be reported as soft indicator or hard indicator as defined in 38.214. - If timing information is reported, the timing information at least can be reported via UL RTOA or gNB Rx-Tx time difference as defined in 38.215. - Note: details of the report are pending further discussion.

In RAN1#117, the following options for Case 2b regarding reference time was proposed.

Proposal 2.2.2-1 For AI/ML based positioning Case 2b, for UE channel measurements reported to LMF, the timing information is represented relative to a reference time, where the reference time is selected from the following. - Option 1: The reference time is TSubframeRxi, as defined in TS 38.215, clause 5.1.29. - Option 2: The reference time is TUE-TX, as defined in TS 38.215, clause 5.1.30. - Option 3: Introduce a DL RTOA reference time T0+tPRS

For Case 2b, some companies argue that reference time needs to be aligned for training and inference, or aligned between different devices. In our view, reusing the existing reference time should be sufficient, which means option 1 and option 2 are supported. Taking UE RSTD measurement as an example, UE reports DL RSTD to LMF using a relative time difference between the reference TRP and other TRPs. Although different UEs may use different TRPs as their reference TRP, LMF is able to figure out the timing difference relations among TRPs based on the information provided by UEs (e.g. choosing a common TRP as assumed reference). In this manner, the related timing information can be aligned for training and inference, as well as among different devices. Therefore, reusing existing reference time in Option 1 or Option 2 with certain implementation method can address this issue of Case 2b raised by companies.

For case 1 direct positioning with UE side model, how UE calculates its position, or how to make use of timing information is up to implementation as all data can be processed within UE.

For Case 2b, support the reuse of legacy reference time for UE. For Case 1, it’s up to implementation.

Phase information

In RAN1#116, the following agreements were achieved.

Agreement For AI/ML based positioning case 3b, at least the following types of time domain channel measurements are supported for reporting: - timing information - paired timing information and power information. Agreement For AI/ML based positioning for all use cases, RAN1 investigate the necessity and feasibility of using phase information (in addition to timing information and power information) for determining model input. The issues to study include: - Tradeoff of positioning accuracy and signaling overhead - The impact of transmitter and receiver implementation - Specification impact - Other aspects are not precluded Note: the phase information may be used in different ways, e.g., one phase value for the first path or first sample only; triplet of {timing information, power information, phase information} for CIR, etc.

From RAN1#116b to RAN1#118, there is an ongoing discussion of phase information, and the below proposal seems difficult to reach a consensus among companies.

Conclusion 2.4.2-1 For Rel-19 AI/ML based positioning for all use cases, there is no consensus in RAN1 to support CIR for determining model input.

During R18 study phase, the evaluation of different types of model input regarding positioning accuracy has been conducted by many companies, the related evaluation results are listed below:

Evaluation results show that if changing model input type while holding other parameters (e.g., Nt, N't, Nport, N'TRP) the same, - When comparing PDP and CIR as model input, - 9 sources showed evaluation results where the positioning error of PDP as model input is 1.06 ~ 1.62 times the positioning error of CIR as model input. - 5 sources showed evaluation results where the positioning error of PDP as model input is 0.61 ~ 0.96 times the positioning error of CIR as model input.

Phase information for model input draws companies’ attention regarding Case 3b. The above results show that most companies observed CIR as model input which contains phase information other than time and power information can help achieve better positioning accuracy. In other words, the AI model can extract important features from phase information, leading to improved positioning performance. In addition, phase information is already supported in current specification, such as R18 measurements DL RSCPD, DL RSCP, UL RSCP, thus the workload for specifying phase reporting is relatively minimal or manageable. Thirdly, most of the concerns of CIR from companies are about signaling overhead. But in our view, this cost of providing more information can be balanced in practice, for example, by parameter settings such as the Nt, Nt’. Specifically, we may consider using a smaller Nt for CIR measurements which is half of the PDP setting to shorten the data to be transferred. Therefore, CIR containing phase information gives an extra option with the potential which has been evaluated to improve the positioning accuracy, and it should not be precluded at this early stage of WI due to concerns such as overhead.

Support phase information reporting for Case 3b.

Model output

*LoS/NLoS indicator for Case 3a

LOS/NLOS indicator may be used in some AI/ML algorithms as model output. For Case 3a, we think the existing IE “LoS/NLos information” in 38.455 can be reused, and it’s unnecessary to indicate that LoS/NLoS indicator is generated by AI/ML in an explicit way. For example, when using AI positioning in Case 3a, the RTOA value indicates the time takes for a light-of-sight virtual path propagation between UE and TRP. In practice, by reusing soft LoS/NLos indicator with special values such as 0 or 1 (representing 0% or 100% respectively, which are the special values that an AI model can hardly output), the indicating of AI/ML can be achieved.

TS38.455 ### 9.2.77 LoS/NLoS Information This IE contains the LoS/NLoS information for UL measurement. IE/Group Name; Presence; Range; IE type and reference; Semantics description. CHOICE LoS/NLoS Indicator; M. >Soft Indicator. >>LoS/NLoS Indicator Soft; M; INTEGER (0..10); Values provide the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Value '0' indicates NLOS and value '1' indicates LOS.. >Hard Indicator. >>LoS/NLoS Indicator Hard; M; ENUMERATED (NLoS, LoS).

Reuse existing “LoS/NLoS information” in TS 38.455 for Case 3a. In practice, by reusing soft LoS/NLos indicator with special values such as 0 or 1 (representing 0% or 100% respectively, which are the special values that an AI model can hardly output), the indicating of AI/ML can be achieved.

Method to ensure consistency between training and inference regarding NW-side additional conditions

Generally, additional conditions are used to represent aspects (e.g., dynamic changing environment factors) related to model training but are not a part of the UE capability. Relations between UE capability, conditions, additional conditions, and models’ information have been discussed in the SI. Although additional conditions show up in both functionality-based LCM and model-ID-based LCM, it is more often discussed in functionality-based LCM as functionality-based LCM needs an innovative mechanism to ensure the consistency of additional conditions between training and inference. Comparatively, for the model-ID-based LCM, the consistency between training and inference can be easily ensured via model ID after the mode identification process.

Due to the limitation of a model’s generalization ability, a model’s performance may face heavy degradation when there is mismatch between the model training environment and model deploying environment. Unfortunately, the environment in wireless communication is usually dynamic, e.g., changing scenarios/sites as the UE is moving. To overcome this issue, operations such as model switching are needed to ensure the most suitable model under specific scenario/site can be deployed. Assuming that there is one control entity that performs the management, and a model deployment entity which runs the model, there are three major steps to address the dynamic changing environment/additional conditions problem under the functionality-based LCM background. In the following example, we assume that UE is the model deployment entity and NW is the control entity such that they follow the 3GPP network interaction manner that UE behaves following the configuration of NW.

Step 1. The model deployment entity needs to be informed of the changing of the environment/additional conditions. In the case when UE is the model deployment entity, the NW-side additional conditions e.g., current cell information, SNR are provided to UE.

Step 2. The model deployment entity reports the applicable AI/ML capability under the specific environment.

Legacy UE capability reporting is used only for reporting static applicable conditions or RRC configurations. If a legacy-like UE capability report is used for functionality-based LCM, it may not handle dynamic scenarios/sites problem well. For example, in functionality-based LCM, a UE may only support AI/ML feature X or configuration X in site A but not in site B. Under this situation, the legacy-like UE capability reporting that the UE supports AI/ML feature X or configuration X may lead to serious problem when UE is in site B. Therefore, UE may need to dynamically report its applicable capability (e.g., UE supports AI/ML feature Y in site B) for supporting the functionality under certain scenarios/sites instead of legacy-like static UE capability report.

Step 3. The control entity further config the model deployment entity based on the above applicable capability. In this step, even if the control entity (NW) may not be aware of the model details at UE side, it can config the UE using traditional configuration mechanism such as RRC. After receiving the RRC configuration, UE as the model deployment entity can select the model, which is best suitable to the RRC config, or to the current environment.

As discussed above, by providing information and/or indication on NW-side additional conditions to UE (Step 1), consistency between training and inference regarding NW-side additional conditions is ensured such that UE can select the best suitable UE-side models for model inference/deployment without NW being aware of the detail model information. In this manner, NW can achieve the control of UE behavior reusing the existing configuration methods.

Regarding specification impact, RAN1 and other WGs can start by discussing aspects which fall within or outside UE capabilities, or by firstly identifying the NW-side additional conditions for positioning. Then further specify the procedure how NW-side additional conditions are provided to UE.

To explore methods that ensure consistency between training and inference, RAN1 may need to firstly identify the NW-side additional conditions regarding positioning.

UE-side additional conditions

In our understanding, UE-side additional conditions can be referred to UE-side information which is not part of the UE capability. We believe that by providing UE-side information especially model information can be beneficial for the NW side control of UE behavior to achieve better positioning performance. Therefore, we support that UE-side model information can be transferred to LMF by LPP protocol.

For Case 1, support the transfer of UE-side model information to LMF via LPP.

Training data collection

Case 3a

Table 2-2 shows the model input and model output related information for Case 3a.

Table 2-2  model input and model output/label related information for Case 3a

| Measurements for model input | CIR/PDP | By TRP |
| --- | --- | --- |
| label | Intermediate measurement: -New measurement report: e.g., ToA, path phase -Existing measurement report: e.g., RSTD, LOS/NLOS indicator, RSRPP -Enhancement of existing measurement report: e.g., soft information/high resolution of RSTD | By network entity with known PRU location |

In preceding SI, the following consensus was reached.

Model training: - For gNB-side model, training data can be generated by the gNB, while the termination point for training data may include the gNB, or OAM. - Note: RAN2 identified the case in which LMF may be used for gNB-side model training. However, no conclusion was reached, as this depends on the RAN1 progress.

As no conclusion was reached whether LMF can be used for gNB-side model training, in our understanding, the primary focus of normative work for Case 3a model training should be to explore the feasibility of training models at gNB side. In this logic, a start for RAN1 may be to discuss how labels are provided to gNB by NW entity (e.g., LMF).

For Case 3a model training, support to explore the feasibility of training models at gNB side as the primary focus of the normative work. Consider discussing how labels are provided to gNB by NW entity (e.g., LMF) as starting point.

Case 3b

Table 2-3 shows the model input and model output related information for Case 3b.

Table 2-3  model input and model output related information for Case 3b

| Measurements for model input | -Potential new measurement: CIR/PDP -Existing measurement: e.g., RSRP/RSRPP/RSTD | By TRP |
| --- | --- | --- |
| Model output | UE position | By LMF with known PRU location |

For Case 3b model training, the entity that collects the data and performs the training is considered as NW side. Therefore, new measurements such as CIR/PDP may need to be specified same as Case 3b model inferencing.

For Case 3b data collection for model training, new measurements (if necessary) such as CIR/PDP may need to be specified same as Case 3b model inferencing.

The existing traditional positioning framework and procedures are designed to derive UE position via UE-RAN-CN cooperation, which can be reused or provide designing reference for the model inference procedure. However, for data-driven AI/ML based positioning, the training data collection procedure becomes as important as inference procedure and may need to redesign such that it can meet various data collection requirements. For example, when a large scale of data is required for offline model training, it may have a lower real-time requirement compared to the existing positioning procedure while the number of types of the measurements within one transfer may increase significantly such that data with certain relationships can be collected in certain pattern for different training purpose.

For data-driven AI/ML based positioning, the training data collection procedure may need to redesign such that it can meet various data collection requirements which is different from the existing positioning procedure.

Based on the above analysis, there may be several potential signalling enhancements to facilitate training data collection in our understanding.

Firstly, in existing positioning procedures, the SRS configuration is decided by the serving gNB, then the specific SRS configuration will be provided to LMF. However, when LMF needs to collect data for Case 3b model training, LMF is expected to directly indicate the specific SRS configuration under which the data will be collected since it is the LMF or the NW side entity which has the full knowledge/control of the data and NW-side models. Therefore, for Case 3b data collection for model training, the LMF should indicate its desired SRS configuration in data collection request, and the serving gNB should provide the configuration result to LMF in the response message.

For Case 3b data collection for model training, the LMF should indicate its desired SRS configuration in data collection request, and the serving gNB should provide the configuration result to LMF in the response message.

Secondly, considering the number of types of the measurements within one transfer for Case 3b data collection may be large, and LMF may request multiple gNBs to transfer the data during the procedure, it would be beneficial that LMF is aware that whether all selected gNBs are able to provide all required measurements/data before the start of measurements reporting. Otherwise gNBs may provide different number of measurement types of the data that can only be aligned after further processing at NW side to form a dataset, which will cause heavy waste of the bandwidth transferring data that cannot be used, as well as extra NW-side computation resources when organizing the dataset.

For Case 3b data collection for model training, due to different gNB capabilities and other factors, gNBs may provide different number of measurement types of the data that can only be aligned after further processing at NW side to form a dataset, which will cause heavy waste of the bandwidth transferring data that cannot be used, as well as extra NW-side computation resources when organizing the dataset.

Therefore, we think it’s necessary to have a measurement request indicating the desired measurements for data collection from LMF, as well as a measurement response indicating the supported measurements by gNBs.

For Case 3b data collection for model training, a measurement request indicating the desired measurements for data collection from LMF, and a measurement response indicating the supported measurements by gNBs can be beneficial to better support data collection.

Quality indicator

Quality indicator for training data collection

In RAN1#118bis, following agreement was reached.

Agreement For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is: - When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455; - FFS: details on how to associate quality indicator to timing information

Here we mainly consider the Case 3b scenario. Definition of Timing Measurement Quality is shown in Table 2-4 as in NRPPa. For traditional positioning methods in NRPPa, Timing Measurement Quality as Measurement Quality can be carried in TRP Measurement Result in the MEASUREMENT RESPONSE Message along with the corresponding measurement such as UL RTOA as shown in Table 2-5. Following the same logic, timing information in Part A for each TRP as a new type of measurement can be transferred to LMF along with the optional Timing Measurement Quality. In this way, the normative workload for the quality indicator can be reduced.

In NRPPa, the Timing Measurement Quality can be associated with the timing information of Part A for each TRP, which can later be transferred to LMF. In this way, the normative workload for the quality indicator can be reduced.

Table 2-4 Timing Measurement Quality in 38.455

Table 2-5 TRP Measurement Result in 38.455

Quality indicator for inference

In RAN1#118bis, following agreements were reached.

Agreement From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) Channel measurement; - (Optional) Quality of the channel measurement; - FFS: details of the quality - (Mandatory) Time stamp of the channel measurement. Agreement From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF: - (Mandatory) timing information; - (Optional) Quality of the timing information; - Existing IE “Timing Measurement Quality” can be reused. - (Mandatory) Time stamp. FFS: LOS/NLOS indicator. Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

In the last meeting, companies have different views regarding quality indicator during inference. For example, whether the quality indicator should include timing and power separately, or whether quality indicator should be per measurement or per measurement element. In our view, existing IE “Timing Measurement Quality” can be supported per measurement to align the measurement for both training and inference with traditional positioning methods, while power quality indicator needs further analysis given that it’s not well evaluated in the past. In our understanding, there is no strong need in the power quality indicator for now, considering that it’s only for the PDP measurement rather than DP.

For Case 3a and 3b inference, support reusing Existing IE “Timing Measurement Quality” to align the measurement for both training and inference with traditional positioning methods.

There is no strong need for power quality indicator considering it’s only for PDP and has not been well evaluated in the past.

Quality indicator for Case 1 label

During last meeting, quality indicator of the label was discussed as below.

Proposal 4.1.5-1A For training data collection of AI/ML based positioning Cases 1, from RAN1 perspective, when label reporting is applicable, quality indicator of label is represented using one of the following options. RAN1 study and select between Option 1 and Option 2. - Option 1: reuse an existing IE. For each case, companies provide input on the existing IE to use. - Option 2: define a new IE on UE location quality provided by LMF to the UE. Use hard (1 or 0) and soft values (0, 0.1, 0.2, …, 1.0), with value 1 indicate the highest quality. - Proponent companies provide information on how to determine the value, entity to generate the label quality, and the applicable signaling.

In our view, Option 1 should be sufficient if it is needed. For example, LocationUncertainty-r16 in 37.355 can be reused.

For quality indicator of Case 1 label, support Option 1, reusing an existing IE.

Model monitoring

*Case 1

RAN1#119 Agreement For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B RAN1#118 Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4. RAN1#116bis Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

As suggested in the preceding SI, on one hand, the UE can monitor the performance of its UE-side model; on the other hand, for monitoring at the LMF side, the gNB or UE can generate, if needed, calculated performance metrics or data required for performance metric calculation, while the termination points for these metrics is the LMF.

Table 2-6 Analysis on different monitoring options of Case 1

|  | Potential specification impact | Feasibility/concerns |
| --- | --- | --- |
| Option A-1 | LMF sends UE location to target UE. | -Existing procedure/signaling can be reused or as reference. -The scenarios under which the specific monitoring method can be used may be very limited. For example, in Option A-1, legacy measurement only works well under LOS condition which may seldom exist, thus the model can’t be monitored all the time. -The availability of certain types of assistance data (e.g., gNB position). |
| Option A-2 | LMF sends assistance data to target UE. |  |
| Option A-3 | LMF sends PRU measurement and corresponding PRU location to target UE. | More signaling overhead compared with Option A-1 and Option A-2. |
| Option B-1 | No spec impact expected. | It’s feasible, straightforward and efficient. PRU and UE location can be sent to LMF using existing signaling for comparison. |
| Option B-2 | LMF sends measurement information of PRU to target UE, and UE sends estimated location to LMF. | This procedure seems complicated, time consuming for implementation and unnecessary. Both PRU and UE sending their measurements and ground truth/estimated location to LMF is a better monitoring approach. |

Based on above analysis, we think Option A-1, A-2 are feasible and can be used based on specific scenarios. In addition, Option B-1 can be achieved using existing mechanism.

Support Option A-1, A-2 for Case 1 monitoring. Support Option B-1 for Case 1 monitoring, which can achieve minimum specification effort.

Case 3a

In our understanding, the monitoring can be either at gNB side or LMF side. If the monitoring is at gNB side, minimum specification effort is expected as it can be up to implementation. If LMF can perform the monitoring, performance metrics or data required for performance metrics will be provided to LMF via NRPPa.

gNB-side or LMF-side monitoring for Case 3a are both feasible.

Case 3b

As the model is deployed in LMF, model monitoring can be up to implementation, hence no specification impact is expected.

No specification impact is expected for Case 3b model monitoring since monitoring can be deployed within the NW and based on implementation.

Conclusions

Observation 1.	Support sample-based measurements as model input at least for Case 3b.

Observation 2.	To explore methods that ensure consistency between training and inference, RAN1 may need to firstly identify the NW-side additional conditions regarding positioning.

Observation 3.	For Case 3b data collection for model training, new measurements (if necessary) such as CIR/PDP may need to be specified same as Case 3b model inferencing.

Observation 4.	For data-driven AI/ML based positioning, the training data collection procedure may need to redesign such that it can meet various data collection requirements which is different from the existing positioning procedure.

Observation 5.	For Case 3b data collection for model training, the LMF should indicate its desired SRS configuration in data collection request, and the serving gNB should provide the configuration result to LMF in the response message.

Observation 6.	For Case 3b data collection for model training, due to different gNB capabilities and other factors, gNBs may provide different number of measurement types of the data that can only be aligned after further processing at NW side to form a dataset, which will cause heavy waste of the bandwidth transferring data that cannot be used, as well as extra NW-side computation resources when organizing the dataset.

Observation 7.	For Case 3b data collection for model training, a measurement request indicating the desired measurements for data collection from LMF, and a measurement response indicating the supported measurements by gNBs can be beneficial to better support data collection.

Observation 8.	gNB-side or LMF-side monitoring for Case 3a are both feasible.

Observation 9.	No specification impact is expected for Case 3b model monitoring since monitoring can be deployed within the NW and based on implementation.

Proposal 1.	For the value ranges of Nt’, k:

	Nt’ set includes 16,32,64,128;

	k set includes 0…5.

Proposal 2.	Regarding starting time for model input measurement for UE-side measurement, RAN1 to consider supporting multiple options, such as Option B and Option D.

Proposal 3.	For Case 2b, support the reuse of legacy reference time for UE. For Case 1, it’s up to implementation.

Proposal 4.	Support phase information reporting for Case 3b.

Proposal 5.	Reuse existing “LoS/NLoS information” in TS 38.455 for Case 3a. In practice, by reusing soft LoS/NLos indicator with special values such as 0 or 1 (representing 0% or 100% respectively, which are the special values that an AI model can hardly output), the indicating of AI/ML can be achieved.

Proposal 6.	For Case 1, support the transfer of UE-side model information to LMF via LPP.

Proposal 7.	For Case 3a model training, support to explore the feasibility of training models at gNB side as the primary focus of the normative work. Consider discussing how labels are provided to gNB by NW entity (e.g., LMF) as starting point.

Proposal 8.	In NRPPa, the Timing Measurement Quality can be associated with the timing information of Part A for each TRP, which can later be transferred to LMF. In this way, the normative workload for the quality indicator can be reduced.

Proposal 9.	For Case 3a and 3b inference, support reusing Existing IE “Timing Measurement Quality” to align the measurement for both training and inference with traditional positioning methods.

Proposal 10.	There is no strong need for power quality indicator considering it’s only for PDP and has not been well evaluated in the past.

Proposal 11.	For quality indicator of Case 1 label, support Option 1, reusing an existing IE.

Proposal 12.	Support Option A-1, A-2 for Case 1 monitoring. Support Option B-1 for Case 1 monitoring, which can achieve minimum specification effort.

- References

[1] RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface RAN#102 December 2023.

- Appendix

Figure A-1  Sub-use cases with different priorities