3GPP TSG RAN WG1 #120	  	                                      R1-2500519

Athens, Greece, February 17th – 21st, 2025

Agenda Item: 	9.11.1

Source: 	Baicells

Title:	Discussion on downlink coverage enhancement for NR NTN

Document for:	Discussion and Decision

1. Background

For NTN downlink coverage enhancement, WID/agreements/conclusions/observations in previous meetings are captured in this part.

- RAN#102

In RAN#102, a new WID was defined for Non-Terrestrial Networks (NTN) for NR Phase 3. Downlink coverage enhancement is one of its objectives. Justification and objective for downlink coverage enhancement is captured as below [1].

Justification:

- Offer optimized performance especially when addressing handset terminals (including smartphones with -5.5 dBi antenna gain) w.r.t. downlink coverage considering the NTN deployment constraints such as payload power limitation, large satellite foot print and limited feeder link bandwidth. DL coverage enhancements are needed to accommodate satellite payload constraints which may be unable to have all its beams active with the « nominal » EIRP density per beam (see Section 6.1.1 in TR 38.821) at a given time due to limited power and limited feeder link bandwidth, while maximizing the number of beams that can be activated simultaneously, and ensuring that all user terminals can be served across the satellite foot print while maximizing the overall satellite throughput and ensuring that all satellite’s radio cells are kept alive even without traffic but allowing new users to join or preventing impact on end-user QoS.

DL coverage enhancements can be considered at both

- Link level to improve the link margin of selected physical channels in order to accommodate the EIRP reduction in FR1-NTN. A link margin improvement for physical channels (e.g. PDSCH and PDCCH) may be considered without impact on SSB design.

- System level to support an efficient dynamic and flexible power sharing between beams or different beam pattern/size (i.e., wide or narrow) across the satellite foot print for FR1-NTN and FR2-NTN.

Objective:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4]

- Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth.

- Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements.

- Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.

- Notes for this objective:

  - SSB channel enhancement is not considered

  - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE

  - NGSO to be considered in priority: LEO Set-1 @ 600 km

  - Rel-18 network energy saving techniques should be considered as baseline in the system level study

- RAN1#116

In RAN1#116, the following agreements were made [2]:

Agreement

For DL coverage study, consider the following additional reference satellite parameters scenarios for LEO600km Set1 in FR1 (i.e., S-band), referred to as Set1-1 FR1, Set1-2 FR1 and Set1-3 FR1:

| LEO600km Set1-1 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size(Note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 31.24 |
| Aggregated EIRP (Total) (dBW) | 61.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints*** | 1058 |
| Total number of simultaneously active beams ** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 61.24 dBm for the reference configuration. **Assuming 100 % Resource Block utilization within the same beam at max power. Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) *** For a constellation design at 600km with low elevation angle with 30° and selected (i.e Set 1 parameters) beam size Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set1-2 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 34 |
| Payload Total DL power level (dBW) | 23 |
| Aggregated EIRP (Total) (dBW) | 53* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 41 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 16 |
| % simultaneously active beams** | 1.5 % |
| *Note: EIRP limit is 53 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 16 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

| LEO600km Set 1-3 FR1 (i.e., S-band) |
| --- |
| Maximum Bandwidth per beam | 5 MHz |
| SCS | 15 kHz |
| Beam size (note 1) | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 26 |
| Payload Total DL power level (dBW) | 23.24 |
| Aggregated EIRP (Total) (dBW) | 53.24* |
| Satellite Tx max Gain | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 33 |
| Total number of beam footprints | 1058 |
| Total number of simultaneously active beams** | 106 |
| % simultaneously active beams** | 10.02 % |
| *Note: EIRP limit is 53.24 dBm for the reference configuration. **Absolute number of simultaneously active beams is up to 212 (due to limitation of RF) Note 1: At least this beam size is considered in this scenario, larger beam sizes maybe evaluated and reported by companies |  |

Note: RAN1 will aim to identify necessary enhancements for these scenarios in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage study at system level, consider the following additional reference satellite payload parameters for LEO600km in FR2 (i.e., Ka-band):

| LEO600km Set1-1 FR2 (i.e., Ka-band) |
| --- |
| Maximum Bandwidth per beam | 400 MHz |
| SCS | 120 kHz |
| Beam size | TBD in next meeting |
| Satellite EIRP density /beam (dBW/MHz) |  |
| Payload Total DL power level (dBW) |  |
| Aggregated EIRP (Total) (dBW) |  |
| Satellite Tx max Gain |  |
| EIRP per Satellite beam (dBW) |  |
| Total number of beam footprints | 800 (note 1) |
| Total number of simultaneously active beams | 12 |
| % simultaneously active beams | 1.5 % |
| Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF) |  |

Agreement

Adopt the following phased array antenna parameters for LEO 600km in FR1:

| Satellite phased array antenna Characteristics | LEO-600 |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | [65] for H [65] for V |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | [400 elements (20 x 20)] |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | [4dB] |

Agreement

RAN1 to consider the following performance metrics for DL Coverage enhancement evaluation at system level:

At least:

- CDF of the received SINR

- The dwell time and revisit time interval for each beam illumination across the coverage

- Periodicity of common control channels (e.g. SSB, CORESET0/SIB1, SIB19) and corresponding coverage ratio

Other metrics may be reported such as

- CDF of the cell throughput

- CDF of user perceived throughput (UPT)

- CDF of Latency

- Ratio of mean served cell throughput and offered cell throughput, denoted by 𝜌 (refer to TR36.889)

For system level study based on analytical evaluation:

- N1 beam footprints are in state “off”

  - These beam footprints are not served by any signal (no satellite service in this area)

- N2 beam footprints are in state “common messages only”

  - These beam footprints do not have any active user traffic, and are served the necessary information for cell discovery and initial access.

  - Optionally, companies may consider user arrival (e.g. RACH access) in this type of cell, and should describe how this is taken into account in the analytical evaluation

- N3 beam footprints are in state “active traffic”

  - These beam footprints have X active (e.g. VoNR) users each.

  - These beam footprints are also served the necessary information for cell discovery and initial access

- N1 + N2 + N3 = “Total number of beam footprints “

- N1, N2, N3, X are to be reported by companies.

- Resource utilization obtained under the assumptions above is to be reported by companies.

- Other assumptions made in the evaluation are to be reported by companies, e.g. power sharing scheme, beam hopping scheme, etc.

Agreement

For NR NTN Rel-19 DL coverage evaluation, UE characteristics for handheld terminals in Table 6.1.1.1-3 in TR 38.821 can be reused, with the following:

- -5.5 dBi antenna gain is assumed

- at least 2Rx are considered at the UE

  - 4Rx can be optionally considered and reported

Note: Redcap device is not considered in the scope of DL coverage study

Agreement

The following traffic models are considered for system level evaluation of DL coverage:

- FTP3: as in Table 6.1.1.1-7 of TR 38.821: 0.5MB as packet size, 200ms as mean inter-arrival time

- FTP3 IM: 0.1MB as packet size, 2s as mean inter-arrival time

- VoIP can be considered in the evaluation.

It is up to company report which traffic model is used among the discussed traffic models in their evaluations.

- Other models may be used as well, and parameter (e.g. packet size and arrival rate) adjustment can be optionally considered and reported.

| Traffic type | FTP | IM | VoIP |
| --- | --- | --- | --- |
| Model | FTP model 3 | FTP model 3 | As defined in Rel-18 NTN CE. |
| Packet size | 0.5 Mbytes | 0.1 Mbytes |  |
| Mean inter-arrival time | 200 ms | 2 sec |  |

Agreement

For NR NTN Rel-19 DL coverage evaluation, Beam layout defined in Table 6.1.1.1-4 in TR 38.821 can be reused.

- Using other beam layouts is not precluded, and should be reported by companies

Agreement

For NR NTN Rel-19 DL coverage evaluation, a value of beam steering latency equal to 0 at least if phase array antenna is assumed.

Values different from 0 can be optionally reported

Agreement

DL coverage is evaluated at link level with the following considerations:

- NGSO at LEO-600 operating in FR1 is considered in priority

- Additional satellite payload parameters defined for system level evaluation are used

- FFS: Antenna gain reduction due to steering loss can be considered

Agreement

For the evaluation of NTN downlink coverage at link level, reuse the target data rate from Rel-18 NTN Coverage enhancements:

- For VoIP: AMR 4.75 kbps (TBS of 184 bits without CRC in physical layer) with 20 ms data arriving interval

- For data rate service: both 3 kbps and 1Mbps can be considered

  - Companies can also use the data rates corresponding to the traffic types used for system level evaluations

Agreement

For link-level study, downlink coverage performance in NR NTN is evaluated according to the following steps.

Step 1: CNR is calculated as defined in 6.1.3.1 of TR 38.821

Step 2: Required SNR of target service is evaluated by LLS

Step 3: The CNR and the required SNR are compared

Agreement

For link-level study, for NR NTN DL coverage enhancement, the following channels/signals can be considered for evaluations:

- PDSCH for VoIP

- PDSCH for low data rate service

- PDSCH Msg.2

- PDSCH Msg.4

- PDSCH carry SIB, e.g., SIB1, SIB 19

- PDSCH for paging

- PDCCH

- Broadcast PDCCH (e.g. PDCCH of Msg.2, paging)

- SSB

Note: RAN1 will aim to identify necessary link-level enhancements for these channels in the study phase. At the end of the study phase, RAN1 will further discuss whether the potential link-level enhancements will be specified within Rel-19 framework.

Agreement

For DL coverage performance evaluation, the following are assumed for all channels/signals

- Channel model/Delay spread:

  - Channel model as in Table 6.1.2-4 of TR38.821, NTN-TDL-C (LOS)

- Evaluation scenario:

  - Rural (LOS)

- Channel estimation: Realistic estimation:

  - Companies are encouraged to report channel estimation method.

- SCS:

  - 15 kHz only

- UE speed: 3 km/h

- Frequency drift: TBD

- Frequency offset: 0.1 ppm

Agreement

For link budget calculation, parameters in the following table are assumed:

| Parameters |  |
| --- | --- |
| Carrier frequency | 2 GHz for DL (S-band) |
| Satellite altitude | 600 km |
| Target elevation angle | 30° (LEO) |
| Atmospheric loss | Equation (6.6-8) in [38.811] |
| Shadowing margin | 3 dB |
| Scintillation loss | Section 6.6.6 in [38.811] Ionospheric loss: = 2.2 dB Tropospheric loss: Table 6.6.6.2.1-1 of [38.811] |
| Additional loss | 0 dB |
| Clear sky conditions | Yes |
| Satellite antenna polarization | Circular polarization |
| Terminal type | [S band: (M, N, P) = (1,1,2)] |
| UE antenna gain | -5.5dBi |
| Free space path loss | Equation (6.6-2) in [38.811] |
| Polarization loss | 3dB |
| Outcome | CNR |

- RAN1#116-bis

In RAN1#116-bis, the following agreements or observation were made [3]:

Agreement

For coverage evaluation of PDCCH in NR NTN, the following table is assumed:

| Parameter | Value |
| --- | --- |
| Number of UE receive chains | 2 for 2GHz |
| Aggregation level | 8 |
| Payload | 40 bits |
| CORESET size | 2 symbols, 24 PRBs |
| Tx Diversity | Reported by companies |
| BLER | 1% BLER optional for 10% BLER |
| Number of SSB for broadcast PDCCH of Msg.2 | Reported by companies |
| Other parameters | Reported by companies |

Agreement

For coverage evaluation of PDSCH in NR NTN, the following table is assumed:

| Parameter | Value |
| --- | --- |
| BLER | For low data rate service, w/ HARQ, 10% iBLER; w/o HARQ, 10% iBLER. For VoIP, 2% rBLER. |
| Waveform | CP-OFDM |
| Number of UE receive chains | 2 for 2GHz |
| HARQ configuration | Whether/How HARQ is adopted is reported by companies. |
| DMRS configuration | 3 DMRS symbols is used for PDSCH of Msg.2. For 3km/h: Type I, 1 or 2 DMRS symbol, no multiplexing with data. PDSCH mapping Type, the number of DMRS symbols and DMRS position(s) are reported by companies. |
| PRBs/TBS/MCS for data rate service | Any value of PRBs, and corresponding MCS index, reported by companies will be considered in the discussion. TBS can be calculated based on e.g. the number of PRBs, target data rate, frame structure and overhead. 24 PRBs for SIB1 and SIB19 |
| PRBs/MCS for VoIP | Any value of PRBs reported by companies will be considered in the discussion. QPSK |
| PDSCH duration | 12 OS |
| Payload size for PDSCH of Msg.2 | 72 bits |
| Payload size for PDSCH of SIB1 | FFS |
| Payload size for PDSCH of Msg.4 | 1040 bits |
| Payload size for PDSCH of SIB19 | FFS |
| Other parameters | Reported by companies. |

Agreement

Antenna gain reduction due to steering loss is not considered in the link level evaluation.

Note: This is aligned with the assumptions made in Rel-18 UL coverage enhancement

Observation

The CNRs for the satellite payload parameters Set 1-1, Set 1-2 and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively.

Agreement

Confirm the Satellite phased-array antenna parameters for LEO 600km in FR1 defined in RAN1#116.

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | Table7.3-1 in TR 38.901 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 65 for H 65 for V |
| Antenna element spacing | 0.667 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 400 elements (20 x 20) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi |
| Steering loss at 30° elevation angle | 4 dB |

Al least the above model is considered for SLS to ease the alignment between evaluation results. The model below can be optionally considered:

| Satellite phased array antenna Characteristics |  |
| --- | --- |
| Orbit | LEO-600km |
| Frequency range/band | FR1/S-Band |
| Antenna element pattern | TR38.820 section 7.2.4 |
| Horizontal/vertical 3 dB beam width of single element (degree) | 90 for H 90 for V |
| Antenna element spacing | 0.5 lambda |
| Antenna polarization | Circular (RHCP or LHCP) |
| Number of antenna elements | 676 elements (26 x 26) |
| Equivalent satellite antenna aperture | 2m |
| Element maximum gain | 4 dBi |
| Antenna maximum gain | 30 dBi (Note 1) |
| Steering loss at 30° elevation angle | 2.5 dB |

Note 1: The maximum antenna gain is determined by considering an overall array efficiency [of 50%.]

R1-2401992	FL Summary #2: NR-NTN downlink coverage enhancements	THALES

Agreement

For coverage evaluation of PDSCH in NR NTN, the following payload sizes for PDSCH are assumed:

| Payload | value |
| --- | --- |
| Payload size for PDSCH of SIB1 | Option 1: 800 bits Option 2: 1280 bits |
| Payload size for PDSCH of SIB19 | 616 bits |

Note: At least the above values are simulated and reported. Other values can be considered.

Note: the values above are not the TBS.

Agreement

For DL coverage study at system level, it is up to companies to report the following parameters for LEO600km Set1-1 FR2:

Beam size

Satellite EIRP density /beam (dBW/MHz)

Payload Total DL power level (dBW)

Aggregated EIRP (Total) (dBW)

Satellite Tx max Gain

EIRP per Satellite beam (dBW)

Agreement

For coverage performance evaluation of DL channels/signals before the SIB19 acquisition, the maximum Doppler frequency drift is assumed to be equal to 0.27 ppm/s based on TR 38.821.

- RAN1#117

In RAN1#117, the following observations were agreed[4]:

Observation

Based on LLS results on PDCCH coverage evaluation collected from different sources:

- It is observed that the required SNR for PDCCH is in average equal to -6dB (17 sources)

  - With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 17 sources observed that there is no coverage gap with Set1-1/1-2 FR1.

      - The coverage margin is around 4 dB compared to CNR of -1.9 dB.

  - With parameter LEO600km Set1-3 FR1:

    - 15 sources observed that there is a PDCCH coverage gap of 3.9dB in average compared to CNR of -9.9 dB.

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDCCH.

Observation

Based on LLS results on PDSCH Msg2 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg2 is in average equal to – 10.9 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is around 9 dB compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 12 sources observed that there is no coverage gap for PDSCH with Msg2:

      - The coverage margin is around 1 dB on average compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH Msg2.

Observation

Based on LLS results on PDSCH Msg4 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying Msg4 is in average equal to – 5.2 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 14 sources observed that there is no coverage gap for PDSCH with Msg4:

      - The coverage margin is around 3.3 dB on average compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with Msg4:

    - The coverage gap is around 4.7 dB on average compared to CNR of -9.9 dB

  - 1 source observed that there is no coverage gap for PDSCH with Msg4 with a coverage margin of 0.3 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH Msg4.

Observation

Based on LLS results on PDSCH SIB1 coverage evaluation collected from different sources:

- For PDSCH carrying SIB1 option 1 (with a payload size of 800bits) it is observed that the required SNR is in average equal to – 5.8 dB (14 sources)

- For PDSCH carrying SIB1 option 2 (with a payload size of 1280bits) it is observed that the required SNR is in average equal to – 3.4 dB (12 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2FR1:

  - 14 sources observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is around 3.9 dB on average compared to CNR of -1.9 dB

  - 12 sources observed that there is no coverage gap for PDSCH with SIB1 option 2:

      - The coverage margin is around 1.5 dB on average compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 11 sources observed that there is a coverage gap for PDSCH with SIB1 option 1:

      - The coverage gap is around 4.1 dB on average compared to CNR of -9.9 dB

  - 1 source observed that there is no coverage gap for PDSCH with SIB1 option 1:

      - The coverage margin is 3.4 dB compared to CNR of -9.9 dB

  - 10 sources observed that there is a coverage gap for PDSCH with SIB1 option 2:

      - The coverage gap is around 6.5 dB on average compared to CNR of -9.9 dB

- Note: some results assumed SIB1 combination (where SIB1 is repeated within 160 ms) and some results assumed no SIB1 combination

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB1.

Observation

Based on LLS results on PDSCH SIB19 coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH carrying SIB19 is in average equal to – 6.9 dB (14 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

    - 12 sources observed that there is no coverage gap for PDSCH with SIB19:

      - The coverage margin is around 4.2 dB on average compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 10 sources observed that there is a coverage gap for PDSCH with SIB19:

      - The coverage gap is around 3.5 dB on average compared to CNR of -9.9 dB

- Note: all the results above assumed no SIB19 combination

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH carrying SIB19.

Observation

Based on the results of DL coverage ratio evaluation at system level collected from 7 sources for all the three LEO600km satellite parameter sets where the beam footprint diameter is 50 km:

- For Set 1-1/1-3, the coverage ratio can be improved from 10% to 100% if the SSB periodicity is increased from 20ms to 80ms and beam hopping is applied

- For Set 1-2, the coverage ratio can be improved from 1.5% to 96.8% if the SSB periodicity is increased from 20ms to 320ms and beam hopping is applied.

- Note: coverage ratio is N2+N3/ total beam footprints

- Note: the baseline assumes no beam hopping. TDM between SIB1 and SIB19 is assumed in those results, following current specs.

Based on the results of DL coverage ratio evaluation at system level collected from 3 sources for a deployment scenario implementing wide beam footprint:

- 1 source reports that with a deployment of wide beam covering 4 narrow (of 50km size) beams, which means Set 1-2 FR1 with additional EIRP reduction of 6dB, using SSB periodicity of 80 ms can provide coverage ratio of 96.8%, and Set 1-1/1-3 FR1 with additional EIRP reduction of 6dB, SSB periodicity of 80 ms can provide coverage of 100%.

- 1 source observed that for Set 1-1, 1-2 and 1-3, the coverage ratio can be improved from 1.5% to 100% using the legacy default SSB periodicity of 20ms during initial access, by choosing a wide beam footprint with beam footprint sizes of 84 km and 56 km respectively.

  - Note: the PDCCH and the PDSCH for SIB19 is assumed to be transmitted within 2 OFDM symbols and 5 MHz bandwidth. the PDSCH for SIB1 is assumed to be transmitted within 3 OFDM symbols and 5 MHz bandwidth. This assumes no SIB1 and SIB19 transmission in N2 beam footprints. This assumes non-aligned SFN timing across different beams.

- 1 source observed, for Set 1-1 with increased beam size, that the legacy SSB periodicity of 20ms during initial access is usable with NTN beam hopping, by choosing a deployment scenario implementing wide beam footprint with beam footprint sizes of 70.7 km and 86.6 km, leading to a total of 529 and 353 beam footprints within the satellite coverage area, respectively, and the coverage ratio is 80% and 90%, respectively, and a ratio of simultaneously active beam footprints to the total number of beam foot prints equal to 20% and 30%.

  - Note: Beam footprint size is increased by increasing only the adjacent beam spacing without increasing the 3dB beamwidth.

Note: RAN1 will further investigate the impact of SSB periodicity extension

Note: Any needed clarification “SSB channel enhancement is not considered” in the WID is up to RAN plenary

Note: RAN1 will further investigate the impact of wider beam of SSB and/or other channels on performance (e.g. link budget, capacity...)

Observation

Based on LLS results on PDSCH for VoIP coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for VoIP is in average equal to – 11 dB (11 sources)

- With parameter LEO600km Set1-1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 11 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 9.1 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 9 sources observed that there is no coverage gap for VoIP:

    - The coverage margin is around 2.3 dB on average, compared to CNR of -9.9 dB

  - 1 source observed that even with 8 PDSCH repetitions there is a coverage gap of 1.5 dB compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH for VoIP.

Observation

Based on LLS results on PDSCH 3kbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH for low data rate is in average equal to – 11 dB (8 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - When PDSCH repetition is enabled, 8 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 9.1 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - When PDSCH repetition is enabled, 6 sources observed that there is no coverage gap for PDSCH with 3kbp:

  - The coverage margin is around 1.6 dB on average, compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH 3kbps.

Observation

Based on LLS results on PDSCH 1Mbps coverage evaluation collected from different sources:

- It is observed that the required SNR for PDSCH with 1Mbps data rate is in average equal to – 4.1 dB (7 sources)

- With parameter LEO600km Set1-1 FR1 and 1-2 FR1:

  - 7 sources observed that there is no coverage gap for PDSCH with 1Mbps:

  - The coverage margin is around 2.2 dB on average, compared to CNR of -1.9 dB

- With parameter LEO600km Set1-3 FR1:

  - 5 sources observed that, there is a coverage gap for PDSCH with 1Mbps:

  - The coverage gap is around 5.5 dB on average, compared to CNR of -9.9 dB

- Note: the results above are obtained independently from the performance of other channels or signals, and it doesn’t imply the successful reception for other channels or signals before or after the detection of PDSCH 1Mbps.

- RAN1#118

In RAN1#118, the following observations and agreement were agreed[5]:

Observation

Based on the results of DL coverage evaluation at system level collected from different sources, it is observed that extending the default value of SSB periodicity (different from 20ms) in NTN with LEO600km satellite parameter sets where the beam footprint diameter is 50 km, is beneficial in terms of reduction of common control channel overhead, when targeting a full coverage of 1058 beam footprints:

- With Set 1-1 FR1 and Set 1-3 FR1, the common messages (SSB, SIB1) overhead is around 40% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead ratio could be reduced to less than 14% when 160ms SSB/SIB1 periodicity is used.

- With Set 1-2 FR1, the common message (SSB, SIB1) overhead is greater than 100% assuming 5 MHz BW when SSB/SIB1 periodicity of 20ms is in use, this overhead could be reduced to around 25.8% when 640ms SSB/SIB1 periodicity is used.

- Note: the overhead of SIB19 was included in some of the results

- Note: an observation when SSB/SIB1 periodicity is 320 ms will be discussed and added to the observation

Agreement

As part of the NTN DL coverage enhancements at both system level and link level, RAN1 to consider:

- Extending the periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

  - Default value[s] with extended periodicity assumed by NTN UE for initial access can be:

    - One [or more] values from the list {40ms, 80 ms, 160 ms, 320ms, 640ms}

- Potential enhancements for transmitting the DL common channels using a wider beam footprint, while DL/UL dedicated channels (incl. PRACH) may be transmitted using a narrower beam footprint

- Link-level enhancements for the following channels:

- PDCCH

- PDSCH with Msg 4

- PDSCH with SIB1/SIB19.

  - Note: link-level enhancements for PDSCH with SIB1/SIB19 may be applicable to other SIBs, without additional specification impact.

- Note: the above does not imply that all the channels above will be enhanced, but all of them should be considered based on this agreement

- RAN#105

In RAN#105, WID NR NTN was updated at the following points [6].

RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any)

RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements.

SSB channel enhancement other than SSB periodicity extension is not considered.

RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension.

The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation.

- RAN1#118-bis

In RAN1#118-bis, the following observations and agreement were agreed[7]:

Agreement

For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

- The maximum of the additional default value (apart from the existing 20ms value) is at least 160 ms.

  - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms

Agreement

Support PDCCH CSS Link level enhancement in Rel-19 for all CSS types except type 3.

- The following techniques are for further study:

  - PDCCH repetition, including:

    - Option 1: Intra-slot PDCCH repetition

    - Option 2: Inter-slot PDCCH repetition

  - CORESET length (i.e. number of OFDM symbols) extension

  - DCI format optimization (e.g. size reduction, etc)

- Note: the same technique is intended to apply to all search space types targeted for link level enhancements

- For the above techniques, at least the following aspects should be discussed for the relevant candidate techniques:

  - Configuration

  - Backward compatibility and UE behaviour of legacy UE

  - Linked Search Space

  - Blocking probability

  - DCI format size budget

  - Resource overhead

  - Impact on CORESET0

- Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

- FFS: whether to apply the selected solution to PDCCH CSS type3 and PDCCH USS

Agreement

For PDSCH with Msg4 Link level enhancement:0

- Continue studying PDSCH repetition

  - Further discuss the specification impact for at least the following:

    - Procedure and signaling

    - Repetition factor

  - Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

- RAN1#119

In RAN1#119, the following observations and agreement were agreed [8]:

Agreement

For PDSCH with Msg4 Link level enhancement:

- Support PDSCH repetition

  - FFS: signalling design including number of repetitions

  - FFS: impact on UE capability

- Note: the target coverage enhancement to bridge the gap with respect to single Msg4 transmission is 2.8 dB

- Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

Observation

Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan.

Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

Agreement

For link level enhancement of PDSCH with SIB1:

- Support PDSCH repetitions within 20 ms duration

  - The number of repetitions is fixed to 2 repetitions

  - Further discuss the specification impact for at least the following:

    - Procedure and signaling (enabling repetitions, associated time resource determination, etc.)

- Note 1: without the above PDSCH repetitions, the coverage gap is 2.2 dB to 4.6 dB depending on SIB1 size.

- Note 2: Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level.

- Note 3: the above is not related to multiple SIB1 transmissions across 20 ms periodicities of SSB, which may not be available when the SSB periodicity is 160 ms or larger (if supported) depending on the SSB and CORESET multiplexing pattern.

Agreement

For PDCCH CSS (except Type-3) link level enhancements, support only PDCCH repetition for NTN.

- FFS: intra-slot and/or inter-slot

- RAN#106

In RAN#106, WID for NTN for NR  Phase 3 was revised regarding link level enhancements and system level enhancements. [9]

For the part of downlink coverage enhancement, the objectives are:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4]

- Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth.

- Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements.

- Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint.

  - Link level enhancements are to be specified for the following channels:

    - PDCCH at least for CSS (except for Type-3) via PDCCH repetition

    - PDSCH with Msg4 via PDSCH repetition

    - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

  - System-level enhancements are to be specified for the following:

    - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

      - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms

- Notes for this objective:

  - SSB channel enhancement other than SSB periodicity extension is not considered

    - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension

  - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation

  - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE

  - NGSO to be considered in priority: LEO Set-1 @ 600 km

Among the above description, the key modification are in the following part:

- Link level enhancements are to be specified for the following channels:

  - PDCCH at least for CSS (except for Type-3) via PDCCH repetition

  - PDSCH with Msg4 via PDSCH repetition

  - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration

- System-level enhancements are to be specified for the following:

  - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access.

    - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms

- Discussion

- Issue 1. Parameter alignment

In RAN1#116-bis, it was agreed that ‘The CNRs for the satellite payload parameters Set 1-1, Set 1-2 and Set 1-3 are equal to -1.9 dB, -1.9 dB and -9.9 dB respectively’. Then in RAN1#118-bis, it was agreed to ‘Focus on coverage enhancement for set 1-3 with a target CNR of -8 dB for NR NTN DL coverage enhancements at link level’.

To make the parameters set 1-3 align with the new target CNR, at least one of the parameters ‘Total DL power level’ and ‘Total number of simultaneously active beams’ should be adjusted.

One option is to keep ‘Total DL power level’ unchanged, and decrease the ‘Total number of simultaneously active beams’ from 106 to 68, so that ‘EIRP per Satellite beam’, ‘Satellite EIRP density /beam’ can increase by 1.9 dB and CNR can reach -8 dB.

Another option is to keep ‘Total number of simultaneously active beams’ unchanged, and increase the ‘Total DL power level’ from 23.24 dBW to 25.14 dBW, so that ‘EIRP per Satellite beam’, ‘Satellite EIRP density /beam’ can increase by 1.9 dB and CNR can reach -8 dB.

The refinement of parameter set 1-3 can be summarized in the table below.

Table 1. Refinement of payload parameter set 1-3

|  | LEO600km Set 1-3 FR1 (before refinement) | LEO600km Set 1-3 FR1 (refined, option1) | LEO600km Set 1-3 FR1 (refined, option2) |
| --- | --- | --- | --- |
| Maximum Bandwidth per beam | 5 MHz | 5 MHz | 5 MHz |
| SCS | 15 kHz | 15 kHz | 15 kHz |
| Beam size(Note 1) | 50km | 50km | 50km |
| Satellite EIRP density /beam (dBW/MHz) | 26 | 27.9 | 27.9 |
| Payload Total DL power level (dBW) | 23.24 | 23.24 | 25.14 |
| Aggregated EIRP (Total) (dBW) | 53.24 | 53.24 | 55.14 |
| Satellite Tx max Gain | 30 dBi | 30 dBi | 30 dBi |
| Maximum EIRP per Satellite beam (dBW) | 33 | 34.9 | 34.9 |
| Total number of beam footprints | 1058 | 1058 | 1058 |
| Total number of simultaneously active beams | 106 | 68 | 106 |
| % simultaneously active beams | 10.02% | 6.43% | 10.02% |
| Note: Target CNR on earth | -9.9 dB | -8 dB | -8 dB |

Observation 1: Given that RAN1 has agreed on a CNR target of -8 dB for satellite payload parameter set 1-3, which is 1.9 dB higher than originally specified, the parameter set 1-3 should be refined to maintain consistency with this new target.

Proposal 1: To align the satellite payload parameter set 1-3 with the target CNR of -8 dB, consider the following options:

- Option 1: Decrease the ‘Total number of simultaneously active beams’ from 106 to 68. This adjustment will increase the ‘EIRP per Satellite beam’ and ‘Satellite EIRP density per beam’ by 1.9 dB, thereby achieving the desired CNR of -8 dB.

- Option 2: Increase the ‘Total DL power level’ from 23.24 dBW to 25.14 dBW. This change will also result in an increase of the ‘EIRP per Satellite beam’ and ‘Satellite EIRP density per beam’ by 1.9 dB, leading to the desired CNR of -8 dB.

- Issue 2. Number of SSB index

In RAN#106, it was agreed to support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. The maximum of the additional default value, apart from the existing 20ms value, is 160 ms.

Based on this agreement, it is expected that the extended periodicity will increase the maximum number of beam footprints that can be covered with SSBs by an active beam through beam hopping within an SSB periodicity.

However, it is important to note that the maximum number of beam footprints that can be covered with SSBs in a cell (i.e., the number of candidate SSBs) is determined by the specification of SSB cases, rather than the SSB periodicity. In other words, extending the SSB periodicity alone does not increase the maximum number of SSBs that can be transmitted by the gNB in a cell.

Therefore, It is necessary for RAN1 to further discuss and define the mechanism for extending the SSB coverage within a cell.

Observation 2: The maximum number of beam footprints that can be covered with SSBs in a cell is determined by the specification of SSB cases, rather than the SSB periodicity. This means that extending the SSB periodicity alone will not increase the number of candidate SSBs that can be transmitted by the gNB within a cell.

Proposal 2: It is necessary for RAN1 to discuss and define mechanisms to effectively increase SSB coverage. This discussion should focus on strategies beyond simply extending the SSB periodicity, such as optimizing beam hopping patterns, improving beam management techniques, or enhancing the specification of SSB cases.

In our previous proposals for RAN1#118-bis and RAN1#119, we introduced a mechanism involving beam hopping and SSB index reusing among multiple cells to increase the SSB coverage ratio[10][11].  The approach can be illustrated with the following example:

Example: SSB periodicity is 320 ms, divided into 16 segments of 20 ms each. For the first 20 ms segment, SSB indices 0, 1, 2, and 3 are used for one cell. For the next 20 ms segment, the same SSB indices are reused for another cell, and so on.

For a UE in one of these cells, the SSB can be detected by assuming an SSB periodicity to be 160 ms during initial access. This method allows for efficient reuse of SSB index across multiple cells, thereby increasing the overall SSB coverage ratio.

Advantages: Transparency to UE. This method is transparent to the UE, meaning it does not require any change to the UE specification. The UE can detect SSBs using the assumed periodicity without additional complexity.

Disadvantages: Limited SSB index per cell. The number SSB indices available within a single cell remains limited. This limitation necessitates dividing the entire satellite coverage into many cells with asynchronous frame timing, which can complicate network management and synchronization.

Given these advantages and disadvantages, it remains an open issue for RAN1 to decide whether or not to expand the number of candidate SSBs in a single cell. Expanding the number of candidate SSBs would likely require redefining the SSB cases for NTN. This decision involves balancing the need for enhanced coverage against the potential complexity and specification changes required.

Observation 3: To enhance the satellite coverage ratio for SSB, it is feasible to reuse SSB index among multiple cells under the same satellite coverage. Alternatively, redefining the SSB cases for NTN might be necessary to accommodate more SSBs within a single cell, thereby increasing the number of beam footprints that can be supported.

Proposal 3: We propose two approaches to enhance SSB coverage:

- Redefine SSB cases for NTN to support more beam footprints within a single cell.

- Implement efficient reuse of the SSB index across multiple cells under the same satellite coverage.

- Issue 3. Link level enhancements

In RAN1#119 and RAN#106, it has been agreed that link level enhancements are to be specified for the following channels:

- PDCCH at least for CSS (except for Type-3) via PDCCH repetition.

- PDSCH with Msg4 via PDSCH repetition.

- PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration.

Additionally, it was observed or agreed that the coverage gaps are as follows:

- 2.0dB for PDCCH (40bit).

- 2.8 dB for Msg4 (1040bit).

- 2.2 dB to 4.6 dB for SIB1 depending on SIB1 size.

Observation 4: The coverage gaps identified are: 2.0 dB for PDCCH (40bit), 2.8 dB for Msg4 (1040bit), and 2.2 dB to 4.6 dB for SIB1 depending on SIB1 size. Given these coverage gaps, it is reasonable to specify the number of repetitions to be 2 for all three channels (PDCCH, PDSCH for Msg4, and PDSCH for SIB1) to bridge their coverage gaps for satellite parameter set 1-3.

Proposal 4: RAN1 should further specify the repetition number of PDCCH for:

- PDCCH for CSS (except for Type-3) to be 2.

- PDSCH for Msg4 to be 2

These specifications will help bridge the coverage gaps in the case of satellite parameter set 1-3.

Notably, the repetition number for PDSCH for SIB1 has already been agreed upon.

To further simplify the signaling process and optimize resource usage, considering that the repetition numbers are the same for different parameter sets (e.g., 2 for Set 1-3 and 1 for Sets 1-1/1-2), a single common indication can be used for multiple channels. This approach not only reduces signaling overhead but also ensures efficient system operation while meeting coverage requirements.

Observation 5: Given that the repetition number for CSS, Msg4 and SIB1 is expected to be the same value, (i.e., 2 for parameter set 1-3 and 1 for parameter sets 1-1/1-2), a single common indication can be used for all these channels. This approach will greatly simplify the indication of the repetition factor and save signaling resources.

Proposal 5:  Use a single common signaling indication for the repetition factor of:

- PDCCH for CSS (except for Type-3)

- PDSCH for Msg4

- PDSCH for SIB1

By adopting this common indication, RAN1 can streamline the signaling process and optimize resource usage, making the system more efficient without compromising coverage requirements.

3. Conclusions

Observation 1: Given that RAN1 has agreed on a CNR target of -8 dB for satellite payload parameter set 1-3, which is 1.9 dB higher than originally specified, the parameter set 1-3 should be refined to maintain consistency with this new target.

Proposal 1: To align the satellite payload parameter set 1-3 with the target CNR of -8 dB, consider the following options:

- Option 1: Decrease the ‘Total number of simultaneously active beams’ from 106 to 68. This adjustment will increase the ‘EIRP per Satellite beam’ and ‘Satellite EIRP density per beam’ by 1.9 dB, thereby achieving the desired CNR of -8 dB.

- Option 2: Increase the ‘Total DL power level’ from 23.24 dBW to 25.14 dBW. This change will also result in an increase of the ‘EIRP per Satellite beam’ and ‘Satellite EIRP density per beam’ by 1.9 dB, leading to the desired CNR of -8 dB.

Observation 2: The maximum number of beam footprints that can be covered with SSBs in a cell is determined by the specification of SSB cases, rather than the SSB periodicity. This means that extending the SSB periodicity alone will not increase the number of candidate SSBs that can be transmitted by the gNB within a cell.

Proposal 2: It is necessary for RAN1 to discuss and define mechanisms to effectively increase SSB coverage. This discussion should focus on strategies beyond simply extending the SSB periodicity, such as optimizing beam hopping patterns, improving beam management techniques, or enhancing the specification of SSB cases.

Observation 3: To enhance the satellite coverage ratio for SSB, it is feasible to reuse SSB index among multiple cells under the same satellite coverage. Alternatively, redefining the SSB cases for NTN might be necessary to accommodate more SSBs within a single cell, thereby increasing the number of beam footprints that can be supported.

Proposal 3: We propose two approaches to enhance SSB coverage:

- Redefine SSB cases for NTN to support more beam footprints within a single cell.

- Implement efficient reuse of the SSB index across multiple cells under the same satellite coverage.

Observation 4: The coverage gaps identified are: 2.0 dB for PDCCH (40bit), 2.8 dB for Msg4 (1040bit), and 2.2 dB to 4.6 dB for SIB1 depending on SIB1 size. Given these coverage gaps, it is reasonable to specify the number of repetitions to be 2 for all three channels (PDCCH, PDSCH for Msg4, and PDSCH for SIB1) to bridge their coverage gaps for satellite parameter set 1-3.

Proposal 4: RAN1 should further specify the repetition number of PDCCH for:

- PDCCH for CSS (except for Type-3) to be 2.

- PDSCH for Msg4 to be 2

These specifications will help bridge the coverage gaps in the case of satellite parameter set 1-3.

Notably, the repetition number for PDSCH for SIB1 has already been agreed upon.

Observation 5: Given that the repetition number for CSS, Msg4 and SIB1 is expected to be the same value, (i.e., 2 for parameter set 1-3 and 1 for parameter sets 1-1/1-2), a single common indication can be used for all these channels. This approach will greatly simplify the indication of the repetition factor and save signaling resources.

Proposal 5:  Use a single common signaling indication for the repetition factor of:

- PDCCH for CSS (except for Type-3)

- PDSCH for Msg4

- PDSCH for SIB1

By adopting this common indication, RAN1 can streamline the signaling process and optimize resource usage, making the system more efficient without compromising coverage requirements.

4. References

RP-234078 Rel-19 Moderator WID NR NTN phase 3

Chair notes RAN1#116 eom0

R1-2403666 Chair notes RAN1#116bis 9.11 R19 NTN v04 (eom)

Chair notes RAN1#117 v04 eom4

R1-2407482 Chair notes RAN1#118 (9.11 R19 NTN) v04 (eom)

RP-241789 Rel-19 WID NR NTN phase 3_v05

R1-2409226 Chair notes RAN1#118bis (9.11 R19 NTN) v06 (eom)

Chair notes RAN1#119 v20

RP-243300 Revised WID Non-Terrestrial Networks (NTN) for NR Phase 3

R1-2408310_Baicells_Discussion on downlink coverage enhancement for NR NTN

R1-2410415_Baicells_Discussion on downlink coverage enhancement for NR NTN