3GPP TSG RAN WG1 #120			R1-2500520

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.3.1

Source:	InterDigital, Inc.

Title:	Views on SBFD TX_RX_measurement procedures

Document for:	Discussion and Decision

# Introduction

In Rel-19, a work item on evolution of NR duplex operation for Sub-band full duplex (SBFD) was approved and updated [1], where the objectives identified for the work item are as follows:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of time location of SBFD subbands in SIB is not precluded - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2] - Indication of frequency domain location of SBFD subbands in SIB is not precluded - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC-CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2] - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon - UL transmissions within UL subband only - DL receptions within DL subband(s) only, except for CLI measurement by the UE outside of the DL subbands Note: When flexible symbols are used, it is not expected that any legacy Uplink symbol is converted to Downlink/SBFD symbols - Enhancement on resource allocation in frequency domain in SBFD symbols, including - resource allocation in frequency domain for PDSCH/CSI-RS across two DL subbands in SBFD symbols - handling of unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols, including - resource allocation in frequency domain for transmission or reception in SBFD symbols and non-SBFD symbols with different available frequency resource in different slots - CSI report of which associated CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol - Followings are assumed based on TR 38.858 - SBFD at the gNB side - Half duplex operation at the UE side - FR1 and FR2-1 - SBFD operation Option 4, i.e., both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs - Coexistence between non-SBFD aware UEs (including legacy UEs) and SBFD aware UEs in the cell operating SBFD at gNB side - SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies - One UL subband for SBFD operation in an SBFD symbol (excluding legacy UL symbol/slot) within a TDD carrier - Mechanisms for SBFD operation shall also consider the adjacent channel coexistence between two operators - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]: - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4] - Note: RAN3 will not specify enhancements to network signalling to support inter-operator coordination for CLI handling - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

Related agreements made in this agenda from RAN1#119 meeting [2] were outlined:

Agreement For a physical channel/signal occasion mapped to SBFD and non-SBFD symbols within a slot, an SBFD aware UE does not transmit or receive the physical channel/signal within the slot, except - PRACH transmission in a valid RO across SBFD symbols and non-SBFD symbols is subject to the agreements in AI 9.3.2. - For PUSCH repetition type B, down-select from the following options. - Option 1: A nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols. - Option 2: A nominal repetition is segmented into actual repetitions as in legacy operation. An actual repetition mapped to both SBFD and non-SBFD symbols is dropped. Agreement Confirm the following working assumption made in RAN1#118. Working Assumption For cell-specific configuration of frequency locations of SBFD UL subband, for each SCS configuration in SCS-SpecificCarrierList for UL, starting RB and bandwidth of SBFD UL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. For cell-specific configuration of frequency locations of SBFD DL subband(s), for each SCS configuration in SCS-SpecificCarrierList for DL, starting RB and bandwidth of each SBFD DL subband are indicated by a RIV-based indication as defined in 38.214 setting NBWPsize=275. - One or two SBFD DL subbands can be configured Agreement For configuration 1 and frequency resource allocation Type 0, if the valid symbol type for SP-CSI on PUSCH is SBFD symbols, - When an assigned RBG overlaps with the subband boundary, only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. - SBFD aware UE does not expect to be assigned with a RBG for PUSCH which is fully outside UL usable PRBs. Agreement Update the following agreement made in RAN1#118 meeting: Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) with Configuration 1, - For PUSCH repetition type A with available slot counting, A-SRS with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type. - For CG PUSCH with neither TBoMS nor PUSCH repetition type A with available slot counting,and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped. Agreement Update the following agreement made in RAN1#118bis meeting in cyan: Agreement For PUSCH intra-slot frequency hopping in SBFD symbols, the starting RB in each hop is given by: RBstart=RBstart, i=0RBUL SB start+(RBstart-RBUL SB start+RBoffset)modNUL SBsize, i=1 For PUSCH inter-slot frequency hopping in SBFD symbols and when pusch-DMRS-Bundling is not enabled, or for inter-slot frequency hopping for a PUSCH in SBFD symbols scheduled by RAR UL grant or DCI format 0_0 with CRC scrambled by TC-RNTI, the starting RB during slot nsμ is given by: RBstart(nsμ)=RBstart, nsμmod2=0RBUL SB start+(RBstart-RBUL SB start+RBoffset)modNUL SBsize, nsμmod2=1 Where - RBUL SB start is the starting PRB index of UL usable PRBs with reference to the start of UL active BWP. - NUL SBsize is the number of UL usable PRBs. - RBstart is the starting PRB index of the first PUSCH hop with reference to the start of UL active BWP. For PUSCH transmissions with Configuration 2, RBstart is the starting PRB index with reference to the start of UL active BWP after applying RB offset between non-SBFD symbols and SBFD symbols. - RB offset is the frequency hopping offset for PUSCH in SBFD symbols - Note: Definition of nsμ is unchanged from existing specifications Agreement For dynamically scheduled PDSCH repetitions with Configuration 2, for TBS determination. - Option 1: - If the first repetition occasion indicated by scheduling DCI is in SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs within DL usable PRBs. - If the first repetition occasion indicated by scheduling DCI is in non-SBFD symbols, the number of PRBs for TBS determination is based on assigned PRBs. Conclusion There is no RAN1 consensus to support mTRP scenario for SBFD in Rel-19 Agreement The maximum number of power control loops is the same as legacy (i.e. no more than 2) for SBFD operation in Rel-19. Agreement For a serving cell configured with SBFD subband time and frequency location, for an SBFD aware UE, SFI in DCI format 2_0 is only applicable to non-SBFD symbols but not applicable to SBFD symbols - Note: no additional specification efforts are expected to support joint operation of SBFD and dynamic SFI. Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, - For type 2 CG PUSCH, SPS PDSCH, down-select from the following options: - The valid symbol type for type 2 CG PUSCH is determined based on the symbol type of the first CG PUSCH associated with activation DCI. - The valid symbol type for SPS PDSCH is determined based on the symbol type of the first SPS PDSCH associated with activation DCI. Agreement At least in case when PUSCH frequency hopping is not enabled, for a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols (i.e. Configuration 2), for determining starting PRB for PUSCH transmissions in SBFD symbols, - Equation 1-C2: RBstartSBFD=RBstartUL SB+RBstartnon-SBFD+RBoffsetSBFDmodNUL SBsize - If RBoffsetSBFD is not configured, it is zero Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable. Agreement It is clarified that ‘the first transmission/reception’ in the following agreement in RAN1#118 refers to the first transmission/reception occasion indicated by scheduling DCI. - It is not expected that the first transmission/reception occasion indicated by scheduling DCI is mapped to both SBFD and non-SBFD symbols. Agreement For Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only, the valid symbol type is determined as following. <Unrelated part omitted> - For dynamically scheduled transmissions/receptions, the valid symbol type is determined based on the symbol type of the first transmission/reception. <Unrelated part omitted>

In this contribution, we discuss issues on SBFD Tx, Rx, measurement procedures, including separate SRS configurations across different symbol types, slot collision due to timing advance for one UE perspective, SBFD operation in SSB symbols, and extending SBFD beyond Rel-19 for flexibility and scalability.

# Discussion on SBFD TX, RX, measurement procedures

NR supports dynamic/flexible time division duplex (TDD) based on a slot format indicator (SFI) that can be indicated to a group of UEs by a group-common (GC) DCI (format 2_0). In addition, semi-static configurations via tdd-UL-DL-config-common/dedicated can be configured, where the transmission pattern for each slot/symbol can be configured as either of ‘D’ as downlink, ‘U’ as uplink, and ‘F’ as flexible.

Up to NR Rel-18, most practical assumptions for duplexing are half duplex (HD) for both gNB and UE. In Rel-19, enhancements to support full duplex (FD) at least for gNB were proposed and endorsed as the work item, see Figure 1. Moreover, subband non-overlapping FD (SBFD), as illustrated in Figure 2, has been identified as a promising approach, since it offers greatly reduced FD implementation complexity in terms of cancelling self-interference (SI) and mitigating cross-link interference (CLI), at least, at the gNB side.

Figure 1. Illustration on NR TDD framework based on FD-gNB and HD-UEs in a cell

Figure 2. Illustration on SBFD configurations including DL/UL subbands

## On separate SRS configurations

In RAN1#119 meeting, it was agreed to support separate SRS configurations for SBFD and non-SBFD symbols, by taking the set-level separation (Option 1) with related behaviors in details as follows.

Agreement Red text is working assumption. For separate SRS configurations for SBFD and non-SBFD symbols, support Option 1. Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage. - SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols. - FFS: whether there is impact on frequency hopping counter - SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols. - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped. - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission. - FFS impact on availableSlotOffsetList - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols. - FFS: whether there is impact on frequency hopping counter - The number of SRS resources in a SRS-ResourceSet for SBFD symbols is the same as the number of SRS resources in a SRS-ResourceSet for non-SBFD symbols for the same usage. - For SRS-ResourceSets with usage set to 'nonCodebook', only one SRS port for each SRS resource is configured as legacy. - For SRS-ResourceSets with usage set to 'Codebook', - Except when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2', the numbers of SRS ports are the same for all the SRS resources in the two SRS-ResourceSets - FFS when higher layer parameter ul-FullPowerTransmission is set to 'fullpowerMode2' - Maintain single TRP behaviour with respect to DCI fields and RRC parameters in rrc-ConfiguredUplinkGrant - SRS resource set indicator (SRSI) field is absent from DCI. srs-ResourceSetId is absent in rrc-ConfiguredUplinkGrant. - If txConfig is set to 'codebook', second precoding information (TPMI) field is absent from DCI and precodingAndNumberOfLayers2 is absent from rrc-ConfiguredUplinkGrant. - Second SRS resource indicator (SRI) field is absent from DCI - For DG PUSCH without repetition within a slot, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as that of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as that of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 1, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission, where the SRS resource prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission - For DG PUSCH with repetition/multi-PUSCH scheduled by a single DCI/TBoMS/Type 2 CG PUSCH with Configuration 2, the SRS resource indicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. For PUSCH transmissions in non-SBFD symbols, the indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols, where the SRS resource is prior to the PDCCH carrying the SRI. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, the precoding information and number of layers (TPMI) field in DCI corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If only a single SRS resource is configured in both SRS resource sets, the SRI field is absent from DCI. In this case “SRS resource identified by the SRI” in the above bullets corresponds to the single SRS resource in each SRS resource set. - srs-ResourceIndicator2 is absent from rrc-ConfiguredUplinkGrant. - For Type 1 CG PUSCH with Configuration 1, srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in the same symbol type as the valid symbol type of PUSCH transmission. - If txConfig is set to 'codebook', precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to the same symbol type as the valid symbol type of PUSCH transmission. - For Type 1 CG PUSCH with Configuration 2, srs-ResourceIndicator is applicable to both SBFD and non-SBFD SRS-ResourceSets. For PUSCH transmissions in SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in SBFD symbols. For PUSCH transmissions in non-SBFD symbols, the srs-ResourceIndicator is associated with the most recent transmission of SRS resource identified by the SRI in non-SBFD symbols. - If txConfig is set to 'codebook', for PUSCH transmission in SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to SBFD symbols. For PUSCH transmission in non-SBFD symbols, precodingAndNumberOfLayers corresponds to the SRS resource identified by the SRI in the SRS-ResourceSet corresponding to non-SBFD symbols. - If txConfig is set to 'nonCodebook', each SRS resource set is associated with an associated CSI-RS. - Applicable to usage set to 'codebook' or 'nonCodebook' - FFS usage set to 'beamManagement' - Not applicable to usage set to 'antennaSwitching’

Option 1, as agreed, intends to separate the whole considered SRS resource set for only one symbol type, i.e., either an SBFD symbol type or a non-SBFD symbol type, which can simplify the specification and minimize any unexpected issue in relation with the legacy behaviors in UL transmission cases.

Regarding the applicable usages, the agreements are at least applicable to usage set to 'codebook' or 'nonCodebook', as these are the supported UL transmission schemes, and not applicable to usage set to 'antennaSwitching'. Regarding the FFS usage when set to 'beamManagement', it can be decided equally as the antenna switching case, as the UE can transmit SRSs (by beam sweeping) for beam management regardless of a symbol type of a transmission occasion.

Regarding the red texts, these are aligned descriptions based on the existing behaviors, and the working assumptions can be confirmed.

Observation 1. Regarding the FFS usage when set to 'beamManagement', it can be decided equally as the antenna switching case, as the UE can transmit SRSs (by beam sweeping) for beam management regardless of a symbol type of a transmission occasion.

Proposal 1. Clarify the agreed separate SRS configurations for SBFD and non-SBFD symbols are not applicable to usage set to 'beamManagement'.

Observation 2. Regarding the red texts as part of the agreements on separate SRS configurations for SBFD and non-SBFD symbols, these are aligned descriptions based on the existing behaviors on UL transmission schemes.

Proposal 2. Confirm the working assumption on the red texts as part of the agreements on separate SRS configurations for SBFD and non-SBFD symbols.

## Slot collision due to timing advance for one UE perspective

In conventional TDD systems, the UL and DL slots are considered separately in time domain. Therefore, subsequent slots are split in DL-only, UL-only, and flexible slots. The symbols in flexible slots can be scheduled to be used as DL or UL based on received configurations. The flexible symbols can also be used as guard period for timing alignment requirements (e.g., DL/UL switching, UL timing advance (TA)).

In SBFD schemes with one or more subbands allocated for UL transmission in DL slots, a non-zero timing advance or switching time in SBFD could result in slot collision cases. As an example, shown in Figure 3, where the UL signals in UL SBs and DL signals in DL SBs/slots are collided by each other due to timing advance. Also, another example on slot-boundary collision case is shown in Figure 4 that is affecting the SBFD slots due to required switching time for UE between DL and UL that is also a factor for the timing alignment requirements.

For example, especially for a semi-statically scheduled resource (e.g., configured grant PUSCH), every time when UE receives a TA command which affects all related UL transmission including the CG-PUSCH, the UE has to check whether the adjusted/accumulated TA and the switching time requirement result in overlap with DL symbol in a prior DL slot, if scheduled, for SBFD operation. And if any slot-boundary collision happens, the UE does not know which selected one of a DL reception and a UL transmission the UE should perform as scheduled and how to perform the other scheduled one that is currently overlapped in symbols, since the UE performs either DL only or UL only on a given set of symbols as a half-duplex UE. This does not happen in legacy TDD system as there is the flexible slot for this reason at least, while in SBFD operation, it is always possible to face a back-to-back scheduling between DL and UL for one UE perspective.

Figure 3. Slot collision issue due to timing advance for one UE perspective

Figure 4. Slot collision issue due to switching time between DL and UL

While the gNB might address the required timing advance/switching time based on resource allocations and scheduling (e.g., as part of gNB implementation), this requires significantly increased scheduling complexity at the gNB side and also increases the signaling overhead, especially for taking such semi-statically scheduled resources for both DL and UL into account. In another aspect, in case a scheduled (dynamic) grant for UL or DL with high priority is received and if the timing alignment requirement including required Rx/Tx or Tx/Rx switching time is not met, e.g., resulting in some DL/UL overlapped symbols being faced at the UE, this could again result in a severe performance degradation or dropping corresponding slots.

Observation 3. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in slot collision issues for one UE perspective and possible dropping of respective slots, especially for back-to-back scheduling cases between DL and UL.

In case a UL transmission in SBFD UL subband overrides the symbols from a previous slot (e.g., a legacy ‘D’ slot) due to the non-zero timing advance, the DL scheduled in those symbols results in the slot collision case and may experience inter-subband CLI as well as inter-slot interference. The slot collision issue due to non-zero TA becomes critical, as SBFD framework in Rel-19 has no ‘Special’ slot in-between the legacy D slot and the SBFD slot back-to-back. The slot collision case with the inter-slot interference is more severe for the legacy UEs scheduled in the previous slot for DL reception, as such a timing misalignment results in intra-subband (directly colliding) and inter-slot interference on the symbols close to the end of the DL slot. The legacy UEs may be scheduled to receive SSB, CORESET#0, or DMRS in PDSCH in the symbols that are overridden by the misaligned UL transmission in the UL subband in the right next slot. As such, the performance of legacy UEs may be affected due to the non-zero TA.

Therefore, there should be a means for UE to detect if the allocated UL/DL timing alignment is accurate and enough. In case UE has detected such timing discrepancies, the UE could report the issues along with requesting for adjustment or at least should be able to determine which overlapped portion of either DL symbol(s) or UL symbol(s) can be dropped or punctured. For example, the UE could also determine to include the time required for the timing alignment inside respective UL/DL SB in SBFD slot, where the scheduled UL transmission on the SBFD slot can have a rate-matched or punctured symbol(s) in the front symbol positions of the UL transmission, or alternatively DL symbol(s) can be punctured, which should depend on gNB’s flexible configurations for such UE behaviours.

Observation 4. The slot collision issue due to non-zero TA from UL transmission in UL subband in SBFD configuration could increase the inter-slot and inter-subband interference including other UEs, specifically on the DL symbols scheduled in the previous slot.

Observation 5. The slot collision issue and the time misalignment due to non-zero TA for an UL transmission in UL subband in an SBFD configuration could affect the legacy UE that is configured to receive critical DL signals such as SSB, CORESET#0, or DMRS close to the end of a preceding DL slot.

Proposal 3. Support methods to handle the slot collision and time misalignment issues due to non-zero TA for a scheduled UL Tx in an SBFD slot, right after the legacy ‘D’ slot back-to-back, e.g., including which channel direction the UE can prioritize and UE reporting on a minimum PUSCH starting symbol to aid gNB’s scheduling.

## SBFD operation in SSB symbols

During the study phase, the support for the SBFD operation in SSB symbols was discussed. Based on the discussions, it was clear that not supporting the SBFD operation in SSB symbols may degrade the performance and objectives of the SBFD operation.

Contiguous UL subband resources in time domain

Considering the UL repetition, the contiguous UL subband resources should be provided in time domain. However, in case the SBFD operation is not allowed on SSB symbols, this requires switching to DL-only symbols each time there is an SSB transmission. As such, switching between SBFD symbols and DL-only symbols will affect the resources for UL repetition in SBFD symbols and will degrade the coverage enhancement purposes.

Latency in UL transmission

In addition to coverage enhancement, one of the objectives in SBFD operation is to reduce latency in UL transmissions. However, in case the SSB symbols are not transmitted in SBFD resources, this will change the TDD model back to DDDSU each time there is an SSB burst transmission. On the other hand, considering SSB transmission in SBFD symbols may improve the latency even in initial access or cell reselection procedures due to availability of resources for sending PRACH preamble and random-access procedures.

Observation 6. In case the SBFD operation is not supported in SSB symbols, the SBFD objectives such as UL coverage enhancement and latency reduction are affected and degraded.

In RAN1 #117, following agreement was made:

Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

Based on this agreement, in case an SSB transmission overlaps with an SBFD symbol, the SBFD symbol is converted to a full DL slot. Alternatively, the UE expects to only receive DL within DL usable PRBs and no UL is allowed for the SBFD-aware UE in the corresponding SBFD symbol. One of the main reasons for not transmitting UL in the overlapping SBFD symbols is handling the collision between the UL transmission and SSB reception, as the potential CLI caused by UEs transmitting UL could degrade SSB reception for the other UEs.

The CLI due to UL transmission in UL subbands of SBFD symbols that overlap in time with SSB transmission could also be caused by UL transmission by SBFD-aware UEs in an adjacent cell with SBFD configuration in the same symbol (although the entire network has a common “DXXXU” pattern for all the cells for example). That is, while the SBFD-aware UEs in the serving cell are refrained from UL transmission in the SBFD symbol overlapping with SSB transmission, the UL transmission from SBFD-aware UEs in an adjacent cell could affect SSB reception performance. As an example, see Figure 5, where the UEs in both Cell 1 and Cell 2 are configured commonly with DXXXU SBFD configuration. In Figure 5, Cell1 is configured with SSB transmission in SBFD Slot2, Cell2 is configured with SSB transmission in SBFD Slot 3, and so the UEs in Cell2 are refrained from UL transmission in Slot 3. On the other hand, the UEs in Cell1 may perform UL transmissions in Slot 3, as there are no SSB transmission from their serving Cell1 in Slot3, which may cause CLI and degradation in SSB reception for UEs in Cell2.

Figure 5. Issue on neighbor-cell’s SSB slot to be considered for the ‘D’ slot fallback on UEs in serving-cell

Observation 7. The UL transmission from SBFD-aware UEs in an adjacent cell in an SBFD symbol that coincides with SSB transmission in a serving cell degrades the SSB reception performance for the UEs in the serving cell.

To mitigate the CLI and potential degradation in SSB reception in a first cell, caused by UL transmission from the SBFD-aware UEs in an adjacent second cell, the respective UEs can explicitly receive information on additional neighbor-cell(s) with their SSB transmission slot information to additionally apply the ‘D’ slot fallback, or could verify some conditions before UL transmission in the SBFD symbols that overlap in time with SSBs from another cell. For example, in Figure 5, UE1 that is closer to the center of Cell1 and far from Cell2 can still transmit UL in the SBFD symbols that overlap in time with Slot3, where SSB is transmitted in Cell2. On the other hand, in Figure 5, UE2 that is far from the center of Cell1 and close to Cell2 cannot transmit UL in Slot3, where SSB is transmitted in Cell2.

Proposal 4. Support mechanisms for collision handling between coincidence of UL transmission in a serving-cell cell and SSB transmission in a neighbour cell, when information on the SSBs of the neighbour cell is configured.

## On extending SBFD beyond Rel-19 for flexibility and scalability

SBFD operation in Rel-19 is based on a strict assumption on semi-statically aligned time/frequency locations of DL/UL subbands in the network, mainly due to challenges of CLI handling [3]. However, in reality, traffic conditions are varying among cells in the network, and semi-statically fixed DL/UL resource partitioning may result in significantly reduced resource utilization efficiency. For example, even in a low-load scenario, the SBFD in Rel-19 does not support a dynamic fallback to a legacy downlink slot opportunistically which results in a DL performance degradation.

In this section, we provide an initial analysis on extending SBFD beyond Rel-19 for flexibility and scalability. For the system-level simulations we focus on an outdoor UMa scenario that utilizes a clustered dropping of UEs. The simulation assumptions are based on those provided in section 7 of [4]. We considered two different FD schemes:

- Scheme 1: Semi-static SBFD with 20% UL subband resource in X slot, with frame structure (XXXXU)

- Scheme 2: Flexible/Dynamic FD operation with adaptation where X slot in (XXXXU) may fallback to a full DL slot

We compare the relative DL and UL performance of the schemes based on mean UPT. Figure 6 provides the relative performance gain/loss of Scheme 2 relative to Scheme 1. From the figure we observe that at low and medium loads, Flexible/Dynamic FD (scheme 2) offers significant gains for mean DL UPT (~22%) and greater than 8% gain at high load. This is coupled with slight gains for UL performance for the flexible/dynamic FD, mainly due to faster DL packet completion rates by the flexible/dynamic FD, resulting in less inter-SB CLI. These results show that utilizing a flexible FD scheme which allows for a fallback to legacy downlink slot can significantly boost DL performance, without any loss in UL performance.

Figure 6: DL and UL mean-UPT gains for different traffic loads

Observation 8. Flexible adaptation (for e.g., dynamic fallback of SBFD slots) for SBFD can result in improved performance over semi-statically fixed DL/UL resource configuration.

Proposal 5. Further study on how to enhance SBFD performance beyond Rel-19 for improved flexibility and scalability, in consideration of dynamic and future-proof full duplex design approach.

# Conclusion

In this contribution, we discussed issues on SBFD Tx, Rx, measurement procedures, including separate SRS configurations across different symbol types, slot collision due to timing advance for one UE perspective, SBFD operation in SSB symbols, and extending SBFD beyond Rel-19 for flexibility and scalability. From the discussions, we made following observations and proposals:

[2.1	On separate SRS configurations]

Observation 1. Regarding the FFS usage when set to 'beamManagement', it can be decided equally as the antenna switching case, as the UE can transmit SRSs (by beam sweeping) for beam management regardless of a symbol type of a transmission occasion.

Proposal 1. Clarify the agreed separate SRS configurations for SBFD and non-SBFD symbols are not applicable to usage set to 'beamManagement'.

Observation 2. Regarding the red texts as part of the agreements on separate SRS configurations for SBFD and non-SBFD symbols, these are aligned descriptions based on the existing behaviors on UL transmission schemes.

Proposal 2. Confirm the working assumption on the red texts as part of the agreements on separate SRS configurations for SBFD and non-SBFD symbols.

[2.2	Slot collision due to timing advance for one UE perspective]

Observation 3. The issues in UL/DL timing alignment (between UL/DL SBs) in SBFD slots could result in slot collision issues for one UE perspective and possible dropping of respective slots, especially for back-to-back scheduling cases between DL and UL.

Observation 4. The slot collision issue due to non-zero TA from UL transmission in UL subband in SBFD configuration could increase the inter-slot and inter-subband interference including other UEs, specifically on the DL symbols scheduled in the previous slot.

Observation 5. The slot collision issue and the time misalignment due to non-zero TA for an UL transmission in UL subband in an SBFD configuration could affect the legacy UE that is configured to receive critical DL signals such as SSB, CORESET#0, or DMRS close to the end of a preceding DL slot.

Proposal 3. Support methods to handle the slot collision and time misalignment issues due to non-zero TA for a scheduled UL Tx in an SBFD slot, right after the legacy ‘D’ slot back-to-back, e.g., including which channel direction the UE can prioritize and UE reporting on a minimum PUSCH starting symbol to aid gNB’s scheduling.

[2.3	SBFD operation in SSB symbols]

Observation 6. In case the SBFD operation is not supported in SSB symbols, the SBFD objectives such as UL coverage enhancement and latency reduction are affected and degraded.

Observation 7. The UL transmission from SBFD-aware UEs in an adjacent cell in an SBFD symbol that coincides with SSB transmission in a serving cell degrades the SSB reception performance for the UEs in the serving cell.

Proposal 4. Support mechanisms for collision handling between coincidence of UL transmission in a serving-cell cell and SSB transmission in a neighbour cell, when information on the SSBs of the neighbour cell is configured.

[2.4	On extending SBFD beyond Rel-19 for flexibility and scalability]

Observation 8. Flexible adaptation (for e.g., dynamic fallback of SBFD slots) for SBFD can result in improved performance over semi-statically fixed DL/UL resource configuration.

Proposal 5. Further study on how to enhance SBFD performance beyond Rel-19 for improved flexibility and scalability, in consideration of dynamic and future-proof full duplex design approach.

# References

RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei.

“Report of 3GPP TSG RAN WG1 #119”, 3GPP RAN1 Meeting #119, November 2024.

TR 38.858, “Study on Evolution of NR Duplex Operation”, V18.0.0.

R1-2212993, “Final Summary on evaluation on NR duplex evolution”, Moderator (CMCC).

# Appendix – Simulation assumptions

Table 1. SLS simulation assumptions, based on section 7 of [4]

| Parameter | Deployment Scenario |
| --- | --- |
|  | Urban Macro (UMa) (from 38.913) |
| Layout | 21 cells with wraparound ISD: 500m |
| Channel Model | UMa (38.901) |
| UE Distribution | 80% indoor, 20% outdoor. 20 UEs per cell. UE clustering distribution with 2 UE clusters and 8 UEs per cluster. |
| UE Mobility | 3 Km/hr |
| Carrier frequency | 3.5 GHz |
| System bandwidth | 100 MHz |
| Subcarrier spacing | 30 kHz |
| BS height | 25 m |
| UE height | The UE height for indoor UEs is updated as following based on Table 6-1 in TR 36.873. 1.5m |
| Open-loop power control | Default: P_0 = -92 dBm, alpha=1.0 |
| BS/UE TX power | BS: 53 dBm, UE: 23 dBm |
| BS antenna configuration | SBFD: Separate antenna array for Tx/Rx Option 2 (# of antenna elements for SBFD is two times that of TDD): (M,N,P,Mg,Ng,Mp,Np,dV,dH)=(4, 4, 2, 1, 2, 4, 4, 0.8, 0.5). Separate Tx/Rx antenna array – 16 TxRUs for DL, 16 TxRUs for UL. |
| Slot structure | Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): SBFD Alt. 2 - Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. SBFD Subband configuration#1 with {DUD} pattern, one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. All cells use the same SBFD slot configuration Semi-static SBFD (Scheme 1): SBFD operation with 20% UL subband resource in ‘X’ slot with (XXXXU). Flexible/Dynamic SBFD (Scheme 2): Flexible/Dynamic SBFD operation where ‘X’ slot may fallback to full ‘DL’ slot. |
| gNB self-interference | 1 dB UL desense |
| CO-site inter sector inter SB CLI | Option 2: 93 dB (spatial isolation), 0 dB digital isolation |
| BS noise figure | 5 dB |
| UE noise figure | 9 dB |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| UE antenna configuration | (M,N,P,Mg,Ng,Mp,Np,dV,dH) = (1, 2, 2, 1, 1, 1, 2, 0.5, 0.5). 4 ports 4 Tx, 2 Rx |
| Channel estimation | Realistic |
| Transmission scheme | 16/32 Tx Type 1 Codebook |
| Scheduler | SU-MIMO (with PF) |
| Target BLER | 10% first transmission BLER |
| HARQ/repetition | 3 HARQ retransmission |
| Metric | DL/UL User Perceived Throughput |
| Traffic model | FTP3 (0.5MB for DL, 0.125 MB for UL) Traffic: λd, λu are number of packet arrivals per UE (each packet is 0.5MB) DL:UL traffic ratio 1:1 and 2:1 are considered Load scenario Low load Type-2 RU < 10%) Medium load Type-2 RU 20%-35% High load Type-2 RU >50% |