3GPP TSG RAN WG1 #120			R1-2500522

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.3.3

Source:	InterDigital, Inc.

Title:	Discussion on CLI handling for SBFD operation

Document for:	Discussion and Decision

# Introduction

In Rel-19, a work item on evolution of NR duplex operation for Sub-band full duplex (SBFD) was approved and updated [1], where the objectives identified for the work item for the CLI handling enhancements as follows:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Notes: Following are assumed for information exchange among gNBs - No normative behavior is implied for the gNB receiving the CLI-mitigation request. No need to further send a stop message for CLI mitigation. Detailed signaling can be discussed in RAN3 - The information exchanges are subject to RAN3’s assessment of feasibility and specification impact - Notes: Following are assumed for UL resource muting - No new DCI field / MAC CE is introduced for indication/determination of UL resource muting - No impact on data and control multiplexing as defined in section 6.2.7, TS 38.212 - UL resource muting does not apply for Msg A PUSCH and Msg 3 PUSCH - UL resource muting applies only for UEs in RRC_CONNECTED mode - UE assumes that the UL resource muting pattern does not overlap UL DMRS or PT-RS in the same symbol - Power boosting is assumed for REs in the symbol with UL resource muting. PUSCH transmit power does not change across symbols - No changes on TBS determination for PUSCH - Check impact on transmit signal quality/MPR requirement, if any, with RAN4 - This feature is subject to UE capability - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

In this contribution, we discuss enhancements on CLI handling schemes based on the revised WID [1] and the TR38.858 [2] for both UE-to-UE and gNB-to-gNB co-channel CLI handling.

# Discussions

NR supports dynamic/flexible time division duplex (TDD) based on a slot format indicator (SFI) that can be indicated to a group of UEs by a group-common (GC) DCI (format 2_0). In addition, semi-static configurations via tdd-UL-DL-config-common/dedicated can be configured, where the transmission pattern for each slot/symbol can be configured as either of ‘D’ as downlink, ‘U’ as uplink, and ‘F’ as flexible.

Up to NR Rel-18, most practical assumptions for duplexing are half duplex (HD) for both gNB and UE. In Rel-19, enhancements to support full duplex (FD) at least for gNB were proposed and endorsed as the work item, see Figure 1. Moreover, subband non-overlapping FD (SBFD), as illustrated in Figure 2, has been identified as a promising approach, since it offers greatly reduced FD implementation complexity in terms of cancelling self-interference (SI) and mitigating cross-link interference (CLI), at least, at the gNB side.

Figure 1. Illustration on NR TDD framework based on FD-gNB and HD-UEs in a cell

Figure 2. Illustration on SBFD configurations including DL/UL subbands

## L1-based UE-to-UE CLI measurement and reporting

Up to NR Rel-17, the UE-to-UE CLI measurements and reporting are based on long-term measurements and layer 3 filtering. As identified during Rel-18 NR-duplex study [2], Layer1(L1)/L2 based UE-to-UE CLI measurement can be optimized for short term interference measurement and low latency, which results in enhancing the CLI mitigation performance by providing accurate CLI measurement. Hence, the L1/L2 based UE-to-UE CLI measurement and reporting can facilitate gNB adjusting UE scheduling.

### Configuration of CLI measurement resources and reporting

In the RAN1 #119 meeting [5], the following agreements/conclusions were reached:

Conclusion For Method#1 (RSSI measurement within DL subband) with the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands), two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband is not supported. Agreement For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig. Agreement Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement. - Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis) Define CLI-RSSI-MeasurementResource with the following parameters: - CLI-RSSI measurement resource ID - Starting PRB index - Number of PRBs - Starting symbol of the CLI-RSSI resource within a slot - Number of symbols of the CLI-RSSI resource within a slot - Periodicity and slot offset for the CLI-RSSI resource - FFS: Other parameters Agreement (RAN1#118bis) For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

In the previous meeting, it has been agreed to support Method #3 (UE measures RSSI within UL subband) for CLI measurements and reporting. Also, it is indicated that the UE cannot be configured to perform simultaneous UL and DL CLI-RSSI measurement.

For the CLI-RSSI measurement resources, the agreements indicate that the configuration include the starting PRB and number of PRB to perform the measurement on, with a precision that for Method #1 with resource type #2 (One CLI-RSSI measurement resource across two DL subbands), the UE excludes the resources not within the DL usable PRBs. However, current parameters may be ambiguous for the cases where the CLI-RSSI resources are configured over the UL subbands, and the UE is not indicated whether to perform a measurement of CLI-RSSI over the UL subband (Method #3) or a CLI-RSSI measurement over DL subband(s) and excluding the non-usable DL PRBs (Method #1).

For clarification, one of the following alternatives need to be considered:

- Alt.1: implicit indication: if the CLI-RSSI resources are exclusively configured within UL usable PRBs, the UE considers the CLI-RSSI resource to be for Method #3: UE measures RSSI within UL subband; otherwise, the UE considers that the measurement is for Method #1 and excludes any non-usable DL PRBs from the measurement/reporting if any.

- Alt.2: explicit indication: A new parameter can be added within the CLI-RSSI-MeasurementResource or CLI-RSSI-MeasurementResourceSet to indicate the method (Method #1 or Method #3) and/or type of resource (resource type #1 or resource type #2). The UE then excludes the frequency resources outside the corresponding usable PRBs from the measurements.

Proposal 1. Support one of the following alternatives for the configuration of the CLI-RSSI measurement resources:

- Alt.1: UE assumes a Method #3 measurement configuration if the configured set of CLI-RSSI-MeasurementResource within a CLI-RSSI-MeasurementResourceSet indicate frequency resources exclusively within the configured UL subband; and Method #1 otherwise.

- Alt.2: A new parameter indicating the Method type is added in CLI-RSSI-MeasurementResource or CLI-RSSI-MeasurementResourceSet and the UE excludes any frequency resources not included within the corresponding usable PRBs.

In the RAN1 #119 [5] meeting, it was agreed that UE may report multiple values, at least for the strongest received interferences. Discussions and proposals were presented [3] to support reporting the least interfered CLI resources, yet without consensus:

Agreement [RAN1 #119] For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’, the following are supported - The number of reported CLI measurement resources M can be configured as {1, 2, 3, 4} - The M CLI measurement resource ID(s) are reported - The absolute (not differential) value of the most interfered (largest) measured L1-CLI-RSSI or L1-SRS-RSRP is reported - When the number of reported CLI measurement resources is larger than one, a differential reporting method is used taking the most interfered measured L1-CLI-RSSI or L1-SRS-RSRP as the reference - FFS: other reporting criteria, e.g., the least interfered measured L1-CLI-RSSI or L1-SRS-RSRP - FFS: whether/how to report differential value in case the largest measurement is out of range #### Proposal 2-11 [R1-2410790] #### For L1 based SRS-RSRP and CLI-RSSI reporting, UE can be configured to report the most X interfering or Y least interfering CLI measurement resources.

We think it’s beneficial that the UE can be configured with reporting most and least inference resources. If one focuses only on the CLI identification, the most interfered resources may be sufficient for resource management. However, reporting the least interfered CLI measurement resources may also be useful information for the network coupled with beam management, e.g., to enable the network to schedule UEs to associate beam pairs for UEs without CLI. See more details on beam aspects in section 2.1.4.

Proposal 2: For L1 based SRS-RSRP and CLI-RSSI reporting, support that the UE report the most X interfering or Y least interfering CLI measurement resources.

### Subband and edge-specific measurement and reporting

In RAN1 #119 [5], following conclusion was made on UE CLI measurement in DL sub-band(s):

Conclusion There is no RAN1 consensus on the following: - Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19.

The inter-subband CLI in SBFD may impose higher interference on subband-edge RBs within the SBFD DL subband and guard-bands, whereas the resources in the middle of the SBFD DL subband may not experience CLI as much. As such, measuring and averaging the CLI over the whole SBFD subband may result in down estimation of the CLI.

Observation 1. Inter-subband UE-to-UE CLI measurement in SBFD DL subbands based on measuring over configured RB resources and averaging may result in down-estimation, as the subband-edge RBs experience higher CLI compared to RBs in the middle of the subband.

In one proposal, to achieve an accurate inter-subband CLI measurement, the UE can measure a delta parameter that is based on measuring and calculating the difference between a first CLI-RSSI measured from the resources in the edge of the configured RBs in the SBFD DL subband or within in-between configured guard-bands and a second CLI-RSSI measured from the resources located in the middle of the configured RBs in the SBFD DL subband. As such, the difference between the CLI-RSSI measured in the edge or guard-bands and CLI-RSSI measured in the middle can be used as an indication of inter-subband CLI.

The indication of a significant difference of CLI-RSSI values for different parts of the measurement resource may be used by the network to consider a reconfiguration of the CLI-RSSI resource size and positions for a finer granularity and accuracy of the measurements.

Proposal 3. In UE-to-UE CLI-RSSI measurement techniques within active DL BWP, support measuring and reporting delta-CLI-RSSI based on differences in measured CLI-RSSI in subband-edge or guard-bands with measured CLI-RSSI in the middle of the DL subband.

### CLI measurements and reporting enhancements

#### Periodic and semi-persistent measurement and reporting

Currently, aperiodic reporting has been agreed to be supported, while periodic and semi-persistent reporting are not yet agreed.  In our view, UE CLI measurement resources and reporting shall support semi-persistent and periodic reporting, e.g., reusing the CSI framework as a baseline for activation/deactivation, without requiring a change in specification.

Although CLI may quickly change over time, periodic/semi-persistent measurement and reporting for CLI may be used by the network to order regular measurements by the UE and prepare its scheduling accordingly to enable CLI measurements, without increasing the overhead of dynamically signalling each CLI measurement and reporting. It is then up to network implementation to configure the periodic and semi-persistent reporting based on its requirements.

Proposal 4. For L1 based UE-to-UE CLI, support periodic and semi-persistent L1 CLI reporting.

#### Triggers for CLI measurements and reporting

In general, DL/UL resource assignment for a UE can be based on CSI/beam measurement and reporting procedures supported in NR. Based on the CSI/beam measurements performed on a configured DL RS (e.g., CSI-RS) resource, the UE can report one or more preferred beam/RS indexes along with corresponding quality metrics such as CQI, L1-RSRP, or L1-SINR. The reported contents can be subband-wise, depending on gNB’s configuration, and those can be used for gNB’s scheduling on the DL/UL resource assignment for the UE. The beam information for the DL/UL resource assignment can be based on an indicated transmission configuration indicator (TCI) for a DL/UL channel/signal. When the UE receives a grant (DCI), the UE can apply a spatial filter determined by the indicated beam information to perform the DL reception or the UL transmission. These general procedures are not based on any dynamic information related to CLI (e.g., UE-to-UE CLI) as the UE’s measurement and reporting is based on a configured RS resource which is relevant only with the gNB.

A CLI due to a signal transmitted by other (aggressor) UE can be present and severely degrade a reception performance of a DL signal scheduled by a grant at a victim UE, where the presence of the interference may not be known prior to the grant, e.g., in case when the aggressor UE is associated with a different serving-cell/TRP. Due to the CLI which is not captured in the general CSI/beam reporting, the victim UE may fail to receive the DL signal, which degrades the DL performance. The failure in receiving the DL signal can continue to happen, in case when such unexpected UE-to-UE CLIs exist for a duration of time.

Since such a UE-to-UE CLI can happen unexpectedly, it is beneficial to consider a conditional CLI handling behaviour based on monitoring the beams at the (victim) UE side, where the monitoring can be conducted per subband which can be configured to the victim UE as performing CLI measurement and reporting. Then, the victim UE can report the CLI monitoring results, which can be used at the gNB to determine whether the assigned DL/UL resource to the victim UE should be changed to a different subband to avoid the CLI on a subband. The conditional behavior on CLI monitoring done at the victim UE could greatly simplify the network and UE implementation for CLI handling, as this procedure is transparent to the aggressor UE side. The condition can at least include a case when the victim UE detects a PDSCH reception failure, e.g., along with NACK transmission in general.

Observation 2. Since a general CSI/beam reporting in NR is not based on dynamic CLI-related information, a victim UE may unpredictably experience DL performance degradation if a UE-to-UE CLI occurs especially when an aggressor UE is served by a different serving gNB/TRP.

Proposal 5. Support a conditional CLI handling behaviour based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a CLI measurement/reporting for a subband switching to avoid the CLI.

### Beam aspects for CLI-based measurement and reporting

In RAN1 #118bis[4] and #119 meetings, the following agreement was made on beams:

Agreement [RAN1 #118bis] For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList: - In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic - If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource: - If unifiedTCI-StateType is not configured - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET” - If unifiedTCI-StateType is configured - “Indicated” DL only/joint TCI state as specified in TS 38.214 - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources Agreement [RAN1 #119] For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter - For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD. - If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE - TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none. If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

It has been agreed that the UE may be configured with different TCI states for different CLI measurement resources, enabling directional measurements, at least for aperiodic reporting, although it is limited to a common TCI state for all the CLI resources of a given CLI measurement resource set. Beam management can still be performed using multiple CLI measurement resource sets in different measurement and reporting configurations. Also, the restriction is on the receiver beam, while the (victim) UE may still perform measurement of multiple CLI measurement resources, which can be transmitted with different beams from another (aggressor) UE.

In beam selection, the UE selects the best beam for Rx/Tx from gNB based on measured RSRP, SINR, etc. In SBFD systems, the beam management techniques need further enhancement using the direction CLI measurements. That is, a beam that was selected as the best beam direction between a victim UE and the gNB may not be the preferred beam in case a directional CLI is received at the victim UE from an aggressor UE, see Figure 3. As such, a beam that was selected by UE to be the best beam, for example based on wideband RSRP measurements, may not be the best beam in one or more of the subbands due to potential CLI in the corresponding subbands.

In NR, joint beam management has proven to be an effective procedure for finding the best beam pairs between the UE and gNB. For joint beam management in SBFD systems, two main aspects can be considered.  One aspect would be to detect and identify the aggressor UEs and the beam directions from the aggressor UEs that are causing CLI. As such, instead of blind CLI measurement and reporting, the victim UE could report the unfavorable beam-pairs for which the aggressor UEs are causing CLI.

In this way, the gNB could prevent the CLI between the victim and aggressor UEs by spatial coordination methods. That is, the gNB could only avoid the beams at the aggressor UEs that are causing the CLI for the victim UE. In other words, if a potential aggressor UE is not actually causing CLI to a victim UE, the gNB could know that based on the report received from the victim UE, and the gNB could allow the non-interfering potential aggressor UE to continue its operation. Similarly, for an actual aggressor UE that is causing CLI to the victim UE, the gNB could still schedule the aggressor UE but just avoiding the beam directions that are causing the CLI to the victim UE. This could result in more enhanced and flexible scheduling and resource management at the gNB-side despite the CLI.

Another aspect is to enable the victim UE to measure, detect, and report the optimal beam direction for the UE-to-gNB communication, based on the direction of the beams at the aggressor UE. In contrast to UE-gNB beam selection that is based on highest RSRP or SINR, the beam selection between the victim UE-aggressor UE can be based on the lowest measured SRS-RSRP or CLI-RSSI. This can be achieved by performing a two-step beam sweeping procedure as part of spatial coordination procedure. As such, the first beam sweeping procedure could be between the victim UE and the aggressor UE, and the second beam sweeping procedure could be between the victim UE and the gNB.

Figure 3. Directional CLI from aggressor UE in UL received at the victim UE in DL

The proposed aspects can be summarized in below two steps:

Step 1. Best beam selection based on wideband measurements via beam sweeping/selection between victim UE and the gNB. As the first step, the victim and the gNB can perform a beam sweeping procedure to find the best beam to Rx/Tx from/to gNB. The beam selection between the UE and gNB can be based on wideband RSRP measurements.

Step 2: Best beam selection based on measurements via beam sweeping between victim and aggressor UEs: As the second step, the victim and the aggressor UEs can perform a beam sweeping procedure to find the best and the worst beam pairs, where the pairs include pairing of {UE-gNB beam index, SRI from aggressor UE}. The SRI may be indicated to the UE as part of SRS measurement configurations. This may be called an opportunistic beam pairing, as the victim UE finds its best beam to communicate with the gNB based on the beams causing the least CLI from the aggressor UE. As the result of this step, the gNB can choose the best beam to communicate with the UE based on the reported best beam pair, even in the existence of the directional CLI. On the other hand, by reporting the results of this step to the gNB, the gNB can mitigate the CLI by preventing the victim UE to use the UE-gNB beam index and preventing the aggressor UEs to use the worst or unfavourable beams QCL-ed with the indicated SRIs in the reported worst beam pairs causing the highest CLI. The beam pairs determination and reporting may be based on CLI measurements to include the CLI measurements in full-duplex operations.

To enable the CLI beam sweeping measurements and reporting, a UE may be configured by the network with a set of SRS resources (that will be transmitted by another UE) and be configured to report one or more measurement results from the different SRS resources. For example, the UE may be configured to report one or more of the (L1-)SRS-RSRP corresponding to the highest or lowest levels of CLI, as discussed in previous section.

The UE may also be configured with one or more RX beams to receive the different resources within a SRS resource set or to receive different SRS resources sets.

Observation 3. Transmitting and monitoring other UE’s SRS can help UE in the following aspects to be considered:

- Preventive aspects, that is determining the victim and aggressor UEs beam pairs to be avoided.

- Beam pairing aspects, that is determining the gNB and victim UE beam pairs to be used based on directional CLI from the aggressor UEs.

Although a TCI state indication for CLI measurement has been agreed previously, it was only for aperiodic reporting and we propose that it can also be used for semi-persistent and periodic reporting, e.g., to enable regular CLI directional measurements or CLI beam sweeping.

Proposal 6. Support UE configuration where the UE is indicated one or more RX beam(s) for CLI resources (e.g., L1-SRS-RSRP), e.g., by indication of a CSI resource indicator (CRI) from the network for semi-persistent and periodic reporting.

Proposal 7. Support UE configurations to enable CLI-based beam-sweeping, where the UE may determine and report the most and least favourable beam(s) or beam pairing(s) between the victim and aggressor UEs.

Proposal 8. Support the UE determination of beam pairing between (victim) UE and gNB based on directional CLI measurements and reports.

#### Further enhancements for scheduling based on directional CLI reporting

Based on the measurement and reporting of (L1-)SRS-RSRP helping to identify aggressor and victim UEs, an aggressor UE could determine the UL beam directions that could cause CLI on other (nearby) UEs. In cases where an aggressor UE is configured with UL transmission based on a configured grant, one way to handle the CLI is to restrict the aggressor UE to not transmit the configured UL on the UL beam directions that may cause interference. However, the restricted UL beam direction may be the best beam direction detected by the gNB or UE for UL transmission from the aggressor UE. As such, the beam restriction could result in lower performance in UL transmission as the aggressor UE now has to transmit UL based on a suboptimal UL beam direction.

Moreover, the victim UEs that are impacted by UL from the aggressor UE in the restricted UL beam direction may not always be configured with DL reception in the same time instance as the aggressor UE is configured for UL transmission. In other words, restricting the UL beam directions may not even be required if the victim UEs are not scheduled to receive DL.

Observation 4. Restricting one or more UL beam directions at the aggressor UE due to causing CLI on victim UEs, throughout all occasions of a configured UL transmission, could degrade the UL performance, as the aggressor UE may be restricted to transmit based on suboptimal UL beam directions.

Observation 5. Restricting one or more UL beam directions at the aggressor UE due to causing CLI on victim UEs, throughout all occasions of a configured UL transmission, could be unnecessary as the respective victim UEs may not be scheduled for DL reception in all corresponding configured UL occasions.

Proposal 9. Support methods to restrict UL beam directions for a configured UL transmission at an aggressive UE based on scheduled victim UEs, that is only for the occasions that a respective victim UE is scheduled for DL reception.

The aggressor UE can be configured with a candidate UL beam direction, based on the L1-SRS-RSRP report,  that is different from the active/best UL beam direction, where the aggressor UE could select to use the candidate UL beam in case a victim UE is scheduled for DL reception. That is, the aggressor UE can select to use the active/best UL beam direction for UL transmission in case none of the victim UEs are scheduled.

The aggressor UE can receive configurations on the victim UEs that are scheduled for DL receptions that coincide with the configured UL transmission occasions. The aggressor UE can receive the indications on the scheduled DL transmissions from gNB based on a list of corresponding DL reference signals or TCI-states, where the DL beam directions correspond to or are QCL-ed with the indicated reference signals or TCI-states. Alternatively, the aggressor UE can receive the indications on the scheduled DL transmissions from gNB based on a list of corresponding SRIs, where the indicated SRIs imply the victim UEs that are scheduled for DL reception.

Moreover, a potential aggressor UE (UE #1 in Figure 4) that has measured UE-to-UE CLI with a potential victim UE (e.g., UE#2 in Figure 4) can determine that if the victim UE is configured for DL reception in the same symbol that the aggressor UE is configured for UL transmission, this could result in CLI caused for the victim UE. In such case scenarios, the aggressor UE might be configured to drop the UL transmission. Alternatively, the aggressor UE could be configured with a second candidate UL beam direction (green beam in Figure 4) to be used in such cases where UL transmission in the first UL beam direction (blue beam in Figure 4) could cause CLI on the victim UE.

On the other hand, if UE#2 is not configured for DL, the UL transmission from UE#1 could be accomplished based on the first UL beam direction (blue beam in Figure 4).

Figure 4. UL beam configuration in aggressor UE

Observation 6. An aggressor UE can be configured with a first UL beam direction and a second candidate UL beam direction, where the first UL beam direction can be used in case no CLI is caused, and the second candidate UL beam direction can be used in case CLI is caused.

Proposal 10. Support CLI mitigation techniques based on configuring a second candidate UL beam direction at the aggressor UE to be used in case the UL transmission based on the first UL beam direction could cause CLI to other UEs.

## UL resource muting for PUSCH

The UL resource muting for PUSCH is considered to enhance measurements accuracy for the network. Two use-cases benefit from the UL resource muting, i.e., the gNB measurement of the gNB-to-gNB CLI, and the gNB measurements of interference covariance matrix.

The updated WID focuses the gNB-to-gNB CLI handling on UL resource muting for PUSCH, to improve how gNB may perform measurements with reduced interference from other UL transmissions or DL CLI. In previous meetings [5][4], the following agreements were also reached:

Agreement [RAN1 #119] For the frequency location configuration of UL resource muting for a PUSCH, at least for CP-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols. Working assumption [RAN1 #119] For the frequency location configuration of UL resource muting for a PUSCH, for DFT-S-OFDM - A comb offset {0, 1} is configured for the up to two UL muting symbols. Agreement [RAN1 #119] If an UL resource muting symbol overlaps with a symbol containing UL DMRS for a PUSCH, DMRS is prioritized, i.e., UE does not apply UL resource muting in the symbol. Agreement [RAN1 #119] If an UL muting symbol overlaps with a symbol containing PT-RS for a PUSCH and transform precoding is not enabled for the PUSCH, the UE does not expect the muted REs overlaps the REs occupied by PT-RS. Agreement [RAN1 #118bis] If UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol.

It has been agreed that the UL resource muting would be based on a comb-2 muting, e.g., to avoid strong PAPR changes, and that no new DCI/MAC indication should be introduced for the determination/indication of the muted resources. It has been pointed out that even if the RE muting may be over up to 2 symbols in the time domain, if one of these symbols is placed at the end of a slot, the channel estimation and proper MMSE received configuration may be delayed to be ready, increasing overall delays and implementation constraints.

It has also been agreed that the resource muting, if any, would cover all the allocated PRBs of the PUSCH on the UL muting symbol(s), which may be viewed as an UL ZP comb-2 RS and the configuration of the time location may reuse the existing RS configuration (e.g., similar to DMRS). Introducing a semi-static configuration of multiple possible time location(s) for the UL resources based on existing RS configuration is an effective solution, where the UE may be configured with reusing existing RS patterns and configurations that supports comb-2, such as DMRS, PT-RS and comb-2 SRS.  The RS resources and ZP may be indicated using RRC configuration, and is flexible, enabling the network to control the time and frequency positions of the muted resources (e.g. to allow which symbol(s) in time to perform measurement – adjacent or not – based on PDCCH transmission parameters and/or the need for early channel estimation for processing), while being possibly (de)activated or triggered reusing existing DCI/MAC indications. Other existing RS patterns (e.g., PT-RS) may also be considered if marginal specification impact is expected.

Proposal 11: For the semi-static configuration of the time location of the UL muting resources, support RS-based pattern indication of the RE to be muted, e.g., based on a ZP comb-2 DMRS configuration with an addiction RRC indication of the RS to be a ZP-RS.

# Conclusion

In this contribution, we discussed issues on CLI mitigation aspects in SBFD systems. From the discussions, we made following observations and proposals:

2.1 L1-based UE-to-UE CLI measurement and reporting

2.1.1 Configuration of CLI measurement resources and reporting

Proposal 1. Support of one of the following alternatives for the configuration of the CLI-RSSI measurement resources:

- Alt.1: UE assumes a Method #3 measurement configuration if the configured set of CLI-RSSI-MeasurementResource within a CLI-RSSI-MeasurementResourceSet indicate frequency resources exclusively within the configured UL subband; and Method #1 otherwise.

- Alt.2: A new parameter indicating the Method type is added in CLI-RSSI-MeasurementResource or CLI-RSSI-MeasurementResourceSet and the UE excludes any frequency resources not included within the corresponding usable PRBs.

Proposal 2: For L1 based SRS-RSRP and CLI-RSSI reporting, support that the UE report the most X interfering or Y least interfering CLI measurement resources.

2.1.2 Subband and edge-specific measurement and reporting

Observation 1. Inter-subband UE-to-UE CLI measurement in SBFD DL subbands based on measuring over configured RB resources and averaging may result in down-estimation, as the subband-edge RBs experience higher CLI compared to RBs in the middle of the subband.

Proposal 3. In UE-to-UE CLI-RSSI measurement techniques within active DL BWP, support measuring and reporting delta-CLI-RSSI based on differences in measured CLI-RSSI in subband-edge or guard-bands with measured CLI-RSSI in the middle of the DL subband.

2.1.3	CLI measurements and reporting enhancements

2.1.3.1	Periodic and semi-persistent measurement and reporting

Proposal 4. For L1 based UE-to-UE CLI, support periodic and semi-persistent L1 CLI reporting.

2.1.3.2	Triggers for CLI measurements and reporting

Observation 2. Since a general CSI/beam reporting in NR is not based on dynamic CLI-related information, a victim UE may unpredictably experience DL performance degradation if a UE-to-UE CLI occurs especially when an aggressor UE is served by a different serving gNB/TRP.

Proposal 5. Support a conditional CLI handling behaviour based on monitoring the beams at the victim UE side, where the condition can at least include a case when the victim UE detects a PDSCH reception failure, which initiates a CLI measurement/reporting for a subband switching to avoid the CLI.

2.1.4	Beam aspects for CLI-based measurement and reporting

Observation 3. Transmitting and monitoring other UE’s SRS can help UE in the following aspects to be considered:

- Preventive aspects, that is determining the victim and aggressor UEs beam pairs to be avoided.

- Beam pairing aspects, that is determining the gNB and victim UE beam pairs to be used based on directional CLI from the aggressor UEs.

Although a TCI state indication for CLI measurement has been agreed previously, it was only for aperiodic reporting and we propose that it can also be used for semi-persistent and periodic reporting, e.g., to enable regular CLI directional measurements or CLI beam sweeping.

Proposal 6. Support UE configuration where the UE is indicated one or more RX beam(s) for CLI resources (e.g., L1-SRS-RSRP), e.g., by indication of a CSI resource indicator (CRI) from the network for semi-persistent and periodic reporting.

Proposal 7. Support UE configurations to enable CLI-based beam-sweeping, where the UE may determine and report the most and least favourable beam(s) or beam pairing(s) between the victim and aggressor UEs.

Proposal 8. Support the UE determination of beam pairing between (victim) UE and gNB based on directional CLI measurements and reports.

Observation 4. Restricting one or more UL beam directions at the aggressor UE due to causing CLI on victim UEs, throughout all occasions of a configured UL transmission, could degrade the UL performance, as the aggressor UE may be restricted to transmit based on suboptimal UL beam directions.

Observation 5. Restricting one or more UL beam directions at the aggressor UE due to causing CLI on victim UEs, throughout all occasions of a configured UL transmission, could be unnecessary as the respective victim UEs may not be scheduled for DL reception in all corresponding configured UL occasions.

Proposal 9. Support methods to restrict UL beam directions for a configured UL transmission at an aggressive UE based on scheduled victim UEs, that is only for the occasions that a respective victim UE is scheduled for DL reception.

Observation 6. An aggressor UE can be configured with a first UL beam direction and a second candidate UL beam direction, where the first UL beam direction can be used in case no CLI is caused, and the second candidate UL beam direction can be used in case CLI is caused.

Proposal 10. Support CLI mitigation techniques based on configuring a second candidate UL beam direction at the aggressor UE to be used in case the UL transmission based on the first UL beam direction could cause CLI to other UEs.

2.2 UL resource muting for PUSCH

Proposal 11. For the semi-static configuration of the time location of the UL muting resources, support RS-based pattern indication of the RE to be muted, e.g., based on a ZP comb-2 DMRS configuration with an addiction RRC indication of the RS to be a ZP-RS.

# References

RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei.

TR 38.858, “Study on Evolution of NR Duplex Operation”, V18.0.0.

R1-2410790, “Summary #3 of CLI Handling”, Huawei (Moderator)

Draft Minutes Report of RAN1#118b v020.

Draft Minutes Report of RAN1#119 v030.