3GPP TSG RAN WG1 #119			R1- 2500529

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.1.1

Source:	InterDigital, Inc.

Title:	Discussion on AI/ML for beam management

Document for:	Discussion and Decision

# Introduction

In RAN#102, a new work item on AI/ML for NR Air Interface has been approved [1], that builds on the previous Rel-18 studies in FS_NR_AIML_Air [2].

For the beam management use case, the objective of the study is to address some of the outstanding issues identified during the Rel-18 Study Item, as summarized in TR 38.843 [3].

With reference to beam management, the study objectives listed in the Rel-19 WID are shown below [1].

WID: - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2

In this contribution, we discuss potential issues and associated standard impacts to support AI/ML for beam management in NR air interface based on BM-Case1 and BM-Case2. Additionally, we provide an updated set of evaluation results for beam prediction.

# Discussions

## Associated ID

In RAN1#118 [8], the following agreement was made on associated ID including the working assumption on configuration of the associated ID within CSI framework:

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

While high level principle of inference related configurations, based on CSI framework with two resource sets for Set A and Set B, respectively, is agreed, how to configure data collection related configuration was not actively discussed. However, data collection related configuration is one of the major factors for designing a framework to utilize the associated ID as the motivation of the associated ID is to ensure consistency between training and inference. In that regard, how to configure the data collection related configuration should be decided before confirming the working assumption.

Proposal 1: Clarify the definition and the detailed signaling of data collection related configuration before confirming the working assumption on the support of associated IDs in CSI framework.

According to the observations from the SI [3], it is well known that one generalized AI/ML model trained for mixed scenarios provide performance benefits for the trained scenarios. Therefore, data collection related configuration should be designed to allow different scenarios/configurations associated with one associated ID. For example, multiple sets of sub-data collection configurations (e.g., Set A and Set B) can be configured within one data collection configuration, so that one of the sub-data collection configurations can be dynamically activated.

Proposal 2: Data collection related configuration supports sub-data collection configurations, which can be dynamically activated, considering different scenarios/configurations associated with one associated ID.

## Applicability of inference for UE-side model

In RAN1#118-bis [10], the following agreement was made on applicability for inference for UE-sided model:

Agreement RAN 1 further study the following options for applicability for inference for UE-side model: Option 1: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied. - FFS on whether some IEs in the CSI report configuration can be removed or modified - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig - FFS on one or more of the above CSI-ReportConfig to be reported - FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4 - FFS: whether Step 5 is needed, Option 2: - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - NW configures one set or multiple sets of inference related parameters - Note: the set of inference related parameters is not configured by CSI-ReportConfig - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - The associated ID(s) may be configured - wherein the associated ID(s) may be - FFS: a) part of one set of the inference related parameters, or - FFS: b) independently from the one set of the inference related parameters. - In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated. - In Step 5, NW configures configuration(s) for CSI report for inference Option 3: - In Step 3, following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig, - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter. - In Step 4, UE reports by UAI - the applicable one or multiple sets of inference related parameters may be included. - FFS on the set of inference related parameters, at least including: - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - Note: not applicable may also be replied by UE - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4. - the associated ID(s) may be included - FFS: a) as part of the inference related parameters, or - FFS: b) independently from the set of the inference related parameters. - In Step 5, NW configures configuration(s) for CSI report for inference. Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

In RAN1#119 [11], the following agreement and the conclusion were made on UE applicability reporting and activation of CSI report configurations.

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

For configuration of inference related parameters in Step 3 case B, one option is to provide sets of inference related parameters. The benefit of this approach is that the configuration overhead for applicability report can be minimized compared to using existing IEs of the CSI-ReportConfig, which requires detailed configurations of Set A, Set B and CSI-report related parameters, for applicability reporting.

The activation procedure for periodic CSI report in Step 4 for case A results in activation of multiple CSI report configs with duplicated or overlapping report transmission parameters (i.e., when multiple report configs are applicable) within a same usage. Although activating multiple redundant CSI report configs for a same usage is possible when there’s enough UE capability, the issue becomes more serious when the number of activated CSI reports are higher than UE’s CPU capability and/or configured to be reported simultaneously. One way to handle this issue is by associating a priority with parameters of CSI report configs so that more efficient CSI reports are prioritized.

Proposal 3: For configuration of each set of inference related parameters for applicability report, down select from the following options:

- Option 1: Support simplified parameters at least including number of beams and QCL-related information for Set A and Set B, and reportQuantity.

- Option 2: Reuse existing IEs at least including configuration of Set A, Set B and reportQuantity.

Proposal 4: Support a priority based mechanism to handle duplicated activated periodic CSI report configs.

## Beam prediction, indication and reporting

### Configuration of Set A and Set B

In RAN1#119 [11], the following agreement was made on configuration of Set A and Set B:

Agreement For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration, - Two CSI-ResourceConfigId s are configured for Set A and Set B separately

In RAN1#116-bis [6], the following agreement was made on configurations of Set A and Set B:

Agreement For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting - FFS on the details in the CSI-ReportConfig, at least considering: - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A - Note: separate CSI-ReportConfig for Set A and Set B are not precluded. - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig - FFS on the association between Set A and Set B with or without additional IE - Other necessary configuration are not precluded.

In RAN1#119, Alt.3: “Two CSI-ResourceConfigIds are configured for Set A and Set B separately.” was agreed for the configuration of Set A and Set B. Since all four alternatives provide the same functionality (i.e., configuration for Set A and Set B) in one way or the other, the remaining alternatives should be deprioritized to avoid any redundancy in the spec.

In addition, it is important to support both no transmission (Scenario 1) and transmission with longer periodicity (Scenario 2) for AI/ML beam management operation. For Scenario 1, RS overhead can be less than Scenario 2, however, additional handling of QCL operation for Set A resources, which are not transmitted, is needed. On the other hand, for Scenario 2, although RS overhead can be higher than Scenario 1, existing QCL operation can be reused. Therefore, both Scenario 1 and Scenario 2 should be supported for AI/ML beam management.

Proposal 5: For the configuration of Set A and Set B, no configuration method is supported other than configuration of two CSI-ResourceConfig Ids for Set A and Set B separately.

Proposal 6: Support Set A for both no transmission and transmission with longer periodicity.

For the configuration of Set A and Set B, the best option that we can support both Scenario 1 and Scenario 2 is to support RS resource design which allows both no transmission (Scenario 1) and transmission with longer periodicity (Scenario 2). While resources of Set A can be used only for indicating best CRIs in Scenario 1, resources of Set A should allow measurement of physical resources as well as indicating best CRIs in Scenario 2. Given the both scenarios, the RS resources configured in Set A should be designed to allow no transmission (Scenario 1) as well as physical transmission with different periodicity from Set B (Scenario 2). For Scenario 2, as the existing specification already supports independent configuration of CSI-ResourcePeriodicityAndOffset for each resource, it can be supported without additional specification support. On the other hand, for Scenario 1, the existing specification does not allow RS resources without transmission. The simplest way to achieve Scenario 1 is allowing Set A RS resources without configured parameters related to physical transmission. For example, parameters such as resourceMapping, CSI-ResourcePeriodicityAndOffset and etc. may not be configured for Set A RS resources.

Proposal 7: Support CSI resource configs with/without physical RS transmission parameters (e.g., resourceMapping, CSI-ResourcePeriodicityAndOffset and etc.).

### QCL measurements for unmeasured Set A beams.

For successful PDCCH/PDSCH reception and decoding, the UE needs to measure associated QCL parameters (e.g., Doppler spread, Doppler shift, delay spread, average delay and spatial Rx parameter) as shown in section 5.1.5 of TS 38.214 [4].

### 5.1.5 Antenna ports quasi co-location The UE can be configured with a list of up to M TCI-State configurations within the higher layer parameter PDSCH-Config to decode PDSCH according to a detected PDCCH with DCI intended for the UE and the given serving cell, where M depends on the UE capability maxNumberConfiguredTCIstatesPerCC. Each TCI-State contains parameters for configuring a quasi co-location relationship between one or two downlink reference signals and the DM-RS ports of the PDSCH, the DM-RS port of PDCCH or the CSI-RS port(s) of a CSI-RS resource. The quasi co-location relationship is configured by the higher layer parameter qcl-Type1 for the first DL RS, and qcl-Type2 for the second DL RS (if configured). For the case of two DL RSs, the QCL types shall not be the same, regardless of whether the references are to the same DL RS or different DL RSs. The quasi co-location types corresponding to each DL RS are given by the higher layer parameter qcl-Type in QCL-Info and may take one of the following values: - 'typeA': {Doppler shift, Doppler spread, average delay, delay spread} - 'typeB': {Doppler shift, Doppler spread} - 'typeC': {Doppler shift, average delay} - 'typeD': {Spatial Rx parameter}

When PDCCH/PDSCH are to be transmitted via a predicted best beam which is not measured, the UE cannot acquire QCL-related parameters thereby failing to successfully decode the PDCCH/PDSCH. Therefore, a procedure to obtain QCL-related parameters for an unmeasured predicted beam is needed before PDCCH/PDSCH transmission. One possible way to acquire QCL-related parameters for an unmeasured predicted beam is utilizing neighbouring beams of the unmeasured predicted beam.

In Figure 1 and Figure 2, below, we show performance of doppler shift and delay spread estimation for an unmeasured beam based on neighbouring beams. In the Figures below, estimation of the QCL parameters for missing Tx beams is performed using measurement of QCL parameters based on adjacent beams on each side of the missing beam along the azimuth direction. The UE is configured to sweep through 64 Tx beams.

Figure 1. CDF of the absolute error in estimating doppler shift for unmeasured/missing beams based on adjacent beams on each side of the missing beams.

Figure 2. CDF of the absolute error in estimating RMS delay spread for unmeasured/missing beams based on adjacent beams on each side of the missing beams.

It can be seen from the figures above that estimation of QCL parameters for unmeasured/missing beams based on neighbouring/adjacent is possible within an acceptable margin of error. For example, Figure 1 show that using adjacent beams on each side of an unmeasured/missing beam, the doppler shift, which would be measured by the missing beam at the UE, can be estimated such that the estimation error falls below 10 Hz 90% of the time. Likewise, estimation of the RMS delay spread which would be measured by the missing beam for the UE can be managed to 15ns or less in absolute error 90% of the time.

Observation 1: When PDCCH/PDSCH are to be transmitted via a predicted best beam which is not measured, the UE cannot acquire QCL-related parameters, therefore a procedure to obtain QCL-related parameters for the unmeasured beam is needed.

Observation 2: Estimation of QCL-related parameters via neighbouring beams achieves Doppler shift estimation with error below 10 Hz for 90% of the time and RMS delay spread below 15 ns for 90% of the time.

Proposal 8: Support configuration of RS resources to estimate QCL-parameters for a RS resource without transmission in Set A.

### Pattern based beam indication

As best beams can be successfully predicted by AI/ML model in BM-Case 2, enhanced beam indication mechanism is needed to enable future beam indication based on the prediction. One possible way is updating the current beam mechanism from indication of one beam to indication multiple beams with corresponding time resources. An efficient way to enable multiple beam indication can be introducing a beam indication mechanism based on beam patterns. For example, a beam pattern and corresponding TCI states required for the indicated beam pattern can be used based on AI/ML based beam prediction.

Observation 3: Enhanced beam indication mechanism is needed to enable future beam indication based on prediction of AI/ML model in BM-Case 2.

Proposal 9: Support a beam indication mechanism with a beam pattern and corresponding TCI states required for the indicated beam pattern.

### Reporting beam measurements/predictions

In RAN1#118-bis [10], the following agreement was made on CPU mechanism for AIML-based CSI processing:

Agreement For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2

The benefit of a separate CPU counter for AI/ML-based CSI processing is not clear. Separately allocating CPU resources for each type of CSI-processing (i.e., legacy and AI/ML) results in an inefficient utilization of UE CSI-processing capability. For example, in a scenario when one type of CSI-processing is underutilized (i.e., idle CPUs) and the other type of CSI-processing is bottlenecked by the number of allocated CPUs. Therefore, it is beneficial to have shared CPUs between legacy and AI/ML-based processing.

Proposal 10: Support a shared CPU counter between legacy and AIML-based CSI reporting.

In TR 38.843 [3], the following potential specification impact was included for assessment.

For BM-Case1 and BM-Case2 with a NW-side AI/ML model: - L1 beam reporting enhancement for AI/ML model inference: - UE to report the measurement results of more than 4 beams in one reporting instance - Other L1 reporting enhancements can be considered

In RAN1#116 [5], the following agreement was made on beam measurement reporting:

Agreement For NW-sided model, for inference, in a beam report initiated by network, based on one measurement resource set, support the report of more than 4 beam related information in L1 signaling - Note: Purpose, such as above “For NW-sided model, for inference”, will not be specified in RAN 1 specifications - FFS on the report content for beam related information - FFS on max number of reported beam related information in one report

In RAN1#117 [7], the following agreements were made for inference report for NW-side model.

Agreement For NW-sided model, for inference report, at least for BM-Case 1, the content in a beam report in L1 signaling, support - L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB - If M = the size of the measurement resource set, the content is all L1-RSRPs and one beam index (i.e., CRI/SSBRI) for the largest measured value of L1-RSRP of a measurement resource set - FFS: L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP, X and M are configured by gNB, and whether/how to report number of reported beams - FFS on the maximum value of M (where M can be larger than 4) based on UE capability (M may or may not be different for different reporting contents) - FFS on beam information - Note: Purpose, such as above “For NW-sided model, for inference report, at least for BM-Case 1”, will not be specified in RAN 1 specifications Agreement At least for NW sided model, for the quantization of a reported L1-RSRP value at least for the report in L1 signaling, support - Support differential L1-RSRP reporting with legacy quantization step and range - FFS: larger quantization step(s) than the already supported legacy quantization step for differential L1-RSRP and/or for absolute L1-RSRP - FFS: Smaller range(s) for differential L1-RSRP than the already supported legacy range

Although it is beneficial to report measurement results of more than 4 beams in one UE reporting instance, however, having UE to report all AIML model input measurements for every AIML prediction instance to gNB greatly increases UE reporting overhead. Therefore, a procedure to reduce reporting overhead by UE is beneficial in some scenarios. For example, when channel condition or UE position does not change significantly, the UE can skip reporting of a subset of AIML input measurements to gNB.

Tables 1 and 2 show beam prediction accuracy considering different UE beam reporting methods. In this evaluation, we consider a sequence of spatial beam predictions for a UE moving at speed of 30 kmh. In addition, RSRP measurements are taken every ΔT (ΔT = 80 ms or 640 ms). RSRP measurements from set B, consisting of 16 Tx beams following the pattern shown below (i.e., Pattern #1), were used from set A consisting of 64 Tx beams.

|  | Beam is member of set A and set B |
| --- | --- |
|  | Beam is member of set A |

Azimuth Angles

Elevation Angles

Pattern #1

Figure 3. Beam patterns of Set B

We consider multiple cases for selecting reported RSRP measurements for set B:

- Case 1 (Baseline): Every ΔT ms, all beams of set B are measured and reported.

- Case 2: Every ΔT ms, 50% of the beams of set B are measured and reported alternatively (e.g., odd beams are measured and reported at time instant T and even beams are measured and reported at time instant T+1). For spatial beam prediction at time instant T, odd beam measurements are combined with even beam measurements from time instant T-1 as input to AIML model. Similarly, at time instant T+1, even beam measurements are combined with odd beam measurements from time instant T-1 as input to AIML model.

- Case 3: Every ΔT ms, 50% of the beams of set B, that have highest RSRP at time instant T-1, are measured and reported at time instant T. For spatial beam prediction at time instant T, best beam measurements are combined with worst beam measurements from time instant T-1 as input to AIML model.

- Case 4a/4b: Every ΔT ms, a subset of beams from set B that are within X dB (12 dB for case 4a, 18 dB for case 4b) from the highest RSRP at time instant T-1, are measured and reported at time instant T. For spatial beam prediction at time instant T, the subset of high RSRP beam measurements are combined with weakest beam measurements from time instant T-1 as input to AIML model.

Table 1: Sequential Spatial Beam Prediction Accuracy of AIML-based Classification (Reduced Overhead Reporting) – ΔT = 80 ms

| Case | Measured Beams at Time Instant T | Top-1 Accuracy | Top-3/1 Accuracy | Top-5/1 Accuracy |
| --- | --- | --- | --- | --- |
| Case 1 (Baseline) | All Beams | 74.58% | 92.70% | 96.52% |
| Case 2 | Odd/Even beams alternatively | 72.90% | 92.26% | 96.19% |
| Case 3 | Best 50% of beams at time instant T-1 | 74.05% | 92.53% | 96.50% |
| Case 4a | Subset of beams within 12 dB from best beam (Average number of measured beams = 6.5323) | 73.65% | 92.58% | 96.46% |
| Case 4b | Subset of beams within 18 dB from best beam (Average number of measured beams = 11.4063) | 74.47% | 92.83% | 96.50% |

Table 2: Sequential Spatial Beam Prediction Accuracy of AIML-based Classification (Reduced Overhead Reporting) – ΔT = 640 ms

| Case | Measured Beams at Time Instant T | Top-1 Accuracy | Top-3/1 Accuracy | Top-5/1 Accuracy |
| --- | --- | --- | --- | --- |
| Case 1 (Baseline) | All Beams | 73.25% | 92.58% | 96.15% |
| Case 2 | Odd/Even beams alternatively | 62.96% | 87.50% | 93.56% |
| Case 3 | Best 50% of beams at time instant T-1 | 67.56% | 90.14% | 94.83% |
| Case 4a | Subset of beams within 12 dB from best beam (Average number of measured beams = 6.5321) | 65.86% | 89.37% | 94.78% |
| Case 4b | Subset of beams within 18 dB from best beam (Average number of measured beams = 11.4097) | 68.93% | 91.29% | 95.69% |

For ΔT = 80 ms, as shown in Table 1, Case 2, Case 3, Case 4a and Case 4b showed marginal difference in beam prediction accuracy with 50% reduction of UE reporting overhead. Also, for ΔT = 640 ms, as shown in Table 2, minor difference in Top-3/1 and Top-5/1 beam prediction accuracy is shown with 50% reduction of UE reporting overhead. Therefore, UE beam reporting overhead can be reduced without sacrificing beam prediction accuracy. Moreover, beam prediction accuracy with variable overhead reduction (i.e., reporting beams within X dB of the best beam) is also as good as baseline. It provides finer control on the number of beams to be reported by configuring a value for X. It should be noted that, CSI reporting with variable payload sizes is already supported by spec in NR through multi-part CSI-Report.

Observation 4: The UE’s frequent reporting of AIML input measurements for inference at gNB greatly increases UE reporting overhead.

Observation 5: Beam prediction accuracy with both fixed overhead reduction (e.g., reporting odd/even beams alternatively or best 50% of beams) and variable overhead reduction (i.e., reporting beams within X dB of the best beam) is as good as baseline case (i.e., reporting all beams in Set B).

Proposal 11: Support an overhead reduction mechanism for reporting data collection for inference (e.g., based on channel conditions).

Proposal 12: Support reporting of L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP.

A useful method to report beam measurements is through pattern-based reporting. In case when the number of Set B beam is large, reporting information of all the set B beams may not be efficient. In this case, supporting a pattern-based reporting enables the UE to report large number of beam measurements without indicating additional CRI overheads, thus reducing UE’s reporting overhead. Moreover, the pattern-based reporting also enables overhead reduction in reporting by the UE by associating a subset of Set B beams with pattern ID(s). When overhead reduction for reporting is enabled, the UE reports beam measurements with corresponding indication of pattern ID(s) for a subset of all pattern ID(s) associated to Set B.

Proposal 13: Support a pattern-based reporting mechanism for data collection for inference wherein the UE reports a pattern ID(s) and corresponding beam measurements of a subset of beams in Set B.

In RAN1#116-bis [6], the following agreements were made on beam measurements and inference reporting:

Agreement For network-sided AI/ML model for BM-Case1 and BM-Case2, - support using existing CSI framework for configuration of Set A as the starting point - support using existing CSI framework for configuration of Set B as the starting point - Note: Purpose, such as above “For NW-sided model, for BM-Case1 and BM-Case2” and “Set A” and “Set B”, will not be specified in RAN 1 specifications Agreement For report content of inference results for UE-sided model for BM-Case 1, for the RSRP of predicted Top K beam(s) in the report of inference results, when applicable, further study the following options: - Option A: Predicted RSRP - Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement - Where the predicted RSRP is based on AI/ML output - Note: Support both Option A and Option B is not precluded.

In RAN1#116-bis [6], the following agreements was on CSI reporting in agenda item 9.1.3.1:

Agreement For the AI/ML based CSI prediction, for CSI report, adopt following as a baseline for evaluation purpose - N4 value: 1, 4 - Others can be additionally submitted, e.g., 2, 8 - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1) - Others can be additionally submitted. - Note: The same selected parameter combination shall be applied for benchmarks. - CSI report periodicity: 5ms, 20ms (encouraged) - Others can be additionally submitted, e.g., 10ms

In RAN1#118 [8], the following agreements were made on beam measurements and inference reporting:

Agreement For UE-sided model, for the quantization of a RSRP value at least for the report of inference results, support - Support differential RSRP reporting with legacy quantization step and range for L1-RSRP reporting - For BM-Case 1, support differential RSRP report among multiple beams - For BM-Case 2, support differential RSRP report among multiple beams over multiple time instances - FFS details Agreement For UE-sided model for BM-Case 2, for inference results report, support to configure UE with N future time instance(s) for inference by NW when applicable - FFS: how to determinate reference time for the time instance(s) - FFS: duration values of the N time instance(s) that can be predicted.

In RAN1#118-bis [10], the following agreement was made on determination of reference time for BM-Case 2:

Agreement For BM-Case 2 of UE-side model, for the reference time of the earliest time instance for the predicted results, consider at least the following alternatives for potential down-selection: - Option 1: Based on the uplink slot for the report - Option 2: Based on the CSI reference resource corresponding to the report - Option 3: Based on the latest transmission occasion of the CSI-RS/SSB resource in Set B for measurement for the report, wherein the transmission occasion is no later than the CSI reference resource

For a UE-side AI/ML model, required specification impact can be minimized as UE determines best beams based by using the UE-side AI/ML model. For example, beam(s) that is based on the output of AI/ML model inference was agreed as a potential specification support. For BM-Case 1, the current beam reporting with CRIs/SSBRIs and corresponding L1-RSRP values is enough to indicate the best beam(s) from AI/ML model inference. For BM-Case 2 with a gNB-side model, time domain characteristics of beam measurements are essential for time domain prediction. Especially, such time domain information will be crucial for certain predictable scenarios such as highway or HST. Information about measured past instances (e.g., the time stamp) for reported beam related information (e.g., CRIs/SSBRIs/RSRPs) can be further considered.

Additionally, for BM-Case2 with a UE-sided model, providing multiple sets of predicted beam information for future time instances (e.g., future beam predictions by the UE) should be beneficial. Reporting information of multiple future time instances reduces latency as network will obtain information of multiple future time instances in advance and does not have to wait for UE reports in queue. For the design for the reporting information of multiple future time instances, the aforementioned agreements in agenda items 9.1.1 and in 9.1.3.1 (CSI Prediction) can be a starting point. For example, the choice of N for reporting BM-Case 2 inference results can be either same with or multiple of CSI reporting periodicity in CSI prediction.

For the determination of reference time for BM-Case 2 report, the benefit of Option 1 is not clear since the purpose of reference time is to determine the time of applicability of the UE report. From the perspective of a temporally predicting AI/ML model, the time at which Set B measurements are performed is the reference time or time 0. The time of applicability of a prediction made by the AI/ML model is calculated/outputted by the AI/ML model relative to the reference time (i.e., time 0) mentioned earlier. Moreover, Option 2 encompasses Option 3 since CSI reference resource (as stated in Option 2) also allows for the configuration of reference resource according to Option 3.

The temporal predictions made by an AI/ML model become less reliable, the farther in time they are especially in a rapidly changing environment (e.g., Urban moving UE). Similarly, the predictions closer to the time of obtaining AIML inputs (i.e., Set B beam measurements) are expected to be more accurate. For example, for a static or slow speed moving UE, the report containing information (e.g., beam ID and predicted RSRP) of a larger number of future time instances is beneficial as the temporal predictions are likely to be valid for a longer duration. Conversely, when a UE is moving at high speed, it is more beneficial to report for a smaller number of future time instances since predictions farther in time are less likely to be valid. Therefore, it is beneficial to have a control on the number of future time instances and the number of reported beams for each future time instance.

For report content of inference results for UE-sided model for BM-Case 1, both ‘Option A: Predicted RSRP’ and ‘Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement’ are useful. In a high noise scenario, a predicted RSRP, usually obtained through regression based AIML model can be closer to the ground truth than a real measurement (e.g., RSRP), which is subject to high measurement errors due to high noise. This is because regression-based prediction also acts as a denoiser for the input. On the other hand, when there’s low measurement noise, the real measurement is usually closer to ground truth than a predicted measurement. Therefore, it is beneficial to have a UE based selection procedure to report according to Option A or Option B.

Observation 6: For BM-Case 1 with a UE-side AI/ML model, the current beam reporting with CRIs/SSBRIs and corresponding L1-RSRP values is enough to indicate the best beam(s) from AI/ML model inference.

Observation 7: For BM-Case 2 with a gNB-side AI/ML model, information about time stamp on measurement instances for reported beam related information needs to be reported for time domain prediction at gNB.

Observation 8: For BM-Case 2 with a UE side AI/ML model, reporting information of multiple future time instances in one report reduces latency as gNB does not have to wait for UE reports in queue.

Proposal 14: Information about the time stamp for measurement instances should be supported.

Proposal 15: Reporting prediction results of multiple future time instances in one report should be supported. Beam inference reporting periodicity should be aligned with CSI-Reporting periodicity as a baseline.

Proposal 16: Support “Option 2: Based on the CSI reference resource corresponding to the report” for determination of reference time.

Proposal 17: Support flexible configuration of multiple future time instance reporting (e.g., based on channel conditions, gNB configuration).

Proposal 18: Support both

‘Option A: Predicted RSRP’

‘Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.’

Additionally, support a UE based selection procedure to report according to Option A or Option B.

In RAN1#116, whether to support beam reporting for only inference or both training and inference was discussed especially for network sided model. As how to utilize reported information, there’s no need to limit the use case of beam reporting. However, enhancement of UE reporting can be considered to allow both training and inference via UE beam reporting. For example, in BM-Case 1, current beam reporting with up to 4 CRIs/SSBRIs and corresponding L1-RSRP values is not enough for AI/ML model training, so that beam reporting up to 64 RSRPs can be considered given the maximum number of beams supported in the current specification. Compared to RSRPs, additional reporting of CRIs/SSBRIs may not be needed as the UE need to report all measured RSRPs from configured RSs (but needs an association between reported RSRPs and measured RSs). For BM-Case 2, time domain characteristics of beam measurements are essential for time domain prediction. Especially, such time domain information will be crucial for certain predictable scenarios such as highway or HST. Information about measured past instances (e.g., time stamp) for reported beam related information should be considered.

Observation 9: No need to restrict purpose of UE beam reporting, especially for network sided model, as how to utilize reported information is up to gNB implementation.

Observation 10: For network sided model, enhancement of UE reporting can be considered to allow both training and inference via UE beam reporting.

Proposal 19: For network sided model, support enhanced UE reporting to report up to 64 RSRP values for whole Set A over multiple time instances.

- No CRIs/SSBRIs are reported and implicit beam indexes (e.g., by association with RSs and reported RSRPs) are used.

- Information on measured past instances (e.g., time stamp) is supported.

### LCM and Consistency and Performance Monitoring

#### Considerations on beam patterns

In TR 38.843 [3], the following conclusion was drawn for BM-specific conditions for AIML models.

Conclusion For BM-Case1 and BM-Case2 with a UE-side AI/ML model, the necessity and potential BM-specific conditions/additional conditions for functionality(ies) and/or model(s) are considered at least from the following aspects: - information regarding model inference - Set A / Set B configuration - performance monitoring - data collection - assistance information

For a UE-sided model, to ensure consistency between training and inference, configuration information associated with the Set B should be indicated to UE so that the UE selects an appropriate AIML model for inference, i.e., an AIML model whose training data/inputs are a good match for the indicated Set B. The configuration signalling associated to a Set B should include information on beam IDs, beam periodicity, beam widths etc.

For gNB-sided model, we consider the spatial beam prediction for multiple cases of reported RSRP measurements selection (i.e., set B selection) when the reported RSRP measurements are subset of the total RSRP measurements (i.e., set B is a subset of set C). We consider a set C of RSRP measurements consisting of 16 Tx beams, following the pattern shown below (i.e., Pattern #1), which a subset of set A consisting of 64 Tx beams.

|  | Beam is member of set A and set C |
| --- | --- |
|  | Beam is member of set A |

Azimuth Angles

Elevation Angles

Pattern #1

|  | Beam is member of set A and set C |
| --- | --- |
|  | Beam is member of set A |

Pattern #2

Figure 4. Beam patterns of Set B for Uniformly selected 16 and 8 Tx beams

We consider multiple cases for selecting reported RSRP measurements for set B:

- Case 1 (Baseline): Size of set B = Size of set C = 8 following Pattern #2.

- Case 2-1: Size of set B = 8, where the members of set B are selected as the best 8 beams from set C.

- Case 2-2: Size of set B = 8, where the members of set B are selected randomly from set C.

- Case 3-1: Size of set B is variable, where the members of set B are selected as the beams of set C that are within 12.621 dB from best beam (so 8 beams on average are selected).

- Case 3-2: Size of set B is variable, where the members of set B are selected as the beams of set C that are within 12 dB from best beam. Also, a minimum of 4 beams must be selected.

- Case 4 (Upper Bound): Size of set B = Size of set C = 16 following Pattern #1.

Table 3: Spatial Prediction Accuracy of AIML-based Classification (Set B Selection)

| Case | Size of Set C | Size of Set B | Set B Selection | Top-1 Accuracy | Top-3/1 Accuracy | Top-5/1 Accuracy |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 (Baseline) | 8 | 8 | All in set C | 55.36% | 86.04% | 93.33% |
| Case 2-1 | 16 | 8 | Best Beams in Set C | 70.19% | 94.14% | 97.50% |
| Case 2-2 | 16 | 8 | Random Beams from Set C | 51.60% | 80.21% | 89.07% |
| Case 3-1 | 16 | variable (Average Size of Set B = 7.6852 Beams) | Beams within 12.621 dB from best beam | 71.81% | 94.83% | 98.41% |
| Case 3-2 | 16 | max(variable,4) (Average Size of Set B = 7.9974 Beams) | Beams within 12 dB from best beam | 70.86% | 94.81% | 98.24% |
| Case 4 (Upper Bound) | 16 | 16 | All in set C | 76.67% | 96.29% | 98.86% |

We note that selecting set B as a subset of best measured beams (Case 2-1) outperforms measuring and reporting a uniform set of beams (Case 1). Moreover, the achieved performance with variable beam set selection (Case 3-1, Case 3-2) is slightly less than the upper bound (Case 4).

Observation 11: For a UE-sided model, configuration information associated to a Set B can help the UE in ensuring consistency between training and inference.

Observation 12: Set B selection as a subset of the best measured beams for input to the gNB-sided AIML model achieves a higher beam prediction accuracy than reporting a uniformly or randomly selected subset of the measured beams.

Proposal 20: Indicating configuration information associated with Set B to UE should be supported.

Proposal 21: Support reporting of UE selected Set B based on a rule (e.g., subset of best measured beams).

#### Considerations on Performance monitoring

In RAN1#118-bis [10], the following agreements was made on Option 2, UE-assisted performance monitoring:

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

In RAN1#119 [11], the following agreements was made on CSI report configuration for UE side performance monitoring:

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for infernece report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

In RAN1#119 [12], the following was proposed to introduce beam accuracy indicator for the discussion:

(FL0) Proposal 2.1-1a For BM-Case1 and at least for BM-Case2 with single time instance in a report with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), introduce a new quantity (i.e., beam accuracy indicator, BAI) for beam prediction accuracy in the CSI report for monitoring, - BAI = Np, the accuracy corresponds to the BAI is Np/N. - FFS: the size of CSI field associated with the BAI is log2N - Where Np as the number of the reported inference result(s) linked with performance monitoring instance(s) to (out of N) for which, FFS the following statement holds with potentially downselection (at least for the case that full set of Set A can be measured) - Option 1 (Top-1/1): the Top-1 beam with largest measured value of the resource set(s) for monitoring is Top-1 predicted beam - Option 2 (1/Top-K): the Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams - K >1, or K is equal to the number of reported predicted beam configured by inference report configuration - Option 3 (Top-K/M): the Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring - K= 1, FFS other values - M is configured - K<=M - Option 4 (best of Top-K/1 with margin): The beam with largest measured value of L1-RSRP of Top-K predicted beams is within a margin X dB of largested measured value of L1-RSRP of the resource set(s) for monitoring - X=0, 1, FFS on whether to support other values - K=1 or K is equal to the number of reported predicted beam configured by inference report configuration, FFS other values - Further study on whether to update some definition of the description of above options. - Note: the above options at least with the assuming that Top-K predicted beam are included in monitoring set(s). - Other options are not precluded. - Note: The above options are at least for discussion purpose, the specification - Where N is the number of the reported inference result(s) linked with performance monitoring instance(s) - FFS on how to link an inference result (e.g., in the inference report) to a performance monitoring instance - FFS on whether to exclude some linked reported inference result(s)/monitoring instance(s)

A simple approach to link monitoring RS resource sets to Set A beams is to link them via a TCI-state. For example, TCI-state associated with the latest transmission of a control/data channel can indicate the activation of appropriate monitoring RS resource set i.e., the monitoring RS resource set containing the RS resource associated with the indicated TCI-state. While the NW can choose any beam for its transmission, the most likely scenario is that NW will use one of the Top-K predicted best beams for its transmission, and therefore the monitoring RS resource set associated with the Top-K beam will be activated. Another benefit of this approach is that any additional configurations/indications from the NW are not required to indicate the link between monitoring resource set and Set A beams.

Regarding the choice of performance metrics for UE assisted performance monitoring, the practical benefit of Option 1, “the Top-1 beam with largest measured value of the resource set(s) for monitoring is Top-1 predicted beam”, is not clear as it is restrictive in its definition. In a commonly occurring scenario, where the Top-1 predicted beam’s performance is slightly lower than the genie-aided Top-1 beam (e.g., Top-1 measured beam), the prediction will be considered inaccurate according to Option 1 even if the Top-1 predicted beam is as good as the Top-1 measured beam in a practical sense. Moreover, Option 1 is a special case of Option 2, “the Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams”, with K = 1.

For the case when Top-K predicted beams are not included in the monitoring resource set, the performance metrics presented in the (FL0) Proposal 2.1-1a are inapplicable. Therefore, a performance metric to handle such scenario is beneficial. For example a performance metric based on the RSRP difference wherein a prediction is counted as accurate when the average difference between the measured and predicted RSRP of the beams associated with monitoring resource set is within X dB, and inaccurate otherwise.

One way to activate performance monitoring at a UE is to couple it with the transmission configuration of the monitoring resource(s) i.e., the UE activates performance monitoring when it receives RSs for performance monitoring. For example, based on the reception of RSs associated to Set A beams (i.e., when configured with physical transmission parameters). If the performance of an AI/ML model falls lower than an acceptable level, then there is no benefit in continually monitoring the performance of the model until model recovery procedure is completed. In this context, a timer based performance monitoring procedure is useful wherein the UE continually monitors the performance of AI/ML model at periodic time instances until the timer expires. The timer is reset based on gNB signalling or when determined model performance is within acceptable range.

One good candidate for UE reported assistance information in the context of performance monitoring is an indication on AIML model validity. For example, instead of reporting the calculated metrics, the UE sends an indication of AIML model validity based on a performance metric computed by the UE.

Proposal 22: For UE-assisted performance monitoring, Support TCI-state based activation of RS resource sets for performance monitoring.

Proposal 23: For UE-assisted performance monitoring, for the case when Top-K beams are included in the monitoring resource set, down select from one of the following performance metrics:

- Option 2 (1/Top-K): The Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams

- Option 3 (Top-K/M): The Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring.

- Option 4 (best of Top-K/1 with margin): The beam with largest measured value of L1-RSRP of Top-K predicted beams is within a margin X dB of largest measured value of L1-RSRP of the resource set(s) for monitoring

Proposal 24: For UE-assisted performance monitoring, for the case when Top-K beams are not included in the monitoring resource set, support a performance metric based on difference between measured RSRPs and predicted RSRPs.

Proposal 25: For UE-assisted performance monitoring, support the following options for activation of performance monitoring at UE:

- Based on the reception of RSs associated to Set A beams.

- Based on a periodic timer which starts/stops performance monitoring procedure based on gNB signaling or model performance.

Proposal 26: Support a UE based indication of AIML model validity.

### Location of AIML Inference

AI/ML inference/training at UE side can be limited due to limited computational power and battery consumption at UE implementation, however, UE can easily utilize more information that the UE acquired by measuring SSB/CSI-RS without consuming any reporting overhead. 
On the other hand, AIML inference at gNB-side can lead to significant reporting overhead since inference generally requires detailed explicit information. Although gNB has much greater computational capacity than UE, gNB computation capacity can be insufficient in some scenarios. For example, for a pair of gNB and UE, the reporting overhead and computation requirement for inference is not a critical issue for the network, however, if we consider all available UEs in a cell, acquiring information for the inference from multiple UEs requires huge overheads. Moreover, gNB may not have enough computational capacity for performing inference operations for multiple UEs especially in UE dense environments. In such scenario, it is beneficial to dynamically enable UE-sided AIML inference for a subset of UEs e.g., based on UE capability or UE’s prediction performance. The network can switch back and forth between NW-side and UE-side inference based on its computational workload, capacity and/or UE’s prediction performance.

Observation 13: AI/ML inference at UE side can be limited due to limited computational power and battery consumption at UE implementation, however, UE can easily utilize more information that the UE acquired by measuring SSB/CSI-RS without consuming any reporting overhead.

Observation 14: AI/ML inference at NW side could be a good implementation option as UE implementation is generally limited due to computational power and battery consumption than gNB implementation. However, AI/ML inference generally requires more detailed explicit information which leads significant reporting overhead. Moreover, although gNB has greater computational capacity than UE, the capacity can still prove to be insufficient in dense UE scenarios.

Proposal 27: Support a procedure to dynamically switch AIML inference location (e.g., based on NW workload and/or UE’s prediction performance).

# Summary

In this contribution, we discuss potential issues and associated standard impacts to support AI/ML for beam management. From the discussions, we made the following observations and proposals:

Observation 1: When PDCCH/PDSCH are to be transmitted via a predicted best beam which is not measured, the UE cannot acquire QCL-related parameters, therefore a procedure to obtain QCL-related parameters for the unmeasured beam is needed.

Observation 2: Estimation of QCL-related parameters via neighbouring beams achieves Doppler shift estimation with error below 10 Hz for 90% of the time and RMS delay spread below 15 ns for 90% of the time.

Observation 3: Enhanced beam indication mechanism is needed to enable future beam indication based on prediction of AI/ML model in BM-Case 2.

Observation 4: The UE’s frequent reporting of AIML input measurements for inference at gNB greatly increases UE reporting overhead.

Observation 5: Beam prediction accuracy with both fixed overhead reduction (e.g., reporting odd/even beams alternatively or best 50% of beams) and variable overhead reduction (i.e., reporting beams within X dB of the best beam) is as good as baseline case (i.e., reporting all beams in Set B).

Observation 6: For BM-Case 1 with a UE-side AI/ML model, the current beam reporting with CRIs/SSBRIs and corresponding L1-RSRP values is enough to indicate the best beam(s) from AI/ML model inference.

Observation 7: For BM-Case 2 with a gNB-side AI/ML model, information about time stamp on measurement instances for reported beam related information needs to be reported for time domain prediction at gNB.

Observation 8: For BM-Case 2 with a UE side AI/ML model, reporting information of multiple future time instances in one report reduces latency as gNB does not have to wait for UE reports in queue.

Observation 9: No need to restrict purpose of UE beam reporting, especially for network sided model, as how to utilize reported information is up to gNB implementation.

Observation 10: For network sided model, enhancement of UE reporting can be considered to allow both training and inference via UE beam reporting.

Observation 11: For a UE-sided model, configuration information associated to a Set B can help the UE in ensuring consistency between training and inference.

Observation 12: Set B selection as a subset of the best measured beams for input to the gNB-sided AIML model achieves a higher beam prediction accuracy than reporting a uniformly or randomly selected subset of the measured beams.

Observation 13: AI/ML inference at UE side can be limited due to limited computational power and

battery consumption at UE implementation, however, UE can easily utilize more information that the UE acquired by measuring SSB/CSI-RS without consuming any reporting overhead.

Observation 14: AI/ML inference at NW side could be a good implementation option as UE implementation is generally limited due to computational power and battery consumption than gNB implementation. However, AI/ML inference generally requires more detailed explicit information which leads significant reporting overhead. Moreover, although gNB has greater computational capacity than UE, the capacity can still prove to be insufficient in dense UE scenarios.

Proposal 1: Clarify the definition and the detailed signaling of data collection related configuration before confirming the working assumption on the support of associated IDs in CSI framework.

Proposal 2: Data collection related configuration supports sub-data collection configurations, which can be dynamically activated, considering different scenarios/configurations associated with one associated ID.

Proposal 3: For configuration of each set of inference related parameters for applicability report, down select from the following options:

- Option 1: Support simplified parameters at least including number of beams and QCL-related information for Set A and Set B, and reportQuantity.

- Option 2: Reuse existing IEs at least including configuration of Set A, Set B and reportQuantity.

Proposal 4: Support a priority based mechanism to handle duplicated activated periodic CSI report configs.

Proposal 5: For the configuration of Set A and Set B, no configuration method is supported other than configuration of two CSI-ResourceConfig Ids for Set A and Set B separately.

Proposal 6: Support Set A for both no transmission and transmission with longer periodicity.

Proposal 7: Support CSI resource configs with/without physical RS transmission parameters (e.g., resourceMapping, CSI-ResourcePeriodicityAndOffset and etc.).

Proposal 8: Support configuration of RS resources to estimate QCL-parameters for a RS resource without transmission in Set A.

Proposal 9: Support a beam indication mechanism with a beam pattern and corresponding TCI states required for the indicated beam pattern.

Proposal 10: Support a shared CPU counter between legacy and AIML-based CSI reporting.

Proposal 11: Support an overhead reduction mechanism for reporting data collection for inference (e.g., based on channel conditions).

Proposal 12: Support reporting of L1-RSRPs and corresponding beam information of up to M beams within X dB gap to the largest measured value of L1-RSRP.

Proposal 13: Support a pattern-based reporting mechanism for data collection for inference wherein the UE reports a pattern ID(s) and corresponding beam measurements of a subset of beams in Set B.

Proposal 14: Information about the time stamp for measurement instances should be supported.

Proposal 15: Reporting prediction results of multiple future time instances in one report should be supported. Beam inference reporting periodicity should be aligned with CSI-Reporting periodicity as a baseline.

Proposal 16: Support “Option 2: Based on the CSI reference resource corresponding to the report” for determination of reference time.

Proposal 17: Support flexible configuration of multiple future time instance reporting (e.g., based on channel conditions, gNB configuration).

Proposal 18: Support both

‘Option A: Predicted RSRP’

‘Option B: Predicted RSRP, if the beam is not configured for corresponding measurement, and measured L1-RSRP if the beam is configured for corresponding measurement.’

Additionally, support a UE based selection procedure to report according to Option A or Option B.

Proposal 19: For network sided model, support enhanced UE reporting to report up to 64 RSRP values for whole Set A over multiple time instances.

- No CRIs/SSBRIs are reported and implicit beam indexes (e.g., by association with RSs and reported RSRPs) are used.

- Information on measured past instances (e.g., time stamp) is supported.

Proposal 20: Indicating configuration information associated with Set B to UE should be supported.

Proposal 21: Support reporting of UE selected Set B based on a rule (e.g., subset of best measured beams).

Proposal 22: For UE-assisted performance monitoring, Support TCI-state based activation of RS resource sets for performance monitoring.

Proposal 23: For UE-assisted performance monitoring, for the case when Top-K beams are included in the monitoring resource set, down select from one of the following performance metrics:

- Option 2 (1/Top-K): The Top-1 beam with largest measured value of L1-RSRP of the resource set(s) for monitoring is one of the Top-K predicted beams

- Option 3 (Top-K/M): The Top-K predicted beams are among Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of the resource set(s) for monitoring.

- Option 4 (best of Top-K/1 with margin): The beam with largest measured value of L1-RSRP of Top-K predicted beams is within a margin X dB of largest measured value of L1-RSRP of the resource set(s) for monitoring

Proposal 24: For UE-assisted performance monitoring, for the case when Top-K beams are not included in the monitoring resource set, support a performance metric based on difference between measured RSRPs and predicted RSRPs.

Proposal 25: For UE-assisted performance monitoring, support the following options for activation of performance monitoring at UE:

- Based on the reception of RSs associated to Set A beams.

- Based on a periodic timer which starts/stops performance monitoring procedure based on gNB signaling or model performance.

Proposal 26: Support a UE based indication of AIML model validity.

Proposal 27: Support a procedure to dynamically switch AIML inference location (e.g., based on NW workload and/or UE’s prediction performance).

# References

- RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Work Item Description,

- RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Study Item Description,

- 3GPP TR38.843 V18.0.0 (2023-12), “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”,

- 3GPP TS38.214 V18.1.0 (2023-12), “Physical layer procedures for data (Release 18)”,

- “Final Report of 3GPP TSG RAN WG1 #116”, 3GPP RAN1 Meeting #116, February 26th – March 1st, 2024,

- “Final Report of 3GPP TSG RAN WG1 #116-bis”, 3GPP RAN1 Meeting #116-bis, April 15th – April 19th, 2024,

- “Final Report of 3GPP TSG RAN WG1 #117-bis”, 3GPP RAN1 Meeting #117-bis, May 20th – May 24th, 2024,

- “Final Report of 3GPP TSG RAN WG1 #118”, 3GPP RAN1 Meeting #118, August 19th – August 23rd, 2024,

- R1-2407604, “LS on applicable functionality reporting for beam management UE-sided model”,

- “Final Report of 3GPP TSG RAN WG1 #118-bis”, 3GPP RAN1 Meeting #118-bis, October 14th – October 18th, 2024.

- “Final Report of 3GPP TSG RAN WG1 #119”, 3GPP RAN1 Meeting #119, November 18th – November 22nd, 2024,

- R1-2409114, “FL summary #0 for AI/ML in beam management”.