3GPP TSG RAN WG1 #120			R1-2500530

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	9.6.1

Source:	InterDigital, Inc.

Title:	Discussion on LP-WUS and LP-SS design

Document for:	Discussion and Decision

# Introduction

In RAN#102 [1], a work item on low-power wake-up signal and receiver for NR (LP-WUS/WUR) was agreed. Based on the work item scope, in RAN#103 [2] and RAN#105 [7], the work scope has been updated including the following work scope:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements.

In this contribution, we discuss LP-WUS and LP-SS design framework.

# Discussions

## LP-SS design framework

The time and frequency synchronization procedure needs enhancements for operation with low-power-wake up receivers (LP-WUR). In systems based on low-power wake-up signaling, the LP-WUR is configured with monitoring windows to monitor and detect potential LP-WUSs. The LP-WUR could be configured with duty cycle for the monitoring occasions, where the duty cycle and monitoring windows should be selected to match with LP-WUS transmission time from NW. In NR, the time and frequency synchronization are based on receiving SSBs and using PSS/SSS for synchronization.

In systems based on low-power wake-up signaling, the UE could receive the SSBs during main receiver’s (MR) “ON mode”, where the UE could use the received SSB for synchronization. However, in cases where the MR is configured with long “OFF mode” or sleeping periods, the clock frequency could drift at the UE. The clock frequency drift or frequency error could result in inaccuracy in LP-WUR’s duty cycle. The difference in NW’s clock and LP-WUR’s clock frequency could result in time mismatch between the LP-WUS transmission time from NW and LP-WUR’s monitoring window. The time mismatch could lead to failed detection of LP-WUS.

To avoid the time mismatch between the LP-WUS transmission time from NW and LP-WURs monitoring window, the UE could be configured to detect and receive periodic low power synchronization signals (LP-SS) to achieve accurate synchronization at the LP-WUR. Based on the discussions in study item phase on LP-SS, as captured in TR38.869 [2], and the objectives of the work item, the LP-SS could be based on OOK-1 and/or OOK-4 waveforms, where the LP-SS could be with or without overlaid OFDM sequences.

### LP-SS Periodicity

In RAN1 #119 [10], following agreement was agreed:

Agreement Regarding whether to support additional sync signal to LP-SS, down-selection from the following: - Option 1A: No additional sync signal, LP-SS periodicity =320ms - Additionally support at least one of [80ms,160ms] - FFS: whether different values can be applicable for different M values - Option 1B: No additional sync signal, LP-SS periodicity =320ms - No additional support of other periodicities - Option 2: With additional sync signal, LP-SS periodicity =320ms - FFS additionally support other LP-SS periodicities - FFS details on additional sync signal - FFS additional sync signal is configurable and/or conditionally present

In the above agreement, different options on the periodicity of LP-SS as well as whether to support additional sync signal are provided for down selection. In Options 1A and 1B, this additional sync signal is not supported, and LP-SS periodicity is considered to be 320ms. As for other periodicities for LP-SS, Option 1B considers no additional support of other periodicities, whereas Option 1A proposes additionally supporting 80ms and 160ms periodicities. In Option 2, additional sync signal is supported.

Compensating the frequency drift in determining DL timing for receiving LP-WUS is one of the main reasons of introducing and using LP-SS. This can be achieved by LP-SS periodicity of 320ms. Using shorter periodicities could increase the synchronization overhead without remarkable added efficiency. Therefore, we support no additional other periodicities.

On the other hand, adding an additional sync increases the complexity at the receiver for receiving and processing the additional sync signal. Also, adding additional sync signal to the LP-SS could increase the overhead even higher than LP-SS with shorter periodicities.

Observation 1. Considering shorter periodicities for the LP-SS or additional sync signal increases the synchronization overhead as well as receiver’s complexity.

Proposal 1. Support Option 1B with no additional sync signal, LP-SS periodicity = 320ms, and no additional support of other periodicities.

### LP-SS Waveform Selection

In RAN1 #116, LP-SS waveform properties were discussed. It was discussed that it is desired for LP-SS to have similar modulation formats as LP-WUS, to achieve unified design for both channels. The following agreement was made:

Agreement Further study the following options for LP-SS: - Option 1: OOK-1 - Option 2: OOK-4 with M=1,2,4,[8] - The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation - FFS: different SCS

In RAN1 #116-bis [5], following working assumption was made:

Working Assumption Support the following options for LP-SS - Option 1: OOK-1 - Option 2: OOK-4 with M=2,4, FFS:1,8,16 - FFS whether value of M depends on SCS - The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation FFS how OOK-1 and OOK-4 are specified

In the working assumption, two options are provided, where Option 1 considers OOK-1 for LP-SS waveform design and Option 2 considers OOK-4 for LP-SS waveform design. In Option 2, the modulation order for OOK-4 is considered to be M = 2, 4, and the other values for M = 1, 8, and 16 are left for FFS.

In RAN1 #119, the following agreement was agreed:

Agreement For the M value for LP-WUS and LP-SS, down-select one alternative from the following in RAN1#120: - Alt1: The M values for LP-WUS and LP-SS are always same. - Alt2: The M values for LP-WUS and LP-SS can be configured to be same or different. M value for LP-WUS cannot be larger than that of LP-SS. - Alt3: The M values for LP-WUS and LP-SS can be configured to be same or different.

In this agreement, different alternatives on M values for LP-SS and LP-WUS are proposed for down selection. In Alt1, the same M values are considered, whereas in Alt2 and Alt3, the M values are proposed to be the same or different.

Considering the below agreement in RAN1 #118, LP-WUS is already agreed to support M=2 and M=4 (working assumption) with OOK-4 and M=1 is FFS. That is, it is agreed to support limited number of M values for LP-WUS.

Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4

As mentioned above, LP-SS is the periodic signaling that is used by LP-WURs to obtain and maintain the synchronicity in time and frequency. Also, the LP-WURs could use LP-SS for RRM measurements to determine the potential coverage and detectability for further LP-WUS reception and detections. As such, the LP-SS waveform design should consider the complexity issues as one main criteria. Especially, the repetitive nature of RRM measurements based on LP-SS is another reason for carefully deciding the LP-SS waveform design and applying low complexity solutions.

In case LP-SS is considered with different values of M for OOK-4, this diversity may cause undesired complexity at LP-WURs, as LP-WURs will be obliged to detect LP-SS with different hypothesis based on different OOK modulations and values of M. Consider a UE configured with an LP-SS (binary) sequence for serving cell and several LP-SS sequences for non-serving cells, where the different LP-SS sequences could potentially have different OOK modulations with different M values. In this case, the UE needs to switch the LP-WUR modulation order for each LP-SS measurement for the serving cell as well as for each of the non-serving cells. This could result in time gaps, higher complexity, as well as undesired power consumption.

Observation 2. In LP-SS based on OOK-4 with diverse values of M, the LP-WURs experience frequent need to change modulation order resulting in increased implementation complexity due to configured LP-SS having OOK-4 with different M values in serving and non-serving cells.

Proposal 2.  Confirm the working assumption on modulation orders for LP-SS to include both Option 1 with OOK-1 and Option 2 with OOK-4 and M = 2 or 4.

- Do not support additional M values.

Considering our abovementioned arguments, it is necessary to support limited range of M values for LP-SS for the sake of complexity at the LP-WUR. As such, we support Alt1, where the M values for LP-WUS and LP-SS are always same.

Proposal 3.  Support Alt1 for M values selection, that is, the M values for LP-WUS and LP-SS are always same.

### LP-SS Binary Sequences

#### LP-SS Binary Sequences Length

In RAN1 #118-bis, below agreement on the different lengths for LP-SS binary sequences was agreed:

Agreement To determine the binary sequences for LP-SS, for each M value, down-select the sequence length L from the corresponding candidate values: - M=1, L= {4,6,8} - M=2, L= {8,12,16,24} - M=4, L= {16,24,32,56} - FFS whether one or more than one L value (s) applied to each of M values M=1,2,4 - FFS the L values applied to other applicable SCS(s) - FFS the L values if other M values in addition to M=1,2,4 are supported. - FFS whether the time estimation averaged across multiple LP-SS occasions is applied - Note 1: with sequence length L, it includes Manchester coding (if supported for LP-SS) - Note 2: This doesn’t preclude any of three agreed sequence types Above applies at least for both 15kHz and 30kHz.

In RAN1 #119, the following agreement was agreed:

Agreement For the length L of LP-SS binary sequence, limit the selection to the following: - M=1, L= {4, 6, 8} - M=2, L= {8, 12, 16} - M=4, L= {16, 32}

In this agreement, different lengths (L) for LP-SS binary sequences with OOK-4 and different M values are proposed. It is left for FFS on whether one or more than one of the proposed L values should be applied per M value.

Considering LP-SS sequences with different lengths could result in different number of OFDM symbols. For a UE in Idle/Inactive mode trying to blindly detect the LP-SS sequences with possible different lengths, this could result in more trial and errors and more missed detection of LP-SSs. This could increase the complexity, latency, power consumption, etc. As a result, one L value should be selected for each M value.

Observation 3. In determining the binary sequences for LP-SS, considering more than one L-value per M-value could result in increased complexity, latency, and power consumption at UE-side.

Proposal 4. In determining the binary sequences for LP-SS, support selecting one L-value applied to each M-value.

On the other hand, in determining the length for binary sequences for different M-values, it should be noted that the number of OFDM symbols used for LP-SS should remain the same for different M-values, regardless of the M-value. This is again to avoid the UE in Idle/Inactive mode to blindly detect LP-SSs with different number of OFDM symbols resulting in increased complexity, latency, and power consumption.

As such, the L-values for M=1, 2, or 4 should be scaled with the M values so that the number of OFDM symbols is the same for different M values. For example, if L1=8 is selected for M=1, the L2 value for M=2 and L3 value for M=4 should be selected in association with L1. That is, L2 = 16 and L3 = 32 should be selected.

Observation 4. In determining the binary sequences for LP-SS, the number of OFDM symbols could be different if the selected L-values for different M-values are not scaled with regards to each other.

Proposal 5. In determining the binary sequences for LP-SS, support selecting the L-value applied to different M-values, such that the number of OFDM symbols is the same for different M-values.

In RAN1 #118-bis meeting, the results of considering different binary sequences for LP-SS to satisfy both sync accuracy requirement and RRM measurement accuracy requirement were provided. Based on the results, the sync accuracy and RRM measurement accuracy could be fulfilled with larger LP-SS binary sequences. Considering the L values in the above agreement, we propose to use L = 8 for M = 1, L = 16 for M = 2, and L = 32 for M = 4.

Proposal 6. In determining the binary sequences for LP-SS, support following L-values:

- For M = 1, support L = 8,

- For M = 2, support L = 16,

- For M = 4, support L = 32.

#### RRM measurements based on LP-SS Binary Sequences

In RAN1 #118, many discussions on LP-SS binary sequence design took place. With regards to selecting the binary sequences for LP-SS, the discussions led to the evaluation assumptions for OOK-based LP-WUR, based on which the following agreement was made:

Agreement To determine the binary sequences for LP-SS, the evaluation assumes the following: - Sync accuracy: timing estimation error smaller than T us for P=90 % of the time for at least SNR=-3dB and SNR=-6dB (lower priority), - T=2us(optional), 5us for OOK-1. - T=1us(optional), 2us for OOK-4 with M=2. - T=0.5us(optional), 1us for OOK-4 with M=4. - Other values of SNR, T, M are up to company report. - Additionally, companies can submit results on SINR with details on how the interference was modelled. - Assume one-shot estimation. - The time error is based on 20ppm maximum frequency error for the detection of the first LP-SS. - Note: Companies can assume other values inside of 20ppm maximum as long as the values are justified. - RRM measurement accuracy: measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s for at least SNR=-3dB and SNR=-6dB (lower priority), X, Y, and Z is up to company report, other values of SNR are up to company report. - As a starting point, the time error is based on 5-10ppm residual frequency error. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance. - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified. - The frequency error: up to company report. - Sampling rate: 3.84MHz (optional) or 7.68MHz assumed for 30kHz SCS. - Channel: AWGN and TDL-C 300ns is assumed. - Consider cross-correlation for 4 binary sequences under time and frequency error. - Companies are encouraged to provide details on other additional simulation assumptions, high level description of their receiver algorithm, and assessment on power consumption.

In RAN1 #118-bis, following agreement on performance accuracy based on binary sequences for LP-SS was agreed:

Agreement To determine the binary sequences for LP-SS, consider the following for performance comparison: - Sync accuracy T’: the achieved sync accuracy T’ for a set of 4 sequences with the same number of occupied OFDM symbols (timing estimation error smaller than ≤T’ us for P=90 % of the time for SNR=-3dB, -6dB (low priority)) - Assume the sliding window size is at least [-25us, 25us]. - RRM measurement accuracy X: the achieved measurement accuracy within range ± XdB for Q=90% measurements based on Y LP-SS samples within a period comparable to Z=the length of I-DRX cycle that is larger or equal to 1.28s - Assume X= 3.5dB shall be satisfied for LP-RSRP and LP-RSRQ, based on Y=1, 4 and Z=1.28s for comparison - Cross correlation: maximum cross-cor value/ maximum auto-cor value - FFS how to use cross-correlation for comparison - Note: refer to RAN4 RRM assumption for the inter-cell interference

In these agreements, both synchronization accuracy and RRM measurement accuracy aspects are taken into consideration. As captured in the agreement, RRM measurement usually has timing error and frequency error even after adjusting synchronization based on LP-SS. Such errors may result in inaccuracies in RRM measurements causing inconsistencies in triggering events based on the measurements.

Consider a UE that is configured to perform RRM measurements based on LP-SS in the serving cell. The UE can identify events based on RRM measurements, where for example, the RRM measurements from the serving cell based on LP-SS is lower than corresponding thresholds. As such, the UE could wake up MR for performing further measurements and calculations based on NR-SS, SSB, or other RSs as part of cell (re)selection or CHO procedures.

In a special case, if the measurements via MR based on NR-SS are not in line with the measurements via LP-WUR based on LP-SS, this could result in frequent false waking up of the MR, resulting in unnecessary power consumption. That is, while the measurements based on LP-SS via LP-WUR indicate an event (e.g., low RRM measurements) that requires waking up the MR, the measurements based on NR-SS via MR could show that triggering the event and the waking up the MR was not required due to RRM measurements being in the acceptable range, i.e., higher than corresponding thresholds.

Observation 5. Inaccuracy in RRM measurements based on LP-SS could cause inconsistency between RRM measurements based on LP-SS and NR-SS that could result in frequent false MR wake up.

Proposal 7. Procedures for handling inconsistencies in RRM measurements based on LP-SS and RRM measurements based on NR-SS, e.g., due to RRM measurements inaccuracies based on LP-SS, should be supported.

#### Configured LP-SS Binary Sequences

In RAN1 #116, it was agreed that LP-SS is based on binary sequences, as below:

Agreement The ‘ON-OFF’ pattern for OOK symbols of LP-SS is based on binary sequence(s) - FFS binary sequence(s) details, including the sequence type, the number of sequences, and the sequence length. - FFS overlaid OFDM sequences, if supported.

In RAN1 #116-bis, it was agreed that multiple binary LP-SS sequences for the ‘ON-OFF' pattern may be used, where the multiple sequences are used for avoiding inter-cell interference as well as for differentiating LP-SS from different cells, see below:

Agreement Support to specify multiple binary LP-SS sequences for the ‘ON-OFF’ pattern: - The LP-SS sequence used in a cell is - Option 1: a sequence is configured - Option 2: a sequence is determined by predefined rule - FFS: Whether both options will be supported or only one will be supported - FFS: the number of LP-SS sequences - Note: Multiple sequences are used to differentiate LP-SS from different cells

In NR, the UEs determine the cell settings and configurations including Cell-ID during initial access based on the detected SSB and after decoding the PSS and SSS and the corresponding sequences. The UEs monitor and detect the PSS followed by SSS, based on blind detection and that is the reason for which the sequences used in PSS and SSS are determined based on a predefined rule.

However, in the LP-SS sequence do not necessarily need to be identified based on a Cell-ID as the UEs receive configurations on the LP-SS time and frequency resources, as well as corresponding LP-SS sequences during the wake-up mode, via MR, and as part of LP-WUR RRC configurations (e.g., for CONNECTED mode) or system information (e.g., for IDLE/INACTIVE modes).

In RAN1 #117, Option 1 was agreed to have the information required for determining the sequence is explicitly configured. It was left as FFS on whether to support determining the LP-SS sequence based on predefined rule. Below is the agreement:

Agreement For the LP-SS sequence used in a cell, - Option 1: the information necessary for determining the sequence is explicitly configured - FFS: Additional support of determining the sequence by predefined rule without configuration

LP-SS sequence configured

Consider a UE that is configured with an LP-SS sequence and corresponding time and frequency resources for the serving cell in addition to several other LP-SS sequences and time and frequency resources for the neighbour/non-serving cells. The UE uses the LP-SS sequence and configurations corresponding to the serving cell for synchronization within the serving cell as well as RRM measurements by using LP-WUR. In case the RRM measurements for the serving cell show lower measured values (e.g., RSRP, RSRQ, RSSI, SNR, etc.) the UE could use the non-serving cells’ LP-SS sequences and configured time and frequency resources for respective RRM measurements.

In our opinion, in case the LP-SS sequences are configured, this gives more flexibility to gNB to control the sequences in different cells. So, gNB can dynamically change the LP-SS sequences and inform the UEs of the changes based on traffic or scheduling requirements.

Observation 6: In specifying binary LP-SS sequences, in case the LP-SS sequences are configured, gNB can dynamically change and update LP-SS sequences and indicate it to UEs, based on traffic or scheduling requirements.

Also, in case the candidate LP-SS sequences corresponding to non-serving cells are configured for UEs, gNB can control the traffic flow toward different cells. That is, gNB can intentionally not configure a neighbour cell’s LP-SS sequence for RRM measurement for a number of low-priority UEs. As such, gNB can allow only a specific (high priority) UEs to measure, select, HO, or connect to that specific neighbour cell, where gNB configures the LP-SS sequence corresponding to the specific cell as one of the candidate cells for the respective UEs.

Observation 7: In case the LP-SS sequences are configured, gNB can control traffic toward cells by not configuring corresponding LP-SS sequences as candidate non-serving cells for some specific UEs, based on UE capabilities, priorities, latency requirements, gNB preferences, etc.

LP-SS sequence determined based on predefined rule

In LP-SS based on predefined rules, the UEs can select any cell, for example during cell (re)selection procedure, since the UEs can determine the cells’ settings such as Cell-IDs potentially based on the received LP-SS of any of the neighbouring non-serving cells. In this case, the UE may need to switch cells or HO later due to traffic issues and gNB preferences, where such frequent cell switching could degrade the performance by increasing the latency as well as power consumption.

Observation 8: In case the LP-SS sequences are determined based on predefined rules, gNB does not have that much control on cell (re)selection procedure. This may result in frequent or ping-pong cell (re)selections causing increased latency, complexity, and unnecessary power consumption.

Proposal 8. In specifying binary LP-SS sequences, do not support determining the sequences by predefined rule without configuration.

### Configurability of overlaid OFDM sequence(s) for LP-SS

In RAN1#118bis [9], the following agreement was made on the configurability of overlaid OFDM sequence(s) for LP-SS.

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

According to the discussion from the SI, the benefits of supporting overlaid OFDM sequence for LP-SS and LP-WUS are two-fold. Firstly, OFDM based LR, which can be relatively easily implemented by using existing NR receiver and show better coverage, can be utilized as well as OOK based LR, which consumes less power. In addition, application of overlaid OFDM sequence reduces interference and leakage to NR bands, so that specification support and additional implementation can be minimized. For LP-SS, the former benefits may not be applicable as OFDM based LR may receive NR-SS, but the latter benefits are still crucial and need to be supported.

The above agreement allows configurability of overlaid OFDM sequence for OOK-1. According to the supporting companies, implementation based overlaid OFDM sequences can be easily utilized by transmitting existing NR signals (e.g., PDCCH) in OOK ON symbols for OOK-1, but no clear solution was suggested for OOK-4 with M>1. In addition, as supporting OOK-4 with M>1 was agreed as a working assumption, transmission of the existing NR signals without DFT/LS operation is not aligned with the agreed OOK-4 with M=1. As well as OOK-4 with M>1, how to support implementation-based solution for OOK-4 with M=1 is also not clear. If a sequence which is similar to specified ZC sequence is applied by gNB implementation, we seriously doubt that what would be additional benefits by supporting implementation-based solution rather than using well discussed/evaluated/defined sequences from RAN1 discussion.

Observation 9: Overlaid OFDM sequence should be always applied to LP-SS to reduce interference and leakage to other NR bands.

Observation 10: As OOK-4 with M=1 is supported by the working assumption, how to implement gNB-implementation based overlaid OFDM sequence is also not clear as well as OOK-4 with M>1.

Observation 11: If an overlaid OFDM sequence similar to specified ZC sequence is applied before DFT/LS operation by gNB implementation, the benefits of utilizing implementation-based solution, instead of utilizing well discussed and specified solution, for overlaid OFDM sequence of LP-SS are doubted.

Proposal 9. Do not support configurability of overlaid OFDM sequence for LP-SS in case of OOK-4 with M>1.

- Configurability of overlaid OFDM sequence for LP-SS in case of OOK-4 with M=1 is also revisited.

## LP-WUS design framework

### Discussion on LP-WUS design

In RAN1#118 [8], support of Manchester coding was agreed as a coding scheme of LP-WUS. In addition, the following coverage enhancement schemes were listed as candidates for meeting target requirements in addition to Manchester coding in RAN1#118bis [9].

Agreement Down-select among the following alternatives for LP-WUS information for OOK to meet target requirements: - adding CRC - channel coding other than Manchester coding, including rate matching if any - mapped to a binary sequence/codeword, include truncation if any - repetition, including rate matching if any - Note: combination of options above is not precluded - Note: combination of Manchester coding with above is assumed FFS how LP-WUS is transmitted within a MO/MOs

As LR is a receiver being implemented in addition to MR, it is important to achieve both low implementation complexity for reducing the cost and low power consumption for achieving better power saving gain. In this context, time domain repetition is the most practical coverage enhancement scheme considering their implementation simplicity, low power consumption and achievable coverage.

Observation 12. Time domain repetition is the most practical coverage enhancement schemes considering their implementation simplicity low power consumption and achievable coverage.

Proposal 10. For LP-WUS signal structure, time domain repetition is supported to meet target requirements in addition to Manchester coding.

### Evaluation results for LP-WUS coverage

In this section, we provide link level evaluation results for the performance of LP-WUS based on overlaid OFDM with ZC sequences over OOK symbols. Different types of receivers (i.e., energy detection for OOK receivers and time domain correlation for OFDM receivers) and different number of repetitions with Manchester coding were considered for OOK-1 and OOK-4 with M=4 compared to PUSCH for Msg 3. Figure 1 and 2 show evaluation results of OOK schemes compared to PUSCH for Msg3 in 700 MHz (15 kHz SCS) and 4 GHz (30 kHz SCS), respectively.

(a) OOK-1						(b) OOK-4 (M=4)

Figure 1 Evaluation results for OOK-1/4 in 700 MHz and 15 kHz SCS

(a) OOK-1						(b) OOK-4 (M=4)

Figure 2 Evaluation results for OOK-1/4 in 4 GHz and 30 kHz SCS

It can be observed from Figures 1­2 that, OFDM receivers with time domain correlation detector provide better performance than OOK receivers with energy detection in OOK-1 with overlaid OFDM. For example, in 700 MHz with 15 kHz SCS, OFDM receivers achieve better performance than PUSCH for Msg 3 even without repetition, on the other hand, OOK receivers require 2 and 5 repetitions to achieve comparable performance with PUSCH for Msg 3 without repetition and 4 repetitions, respectively. In 4 GHz with 30 kHz SCS, both OFDM and OOK receivers require more number of repetitions (i.e., 10 repetitions for OOK and 5 repetitions for OFDM) to achieve comparable performance with PUSCH for Msg 3. For OOK-4, obviously, both OFDM and OOK receivers require more repetitions than OOK-1 as OOK-4 multiplexes 4 bits in one OFDM symbol. However, for OOK-4 with overlaid OFDM, OFDM receivers tend to experience more performance degradation compared to OOK receivers due to reduced correlation window and sequence. For example, OFDM receivers with 10 repetitions even show worse performance than OOK receivers without repetition. In general, for OOK-4 with up to 10 repetitions, both OFDM and OOK receivers do not achieve comparable performance with PUSCH for Msg 3, but this can be acceptable as OOK-4 will be used only in limited high SINR cases.

Observation 13. For OOK-1, OFDM receivers with time domain correlation provide better performance than OOK receivers with energy detection.

- To achieve comparable performance with PUSCH for Msg 3, OOK receivers require 10 repetitions while OFDM receivers require 5 repetitions.

Observation 14. For OOK-4, OFDM receivers experience more performance degradation compared OOK receivers due to reduced correlation window and sequence.

- In general, for OOK-4 with up to 10 repetitions, both OFDM and OOK receivers do not achieve comparable performance with PUSCH for Msg 3, but this can be acceptable as OOK-4 will be used only in limited high SINR cases.

Proposal 11. Support up to 10 repetitions of LP-WUS with Manchester coding to achieve comparable performance with PUSCH for Msg 3.

For the support of OOK-4 with M=4, while the support of 15 kHz was confirmed, the support of 30 kHz has remained as a working assumption in RAN1#118 [8].

Agreement [H][FL1] Proposal 3.1-1: Confirm the Working Assumption that OOK-4 with M=4 for 15KHz SCS is supported for LP-WUS. Agreement For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - M=4 for 15KHz SCS - M=4 for 30KHz SCS (working assumption) - FFS M=1 for OOK-4.

As shown in Figure 2, OOK-4 with M=4 in 30 kHz provides relatively lower performance than OOK-1. However, OOK-4 with M=4 and 10 repetitions shows similar performance with PUSCH for Msg3 without repetition. Considering the efficient resource utilization from higher SCS/large M and simple specification support without consideration on SCS, it is preferred to confirm M=4 for 30 kHz.

Proposal 12. Support of M=4 for 30 kHz is confirmed.

### Evaluation results for overlaid OFDM sequences

In RAN1#118bis [9], a working assumption was made on overlaid OFDM sequences.

Working Assumption For overlaid OFDM sequences for LP-WUS in time or frequency domain, for OOK-1 and OOK-4 M=1 - Option 1-1(compromised): overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing for OOK-4 with M=1. - ZC sequence in time domainDFT/LSIFFT to result in a frequency domain sequence that is ZC sequence or a sequence that is close to ZC sequence, no additional operation - It doesn’t preclude additional operation for OOK-4 with M>1. - DFT size is 2^n Note1: OOK-1 is considered as specific case of OOK-4 with M=1. Note: Concerns were raised by Ericsson and CATT that the above is not beneficial in terms of implementation complexity

In the following, we provide BLER comparison among ZC sequence, M-sequence and Gold sequence in Figure 3.

(a) 700 MHz with 15kHz				(b) 4 GHz with 30kHz

Figure 3 BLER results for overlaid OFDM sequences

As shown in the above, M-sequence and ZC sequence provide better BLER performance compared to Gold sequence. While Gold sequence provides comparable performance in OOK-4 with M=1, the sequence provides significant performance degradation in OOK-4 with M=2 and 4, especially in 4 GHz. Based on the observation, Figure 4 provides timing offset estimation performance comparison between ZC sequence and M-sequence.

(a) 700 MHz with 15kHz				(b) 4 GHz with 30kHz

Figure 4 Timing offset estimation for overlaid OFDM sequences

Although M-sequence generally provides better BLER performance assuming identical timing offset, as shown in the above, ZC sequence achieves better timing offset especially in low SNR for 4 GHz. As increased timing offset significantly impacts on the performance of OOK-4 with M>1, more performance degradation is expected for M-sequence. While M-sequence and ZC sequence show performance benefits in either of BLER performance or timing offset, ZC sequence provides better implementation flexibility considering for DFT operation as combination of DFT operation and ZC sequence can be represented as another ZC sequence.

Observation 15. For BLER, M-sequence and ZC sequence generally provides best performance.

- While Gold sequence provides comparable performance in OOK-4 with M=1, Gold sequence provides significant performance degradation in OOK-4 with M=2 and 4, especially in 4 GHz.

Observation 16. For timing offset, ZC sequence achieves better timing offset especially in low SNR for 4 GHz.

Observation 17. ZC sequence provides simpler implementation when DFT is utilized.

Proposal 13. Confirm the working assumption with OOK-4 M=1 and ZC sequence for overlaid OFDM sequences.

### Discussion on LP-WUS Information

## Triggering PDCCH monitoring for CONNECTED mode

In RAN1#119 [10], the following proposal was discussed on LP-WUS information for CONNECTED mode.

For future meetings: Companies are encouraged to consider the following for future discussions Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, - Option 1: A bitmap with each bit corresponding to [one or more] UEs - 1A No CRC - Per bit coding, e.g., one bit is mapped to one of 2 codeword/sequences - The maximum number of codewords targeting per bit position in the bitmap for a UE is 1 - FFS coded bit length - Single MO - 1B No CRC - Bitmap-level coding, e.g., one bitmap of L bits is mapped to one of 2L codeword/sequences - The maximum number of codewords/sequences targeting for a UE is 2L-1 - Single MO - 1C With CRC - FFS CRC length - Single MO - Option 2: A codepoint value corresponding to [one or more] UEs - 2A only 1-1 mapping from codepoint to UE - The maximum number of codepoints targeting per MO for a UE is 1 - FFS the maximum number of MOs to be detected by a UE - 2B 1-to-1, 1-to-all mapping from codepoint to UEs - The maximum number of codepoints targeting per MO for a UE is up to 2 - FFS the maximum number of MOs to be detected by a UE - 2C 1-to-X mapping from codepoint to UEs, - The maximum number of codepoints targeting per MO for a UE is up to - FFS the maximum number of MOs to be detected by a UE - Note: L is the number of UEs associated monitoring the same WUS

While Option 1 (a bitmap) supports the simplest solution for indicating subgroups that needs to be supported, Option 1 requires relatively larger container size than other options when a same number of UEs are triggered for PDCCH monitoring. Although probability to wake up multiple UEs can be relatively higher compared to IDLE/INACTIVE modes, need for supporting full flexibility is still low. In that regard, supporting Option 1 can be inefficient as Option 1 consumes a bit for each UE all the time. On the other hand, Option 2 (a codepoint value) can provide better time/frequency resource efficiency by adopting encoding mechanism. Among the sub-options of Option 2, 2B should be supported to enable triggering each UE and all the UEs when multiple UEs need to be activated.

Proposal 14.  For CONNECTED mode, Option 2A ‘A codepoint value with 1-to-1, 1-to-all mapping from codepoint to UEs’ is supported.

## Extension for the UEs configured with CA

DCP supports wake-up indication and SCell dormancy. Therefore, to avoid limiting the use of LP-WUS monitoring, supporting both wake-up indication and SCell dormancy indication in LP-WUS is essential. In the case only wake-up indication is supported in LP-WUS, UEs configured with CA needs to decode DCP to acquire SCell dormancy related information before receiving PDCCH scheduling PDSCH/PUSCH and the additional latency/power consumption is expected due to additional decoding behavior.

Proposal 15. For LP-WUS information in CONNECTED mode, support wake-up indication and SCell dormancy indication when UE is configured with CA.

In DCP, each UE can receive up to 6 bits; 1 bit for wake-up signal and up to 5 additional bits for SCell dormancy indication. Using a similar design for indicating SCell dormancy via LP-WUS is not efficient. For example, in the case UE is indicated not to wake-up, bits allocated for SCell dormancy indication are wasted/unused. Therefore, a new design which optimizes the use of available LP-WUS payload is needed. To this end, joint indication of wake-up indication and SCell dormancy indication can be considered. For example, considering the requirements of each UE (e.g., number and required activation pattern of SCells/SCell groups, etc.), gNB can configure a set of codepoints for the UE and each codepoint can indicate wake-up indication as well as required dormancy indication for SCells based on the configuration.

Proposal 16. When UE is configured CA, support SCell dormancy indication by configuring codepoints for joint indication of wake-up indication and SCell dormancy indication.

# Conclusion

In this contribution, we discussed LP-WUS and LP-SS design frameworks. From the discussions, we made following observations and proposals:

Observation 1. Considering shorter periodicities for the LP-SS or additional sync signal increases the synchronization overhead as well as receiver’s complexity.

Observation 2. In LP-SS based on OOK-4 with diverse values of M, the LP-WURs experience frequent need to change modulation order resulting in increased implementation complexity due to configured LP-SS having OOK-4 with different M values in serving and non-serving cells.

Observation 3. In determining the binary sequences for LP-SS, considering more than one L-value per M-value could result in increased complexity, latency, and power consumption at UE-side.

Observation 4. In determining the binary sequences for LP-SS, the number of OFDM symbols could be different if the selected L-values for different M-values are not scaled with regards to each other.

Observation 5. Inaccuracy in RRM measurements based on LP-SS could cause inconsistency between RRM measurements based on LP-SS and NR-SS that could result in frequent false MR wake up.

Observation 6: In specifying binary LP-SS sequences, in case the LP-SS sequences are configured, gNB can dynamically change and update LP-SS sequences and indicate it to UEs, based on traffic or scheduling requirements.

Observation 7: In case the LP-SS sequences are configured, gNB can control traffic toward cells by not configuring corresponding LP-SS sequences as candidate non-serving cells for some specific UEs, based on UE capabilities, priorities, latency requirements, gNB preferences, etc.

Observation 8: In case the LP-SS sequences are determined based on predefined rules, gNB does not have that much control on cell (re)selection procedure. This may result in frequent or ping-pong cell (re)selections causing increased latency, complexity, and unnecessary power consumption.

Observation 9: Overlaid OFDM sequence should be always applied to LP-SS to reduce interference and leakage to other NR bands.

Observation 10: As OOK-4 with M=1 is supported by the working assumption, how to implement gNB-implementation based overlaid OFDM sequence is also not clear as well as OOK-4 with M>1.

Observation 11: If an overlaid OFDM sequence similar to specified ZC sequence is applied before DFT/LS operation by gNB implementation, the benefits of utilizing implementation-based solution, instead of utilizing well discussed and specified solution, for overlaid OFDM sequence of LP-SS are doubted.

Observation 12. Time domain repetition is the most practical coverage enhancement schemes considering their implementation simplicity low power consumption and achievable coverage.

Observation 13. For OOK-1, OFDM receivers with time domain correlation provide better performance than OOK receivers with energy detection.

- To achieve comparable performance with PUSCH for Msg 3, OOK receivers require 10 repetitions while OFDM receivers require 5 repetitions.

Observation 14. For OOK-4, OFDM receivers experience more performance degradation compared OOK receivers due to reduced correlation window and sequence.

- In general, for OOK-4 with up to 10 repetitions, both OFDM and OOK receivers do not achieve comparable performance with PUSCH for Msg 3, but this can be acceptable as OOK-4 will be used only in limited high SINR cases.

Observation 15. For BLER, M-sequence and ZC sequence generally provides best performance.

- While Gold sequence provides comparable performance in OOK-4 with M=1, Gold sequence provides significant performance degradation in OOK-4 with M=2 and 4, especially in 4 GHz.

Observation 16. For timing offset, ZC sequence achieves better timing offset especially in low SNR for 4 GHz.

Observation 17. ZC sequence provides simpler implementation when DFT is utilized.

Proposal 1. Support Option 1B with no additional sync signal, LP-SS periodicity = 320ms, and no additional support of other periodicities.

Proposal 2.  Confirm the working assumption on modulation orders for LP-SS to include both Option 1 with OOK-1 and Option 2 with OOK-4 and M = 2 or 4.

- Do not support additional M values.

Proposal 3.  Support Alt1 for M values selection, that is, the M values for LP-WUS and LP-SS are always same.

Proposal 4. In determining the binary sequences for LP-SS, support selecting one L-value applied to each M-value.

Proposal 5. In determining the binary sequences for LP-SS, support selecting the L-value applied to different M-values, such that the number of OFDM symbols is the same for different M-values.

Proposal 6. In determining the binary sequences for LP-SS, support following L-values:

- For M = 1, support L = 8,

- For M = 2, support L = 16,

- For M = 4, support L = 32.

Proposal 7. Procedures for handling inconsistencies in RRM measurements based on LP-SS and RRM measurements based on NR-SS, e.g., due to RRM measurements inaccuracies based on LP-SS, should be supported.

Proposal 8. In specifying binary LP-SS sequences, do not support determining the sequences by predefined rule without configuration.

Proposal 9. Do not support configurability of overlaid OFDM sequence for LP-SS in case of OOK-4 with M>1.

- Configurability of overlaid OFDM sequence for LP-SS in case of OOK-4 with M=1 is also revisited.

Proposal 10. For LP-WUS signal structure, time domain repetition is supported to meet target requirements in addition to Manchester coding.

Proposal 11. Support up to 10 repetitions of LP-WUS with Manchester coding to achieve comparable performance with PUSCH for Msg 3.

Proposal 12. Support of M=4 for 30 kHz is confirmed.

Proposal 13. Confirm the working assumption with OOK-4 M=1 and ZC sequence for overlaid OFDM sequences.

Proposal 14.  For CONNECTED mode, Option 2A ‘A codepoint value with 1-to-1, 1-to-all mapping from codepoint to UEs’ is supported.

Proposal 15. For LP-WUS information in CONNECTED mode, support wake-up indication and SCell dormancy indication when UE is configured with CA.

Proposal 16. When UE is configured CA, support SCell dormancy indication by configuring codepoints for joint indication of wake-up indication and SCell dormancy indication.

# References

RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, 3GPP RAN meeting#102, December 11th – 15th, 2023,

RP-240801, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, 3GPP RAN meeting #103, March 18th – 21st, 2024,

TR38.869, “Study on low-power wake-up signal and receiver for NR”, Release 18,

“Final Report of 3GPP TSG RAN WG1 #113”, 3GPP RAN1 Meeting #113, May, 2023,

“Final Report of 3GPP TSG RAN WG1 #116-bis”, 3GPP RAN1 Meeting #116-bis, April, 2024,

“Final Report of 3GPP TSG RAN WG1 #117”, 3GPP RAN1 Meeting #117, May, 2024,

RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)” 3GPP RAN meeting #105, September 9-12, 2024,

“Final Report of 3GPP TSG RAN WG1 #118”, 3GPP RAN1 Meeting #118, August, 2024,

“Final Report of 3GPP TSG RAN WG1 #118-bis”, 3GPP RAN1 Meeting #118-bis, October, 2024,

“Final Report of 3GPP TSG RAN WG1 #119”, 3GPP RAN1 Meeting #119, November 2024.

# Appendix – Simulation assumptions

## Common evaluation assumptions

| Parameter | Value |
| --- | --- |
| System bandwidth | 20 MHz for 700MHz / 100 MHz for 2.6GHz and 4GHz |
| Scenario and frequency | Rural: 700 MHz (FDD) UMa: 2.6 GHz and 4 GHz (TDD) |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,2,2,1,1) for 700MHz (M,N,P,Mg,Ng) = (12,8,2,1,1) for 2.6GHz and 4GHz |
| Channel model for link-level simulation | CDL-C |
| Delay spread | 300ns |
| UE velocity | 3 km/h |

## Evaluation assumptions of LP-WUS

| Parameter | Value |
| --- | --- |
| UE antenna configuration | 1 RXs for 700MHz, 2.6GHz, and 4GHz |
| SCS | 15kHz for 700MHz, 30kHz for 2.6 GHz and 4GHz |
| LP-WUS duration | 8 OFDM symbols for OOK-1, 2 OFDM symbols for OOK-4 |
| Overlaid OFDM sequence mapping | Option 1: Single overlaid sequence is on each OOK ‘ON’ symbol. |
| No. of bits | 4 bits |
| LP-WUS BW | 25 PRBs at 15 kHz SCS and 12 PRBs, 4 PRBs at 30 kHz SCS |
| LP-WUS Coding scheme | Manchester coding (code rate: ½) |
| Impairment modelling | 1-Bit ADC: Quantization error, Ideal ADC Sampling jitter – Resulting in timing error (10ppm) |

## Evaluation assumptions of PUSCH for Msg 3

| Parameter | Value |
| --- | --- |
| UE antenna configuration | 2 TXs for 700MHz, 2.6GHz, and 4GHz |
| SCS | 16 |
| Occupied RB | 2 |
| MCS, MCS Table | 0, 2 |
| DMRS overhead | 2 DMRS OFDM symbols |
| Number of repetitions | Practical |
| Frequency hopping | Inter-slot hopping |
| Channel estimation | Practical |
| Receiver type | MMSE |