3GPP TSG RAN WG1 Meeting #120                                                 	R1-2500541

Athens, Greece, 17 Feb – 21 Feb, 2025

Source:           BUPT, Spark NZ Ltd

Title:              Discussion on channel model validation of TR38.901 for 7-24 GHz

Agenda item:    	9.8.1

Document for:  	Discussion and Decision

- Introduction

In RAN1 #119, a summary on channel modelling validation for 7-24 GHz was approved [1]:

Agreement - Study further on the need for introduction of intra-cluster power profile. - Potential options for introduction of intra-cluster power profile: - Introduce unequal power per ray within a cluster and change the unequal angle offset to equal angle offset among rays within a cluster to maintain the same PAS characteristic as in TR 38.901 (example in R1-2409724) - Power scaling within a cluster based on DS associated with the cluster (example in R1-2410213)

The new mid-band is a key candidate band for future mobile communication systems [2]. The intra-cluster power profile is one of the key factors affecting the accuracy of the model. In this contribution, we provide our views on the validation and the details for Rel-19 channel modeling enhancements for 7-24 GHz. Key consideration is the modification of the cluster structure and the performance of the channel parameters.

- Cluster Structure

2.1 The ray offset angles within a cluster based on the intra-cluster power profile

In existing channel models, the power angular spectrum (PAS) is assumed to obey a Laplace distribution. The intra-cluster rays approximate the Laplace angular power distribution by utilizing equal power per multipath but unequal angular distances. However, the results of measurements at 7-24 GHz indicate that there is a dominant ray within the cluster that contains most of the cluster power, as shown in Fig. 1. This phenomenon does not match the multipath power equality of existing channel models.

Figure 1: The intra-cluster power distribution in the 3GPP model and measurement.

Therefore, we introduce the intra-cluster power factor (ICP) to characterize this phenomenon. That is, the intra-cluster power is modeled as

pm=pm=II+1∙Pn,             αm=min⁡(A)pm=1I+1∙PnM−1,             αm≠min⁡(A)

where pm represents the power of the mtℎ ray. Pn represents the ntℎ cluster power.  αm is the mtℎ ray offset angle and A is the set of the ray offset angles. I and M represent the ICP and ray number, respectively.

As the power percentage of the ray changes, the angular spacing of the ray needs to change in order to ensure the sampling theorem for PAS. The PAS obeys the Laplace variation, so its CDF is expressed as

Fθ=12exp2θ−θ0σASA,              θ<θ01−12exp−2θ−θ0σASA,          θ≥θ0

where θ and θ0 represent the ray angle and the cluster angle, respectively. σASA represents the cluster angle spread. We assume that the clusters angle is 0° and the cluster angle spread is 1°. The CDF formula of PAS was utilized to divide the power of 1 dominant ray and 19 remaining rays. The corresponding ray angles are calculated from the inverse function of Fθ, expressed as

θm=F−1m−0.5M.

Fig. 2 illustrates the PAS using the average distribution of power and after the introduction of ICP. The results indicate that the introduction of ICP with changing the power and the angular distance of the rays still makes the PAS obey the Laplace distribution.

(a)                                                                                       (b)

Figure 2: The PAS using the average distribution of power and after the introduction of ICP.

The updated Ray offset angles within a cluster are shown in Table 1.

Table 1: Ray offset angles within a cluster.

| Ray number m | Basis vector of offset angles αm |
| --- | --- |
| 1,2 | -0.0901, -0.3611 |
| 3,4 | ±0.2729 |
| 5,6 | ±0.3614 |
| 7,8 | ±0.4626 |
| 9,10 | ±05808 |
| 11,12 | ±0.7227 |
| 13,14 | ±0.9004 |
| 15,16 | ±1.1383 |
| 17,18 | ±1.4995 |
| 19,20 | ±2.2763 |

The ICP uses delay spread and normalized cluster delay (obtained by Step 5) to change the ray power in the channel coefficients 7.5-22. The performance of ICP is verified in UMa scenarios using the Gini index. We simulate the channel sparsity based on the 3GPP model framework and compare the results from measurement, 3GPP model, and ICP model, as illustrated in Fig. 3.

(a)                                                                  (b)

Figure 3. The CDF of Gini index generated by measured channels, by the 3GPP channel model, and by the 3GPP model with proposed intra-cluster power allocation model (modified model). (a) LoS scenario. (b) NLoS scenario.

The dashed lines in Fig. 3 show that the Gini index obtained from the 3GPP channel model is generally smaller than the measurement results. Additionally, the Gini index obtained from the 3GPP channel model cannot reflect the sparsity differences between frequencies, especially in NLoS condition.

The dotted lines in Fig. 3 show that the Gini index obtained by the modified 3GPP channel model with ICP are closer to the measurement results. These results indicate that the modified 3GPP channel model accurately characterizes the sparsity of mid-band MIMO channels in the UMa scenario.

Observation 1: The introduction of ICP with changing the power and the angular distance of the rays still makes the PAS obey the Laplace distribution. When ICP is introduced into the 3GPP model, the level of sparsity is closer to the measured results.

Proposal 1: The ICP model should be introduced in TR38901, which changes the angular spacing and power of the multipaths within a cluster to make the 3GPP model describe channel sparsity more accurately. The ICP can be modeled as ICP=b∙10−a∙τnDS, which is related to cluster delay and delay spread. The ray offset angles within a cluster are updated with the ICP.

Hu,s,nNLOSt=ICP∙PnICP+1Frx,u,θθn,1,ZOA,ϕn,1,AOAFrx,u,ϕθn,1,ZOA,ϕn,1,AOATexpjΦn,1θθκn,1−1expjΦn,1θϕκn,1−1expjΦn,1ϕθexpjΦn,1ϕϕ

Ftx,s,θθn,1,ZOD,ϕn,1,AODFtx,s,ϕθn,1,ZOD,ϕn,1,AODexpj2πrrx,n,1T∙drx,uλ0expj2πrtx,n,1T∙dtx,sλ0expj2πrrx,n,1T∙vtλ0

+PnICP+1M−1m=2MFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m−1expjΦn,mθϕκn,m−1expjΦn,mϕθexpjΦn,mϕϕ

Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vtλ0(7.5-22)

2.2 Verification of channel characteristics based on simulation

In order to investigate the effect of varying the angular spacing and power of the intra-cluster rays on the channel characteristics, we carry out simulations based on the 3GPP channel modeling framework. The parameters of the simulations are shown in Table 2.

Table 2: Simulation parameters

| Parameter | Type/Value |
| --- | --- |
| Carrier Frequency | 13 GHz |
| RF bandwidth | 400 MHz |
| Scenarios | UMa, NLoS |
| Sample Number | 500 |
| Tx antenna | Omnidirectional antenna |
| Rx antenna | Omnidirectional antenna |

Figure 4: The channel coefficients follow a Gaussian distribution after the introduction of ICP.

Figure 5: Comparison of CDFs for the K-S test.

The simulation results demonstrate that the channel coefficients still obey a Gaussian distribution when the introduction of ICP changes the angular distance and power of the rays within a cluster, as shown in Fig. 4 and Fig. 5. The KS statistics (D) are both less than 0.03. This also demonstrates that improving the intra-cluster power profile does not change the original channel model framework, but only enhances the accuracy of the channel model in characterizing sparsity.

Observation 2: The modification of the intra-cluster power profile does not change the original channel model framework, but only enhances the accuracy of the channel model in characterizing sparsity. The channel coefficients still obey a Gaussian distribution when the angular distance and power of the rays within a cluster are changed.

2.3 Impact of the ICP model on system performance

The modification of the cluster structure has an impact on the system performance such as channel capacity, channel sparsity (Gini index). This work simulates the Gini index and channel capacity of the 3GPP model with and without ICP based on the measurement setup. From Fig. 6 (a), it is found that the Gini index is closer to the measurements when the cluster contains a dominant ray within cluster. Similarly, the results of channel capacity are shown in Fig. 6 (b). The results show that the channel capacity simulated with the ICP model is closer to the measured channel capacity.

| (a) Gini index | (b) Capacity |
| --- | --- |

Figure 6: The Gini index and capacity of measurements and simulations. The simulations are conducted in the LOS condition of UMa scenario at 13 GHz utilizing a 128-array element TX antenna and a 64-array element RX antenna.

Observation 3: The measured cluster structure does not match that of TR38901. The cluster structure has an impact on system performance, such as channel capacity and sparsity. The performance of the system with the improved cluster structure better matches the measured channel.

- Conclusion

In this contribution, we provide our views on the validation and the details for Rel-19 channel modeling enhancements for 7-24 GHz. Key consideration is the modification of the cluster structure. The observation and proposals are as follows:

Observation 1: The introduction of ICP with changing the power and the angular distance of the rays still makes the PAS obey the Laplace distribution. When ICP is introduced into the 3GPP model, the level of sparsity is closer to the measured results.

Observation 2: The modification of the intra-cluster power profile does not change the original channel model framework, but only enhances the accuracy of the channel model in characterizing sparsity. The channel coefficients still obey a Gaussian distribution when the angular distance and power of the rays within a cluster are changed.

Observation 3: The measured cluster structure does not match that of TR38901. The cluster structure has an impact on system performance, such as channel capacity and sparsity. The performance of the system with the improved cluster structure better matches the measured channel.

Proposal 1: The ICP model should be introduced in TR38901, which changes the angular spacing and power of the multipaths within a cluster to make the 3GPP model describe channel sparsity more accurately. The ICP can be modeled as ICP=b∙10−a∙τnDS, which is related to cluster delay and delay spread. The ray offset angles within a cluster are updated with the ICP.

Hu,s,nNLOSt=ICP∙PnICP+1Frx,u,θθn,1,ZOA,ϕn,1,AOAFrx,u,ϕθn,1,ZOA,ϕn,1,AOATexpjΦn,1θθκn,1−1expjΦn,1θϕκn,1−1expjΦn,1ϕθexpjΦn,1ϕϕ

Ftx,s,θθn,1,ZOD,ϕn,1,AODFtx,s,ϕθn,1,ZOD,ϕn,1,AODexpj2πrrx,n,1T∙drx,uλ0expj2πrtx,n,1T∙dtx,sλ0expj2πrrx,n,1T∙vtλ0

+PnICP+1M−1m=2MFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m−1expjΦn,mθϕκn,m−1expjΦn,mϕθexpjΦn,mϕϕ

Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vtλ0(7.5-22)

References

[1] R1-2407467, Summary #4 of discussions for Rel-19 7-24 GHz Channel Modeling Validation, RAN WG1 Meeting#118, 19th – 23th August, 2024.

[2] J. Zhang, H. Miao, P. Tang et al., “New mid-band for 6G: Several considerations from channel propagation characteristics perspective,” IEEE Communications Magazine., Jun. 2024.