3GPP TSG RAN WG1 Meeting #69										  R1-122440

Prague, Czech Republic, 21th - 25th May 2012

Source:	ITRI

Title:	Interference mitigation for TDD reconfiguration in multi-Picocell scenario

Agenda Item:	7.10.1

Document for:	Discussion and Decision

# Introduction

Four single operator multi-cell scenarios in LS [1] are feasible under an assumption that DL-UL interference management mechanisms are utilized, and the multi-cell Outdoor Pico – Outdoor Pico deployment scenario was agreed by RAN1 for further evaluation. In RAN1 #68bis meeting, Intel Corporation reported a performance analysis of DL-UL interference manage and traffic adaptation in multi-cell Pico-Pico deployment scenario [2]. In this document, the evaluated interference management improves the UL packet throughput but introduces some degradation to the DL performance comparing to the case when only traffic adaptation is applied. And in all conducted evaluations the 10ms adaptation consistently outperforms the 640ms adaptation time scale.

In this contribution, we discuss the tradeoff and benefits of traffic adaptation and interference mitigation for multi-Picocell scenario.

# DL-UL Interference Mitigation

For the enhancement of entire system throughput, we consider the traffic adaptation and interference mitigation simultaneously for TDD reconfiguration in this contribution. For TDD reconfiguration, the DL-UL interference has a significant impact on the UL SINR of Pico UEs if Pico stations randomly choose transmission directions. In Figure 1, each Pico station chooses the configuration according to its UL-DL traffic condition based on Table 1. However, for example, when a Pico station with configuration 3 has a DL transmission in subframe 7, it may cause a severe interference to the neighbor Pico station with configuration 0 to receive an UL signal in subframe 7. Table 1 shows 7 configurations for LTE TDD reconfiguration. Configuration 0, 1, 2, 6 are with 5ms switch-point period, and configuration 3, 4, 5 are with 10 ms period. We observe that an UL SINR of a Pico UE is suffered much severer DL interference during the second half of one frame, since configurations with 10ms switch-point period have DL transmission from subframe 5 to 9.

Figure 1: DL-UL interference in multi-Picoell environments

| UL-DL Configuration | DL-to-UL Switch-point periodicity | Subframe number |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 0 | 5 ms | D | S | U | U | U | D | S | U | U | U |
| 1 | 5 ms | D | S | U | U | D | D | S | U | U | D |
| 2 | 5 ms | D | S | U | D | D | D | S | U | D | D |
| 3 | 10 ms | D | S | U | U | U | D | D | D | D | D |
| 4 | 10 ms | D | S | U | U | D | D | D | D | D | D |
| 5 | 10 ms | D | S | U | D | D | D | D | D | D | D |
| 6 | 5 ms | D | S | U | U | U | D | S | U | U | D |

Table 1: UL-DL Configuration (Table 4.2-2 in TS 36.211) for LTE TDD

In this contribution, a method to diminish the DL-UL interference during the second half of one frame is proposed. The Pico stations which the Pico-Pico links have low pathloss are divided into a group. Then only configurations with the same switch-point period are operated within a group of Pico stations. Figure 2 shows an example. The Pico stations are divided into 3 groups and Pico-Pico links of Pico stations have low pathloss within a group. Each group decides its corporate configuration class, then a Pico station chooses its configuration from the corporate configuration class which is proximal to its traffic condition.

Figure 2: Interference Mitigation in a group structure for TDD reconfiguration

The method has two features. Since each Pico eNB is able to do its utmost to choose the configuration according to its traffic condition, the traffic adaption gain is maintained within a group. The second is that since the configurations within a group of Picocells belong to the same configuration class, the severe DL interference on UL SINR in the second half of one frame is alleviated. Although the DL interference may still cause a negative impact on UL transmissions during subframe 3, 4, 8 and 9, we think that it is a trade-off between TDD reconfiguration gain and interference mitigation. Four cases of the multi-Picocell evaluation are analyzed in this document:

- Fixed UL-DL configuration 1 in all cells

- Dynamic traffic adaptation without DL-UL interference management (TA);

- Dynamic traffic adaptation with DL-UL interference management described by Intel (IMTA-Intel);

- Dynamic traffic adaptation with DL-UL interference management described in the document (IMTA-ITRI).

# Simulation Assumptions

We follow similar assumptions agreed for outdoor pico scenario in RAN1 #68bis, and the key parameters used are summarized in Table 2 (see further information in Appendix 7.1). The packet throughput and cell average packet throughput was selected as a main performance metric for current analysis.

Table 2. Main parameters for traffic adaptation in multi cell Pico-Pico scenario

| File Size, [MBytes] | FTP packet mean arrival rate in DL, [1/s] | Adaptation time scale, [ms] | DL/UL ratio of FTP packet arrival rate |
| --- | --- | --- | --- |
| 0.5 | [0.5, 1, 2] | 10, | {1/1, 1/0.5} |

# Performance Analysis

# Cumulative Distribution Functions of Packet Throughput

In this section we analyze the packet throughput CDF with different packet arrival rates, and compare the configuration class method with the static configuration #1 case, traffic adaptation without interference management and Intel’s clustering method. The simulation results are shown in Figure 3 and Figure 4.

Figure 3: CDF of packet throughput with traffic rate DL:UL = 1:1

Figure 4: CDF of packet throughput with traffic rate DL:UL = 2:1

Observations 1:

- Similar to multi-cell scenario [2] the traffic adaptation provides DL and UL packet throughput gains, especially in a symmetrical traffic model.

- With traffic rate DL:UL=1:1, the TA, interference management combined with traffic adaptation, IMTA-Intel and IMTA-ITRI all have better performance than fixed reconfiguration case.

－ In downlink, some degradation is observed of the two IMTA methods compared with the traffic adaptation without interference management (TA).

－ In uplink, since by IMTA-ITRI method, each Pico eNB is able to do its utmost to choose the configuration according to its traffic condition, the traffic adaption gain is maintained within a group. Furthermore, the configurations within a group of Picocells belong to the same configuration class, the severe DL interference on UL SINR in the second half of one frame is alleviated. These two special features can explain why IMTA-ITRI has much more performance gain.

- With traffic rate DL:UL=2:1, TA and the two IMTA methods have similar performance with different packet arrival rate.

－ In downlink, the TDD reconfiguration has obvious gain compared with the fixed configuration 1 case.

－ In uplink, the TDD reconfiguration has a more evident improvement compared with the fixed configuration 1 case only in a medium and high packet arrival rate,. By applying the interference mitigation, IMTA-Intel and IMTA-ITRI performs than TA.

# Cell Average Packet Throughput Analysis vs. Traffic Loading

### Figure 5: UL/DL cell average packet throughput with traffic rate DL:UL = 1:1

### Figure 6: UL/DL cell average packet throughput with traffic rate DL:UL = 2:1

The main goal of this section is to compare performance of the configuration class method with the static configuration #1 case, traffic adaptation without interference management and Intel’s clustering method. The packet throughput characteristics are shown in Figure 5 and Figure 6.

Observations 2:

- Similar to multi-cell scenario [2] the packet throughput gains are higher at low loading and medium loading.

- In downlink, two IMTA methods have slight performance loss comparing to the traffic adaptation without interference management (TA).

－ For traffic rate DL:UL=1:1, IMTA-ITRI performs slightly worse than IMTA-Intel and TA. This is because IMTA-ITRI has more freedom in choosing configurations within a Pico group, a higher probability for a group to decide its corporate configuration class to be with 5ms switch-point period (configuration 0, 1, 2, 6), in which the DL transmission is limited.

－ For traffic rate DL:UL=2:1, the three approaches including IMTA-Intel, IMTA-ITRI, and TA all have a traffic adaptation gain compared to fixed configuration 1 case.

- In uplink, two IMTA methods show better performance than TA.

－ For traffic rate DL:UL=1:1, IMTA-ITRI performs better than IMTA-Intel and TA. This is because IMTA-ITRI has more freedom in choosing configurations within a Pico group, a higher probability for a group to decide its corporate configuration class to be with 5ms switch-point period (configuration 0, 1, 2, 6), in which the UL transmission is assured. Due to considering the traffic adaption and interference mitigation simultaneously, IMTA-ITRI has a much better performance.

－ For traffic rate DL:UL=2:1, the three approaches including IMTA-Intel, IMTA-ITRI and TA have similar performance resulted from the reason that fewer DL-UL collisions occur. However, their performance degrades dramatically in high loading compared to fixed configuration 1 case. This implies TDD reconfiguration has lower flexibility and severer DL interference destroy the UL transmission in an asymmetrical and high traffic loading.

The TDD reconfiguration provides substantial traffic adaptation gain at high and medium loading. By applying the interference mitigation, UL performance is generally improved, but DL throughput has a slightly degradation. If we add the DL and UL packet throughput together, it is worth noting that the overall system throughput is increased, especially for IMTA-ITRI case.

# Conclusions

In this contribution, the traffic adaptation provides DL and UL packet throughput gains similar to the past work. Although in downlink, two IMTA methods have slight performance loss comparing to the traffic adaptation without interference management (TA), they show better gain than TA in uplink. Due to considering the traffic adaption and interference mitigation simultaneously, IMTA-ITRI has a much better performance in uplink with traffic rate DL:UL=1:1. If we add the DL and UL packet throughput together, it is worth noting that the overall system throughput is increased, especially for IMTA-ITRI case.

# References

- R1-120922 “LS on the feasibility of applying different UL-DL configurations in multi-cell scenarios” 3GPP RAN WG4, February 2012

- R1-121529 “Performance analysis of DL-UL interference management and traffic adaptation in multi-cell Pico-Pico deployment scenario” 3GPP RAN WG1, March 2012

# Appendix

  - Simulation assumption

Table A: Simulation assumptions for multiple pico cell scenario

| Parameters |  | Agreement |
| --- | --- | --- |
| Simulation Scenario |  | Multi-pico cells deployed on the same frequency |
| System bandwidth |  | 10MHz |
| Carrier frequency |  | 2 GHz |
| Inter-site distance |  | 500 m [case1 in 36.942] |
| Macro deployment |  | The typical 19-cell and 3-sectored hexagon system layout [36.942]. |
| Outdoor Pico deployment |  | 40m radius, random deployment [36.814] |
| Number of Pico cells per sector |  | 4 |
| Minimum distance between outdoor Pico cells |  | 40m [36.814] |
| Minimum distance between outdoor Pico and Macro |  | 75m |
| Minimum distance between UE and outdoor Pico |  | 10m [36.814] |
| Minimum distance between UE and Macro |  | 35m [36.814] |
| Outdoor Pico antenna pattern |  | 2D, Omni-directional [36.814] |
| Outdoor Pico antenna gain |  | 5 dBi [36.814] |
| UE antenna gain |  | 0 dBi [36.942] |
| Outdoor Pico noise figure |  | 13 dB [36.104] |
| UE noise figure |  | 9 dB [36.814] |
| Outdoor Pico max transmission power |  | 24 dBm as in [36.104] |
| Macro DL power control |  | Not modeled, i.e. assuming max macro Tx power |
| UE power class |  | 23 dBm (200 mW) [36.814] |
| Number of UEs per Pico cell |  | 10 UEs uniformly dropped around each of the Pico cells within a radius of 40m |
| User distribution |  | Cluster, Photspot = 2/3 |
| Shadowing standard deviation between outdoor Pico cells |  | 6dB [36.814] |
| Shadowing correlation between UEs |  | 0 |
| Shadowing correlation between outdoor Picos |  | 0.5 [36.814] |
| Pathloss model |  |  |
| Outdoor Pico to outdoor Pico |  | LOS: if R<2/3 km, PL(R)=98.4+20log10(R) [ free space loss] else, PL(R)=101.9+40log10(R), R in km [ Dual slop model TR25942 section5.1.4.3] NLOS: PL= 40log10(R)+169.36, R in km [25.942:section 7.4.1.2.1.4 TR 101 112(ETSI):Annex B1.8.1.2] Case 1: Prob(R)=0.5-min(0.5,5exp(-0.156/R))+min(0.5, 5exp(-R/0.03)) [36.814: table A.2.1.1.2-3 the probobility of Relay-UE case1] |
| Outdoor Pico to UE |  | PLLOS(R)=103.8+20.9log10(R) PLNLOS(R)=145.4+37.5log10(R) For 2GHz, R in km Case 1: Prob(R)=0.5-min(0.5,5exp(-0.156/R))+min(0.5, 5exp(-R/0.03)) [36.814: table A.2.1.1.2-3 Pico-UE] |
| UE to UE |  | If R<=50m, PL=98.45+20*log10(R), R in km If R>50m, PL=55.78 +40*log10(R), R in m (Xia model) [Section 7.4.1.2.1.4 of TS25942, Annex B1.8.1.2 of TR 101 112(ETSI), ETSI STC SMG2 UMTS L1#9 Tdoc 679/98] |
| Time scale for reconfiguration |  | 10ms |
| Scheduler |  | FIFO |
| Pico antenna configuration |  | 1Tx 2Rx |
| UE antenna configuration |  | 1Tx, 2Rx |
| Adaptation method of UL-DL reconfiguration |  | Based on amount of data in queue and amount of data transmitted |
| Link adaptation |  | MCS selection with 10% BLER DL based on CQI/PMI/RI reports and UL based on SRS measurement |
| DL CSI feedback |  | DL CSI modeled as following: -- PUCCH mode 1-1, wideband CQI/PMI reported every 10ms -- CSI reporting based on ideal channel estimation and ideal interference estimation in the reported subframe -- A minumum 5ms CSI feedback delay is modeled -- Error free feedback |
|  |  | UL CSI modeled as following --1 symbol SRS per 10ms (Last UL symbol in subframe#1) -- UL CSI based on ideal channel estimation and ideal interference estimation in the SRS subframe -- A minimum 5 ms CSI delay is modeled |
| Channel estimation |  | Ideal |
| Outdoor Pico DL power control |  | No power control |
| UE UL Power control |  | Open loop power control |
| Set of TDD UL-DL configurations |  | All 7 configurations are used |
| Small scaling fading channel |  | Not modelde |
| CP length |  | normal CP in both downlink and uplink. |
| Special subframe configuration |  | Special subframe configuration #8 |
| Packet drop time |  | The packet drop time is either not modeled or model according to 36.814 (i.e. 8s for 0.5MB and 32s for 2MB). Details to be provided by each company |
| Receiver type |  | MMSE receiver |
| UL modulation order |  | All modulations {QPSK, 16QAM, 64QAM} can be used as the UL modulation order |
| Shadowing standard deviation between outdoor Pico and UE | 3dB for LOS and 4dB for NLOS [ ITU-R M.2135 UMi] |
| Traffic model | Same traffic generation methodology and arriving rate as agreed in isolated cell case [R1-120080], independent traffic generation per cell. Same arriving rate for all the cells |
| Reference TDD configuration | TDD UL-DL configuration 1 with ratio of DL and UL arrival rate = { 1/1, 2/1} |
| HARQ modeling | A HARQ ACK/NACK is transmitted in the first available subframe after 4ms and the retransmission can happen in the first available subframe after another 4ms. In addition, a TB will be put back to the front of the data buffer if the TB has been retransmitted over the maximum number of HARQ retransmissions. |
| HARQ retransmission scheme | CC |
| Simulation cases | - Fixed UL-DL configuration 1 in all cells - Dynamic traffic adaptation without DL-UL interference management (TA); - Dynamic traffic adaptation with DL-UL interference management described by Intel (IMTA-Intel); - Dynamic traffic adaptation with DL-UL interference management described in the document (IMTA-ITRI). |
| Control channel and reference signal overhead | DL: • Overhead for CRS according to 36.211; • Overhead for PDCCH: 2 OFDM symbols; UL: • overhead for SRS defined above; • Overhead for PUCCH: 2 PRBs; • Overhead for UL DMRS: 2 symbols per subframe. |
| Shadow fading for Macro-UE link | 8dB |
| Cell-UE penetration loss | 20dB |
| Cell-Cell penetration loss | 0dB |
| UE-UE penetration loss | 20dB |