3GPP TSG-RAN WG1#73	R1-132419

Fukuoka, Japan, May 20th – 24th, 2013

Source:      CATR

Title:        Comparison of 4 Tx codebook for rank1 and rank2

Agenda Item: 6.2.2.1.1

Document for: Discussion and Decision

Introduction

In RAN1#72 bis, the agreed working assumption related to 4Tx codebook enhancement was

- New 4 tx codebook is constant modulus

  - Companies can provide further evaluations for Scenario C1 until RAN1#73 if desired

- At least for rank 1-2:

  - 4Tx codebook enhancement is based on a grid of beams design similar to the Rel 10 8Tx codebook

    - (this does not preclude that some of the Rel-8 codewords might be included in the new codebook)

  - a same W1 codebook is assumed for rank 1 and rank 2 feedback

- FFS until RAN1#73 whether rank 3-4 is:

  - same as Rel-8 or

  - enhanced or

  - not supported in the new codebook

- If rank 3-4 is supported in the new codebook, a same W1 codebook is assumed for rank 3 and rank 4 feedback

The working assumption for structure of W1 was given as:

- For each rank R =1,2 (and 3 and 4, if 4Tx codebook enhancement of rank 3 and 4 is supported in Rel 12),  W1 is defined as below

,.

Where Xn is a 2xCR matrix with DFT columns and

The working assumption for structure of W2 was given by:

- For each rank R =1,2 (and 3 and 4, if 4Tx codebook enhancement of rank 3 and 4 is supported in Rel 12), W2 is a 2CR x R matrix defined as

.

In the following email discussion, lots of companies provide the details of codebook for evaluation [1-6]. In this contribution we give the evaluation results of these candidate codebooks.

Enhanced Codebook Analysis and evaluation

Overhead of each codebooks

Here we just evaluate codebooks from CATT, Qualcomm, Huawei, ZTE, TI and LG, the overhead of each codebook with PUSCH mode 3-2 is shown in table1 compared with codebooks of Release 10 with PUSCH mode3-1.

Table 1 the overhead of each codebooks

| Overhead (bits) |  | R10 codebook | Qualcomm | Huawei | CATT | ZTE | TI | LGE |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | rank=1 | 4 | 4+3N | 4+4N | 4+4N | 4+3N | 4+4N | 4+4N |
|  | rank=2 | 4 | 4+3N | 4+4N | 4+3N | 4+3N | 4+4N | 4+4N |

Comparison of evaluation results

In this section, the system performances of above codebooks are evaluated. We just evaluated 20% outdoor/80% indoor with full buffer traffic model, and PUSCH mode 3-2 is used. The details of simulation assumptions are given in Annex.

Table 2 the evaluation results for 20% outdoor/80% indoor with PUSCH Mode 3-2

|  | R10(PUSCH 3-1) | Qualcomm | CATT | Huawei | ZTE | TI | LG |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cell.Avg | 2.15 | 2.16 | 2.16 | 2.146 | 2.161 | 2.143 | 2.169 |
| Edge UE SE | 0.0464 | 0.05326 | 0.05353 | 0.05363 | 0.05193 | 0.05243 | 0.05337 |
| Gain | 0%/0% | 0.5%/14.8% | 0.5%/15.4% | -0.2%/15.6% | 0.5%/11.9% | -0.3%/13% | 0.9%/15% |

In above tables 2, the performance gains of the enhanced codebook with feedback mode3-2 are compared. It can be observed that the gain of enhanced codebooks compared with R10 codebook (PUSCH 3-1) is to the cell edge users, about 11.9%~15.6% and the gain of average cell spectrum efficiency is small, about 0.5%~0.9% and even worse than R10 codebooks (PUSCH 3-1), about -0.3%~-0.2%.

Conclusions

In this contribution, we give our evaluation results of several codebooks, based on the evaluation results, we can observe that:

- The enhanced codebooks can get cell edge gain compared with R10 codebooks with PUSCH mode 3-1, the gain is from 11.8%~15.6%.

- The gain of average cell spectrum efficiency of enhanced codebooks is marginal.

References

- R1-131803，Design of the enhanced 4Tx codebook, Qualcomm Incorporated

- R1-131798 Discussion on 4Tx codebook for rank1-2,CATT

- R1-131796 Proposals for 4Tx Codebook Huawei

- R1-131809 Proposal on 4tx codebook for DL MIMO enhancement LG Electronics

- R1-131808 Proposal of enhanced 4Tx Codebook  ZTE

- R1-131805 On 4Tx codebook enhancement in Rel.12 Texas Instruments

Annex

Simulation assumptions

| Parameter | Assumption |
| --- | --- |
| Scenario | Scenario A |
| Deployment model | Macro cell: Cross-polarized Macro-sites, Reuse the macro part of the baseline simulation case for scenario 4 in TR36.819 |
|  | Hexagonal grid, 19 macro sites, 3 sectors/site |
| Traffic model | Full buffer |
| UE speeds of interest | 3 km/h |
| Bandwidth | 10 MHz |
| Carrier frequency | 2 GHz |
| Max number of HARQ retransmissions | 4 |
| Channel model | - ITU UMA channel with 80% indoor/20% outdoor UEs |
| Transmit power | 46dBm |
| Number of eNB antennas | 4 |
| Number of UE antennas | 2 |
| Number of UE per macro area | 10 UEs with uniform distribution |
| Antenna configuration | TX: 0.5 λ-spaced cross-polarized ±45° RX: cross-polarized ±45° |
| Receiver | MMSE-IRC |
| Feedback type | PUSCH reporting mode 3-1 /mode 3-2 |
| Feedback delay | 5ms |
| Feedback interval | 5ms |
| Scheduler | Proportional fair in time and frequency |
| Maximum number of co-scheduled UEs | 2 UEs,with1 layer per UE |
| Channel estimation | Non-Ideal |
| Overhead | 3 OSs DL control/ 2 CRS ports/ 12 REs DM-RS per PRB |