3GPP TSG RAN WG1 Meeting #76		R1-140423

Prague, CZ Rep. 10th – 14th February 2014

Agenda item:	7.2.7.4

Source:	NEC Group

Title: 		   Baseline performance results

Document for:	Discussion and Decision

Introduction

Basically, all parameters of 3D channel model have been approved [1], either as agreements or working assumptions after RAN1#75 email discussion. The simulation assumptions were also provided in session [75-29] of the email discussion. According to the working assumptions, we provide the simulation results for the baseline performance including the cell spectrum efficiency (SE) and the edge UE spectrum efficiency, based on different alternatives. We also provide the details of our simulation assumptions in the appendix as the reference.

Discussion

After the RAN1#75 email discussion, all simulation parameters have been approved, excepted for following alternative working assumption:

- Choice of wrapping method for 3D-channel model [2]

- Alt1: Geographical distance based wrapping

- Alt2: Radio distance based wrapping

- Polarized antenna modelling [3]

- Alt1: R1-136036 (yellow part)

- Alt2: 36.814

- UE orientation[4]

- Alt1:  ΩUT, uniformly distributed on [0,360] degree, ΩUT, = 90 degree, ΩUT, = 0 degree

- Alt2:  ΩUT, uniformly distributed on [0,360] degree, ΩUT, = arccos(X) with X ~ U[-1,1], ΩUT, uniformly distributed on [0,360] degree

- UE antenna pattern[4]

- Alt1:  Isotropic antenna gain pattern A’(θ’,ф’) = 1

- Alt2:  Dipole antenna gain pattern A’(θ’,ф’) = D(cos((π/2)cosθ’)/sinθ’)2 with D=1.63

In this contribution, we provide a series of simulation results, including the comparison on the system level simulations between AAS pattern (Table 7.1-1 in 36.873 of version 8) and previous antenna pattern (Table A.2.1.1-2 in 36.814), as well as some other alternative assumptions.

Table 1 Performance results on UMa scenario

| Simulation assumption |  |  |  |  | Cell average SE | Cell edge SE (5% user cdf ) |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna configuration | BS antenna pattern | Polarization method | UE antenna pattern | UE orientation |  |  |
| XX | 36.873 | Alt 1 | Alt 1 | Alt 1 | 1.9785 | 0.054108 |
| ¦¦¦¦ |  |  |  |  | 1.9935 | 0.064655 |
| XX | 36.873 | Alt 1 | Alt 1 | Alt 2 | 1.97323 | 0.050219 |
| XX | 36.873 | Alt 2 | Alt 1 | Alt 1 | 1.94902 | 0.049644 |
| XX | 36.814(1) | Alt 2 | Alt 1 | 36.814(2) | 1.59672 | 0.04425 |
| ¦¦¦¦ |  |  |  |  | 1.61178 | 0.047288 |
| XX | 36.873 | Alt 1 | Alt 2 | Alt 2 | 1.96176 | 0.051828 |
| ¦¦¦¦ |  |  |  |  | 1.9694 | 0.058473 |

(1)  Antenna pattern as in Table A.2.1.1-2 of 36.814

(2)  There is no orientation change, i.e. ΩUT,, ΩUT, , ΩUT, are all zeros degree.

Table 2 Performance results on UMi scenario

| Simulation assumption |  |  |  |  | Cell average SE | Cell edge SE (5% user cdf ) |
| --- | --- | --- | --- | --- | --- | --- |
| Antenna configuration | BS antenna pattern | Polarization method | UE antenna pattern | UE orientation |  |  |
| XX | 36.873 | Alt 1 | Alt 1 | Alt 1 | 1.9169 | 0.046651 |
| ¦¦¦¦ |  |  |  |  | 1.86808 | 0.50408 |
| XX | 36.873 | Alt 1 | Alt 1 | Alt 2 | 1.8981 | 0.046573 |
| XX | 36.873 | Alt 2 | Alt 1 | Alt 1 | 1.88294 | 0.042632 |
| XX | 36.814 | Alt 2 | Alt 1 | 36.814 | 1.57179 | 0.03928 |
| ¦¦¦¦ |  |  |  |  | 1.47163 | 0.042894 |
| XX | 36.873 | Alt 1 | Alt 2 | Alt 2 | 1.90739 | 0.047509 |
| ¦¦¦¦ |  |  |  |  | 1.86632 | 0.050425 |

As shown in Table 1 and Table 2, the trends of results with cross-polarized antenna, are similar both for UMa and UMi scenario. Different working assumptions except for BS antenna pattern have marginal impact on the simulation results, especially for the cell average sepctral efficiency. Compared with the cross-polarized antenna, larger performance gain on the cell edge can be found in the co-polarized antenna case since the higher antenna correlation can provide larger beamforming gain for the cell edge UEs, which are often with Rank-1.

Conclusion

In this contribution, we provided baseline performance results according to the RAN 1 agreements / working assumptions which have been approved in the past 3D channel modelling meetings.

References

[1] Chairman’s Notes 3GPP TSG RAN1#75, San Francisco, USA, 11th – 15th November 2013.

[2] Email discussion of 3GPP TSG RAN1#75 on [75-13],＂Phase 1 calibration for 3D-channel＂

[3] Email discussion of 3GPP TSG RAN1#75 on [75-10],＂TP for mechanical tilt in 3D-channel＂

[4] Email discussion of 3GPP TSG RAN1#75 on [75-11], ＂WF on UE orientation and UE antenna pattern for 3D-channel＂

Appendix

Table 3 Simulation assumption

|  | Baseline |
| --- | --- |
| Scenarios | 3D-UMa, 3D-UMi |
| Number of drops | 10 |
| TTI per drop | 5000 |
| BS antenna configurations | K=M=10, N=2, X-pol (+/-45), 0.5λ H/V, θetilt = 12 degrees |
| MS antenna configurations | 2Rx X-pol (0/+90) |
| System bandwidth | 10MHz (50RBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Carrier Frequency | 2GHz |
| Duplex | FDD |
| Network synchronization | Synchronized |
| Number of UEs per cell | 10 |
| UE distribution | Follows 36.873 3D-UMa, 3D-UMi |
| UE Speed | 3km/h |
| Traffic model | Full buffer |
| Scheduler | PF, 1 UE per TTI allocation |
| Receiver | Ideal channel estimation |
|  | Ideal interference modeling |
|  | MMSE-IRC receiver |
| Interference model | Ideal interference from PDSCH which can be measured from IMR |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | PUSCH 3-1 |
|  | CQI and PMI reporting triggered per 5ms |
|  | Feedback delay is 5 ms |
|  | Rel-8 4Tx codebook |
| Overhead | 3 symbols for DL CCHs, 4 CRS ports and DM-RS with 12 REs per PRB |
| Transmission scheme | TM10, single CSI process, SU-MIMO with rank adaptation |
| Interference model | Ideal interference from PDSCH, can be measured from IMR |
| Wrapping method | Geographical distance based (baseline) |
| Cluster elimination step 6 | scaling factor not changed after cluster elimination |
| Handover margin (for calibration) | 0 dB |
| Metrics | Cell average SE |
|  | 5% cell-edge SE |