3GPP TSG-RAN WG1 #76 	R1-140460

February 10 – 14, 2014

Prague, Czech Republic

Agenda item:	7.2.7.4

Source: 	Qualcomm Incorporated

Title: 	Initial baseline results

Document for:	Discussion/Decision

# Introduction

As agreed in RAN1#73 [1], the baseline performance of the existing transmission scheme will be calibrated besides the Phase 1 and Phase 2 calibration of the 3D channel model.  In RAN1#75 [2] and the followed email discussion [3], the baseline simulation assumptions were agreed.  In this contribution, we present some initial results of the baseline performance under the 3D-UMa and 3D-UMi scenarios.

# Initial baseline performance

We follow the agreed simulation assumption as follows.

- Antenna configuration

  - 4 horizontal antenna ports at BS, cross-polarization, antenna spacing  0.5λ

  - 2 receive antennas at UE, cross-polarization

- Transmission scheme

  - TM10

    - Where the ideal interference is from PDSCH, which can be measured by IMR. A single CSI process is assumed.

  - SU-MIMO with rank adaption

- CRS port 0 is used for RSRP computation for UE attachment

- Output of baseline performance simulation

  - Performance of the transmission scheme under 3D channel model

    - UE distribution follows TR36.873

    - K = M = 10, N = 2 , the complex weight factor for vertical element  is defined in TR36.873

  - Evaluation metrics:  cell average spectrum efficiency, 5th percentile cell edge spectrum efficiency.

Detailed assumptions are listed in Table 1.

The baseline performance results are summarized in Table 2.

Table 1  Simulation assumptions for baseline performance.

| Parameter | Description |
| --- | --- |
| Scenarios | 3D-UMa and 3D-UMi |
| BS antenna array | K = M = 10, N = 2, ±45° cross-polarized antenna array with 0.5λ horizontal/vertical antenna spacing and 102° electrical downtilt |
| MS antenna array | 2 0°/90° cross-polarized antenna array |
| System bandwidth | 10MHz (50RBs) |
| UE attachment | Based on RSRP from CRS port 0 |
| Carrier Frequency | 2 GHz |
| Duplex | FDD |
| Network synchronization | Synchronized |
| Number of UEs per cell | 10 |
| UE distribution | Follows 36.873 3D-UMa, 3D-UMi |
| UE Speed | 3 km/h |
| Polarized antenna modeling | 1) R1-136021 (yellow part); 2) 36.814 |
| UE array orientation | ΩUT,α ~U[0, 2π], ΩUT,β = 90°, ΩUT,γ = 0° |
| UE antenna pattern | Isotropic antenna gain pattern A'(θ', ϕ') = 1 |
| Traffic model | Full buffer |
| Scheduler | PF, 1 UE per TTI allocation |
| Receiver | MMSE-IRC receiver with Ideal channel estimation |
| Interference model | Ideal interference from PDSCH which can be measured from IMR |
| Hybrid ARQ | Maximum 4 transmissions |
| Feedback | PUSCH 3-1, CQI and PMI reporting triggered per 5ms |
|  | Feedback delay is 5 ms |
|  | Rel-8 4Tx codebook |
| Overhead | 3 symbols for DL CCHs, 4 CRS ports and DM-RS with 12 REs per PRB |
| Transmission scheme | TM10, single CSI process, SU-MIMO with rank adaptation |
| Interference model | Ideal interference from PDSCH, can be measured from IMR |
| Wrapping method | Geographical distance based |
| Cluster elimination step 6 | Scaling factor not changed after cluster elimination |
| Handover margin (for calibration) | 0 dB |

Table 2 Initial baseline performance results

| Polarization Model | 3D-UMa |  | 3D-UMi |  |
| --- | --- | --- | --- | --- |
|  | Cell average SE [bps/Hz] | 5% cell-edge SE [bps/Hz] | Cell average SE [bps/Hz] | 5% cell-edge SE [bps/Hz] |
| R1-136021 | 2.52 | 0.071 | 2.62 | 0.068 |
| TR 36.814 | 2.49 | 0.067 | 2.58 | 0.064 |

# Conclusion

In summary, we presented some initial baseline performance results.

# References

- R1-132861, Final Report of 3GPP TSG RAN WG1 #73, v1.0.0.

- Draft Report of 3GPP TSG RAN WG1 #75, v0.2.0.

- Email discussion [75-29] Calibration for 3D channel model.

- IST-4-027756, D1.1.2 WINNER II Channel Models, v1.2.