3GPP TSG-RAN WG1 #77 	R1-141969

19th May – 23rd May, 2014

Seoul, Korea

Agenda item:	6.2.5.2.1

Source:	Qualcomm Incorporated

Title:	Resource allocation for Mode 2 D2D broadcast communication

Document for: 	Discussion/Decision

# 1	Introduction

In this contribution, we focus on Mode 2 resource allocation for the out-of-coverage case, with a focus on VoIP traffic. Through system-level simulations, we compare the performance of the following three resource allocation schemes, whose descriptions are provided in Section 2:

- Random resource selection

- Randomized RPT selection (called RRPT)

- CSMA based on SA decoding (called CSMA/SA)

As the main conclusion of this contribution, we propose that the random resource selection should be considered as the baseline due to its simplicity and performance close to the other schemes considered.

# 2	Description of resource allocation schemes

We consider three resource allocation schemes for Mode 2 as follows:

- Random resource selection:  a transmitter that has a data packet to transmit, selects four resources uniformly at random (randomized both in time and frequency), and transmits the data packet on those resources. We assume that this randomization is done based on the Destination ID and hence known at the receiver.

- Randomized RPT (RRPT) – this scheme is motivated by [4] and [6] where orthogonal RPTs are proposed. By numerical simulations, we pick about 200 RPTs such that any time RPTs collide at most two times. Each RPT is a set of 4 numbers (corresponding to 4 transmissions) from 1 to 20 (corresponding VoIP packet periodicity). In this scheme, each transmitter picks a RPT randomly for each transmission.

- CSMA based on SA decoding (CSMA/SA) -- this scheme builds on the RRPT scheme where we assume that each transmitter is able to decode SA from other transmitters and picks an RPT that minimizes conflict with its neighbors (motivated by [4], [5],[6]). We assume idealized SA decoding and only a link budget constraint of 135 dB is modeled –half duplex constraint in SA is not modeled.

Before we move to a quantitative simulation comparison of the proposed schemes, we qualitatively compare the schemes in Table 1.

Table 1 Qualitative comparison of schemes

|  | Random | RRPT | CSMA/SA |
| --- | --- | --- | --- |
| Direct collision avoidance | No | No | Yes |
| Hidden node collision avoidance | No | No | No |
| Interference diversity (across retransmissions) | Yes | Yes | No |
| Frequency diversity | Yes | Yes | Yes |
| Time diversity | No | No | No |

We note that the main difference between the three algorithms described above is the mechanism of collision avoidance. The random and RRPT algorithms achieve collision avoidance through interference averaging across retransmissions, while the CSMA/SA algorithm avoids collisions using direct measurements. We argue that interference averaging is a simple way to avoid collisions and provides comparable performance. In the next section, we present simulation results to justify this claim.

Observation 1: CSMA/SA scheme relies on collision avoidance based on either direct measurements, whereas the random and RRPT schemes rely on interference averaging across retransmissions to deal with collisions.

# 3	Performance evaluation

In this section, we evaluate the resource allocation schemes described in Section 2 with the help of system-level simulations. The simulation assumptions are given in Table 2 below:

Table 2 Simulation assumptions

| Parameters | Assumptions |
| --- | --- |
| Layout | Option 5 (ISD = 1732m) uniform |
| Carrier Frequency, System Bandwidth | 700 MHz, 10 MHz |
| Num TX | 12/cell |
| Number of UEs | 32/Cell |
| TX Power | 23 dBm |
| Num RX antennas | 2 |
| Channel Model/ Fading | As per [2] |
| IBE Model | W,X,Y,Z = {3,6,3,3} |
| VOIP Packet Size | 44 Bytes (incl. CRC) |
| VOIP Coding/Modulation | Turbo/QPSK |
| Transmission bandwidth (PRBs) | 2 |
| Number of transmissions per packet | 4 |
| Diversity techniques | Frequency |
| RSRP threshold | -107 dBm |

The simulation results are summarized in Table 3 below, and detailed in Figure 1.

Table 3 Fraction of successful VOIP links

| Drop | Random | RRPT | CSMA/SA |
| --- | --- | --- | --- |
| Option 5 (Uniform) | 96% | 96% | 94% |

Figure 1 VOIP Simulation results

While we present simulation results only for the uniform drop, we expect the same trend to hold for in-out and hotspot drops.

Observation 2: we observe that all the three schemes give very similar performance.

Based on the above discussion, we propose that:

Proposal 1: Random resource selection should be used as a baseline for resource allocation for Mode 2 D2D broadcast communication.

Further, we propose to use the Destination ID (“ID in SA”) for pseudo-randomly determining the resources both in time and frequency.

Proposal 2: For random resource selection, the resources should be randomized both in time and frequency. Destination ID should be used to determine these resources in a pseudo-random fashion.

We note that different RPTs were agreed both for Mode 1 and Mode 2 at RAN1 #76BIS [3]:

Agreements: - One or more resource patterns for transmission (RPT) of time and/or frequency resources for multiple transmission opportunities of data TBs can be defined - RPT is either implicitly or explicitly signaled by the eNB or Rel-10 Relay for Mode1 - RPT is either implicitly or explicitly signaled in SA

As discussed in a companion contribution [7], we propose that RPT is explicitly signaled in SA. For Mode 2 communication, RPTs will be used to indicate the mean inter-transmission period.

Proposal 3: different RPTs are used to indicate different mean inter-transmission period for Mode 2

# 4	Conclusion

In this contribution, we evaluated the performance of a variety of resource allocation schemes for the out-of-network D2D broadcast communication scenario. We observed that the random resource selection algorithms provides a good performance – this scheme performs comparable to the two other schemes studied (CSMA/SA and RRPT). Due to simplicity of implementation, we propose to use random resource allocation as baseline for Mode 2 broadcast communication.

Proposal 1: Random resource selection should be used as a baseline for resource allocation for Mode 2 D2D broadcast communication.

Proposal 2: For random resource selection, the resources should be randomized both in time and frequency. “ID in SA” should be used to determine these resources in a pseudo-random fashion.

Proposal 3: different RPTs are used to indicate different mean inter-transmission period for Mode 2

This is based on the following two observations:

Observation 1: CSMA/SA scheme relies on collision avoidance based on either direct measurements, whereas the random and RRPT schemes rely on interference averaging across retransmissions to deal with collisions.

Observation 2: we observe that all the three schemes give very similar performance for D2D VoIP communication.

# References

- Chairman’s notes, RAN WG1 #76, Prague, Czech Republic, Feb, 2014

- 3GPP TR 36.843

- Chairman’s notes, RAN WG1 #76BIS, Shenzhen, China, Apr, 2014

- R1-141256, “Distributed resource allocation for D2D communication”, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, RAN1 #76BIS

- R1-141425, “Distributed Resource Allocation for D2D Communication”,  ZTE

- R1-141546, “Discussion on D2D Operation Outside of Network Coverage (Mode-2)”, Intel Corporation

- R1-141966, “Control for D2D broadcast communication”, Qualcomm Inc.