3GPP TSG RAN WG1 Meeting #77	R1-142136

Seoul, Korea, 19th – 23rd, May 2014

Agenda Item:	6.2.3.2.2

Source: 	LG Electronics

Title:		        Cell detection performance using discovery signals

Document for:	Discussion and decision

Introduction

In last RAN1 #77 meeting, the following agreements on small cell discovery were made.

- For enhanced discovery procedure:

  - UE can be configured with at least one DRS measurement timing configuration per frequency

    - Note: DRS measurement timing configuration indicates when UE may perform cell detection and RRM measurement based on DRS

    - Note: UE can detect multiple cells based on a DRS measurement timing configuration on the frequency

    - UE can expect DRS in DRS measurement timing configuration

    - DRS measurement timing configuration includes at least period and offset and potentially duration

      - The reference timing for the offset is a serving cell timing

        - FFS: Which serving cell

- A DRS comprises following signals

  - Both PSS and SSS are transmitted

  - Additional reference signal(s) include CRS and/or CSI-RS

- FFS: Changing mapping of PSS/SSS, CRS, CSI-RS

- Further down select of following alternatives of DRS in RAN1 #77 meeting

  - Alt. 1: DRS is PSS/SSS/CSI-RS/CRS or PSS/SSS/CRS with configurable

  - Alt. 3a: DRS is PSS/SSS/CRS

  - Alt. 3b: DRS is PSS/SSS/CSI-RS

  - Alt. 5: DRS is PSS/SSS/CSI-RS/CRS or PSS/SSS/CSI-RS with configurable

- Rel. 12 discovery signal should identify transmission point

- Rel. 12 discovery signal should facilitate small cell on/off

In this contribution, we will give performance evaluation results of discovery signals options.

Discovery signal options

2.1 	SSS location

According to the agreement in last RAN1 meeting, PSS/SSS will be transmitted as a part of discovery signal. However, it is still FFS where the discovery signals will be located. Hence, we will provide the evaluation results of cell detection performance depending on the SSS location. We consider the different SSS location depending on PSS id (cell group id) with SSS muting among small cells. It means that SSS can be located in 3 different locations. The detailed simulation assumptions and evaluation results are shown in appendix. The cell detection performance is compared with that using legacy PSS/SSS in Table 1. As shown in Table 1, the cell detection performance can be significantly improved by changing SSS location depending on PSS id.

Table 1. Percentage of UE with detection probability of Pd < 0.9

|  | 1st cell | 2nd cell | 3rd cell |
| --- | --- | --- | --- |
| Legacy PSS/SSS location: Prob (UE¦ Pd < 0.9) | 0 % | 32.3 % | 100 % |
| SSS location depending on PSS id: Prob (UE¦ Pd < 0.9) | 0 % | 0.8 % | 14.8 % |

Observation 1:The cell detection performance can be significantly improved by changing SSS location depending on PSS id with SSS muting among small cells.

2.2 	Transmission point (TP) identification

Since transmission point (TP) should be identified by discovery signals, it would be desirable to adopt CSI-RS as a part of discovery signals (call it as DRS-CSI-RS) for the purpose of TP identification in addition to PSS/SSS. We also evaluate the cell detection performance using DRS-CSI-RS. In this evaluation, it is assumed that physical cell id can be detected without error using PSS/SSS and thus performance of TP identification of DRS-CSI-RS is evaluated. For DRS-CSI-RS transmission, we assume that DRS-CSI-RS is transmitted with muting among TPs by configuring zero power CSI-RS. The detailed simulation assumptions and results are also shown in appendix. The TP detection performance is shown in Table 2.

As shown in Table 2, the TP can be reliably detected by DRS-CSI-RS in case of system bandwidth of 25 RBs. However, the TP detection performance degrades in case of system bandwidth of 6 RBs, which may require enhanced DRS-CSI-RS transmission (e.g., increased density) if 6 PRB bandwidth needs to be supported. Given that small cell layer may utilize high frequency band such as 3.5 GHz where wideband would be available, supporting 6 PRB for on/off operation may not be so essential. Thus, we consider DRS-CSI-RS performance for TP identification is satisfactory.

Table 2. Percentage of UE with detection probability of Pd < 0.9

|  | 1st cell | 2nd cell | 3rd cell |
| --- | --- | --- | --- |
| DRS-CSI-RS for TP detection (25 RB): Prob (UE¦ Pd < 0.9) | 0 % | 0.28 % | 3.12 % |
| DRS-CSI-RS for TP detection (6 RB): Prob (UE¦ Pd < 0.9) | 2.13 % | 14.08 % | 46.11 % |

Observation 2:The TP can be reliably detected by orthogonal DRS-CSI-RS configuration with muting  among small cells for system bandwidth of larger than 25 RBs.

2.3 	RRM measurement

As discovery signal will be used for RSRP and RSRQ-like measurements, the measurement performance of discovery signal should be also analysed. Hence, we evaluate the RSRP measurement accuracy when  DRS-CSI-RS is used for RSRP measurement as discovery signal. The detailed evaluation assumptions are in Table 3.

Table 3. Evaluation assumptions for RSRP measurement using discovery signal (DRS-CSI-RS)

| Parameter | Value |
| --- | --- |
| System bandwidth | 10 MHz |
| Carrier frequency | 3.5 GHz |
| Channel profile | EPA, 3km/h |
| Frequency error | Uniformly distributed in [-200, 200] Hz |
| Timing error | Perfectly known to UE |
| Measurement duration | 1ms / 4ms |
| Measurement interval | 1ms |
| Measurement bandwidth | 6PRBs / 50PRBs |
| Number of antenna ports for CSI-RS | 1 |
| State of a cell | Small cell off-mode |
| SINR | -9dB, -6dB, -3dB |

According to current specification in [1], the RSRP measurement accuracy requirement is 6 dB on the condition of SINR  -6 dB in normal condition. We show the RSRP measurement performance with discovery signal in Figure 4 (including RSRP measurement performance using CRS as a reference). As can be seen in Figure 1, the accuracy requirement can be fulfilled with SSS. In case of CSI-RS, the accuracy requirement can be fulfilled when 50 RBs or multiple subframes are used for measurement.

(a)

(b)

(c)

Figure 1. RSRP measurement performance using discovery signal: (a) CRS (reference), (b) DRS-CSI-RS (6 RB), and (c) DRS-CSI-RS (25 RB)

Observation 3:RRM measurement using DRS-CSI-RS can satisfy the accuracy requirement for system bandwidth of 25 RBs.

Conclusions

We discuss the discovery signal options and have following observations.

Observation 1:The cell detection performance can be significantly improved by changing SSS location depending on PSS id with SSS muting among small cells.

Observation 2:The TP can be reliably detected by orthogonal CSI-RS configuration with muting  among small cells for system bandwidth of larger than 25 RBs.

Observation 3:RRM measurement using DRS-CSI-RS can satisfy the accuracy requirement for system bandwidth of 25 RBs.

- 4	References

- 3GPP TS 36.133 v10.5.0, Requirements for support of radio resource management (Release 10)

Appendix

The link and system level simulation assumptions for cell detection are shown in Table 1 and Table 2, respectively.

Table A1. Link level simulation assumptions for cell detection.

| System bandwidth | 25 RB |
| --- | --- |
| Carrier frequency | 3.5G |
| Channel model and Doppler frequency | EPA |
| Time offset (us) | Random and uniform timing offset within +/- 3 us for each signal |
| Frequency offset (Hz) | Random and uniform frequency offset within 0.1ppm at the UE |
| UE speed | 3km/h |
| PSS/SSS transmission | 5 strongest cells |
| Antenna configurations | 2x2 |
| Number of CSI-RSsubframes used for cell detection | Single subframe |
| Threshold for RSRP gap with respect to the 1st strongest cell | 12 dB |
| Interference modelling | AWGN for the interference from cells other than the 5 strongest cells |

Table A2. System level simulation assumptions for cell deployment.

| Scenario | Scenario #2a |
| --- | --- |
| Number of macro site | 7 |
| Total Small cell TX Power | 30 dBm |
| Number of clusters/buildings per macro cell geographical area | 1 |
| Number of small cells per cluster | 10 |
| Distance-dependent path loss | ITU model as baseline |
| Antenna configuration | 2x2, cross-polarized |
| Selection of target cell for detection | 3 strongest cells based on RSRP |

In Figure A1, we show cell detection performance with legacy PSS and SSS as a reference.

- (b)

Figure A1. Cell detection performance with legacy PSS and SSS. (a) 1st cell, (b) 2nd cell, and (c) 3rd cell l detection performance.

In Figure A2, we show cell detection performance of PSS and SSS location depending on the PSS id.

(a)							(b)

(c)

Figure A2. Cell detection performance of PSS and SSS location depending on PSS id. (a) 1st cell, (b) 2nd cell, and (c) 3rd cell detection performance.

In Figure A3, we show TP detection performance using DRS-CSI-RS in case of system bandwidth of 25 RBs.

(a)							(b)

(c)

Figure A3. Cell detection performance using DRS-CSI-RS with system bandwidth of 25 RB. (a) 1st cell, (b) 2nd cell, and (c) 3rd cell detection performance.

In Figure A4, we show TP detection performance using DRS-CSI-RS in case of system bandwidth of 6 RBs.

(a)							(b)

(c)

Figure A4. TP detection performance using DRS-CSI-RS with system bandwidth of 6 RB. (a) 1st cell, (b) 2nd cell, and (c) 3rd cell detection performance.