3GPP TSG RAN WG1 Meeting #77						      R1-142287

Seoul, Korea, 19th May– 23th May, 2014

Agenda item:	6.2.4

Source:	FiberHome

Title: 	Discussion on Dynamic Power-sharing of DC UL transmission

Document for:	Discussion and Decision

Introduction

In RAN1 #76b meeting, many issues about dual connectivity were discussed. In the emails, most company focus on TPC of DC UL transmission, several questions were proposed [1]:

Question 0: Is dynamic power-sharing between two CGs/eNBs necessary?

Question 1: Are PMeNB and PSeNB (maximum transmit power per CG/eNB) need to be defined?

Question 2: In unsync case, how to define the power-limited case?

Question 3: Do UE need to take into account of power requirement of the other eNB/CG before allocating its available power for the first eNB/CG, in the following two cases

Case 1) when the first eNB/CG is MeNB/MCG

Case 2) when the first eNB/CG is SeNB/SCG

Question 4: Is the additional processing time reduction (maximum close to 1ms) acceptable?

Question 5: Is it allowable to keep some aspects as UE implementation matters?

Question 6: What should be decided in RAN1 and what in RAN4?

Question 7: Is configuration of UE maximum transmit power per serving cell applicable in dual connectivity?

In this contribution, we discuss some aspects about the Dynamic Power-sharing of DC UL transmission which could answer Question 0/1/2.

Discussion

Due to independent schedulers at the MeNB and the SeNB and the non-zero latency of the backhaul link, it is possible for a UE that a combined power of transmission to MeNB/MCG and SeNB/SCG in a same subframe may exceed the maximum power value. So a power split mechanism between MeNB/MCG and SeNB/SCG has to be done to avoid the exceeding. The main alternatives are [2]:

Alternatives 1: Static Power Split between MeNB and SeNB

The power scaling between MeNB link and SeNB link is provided statically. A fixed power sharing between the two eNBs is introduced to guarantee no exceeding. A notable waste of UE’s power will happen when uplink transmission happens only within one CG. On the other way, the fixed power on each eNB/CG will compromise its coverage especially when the power can’t maintain transmission link getting worse.

Alternatives 2: Semi-static Power Split Between MeNB and SeNB

The power scaling between MeNB link and SeNB link is provided semi-statically by certain configuration according to the actual requirements (e.g. the pathloss change and ect.). “Semi-static” means the ratio of power scaling varies when the transmission environments changes which is not an efficient solution. The power assignment can be given by MeNB or UE itself.

Alternatives 3: Dynamic Power Sharing between MeNB and SeNB

Dynamic Power Sharing can achieve the purpose of make full uses of UE UL transmission power. For example, there are two modes in the network, and some subframes are merely DL subframes when UE links to TDD eNB. In this condition, it would be beneficial to utilize all the UE power to the UL subframes transmission. Besides, Dynamic Power Sharing also own the advantage of Semi-static Power Split when uplink transmission changes which has a better power-sharing efficiency.

Based on the discussions above, Dynamic Power Sharing is necessary to make the UE transmit power utilization as efficient as possible compare to the other two alternatives. So Dynamic Power-sharing between two CGs/eNBs is necessary.

Proposal 1: It is necessary to support the dynamic power sharing between two CGs/eNBs.

Before Dynamic Power-sharing scheme to be made, restrict the actual UL transmission at one subframe with the maximum power value PMeNB and PSeNB(defines maximum transmit power per eNB/CG) is worth doing which takes some effects on avoiding UE power-limited case preliminarily. The uses of PMeNB and PSeNB ensure a better coordinate power allocation between different CGs even if the two CGs has difficulties on exchanging scheduling information timely,

Proposal 2: PMeNB and PSeNB (maximum transmit power per CG/eNB) need to be defined.

The following candidate solutions for Dynamic Power-sharing were discussed [1]:

Candidate 1: Dynamic power-sharing without PMeNB and PSeNB

- Maximum transmit power per serving cell is PCMAX,c

- Maximum transmit power per eNB/CG is PCMAX

- Maximum transmit power per UE is PCMAX

- When UE is power-limited, depending on some prioritization rules or transmission timing, power-scaling/dropping is applied

- PHR is calculated using PCMAX,c

Candidate 2: Dynamic power-sharing with PMeNB and PSeNB, where maximum transmit power per eNB/CG cannot exceed PMeNB or PSeNB

- Maximum transmit power per serving cell is PCMAX,c

- Maximum transmit power per eNB/CG (for non-PRACH transmission, FFS on PRACH) is PMeNB or PSeNB

- Maximum transmit power per UE is PCMAX

- When UE is power-limited, depending on some prioritization rules or transmission timing, power-scaling/dropping is applied

- FFS PHR is calculated using PCMAX,c or PMeNB/PSeNB

Candidate 3: Dynamic power-sharing with PMeNB and PSeNB, where maximum transmit power per eNB/CG can exceed PMeNB or PSeNB

- Maximum transmit power per serving cell is PCMAX,c

- Maximum transmit power per eNB/CG (for non-PRACH transmission, FFS on PRACH) is (PCMAXPMeNB) for SeNB and (PCMAXPSeNB) for MeNB

- Maximum transmit power per UE is PCMAX

- PMeNB + PSeNB is equal or less than UE total maximum output power PCMAX

Candidate 4: Dynamic power-sharing with PMeNB and PSeNB, where PMeNB/PSeNB are the signalling exchanged b/w eNBs (not signalled to UE)

- Maximum transmit power per serving cell is PCMAX,c

- Maximum transmit power per eNB/CG is PMeNB or PSeNB, but not indicated to a UE configured with dual connectivity; therefore, the rule can be broken by the controlling eNB (i.e., MeNB)

- Maximum transmit power per UE is PCMAX

- When UE is power-limited, depending on some prioritization rules or transmission timing, power-scaling/dropping is applied

- PHR is calculated using Pcmax,c

Candidate 5: Dynamic power-sharing with PMeNB and PSeNB, where maximum transmit power per eNB/CG cannot exceed PMeNB or PSeNB in power-limited case

- Maximum transmit power per serving cell is PCMAX,c

- Maximum transmit power per eNB/CG (for non-PRACH transmission, FFS on PRACH) is PMeNB or PSeNB in power-limited case, where PMeNB+PSeNB<=PCMAX

- Maximum transmit power per eNB/CG is PCMAX in non-power limited case

- Maximum transmit power per UE is PCMAX

- When UE is power-limited, depending on some prioritization rules or transmission timing, power-scaling/dropping is applied

- FFS PHR is calculated using Pcmax,c or PMeNB/PSeNB

Candidate 1 and 4 are almost the same on the side of UE. Candidate 1 can’t make use of the advantages of PMeNB and PSeNB. With Candidate 4, UE can’t make flexible change in the extreme case that only one eNB link left so that the utility of Dynamic Power-sharing is weakened. Candidate 2 makes sense of restriction. When the transmit power per eNB/CG does not exceed PMeNB and PSeNB, the surplus power can’t be shared by the other eNB/CG. Candidate 3 needs to exchange PMeNB and PSeNB between MeNB and SeNB. If PMeNB and PSeNB need to be varied by the alteration of transmission condition, it is less likely that the non-ideal delay between MeNB and SeNB can afford the frequency of variation.

Candidate 5 is more friendly than the mentioned up ones: in power-limited case, the restriction works; in non-power limited case, surplus power to one eNB/CG can be offered to the other. But we think the restriction of PMeNB+PSeNB<=PCMAX may be broaden to PCMAX ≤ PMeNB+ PSeNB which is the same way as Pcmax,c to Pcmax in CA for each eNB/CG.

Based on the analysis, Candidate 5 is preferred. In addition, the relation among PMeNB, PSeNB and Pcmax and the way of exchange PMeNB and PSeNB between MeNB and SeNB need further study.

Proposal 3: We slightly prefer the candidate 5 and the relation among PMeNB, PSeNB and Pcmax and the way of exchanging PMeNB and PSeNB between MeNB and SeNB need further study.

Given that Dynamic Power-sharing scheme has been made, how to design appropriate prioritization among channels/signals/CGs is a crucial event. In each CG, R-11 CA prioritization can be reused. However once associated with two CGs, it is obvious that some connectivity to MCG is more important. Such as PRACH and PUCCH with HARQ-ACK to MCG should have higher priority than others. But it doesn’t mean any channels/signals to SCG are tagged with lower priority which may vary at different transmission requirements.

Proposal 4: For simple and convenient, Rel-11 CA prioritization should be reused as much as possible in the DC case.

In synchronized case, Dynamic Power-sharing can operate smoothly. However when MeNB and SeNB is unsynchronized, UL subframes transmitted to different eNBs may overlap partially as illustrated below [1]：

Figure 1: the unsynchronized case of DC UL transmission

The current UE UL transmission power is not clearly defined because the starting and ending points of different cells do not match. In other words, perfect Dynmic Power-sharing scheme can ensure no power-limited case in synchronized case which may not work any more in unsynchronized case. So in unsynchronized case, UE needs to know the actual UL transmission power at current point of time. If UE knows the timing offset between MeNB and SeNB(e.g. UE may acquire the timing offset at DL synchronization procedure), UE can decide how to perform the dynamic power sharing in the overlap subframes.

Proposal 5: Given that the timing offset between MeNB and SeNB, UE can calculate the actual subframes overlapping pattern which can guide Dynamic Power-sharing scheme work effectively.

Conclusion

In this contribution, we discuss some aspects about the Dynamic Power-sharing of DC UL transmission. We propose that:

Proposal 1: Dynamic power-sharing between two CGs/eNBs is necessary.

Proposal 2: PMeNB and PSeNB (maximum transmit power per CG/eNB) need to be defined.

Proposal 3: We slightly prefer the candidate 5 and the relation among PMeNB, PSeNB and Pcmax and the way of exchanging PMeNB and PSeNB between MeNB and SeNB need further study.

Proposal 4: For simple and convenient, R-11 CA prioritization should be reused as much as possible in the DC case.

Proposal 5: Given that the timing offset between MeNB and SeNB, UE can calculate the actual subframes overlapping pattern which can guide Dynamic Power-sharing scheme work effectively.

Reference

- R1-141899, “Outcome of the email discussion [76b-08] on TPC aspects of Dual Connectivity”, NTT DOCOMO.

- R1-141645, “UL Power control in dual connectivity”, Ericsson