3GPP TSG RAN WG1 Meeting #77			R1-142362

Seoul, Korea, 19th – 23rd May 2014

Agenda Item:	6.2.5.2.1

Source: 	CATR

Title: 	Evaluation on D2D distributed resource allocation for discovery

Document for:	Discussion and Decision

Introduction

In RAN1 76 meeting, for discovery signal, working assumption is shown as following,

- Further to the baseline agreed above, the following FFS options can be further studied (including number of discovered devices and latency) for a UE’s transmission on a discovery resource (or on a set of resources if repetition is supported)

  - Option 1: based on Tx UE transmission period and offset

  - Option 2: based on a fixed or adaptive transmission probability derived from a preconfigured/configured nominal transmission probability

  - Others

In this contribution, we give the comparison results of these two options, and also our preference.

Comparison of these two options

If the discovery resources are sufficient for all D2D UEs, the interference among D2D UEs is acceptable, and the discovery performance of option 1 is certainly better than that of option2, but when the discovery resources are limited, the interference among D2D UEs are larger, and it may be worse than that each UEs has a fixed or adaptive transmission probability.

We also simulated the different resource allocation method for layout option 3, as shown in figure 1 and figure 2, and found that no matter the discovery frames of one period is 64 or 16, option 1 is better at the first several discovery periods, and option 2 could discover minor larger number of UEs with the increase of discovery periods. The simulation assumptions are shown in the appendix.

Figure1Comparison of different resource allocation method（44*16）

Figure2Comparison of different resource allocation method（44*64）

Conclusion

In this contribution, we analyzed the two options of resource allocation and also gave the evaluation results. From the simulation results, we find that option 1 could get good performance at the several first discovery periods, and with the increase of the discovery periods option 2 shows minor performance gain than option 1.

References

[1] Draft Report of 3GPP TSG RAN WG1 #76bis

[2] R1-140098  Further discussion on resource allocation for D2D discovery CATT

Appendix

Table 1: System level simulation assumptions

| Parameter | Assumption |
| --- | --- |
| Layout | Hexagonal grid, 3 sectors per site with 19 macro-site with wraparound |
| Channel model | According to TR 36.843 v1.0.0 |
| Carrier frequency | 700 MHz |
| System bandwidth | 10 MHz |
| Network synchronization | All eNodeBs are synchronized |
| UE antenna configuration | 1 TX 1 RX |
| Transmit power | 31dBm |
| Number of D2D UEs per sector | 150 UEs |
| UE drop for D2D UEs, for discovery | As described in TR 36.843 v1.0.0 |
| Discovery Bandwidth | 44RBs |
| Discovery subframes number in one period | 16 |
| Discovery signal format | 1 PRB PUSCH with two slots |
| Resource allocation | Random allocation within each period as baseline |
| In-band emission | [W,X,Y,Z] = [3,6,3,3]dB |
| Message size | 104bits |