3GPP TSG RAN WG1 #79	R1-144602

November 17th- 21st, 2014

San Francisco, USA

Agenda item:	6.3.2.1

Source: 	Cisco Systems

Title: 	Evaluation Assumptions and Methodologies for Modeling of PHY layer Options for LAA

Document for:	Discussion and Decision

# Introduction

RAN1 #78bis marked the beginning of SID for licensed-assisted access using LTE [1]. Preliminary agreements were reached regarding the functionalities for LAA. Three co-existence scenarios were drawn for co-existence evaluation:

- Following scenarios are used for evaluation

  - Three coexistence scenarios should be evaluated (See Figures in R1-144375)

    - Coexistence scenario a:  Operator #1 deploys Wi-Fi and operator #2 deploys Wi-Fi

    - Coexistence scenario b:  Operator #1 deploys LAA and operator #2 deploys LAA

    - Coexistence scenario c:  Operator #1 deploys Wi-Fi and operator #2 deploys LAA

  - Both outdoor and indoor deployments should be considered in these scenarios

  - Coexistence scenarios with single and multiple unlicensed channels should be evaluated

    - Note: this may not need two separate simulation scenarios

  - Async between different LAA operators are baseline

    - Sync between different LAA operators can also be evaluated

For LAA-Wi-Fi coexistence, the following methodology was defined:

- Wi-Fi-LAA coexistence

  - For each UE and eNB/AP drop

    - Step 1: Performance metrics for two Wi-Fi networks coexisting in a given evaluation scenario are evaluated and recorded.

    - Step 2: Wi-Fi is replaced with LAA for the group of eNBs and UEs served by one of the Wi-Fi operators. Performance metrics of the Wi-Fi network coexisting with the LAA network are evaluated and recorded.

  - Performance metrics for the Wi-Fi operator common to the two steps are compared.

- LAA-LAA coexistence

  - Performance metrics for two LAA operators coexisting in a given evaluation scenario are evaluated and recorded.

  - Performance metrics for the two LAA operators are compared.

Certain aspects of the coexistence studies related to AP and UE dropping, traffic modeling, and PHY modeling for LAA-LTE and Wi-Fi were discussed as part of the email discussion  [2]. This contribution outlines Cisco Systems’ preferences on some of the outstanding issues that were discussed as part of the evaluation assumptions and methodologies for modeling on PHY layer options for LAA.

# Layout Scenarios for Nodes

For indoor scenario our view is that the AP deployment should be modelled after taking into account practical guidelines for enterprise AP deployments. Indoor enterprise AP deployments are typically optimized for capacity, not coverage. As an example, [3] suggests that a good rule of thumb for indoor enterprise AP density is between 3000 – 5000 sq. ft. depending on the end application (data, voice, location services etc.).

Even higher node densities have been assumed in IEEE .11ax studies [4] wherein for enterprise deployment, as shown in Figure 1, where 4 access points are deployed within a 400 sq. meter area, corresponding to an AP density equalling 100 sq. meter  (1077 sq. ft.) per AP.

Some companies suggest modelling the AP density of 4 AP within a 6000 sq. meter floor plan (120 m x 50 m) resulting in an indoor enterprise AP density of 1500 sq. meter /AP (approximately 16145 sq. ft./AP) however in our view this density is neither reflective of a capacity optimized AP deployment nor what is the observed AP density in congested areas such as malls, exhibition and in densely populated areas (e.g. 1854 AP/sq. km. in Manhattan, 4664 AP/sq. km in Vienna) [5]

Figure 1: AP positions in an indoor enterprise scenario [4]

Therefore, in order to ensure that the AP deployment modelling is rooted in reality, our recommendation is that 3GPP align their simulation assumptions to be aligned with practical enterprise AP deployment guidelines and prior simulation studies in IEEE WGs [3] [4].

Proposal 1: Adopt an indoor enterprise AP density of 3000 sq. ft. per AP to model practical field deployment recommendations [3], IEEE WG models [4] and real-world AP measurements [5].

- With the proposal of 4 AP/operator, adopt a 56m x 22m floor plan.

- No synchronization across AP transmissions even if they belong to the same operator.

# Unmanaged Wi-Fi

In reality, enterprise AP deployments have to co-exist with unmanaged devices such as Wi-Fi AP belonging to third-party operator, home APs, MiFi Internet hotspot etc. Such devices typically employ either static channel selection schemes or simple greedy channel selection schemes without any coordination with neighbouring Wi-Fi AP STAs. While enterprise APs can employ sophisticated SON schemes for channel selection, power control etc., in practice, the gains from these schemes will likely be dictated by the ratio of the unmanaged Wi-Fi devices to the managed Wi-Fi devices.

Other issues such as hidden terminal scenarios are more likely to occur in presence of unmanaged Wi-Fi devices and diminish the gains deployed in an enterprise operator deployment of LAA-LTE. Note that Wi-Fi by itself is quite optimized for addressing hidden terminal scenarios via RTS/CTS mechanism [6]. For modelling simplicity and for sake of being pragmatic in terms of simulation models, we propose at least 4 unmanaged Wi-Fi devices on each channel. However, our view is that number of unmanaged devices should not be taken into account for calculating the number of managed enterprise AP as identified in Proposal 1.

Proposal 2: Model at least 4 unmanaged Wi-Fi devices on each channel, with the caveat that the number of unmanaged devices should not be taken into account for calculating the Operator managed AP density as identified in Proposal 1.

# Carrier Number

One of the main motivations for performing the coexistence evaluations is to perform an “apples-to-apples” comparison on the relative impact of LAA-LTE versus Wi-Fi deployment of operator 1 on operator 2’s Wi-Fi network. Considering the increasing adoption of the 5 GHz band and the general trend towards ever increasing data demand, it is inevitable that the number of deployed AP for each operator will significantly outnumber the number of available channels.

The higher density of APs on a single channel has ramifications in terms of the following: -

- In terms of Listen-Before-Talk (LBT) for Wi-Fi, Wi-Fi APs for a given operator will defer to the medium upon sensing neighbour WI-FI devices belonging to either the same/different operator. Thus, the aggregate served load for the Wi-Fi (Operator 1) + Wi-Fi (Operator 2) scenario is likely to saturate (or even slightly decrease) with increasing density of AP/channel.

- Since the LTE standard by itself lacks LBT support, it is quite likely that LAA devices will be quite disruptive in terms of interference to LAA/Wi-Fi devices belonging to a different operator, as has been demonstrated in our companion contribution [7]. Even worse, due to lack of a preamble-based CCA mechanism in LTE, there is a large potential for hidden terminal occurrences due to an Wi-Fi transmitter not being able to detect MEDIUM_BUSY due to LAA transmission if the received power from the LAA device at the Wi-Fi transmitter is below the Wi-Fi STA’s energy detect threshold (typically -62 dBm).

To summarize, it is important to design LBT capability for LAA-LBT from the ground-up targeting a high density of AP for each operator on a given channel. Our preference is to model at least 4 AP for each operator on a single carrier as a simulation baseline, keeping in mind the recommended enterprise AP density.

Proposal 3:  Baseline simulation model should consider at least X = 4 AP per operator along with X (FFS) unmanaged AP (s) on a single channel (Y = 1).

- Any evaluation of multi-channel alternative (e.g. X = 4, Y = 4) should model Wi-Fi STA transmissions using channel bonding (.11ac, Wave 1) on up to a single 80 MHz channel.

# Number of UEs/Client STAs and Traffic Model

The current number (60 devices per operator) in our view does not model realistic indoor enterprise AP density. In reality assuming a given AP density and a certain density of people on each floor, this number should scale with # APs per operator. Most people in indoor enterprises have 2-3 Wi-Fi devices. Assuming AP density of 3000 sq. ft. per AP and assuming around 20-40 people in that area, this translates to between 40 - 120 Wi-Fi devices per AP in a 3000 sq. ft. area. In a 12000 sq. ft. area with 4 APs, this maps to nearly 160 – 480 Wi-Fi client associations per operator.

For sake of being pragmatic with respect to system simulation complexity, we therefore propose to model the UE density on each channel to be the lower limit (160 UE/client STA associations) yielding a maximum of 40 UE per AP per operator on each channel.

Proposal 4: Model up to 40 UE/client STAs per AP per operator on each channel.

# Traffic Modelling and DL/UL Multiplexing

In keeping with the rapid proliferation of delay-sensitive applications in unlicensed band (e.g. Face Time, Wi-Fi calling [8]), the existing 3GPP methodology of focusing solely on FTP traffic is not altogether representative of typical user activity. Hence we would prefer the coexistence studies prioritize modeling delay-sensitive traffic in addition to best effort services (e.g. FTP).

Proposal 5: Model as part of simulation a mix of best effort (FTP Model 1/3) and delay-sensitive traffic (e.g. bi-directional VoIP and video).

Current phase in the LAA-LTE SID considers only downlink transmissions over LAA devices. For a fair comparison during coexistence evaluations, it is important to ensure that the same set of hearing relationships (in terms of the set of potential transmitting devices) are maintained when the LAA-LTE devices belonging to operator 1 are replaced with a (equivalent) Wi-Fi BSS.  A similar point has been made in [10]. Our preference is that the “aggressor” LAA/Wi-Fi network transmits only over DL whereas the “victim” Wi-Fi BSS (Operator 2) shall consist of both UL and DL traffic. This ensures that the number of contending devices stays the same during co-existence studies.

Proposal 6: “Aggressor” LAA/Wi-Fi network transmits only over DL whereas the “victim” Wi-Fi BSS shall consist of both UL and DL traffic.

# Performance Metrics

Proposal 7: While modeling delay-sensitive traffic, we would like to model the Latency CDF and # STA/LAA-UE whose 98% delay <= 50 ms (voice) and <= 100 ms (video).

Proposal 8: Capture performance fairness in terms of, for a given traffic and given density of STA at each operator, how fair one technology is, while allowing the other technology to access the medium.  One possibility is to capture the TXOP probability of success as suggested by CableLabs during the email discussion [78bis-15].

# Design Methodology, Targeted Functionality and PHY/MAC Modelling Aspects

Our view is that the goals for coexistence studies should to be identify what are the requisite changes in the LTE standard in order to ensure meaningful coexistence with other RATs (especially Wi-Fi) in unlicensed spectrum.  The design methodology and the targeted functionality should be that the performance impact introduced by adding an LAA-LTE AP serving X number of UE should be no different (in terms of a reasonable set of performance metrics such as the aggregate packet throughput, PDCP delays for delay-sensitive traffic, number of retransmissions etc.) when compared adding an additional reference Wi-Fi BSS with the same number of STAs.

For meaningful comparison in our view, the reference Wi-Fi standard should reflect in a way the best possible performance in terms of state-of-the-art capabilities of Wi-Fi (e.g. A-MPDU, 256 QAM, Channel Bonding etc.).

Proposal 9: Model PHY/MAC capabilities assuming .11ac Wave 1 capabilities. More details are provided in Table 1.

Table 1: Wi-Fi device capabilities for coexistence studies

| Parameter | Recommended Value/Setting | Justification |
| --- | --- | --- |
| Frame Aggregation and Block-ACK | Enabled (burst up to 64 KB) | Both features reduce contention related overheads incurred due to RTS/CTS/IFS and individual ACKs [9]. |
| RTS/CTS | Explicitly modeled | RTS/CTS are regularly deployed in tandem with MPDU frame aggregation in .11ac access points |
| Antenna Configuration | 4 x 4: 3 | Typical .11ac antenna configuration in enterprise APs |
| MCS | Support for 256 QAM (is this common) | Feature of .11ac Wave 1 (improves the peak rate) supported in commercial devices (e.g. iPad) |
| Channel Bonding | Ability to bond 80 MHz channels | Feature of .11ac Wave 1 (improves the peak rate) supported in commercial devices (e.g. iPad) |

# Conclusions

In conclusion, the take-away from this contribution is that the coexistence evaluations should be rooted in reality in terms of modeling AP/UE densities and the technological capabilities of Wi-Fi devices. Our recommendations are based on this principle. Pragmatic choices for simulation parameters can be made in order to reduce simulation running time, complexity etc., but the methodology should be representative of real-world user activity, device capability etc.

Proposal 1: Adopt an indoor enterprise AP density of 3000 sq. ft. per AP to model practical field deployment recommendations [3], IEEE WG models [4] and real-world AP measurements [5].

- With the proposal of 4 AP/operator, adopt a 56m x 22m floor plan.

- No synchronization across AP transmissions even if they belong to the same operator.

Proposal 2: Model at least 4 unmanaged Wi-Fi devices on each channel, with the caveat that the number of unmanaged devices should not be taken into account for calculating the Operator managed AP density as identified in Proposal 1.

Proposal 3:  Baseline simulation model should consider at least X = 4 AP per operator along with X (FFS) unmanaged AP (s) on a single channel (Y = 1).

- Any evaluation of multi-channel alternative (e.g. X = 4, Y = 4) should model Wi-Fi STA transmissions using channel bonding (.11ac, Wave 1) on up to a single 80 MHz channel.

Proposal 4: Model up to 40 UE/client STAs per AP per operator on each channel.

Proposal 5: Model as part of simulation a mix of best effort (FTP Model 1/3) and delay-sensitive traffic (e.g. bi-directional VoIP and video).

Proposal 6: Model the aggressor eNB/Wi-Fi BSS to transmit DL only traffic. The victim Wi-Fi BSS shall transmit traffic over both UL and DL.

Proposal 7: While modeling delay-sensitive traffic, we would like to model the Latency CDF and # STA/LAA-UE whose 98% delay <= 50 ms (voice) and <= 100 ms (video).

Proposal 8: Capture performance fairness in terms of, for a given traffic and given density of STA at each operator, how fair one technology is, while allowing the other technology to access the medium.  One possibility is to capture the TXOP probability of success as suggested by CableLabs during the email discussion [78bis-15].

Proposal 9: Model PHY/MAC capabilities assuming .11ac Wave 1 capabilities identified in Table 1.

# References

- RP-141664, Study on Licensed-Assisted Access using LTE.

- Email discussion [78bis-15] on Detailed Coexistence Evaluation Assumptions for LAA.

- Cisco Aironet Series 2700/3700 Access Points Deployment Guide, available at http://www.cisco.com/c/en/us/td/docs/wireless/technology/apdeploy/7-6/Cisco_Aironet_3700AP.html#pgfId-92249)

- IEEE P802.11 Wireless LANs “TGAx Simulation Scenarios”, July 18, 2014.

- European Commission Joint Research Center, “JRC inputs on RLAN deployment density”

- Cisco white paper on Aironet 3600 series available at http://www.cisco.com/c/en/us/products/collateral/wireless/aironet-3600-series/white_paper_c11-713103.html.

- R1-144603, “Initial Coexistence Evaluations comparing LAA Versus Wi-Fi”, Cisco Systems.

- TMobile Wi-Fi Calling and Text (http://www.t-mobile.com/offer/wifi-calling-wifi-extenders.html).

- Boris Ginzburg, Alex Kesselman, “Performance Analysis of A-MPDU and A-MSDU Aggregation in IEEE 802.11n”.

- R1-145131, “User Plane Traffic Duplex Considerations”, CableLabs, Broadcom.