3GPP TSG RAN WG1 Meeting #80                                                                            R1-150295

Athens, Greece, 9th – 13th February 2015

Agenda Item:	7.2.3.1

Source:	              NEC

Title:	              Performance evaluation for LBT

Document for:	Discussion/Decision

Introduction

In RAN1#79, the simulation assumptions have been agreed [1]. And in [2], the initial co-existence evaluation has been summarized. It is agreed that

Agreements:

- Companies should at least provide results with LBT for coexistence evaluations.

In our previous contribution [3], we have shown that coexistence mechanism should be applied to LAA to guarantee the fairness between LAA and WiFi system. In this contribution, we will provide the initial simulation results for WiFi-LAA coexistence scenario with LBT using the agreed assumptions.

Simulation assumptions

In this contribution, we simulated 4 cases as described below:

Case 1: 4 AP + 4 LAA per cluster, λ = 0.2, file size = 0.5MB

Case 2: 2 AP + 2 LAA per cluster, λ = 0.2, file size = 0.5MB

Case 3: 4 AP + 4 LAA per cluster, λ = 0.1, file size = 0.5MB

Case 4: 2 AP + 2 LAA per cluster, λ = 0.1, file size = 0.5MB

In each case, the UE performance when LAA deploys coexistence mechanism (i.e. LBT) will be compared to that when LAA deploys nothing. The other simulation assumptions follow those in A. 1.2 and A. 2 in [4].

In addition to the agreed scenario with 4AP + 4LAA, we provide more cases for a better comparison. The 2AP+2LAA cases are simulated to evaluate the performance of LBT in sparse deployment scenario. The user perceived throughput and latency have been evaluated.

User perceived throughput

The simulation results of user perceived throughput are illustrated in Fig.1. From the simulation results we can see that LBT for LAA can dramatically increase the UPT performance of WiFi, especially in dense deployment and low data arrival case, i.e. case 3. The reason is that, if LBT is applied to LAA, the interference will be avoided and the channel quality of WiFi will be improved. In high data arrival rate cases, i.e. case 1 and case 2, LAA performance will be slightly affected. In low data arrival rate cases, i.e. case 3 and case 4, the LBT scheme can both increase the performance of WiFi and LAA. In the simulation, the data arrival rates for LAA and WiFi are the same. With high data arrival rate, LAA will suspend for longer time to avoid interference to WiFi, and the available resources will be shared by more UEs. Since LBT will also improve the channel quality of LAA, the UPT will be slightly affected.  With low data arrival rate, LAA will suspend for relatively less time to avoid interference to WiFi, and the available resources will be shared by less UEs. Since LBT will also improve the channel quality of LAA, the UPT will be improved. Detailed simulation results are shown in Table 1 and Table 2.

Observation 1: LBT for LAA can dramatically increase the UPT performance of WiFi, especially in dense deployment and low data arrival rate case.

Observation 2: In low data arrival rate cases, the UPTs of LAA are also improved.

Fig.1 Average user perceived throughput

Table 1: User perceived throughput of WiFi

| Cases |  | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) | Average UPT (Mbps) |
| --- | --- | --- | --- | --- | --- |
| Case 1 | LAA w/o LBT | 0.38 | 3.38 | 25.97 | 6.83 |
|  | LAA w/ LBT | 1.03 | 9.41 | 40.82 | 12.98 (+90.0%) |
| Case 2 | LAA w/o LBT | 0.42 | 6.73 | 37.74 | 10.86 |
|  | LAA w/ LBT | 0.95 | 9.43 | 38.46 | 12.93 (+19.1%) |
| Case 3 | LAA w/o LBT | 0.300 | 4.51 | 35.71 | 8.81 |
|  | LAA w/ LBT | 1.72 | 16.06 | 50.00 | 21.53 (+144.4%) |
| Case 4 | LAA w/o LBT | 0.37 | 9.13 | 44.94 | 13.54 |
|  | LAA w/ LBT | 1.68 | 13.99 | 50.00 | 18.59 (+40.3%) |

Table 2: User perceived throughput of LAA

| Cases |  | 5% UPT(Mbps) | 50% UPT(Mbps) | 95% UPT(Mbps) | Average UPT (Mbps) |
| --- | --- | --- | --- | --- | --- |
| Case 1 | LAA w/o LBT | 2.14 | 10.64 | 40.82 | 14.60 |
|  | LAA w/ LBT | 1.03 | 7.56 | 59.70 | 14.03 (-3.9%) |
| Case 2 | LAA w/o LBT | 1.97 | 14.44 | 66.67 | 21.00 |
|  | LAA w/ LBT | 1.96 | 13.11 | 62.50 | 20.32 (-3.2%) |
| Case 3 | LAA w/o LBT | 1.71 | 10.96 | 41.67 | 16.07 |
|  | LAA w/ LBT | 1.67 | 12.74 | 66.67 | 23.55 (+46.5%) |
| Case 4 | LAA w/o LBT | 2.17 | 17.32 | 66.67 | 23.54 |
|  | LAA w/ LBT | 2.53 | 18.87 | 66.67 | 27.28 (+15.9%) |

Latency

The latency performances of the 4 cases are also evaluated. From Fig. 2 we can see that the latency of WiFi is dramatically decreased when LAA employs LBT, especially in dense deployment and low data arrival rate case, i.e. case 3. The reasons are the same as mentioned in section 3. The WiFi channel quality is improved because of the LBT scheme of LAA. Hence the required data transmission time for WiFi will be reduced. The latency of LAA will increase, especially in dense deployment and high data arrival rate case, i.e. case 1. LAA will experience more waiting time due to the LBT scheme. But in sparse deployment and low data arrival rate case, i.e. case 4, the LBT can also reduce the latency of LAA. This may due to the reason that with LBT, the LAA will experience less interference and hence have higher spectrum efficiency. The detailed simulation results are shown in Table 3 and Table 4.

Observation 3: The LBT for LAA can dramatically decrease the latency of WiFi.

Fig. 2 Averaged latency performance

Table 3: Latency of WiFi

| Cases |  | 5% latency(ms) | 50% latency(ms) | 95% latency(ms) | Average latency(ms) |
| --- | --- | --- | --- | --- | --- |
| Case 1 | LAA w/o LBT | 155 | 1184 | 10570 | 2647.1 |
|  | LAA w/ LBT | 98 | 426 | 3897 | 982.5 (-62.9%) |
| Case 2 | LAA w/o LBT | 106 | 594 | 9494 | 2092.8 |
|  | LAA w/ LBT | 104 | 425 | 4199 | 1035.4 (-50.5%) |
| Case 3 | LAA w/o LBT | 112 | 887 | 13540 | 2981.6 |
|  | LAA w/ LBT | 80 | 250 | 2329 | 642.6 (-78.4%) |
| Case 4 | LAA w/o LBT | 89 | 438 | 10870 | 2138.2 |
|  | LAA w/ LBT | 80 | 287 | 2356 | 694.8 (-67.5%) |

Table 4: Latency of LAA

| Cases |  | 5% latency(ms) | 50% latency(ms) | 95% latency(ms) | Average latency(ms) |
| --- | --- | --- | --- | --- | --- |
| Case 1 | LAA w/o LBT | 99 | 376 | 1870 | 618.3 |
|  | LAA w/ LBT | 98 | 529 | 3739 | 1041.7 (+68.5%) |
| Case 2 | LAA w/o LBT | 60 | 277 | 2028 | 569.3 |
|  | LAA w/ LBT | 64 | 305 | 2044 | 589.8 (+3.6%) |
| Case 3 | LAA w/o LBT | 112 | 365 | 2336 | 645.7 |
|  | LAA w/ LBT | 80 | 314 | 2389 | 670.4 (+3.8%) |
| Case 4 | LAA w/o LBT | 60 | 231 | 1845 | 518.1 |
|  | LAA w/ LBT | 80 | 214 | 1580 | 435.2 (-16.0%) |

Conclusions

In this contribution, we provided simulation results for the WiFi-LAA co-existence scenario with LBT scheme. From the simulation results, we observe that:

Observation 1: LBT for LAA can dramatically increase the UPT performance of WiFi, especially in dense deployment and low data arrival rate case.

Observation 2: In low data arrival rate cases, the UPTs of LAA are also improved.

Observation 3: The LBT for LAA can dramatically decrease the latency of WiFi.

- References

RAN1#79 chairman notes, 3GPP RAN1#79 meeting, 17th -21st November, 2014.

R1- 145320, Huawei, HiSilicon, Cisco, NTT Docomo, ZTE, “Summary of initial co-existence evaluations”, 3GPP RAN1#79 meeting, 17th -21st November, 2014.

R1- 144867, NEC, “Initial simulation results for coexistence evaluation”, 3GPP RAN1#79 meeting, 17th -21st November, 2014.

[4] 3GPP TR 36.889 V0.1.1 (2014-11).