3GPP TSG RAN WG1 Meeting #80		R1-150546

Athens, Greece, 9th – 13th February 2015

Source:	QUALCOMM Incorporated

Title:	On CQI mismatch issue for NAICS

Agenda item:		6.3.2

Document for:	Discussion

# 1	Introduction

The CQI mismatch problem for IC-capable UEs has been discussed in [1]-[2]. In short, due to effects such the delay in CQI feedback, some loss in throughput is to be expected compared to the case when the downlink scheduler had a more up to date knowledge of the CQI. In this contribution, multi-cell downlink simulations are performed to quantify the throughput gain that can be obtained if the CQI mismatch issue is resolved.

# 2	Simulation assumptions

Downlink simulation assumptions are provided in Table 1. The modulation order, the number of codes and TBS in the interfering cells are randomly chosen from Table 2. Two sets of simulations are performed:

- Simulation 1 (Baseline):

  - Use the IC-capable UE receiver discussed in [3]-[4]

  - Randomly select the number of codes, modulation as well as TBS from Table 2 every TTI to schedule the interfering cells

  - Record the scheduled interference loading (codes, modulation and TBS) and the reported CQI of the UE. This would be later used in Simulation 2 to resolve the CQI mismatch issue

- Simulation 2 (Enhanced)

  - Use IC-capable UE receiver discussed in [3]-[4]

  - Schedule serving and interference cells using genie CQI and interference loading recorded in Simulation 1. As a result, no CQI delay is introduced and any CQI mismatch is resolved. The UE is scheduled using exactly the CQI it is going to observe in that TTI.

Table 1: Multi-cell downlink simulation assumptions

| Parameters | Values |
| --- | --- |
| Cell Layout | Three cell |
| Scheduler | CQI-based scheduling, 10% first BLER |
| # of UE Antennas | Dual antenna |
| HS-PDSCH Ec/Ior | -2 dB |
| P-CPICH Ec/Ior | -10 dB |
| P-CCPCH Ec/Ior | -12 dB |
| P-SCH Ec/Ior | -15 dB |
| S-SCH Ec/Ior | -15 dB |
| OCNS | Enabled (for full power transmission) |
| Propagation conditions | AW PA3, PB3, VA30, VA120 |

Table 2: Modulation, the number of codes and TBS for interference cells

| TBS | # of codes | Modulation |
| --- | --- | --- |
| 120 | 1 | QPSK |
| 3202 | 5 | QPSK |
| 4664 | 4 | 16QAM |
| 3202 | 5 | QPSK |
| 4664 | 4 | 16QAM |
| 3202 | 5 | QPSK |
| 4664 | 4 | 16QAM |
| 3202 | 5 | QPSK |
| 6438 | 10 | QPSK |
| 9377 | 8 | 16QAM |
| 9224 | 15 | 16QAM |
| 26504 | 15 | 64QAM |
| 9736 | 15 | QPSK |
| 17568 | 15 | 16QAM |
| 9736 | 15 | QPSK |
| 9719 | 15 | QPSK |
| 17548 | 15 | 16QAM |
| 3498 | 3 | 16QAM |
| 17490 | 15 | 16QAM |
| 6996 | 3 | 64QAM |

# 3	Simulation results

## The simulation results for both baseline and enhanced simulation models are summarized in Tables 3. The results show that when there is no CQI mismatch, the performance gain is less than 10%. For multipath channels such as PB3 or high speed vehicular channels such as VA120, the gain is less than 2%

## Table 3: Simulation results

| Channel | Ior1/Ioc (dB) | Ior2/Ioc (dB) | Ior3/Ioc (dB) | Throughput (kbps) |  | Throughput Gain |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Baseline | Enhanced |  |
| PA3 | 0 | 0 | 0 | 1386.62 | 1469.84 | 6% |
|  | 10 | 0 | 0 | 5603.96 | 5874.92 | 5% |
|  | 10 | 10 | 4 | 3827.48 | 4076.47 | 7% |
|  | 20 | 0 | 0 | 11451.49 | 11707.43 | 2% |
|  | 20 | 15 | 0 | 8739.76 | 9199.22 | 5% |
|  | 20 | 15 | 15 | 5533.98 | 5870.60 | 6% |
|  | 3.8 | -0.3 | -3.5 | 2848.27 | 2998.77 | 5% |
|  | 5.77 | 2.67 | 2.67 | 2983.65 | 3159.00 | 6% |
|  | 4.77 | 4.77 | -1.23 | 2714.63 | 2879.20 | 6% |
|  | 4.47 | 1.47 | -4.03 | 3012.71 | 3180.70 | 6% |
| PB3 | 0 | 0 | 0 | 1391.98 | 1414.48 | 2% |
|  | 10 | 0 | 0 | 5352.22 | 5433.89 | 2% |
|  | 10 | 10 | 4 | 3665.36 | 3721.78 | 2% |
|  | 20 | 0 | 0 | 10168.64 | 10271.74 | 1% |
|  | 20 | 15 | 0 | 7623.39 | 7719.42 | 1% |
|  | 20 | 15 | 15 | 5138.27 | 5207.20 | 1% |
|  | 3.8 | -0.3 | -3.5 | 2855.14 | 2897.88 | 1% |
|  | 5.77 | 2.67 | 2.67 | 2986.63 | 3027.33 | 1% |
|  | 4.77 | 4.77 | -1.23 | 2717.31 | 2769.48 | 2% |
|  | 4.47 | 1.47 | -4.03 | 3019.53 | 3071.47 | 2% |
| VA30 | 0 | 0 | 0 | 724.77 | 791.18 | 9% |
|  | 10 | 0 | 0 | 3757.06 | 3944.25 | 5% |
|  | 10 | 10 | 4 | 2351.51 | 2494.01 | 6% |
|  | 20 | 0 | 0 | 8132.97 | 8251.49 | 1% |
|  | 20 | 15 | 0 | 5598.50 | 5731.37 | 2% |
|  | 20 | 15 | 15 | 3483.98 | 3631.50 | 4% |
|  | 3.8 | -0.3 | -3.5 | 1783.85 | 1920.58 | 8% |
|  | 5.77 | 2.67 | 2.67 | 1840.63 | 1971.26 | 7% |
|  | 4.77 | 4.77 | -1.23 | 1634.84 | 1756.59 | 7% |
|  | 4.47 | 1.47 | -4.03 | 1903.70 | 2041.56 | 7% |
| VA120 | 0 | 0 | 0 | 786.99 | 797.41 | 1% |
|  | 10 | 0 | 0 | 3830.47 | 3869.62 | 1% |
|  | 10 | 10 | 4 | 2480.29 | 2504.37 | 1% |
|  | 20 | 0 | 0 | 7505.91 | 7524.05 | 0% |
|  | 20 | 15 | 0 | 5127.09 | 5154.82 | 1% |
|  | 20 | 15 | 15 | 3525.30 | 3547.89 | 1% |
|  | 3.8 | -0.3 | -3.5 | 1939.87 | 1985.26 | 2% |
|  | 5.77 | 2.67 | 2.67 | 1997.65 | 2031.58 | 2% |
|  | 4.77 | 4.77 | -1.23 | 1775.51 | 1797.50 | 1% |
|  | 4.47 | 1.47 | -4.03 | 2091.13 | 2117.82 | 1% |

# 3	Conclusions

## This contribution analyses the throughput gain when there is no CQI mismatch for IC-capable dual Rx UEs. The simulation results suggest that there are limited gains if the CQI mismatch issue is resolved.

# 4	References

- R1-145141, Network assisted interference cancellation and suppression for UMTS, Huawei, HiSilicon.

- R1-140684, CQI mismatch with NAIC in Hetnet, Huawei, HiSilicon.

- R1-133837, Performance of Network Assisted IC in a Hetnet Framework, QUALCOMM Incorporated.

- TR 25.800, Technical Report on UMTS Heterogeneous Networks.