3GPP TSG RAN WG1 Meeting #80		R1-150650

Athens, Greece, 9th – 13th February 2015

Source:	Ericsson

Title:	Proposed Correction to 36.213: Sidelink Resource Configuration

Agenda Item:	7.1

Document for:	Discussion and Decision

# Discussion

The draft CR for 36.213 [1] describes resource configuration for SA, data (SL-TM 2) and discovery as follows:

# 14 UE procedures related to Sidelink

A UE can be configured by higher layers with one or more PSSCH resource configuration(s). A PSSCH resource configuration can be for reception of PSSCH only, or for transmission and reception of PSSCH. The physical sidelink shared channel related procedures are described in subclause 14.1.

A UE can be configured by higher layers with one or more PSCCH resource configuration(s). A PSCCH resource configuration can be for reception of PSCCH only, or for transmission and reception of PSCCH and the PSCCH resource configuration is associated with either sidelink transmission mode 1 or sidelink transmission mode 2. The physical sidelink control channel related procedures are described in subclause 14.2.

A UE can be configured by higher layers with one or more PSDCH resource configuration(s). A PSDCH resource configuration can be for reception of PDSCH only, or for transmission and reception of PDSCH.  The transmissions of PSDCH according to a PSDCH resource configuration are associated with either sidelink discovery type 1 or sidelink discovery type 2B. The physical sidelink discovery channel related procedures are described in subclause 14.3.

Configuration of tx/rx pools was discussed by RAN1 while listing L1 RRC parameters [2]. In particular it was clarified that each pool can be either a tx, rx or tx+rx pool, as captured also in I1 of [2]: Relevant for Tx or Rx by a UE. Note 'Tx/Rx' refers to Tx and/or Rx.

Based on the above we propose to modify CR 36.213 according to the agreements.

# Text Proposal

# A UE can be configured by higher layers with one or more PSSCH resource configuration(s). A PSSCH resource configuration can be for reception of PSSCH only, for transmission of PSSCH only, or for transmission and reception of PSSCH. The physical sidelink shared channel related procedures are described in subclause 14.1.

# A UE can be configured by higher layers with one or more PSCCH resource configuration(s). A PSCCH resource configuration can be for reception of PSCCH only, for transmission of PSCCH only, or for transmission and reception of PSCCH and the PSCCH resource configuration is associated with either sidelink transmission mode 1 or sidelink transmission mode 2. The physical sidelink control channel related procedures are described in subclause 14.2.

# A UE can be configured by higher layers with one or more PSDCH resource configuration(s). A PSDCH resource configuration can be for reception of PSDCH only, for transmission of PSDCH only, or for transmission and reception of PSDCH.  The transmissions of PSDCH according to a PSDCH resource configuration are associated with either sidelink discovery type 1 or sidelink discovery type 2B. The physical sidelink discovery channel related procedures are described in subclause 14.3.

# References

- R1-145494	36.213 CR0492 (Rel 12, B) Introduction of D2D feature into 36.213	Motorola Mobility

- R1-145285, LS on RRC parameters for ProSe LTE D2D, RAN1