3GPP TSG RAN WG1 Meeting #80					  R1-150758

Athens, Greece, 9th – 13th February 2015

___________________________________________________________________Agenda item: 7.1

Source: LG Electronics

Title: Text proposal for sidelink transmission power

Document for: Discussion and Decision

1. Text proposal for maximum power of sidelink channels

It is proposed to define the maximum sidelink transmission power as follows:

| Maximum power | Condition | is set to |
| --- | --- | --- |
| , | Out-coverage | maxTxPower in ProsePreconfiguration IE |
|  | In-coverage | P-Max in SIB1 |
|  | In-coverage | discMaxTxPower in ProseDiscTxPowerInfo IE |
|  | Out-coverage | maxTxPower in ProsePreconfiguration IE |
|  | In-coverage and triggered for communication | P-Max in SIB1 |
|  | Otherwise | discMaxTxPower in ProseDiscTxPowerInfo IE |

Here, “triggered for communication” means that the SLSS/PSBCH transmission in the subframe is triggered because

- The UE is capable of communication and the eNB instructed to transmit SLSS/PSBCH by dedicated signalling, or

- The RSRP of the serving cell is less than the threshold configured for SLSS/PSBCH transmission, and the UE transmits PSCCH or PSSCH in a PSCCH period which includes the subframe for the SLSS/PSBCH transmission.

The following is the text proposal to capture the above in TS 36.213.

#### 14.1.1.5 UE procedure for PSSCH power control For sidelink transmission mode 1 and PSCCH period i, the UE transmit power is given by the following - if the TPC command field in configured sidelink grant (described in [8]) for PSCCH period i is set to 0 - if the TPC command field in configured sidelink grant (described in [8]) for PSCCH period i is set to 1 - [dBm] where is the bandwidth of the PSSCH resource assignment expressed in number of resource block and is thevalue as defined in subclause 5.1.1.1. and are provided by higher layer parameters mode1DataPo and mode1DataAlpha, respectively. is the value calculated assuming no uplink transmissions in carrier(s) other than the carrier c with setting in [6] to the value indicated by - higher layer parameter P-Max if the UE has serving cell fulfilling the S criterion in carrier c according to [TS 36.304, clause 5.2.3.2] - higher layer parameter maxTxPower otherwise, For sidelink transmission mode 2, the UE transmit power is given by [dBm] , where is the value determined by the UE for the uplink subframe corresponding to the sidelink subframe in which the PSSCH is transmitted and is the bandwidth of the PSSCH resource assignment expressed in number of resource blocks and is thevalue as defined in subclause 5.1.1.1. and are provided by higher layer parameter mode2DataPo and mode2DataAlpha, respectively and that are associated with the corresponding PSSCH resource configuration. #### 14.2.1.3 UE procedure for PSCCH power control For sidelink transmission mode 1 and PSCCH period i, the UE transmit power is given by the following - if the TPC command field in the configured sidelink grant (described in [8]) for PSCCH period i is set to 0 - if the TPC command field in the configured sidelink grant (described in [8]) for PSCCH period i is set to 1 - [dBm] where =1 and is thevalue as defined in subclause 5.1.1.1. and are provided by higher layer parameters saPo and saAlpha, respectively and are associated with the corresponding PSCCH resource configuration. is the value calculated assuming no uplink transmissions in carrier(s) other than the carrier c with setting in [6] to the value indicated by - higher layer parameter P-Max if the UE has serving cell fulfilling the S criterion in carrier c according to [TS 36.304, clause 5.2.3.2] - higher layer parameter maxTxPower otherwise, ### 14.3.1 UE procedure for transmitting the PSDCH For sidelink discovery, the UE transmit power is given by the following [dBm] where is the value calculated assuming no uplink transmissions in carrier(s) other than the carrier c with setting in [6] to the value indicated by higher layer parameter discMaxTxPower, and =2 and is thevalue as defined in subclause 5.1.1.1. and are provided by higher layer parameters discoveryPo and discoveryAlpha, respectively and are associated with the corresponding PSDCH resource configuration. ## 14.4 Physical Sidelink Synchronization related procedures The UE transmit power of primary sidelink synchronization signal and secondary sidelink synchronization signal is given by - If the UE is configured with sidelink transmission mode 1, and if the UE transmits sidelink synchronization signals in PSCCH period i, and if the TPC command field in the configured sidelink grant (described in [8]) for the PSCCH period i is set to 0 - otherwise - [dBm] , where and are provided by higher layer parameter synchPo and synchAlpha respectively and are associated with the corresponding sidelink synchronization signal resource configuration. is the value calculated assuming no uplink transmissions in carrier(s) other than the carrier c with setting in [6] to the value indicated by - higher layer parameter maxTxPower if the UE has no serving cell fulfilling the S criterion in carrier c according to [TS 36.304, clause 5.2.3.2] - higher layer parameter P-Max if the UE has serving cell fulfilling the S criterion in carrier c according to [TS 36.304, clause 5.2.3.2], and if high layer parameter syncSourceControl is configured and set to TRUE or if the UE transmits PSCCH or PSSCH in PSCCH period i - higher layer parameter discMaxTxPower otherwise,

2. Text proposal for sidelink power adjustment

The following proposal is to make the PSSCH transmission power constant in a subframe without changing uplink transmission powers in the other carriers in the power limited case. Similar changes are necessary for other sidelink channels and signals.

#### 14.1.1.5 UE procedure for PSSCH power control For sidelink transmission mode 1 and PSCCH period i, the UE transmit power is given by the following - if the TPC command field in configured sidelink grant (described in [8]) for PSCCH period i is set to 0 - if the TPC command field in configured sidelink grant (described in [8]) for PSCCH period i is set to 1 - [dBm] where is the value determined by the UE for the uplink subframe corresponding to the sidelink subframe in which the PSSCH is transmitted and is the bandwidth of the PSSCH resource assignment expressed in number of resource block and is thevalue as defined in subclause 5.1.1.1. and are provided by higher layer parameters mode1DataPo and mode1DataAlpha, respectively. For sidelink transmission mode 2, the UE transmit power is given by [dBm] , where is the value determined by the UE for the uplink subframe corresponding to the sidelink subframe in which the PSSCH is transmitted and is the bandwidth of the PSSCH resource assignment expressed in number of resource blocks and is thevalue as defined in subclause 5.1.1.1. and are provided by higher layer parameter mode2DataPo and mode2DataAlpha, respectively and that are associated with the corresponding PSSCH resource configuration. If a PSSCH transmission of a UE on sidelink subframe k for a carrier c overlaps in time domain with uplink transmissions on subframe i for carrier(s) other than carrier c, and if exceed , the UE scales such that the condition is satisfied where is the linear value of , is in uplink subframe i in carrier(s) other than carrier c, is the linear value of the UE total configured maximum output power calculated by assuming the simultaneous transmissions of PSSCH on sidelink subframe k for carrier c and uplink transmissions on uplink subframe i for other carrier(s), and is a scaling factor of for carrier where . If a PSSCH transmission of a UE on sidelink subframe k for carrier c overlaps in time domain with uplink transmissions on subframe i and subframe i+1 for carrier(s) other than carrier c, and if exceed or exceed , the UE scales for carrier c in sidelink subframe k such that the condition is satisfied.