3GPP TSG RAN WG1 #80bis                                                                               	R1-151403

20th – 24th April 2015

Belgrade, Serbia

Source: 	Qualcomm Incorporated

Title:	Simulation results for WiFi DL + UL and LAA DL only scenarios

Agenda Item:	7.2.4.1

Document for:	Discussion and Decision

# Introduction

In this contribution, we present the simulation results for the single channel indoor scenario when WiFi network has DL and UL mixed traffic and LAA network has only DL traffic.

# Simulation Assumptions

The following parameters are assumed for WiFi for the single channel indoor scenario

| WiFi Parameter | Value |
| --- | --- |
| CCA-ED | -62dBm |
| CCA-CS | SINR of 4dB |
| Defer period for backoff | Based on AIFS |
| 256 QAM | Not used |
| LDPC Codes | Used |
| RTS/CTS | Not Used |
| Max TXOP | 3ms |
| DL-UL traffic ratio | 80-20 |

The below set of parameters are assumed for LAA deployments for both indoor and outdoor scenarios.

| LAA parameter | Value |
| --- | --- |
| Scheduling assumption | All traffic on unlicensed only |
| CCA-ED | -82dBm |
| LBT Category | 3 |
| q value | 32 |
| Max TXOP | 9.5ms |
| 256 QAM | Not used |
| CCA slot duration | 20us |
| Inter operator sync | eNBs are time synchronized but channel access time may be asynchronous |
| Intra and inter-RAT detection | Not assumed |

We report the statistics at the low, medium and high offered loads in the next section, where the low, medium and high loads refer to the offer rates when the buffer occupancy of the Wi-Fi operator 2 in the step 1 is respectively 20%, 40% and 60%. The simulated results do not exactly align with these buffer occupancy marks and the reported numerical values are linearly interpolated to match these marks.

# Simulation Results

In this section, we show the simulation results for the one channel indoor scenario with LAA using LBT category 3. Table 1 shows the results.

Table 1: WiFi LAA coexistence results with non-replaced WiFi network having DL + UL traffic and replaced WiFi network having DL only traffic for the single channel indoor scenario.

| Reported parameters |  | Low load BO range for Wi-Fi Opt.1 in Step 1: 10%~25% |  |  |  | Medium load BO range for Wi-Fi Opt. 1 in Step 1: 35%~50% |  |  |  | High load BO range for Wi-Fi Opt.1 in Step 1: above 55% |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 | Wi-Fi Opt.1 in step 1 | Wi-Fi Opt.2 in step 1 | Wi-Fi Opt. 1 in step 2 | LAA Opt.2 in step 2 |
| DL: UPT CDF [Mbps] | 5% | 32.398 | 32.682 | 39.481 | 36.720 | 3.432 | 1.596 | 8.150 | 1.585 | 0.196 | 0.079 | 0.787 | 0.123 |
|  | 50% | 66.589 | 67.068 | 74.035 | 70.588 | 45.778 | 45.262 | 53.279 | 46.083 | 26.589 | 25.100 | 34.505 | 20.514 |
|  | 95% | 83.066 | 82.981 | 91.466 | 89.981 | 68.777 | 68.870 | 80.195 | 72.215 | 57.649 | 55.929 | 70.564 | 58.773 |
|  | Mean | 63.601 | 64.283 | 70.884 | 68.093 | 42.530 | 42.353 | 49.538 | 41.447 | 27.166 | 25.745 | 34.893 | 24.247 |
| DL: Delay CDF [s] | 5% | 0.123 | 0.122 | 0.101 | 0.109 | 1.166 | 2.506 | 0.491 | 2.524 | 20.408 | 50.633 | 5.083 | 32.520 |
|  | 50% | 0.060 | 0.060 | 0.054 | 0.057 | 0.087 | 0.088 | 0.075 | 0.087 | 0.150 | 0.159 | 0.116 | 0.195 |
|  | 95% | 0.048 | 0.048 | 0.044 | 0.044 | 0.058 | 0.058 | 0.050 | 0.055 | 0.069 | 0.072 | 0.057 | 0.068 |
|  | Mean | 0.063 | 0.062 | 0.056 | 0.059 | 0.094 | 0.094 | 0.081 | 0.097 | 0.147 | 0.155 | 0.115 | 0.165 |
| UL: UPT CDF [Mbps] | 5% | 25.327 | N/A | 28.724 | N/A | 4.702 | N/A | 7.066 | N/A | 2.615 | N/A | 3.305 | N/A |
|  | 50% | 58.253 | N/A | 64.193 | N/A | 39.405 | N/A | 44.554 | N/A | 21.665 | N/A | 26.903 | N/A |
|  | 95% | 79.472 | N/A | 86.027 | N/A | 64.031 | N/A | 73.461 | N/A | 52.110 | N/A | 63.144 | N/A |
|  | Mean | 56.291 | N/A | 62.033 | N/A | 37.471 | N/A | 42.596 | N/A | 23.752 | N/A | 29.353 | N/A |
| UL: Delay CDF [s] | 5% | 0.158 | N/A | 0.139 | N/A | 0.851 | N/A | 0.566 | N/A | 1.530 | N/A | 1.210 | N/A |
|  | 50% | 0.069 | N/A | 0.062 | N/A | 0.102 | N/A | 0.090 | N/A | 0.185 | N/A | 0.149 | N/A |
|  | 95% | 0.050 | N/A | 0.046 | N/A | 0.062 | N/A | 0.054 | N/A | 0.077 | N/A | 0.063 | N/A |
|  | Mean | 0.071 | N/A | 0.064 | N/A | 0.107 | N/A | 0.094 | N/A | 0.168 | N/A | 0.136 | N/A |
| 𝜌DL |  | 1.00 | 1.00 | 1.00 | 1.00 | 0.98 | 0.97 | 0.99 | 0.98 | 0.94 | 0.92 | 0.96 | 0.91 |
| 𝜌UL |  | 1.00 | N/A | 1.00 | N/A | 0.99 | N/A | 1.00 | N/A | 0.99 | N/A | 0.99 | N/A |
| BO (DL/UL reported separately) |  | 0.17/0.06 | 0.2 | 0.14/0.05 | 0.19 | 0.36/0.16 | 0.4 | 0.31/0.14 | 0.42 | 0.54/0.26 | 0.6 | 0.47/0.24 | 0.61 |
| 𝜆 |  | 1.36 Mbps |  |  |  | 1.74 Mbps |  |  |  | 1.98 Mbps |  |  |  |
| Company/tdoc: LBT category: 3 Additional information: |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the indoor scenario, we assume that the WiFi ED threshold is -62dBm while the LAA network operates using LBT category 3 with an ED threshold of -82dBm and all the traffic is routed only on the unlicensed carrier. Under the assumptions, it can be observed that there is no degradation in WiFi performance and that an LAA deployment can co-exist with a WiFi deployment having both DL and UL traffic in scenarios with low, medium and high traffic loads.

# References

[1]	3GPP TR 36.889, “Study on licensed assisted access to unlicensed spectrum”.