3GPP TSG RAN WG1 Meeting #80bis                                                                       	                      R1-151664

Belgrade, Serbia, 20th – 24th April 2015

Source:	Panasonic

Title: 	Multiple subframe code spreading for MTC UEs

Agenda Item:		7.2.1.3

Document for:	Discussion and Decision

Introduction

This contribution proposes to have multiple subframe code spreading on top of the repetition for PUSCH and PUCCH. This contribution also describes the system design of multiple subframe code spreading and simulation results.

Multiple subframe code spreading

Concept of multiple subframe codes spreading

In order to achieve 15 dB coverage enhancement, repetition over multiple subframes is one of necessary solutions. Repetition degrades significant spectral efficiency for both control and data channels since more physical resources are occupied. Techniques to mitigate the loss by repetition are necessary.

One method to mitigate repetition loss is multiple subframe code spreading on the top of the repetition. Fig. 1 illustrates signal structure of multiple subframe code spreading. Each subframe is repeated over multiple subframes and then, spread by one of orthogonal spreading sequences over multiple subframes.

Fig.1	Multiple subframe code spreading.

Multiple subframe code spreading on top of repetition has two merits. The first merit is to mitigate the spectral efficiency loss incurred by repetition. To apply multiple subframe code spreading especially to the uplink data and control channel (such as PUSCH and PUCCH) can provide more resources. Using NSF orthogonal spreading codes, up to NSF UEs/signals can be simultaneously transmitted within the same resource block over the repetition period. This can minimize the spectral efficiency degradation caused by repetition. A smaller granularity than 1 PRB transmission may have a similar effect on the spectral efficiency, but multiple subframe code spreading allows to keep the same PUSCH/PUCCH structure.

The second merit is to reduce the DCI size for minimizing the overhead. Applying multiple subframe code spreading on top of the repetition of data channels (such as PDSCH and PUSCH) or the uplink control channel (such as PUCCH), more resources can be provided by the system, and semi-static resource allocation is possible. This obviates the need for dynamic resource indication (e.g. the RBA field) in the DCI, resulting in a smaller DCI payload than currently supported.

Observation 1: Multiple subframe code spreading on top of repetition can be utilized to mitigate the spectral efficiency loss incurred by repetition and/or to reduce the DCI size.

Design of multiple subframe codes spreading

In RAN1#80 [1], RAN1 has agreed that multiple repetition levels in time domain are supported for enhanced coverage transmission. In order to realize to multiplex UEs with multiple repetition levels by multiple subframe code spreading, to have a common spreading factor (as shown in Fig. 2) would be reasonable. For larger repetition, the same spreading code sequence could be repeatedly applied.

Fig.2	The use of common spreading factor.

In RAN1#80 [1], to support multiple subframe channel estimation was agreed for both uplink and downlink data channel (such as PUSCH and PDSCH). In order to allow multiple subframe channel estimation, PRB position is the same during at least X subframes. Larger number of X could face the issue on frequency error [2]. Then, relatively smaller number of X, such as X = 4 or 8 would be reasonable. Multiple subframe code spreading is suited to the multiple subframe channel estimation as the receiver operation is almost the same as symbol level combining with multiple subframe channel estimation (Detailed description of receiver operation is shown in Appendix A). Therefore, we propose to support multiple subframe code spreading with a common spreading factor X = 4 or 8 as well as multiple subframe channel estimation. Although both X = 4 or 8 are valid, based on the evaluation of PUCCH [3], we propose to use X = 4.

Observation 2: Multiple subframe code spreading on top of repetition is suited to symbol level combining with multiple subframe channel estimation.

Proposal 1: Coverage enhancement for PUSCH and PUCCH supports multiple subframe code spreading using a common spreading factor X = 4 as well as multiple subframe channel estimation.

In RAN1#80 [1], to support frequency hopping over the system bandwidth was also agreed for both uplink and downlink data channel. Multiple subframe code spreading using a common spreading factor is also suited to the frequency hopping over the system bandwidth, as orthogonal property among different spreading codes can be kept within hopping interval as shown in Fig. 3.

Observation 3: Multiple subframe code spreading using a common spreading factor is suited to frequency hopping over the system bandwidth.

Fig.3	Combination of multiple subframe code spreading and frequency hopping.

Indication of spreading code sequence

There are two options for the indication of spreading code sequence:

- Dynamic indication: Spreading code sequence is indication by DCI. Assuming common spreading factor is 4 or 8, 2 or 3 bits are needed in DCI. Although adding new field for spreading code indication causes the increase of DCI overhead, some fields in DCI that are expected not to be used for MTC coverage enhancement, would be replaced by spreading code indication.

- Semi-static indication: Spreading code sequence can also be indicated via higher layer signalling. When the spectral efficiency is not limiting factor, the network side may not implement the receiver corresponding to multiple subframe code spreading on top of repetition. Even such situation, to configure the spreading sequence like all +1 could receive UE’s transmission. No loss from DCI size overhead is incurred. Other option would be spreading sequence is indicated in an implicit manner such as linked to UE-ID (RNTI).

Proposal 2: Dynamic or semi-static spreading code indication should be specified for multiple subframe code spreading.

Simulation results

In this section, we evaluate the link level performance of applying multiple subframe code spreading to PUSCH.The detailed parameters are listed in the Appendix B.

Figure 4 plots the BLER performance of PUSCH with number NRep of repetition as a parameter and shows the performance comparison for multiple subframe code spreading with 1 UE and 2 UEs multiplexed in code domain on the same T/F resource. The average received power of 2 UEs are assumed to be the same. NSF = 4 is used as the common spreading factor regardless of the value of NRep. Each UE uses a different multiple subframe orthogonal code. Frequency hopping over the system bandwidth with hopping period Nhop = 4 is applied, where the retune/switching time for frequency hopping is not considered in the evaluation. It can be seen from Fig. 4 that no performance degradation by UE multiplexing using multiple subframe spreading code occurs. The same coverage improvement gain can be achieved by symbol level combining with 4(= NSF) subframes [4].

Fig.4 Simulation results of multiple subframe code spreading for PUSCH repetition.

Observation 4: There is no performance degradation caused by UE multiplexing with different spreading codes, regardless of the number of repetitions. The same coverage improvement gain can be achieved as by symbol level combining with 4(= NSF) subframes.

Next, the impact of near-far effect on multiple subframe code spreading for PUSCH is discussed as difference of the power may be seen as the loss of orthogonality. Fig. 5 plots the BLER performance of PUSCH with near-far effect when NRep = 32. NSF = 4 is used as the common spreading factor. In this evaluation, 2 UE multiplexing is evaluated and the interfering UE has 0, 3, 6, 9, and 12dB larger average received SINR than the evaluated UE. It can be seen from Fig. 5 that the SINR degradation from ideal transmit power control case (P0/P1 = 0 dB) is less than 0.5 dB at BLER = 101. So for the case of 1 Hz Doppler frequency, the inter-code interference due to the loss of orthogonality is very small.

From the specification on user equipment radio transmission (TS 36.101) [5], the minimum requirement for absolute power tolerance is given as ±9.0 dB and ±12.0 dB for normal and extreme conditions, respectively. With the condition of no transmit power control command, the worst case of power difference between 2 UEs P0/P1 = 18 dB and 24 dB for normal and extreme condition, respectively. On the other hand, the case in which both UEs have worst case would be rare. Then, in this evaluation, the transmit power control error is assumed to follow a log-normal distribution. The standard deviation  of power control error is set to 3 dB or 4 dB, each corresponds to the scenario in which UEs have transmit power control error within 9 dB or 12 dB with a probability of 99.78 %. Fig.6 plots the BLER performance of PUSCH in above scenarios. NSF = 4 is used as the common spreading factor regardless of the value of NRep. It can be seen from Fig. 6 that there is only little performance degradation from ideal transmit power control case (P0/P1 = 0 dB).

Fig.5 Impact of near-far effect assuming the interfering UE has a larger (constant) average received SINR than the evaluated UE.

(a) = 3dB														(b) = 4dB

Fig.6 Impact of near-far effect assuming multiplexed UEs have power control error which follows a log-normal distribution with standard deviation .

Observation 5: Impact of near-far effect on multiple subframe code spreading is very small.

Conclusion

In this contribution, we discussed multiple subframe code spreading on top of the repetition and showed some simulation results when using repetition and multiple subframe code spreading on PUSCH. Based on the discussion presented, we summarize our views through the following observations and proposal:

Observation 1: Multiple subframe code spreading on top of repetition can be utilized to mitigate the spectral efficiency loss incurred by repetition and/or to reduce the DCI size.

Observation 2: Multiple subframe code spreading on top of repetition is suited to symbol level combining with multiple subframe channel estimation.

Observation 3: Multiple subframe code spreading using a common spreading factor is suited to frequency hopping over the system bandwidth.

Observation 4: There is no performance degradation caused by UE multiplexing with different spreading codes, regardless of the number of repetitions. The same coverage improvement gain can be achieved as by symbol level combining with 4(=NSF) subframes.

Observation 5: Impact of near-far effect on multiple subframe code spreading is very small.

Proposal 1: Coverage enhancement for PUSCH and PUCCH supports multiple subframe code spreading using a common spreading factor X=4 as well as multiple subframe channel estimation.

Proposal 2: Dynamic or semi-static spreading code indication should be specified for multiple subframe code spreading.

- Reference

- 3GPP RAN1#80, Chairman’s note

- R1-133587,	“Implementation Considerations for Coverage Enhancements for MTC,” Qualcomm Incorporated

- R1-151668, “Discussion and performance evaluation on PUCCH for MTC UEs”, Panasonic

- R1-150312, “Discussion and performance evaluation on PUSCH coverage enhancement,” Panasonic

- 3GPP TS36.101, “User Equipment (UE) radio transmission and reception (Release 12),” Dec. 2014.Appendix A: System model of PUSCH repetition with multiple subframe code spreading

Fig.6 illustrates the system model of PUSCH repetition with multiple subframe code spreading. At the UE side, subframe-level orthogonal code spreading is applied

,																										(1)

where Si(n, k) is the frequency-domain transmitted symbol after multiple subframe code spreading at k-th subcarrier of n-th symbol in i-th (i = 0~NRep-1) subframe for PUSCH repetition. NRep denotes the number of repetitions. D(n, k) is the frequency-domain transmitted symbol at k-th subcarrier of n-th symbol and it is the same over the repetition period (for i = 0~NRep-1). Cu(i), i = 0~NRep-1, is the u-th multiple subframe spreading code sequence. When the multiple subframe spreading factor is NSF, multiple subframe spreading code sequence is given by .

At the receiver of eNodeB, subframe-level despreading and symbol combining are carried out by

,																						(2)

where Rcomb,j(n, k), j = 0~NRep/NSF-1, is the frequency-domain received signal after subframe-level despreading and symbol combining. Ri(n, k) is the frequency-domain received signal at k-th subcarrier of n-th symbol in i-th subframe for PUSCH repetition. After despreading and symbol combining, channel estimation is carried out by using the combined received reference symbol of Rcomb,j(n, k) and then, MMSE-FDE is carried out by using channel estimate as

,																							(3)

where Wj(n,k) is the frequency domain equalization weight. After FDE, NRep/NSF equalized received signals are combined and the frequency-domain received signal after repetition combining is given by

.																									(4)

The above combined symbols are input to the SC-FDMA demodulation.

Fig.6	System model of PUSCH repetition with multiple subframe code spreading.

- Appendix B: Simulation parameters

| Parameters | Value |
| --- | --- |
| Number of antennas | 1×2 with low correlation |
| Number of multiplexed UEs | 1, 2 |
| Number of repetition | NRep = 4, 8, 16, 32, 64, 128 |
| Channel model | EPA (Doppler frequency, fD = 1 Hz) |
| Frame format | PUSCH 1 RB |
| MCS | MCS 5 |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz |
| Timing error | Ideal |
| Transmit power control error | Average received power of interfering UE is 0 to 12 dB larger than that of evaluated UE. or Multiplexed UEs have power control error which follows a log-normal distribution with standard deviation  (= 3 or 4 dB). |
| Residual frequency offset | 20Hz |
| Channel estimation | Realistic multiple subframe channel estimation (Averaging over NSF = 4 subframes) |
| Equalization | MMSE-FDE |