3GPP TSG RAN WG1 Meeting #80bis	      R1-151669

Belgrade, Serbia, 20th - 24th April 2015

Source:	Panasonic

Title: 	Rel.13 MTC impact to PBCH

Agenda Item:		7.2.1.6

Document for:	Discussion and Decision

Introduction

This document discusses PBCH contents and PBCH transmission method. We propose following.

Proposal 1: 5 bits in current PBCH is used for TBS, time and frequency position of MTC SIB1.

Proposal 2: PBCH is sent only in SF#0.

Discussion

PBCH content

Related to the discussion on the determination of the start OFDM symbol in the last RAN1 meeting, whether unused bits in PBCH are used or not was discussed. Some offline discussion was held and report is provided in [1]. The same PBCH is used for both current UEs and Rel.13 MTC UEs. Therefore, backward compatibility is required to be kept.

According to RRC spec TS36.331 [3], PBCH content is defined as MIB(MasterInformationBlock). Ten spare bits are available. These 10 bits were intentionally kept reserved for future usage [4].

We propose 5 bits in reserved 10 bits in PBCH are used for the indication of TBS and time/frequency resource usage of MTC SIB1 and not to use "Physical downlink control channel for MTC".. Using "Physical downlink control channel for MTC" for  MTC SIB1 requires additional resources for Rel. 13 MTC and further delay the reception of MTC SIB1. In our view, 5 bits can be sufficient for the indication of SIB1 location. Following is example of 5 bits usage in PBCH.

As MTC SIB1 transport block size, we think 3 sizes could be sufficient. We are tempted to only one size for MTC SIB1 TBS but thinking for future usage/extension, 3 sizes are proposed.

As MTC SIB1 resource utilization, once in every 20, 40 or 80ms order could be reasonable number in order to sets total number of MTC SIB overhead less than 5%. Note that once in every 20, 40 or 80ms  order does not mean to transmit exactly once in every 20, 40 or 80ms. By taking into account frequency hopping , multiple channel estimation gain and DRX operation, how to transmit SIB1 is better needs further discussion. Once in every 20ms could also mean 2 contiguous subframes in every 40ms as the resource utilization. The reason to have different TBS size in the same overhead is SIB1 is constant transmission and the reception delay is up to UE implementation. Larger TBS just means more delay if the resource utilization is same.

As time/frequency location of MTC SIB1, some part of the time/frequency resource would be determined by the PCID of this cell but we see these could be insufficient in order to provide flexible usage of SIB1 position. Note that before SIB1 reception, MBSFN subframe configuration and TDD configuration are not known by UE. MTC SIB1 time/frequency location needs to be flexible to accommodate different MBSFN subframes usage and TDD configurations.

Table 1: Example of 5 bits usage in PBCH reserved bits

| 5 bits reserved bits value | Transport block size of MTC SIB1 | SIB1 resource utilization | Time/frequency location and frequency hopping |
| --- | --- | --- | --- |
| 0 | No Rel.13 MTC support | No Rel.13 MTC support | No Rel.13 MTC support |
| 1 | Small size | Once in every 80ms order | Location X. No hopping |
| 2 | Medium size | Once in every 80ms order | Location X. No hopping |
| 3 | Large size | Once in every 80ms order | Location X. No hopping |
| 4 | Small size | Once in every 80ms order | Location X. With hopping |
| 5 | Medium size | Once in every 80ms order | Location X. With hopping |
| 6 | Large size | Once in every 80ms order | Location X. With hopping |
| 7 | Small size | Once in every 40ms order | Location Y. No hopping |
| 8 | Medium size | Once in every 40ms order | Location Y. No hopping |
| 9 | Large size | Once in every 40ms order | Location Y. No hopping |
| 10 | Small size | Once in every 40ms order | Location Y. With hopping |
| 11 | Medium size | Once in every 40ms order | Location Y. With hopping |
| 12 | Large size | Once in every 40ms order | Location Y. With hopping |
| 13 | Small size | Once in every 40ms order | Location Y. With hopping |
| 14 | Medium size | Once in every 40ms order | Location Y. With hopping |
| 15 | Large size | Once in every 20ms order | Location Y. With hopping |
| 16 | Small size | Once in every 20ms order | Location Y. With hopping |
| 24 | Large size | Once in every 20ms order | Location Z. With hopping |
| 25 | Small size | Once in every 20ms order | Location Z. With hopping |
| 26 | Medium size | Once in every 20ms order | Location Z. With hopping |
| 27 | Large size | Once in every 20ms order | Location Z. With hopping |
| 28 to 31 | Reserved for future usage | Reserved for future usage | Reserved for future usage |

We propose following.

Proposal 1: 5 bits in current PBCH is used for TBS, time and frequency position of MTC SIB1.

PBCH transmission method

In the last RAN1, PBCH transmission method was discussed[5] and following options were agreed.

- Option 1: Repetition in SF#0

- Option 2: Repetition in SF#0 + repetition in SF#5 in odd frames.

- Option 3: Repetition in SF#0 + repetition in 1 other sub-frame in all frames

- Option 4: Repetition in SF#0 + repetition in 3 other sub-frames in all frames

SF#5 in even subframes are used for current SIB1 of normal UEs. If SF#5 in odd subframes are used for PBCH further, MTC SIB1 subframe position is more restricted especially when the subframes usage is congested i.e. cell is operating TDD, eICIC, MBMS and so on. Although MTC SIB1 time/frequency location is proposed to be indicated in MIB, SF#5 in odd subframe is very good candidate for MTC SIB1 transmission subframe.

SIBs are generally bigger payload and more restriction on the transmission opportunity. Therefore, we see SIBs reception time is more critical than PBCH reception. Therefore, even PBCH reception time is reduced by option 2, 3 and 4, the consequence is more restriction on SIB transmission and more delay of SIBs reception. It means more UE power consumption. Therefore, we propose to agree option 1: Repetition in SF#0 for PBCH.

Proposal 2: PBCH is sent only in SF#0.

Conclusion

We discussed PBCH contents and PBCH transmission method. We propose following.

Proposal 1: 5 bits in current PBCH is used for TBS, time and frequency position of MTC SIB1.

Proposal 2: PBCH is sent only in SF#0.

- Reference

- R1-150865, "MIB contents offline discussion", Panasonic, Nokia Networks, Nokia Corporation

- R1-150314, "Starting symbol of MTC", Panasonic

- TS36.331

- R1-084063, " LS on transport block size on BCH", TSG-RAN WG1

- R1-150872, " WF on PBCH Coverage Enhancement"	Huawei, HiSilicon, Ericsson, InterDigital