3GPP TSG RAN WG1 Meeting #80bis								R1-151701

Belgrade, Serbia, April 20th – 24th, 2015

Source:	Sharp

Title:	Common messages for Rel-13 MTC UEs

Agenda Item:	7.2.1.5

Document for:	Discussion and Decision

- Introduction

The following agreements were achieved in RAN1#79 [1]:

Agreement: - RAN1 recommends that RAN2 consider introducing new SIB(s) for Rel-13 low complexity UEs in normal and enhanced coverage. - RAN1 recommends RAN2 to consider limiting support of mobility for Rel-13 low complexity UEs to reduce SIB size at least in enhanced coverage. - RAR/Paging messages for Rel-13 low-complexity UEs and/or UEs operating coverage enhancements (CE) are transmitted separately from RAR/Paging messages for other UEs - RAR/paging message intended for Rel-13 low-complexity UE and/or UE operating CE can support PDSCH subframe bundling/repetition with multiple bundle sizes/repetition levels - For paging, from RAN1 perspective, followings are beneficial - The eNB needs knowledge that the UE to be paged is a Rel-13 low-complexity UE and/or is a UE that is to be paged using CE - If possible, it is beneficial for eNB to have knowledge on the required amount of coverage enhancement during Paging message transmission

In this contribution we share our considerations on the common control messages (SIB/RAR/Paging).

- SIB transmission

According to the above agreements, new SIB(s) should be introduced for Rel-13 low complexity UEs in normal and enhanced coverage. However, the required system information is different for these two types of UEs. According to the objective of this WI, reduction of measurement time and measurement reporting should be considered for power consumption reduction. Hence, the neighbouring-cell-related system information (e.g. SIB4-SIB8) is not required for the low complexity UEs in enhanced coverage. But this kind of system information may be useful for the low complexity UEs in normal coverage.

If the low complexity UEs in normal and enhanced coverage share the same SIBs (or if all the SIBs for MTC UE are shared by the two types of UEs), low complexity UEs in enhanced coverage would have to acquire some system information that are not required for them, which would waste power of low complexity UEs in enhanced coverage. Hence, only the SIB which contains common information for these two types of UE should be shared by low complexity UEs in normal and enhanced coverage. Other SIB(s) (e.g. SIB4) should be dedicated for one of these two types of UEs.

Proposal 1: SIB(s) that are not required for both types of UEs should be separately provided for each one of them.

- RAR transmission

As discussed in [2], joint encoding of RAR intended for different UEs may not be suitable for bandwidth reduced UEs and coverage enhanced UEs. Hence, it makes sense to consider UE-specific RAR for bandwidth reduced UEs and coverage enhanced MTC UEs. However, for UEs with the same repetition level, multiple MAC RARs can be still jointly encoded in one RAR. And RAR transmissions to the same or different repetition level UEs can be frequency multiplexed. So, in our view, UE-specific and/or repetition level specific RAR transmission should be considered further.

Proposal 2: UE-specific and/or repetition level specific RAR transmission should be considered further.

In RAN1#79, it was confirmed that multiple PRACH repetition levels for PRACH enhancement agreed in Rel-12 LC-MTC are applied for Rel-13 low-complexity UEs [1]. In RAN1#80, it was agreed that there is one to one mapping between PRACH repetition level and PRACH resource set [3]. Therefore, an eNB can acquire the UE’s RAR repetition level after receiving the corresponding PRACH preamble. The resources and response window size for RAR transmission can be implicitly indicated based on the RAR repetition level or its corresponding PRACH resource set.

Proposal 3: The resources and response window size for RAR transmission should be implicitly indicated based on RAR repetition levels.

- Paging transmission

Similar to RAR transmission, multiplexing of several paging messages intended for different UEs into a single jointly encoded transport block may not be suitable for bandwidth reduced MTC UEs. If all paging messages are restricted to reduced bandwidth, there will be a negative impact on the coverage and capacity of legacy paging messages. In the coverage enhanced case, where each Paging message needs to be repeated, the problem becomes even worse. Furthermore, a conservative eNB has to unnecessarily repeat the paging messages addressed to low complexity MTC UEs in normal coverage – which is very inefficient from the resource usage perspective.

Hence, it makes sense to consider UE-specific paging messages for bandwidth reduced and coverage enhanced MTC UEs. Given that the Paging messages can be frequency multiplexed or jointly coded, transmission of paging messages to UEs with the same repetition level could be still possible when repetition is applied.

Proposal 4: UE-specific and/or repetition level specific paging should be considered further.

- Conclusions

In this contribution, we discussed the high layer common control messages like SIB, RAR and Paging. The following proposals are concluded accordingly:

Proposal 1: SIB(s) that are not required for both types of UEs should be separately provided for each one of them.

Proposal 2: UE-specific and/or repetition level specific RAR transmission should be considered further.

Proposal 3: The resources and response window size for RAR transmission should be implicitly indicated based on RAR repetition levels.

Proposal 4: UE-specific and/or repetition level specific paging should be considered further.

References

- 3GPP RAN1 #79 chairman’s notes, November 2014.

- R1-142541, “On Reduced UE Bandwidth and Enhanced Coverage for MTC”, Ericsson, Nokia Networks, RAN1#77, Seoul, Korea, May 2014.

- 3GPP RAN1 #80 chairman’s notes, February 2015.