3GPP TSG RAN WG1 Meeting #80bis	                                                                   R1-151985

Belgrade, Serbia, 20th - 24th April 2015

Source:	NTT DOCOMO

Title:	Investigation on Reciprocity Based Elevation Beamforming and FD-MIMO in TDD Systems

Agenda Item:	7.2.5.2.1

Document for: 	Discussion and Decision

# Introduction

FD-MIMO operation in TDD systems is very promising as the channel recoprocity can be exploited for downlink CSI acquisition. At the RAN1#80 meeting, it was agreed that companies shall consider the potential TDD enhancements for EBF/FD-MIMO in the following aspects [1]:

- SRS enhancements

  - Increase number of combs

  - Extend SRS to more resources (e.g. DMRS or PUSCH resources)

  - Precoded SRS

  - 4Tx antenna switching

- CSI feedback without PMI

  - CQI based on beamformed CSI-RS

  - RI feedback

- Other enhancements are not precluded.

In addition, a SRS error model was agreed in order to enable more realistic evaluation for the reciprociry based FD-MIMO transmission [2]. In this contribution, we investigate TDD reciprocity based transmission for FD-MIMO, based on the agreed SRS impairment model. We first study the performance benefit of increasing the number of TXRUs from 8 to 32. Then we make a numerical investigation of the potential benefit of improving the SRS capacity. In addition, we discuss other possible enhancements for elevation beamforming and FD-MIMO operations in TDD systems such as CQI report, etc.

# Performance Evaluation of FD-MIMO in TDD Systems

In this section, we investigate the FD-MIMO performance in the HetNet scenario with separate frequency bands, which seems to be one promising TDD use case. As agreed at the RAN1#78bis meeting, the macro layer is only utilized for cell selection and the performance statistics is only obtained in the small cell layer.  In the simulation, we adopt the agreed SRS error model framework [2], with the parameter  obtained from a lookup table. The detailed derivation of the parameter  is given in Appendix B. The FTP traffic model is adopted for all evaluations. The performance is characterized by the mean, 5% and 50% user packet throughput (UPT).

## Performance Benefit of Large TXRU number

We first evaluate the performance benefit of having a larger TXRU number. For performance comparison, different TXRUs, i.e., Q=8 and 32 are considered based on a same antenna array with (M, N, P) = (4, 4, 2). In order to characterize the actual SRS interference issue, a baseline SRS error model is considered. It is assumed that all UEs are randomly assigned to 4 interfering groups. The SRS sent by the UEs in the same group will interfere with each other. The evaluation results of the UPT values at a medium traffic load situation are summarized in Table 1.

Table 1: System performance with SRS impairment

| (M, N, P, Q) for small cells | Mean UPT (Mbps) | 5%-UPT (Mbps) | 50%-UPT (Mbps) |
| --- | --- | --- | --- |
| (4, 4, 2, 8) | 18.0 (0%) | 2.3 (0%) | 13.9 (0%) |
| (4, 4, 2, 32) | 26.4 (47%) | 6.4 (178%) | 23.7 (71%) |

According to the results in Table 1, we can make the following observations.

Observation 1: In reciprocity-based TDD systems, the antenna array with 32 TXRUs presents a large potential performance gain over the same antenna array with 8 TXRUs, e.g. 47%, 178% and 71% in respects of mean, 5%- and 50%- of UPT, respectively.

Observation 2: FD-MIMO fits together well with reciprocity-based system. Even with small number of antenna elements, recipricity-based system achieves a significant potential perfromance gain according to the number of TXRUs thanks to reciprocity-based CSI acquisition without cost of CSI-RS and CSI feedback overhead.

## Benefit of SRS Capacity Enhancement

It can be seen from the SRS error model [2] that the error is proportional to the SINR. In another word, the interference in the SRS transmission is a performance limitation factor. To investigate this issue, we evaluate three cases, i.e., the ideal SRS, the baseline, and the enhanced SRS cases. In case of ideal SRS, the SRS are assumed to be received without any interference. In the baseline and the enhanced SRS cases, UEs are randomly assigned to 4 and 16 groups respectively. The SRS sent by the UEs in the same group will interfere with each other. Accordingly, in the enhanced SRS case, there is a lower probability of inter-SRS interference as there will be less UEs in the same group, compared to that situation in the baseline case. The simulation results are listed in Table 2.

Table 2: System performance of FD-MIMO (4, 4, 2, 32) with different SRS assumption (capacity)

| SRS assumption (capacity) | Mean UPT (Mbps) | 5%-UPT (Mbps) | 50%-UPT (Mbps) |
| --- | --- | --- | --- |
| Ideal SRS (infinite) | 30.3 (0%) | 10.7 (0%) | 27.4 (0%) |
| Baseline (1x) | 26.4 (-12.9%) | 6.4 (-40.2%) | 23. 7 (-13.6%) |
| Enhanced SRS (4x) | 29.5 (-2.5%) | 9.3 (-12.7%) | 26.7 (-2.5%) |

Based on the evaluation results in Table 2, we can make the following observations.

Observation 3: There is a large performance gap between the ideal and baseline SRS cases due to the inter-SRS interference issue, e.g. -12.9%, -40.2% and -13.6% in respects of mean, 5% and 50% of UPT, respectively.

Observation 4: In reciprocity-based TDD systems, SRS capacity enhancement may improve the performance of FD-MIMO.

It shall be noted that the SRS is not only used for exploitng the channel reciprocity, but also for UL link adaptation. With the demonstration of reciprocity based 3D MIMO performance benefit, it is becoming more important to focus on the SRS enhancement to fulfill the requirement of delivering reliable channel estimation, since it can be easily understood that the required reception quality is higher for channel reciprocity rather than UL link adaptation. Regarding the SRS enhancement, candidate schemes together with the pros and cons are summarized in Table 3.

Table 3: Candidate schemes for SRS enhancement

| Candidate scheme | Pros | Cons |
| --- | --- | --- |
| More combs (>2) | No additional resource. | - Shorter ZC sequence which causes processing gain loss. |
| More cyclic shifts (>8) | No additional resource. | - The inter-CS distance is shortened, therefore, only work for a small delay spread scenario |
| Additional SC-FDMA symbols | Remarkable capacity improvement. | - Additional overhead in the uplink. |
| Precoded SRS | Less SRS may be needed | - Proper precoder design and related signaling shall be defined - Introduction of additional constraints on multi-layer transmission due to partial channel reciprocity caused by SRS precoding |

Proposal: Necessity of SRS capacity enhancement should be determined with a study of  the performance benefit and the associated system impact.

# Other Possible TDD Enhancement Requirements

In the legacy system, the CQI in a TDD system is derived based on the assumption of single antenna or TxD transmission, i.e., TM 1 or 2. It can be easily imagined that informed CQI is highly different from optimam one, since precoding gain is not considered for TM1/2-based CQI. For FD-MIMO, the gap is expected to be enlarged. One possible solution is to derive CQI assuming precoding gain. For instance, UE calculates RI, PMI, CQI as the procedure for TM 9 and informs RI and CQI only. This method may not be much accurate but requires less specification impact and helps to achieve better link adaptation. Another possibility is utilizing precoded CSI-RS. This scheme can be attractive in a TDD system as channel reciprocity can be exploited for designing the CSI-RS precoder, which can help to realize UE-specific CSI-RS and lead to better spatial resolution. The required RE for transmitting the precoded RS is no more proportional to the TXRU number, which leads to a reduction of the DL overhead. In order to support reciprocity based transmission, it may be beneficial to define CQI derivation based on precoded CSI-RS.

Additionally, UE is usually equipped with one transmit antenna but two receive antennas. In this case, only partial reciprocity holds. That is, when the eNB estimates the channel from the UL SRS, it can only measures a rank-1 channel, since SRS can only be sent from the single UE transmit antenna. However, the two receive antennas at the UE indicate the capability of DL rank-2 transmission. If the precoding at the eNB only expliots the partical reciprocity based the rank-1 channel state information, it is unable to make efficient rank-2 transmission in DL. In this sense, additional feedback design is needed to support multi-layer transmission with partical reciprocity. Evaluation results shown in [3] demonstrate the performance gap between reciprocity based transmission with 1-Tx SRS and 2-Tx SRS. Such a practical issue shall be addressed in RAN1 to look for an appropriate solution.

# Summary

In this contribution, we investigate TDD reciprocity based transmission for FD-MIMO with SRS impairment.  Based on the performance evaluation, we make the following observations and proposals.

Observation 1: In reciprocity-based TDD systems, the antenna array with 32 TXRUs presents a large potential performance gain over the same antenna array with 8 TXRUs, e.g. 47%, 178% and 71% in respects of mean, 5%- and 50%- of UPT, respectively.

Observation 2: FD-MIMO fits together well with reciprocity-based system. Even with small number of antenna elements, recipricity-based system achieves a significant potential perfromance gain according to the number of TXRUs thanks to reciprocity-based CSI acquisition without cost of CSI-RS and CSI feedback overhead.

Observation 3: There is a large performance gap between the ideal and baseline SRS cases due to the inter-SRS interference issue, e.g. -12.9%, -40.2% and -13.6% in respects of mean, 5% and 50% of UPT, respectively.

Observation 4: In reciprocity-based TDD systems, SRS capacity enhancement may improve the performance of FD-MIMO.

Proposal: Necessity of SRS capacity enhancement should be determined with a study of  the performance benefit and the associated system impact.

# References

- 3GPP, “Draft Report of 3GPP TSG RAN WG1 #80 v0.1.0,” Feb. 2015.

- 3GPP, R1-150867, ZTE, “WF on SRS estimation error modeling,” Feb. 2015

- 3GPP, R1-150445, CMCC, “Baseline results for 16/32/64 TXRU with channel reciprocity” Feb. 2015.

# Appendix A

Table A: Evaluation Assumptions for Reciprocity Based Transmission Schemes in Heterogeneous Network Scenario

| Parameter | Values |  |
| --- | --- | --- |
|  | Macro cell (only for cell association) | Small cell (for performance evaluation) |
| Carrier frequency | 2 GHz | 3.5 GHz |
| System bandwidth | 10 MHz (50 RBs) | 10 MHz (50 RBs) |
| Macro ISD | 500 m |  |
| eNB antenna configurations | (M, N, P) = (8, 4, 2), MTXRU = 1, (dH, dV) = (0.5, 0.8), θetilt = 100 degs. | (M, N, P) = (4, 4, 2), MTXRU = 1, (dH, dV) = (0.5 , 0.5 ) |
| UE antenna configurations | 2 X-pol (0 / 90 deg.) |  |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |  |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |  |
| Polarized Antenna modeling | Model-2 from TR36.873 |  |
| Total BS Tx power | 46 dBm | 30 dBm |
| Channel Model | 3D-UMa | 3D-UMi |
| Handover margin | 3dB |  |
| Number of clusters per macro cell | 1 |  |
| Number of small cells per cluster | 4 |  |
| UE distribution | 2/3 UEs randomly and uniformly dropped within the clusters, 1/3 UEs randomly and uniformly dropped throughout the macro geographical area. 20% UEs are outdoor and 80% UEs are indoor. |  |
| UE speed | 3 km/h |  |
| Radius of small cell center dropping in a eNB cluster (RC) | 50 m |  |
| Radius of UE dropping in a UE cluster | 70 m |  |
| Minimum distance (2D distance) | Macro – small cell cluster center: 105 m |  |
|  | Small cell area center – small cell area center: 40 m |  |
|  | Small cell cluster center – small cell cluster center: 100 m |  |
|  | Macro – UE: 35 m |  |
|  | Small cell – UE: 10 m |  |
| MIMO scheme | TM10, single CSI process, dynamic SU/MU-MIMO with rank adaptation |  |
| UE receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel. 12 [71-12] assumptions |  |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel. 12 [71-12] assumptions |  |
| Feedback scheme | PUSCH Feedback mode 3-0 |  |
| CSI-RS transmission interval / CSI feedback interval | 5 ms |  |
| SRS transmission interval | 5 ms |  |
| Number of UE transmit antennas | 2 |  |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes |  |
| Scheduler | Proportional fairness based frequency selective scheduling |  |
| Control delay | 5 ms |  |
| HARQ, Round trip delay | Chase combining, 8 ms |  |
| Metrics | Mean, 5%, 50% UPT |  |

# Appendix B

We establish a realistic SRS error model by assuming MMSE based channel estimation at the eNB side, i.e., MMSE-based SRS error model. The estimated channel can be written as:

where  is the actual channel fading vector,  is zero mean complex distributed error vector, i.e.  with  an  identity matrix, and  is a scaling factor for power normalization purpose.

In the model, the covariance matrix of the error vector  can be written as:

where  is the channel covariance matrix, and  is that known at the eNodeB channel estimator, e.g., derived based on the assumption of the power delay profile (PDP).  is the SINR on SRS reception.

In the calculation of SINR, intra-cell interference is disregarded and only inter-cell interference is considered. Above all, all the UEs are uniformly and randomly divided into 4 groups, corresponding to 2 SC-FDMA symbols and 2 combs. Each UE only suffers the interference from the intra-group UEs associated to other cells. Moreover, the processing gain of CAZAC sequence is 8.5 dB.

Considering the fact that actual PDP of the channel cannot be exactly known at the receiver side, without loss of generality, a uniform PDP within cylcic prefix (CP) length is assumed at the channel estimator and therefore the element at the mth row and the nth column of  of can be calculated as

where  and  are the CP length and subcarrier spacing of the OFDM system, respectively.

Finally, the variance of the error  can be obtained by

In other words, the equivalent parameter  can be expressed by

.