3GPP TSG RAN WG1 Meeting #80bis	       R1-152035

Belgrade, Serbia, 20th - 24th April 2015

Source:	CMCC

Title:	Evaluation of SRS enhancements

Agenda Item:	7.2.5.2.1

Document for:	Discussion and Decision

- Introduction

In 3GPP RAN #65 meeting, a new study item (SI) on EBF and Full Dimension MIMO (FD-MIMO) was approved for Release 13 [1].  Reciprocity based operation is sensible for 3D-MIMO system, because the feedback overhead does not scale with number of Tx antenna, i.e., CSI reporting overhead and CSI-RS transmission overhead maintains the same even if the number of transmit antenna increases. Details and suggestions on potential spec impact for reciprocity based operation are discussed in [2].

This contribution gives both link level and system level simulation results for SRS enhancements.

- Possible SRS enhancements

SRS is usually transmitted using UpPTS which consists of 2 SC-FDMA symbols in TD-LTE network with uplink-downlink configuration 1&2. Besides, each SC-FDMA symbol has two combs to transmit SRS, and each comb contains 8 cyclic shifts to carry 8 orthogonal users. Considering the performance degradation of using all 8 cyclic shifts for one comb, only 4 cyclic shifts are usually used in implementation. Thus, every 5ms can support at most 16 orthogonal SRS.

For 1Tx SRS at UE side, there are maximally 16 UEs can transmit wideband SRS every 5 ms, which is likely enough from SRS capacity perspective. For 2Tx SRS at UE side, there are maximally 8 UEs that can transmit wideband SRS every 5 ms, in which case certain SRS capacity enhancement may be beneficial. Such enhancement may include such as extending SRS resource to last two OFDM symbols, using UL DMRS REs, etc. Due to time and resource limitation, SRS capacity enhancement is not evaluated in this contribution.

Another potential area of SRS enhancement is to reduce SRS inter-cell-interference and increase SRS transmit power, mainly to improve cell edge performance. Increasing comb from 2 to 4 can help in this area, in which transmit power per RE can be doubled with same SRS bandwidth.  Meanwhile, the probability of SRS collision is also half as before because SRS in different comb would not collide with each other. In the following sections we evaluate performance gain of RFP from 2 to 4.

- Evaluations of SRS enhancement of RPF=4

Here, both link level and system level simulation results are provided to see the performance with SRS RPF 4.

3.1.  Link level evaluation

UMi channel model is applied in LLS with 1 transmit antenna and 1 receive antenna assumption. The system bandwidth is 10MHz and channel estimation is based on LMMSE. MSE curves with different SRS bandwidth, RPF and power are depicted in Fig.1.

Fig. 1. MSE curves of different SRS transmission schemes

From Fig.1, we can see that while there is only 1 user transmitting on the SRS resource (i.e. only 1 cyclic shift is used), RPF 4 has almost the same MSE performance with RPF 2. If there are 4 users transmitting on the same SRS resource (i.e. 4 cyclic shifts are used), the MSE performance of increasing RPF from 2 to 4 with SRS bandwidth increasing from 8RB to 16RB seems acceptable, while RPF 4 with 8 RB plus power boosting has a non-ignorable performance loss. When wider bandwidth like 24RB is used for SRS transmission, RPF 4 has almost the same MSE performance with RPF 2 even when 4 cyclic shifts are used.

Observation 1:

For wider band SRS transmission like more than 16RB, RPF 4 has almost the same MSE performance with RPF 2, assuming 4 cyclic shifts are used.

3.2 System level evalutaion

SLS results of SRS enhancement are shown in this section, 3D-UMi with 200m ISD and antenna configuration (8,4,2,64) with one-to-one mapping are applied in the simulation. The SRS channel estimation error is modeled following [3], where RPF 4 has a lower probability of inter-cell SRS collision. Table 1 gives the system level simulation results of SRS enhancement using Full Buffer traffic model.

Table 1. System level simulation results

|  | RPF 2 | RPF 4 |
| --- | --- | --- |
| Cell average (bps/Hz) | 2.31939 | 2.36709 |
| Cell edge (bps/Hz) | 0.13356 | 0.16524 |

From Table 1, we can see that SRS with RPF 4 can improve average cell throughput by 2% and cell edge throughput by 24% compared to SRS with RPF 2.

Observation 2:

Compared to SRS with RPF 2, SRS with RPF 4 can improve cell edge throughput by 24% due to lower probability of inter-cell SRS collision.

- Conclusion

This contribution gives both link level and system level simulation results for SRS enhancements with increasing RPF from 2 to 4. Following observations are made.

Observation 1:

For wider band SRS transmission like more than 16RB, RPF 4 has almost the same MSE performance with RPF 2, assuming 4 cyclic shifts are used.

Observation 2:

Compared to SRS with RPF 2, SRS with RPF 4 can improve cell edge throughput by 24% due to lower probability of inter-cell SRS collision.

References

- RP-141644, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE,” Samsung, Nokia Networks, 3GPP RAN #65, Edinburgh, Scotland, 9 - 12 September, 2014.

- R1-144947, ” Spec enhancement for reciprocity based operation”, CMCC, 3GPP RAN1 #79, San Francisco, USA, 17th – 21st November, 2014.

- R1-145389, “Proposal on SRS estimation error modeling”, 3GPP TSG RAN WG1 #79, San Francisco, USA, 17th – 21st November, 2014.

Annex

Table 1 : Link Level Evaluation Assumptions

| Parameters | Values |
| --- | --- |
| Homogeneous scenarios | 3D-Umi ISD 200m |
| System bandwidth | 10MHz (50 PRBs) |
| Carrier Frequency | 3D-Umi ISD 200m: 2GHz |
| UE Speed | 3km/h |
| Number of antenna | 1 Tx and 1 Rx |
| Channel estimation | LMMSE |

Table 2: System Level Evaluation Assumptions

| Parameters | Values |
| --- | --- |
| Homogeneous scenarios | 3D-Umi ISD 200m |
| Polarized antenna modeling | Model-2 from 36.873 |
| Traffic model | Full Buffer |
| Wrapping method | Geographical distance based |
| Handover margin | 3dB |
| System bandwidth | 10MHz (50 PRBs) |
| UE attachment | Based on RSRP (formula) from CRS port 0 |
| Carrier Frequency | 3D-Umi ISD 200m: 2GHz |
| Downtilt | 3D-Umi ISD 200m: 100 degree |
| Network synchronization | Synchronized |
| UE Speed | 3km/h |
| UE distribution | according to 36.873 |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 90 degree, ΩUT,g = 0 degree |
| UE antenna pattern | Isotropic antenna gain pattern A’(θ’,ф’) = 1 |
| Receiver | Non-ideal channel estimation and interference modeling, LMMSE-IRC receiver, detailed guidelines according to Rel-12 [71-12] assumptions, |
| UE Rx configuration | 2 Rx x-polar (+90/0) |
| Duplex Mode | TDD |
| Feedback | PUSCH 3-0 for reciprocity based operation |
| SRS | CQI reporting triggered per 5ms |
| Transmission scheme | Feedback delay is 5 ms |
| SRS | 2Tx, 10 ms periodicity, wideband |
| Transmission scheme | TM10, single CSI process, dynamic SU/MU-MIMO with rank adaptation (no CoMP) |
| Overhead | 3 symbols for DL CCHs, 2 CRS ports and DM-RS with 12 or 24 REs per PRB |
| Scheduler | Frequency selective scheduling (multiple UEs per TTI allowed) |

Table 3: BS Antenna Configuration

| Parameter | Values |
| --- | --- |
| Number of columns (N) | 4 |
| Antenna Polarization (P) | P = 2: cross-pol (eNB: +/- 45 deg, UE: 0/90 deg) |
| Horizontal antenna element spacing dH | 0.5λ |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 8 dBi |
| Vertical antenna element spacing and Number of antenna elements with the same polarization in each column (dV , M) | (0.8 λ, 8) |