3GPP TSG-RAN WG1 Meeting #80bis	R1-152161

Belgrade, Serbia, 20th – 24th April 2015

Agenda Item:	7.2.4.1

Source:	InterDigital Communications

Title:	Evaluation results for DL-only LAA with WiFi and VoIP

Document for:	Discussion

# Introduction

In the RAN1#80 and the March 2015 RAN1 LAA AdHoc meetings, we presented coexistence evaluation results for DL only LAA with FTP NFB traffic in [2][3]. For the 3 coexistence evaluation cases, Wi-Fi/Wi-Fi, LAA/LAA and Wi-Fi/LAA, we showed results for the Indoor Scenario [5], for Y=1 and Y=4 unlicensed carriers in the 5 GHz band, for LBE Cat 3 and Cat 4 schemes and for different assumed TXOP durations. In contribution [1] we present our evaluation results for the Wi-Fi and UL+DL LAA coexistence case with FTP traffic and different traffic ratios.

In this contribution we present and discuss DL-only LAA and Wi-Fi coexistence evaluation results for VoIP for the Indoor scenario.

# Evaluation methodology and assumptions

Overview

For evaluations purposes, we assume that the LAA eNB employs LBT/CCA for channel access in the DL. DTX is used to limit the maximum possible transmission duration for an LAA transmitter. The results presented here are for the Y=1 case (a single 5 GHz channel), so no carrier selection mechanism is employed by the LAA eNB as by assumption.

We use FTP non-full buffer traffic model 3 with 0.5 MB file size. More detailed evaluation assumptions for both Wi-Fi and LTE are summarized in Appendix B of this document.

Channel Access

For DL transmissions, the LAA eNB employs LBE Cat 3 (with fixed window size). LBE Cat 3 by the LAA eNB is modeled as Option B from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. The CW size fixed and set by q at beginning of simulation. The random backoff N is selected from [1;q]. TXOPs are limited by q*13/32 ms. At first a random backoff counter N is drawn to start the LBT procedure. Initial CCA is always immediately followed by extended CCA in durations of integer multiples of 20 µs increments. For each 20 µs increment during the extended CCA duration, if the channel is deemed to be unoccupied, the backoff counter is decremented by 1. The eNB can start DL scheduling immediately after the backoff counter value reaches zero, i.e., earliest after N+1 CCA periods. When the channel is observed to be occupied during the extended CCA period by the eNB, the backoff counter is frozen and the eNB defers back to initial CCA.

Wi-Fi uses RTS/CTS frame exchange sequences in conjunction with EDCA for VoIP. LAA eNBs can decode RTS/CTS packets received from Wi-Fi APs and client STAs and will defer their channel access attempts accordingly, but the LAA eNBs can’t transmit RTS/CTS themselves when scheduling the LAA DL.

DTX

DTX for the LAA DL is modeled as a maximum allowed time duration to schedule a number of consecutive DL subframes by the LAA eNB. We use maximum TXOP durations of 4 ms. The duration of the busy signal when transmitted by the eNB prior to regular DL scheduling does not impact the allowed maximum number of schedulable DL subframes. Therefore, the LTE LAA DL will transmit up to 4 consecutive DL subframes once it obtains channel access using LBT/CCA. When a DL busy signal is transmitted by the LAA eNB, it is accounted for as overhead until the beginning of the next regularly occurring SCell subframe boundary.

# Coexistence evaluation results for DL-only LAA

For the 2 coexistence evaluation cases, Wi-Fi/Wi-Fi and Wi-Fi/LAA, our DL only results for the Indoor Scenario in presence of VoIP traffic are summarized in Appendix A. Table A.1 shows UPT for the FTP NFB traffic and the VoIP outage results for Wi-Fi/Wi-Fi and Wi-Fi-LAA at low, medium and high traffic load points. Figures 1 and 2 summarize our findings for performance of the Wi-Fi network in coexistence Step 1 and Step 2 respectively.

Figure 1: DL UPT for Wi-Fi/LAA with DL only in presence of VoIP at different loads

Figure 2: VoIP outage for Wi-Fi/LAA with DL only in the Indoor Deployment at different loads

From the evaluation results in Appendix A it can be seen that the DL throughput and VoIP outage ratios for Wi-Fi operator A in the Indoor Deployment are not adversely impacted by the presence of DL only LAA. We observe that both for FTP and VoIP traffic, LAA is a better neighbor to Wi-Fi than Wi-Fi is to Wi-Fi, especially as load increases.

VoIP outage ratios drop to virtually zero in presence of the LAA network replacing Wi-Fi in coexistence Step 2 for medium and high loads.

It can also be seen in our evaluation results that LAA performance suffers in comparison to Wi-Fi with increasing load. This stems from the fact that only Wi-Fi APs have the ability to prevent LAA nodes from transmitting, and not the other way, since the LAA nodes do not have the capability to initiate a RTS/CTS frame sequence. As a result, as load increases, the WiFi network starts to dominate the 5 GHz channel.

We note again that our evaluation results are obtained not using any particular optimization with respect to LBE Cat 3 (with fixed window size) employed by the LAA eNB to win channel access in the DL. As already observed in [2] for the case of DL only LAA for Y=1 under high load, LBE Cat 3 schemes and DTX with a fixed maximum transmission duration are not sufficient on their own to prevent starving of the Wi-Fi APs through LAA. It is necessary to implement additional functionality in the LAA eNBs to allow for adjustment of LAA channel usage ratios and claimed air time as a function of observed traffic load from Wi-Fi neighbor BSSs on the channel. This can be done for example through adjusting the lengths of individual LAA TXOPs and by adjusting the number of channel access attempts by the LAA eNB.

# Conclusion

In this contribution we present and discuss DL-only LAA and Wi-Fi coexistence evaluation results for VoIP for the Indoor scenario.

Our coexistence evaluation results show that the DL throughput and VoIP outage ratios for Wi-Fi operator A in the Indoor Deployment are not adversely impacted by the presence of DL only LAA. We observe that both for FTP and VoIP traffic, LAA is a better neighbor to Wi-Fi than Wi-Fi is to Wi-Fi, especially as load increases.

VoIP outage ratios drop to virtually zero in presence of the LAA network replacing Wi-Fi in coexistence Step 2 for medium and high loads.

# References

R1-151458 Coexistence evaluation results for Wi-Fi and UL+DL LAA; InterDigital

R1-151066 Coexistence evaluation results for LTE LAA DL-only and Wi-Fi; InterDigital

R1-150540 Coexistence evaluation results for LTE LAA DL-only and WiFi; InterDigital

R1-145186 On the need to model RTS/CTS in LAA coexistence scenarios; InterDigital

3GPP TR 36.889 v031 Feasibility Study on License-Assisted Access to Unlicensed Spectrum (R13)

# Appendix A – DL only LAA evaluation results with Wi-Fi and VoIP

Table A.1: Wi-Fi/LAA; DL only; Indoor Deployment; Y=1; TXOP=4ms, LBE Cat 3; VoIP

| LAA LBT cat. | Reported parameters |  | Low load BO range for Wi-Fi in Step 1: 10%~25% |  |  | Medium load BO range for Wi-Fi in Step 1: 35%~50% |  |  | High load BO range for Wi-Fi in Step 1: above 55% |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 | Wi-Fi in step 1 | Wi-Fi in step 2 | LAA in step 2 |
| Cat. 3 | UPT CDF [Mbps] | 5% | 11.631 | 12.611 | 5.896 | 4.439 | 4.660 | 1.308 | 1.707 | 2.869 | 0.741 |
|  |  | 50% | 53.605 | 48.265 | 55.162 | 21.008 | 17.637 | 8.263 | 8.947 | 12.556 | 6.044 |
|  |  | 95% | 119.706 | 119.197 | 142.224 | 104.410 | 54.880 | 55.588 | 34.394 | 42.408 | 52.040 |
|  |  | Mean | 60.665 | 58.267 | 66.389 | 33.370 | 29.762 | 16.804 | 12.980 | 16.627 | 14.081 |
|  | Delay CDF [s] | 5% | 0.035 | 0.035 | 0.030 | 0.038 | 0.075 | 0.072 | 0.117 | 0.096 | 0.091 |
|  |  | 50% | 0.078 | 0.089 | 0.078 | 0.209 | 0.239 | 0.548 | 0.478 | 0.337 | 0.704 |
|  |  | 95% | 0.343 | 0.316 | 0.646 | 1.033 | 0.846 | 2.913 | 2.402 | 1.1691 | 4.285 |
|  |  | Mean | 0.117 | 0.126 | 0.169 | 0.323 | 0.314 | 0.897 | 0.737 | 0.515 | 1.301 |
|  | VoIP outage (%) |  | 0.0 | 0.0 | N/A | 6.25 | 0.0 | N/A | 31.25 | 0.0 | N/A |
|  | 𝜌 |  | 0.9114 | 0.9114 | 1.00 | 0.9474 | 0.9289 | 1.00 | 0.8829 | 0.906 | 0.829 |
|  | BO |  | 21.22 | 20.56 | 25.90 | 45.64 | 47.08 | 76.36 | 72.51 | 64.861 | 82.95 |
|  | 𝜆 |  | 0.8 |  |  | 1.0 |  |  | 1.2 |  |  |
| Additional comments |  |  | Additional comments: TXOP=4ms, without licensed carrier. WiFi has RTS/CTS turned on. LAA eNB nodes can decode RTS/CTS and remain silent during subsequent Data/ACK transmission. LBE Cat.3: Option B from ETSI EN 301 893 v1.8.0, Section 4.8.3.2. CW size fixed and set by q at beginning of simulation. Random backoff N selected from [1;10]. TXOP = 4 ms. |  |  |  |  |  |  |  |  |

# Appendix B – Evaluation assumptions

| Parameter | Table B.1: Indoor scenario for LAA coexistence evaluations |
| --- | --- |
| Layout for nodes | Two operators deploy 4 small cells each in the single-floor building. The small cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random. The set of small cells for both operators is centered along the longer dimension of the building. |
| System bandwidth per carrier | 20 MHz |
| Carrier frequency | 5.0 GHz |
| Number of carriers | 1 (to be shared between two operators) |
| Total BS TX power | 18 dBm |
| Total UE TX power | 18 dBm |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH [Table B.1.2.1-1 in TR36.814] UE-to-UE: 3GPP TR 36.843 (D2D). (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability.) |
| Penetration | 0dB |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |
| Antenna pattern | 2D Omni-directional |
| Antenna Height: | 6m |
| UE antenna Height | 1.5m |
| Antenna gain + connector loss | 5dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU InH |
| Number of clusters/buildings per macro cell geographical area | N/A |
| Number of small cells per cluster | N/A |
| Number of small cells per Macro cell | N/A |
| Number of UEs | 10 UEs per unlicensed band carrier per operator |
| UE dropping per network | Uniform random drop within indoor coverage |
| Radius for small cell dropping in a cluster | N/A |
| Radius for UE dropping in a cluster | N/A |
| Minimum UE-to-UE distance (2D distance) | 3m |
| Traffic model | FTP3, 0.5 MB file size Mixed traffic model with each UE carrying only VoIP traffic or only FTP traffic in the Wi-Fi network that is not replaced by LAA. - Two UEs with VoIP traffic in addition to UEs with FTP traffic - The VoIP traffic model is based on G.729A (data rate is 24 kbps) - Packet inter-arrival time: 20 ms - Packet size: 60 bytes (payload plus IP header overhead) - Voice activity is assumed to be 100%. Statistics are independently reported in each direction - No associated control plane traffic is modelled |
| UE receiver | MMSE-IRC as baseline |
| UE noise figure | 9dB |
| UE speed | 3km/h |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. For WiFi STAs, cell selection is based on RSS (Received signal power strength) of WiFi APs. RSS threshold is -82 dBm. |
| UE Bandwidth | UE bandwidth for LAA and Wi-Fi: 20 MHz unlicensed |
| Network synchronization | For the same operator, the network is synchronized. Small cells of different operators are not synchronized. |
| Performance metrics | UPT, Delay, Buffer occupancy, Outage |

| Parameter |  | Table B.2: Additional Wi-Fi system evaluation assumptions |
| --- | --- | --- |
| MCS |  | 802.11ac MCS table without 256 QAM |
| Antenna configuration |  | 2Tx2Rx in DL, Cross-polarized Open loop |
| Channel coding |  | BCC |
| Frame aggregation |  | A-MPDU |
| MPDU size |  | Fixed 1ms (A)-MPDU transmission duration |
| Max PPDU duration |  | 4 ms |
| MAC | Coordination | EDCA AC4 |
|  | SIFS, DIFS | SIFS, DIFS |
|  | Detection | CCA-CS |
|  | RTS/CTS | Yes |
|  | Contention window | FTP Min: 15 slots, Max: 1023 slots VoIP Min: 3 slots, Max: 7 slots |
| CCA-CS |  | -82dBm |
| CCA-ED |  | -62dBm |
| RTS/CTS |  | ON |
| ACK Modeled (successful reception, resources utilized) |  | Yes |
| DL/UL Duplexing |  | DL only |
| Rate control |  | Minstrel algorithm with 100 ms reconfiguration period |
| Channel selection |  | Minimum neighbour |
| OFDM symbol length |  | 4 us |