# 3GPP TSG RAN WG1 Meeting #80-BIS	R1-152166

# Belgrade, Serbia, 20th – 24th April 2015	(Revision of R1-151477)

# Title: 	Evaluation results for LAA and LAA Coexistence with DL-only LAA

# Source: 	Alcatel-Lucent Shanghai Bell, Alcatel-Lucent

# Agenda Item:	7.2.4.1

# Document for: 	Discussion/Decision

# Introduction

In RAN1 #80, LBT schemes were classified into four categories for LAA channel access evaluations [1].

- Classify the evaluated LBT schemes according to the following categories:

  - Category 1: No LBT

  - Category 2: LBT without random back-off

  - Category 3: LBT with random back-off with fixed size of contention window

  - Category 4: LBT with random back-off with variable size of contention window

Note: Contention window is the maximum possible random back-off value

Note: Category classification does not restrict a LBT design investigation

Note: Company is encouraged to evaluate many categories as much as possible

- Illustrative examples

  - FBE procedure as defined in EN BRAN V1.8.0 belongs to category 2

  - LBE procedure with a fixed q for the contention window as defined in EN BRAN V1.8.0 belongs to category 3

  - LBE procedure Op A with a variable q for the contention window as defined in EN BRAN V1.8.0 belongs to category 4

Also, the following simulation assumptions were agreed.

- 256 QAM is mandatory for all cases

- LDPC codes should be used in the simulations for all cases for the WiFi network

In this contribution, the simulation results taking the above agreed simulation assumptions into account are given and updated for LAA-LAA coexistence on top of our previous contribution [2].

# Evaluation Assumptions

The following LAA-LAA coexistence scenarios are evaluated as agreed in [3].

- Indoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic (LAA without licensed carrier)

- Outdoor deployment for LAA and LAA coexistence case with one shared unlicensed carrier and FTP traffic (LAA without licensed carrier)

In addition, the performance of LBT category 2, category 3, and category 4 are evaluated and compared for LAA-LAA coexistence scenario. The FTP-3 traffic model is used for all simulation cases. The detailed LBT parameters and additional simulation assumptions can be found in the appendix. Note that the evaluation results for LAA and Wi-Fi coexistence are given in our companion contribution [4].

# Simulation Results

In this section, simulation results are provided for indoor and outdoor coexistence scenarios. The average user perceived throughput (UPT), latency, the ratio between the mean served cell throughput to the mean offered cell throughput (𝜌) and the mean buffer occupancy (BO) are considered as the performance metrics which are agreed in [3].

# Indoor Scenario

In Tables 1, 2 and 3, the simulation results of different LBT categories for LAA-LAA coexistence are provided in the indoor scenario without licensed carrier support for low, medium, and high traffic load.

Table 1 Simulation results for indoor scenario, low traffic load

| Reported parameters |  | Category 2 |  | Category 3 |  | Category 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 44.72 | 63.15 | 38.42 | 47.57 | 34.75 | 39.51 |
|  | 50% | 112.61 | 112.26 | 108.18 | 105.52 | 118.53 | 118.94 |
|  | 95% | 137.97 | 141.13 | 136.33 | 141.38 | 147.70 | 152.38 |
|  | Mean | 107.87 | 110.28 | 103.18 | 104.18 | 112.36 | 113.74 |
| Delay CDF [s] | 5% | 25.00 | 25.00 | 24.00 | 24.00 | 24.00 | 24.00 |
|  | 50% | 34.00 | 34.00 | 36.00 | 37.00 | 31.00 | 31.00 |
|  | 95% | 163.00 | 135.00 | 180.00 | 175.00 | 179.00 | 146.00 |
|  | Mean | 100.03 | 80.54 | 84.29 | 71.03 | 102.50 | 76.04 |
| 𝜌 |  | 0.76 | 0.81 | 0.78 | 0.82 | 0.78 | 0.82 |
| BO |  | 0.15 | 0.15 | 0.16 | 0.15 | 0.16 | 0.16 |
| 𝜆 |  | 1.0 |  | 1.0 |  | 1.0 |  |
| Additional comments |  | LBT category 2: frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms) LBT category 3: ETSI EN 301 893 v1.8.0 Option B, q=32, Maximum Occupancy Time= 10 ms LBT category 4: ETSI EN 301 893 v1.8.0 Option A |  |  |  |  |  |

Table 2 Simulation results for indoor scenario, medium traffic load

| Reported parameters |  | Category 2 |  | Category 3 |  | Category 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 40.62 | 30.75 | 5.51 | 7.09 | 43.20 | 56.88 |
|  | 50% | 96.03 | 94.32 | 80.48 | 81.79 | 101.66 | 100.45 |
|  | 95% | 123.52 | 119.54 | 116.78 | 117.37 | 131.91 | 132.71 |
|  | Mean | 91.38 | 89.18 | 74.04 | 73.76 | 98.43 | 98.94 |
| Delay CDF [s] | 5% | 25.00 | 25.00 | 24.00 | 24.00 | 24.00 | 24.00 |
|  | 50% | 42.00 | 44.00 | 58.00 | 56.00 | 40.00 | 40.00 |
|  | 95% | 437.00 | 489.00 | 936.00 | 588.00 | 271.00 | 211.00 |
|  | Mean | 123.07 | 121.70 | 208.98 | 173.98 | 106.98 | 81.61 |
| 𝜌 |  | 0.69 | 0.76 | 0.70 | 0.72 | 0.73 | 0.77 |
| BO |  | 0.25 | 0.26 | 0.33 | 0.31 | 0.21 | 0.21 |
| 𝜆 |  | 1.2 |  | 1.2 |  | 1.2 |  |
| Additional comments |  | LBT category 2: frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms) LBT category 3: ETSI EN 301 893 v1.8.0 Option B, q=32, Maximum Occupancy Time= 10 ms LBT category 4: ETSI EN 301 893 v1.8.0 Option A |  |  |  |  |  |

Table 3 Simulation results for indoor scenario, high traffic load

| Reported parameters |  | Category 2 |  | Category 3 |  | Category 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 3.01 | 6.32 | 0.79 | 1.39 | 4.26 | 14.77 |
|  | 50% | 59.21 | 60.77 | 19.43 | 21.82 | 65.68 | 66.62 |
|  | 95% | 101.47 | 109.80 | 77.70 | 87.00 | 109.59 | 109.00 |
|  | Mean | 56.74 | 59.25 | 28.10 | 31.64 | 62.30 | 65.57 |
| Delay CDF [s] | 5% | 25.00 | 25.00 | 26.00 | 26.00 | 24.00 | 24.00 |
|  | 50% | 83.00 | 81.00 | 221.00 | 195.00 | 77.00 | 69.00 |
|  | 95% | 2397.00 | 1929.00 | 3131.00 | 2715.00 | 1545.00 | 1201.00 |
|  | Mean | 443.30 | 381.36 | 684.94 | 603.58 | 299.63 | 281.57 |
| 𝜌 |  | 0.60 | 0.58 | 0.52 | 0.53 | 0.68 | 0.64 |
| BO |  | 0.52 | 0.53 | 0.72 | 0.69 | 0.45 | 0.43 |
| 𝜆 |  | 1.5 |  | 1.5 |  | 1.5 |  |
| Additional comments |  | LBT category 2: frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms) LBT category 3: ETSI EN 301 893 v1.8.0 Option B, q=32, Maximum Occupancy Time= 10 ms LBT category 4: ETSI EN 301 893 v1.8.0 Option A |  |  |  |  |  |

Based on the simulation results from the indoor deployment, it is observed that all LBT schemes could achieve the co-existence fairness between two LAA operators. Note that channel access unfairness issue between asynchronous operators for FBE scheme cannot be observed from these simulation results. In fact it cannot be reflected by the metrics provided because the metric is the average UPT for a UE, and due to the random packet generation, one UE does not always have access advantage for all the files.

With respect to the performance comparison between LBT category 2 and category 3, it is observed that LBT category 2 performs better for all traffic load cases. The performance gain is mainly due to the intra-operator frequency reuse one and slightly quicker channel access with shorter (e)CCA time. Hence, it is important to study further how to support intra-operator frequency reuse one with LBE based LBT.

In the indoor deployment, no matter which traffic load is, LBT category 4 outperforms LBT category 3 due to adaptive back-off mechanism based on sensing results. In addition, the initial contention window size in LBT category 4 is 16, LBT category 4 could experience quicker channel access compared with category 3 in the lower traffic load case.

Observation 1: Co-existence fairness of two LTE operators can be achieved with both LBE-based and FBE-based LBT algorithms.

Observation 2: LBT category 2 outperforms LBT category 3 mainly due to the intra-operator frequency reuse factor one and slightly quicker channel access with shorter (e)CCA time.

Observation 3: LBT category 4 outperforms other LBT mechanism in all traffic load cases for the indoor scenario.

# Outdoor Scenario

In Tables 4, 5 and 6, the simulation results of different LBT categories for LAA-LAA coexistence are provided in the outdoor scenario without licensed carrier support for low, medium, and high traffic load.

Table 4 Simulation results for outdoor scenario, low traffic load

| Reported parameters |  | Category 2 |  | Category 3 |  | Category 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 50.33 | 41.61 | 31.33 | 27.22 | 55.42 | 55.56 |
|  | 50% | 90.24 | 89.33 | 69.50 | 70.22 | 95.30 | 93.51 |
|  | 95% | 119.07 | 122.63 | 108.47 | 108.09 | 128.25 | 128.64 |
|  | Mean | 88.72 | 88.03 | 69.77 | 70.30 | 95.04 | 93.65 |
| Delay CDF [s] | 5% | 25.00 | 25.00 | 24.00 | 24.00 | 24.00 | 24.00 |
|  | 50% | 45.00 | 44.00 | 61.00 | 59.00 | 41.00 | 41.00 |
|  | 95% | 188.00 | 234.00 | 345.00 | 363.00 | 166.00 | 162.00 |
|  | Mean | 69.00 | 76.61 | 111.38 | 121.89 | 61.41 | 61.72 |
| 𝜌 |  | 0.71 | 0.71 | 0.70 | 0.70 | 0.73 | 0.72 |
| BO |  | 0.15 | 0.16 | 0.23 | 0.22 | 0.14 | 0.14 |
| 𝜆 |  | 1.0 |  | 1.0 |  | 1.0 |  |
| Additional comments |  | LBT category 2: frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms) with reuse 1 LBT category 3: ETSI EN 301 893 v1.8.0 Option B, q=32, Maximum Occupancy Time= 10 ms, LBT category 4: ETSI EN 301 893 v1.8.0 Option A |  |  |  |  |  |

Table 5 Simulation results for outdoor scenario, medium traffic load

| Reported parameters |  | Category 2 |  | Category 3 |  | Category 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 13.94 | 11.75 | 1.73 | 1.42 | 16.23 | 12.37 |
|  | 50% | 62.56 | 63.51 | 30.28 | 28.01 | 58.09 | 64.51 |
|  | 95% | 100.65 | 106.96 | 82.29 | 74.88 | 104.78 | 107.73 |
|  | Mean | 62.34 | 62.46 | 33.68 | 32.67 | 59.85 | 62.14 |
| Delay CDF [s] | 5% | 25.00 | 25.00 | 32.00 | 33.00 | 24.00 | 24.00 |
|  | 50% | 69.00 | 71.00 | 151.00 | 148.00 | 78.00 | 70.00 |
|  | 95% | 1288.00 | 1033.00 | 1442.00 | 1391.00 | 739.00 | 706.00 |
|  | Mean | 273.58 | 240.57 | 377.32 | 368.57 | 200.56 | 203.62 |
| 𝜌 |  | 0.66 | 0.66 | 0.59 | 0.60 | 0.67 | 0.67 |
| BO |  | 0.36 | 0.36 | 0.53 | 0.53 | 0.35 | 0.35 |
| 𝜆 |  | 1.2 |  | 1.2 |  | 1.2 |  |
| Additional comments |  | LBT category 2: frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms) with reuse 1 LBT category 3: ETSI EN 301 893 v1.8.0 Option B, q=32, Maximum Occupancy Time= 10 ms, LBT category 4: ETSI EN 301 893 v1.8.0 Option A |  |  |  |  |  |

Table 6 Simulation results for outdoor scenario, high traffic load

| Reported parameters |  | Category 2 |  | Category 3 |  | Category 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 | LAA opt. 1 | LAA opt. 2 |
| UPT CDF [Mbps] | 5% | 6.27 | 4.78 | 0.70 | 0.75 | 4.83 | 5.45 |
|  | 50% | 37.03 | 32.75 | 11.86 | 14.32 | 35.54 | 38.26 |
|  | 95% | 84.49 | 81.33 | 48.28 | 54.14 | 75.43 | 81.85 |
|  | Mean | 38.28 | 36.10 | 17.46 | 19.47 | 37.49 | 40.76 |
| Delay CDF [s] | 5% | 29.00 | 29.00 | 46.00 | 44.00 | 30.00 | 27.00 |
|  | 50% | 159.00 | 184.00 | 269.00 | 249.00 | 155.00 | 137.00 |
|  | 95% | 2667.00 | 2316.00 | 3006.00 | 2359.00 | 1328.00 | 1394.00 |
|  | Mean | 553.57 | 569.25 | 677.82 | 573.77 | 372.72 | 354.64 |
| 𝜌 |  | 0.56 | 0.57 | 0.44 | 0.53 | 0.60 | 0.62 |
| BO |  | 0.64 | 0.67 | 0.76 | 0.73 | 0.57 | 0.56 |
| 𝜆 |  | 1.5 |  | 1.5 |  | 1.5 |  |
| Additional comments |  | LBT category 2: frame period: 10ms(Maximum Occupancy Time= 9.5ms, idle time = 0.5ms) with reuse 1 LBT category 3: ETSI EN 301 893 v1.8.0 Option B, q=32, Maximum Occupancy Time= 10 ms, LBT category 4: ETSI EN 301 893 v1.8.0 Option A |  |  |  |  |  |

Comparing the indoor and outdoor scenarios, it can be observed that the indoor scenario has better performance (higher UPT and lower latency) due to good interference isolation from other neighbor clusters. From the simulation results in the outdoor case, it can be observed that LBT category 4 outperforms category 3in all traffic load cases. .

Observation 4: LBT category 4 can provide better coexistence performance than LBT category 3 in all traffic load cases for the outdoor scenario.

# Conclusion

In this contribution, simulation results of LAA-LAA coexistence performance are provided for DL only LAA system. Based on the simulation results, the following conclusions are made:

Observation 1: Co-existence fairness of multiple LTE operators can be achieved for both FBE-based and LBE-based LBT schemes in both indoor and outdoor deployment.

Observation 2: LBT category 2 outperforms LBT category 3 mainly due to the intra-operator frequency reuse factor one and slightly quicker channel access with shorter (e)CCA time.

Observation 3: LBT category 4 outperforms other LBT mechanism in all traffic load cases for the indoor scenario.

Observation 4: LBT category 4 can provide better coexistence performance than LBT category 3 in all traffic load cases for the outdoor scenario.

# References

- 3GPP RAN1#80 Chairman’s note, Feb. 2015.

- 3GPP R1-151070, “Evaluation results for LAA and LAA Coexistence with DL-only LAA”, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, March 2015.

- 3GPP R1-150890, “Table templates for DL only LAA coexistence evaluation result”, Ericsson, Feb. 2015.

- 3GPP R1-151069, “Evaluation results for LAA and Wi-Fi Coexistence with DL-only LAA”, Alcatel-Lucent Shanghai Bell, Alcatel-Lucent, Mar. 2015.

# Appendix: Simulation Assumptions

Table 7 LTE additional simulation parameters

| Parameter | value |
| --- | --- |
| Outdoor scenario layout | Based on SCE# 2a + unlicensed band X=4, Y=1; 10 UEs per operator per carrier; 10 m for min. distance between small cells of different operators |
| Indoor scenario layout | Based on SCE#3 + unlicensed band X=4, Y=1; 10 UEs per operator per carrier; 3 m minimal distance between small cells of different operators |
| Traffic model | FTP Model 3 file size: 0.5 Mbytes |
| Network synchronization | For the same operator, the network is ideally synchronized Small cells of different operators are not synchronized |
| Total BS TX power | 18 dBm |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized; adaptive stream |
| CCA-ED | -73 dBm/MHz+23-PH / (1 MHz) |
| Link adaptation | Realistic |
| Transmission schemes | TM10, QPSK/16QAM/64QAM/ 256QAM |
| Scheduler | Proportional fair |
| UE receiver | MMSE-IRC |
| Scheduling delay for LAA | 1 ms (Channel ON Status) |
| CQI/RI/PMI Feedback for LAA | Sub-band feedback with 1 ms feedback period and 1ms feedback delay |
| MAC HARQ | Number of maximum ReTx: 3; minimal interval of ReTx: 8 ms (Channel ON Status) |
| Outer loop link adaptation | Enabled |