# 3GPP TSG RAN WG1 Meeting #80bis 	R1-152167

# Belgrade, Serbia, 20th – 24th April 2015	(Revision of R1-151483)

# Title: 	DCI Transmission for the DL Partial Subframe in LAA

# Source: 	Alcatel-Lucent, Alcatel-Lucent Shanghai Bell

# Agenda Item:	7.2.4.2

# Document for: 	Discussion/Decision

# Introduction

For LAA, the following has been agreed on PDSCH transmission in RAN1 LAA ad hoc meeting:

Agreements:

- LAA supports transmitting PDSCH when not all OFDM symbols are available for transmission in a subframe according to LBT, also support delivering necessary control information for the PDSCH

  - FFS starting/ending OFDM symbols of the PDSCH

In our companion contribution [1], it is proposed that both self-scheduling and cross-carrier scheduling should be supported. In this contribution, the discussion focuses on how DCI is transmitted for the partial subframe as agreed in the agreements above.

# Discussion

Partial subframe transmission at the beginning of a transmission burst is more of an issue for LBE-based approach. One important issue to address for the partial subframe transmission is the delivery of DCI for the partial subframe transmission to the UE. The potential solutions include a combination of a few different dimensions:

- Self-scheduling or cross-carrier scheduling

- PDCCH or EPDCCH

- DCI transmitted in the current subframe or in the next subframe

- TB separate from or combined with the TB in the next full subframe

Whether the TB in the partial subframe is combined with the TB in the next full subframe would only affect the contents of the DCI, not how the DCI is transmitted. In terms of when/where DCI is transmitted, there are the following combinations:

- Self scheduling with PDCCH(-like) in the current subframe

In this case, the PDCCH is not exactly the same as in Rel-12. PDCCH is currently being transmitted in the first 3 OFDM symbols, which can no longer work for the initial partial subframe transmission on LAA. But LAA can reuse the idea as PDCCH, but start the transmission only after it grabs the channel. The number of symbols can be fixed, and it can be less than 3 to save overhead, which could still work well if the number of scheduled UEs per subframe is small.

- Self scheduling with EPDCCH in the current subframe

Similar as option 1, EPDCCH would start the transmission after the eNB gets the channel access. Since the number of remaining symbols in the subframe varies, the reliable transmission of EPDCCH could be an issue if the number of remaining symbols is small.

- Self scheduling with (E)PDCCH in the next subframe

When DCI is transmitted in the next subframe, it requires the UE to buffer all the data symbols and start dedomulation/decoding after DCI is received in the next subframe. This requires more UE buffering. It could also require higher UE processing power because in the next subframe the UE may need to decode for two packets at the same time, if the UE is scheduled with a separate TB in the next subframe.

- Cross-carrier scheduling with (E)PDCCH from a licensed carrier

When cross-carrier scheduling is used, PDCCH/EPDCCH would need to be carried in the next subframe on the licensed carrier because it has missed the chance of transmission in the current subframe. This has similar implication on UE complexity as option 3.

Comparing these different approaches, option 3 and 4 requires less change in the DL control channel, but it can require more UE buffering and processing power. Option 1 and 2 on the other hand involves a new way of decoding (E)PDCCH. These options should be discussed further.

The above options assume that the DCI is transmitted only after the eNB successfully performs the (E)CCA. An alternative would be that the eNB transmit DCI on a licensed carrier before the success of (E)CCA. Instead, the DCI is transmitted as an indication of the intention that the eNB wants to transmit. This assumes that the UE does not rely on the DCI to know the starting position of the PDSCH transmission. It has been discussed what functions an initial signal needs to support. If the initial signal allows the detection of the data transmission, the UE can use the DCI that was received previously to decode the packet in the partial subframe once it detects the data transmission. By doing so, it avoids the additional UE buffering that is required when the DCI is delivered in the next full subframe. It also avoids the new (E)PDCCH design in the partial subframe that is needed when the DCI is delivered in the partial subframe itself.

Because it can be uncertain how long the (E)CCA takes, the eNB and the UE would not know in which subframe the eNB may be able to get the channel. To maximize the efficiency and avoid the waste of DCI transmission on the licensed carrier, this pre-transmitted DCI can have a valid duration associated with it.

Figure 1 provides an example of using such an approach.

Figure 1. An example of sending DCI on a licensed carrier in advance for the partial subframe on LAA

On whether the partial subframe should have a combined TB with the next full subframe, from the efficiency point of view, it depends on how many symbols remains in the subframe. If there are very few symbols, it may not worth a separate TB considering the (E)PDCCH overhead. But if there are a considerable number of symbols available, a separate TB can be transmitted.

# Conclusion

In this contribution, we have discussed the pros and cons of different options to transmit DCI for the initial partial subframe. Instead of transmitting DCI only after successful (E)CCA check, an alternative of sending DCI in advance was also discussed.

# References

- R1-151484, “Control Signaling for LAA,” Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, RAN1#80bis, April 2015.