3GPP TSG RAN WG1 Meeting #82                                                                   R1-154662

Beijing, China, 24th - 28th August 2015

Source:	NTT DOCOMO

Title:	Feedback design for beamformed CSI-RS based 3D MIMO

Agenda Item:	7.2.5.3.2

Document for: 	Discussion and Decision

# Introduction

- Specify enhancements on reference signal in the following areas [RAN1]

  - Non-precoded CSI-RS, extending the existing numbers {1,2,4,8} of CSI-RS antenna ports for support of 12 and 16 CSI-RS ports, using full-port mapping

  - Beamformed CSI-RS

  - SRS capacity improvement

    - Support for SRS transmission with 4TX antennas as a second priority

  - Support of additional ports for DMRS targeting higher dimensional MU-MIMO

    - The maximum number of DMRS ports that a UE may be able to receive is kept as 8

- Specify enhancements on CSI reporting in the following areas [RAN1]

  - For non-precoded CSI-RS, codebook for 2D antenna arrays for support of {8,12,16} CSI-RS ports and associated necessary channel state information.

    - If there is not significant gain shown for new codebook for 8 CSI-RS ports, the current codebook for 8 CSI-RS ports is retained.

  - Necessary channel state information for beamformed CSI-RS

  - Extension of Rel-12 CSI reporting mechanism for both periodic and aperiodic CSI reports

- Specify higher layer support of enhancements listed above [RAN2]

- Specify the necessary UE (if any) core requirements [RAN4]

At the RAN1 #81 meeting, the SI on elevation beamforming and full-dimension (FD) MIMO for LTE was successfully completed [1]. Several specification based enhancement are identified for enabling high system performance with 3D MIMO. At the RAN #68 meeting, the WI, elevation beamforming/full-dimension (FD) MIMO for LTE, has been approved with the objective shown as follows [2].

One of the fundamental topics is to design CSI feedback for beamformed CSI-RS based 3D MIMO. In this contribution, we investigate CSI feedback for beamformed CSI-RS based 3D MIMO with system-level simulation results.

# Beamformed CSI-RS Based Enhancement

Following steps and Fig. 1 show beamformed CSI-RS based MIMO transmission scheme evaluated in this paper. We applied stepwise CSI acquisition scheme using cell-specific and UE-specific BF CSI-RSs as detailed procedure shown in our companion contribution [3]. For beamformed CSI-RS based transmission scheme with a number of TXRUs, it is very important to reduce CSI-RS overhead, which increases according to the number of TXRUs and/or beamformed CSI-RS. For the reason, in this paper, cell-specific BF CSI-RSs are transmitted with low duty cycle. Additionally, UE-specific BF CSI-RS is transmitted only towards selected beam direction.

- Step 1: eNB transmits multiple BF CSI-RSs to cover entire cell (cell-specific BF CSI-RSs)

- Step 2: UE selects a preferred beam and feeds back as a beam index (BI)

- Step 3: eNB transmits CSI-RS beamformed by the selected beam (UE-specific BF CSI-RS)

- Step 4: UE feeds back CSI, i.e., RI, PMI and CQI, which is derived based on the UE-specific BF CSI-RS

Figure 1: Beamformed CSI-RS based scheme

# Performance Evaluation

In this evaluation, we clarify optimal number of cell-specific BF CSI-RS for different eNB antenna configuration, e.g., number of antenna element and TXRU virtualization. Major evaluation assumptions are summarized in Table A. We assume 3D-UMi environment with the eNB antenna configuration (M, N, P, Q) of (8, 4, 2, 16), (8, 4, 2, 32) and (16, 4, 2, 64). The subarray partitioning is applied to multiple adjacent vertical antenna elements. The BI and PMI represent vertical and horizontal precoding, respectively, i.e., multiple beamformed CSI-RSs are transmitted with different vertical tilting angles. The number of cell-specific beamformed CSI-RS is parameterized with the oversampling factor from 1 to 4. Here, the oversampling factor is defined as NCSI-RS/NV-TXRU, where NCSI-RS and NV-TXRU represent the number of cell-specific BF CSI-RS and the number of TXRUs in a single column, respectively. Finally, Table 1 shows evaluation results with the relative gain compared to baseline schemes. Note that the best performance among different oversampling factor is colored in red.

Table 1: Impact of oversampling factor (3D-UMi – 2 GHz)

From the results, for all antenna configurations and traffic conditions, the oversampling factor of 2 achieves significant gain compared to that for the oversampling factor of 1. For instance, the mean throughput gain of oversampling factor of 2 compared to that of 1 is in the range of 7.4 – 37.0 % for high traffic cases. On the other hand, the mean throughput gain of oversampling factor of 4 compared to that of 2 is in the range of -3.6 – 1.1 % for high traffic cases. This is due the trade-off between granurality of cell-specific BF CSI-RS and its overhead. Hence, considering the possible eNB antenna configurations in this WI, 3 bits can be enough for the number of feedback bits on beam index for most of the prioritized antenna configurations.

Observation: Optimal oversampling factor for vertical cell-specific BF CSI-RS is 2.

Proposal: The number of feedback bits for beam index should be 3.

# Summary

In this contribution, we investigated CSI feedback for beamformed CSI-RS based 3D MIMO with system-level simulation results. Based on the discussion, we made the following observation and proposal.

Observation: Optimal oversampling factor for vertical cell-specific BF CSI-RS is 2.

Proposal: The number of feedback bits for beam index should be 3.

# References

- 3GPP, TR 36.897 (V13.0.0), “Study on Elevation Beamformeing/Full-Dimension (FD) MIMO for LTE,” July 2015.

- 3GPP, RP-151085, “Elevation Beamforming/Full-Dimension (FD) MIMO for LTE,” June 2015.

- 3GPP, R1-154661, NTT DOCOMO, “Views on Beamformed CSI-RS Based Enhancement,” Aug. 2015.

# Appendix

Table A: Evaluation assumptions

| Parameter | Values |
| --- | --- |
| Scenario / channel model | 3D-UMi (ISD: 200 m) |
| Carrier frequency | 2 GHz |
| System bandwidth | 10 MHz (50 RBs) |
| eNB antenna configurations | (M, N, P, Q) = (8, 4, 2, 16), (8, 4, 2, 32), (16, 4, 2, 64), (dH, dV) = (0.5 , 0.8 ), etilt = 100 deg. |
| Total BS Tx power | 41 dBm |
| UE antenna configurations | 2 X-pol (0/90 deg.) |
| UE speed | 3 km/h |
| Indoor UE ratio | 80 % |
| MIMO scheme | SU/MU-MIMO dynamic switching |
| UE receiver | Non-ideal channel estimation and interference modeling, detailed guidelines according to Rel. 12 [71-12] assumptions |
|  | LMMSE-IRC receiver, detailed guidelines according to Rel. 12 [71-12] assumptions |
| CSI feedback scheme | Subband PMI and CQI |
| CSI-RS transmission interval / CSI feedback interval | 5 ms for RI, PMI and CQI, 200 ms for beam selection |
| Traffic model | FTP Model 1 with packet size 0.5 Mbytes (low: ~20 % RU, medium: ~50 % RU, high: ~70 % RU) |
| Scheduler | Proportional fairness |
| Control delay | 6 ms |
| HARQ | Chase combining with 8 ms RTD |