3GPP TSG RAN WG1 Meeting #82	R1-154768

Beijing, China, 24th - 28th August 2015

Agenda Item:    7.2.4.1

Source:              ZTE

Title:                  Contention window size adaptation for DL LBT in LAA

Document for:  Discussion and Decision

# Introduction

The SI on Licensed-Assisted Access (LAA) to Unlicensed Spectrum [1] was completed and the outcome was captured in [2]. Based on this study, in RAN#68 meeting, a WI on LAA was approved [3]. The goal of this work item is to specify LTE enhancements for a single global solution framework for licensed-assisted access to unlicensed spectrum which enables operation of LTE in the 5GHz unlicensed spectrum for low power secondary cells based on regional regulatory power limits using carrier aggregation.

The TR recommends a Category 4 LBT with random back-off with variable size of contention window to be the baseline at least for LAA DL transmission bursts containing PDSCH where the detailed description of Category 4 LBT scheme is as the following [2]:

Category 4 LBT scheme description

The LBT scheme defined here is based on the procedure in Option B in clause 4.8.3.2 of [4] except for the following modifications to form a category 4 LBT scheme that ensure fairness with Wi-Fi:

- The size of the LAA contention window is variable via dynamic variable  backoff or semi-static backoff between X and Y ECCA slots. Further details are provided below.

-	One candidate for variation of the contention window is exponential backoff. It should be noted that most of evaluations are based on exponential backoff.

-	The value of X and Y is a configurable parameter

-	For PDSCH, the following two approaches to adjust the contention window size should be considered and it should be noted that a combination of the options listed below is not precluded.

-	Based on feedback/report of UE(s) (e.g. HARQ ACK/NACK)

-	Based on eNB’s assessment (e.g. sensing based adjustment)

- Consider minimum ECCA slot size smaller than 20 µs.

- The initial CCA (ICCA) can be configurable to be comparable to the defer periods of Wi-Fi (e.g., DIFS or AIFS)

- When ECCA countdown is interrupted, a defer period (not necessarily the same as ICCA) is applied after channel becomes idle. No ECCA countdown is performed during the defer period.

- The defer period is configurable. It can be configured to be comparable to defer periods of Wi-Fi (e.g. DIFS or AIFS).

- Initial CCA is performed to transmit a DL transmission burst when the eNB has not transmitted any signal/channel although the random backoff counter reached zero in the backoff procedure.

In the above procedure, defer period is defined as the minimum time that a node has to wait after the channel becomes idle before transmission, i.e., a node can transmit if the channel is sensed to be idle for a time period not less than the defer period.

In this contribution, we share our views on contention window size adaptation for DL LBT.

# Contention window size adaption

For LBT Cat 4, an LAA site can adjust the size of contention window (CW) according to the load of an unlicensed carrier. It will reduce contention and resources waste caused by too many number of unsuccessful CCA.

In the following, we discuss two approaches to adjust CW size in detail.

Alt. 1 Based on feedback from UE

In this method, the CW size is adjusted according to HARQ-ACK fed back from UE. For example, if the number of UEs that feedback NACK exceeded a predefined threshold, then the LAA site will increase the CW size for the next LBT (e.g., double the CW size). Since the LAA eNB cannot distinguish whether the NACK/DTX is caused by channel collision or channel fading, hence, this method may waste some channel opportunities if the CW size is increased assuming NACK is caused by channel collision. Several aspects of UE feedback and criteria for CW size adjustment should be considered.

In addition to HARQ-ACK fed back from UE, the interference observed on the unlicensed carrier should be fed back by a UE. The CW size is adjusted according HARQ-ACK and interference measurement fed back from a UE. For example, if a NACK was received and the interference was lower than a threshold, then this NACK could likely be caused by channel fading instead of channel collision. If a NACK was received and the interference was high as well, then this NACK could be caused by channel collision with high probability. If the LAA eNB can effectively distinguish channel fading and channel collision, the adjustment of CW size could be more matched to the channel load.

Another aspect of UE feedback is the time interval during which the UE measured to get the feedback. For example, the feedback from a UE could be based on the most recent measurement interval (e.g., a pre-defined or configured measurement interval on the most recent transmission burst from its LAA eNB). In this case, such feedback would only reflect a short time channel statistics. In another example, the feedback from a UE could be based on a relative long term measurement interval (e.g., multiple transmission bursts from its LAA eNB or a continuous time interval during which its LAA eNB may not occupy the unlicensed carrier). For instance, in a scenario that only operator deployed LAA nodes exist, this longer term statistics may be more accurate to reflect the relatively stable number of LAA nodes and traffic load.

Different from WiFi, there could be multiple UEs scheduled and configured for feedback during a single transmission burst. The number of UEs that feedback information for CW adjustment should be taken into consideration. For example, the LAA node may increase the CW size only if at least M NACK were fed back from UEs.

Alt. 2 Based on LAA eNB measurement

In this approach, the CW size is adjusted only according to measurement of LAA node itself. The basic idea is to characterize the channel load using LAA measurement and hence to adjust CW size.

An example of CW adjustment based on CCA check at LAA eNB is described as the following. Here P is a counter of how many times the unlicensed carrier changes from idle to busy during each ECCA check during the Kth LBT attempt.  The LAA eNB adjusts the CW size for the (K+1)th LBT attempt according to P or P/N by comparing it with a predefined threshold.

In summary, to ensure fairness among nodes for coexistence on the unlicensed carrier, each node should apply the same method of adjusting CW. Hence, the method of adjusting CW should be standardized. For example, criteria for CW adjustment, triggering condition for adjustment should be standardized.

Proposal 1: the contention window size adaptation for DL LBT should be standardized.

Three methods to adjust CW size are evaluated and the results are presented in the appendix. It is observed that the performance of CW size adjustment based on UE HARQ-ACK is the worst among three evaluated methods. The method where CW size adjustment based on UE interference feedback and LAA eNB measurement provides the best performance.

Proposal 2: the contention window size adaptation for DL LBT should be based on UE feedback and LAA eNB measurement.

# Conclusion

In this contribution, we have discussed several aspects of contention window size adaptation for DL LBT. We have the following proposals:

Proposal 1: the contention window size adaptation for DL LBT should be standardized.

Proposal 2: the contention window size adaptation for DL LBT should be based on UE feedback and LAA eNB measurement.

# References

[1] RP-141817, Study on Licensed-Assisted Access to Unlicensed Spectrum

[2] TR 36.889,  Feasibility Study on Licensed-Assisted Access to Unlicensed Spectrum V13.0.0

[3] RP-151045, New Work Item on Licensed-Assisted Access to Unlicensed Spectrum, Ericsson, Huawei, Qualcomm, Alcatel-Lucent

[4] ETSI EN 301 893, Harmonized European Standard, “Broadband Radio Access Networks (BRAN); 5 GHz high performance RLAN”

# Appendix

|  | Licensed cell | Unlicensed cell |
| --- | --- | --- |
| Layout for nodes | For DL-only coexistence evaluations: Two operators deploy 4 small cells each in the single-floor building. The small cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random. The set of small cells for both operators is centered along the longer dimension of the building. 120 m 50 m |  |
| System bandwidth per carrier | 10MHz | 20MHz |
| Carrier frequency | 3.5GHz | 5.0GHz |
| Number of carriers | 2 (one for each operator) | 1 |
| Total BS TX power | 24dBm (Ptotal per carrier) | 24 dBm |
| Total UE TX power | Total UE TX power: 23dBm across aggregated cells Max total UE TX power per cell in licensed spectrum: 23dBm Max total UE TX power across aggregated cells in unlicensed spectrum: 18 dBm |  |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH [referring to Table B.1.2.1-1 in TR36.814] Indoor UE-to-indoor UE: 3GPP TR 36.843 (D2D). (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability and break point distance) |  |
| Penetration | 0dB |  |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |  |
| Antenna pattern | 2D Omni-directional is baseline; directional antenna is not precluded |  |
| Antenna Height: | 6m |  |
| UE antenna Height | 1.5m |  |
| Antenna gain + connector loss | 5dBi |  |
| Antenna gain of UE | 0 dBi |  |
| Fast fading channel between eNB and UE | ITU InH |  |
| Number of clusters/buildings per macro cell geographical area | N/A |  |
| Number of small cells per cluster | N/A |  |
| Number of small cells per Macro cell | N/A |  |
| Number of UEs | 20 UEs per unlicensed band carrier per operator |  |
| UE dropping per network | All UEs should be randomly dropped and be within coverage of the small cell in the unlicensed band. |  |
| Radius for small cell dropping in a cluster | N/A |  |
| Radius for UE dropping in a cluster | N/A |  |
| Minimum distance (2D distance) | 3m |  |
| Traffic model | FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process and the transmission time of a packet is counted from the time instance it arrives in the queue. FTP model file size: 0.5 Mbytes. |  |
| UE receiver | MMSE-IRC as baseline |  |
| UE noise figure | 9dB |  |
| UE speed | 3km/h |  |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. For WiFi STAs, cell selection is based on RSS (Received signal power strength) of WiFi APs. RSS threshold is -82 dBm. |  |
| UE Bandwidth | UE bandwidth for LAA: 10 MHz licensed + 20 MHz unlicensed UE bandwidth for Wi-Fi: 20 MHz unlicensed |  |
| Network synchronization | For the same operator, the network is synchronized. Asynchronous between different operators. |  |
| Performance metrics | - User perceived throughput (UPT) - File throughput is calculated per file - Unfinished files should be incorporated in the UPT calculation. - The number of served bits (possibly zero) of an unfinished file by the end of the simulation is divided by the served time (simulation end time – file arrival time). - User throughput is the average of all its file throughputs - Latency (From packet arrival in devices (eNB, AP, UE, STA) MAC buffer to successful transmission (including retransmission) of packet) - Latency CDF |  |

- Simulation Results

Table A.1 CW size adjust based on UE HARQ-NACK

| Reported parameters |  | CW size adjust based on UE HARQ-NACK |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Op1 | Op2 | Op1 | Op2 | Op1 | Op2 |
| UPT CDF [Mbps] | 5% | 52.083 | 54.829 | 14.635 | 26.999 | 3.414 | 4.541 |
|  | 50% | 71.656 | 75.532 | 33.374 | 34.751 | 15.783 | 12.313 |
|  | 95% | 98.782 | 90.107 | 77.404 | 66.685 | 52.682 | 31.930 |
|  | Mean | 76.328 | 76.184 | 40.397 | 39.715 | 20.686 | 15.226 |
| Delay CDF [s] | 5% | 0.004 | 0.004 | 0.004 | 0.004 | 0.005 | 0.007 |
|  | 50% | 0.007 | 0.006 | 0.024 | 0.025 | 0.056 | 0.090 |
|  | 95% | 0.073 | 0.066 | 0.228 | 0.419 | 0.409 | 0.818 |
|  | Mean | 0.019 | 0.017 | 0.054 | 0.086 | 0.109 | 0.202 |
| 𝜌 | % | 99.922 | 100.000 | 99.946 | 99.377 | 99.677 | 86.879 |
| BO | % | 12.228 | 11.023 | 35.479 | 37.441 | 60.711 | 69.651 |
| 𝜆 |  | 1.6 |  | 2.4 |  | 2.8 |  |

Table A.2 CW size adjust based on UE HARQ-NACK and interference feedback

| Reported parameters |  | CW size adjust based on UE HARQ-NACK and interference feedback |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Op1 | Op2 | Op1 | Op2 | Op1 | Op2 |
| UPT CDF [Mbps] | 5% | 60.539 | 56.072 | 22.485 | 27.077 | 2.240 | 2.940 |
|  | 50% | 70.174 | 77.639 | 35.871 | 36.209 | 14.481 | 14.393 |
|  | 95% | 99.111 | 89.490 | 75.737 | 66.899 | 56.089 | 34.773 |
|  | Mean | 77.053 | 76.959 | 42.450 | 41.154 | 21.861 | 16.005 |
| Delay CDF [s] | 5% | 0.004 | 0.004 | 0.004 | 0.004 | 0.005 | 0.006 |
|  | 50% | 0.006 | 0.006 | 0.023 | 0.021 | 0.046 | 0.084 |
|  | 95% | 0.062 | 0.062 | 0.200 | 0.383 | 0.344 | 0.751 |
|  | Mean | 0.017 | 0.016 | 0.051 | 0.080 | 0.096 | 0.187 |
| 𝜌 | % | 99.843 | 100.00 | 100.000 | 99.896 | 99.493 | 88.741 |
| BO | % | 11.678 | 10.785 | 34.243 | 37.894 | 57.122 | 68.250 |
| 𝜆 |  | 1.6 |  | 2.4 |  | 2.8 |  |

Table A.3 CW size adjust based on UE interference feedback and LAA eNB CCA check

| Reported parameters |  | CW size adjust based on UE interference feedback and LAA eNB CCA check |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Op1 | Op2 | Op1 | Op2 | Op1 | Op2 |
| UPT CDF [Mbps] | 5% | 50.733 | 54.534 | 20.390 | 29.351 | 4.578 | 6.270 |
|  | 50% | 72.377 | 76.293 | 45.006 | 39.976 | 22.322 | 15.146 |
|  | 95% | 101.635 | 90.546 | 87.274 | 66.063 | 66.401 | 42.734 |
|  | Mean | 76.924 | 76.948 | 49.089 | 44.235 | 28.541 | 20.150 |
| Delay CDF [s] | 5% | 0.004 | 0.004 | 0.004 | 0.004 | 0.004 | 0.005 |
|  | 50% | 0.006 | 0.006 | 0.017 | 0.019 | 0.031 | 0.074 |
|  | 95% | 0.054 | 0.058 | 0.122 | 0.310 | 0.281 | 0.722 |
|  | Mean | 0.015 | 0.016 | 0.034 | 0.066 | 0.071 | 0.175 |
| 𝜌 | % | 99.922 | 100 | 100 | 99.844 | 99.493 | 91.578 |
| BO | % | 10.693 | 10.708 | 26.940 | 33.633 | 49.743 | 64.223 |
| 𝜆 |  | 1.6 |  | 2.4 |  | 2.8 |  |