3GPP TSG RAN WG1 Meeting #83	R1-156467

Anaheim, USA, November 15-22, 2015

Agenda Item:	6.2.6.1.1

Source:	Huawei, HiSilicon

Title:	Exception report latency evaluation in standalone operation

Document for:	Discussion and decision

# Introduction

At RAN#69 meeting, the “NB-IoT” WI was approved based on outcome of the GERAN SI on cellular IoT [1]. The objective on exception report latency in study item is stated in TR 45.820 as

For devices supporting such applications a delay requirement of 10 seconds is appropriate for the uplink when measured from the application ‘trigger event’ to the packet being ready for transmission from the base station towards the core network.

The evaluation results following methodologies in [2]  and following conclusion have been captured in TR 45.820:

This section demonstrates that exception report can be delivered to the base station within 10s for all coverage classes with a reliability of at least 99%.

In this contribution, the evaluation results of exception report delivery latency with an UL numerology supporting 2.5 kHz  sub-channel spacing and 1.875 kHz sample rate is presented for standalone operation (see [6] for more information).

# Exception report procedure

The different steps involved in sending an exception report are shown in Figure 1. The time it takes to complete each step depends on the coverage class.

Synchronization

Primary SI

RACH

Uplink Assignment

Uplink data transfer

Ack for uplink data

Tsync

TMIB

TRACH

TUplinkAssignment

TUplinkData

TACK

Figure 1 Steps for exception reporting

## Time to send uplink data

For exception reporting, the field sizes are summarized in Table 1, including application payload size, the IP protocol size, the NAS protocol size and the MAC [2]. Table 2 gives the PUSCH configurations and transmission time with BLER less than 10%, where different channel bonding factors are applied for different coverage conditions.

Table 1 PHY layer uplink payload size

|  | Uncompressed IP |
| --- | --- |
| Application layer report size | 20 |
| Upper layer Protocol header | 65 |
| SNDCP header | 4 |
| LLC header | 6 |
| MAC | 10 |
| Total PHY payload size (bytes) | 105 |

Table 2 PUSCH configurations with less than 10% BLER

| Burst type | PHY burst size | Coupling loss | Channel bonding factor | Symbol rate | modulation | repetition | code rate | Duration (ms) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PUSCH | 105 bytes | 144dB | 16 | 30kHz | GMSK | 1 | 0.77 | 50 |
|  |  | 154dB | 2 | 3.75kHz | GMSK | 1 | 0.77 | 400 |
|  |  | 164dB | 1 | 1.875 kHz | GMSK | 1 | 0.32 | 1920 |

## Total time to send exception report

The different durations for the initial transmission of an exception report are shown in Table 3. This corresponds to higher than 90% confidence of successful delivery because the initial message BLER for the uplink report is less than 10% as shown in Table 2. The following assumptions have been made:

- The results on synchronization refer to [3], and the results on broadcast channel refer to [4].  The evaluation results of other downlink channels refer to [2].

- A NB-EPDCCH configuration where a NB-EPDCCH message for certain UE could be transmitted in one NB-EPDCCH occurrence is assumed. The time to receive the NB-EPDCCH message is obtained according to results in [2]. In the worst case, the base station may have schedule NB-EPDCCH when receiving RACH transmissions (on PUSCH). Therefore, BS needs to wait for next NB-EPDCCH occurrence to transmit uplink assignment.

- The total time to transmit the uplink packet containing the exception report includes an additional delay waiting for PUSCH, which is 40ms in the worst case as in [2]. In addition, a 3ms delay is added to account for the base station processing time.

Table 3 Exception report activity duration for 90% confidence of delivery

| Activity |  | Report with no header compression (105 byte payload) |  |  |
| --- | --- | --- | --- | --- |
| Coupling loss |  | 144 | 154 | 164 |
| Tsync (ms) |  | 140 | 140 | 220 |
| TPSI(ms) | PSCH – to - PSI gap (ms) | 80 | 80 | 320 |
|  | Time to decode PSI Duration (ms) | 80 | 80 | 320 |
| TRACH (ms) | RACH Duration | 40 | 40 | 320 |
|  | PSI to RACH gap | 44 | 44 | 324 |
| TUplinkAssignment (ms) | Wait for NB-EPDCCH | 320 | 320 | 320 |
|  | NB-EPDCCH | 5 | 15 | 110 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |
|  | Transmission time | 50 | 400 | 1920 |
|  | Processing time | 3 | 3 | 3 |
| Total time (ms) |  | 802 | 1162 | 3897 |

If the entire report has to be re-transmitted, then this involves reception of negative acknowledgement, new assignment message, data transmission and subsequent acknowledgement reception. With this simple model, the durations for initial transmission and re-transmission of an exception report are shown in Table 4. This corresponds to higher than 99% confidence of successful delivery because the message BLER for each uplink report transmission is less than 10%. The additional assumptions for re-transmission are as following:

- The time to wait for NB-EPDCCH in receiving uplink ACK and uplink assignment is calculated as 320 – (T mod 320), where 320ms is the NB-EPDCCH periodicity and T is the time required of preceding steps since last NB-EPDCCH transmission.

Table 4 Exception report activity duration for 99% confidence

| Activity |  | Report with no header compression (105 byte payload) |  |  |
| --- | --- | --- | --- | --- |
| Coupling loss |  | 144 | 154 | 164 |
| Tsync (ms) |  | 140 | 140 | 220 |
| TPSI(ms) | PSCH – to - PSI gap (ms) | 80 | 80 | 320 |
|  | Time to decode PSI Duration (ms) | 80 | 80 | 320 |
| TRACH (ms) | RACH Duration | 40 | 40 | 320 |
|  | PSI to RACH gap | 44 | 44 | 324 |
| TUplinkAssignment (ms) | Wait for NB-EPDCCH | 320 | 320 | 320 |
|  | NB-EPDCCH | 5 | 15 | 110 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |
|  | Transmission time | 50 | 400 | 1920 |
|  | Processing time | 3 | 3 | 3 |
| TUplinkAck (ms) & TUplinkAssignment (ms) | Wait for NB-EPDCCH | 222 | 182 | 167 |
|  | NB-EPDCCH | 5 | 15 | 110 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |
|  | Transmission time | 50 | 400 | 1920 |
|  | Processing time | 3 | 3 | 3 |
| Total time (ms) |  | 1122 | 1802 | 6137 |

The further evaluation results for 15 kHz subcarrier spacing downlink with FDMA uplink are provided in Annex A. For comparison, the results of design with 15 kHz sub-carrier spacing downlink and SC-FDMA uplink in [5] are reproduced in Annex B. By comparing the results in Annex A and Annex B, the following could be observed:

Observation 1: For design with 3.75 kHz subcarrier spacing downlink and FDMA with GMSK uplink, the exception report can be delivered to the network with 99% confidence within 10 seconds in standalone operation, for all the target MCLs, including 164 dB.

Observation 2: FDMA with GMSK uplink design has lower latency than SC-FDMA based uplink design (for both numerologies) in delivering exception report to the network in standalone operation.
    - This is without taking into account power back-off for SC-FDMA, see [7] for details.

# Conclusions

This document provides the latency evaluation for transmitting an exception report to the network in standalone operation. Based on the analysis, the following can be observed:

Observation 1: For design with 3.75 kHz subcarrier spacing downlink and FDMA with GMSK uplink, the exception report can be delivered to the network with 99% confidence within 10 seconds in standalone operation, for all the target MCLs, including 164 dB.

Observation 2: FDMA with GMSK uplink design has lower latency than SC-FDMA based uplink design (for both numerologies) in delivering exception report to the network in standalone operation.
    - This is without taking into account power back-off for SC-FDMA, see [7] for details.

# References

RP-151621, “New Work Item: Narrow Band IOT (NB-IOT)”, Qualcomm Incorporated, RAN#69, Phoenix, US, September, 2015.

3GPP TR 45.820, Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things, Release 13, version 13.0.0.

R1-156465, Evaluations of network synchronization, Huawei Technologies Co., Ltd., HiSilicon, RAN1#83, Annaheim, Nov., 2015.

R1-156462, NB-IOT - Downlink physical layer concept description, Huawei Technologies Co., Ltd., HiSilicon, RAN1#83, Annaheim, Nov., 2015.

R1-156027, NB-IoT – Exception report latency evaluation, Ericsson, RAN1#82BIS, Malmo, Oct., 2015.

R1-156013, Coverage analysis in stand-alone operation, Ericsson, RAN1#82BIS, Oct., 2015.

R1-156466, Uplink coverage evaluation, Huawei Technologies Co., Ltd., HiSilicon, RAN1#83, Annaheim, Nov., 2015.

R1-156930, Uplink FDMA physical layer design updates, Neul, Huawei, HiSilicon, RAN1#83, November 2015.

# Evaluation results of 15 kHz subcarrier spacing downlink and FDMA uplink

This section provides the further evaluation results for 15 kHz subcarrier spacing downlink and FDMA uplink. The different durations for transmitting an exception report with 90% and 99% confidence of successful delivery are summarized in Table 5 and Table 6, respectively. The following assumptions have been made:

- The evaluation methodology and results of uplink channels with FDMA refer to section 2, and downlink channels with 15 kHz sub-carrier spacing refer to [5].

- In the worst case, the delay between end of PRACH and next NB-EPDCCH, i.e. waiting for NB-EPDCCH, is assumed as

MAX_NBEPDCCH_TX+MAX_PDSCH+MAX_NBEPDCCH_WAIT=580ms [5].

- The time to wait for NB-EPDCCH in receiving uplink ACK and uplink assignment is calculated as 40 – (T mod 40), where 40 ms is the NB-EPDCCH periodicity and T is the time required of preceding steps since last NB-EPDCCH transmission [5].

Table 5 Exception report activity duration for 90% confidence of delivery

| Activity |  | Report with no header compression (105 byte payload) |  |  |
| --- | --- | --- | --- | --- |
| Coupling loss |  | 144 | 154 | 164 |
| Tsync (ms) |  | 340 | 340 | 520 |
| TPSI(ms) | PSCH – to - PSI gap (ms) | 80 | 80 | 320 |
|  | Time to decode PSI Duration (ms) | 71 | 71 | 311 |
| TRACH (ms) | RACH Duration | 40 | 40 | 320 |
|  | PSI to RACH gap | 40+4 | 40+4 | 320+4 |
| TUplinkAssignment (ms) | Wait for NB-EPDCCH | 580 | 580 | 580 |
|  | NB-EPDCCH | 1 | 4 | 40 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |
|  | Transmission time | 50 | 400 | 1920 |
|  | Processing time | 3 | 3 | 3 |
| Total time (ms) |  | 1249 | 1602 | 4378 |

Table 6 Exception report activity duration for 99% confidence of delivery

| Activity |  | Report with no header compression (105 byte payload) |  |  |
| --- | --- | --- | --- | --- |
| Coupling loss |  | 144 | 154 | 164 |
| Tsync (ms) |  | 340 | 340 | 520 |
| TPSI(ms) | PSCH – to - PSI gap (ms) | 80 | 80 | 320 |
|  | Time to decode PSI Duration (ms) | 71 | 71 | 311 |
| TPRACH (ms) | PRACH Duration | 40 | 40 | 320 |
|  | PSI to PRACH gap | 40+4 | 40+4 | 320+4 |
| TUplinkAssignment (ms) | Wait for NB-EPDCCH | 580 | 580 | 580 |
|  | NB-EPDCCH | 1 | 4 | 40 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |
|  | Transmission time | 50 | 400 | 1920 |
|  | Processing time | 3 | 3 | 3 |
| TUplinkAck (ms) & TUplinkAssignment (ms) | Wait for NB-EPDCCH | 26 | 33 | 37 |
|  | NB-EPDCCH | 1 | 4 | 40 |
| TUplinkData (ms) | Wait for PUSCH | 40 | 40 | 40 |
|  | Transmission time | 50 | 400 | 1920 |
|  | Processing time | 3 | 3 | 3 |
| Total time (ms) |  | 1369 | 2082 | 6418 |

# Evaluation results of SC-FDMA

In this section, the results of design with 15 kHz sub-carrier spacing downlink and SC-FDMA uplink in [5] are reproduced in Table 7 and Table 8 for 90% and 99% confidence of delivery, respectively.

Table 7 Exception report delivery time with 90% confidence

| Activity Coupling loss (dB) | 144 | 154 | 164 |
| --- | --- | --- | --- |
| Tsync(ms) | 340 | 340 | 520 |
| TMIB(ms) | 151 | 151 | 631 |
| TPRACH(ms) | 324 | 688 | 1440 |
| TRAmsg2-4 (ms) | 622 | 708 | 1060 |
| TULgrant(ms) | 48 | 45 | 49 |
| TULdata(ms) | 39 | 553 | 1923 |
| Total time (ms) | 1524 | 2485 | 5623 |

Table 8 Exception report delivery time with 99% confidence

| Activity Coupling loss (dB) | 144 | 154 | 164 |
| --- | --- | --- | --- |
| Tsync(ms) | 340 | 340 | 520 |
| TMIB(ms) | 151 | 151 | 631 |
| TPRACH(ms) | 324 | 688 | 1440 |
| TRAmsg2-4 (ms) | 622 | 708 | 1060 |
| TULgrant(ms) | 48 | 45 | 49 |
| TULdata(ms) | 39 | 553 | 1923 |
| TAck(ms) | 41 | 47 | 77 |
| TULdata(ms) | 39 | 553 | 1923 |
| Total time (ms) | 1604 | 3085 | 7623 |