3GPP TSG RAN WG1 Meeting #83	R1-156984

Anaheim, USA, 15th - 22th November 2015

Agenda Item:    6.2.3.1

Source:              ZTE

Title:                  Contention Window Adaptation based on HARQ-ACK feedback

Document for:  Discussion and Decision

# Introduction

At RAN1 #82b meeting, the procedure of adjusting contention window size is discussed and agreed [1].

Agreements:

- For CWS adjustment based on HARQ-ACKs,

  - Set of CWSs for LBT priority class 3 = {15, 31, 63}

  - The CWS is increased if at least Z % of the HARQ-ACK feedback values for a reference subframe set are NACK. Otherwise, the CWS is reset to the minimum value (i.e., 15).

    - Reference subframe set (to be down selected)

      - Alt. 1: the latest DL subframe for which HARQ-ACK feedback is available

      - Alt. 2: the first DL subframe of the latest DL data burst for which HARQ-ACK feedback is available

      - Alt. 3: all subframes for which HARQ-ACK feedback is available of the latest DL data burst for which HARQ-ACK feedback is available

    - FFS on the Z value. Select one out of {10%, 50%, 75%, 100%}.

- In addition, the CWS is reset to the minimum value (i.e., 15) if the maximum CWS (i.e., 63) is used for K consecutive eCCA for transmission

  - K is selected by NW from the set of values from (1, …,8)

  - FFS: Whether the CWS is reset to the minimum value if there has been no DL transmission by the eNB for a duration of at least T

  - FFS: HARQ-ACK DTX

In this contribution, we discuss these remaining issues of CWS adjustment based on HARQ-ACKs in details.

# Contention window size adaption based on HARQ-ACK feedback

## Reference subframe set

The following three options are considered for adapting the CWS based on the HARQ-ACK feedback of reference subframe set:

- Alt. 1: the latest DL subframe for which HARQ-ACK feedback is available

- Alt. 2: the first DL subframe of the latest DL data burst for which HARQ-ACK feedback is available

- Alt. 3: all subframes for which HARQ-ACK feedback is available of the latest DL data burst for which HARQ-ACK feedback is available

For Alt. 1 and Alt. 2, CWS adjustment is based on HARQ-ACK feedback from a single subframe. For these two methods, since the number of UEs which feedback HARQ-ACK is relatively few (e.g., only one UE is scheduled within a TTI), it is less likely to reflect the actual load of channel.

On the contrary, in Alt. 3, in most cases, HARQ-ACK feedback from multiple subframes is taken into account for CWS adjustment. The HARQ-ACK feedback comes from more UEs. As a consequence, it is likely to sufficiently reflect the load of channel.

Furthermore, Alt. 3 may be more effective in detecting hidden nodes. For example, if LAA eNBs adjust CWS corresponding to HARQ-ACK feedback of a single subframe, HARQ-ACK feedback of UEs in the other subframes during the DL transmission burst is not utilized and the hidden node problem if occurred in those subframes will not be observed. Therefore, the CWS adjustment corresponding to HARQ-ACK feedback of a predefined window in Alt. 3 is more effective than that of Alt. 1 and Alt. 2 considering the problem of hidden node.

Proposal 1: all subframes for which HARQ-ACK feedback is available of the latest DL data burst for which HARQ-ACK feedback is available should be adopted as the reference subframe set of HARQ-ACK feedback.

## Time effectiveness

For all the above discussed reference subframe sets, there is an important issue of time effectiveness. For example, if there’s a long time idle between the reference subframe set (i.e., the latest DL data burst) and current LBT window, the HARQ-ACK feedback from the reference subframe set may not reflect the current channel status at all. In turn, CWS adjustment based on HARQ-ACK feedback would not be logical and reasonable. To overcome this problem, it is suggested that a limitation of time be applied to the reference subframe set for which HARQ-ACK feedback is available. For example, the number of subframes from the reference subframe set for which HARQ-ACK feedback is available to current LBT is not greater than M. The value of M can be derived from COTmax defined in regulation. For example, M=X*COTmax, where X is selected by network from the set of values from (3, …, 10).

Otherwise, if the number of subframes from the reference subframe set for which HARQ-ACK feedback is available to current LBT is greater than M, the CWS is reset to the minimum value.

Proposal 2:  If the number of subframes from the reference subframe set for which HARQ-ACK feedback is available to current LBT is not greater than M, CWS is adjusted based on HARQ-ACKs. Otherwise, the CWS is reset to the minimum value. M=X*COTmax, and X is selected by network from the set of values from (3, …, 10).

## Simulations

Simulations were conducted to investigate the performance of CWS adaptation based on UE HARQ-ACK feedback with different reference subframe set and Z values. Both LAA-LAA coexistence and WiFi-LAA coexistence scenarios in an indoor deployment are considered. Reference subframe set Alt. 1, Alt. 2 and Alt. 3 are implemented. Different Z values are also simulated.

Detailed simulation parameters are presented in the Appendix. Table 1 to 3 show the mean UPT of both operators’ eNBs with different reference subframe set and Z values under different traffic load.

Table 1 LAA-LAA coexistence, Mean UPT, Alt. 1 of reference subframe set

| 𝜆 | 0.3 |  | 0.6 |  |
| --- | --- | --- | --- | --- |
|  | LAA op1 | LAA op2 | LAA op1 | LAA op2 |
| Z = 0.1 | 94.743 | 94.159 | 17.474 | 20.597 |
| Z = 0.5 | 94.743 | 94.159 | 17.474 | 20.597 |
| Z = 1 | 94.743 | 94.159 | 17.474 | 20.597 |

Table 2 LAA-LAA coexistence, Mean UPT, Alt. 2 of reference subframe set

| 𝜆 | 0.3 |  | 0.6 |  |
| --- | --- | --- | --- | --- |
|  | LAA op1 | LAA op2 | LAA op1 | LAA op2 |
| Z = 0.1 | 97.408 | 96.636 | 25.808 | 29.579 |
| Z = 0.5 | 98.302 | 96.157 | 20.908 | 25.546 |
| Z = 1 | 96.597 | 95.450 | 18.132 | 22.318 |

Table A.4 LAA-LAA coexistence, Mean UPT, Alt. 3 of reference subframe set

| 𝜆 | 0.3 |  | 0.6 |  |
| --- | --- | --- | --- | --- |
|  | LAA op1 | LAA op2 | LAA op1 | LAA op2 |
| Z = 0.1 | 98.378 | 99.834 | 27.002 | 30.610 |
| Z = 0.5 | 98.406 | 96.205 | 22.481 | 27.641 |
| Z = 1 | 96.105 | 95.347 | 20.912 | 24.973 |

The following are observed based on the results.

- Alt. 3 of reference subframe set has the best performance in particular for high load case where CWS has a bigger impact to UPT performance.

- Z = 0.1 has a good performance in general.

Proposal 3: Z = 0.1.

# Conclusion

In this contribution, we have discussed several aspects of contention window size adaptation for DL LBT and presented the system simulation results. We have the following proposals:

Proposal 1: all subframes for which HARQ-ACK feedback is available of the latest DL data burst for which HARQ-ACK feedback is available should be adopted as the reference subframe set of HARQ-ACK feedback.

Proposal 2:  If the number of subframes from the reference subframe set for which HARQ-ACK feedback is available to current LBT is not greater than M, CWS is adjusted based on HARQ-ACKs. Otherwise, the CWS is reset to the minimum value. M=X*COTmax, and X is selected by network from the set of values from (3, …, 10).

Proposal 3: Z = 0.1.

# References

[1] Chairman's Notes RAN1_82b - final

# Appendix

|  | Licensed cell | Unlicensed cell |
| --- | --- | --- |
| Layout for nodes | For DL-only coexistence evaluations: Two operators deploy 4 small cells each in the single-floor building. The small cells of each operator are equally spaced and centered along the shorter dimension of the building. The distance between two closest nodes from two operators is random. The set of small cells for both operators is centered along the longer dimension of the building. 120 m 50 m |  |
| System bandwidth per carrier | 10MHz | 20MHz |
| Carrier frequency | 3.5GHz | 5.0GHz |
| Number of carriers | 2 (one for each operator) | 1 |
| Total BS TX power | 24dBm (Ptotal per carrier) | 24 dBm |
| Total UE TX power | Total UE TX power: 23dBm across aggregated cells Max total UE TX power per cell in licensed spectrum: 23dBm Max total UE TX power across aggregated cells in unlicensed spectrum: 18 dBm |  |
| Distance-dependent path loss | Small cell-to-Small cell, Small cell-to-UE: ITU InH [referring to Table B.1.2.1-1 in TR36.814] Indoor UE-to-indoor UE: 3GPP TR 36.843 (D2D). (3D distance between an eNB and a UE is applied. Working assumption is that 3D distance is also used for LOS probability and break point distance) |  |
| Penetration | 0dB |  |
| Shadowing | ITU InH [referring to Table A.2.1.1.5-1 in TR36.814] Working assumption is that 3D distance is used for shadowing correlation distance |  |
| Antenna pattern | 2D Omni-directional is baseline; directional antenna is not precluded |  |
| Antenna Height: | 6m |  |
| UE antenna Height | 1.5m |  |
| Antenna gain + connector loss | 5dBi |  |
| Antenna gain of UE | 0 dBi |  |
| Fast fading channel between eNB and UE | ITU InH |  |
| Number of clusters/buildings per macro cell geographical area | N/A |  |
| Number of small cells per cluster | N/A |  |
| Number of small cells per Macro cell | N/A |  |
| Number of UEs | 20 UEs per unlicensed band carrier per operator |  |
| UE dropping per network | All UEs should be randomly dropped and be within coverage of the small cell in the unlicensed band. |  |
| Radius for small cell dropping in a cluster | N/A |  |
| Radius for UE dropping in a cluster | N/A |  |
| Minimum distance (2D distance) | 3m |  |
| Traffic model | FTP Model 3: Based on FTP model 2 as in TR 36.814 with the exception that packets for the same UE arrive according to a Poisson process and the transmission time of a packet is counted from the time instance it arrives in the queue. FTP model file size: 0.5/8 Mbytes. |  |
| UE receiver | MMSE-IRC as baseline |  |
| UE noise figure | 9dB |  |
| UE speed | 3km/h |  |
| Cell selection criteria | For LAA UEs, cell selection is based on RSRP in the unlicensed band. For WiFi STAs, cell selection is based on RSS (Received signal power strength) of WiFi APs. RSS threshold is -82 dBm. |  |
| UE Bandwidth | UE bandwidth for LAA: 10 MHz licensed + 20 MHz unlicensed |  |
| Network synchronization | For the same operator, the network is synchronized. Asynchronous between different operators. |  |
| Performance metrics | - User perceived throughput (UPT) - File throughput is calculated per file - Unfinished files should be incorporated in the UPT calculation. - The number of served bits (possibly zero) of an unfinished file by the end of the simulation is divided by the served time (simulation end time – file arrival time). - User throughput is the average of all its file throughputs - Latency (From packet arrival in devices (eNB, AP, UE, STA) MAC buffer to successful transmission (including retransmission) of packet) - Latency CDF |  |

Table A.1: Additional LAA system evaluations assumptions

| Parameters | Value |
| --- | --- |
| PCI planning for each NW | Planned |
| Antenna configuration | 2Tx2Rx in DL, Cross-polarized |
| Transmission schemes | TM10, QPSK/16QAM/64QAM |
| Turbo code block interleaving depth | Per LTE specs (1-14 LTE OFDM symbols dependent on MCS and PRB allocation) |
| Scheduling | Proportional fair |
| Link adaptation | Realistic |
| CCA-ED | -62 dBm |
| eNB contention window | CWmin =15, CWmax = 63 |
| CCA slot size | 9 us |
| Defer period | 25 us |
| Max burst length | 10 ms |