3GPP TSG RAN WG1 Meeting #83	R1-156986

Anaheim, USA, 15th - 22th November 2015

Source:             ZTE

Title:                  Frequency Reuse on Unlicensed Carrier

Agenda Item:    6.2.3.3

Document for:  Discussion and Decision

# Introduction

During the SI [1], it has been identified that enabling frequency reuse for transmission by neighbour LAA cells of the same operator is one target of LAA design [2]. Several options to enable frequency reuse were discussed during the SI.

- At least the following options can be further studied to enable improved freq. reuse for LBE for DL LAA

  - CCA threshold adaptation

  - Tx start timing alignment

  - Signal subtraction from ED or modified ED

  - Combination of those options or other alternatives are not precluded.

At the RAN1 #82 meeting, the following consensus has been reached [4]:

- Signal subtraction from ED as a way to enable frequency reuse is not considered in Rel-13

- Note: Frequency reuse is a design target for LAA

In this contribution, we share our views on frequency reuse among the cells within the same operator.

# Frequency Reuse for LAA

When each transmitter performs CCA check individually on unlicensed spectrum, the transmission from a small cell may block neighbouring small cells of the same operator. Therefore, the key issue of frequency reuse design for LAA is related to CCA check.

## CCA Threshold Adaptation

In this method, multiple CCA thresholds should be defined so that LAA nodes can adjust the CCA threshold based on whether the sensed signal on unlicensed carrier is from other LAA nodes or from other RATs, e.g. Wi-Fi. If the sensed signal comes from other RATs, the channel should be considered as occupied with a lower threshold. If the sensed signal comes from other LAA nodes from the same operator, the channel may be considered as clear for frequency reuse with a much higher CCA threshold.

Some simulations were performed to verify the benefits of frequency reuse with CCA threshold adaptation. With certain increment of CCA threshold (e.g., CCA threshold is adjusted to -52dBm/20MHz or higher) for LAA system, frequency reuse factor is closing to one. This means that LAA nodes can share the same carrier to improve intra-LAA system performance, especially for intra-operator deployment scenario. Figure1 show the performance impact of the frequency reuse when two LAA networks coexisted in an outdoor scenario.

- 2.

但如果在此同一时期内按照人均gdp增长速率来比较的话，那么日本表现比美国及欧元区要好（如图1所示）。

Yet if judged by growth in gdp per person over the same period, then japan has outperformed america and the euro zone ( see chart 1).

www.ecocn.org

- 3.

如图1所示，服务器应用使用一个共用的工作队列来存放从远端的来源处到达的任务。

As shown in figure 1, server applications use a common worker queue to hold short tasks that arrive from remote sources.

article.yeeyan.org

- 4.

如图1所示，主要构成要素是一个堵漏装配，它能安装在已损坏的防喷器顶端，例如致使马孔多油井倾油的防喷器。

As the first diagram shows, the main component would be a containment assembly that could fit on top of a damaged blowout preventer, such as the one from which the macondo oil poured forth.

www.ecocn.org

- 5.

把您的利益相关者如图1所示在我们的免费模板权力/兴趣象限方格上画出来并根据他们对您工作的权限以及对您工作的兴趣进行分类。

Map out your stakeholders on a power/ interest grid on our free template as shown in figure 1, and classify them by their power over your work and by their interest in your work.

article.yeeyan.org

Figure 1 UPT performance of frequency reuse when two LAA networks coexisted

For LAA and Wi-Fi coexistence scenarios, as shown in Figure 2, it can be clearly observed that with the increment of CCA threshold (i.e., frequency reuse factor is closing to one), the mean UPT of Wi-Fi is almost not affected while the LAA has some improvement of UPT (the CCA threshold for Wi-Fi is always fixed at -62dBm/20MHz while the CCA threshold for LAA is increased).

Figure 2 UPT performance of frequency reuse when LAA coexisted with Wi-Fi

In addition, the identification of the source of the sensed signal can be realized by utilizing, e.g. muting pattern proposed in section 2.3. For this purpose, we assume that certain RE or PRB in the whole bandwidth are not assigned to any node and that eNB can detect energy on these REs or PRBs to estimate whether the sensed signal is from other LAA nodes or from Wi-Fi.

Proposal 1: In order to support frequency reuse, multiple CCA thresholds should be defined .

## Tx start timing alignment

In this method, the Tx start time of different LAA eNBs within the same operator are aligned such that one eNB is not blocked by others. In order to achieve this, there are several possible candidate solutions as follows.

Alternative 1: Tx start timing alignment by back-off counter coordination

For example, the small cells in an area can coordinate and use the same random back-off counter instead of individually generating different back-off counters. This may require significant amount of inter-eNB signalling to coordinate eNBs. Therefore, it may incur high network overhead. Furthermore, the latency of current X2 interface between eNBs may be impractical for LBT back-off counter coordination. In addition, due to the different location and channel conditions of the LAA nodes, it is likely that CCA check results may not be the same. Besides, different LAA eNBs may have different loading situation in their buffer and thus different sensing starting time. Finally, it is difficult to ensure accurate synchronization between LAA nodes due to typical very short CCA check time interval.

Thus, controlling all eNB’s backoff counter may result in loss of access opportunity if all of them have to wait to transmit at the same time.

Alternative 2: Tx start timing alignment by alignment of ECCA parameters, ECCA starting time position

For this method, with LBT category 4 schemes adopted in DL, with exponential contention window size, it is difficult to always keep alignment of the CCA window size and back-off counter among LAA eNBs since the interference conditions experienced in each eNB may be different.

Therefore, it is difficult to support frequency reuse by Tx start timing alignment, assuming the exponential contention window size being adopted.

Alternative 3: Tx start timing alignment by self-deferral operation

One of the considerations of the deferred transmission or counter decrease is to allow frequency reuse. However, defer one or more ECCA slot time may make frequency reuse more difficult. In addition, there is also a problem that the coordination delay (at least ms level) is much larger than a CCA slot time. Among them, the defer operation can occur in the ECCA procedure. For example, defer will be mis-aligned with other LAA cells if defer times is too much as shown in Figure 3.

Figure 3 mis-aligned time for DL transmission

To sum up, there are some issues to support Tx start timing alignment schemes: difficult to overcome the coordination delay problem, back-off counter N cannot reduced to zero at the same time and coordination problem of self-defer times.

Observation: to support TX start timing alignment, there are some issues as follows:

- difficult to overcome the coordination delay;

- back-off counter N can’t be reduced to zero at the same time;

- coordination problem of self-defer times, etc.

Proposal 2: TX start timing alignment is not recommended for frequency reuse.

## Muting Pattern to Support Frequency Reuse

As discussed in the above, in order to realize the frequency reuse among the cells within the same operator, the sensing LAA nodes need to find ways to distinguish whether the signal observed on the channel is from other LAA nodes or from other RATs. Furthermore, it needs to tell whether the sensed signal is from intra-operator LAA nodes or from other operators’ LAA nodes and/or Wi-Fi nodes.

One possible way is to mute certain REs or RBs for LAA nodes during its initial signal and/or data transmission. Different operators could have different muting patterns, and the LAA nodes within the same operator have the same muting pattern.

Correspondingly, the LAA eNB can detect energy on these muting REs or RBs to estimate the source of interference. This method can be applied to CCA Threshold adaptation. For example, set a default CCA threshold value to T1, e.g. T1 = -62 dBm. If the eNB detect energy on those muted REs or RBs corresponding to its own operator and recognizes that energy is lower than a pre-defined threshold, then another CCA threshold T2 is used (e.g. T2 = -52 dBm). And if the eNB check energy on the whole bandwidth is lower than CCA threshold T2, then the eNB assumes that frequency reuse can be realized.

Similarly, muting pattern can be applied to the initial signal. Other eNBs of the same operator can detect the muted REs or RBs of initial signal to know whether frequency reuse is possible or not.

Proposal 3: Muting pattern should be considered to support frequency reuse.

# Conclusion

In this contribution, we have discussed some details of frequency reuse on unlicensed carrier. We have the following observation and proposals:

Proposal 1: In order to support frequency reuse, multiple CCA thresholds should be defined.

Observation: to support TX start timing alignment, there are some issues as follows:

- difficult to overcome the coordination delay;

- back-off counter N can’t be reduced to zero at the same time;

- coordination problem of self-defer times, etc.

Proposal 2: TX start timing alignment is not recommended for frequency reuse

Proposal 3: Muting pattern should be considered to support frequency reuse.

# References

[1] RP-141817, Study on Licensed-Assisted Access to Unlicensed Spectrum

[2] TR 36.889,  Feasibility Study on Licensed-Assisted Access to Unlicensed Spectrum V13.0.0

[3] RP-151045, New Work Item on Licensed-Assisted Access to Unlicensed Spectrum, Ericsson, Huawei, Qualcomm, Alcatel-Lucent

[4] Chairman’s Notes RAN1_82-final.