3GPP TSG RAN WG1 Meeting #83                                                                   R1-157183

Anaheim, USA, 15th - 22th November 2015

Source:	NTT DOCOMO

Title:	Considerations on DCI design for DMRS enhancements

Agenda Item:	6.2.4.2.3

Document for: 	Discussion and Decision

# Introduction

At the 3GPP RAN1 #82b meeting [1], the following agreements have been reached concerning the enhanced DMRS ports and the new DCI design for MU-MIMO:

Agreement:

- Confirm the working assumption for OCC=4 and 12REs made in RAN1#82, and the following table

| Ports for MU transmission | OCC |
| --- | --- |
| Port 7 | [1 1 1 1] |
| Port 8 | [1 -1 1 -1] |
| Port 11 | [1 1 -1 -1] |
| Port 13 | [1 -1 -1 1] |

Agreement:

- Agree that a new table will be adopted, configurable by RRC, for the signalling of the DMRS configuration

- FFS until RAN1#83 whether 3 or 4 bits are used, and details.

In this contribution, we discuss the DCI design based on the agreed DMRS port configurations.

# DCI Design for DMRS Enhancements

According to the RAN1 progress, it shall be decided at RAN1#83 meeting, how to define a new antenna port, layer, and SCID indication table to be used by the DCI. Supporting MU-MIMO transmission with port 7~13 with OCC = 4 is mandatory according to the previous agreements. Then it shall be further discussed what other features shall be embeded into this table.

In the legacy LTE systems, different multi-user layers are seperated orthogonally by different ports with a same scrambling sequence, as well as semi-orthogonally by different scrambling sequences on the same port. With the combination of port number and scrambling sequence number, the maximal number of multiplexed layers can be extended. The previous agreement on MU-MIMO DMRS enhancement only mentioned about the enhancemnt of different ports with a same scrambling sequence. It is also desired that dufferent scrambling sequences are used on a same port. The four MU DMRS ports, i.e., port 7, 8, 11, and 13, when combined with two scrambling IDs, can support up to 8 MU layers.

Observation 1: It is benefical to apply different scrambling sequences on top of the orthogonal ports to provide a higher multiplexing order.

During the study item of elevation beamforming and FD-MIMO, it was shown that the percentage of single user transmission after the common multi-user scheduling is not negligible, especially when the FTP traffic model is considered. When the number of active users in the cell is small, or when the active users are not suitable to be paired by the MU scheduler, it is benefical if the system can fall back to single user high-rank transmission. Therefore, it is still desired that the system can support the switching between high-order MU-MIMO and high-rank SU-MIMO transmission.

Observation 2: It is beneficial to the system performance if the antenna port, layer, and SCID indication field can support SU/MU dynamic switching.

With the considered DMRS enhancement, 4 bit OCC codes are used generally. However, when only port 7 and port 8 are actually used to schedule two users, OCC = 2 is sufficient to ensure the DMRS port orthogonality. Using OCC = 2, the channel estimation accuracy is expected to be improved, compared to OCC = 4 case. It is more important that the system can fall back to OCC = 2 in order to support high UE mobillity. To support such feature, additoinal information that whether ports 11/13 are used must be notified to UEs scheduled on port 7 and 8. With such information, UEs can decide which OCC length is used during DMRS channel estimation.

Observation 3: If only antenna ports 7 and 8 are used in MU-MIMO transmission, it is beneficial to inform the UE that OCC = 2 can be assumed when making the channel estimation on DMRS ports.

Considering the requirement to support both high-order MU-MIMO and high-rank SU-MIMO, as summarized in Observation 1 and 2, and the requirement to nofity the UE two switch between OCC = 2 and OCC = 4 depending on the scheduling situation, it seems that a 4-bit new antenna port, layer, and SCID indication table is needed. More specifically, we propose the following DCI design to meet the above-mentioned requirements.

Proposal: Extended the antenna port, layer, and SCID indication field to 4 bits where details of each message are shown in Table 1.

Table 1: Messages of extended antenna port, layer, and SCID indication

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |
| --- | --- | --- | --- |
| Value | Message | Value | Message |
| 0 | 1 layer, port 7, SCID=0, w/o port 11/13 | 0 | 2 layers, port 7, 8, SCID=0, w/o port 11/13 |
| 1 | 1 layer, port 8, SCID=0, w/o port 11/13 | 1 | 2 layers, port 7, 8, SCID=1, w/o port 11/13 |
| 2 | 1 layer, port 7, SCID=1, w/o port 11/13 | 2 | 3 layers, port 7~9 |
| 3 | 1 layer, port 8, SCID=1, w/o port 11/13 | 3 | 4 layers, port 7~10 |
| 4 | 2 layers, port 7~8 | 4 | 5 layers, port 7~11 |
| 5 | 3 layers, port 7~9 | 5 | 6 layers, port 7~12 |
| 6 | 4 layers, port 7~10 | 6 | 7 layers, port 7~13 |
| 7 | 1 layer, port 7, SCID=0, w/ port 11/13 | 7 | 8 layers, port 7~14 |
| 8 | 1 layer, port 8, SCID=1, w/ port 11/13 | 8 | 2 layers, port 7, 8, SCID=0, w/ port 11/13 |
| 9 | 1 layer, port 7, SCID=0, w/ port 11/13 | 9 | 2 layers, port 7, 8, SCID=1, w/ port 11/13 |
| 10 | 1 layer, port 8, SCID=1, w/ port 11/13 | 10 | 2 layers, port 11, 13, SCID=0 |
| 11 | 1 layer, port 11, SCID=0 | 11 | 2 layers, port 11, 13, SCID=1 |
| 12 | 1 layer, port 13, SCID=0 | 12 | Reserved |
| 13 | 1 layer, port 11, SCID=1 | 13 | Reserved |
| 14 | 1 layer, port 13, SCID=1 | 14 | Reserved |
| 15 | Reserved | 15 | Reserved |

# Summary

In this contribution, we discussed the DCI table design to support enhanced DMRS for MU-MIMO transmission. Based on our analysis, the following observations and proposal are made.

Observation 1: It is benefical to apply different scrambling sequences on top of the orthogonal ports to provide a higher multiplexing order.

Observation 2: It is beneficial to the system performance if the antenna port, layer, and SCID indication field can support SU/MU dynamic switching.

Observation 3: If only antenna ports 7 and 8 are used in MU-MIMO transmission, it is beneficial to inform the UE that OCC = 2 can be assumed when making the channel estimation on DMRS ports.

Proposal: Extended the antenna port, layer, and SCID indication field to 4 bits where details of each message are shown in Table 1.

# References

- 3GPP, “Draft Report of 3GPP TSG RAN WG1 #82bis v0.2.0.”