3GPP TSG RAN WG1 Meeting #83	R1- 157338

Anaheim, USA, November 15-22, 2015

Agenda Item:	6.2.6.1.1

Source:	Huawei, HiSilicon

Title:	Downlink Capacity Evaluation in Standalone Operation

Document for:	Discussion

# Introduction

At RAN1 #82bis meeting, a number of contributions [1]-[3] showed system-level evaluation results based on various DL/UL numerologies with regard to the standalone operation mode. The methodologies defined in [4] were followed by these evaluations and one observation was drawn:

# Observation:

- All the results presented confirm that the NB-IoT solutions under consideration satisfy the capacity and latency targets

In this contribution, further evaluations for downlink capacity are provided based on 3.75kHz downlink subcarrier spacing. Frequency reuse factor 1/1 is assumed in the evaluations in this contribution instead of frequency reused factor 1/3. The uplink capacity in the standalone operation mode is evaluated in our companion contribution [5].

# BPL modeling

The BPL modeling results are shown in Figure 1, for both BPL scenarios (see Table D.2 and Table D.3 of [4]) and both agreed correlation coefficients (i.e. 0.5 and 0.75). These results show the CDF of the BPL for all the NB-IoT devices in the simulation, after each NB-IoT device has selected its preferred cell based on minimising the overall path plus penetration loss.

Figure 1. BPL modeling results

# Traffic profile and generation

The traffic profile defined in sub-clauses 5.2.2 [4] is followed. In this document, the information exchange due to the initiation of a MAR (mobile autonomous reporting) periodic or NC (network command) attempt is referred to as a “session”.

Due to the limitation on processing capability of the workstations running simulations, the actual simulation time (denoted by , in seconds) is normally in the order of hundreds or thousands of seconds (although the time needed to run the simulation is normally in the order of days).

The traffic profile is generated as follows,

- The number of MAR periodic sessions generated per sector per day is expressed as:

where  is the number of MSs configured per sector (see Annex E.1 of [4]).

- The total number of NC sessions generated per sector per day is expressed as:

- The total number of sessions generated per sector during the simulation is:

Note that the total number of sessions generated per cell site is .

- It is not clear in [4] how the MAR periodic and NC traffic is distributed over time. In this document it is assumed that they are uniformly distributed over time (i.e. similar to traffic profile 2).

# Other simulation settings

The NB-IoT system is assumed to reuse a single 200 kHz carrier in all cells and is deployed in a frequency reuse of 1/1.

Only QPSK is used in the downlink capacity evaluations. The code rates used in the evaluation are presented in Table 1.

Table 1. MCS used for the evaluation

| MCS index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Code rate | 0.01 | 0.02 | 0.03 | 0.08 | 0.17 | 0.33 | 0.5 | 0.7 | 0.93 |

The same L2S mapping methodology as in [5] is followed.

The MCS is determined for each NB-IoT device such that the highest MCS index is selected, subject to the required SINR being lower than or equal to the NB-IoT device’s average SINR (derived by geometry).

An independent measurement error, according to N(0,3 dB), is applied to each base station. This implies that some users will not select the optimum serving cell, and also not the most appropriate MCS/coverage class (the one that minimizes resource utilization).

Data transmission and retransmission described in subclause 7.3.4.6.3 [4] are followed.

Other simulation assumptions follow Table D.1 in Annex D, which are recapped in Table 3.

# Simulation results

## Simulation cases

The simulation case applied is shown in Table 2.

To determine the maximum capacity of the system, the simulation case is run for a number of offered loads (denoted by “#devices per sector”).

Table 2. Definition of simulation case

| IP header compression | BPL scenario | BPL inter-site correlation coefficient |
| --- | --- | --- |
| No | Scenario 2 | 0.75 |

Note 1: Without IP header compression, the protocol overhead above (equivalent of) SNDCP layer is 65 bytes. See Table E.2-3 in [4] for more details. The header overhead of (equivalent of) SNDCP down to MAC (e.g. SNDCP, LLC, RLC/MAC in Gb mode) layer can be estimated to be 15 bytes (4 bytes for SNDCP + 6 bytes for LLC + 2 bytes for MAC + 3 bytes for CRC).

Note 2: BPL scenario  2 is defined in Table D.2 and Table D.3 of [4], respectively.

Suppose the total number of successful downlink packets collected from all cell sites is , the number of simulated cell sites is , and the number of 200 kHz carriers allocated to one cell site is . For each simulation case, the capacity result is given by:

## Results

The capacity is calculated based on counting downlink packets that are successfully delivered, as indicated in section 5.1. The black line represents the “ideal capacity” (i.e. assuming every downlink packet is successfully delivered by the system), so is a straight line through the origin with gradient determined by the parameters of the traffic model. The capacity results for both of the two downlink terminologies (i.e. 3.75kHz subcarrier spacing and 15kHz subcarrier spacing) are shown in Figure 2. Different proportions of downlink overhead among the total available downlink resources are assumed as:

- Resource utilization case 1: 25% of the total downlink resources are allocated synchronization signals, broadcast channels and control channel, as a result the remaining 75% of the total resource elements are available for the data channel.

- Resource utilization case 2: 25% of the total downlink resources are allocated synchronization signals and broadcast channels, as a result the remaining 75% of the total resource elements are shared by the control channel and the data channel.

Figure 2. Downlink capacity

The same distribution of different MCSs for both 3.75kHz subcarrier spacing and 15kHz subcarrier spacing  in the standalone operation mode is shown in Figure 3.

Figure 3 Distribution of different MCSs for DL

From the simulation results, it can be seen that using the frequency reuse factor of 1 the NB-IoT downlink with 3.75kHz sub-channel spacing have excellent downlink capacity performance in the standalone operation mode. And also higher capacity is observed by the downlink with 3.75kHz subcarrier spacing compared to the 15kHz subcarrier spacing mainly due to the higher spectrum efficiency. It can be also seen that if the constraint of control channel capacity is considered (i.e. resource utilization case 2), the overall downlink capacity will be smaller so probably the downlink capacity rather than the uplink capacity becomes the limiting factor of the whole system.

Observation: The NB-IoT downlink with 3.75kHz subcarrier spacing achieves higher capacity than the downlink with 15kHz subcarrier spacing in the standalone operation mode.

# Conclusions

In this document, system level simulation results for the standalone operation mode are provided for the NB-IoT downlink with different subcarrier spacing. Frequency reuse factor of 1 is assumed in all the evaluations. The following observation is made:

Observation: The NB-IoT downlink with 3.75kHz subcarrier spacing achieves higher capacity than the downlink with 15kHz subcarrier spacing in the standalone operation mode.

# References

R1-155967, “System Level Simulation for Capacity and Latency Evaluation”, Huawei, HiSilicon, 3GPP RAN1 #82bis.

R1-156016, “Performance evaluation in stand-alone operation”, Ericsson, 3GPP RAN1 #82bis.

R1-155997, “System simulation of uplink capacity for NB-IoT”, ZTE, 3GPP RAN1 #82bis

3GPP TR 45.820 v2.1.0, “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”.

R1-156468, “Downlink Capacity Evaluation in Standalone Operation”, Huawei Technologies Co., Ltd, HiSilicon Technologies Co. Ltd., 3GPP TSG RAN1 #83.

# Appendix

Table 3. Assumptions for system level simulations in standalone operation mode

| No | Parameter |  | Assumption |
| --- | --- | --- | --- |
| 1 | Cellular Layout |  | Hexagonal grid, 19 cell sites, 3 sectors per site with wrap-around |
| 2 | Frequency band |  | 900 MHz |
| 3 | Inter site distance |  | 1732 m |
| 4 | MS speed |  | 0 km/h |
| 5 | User distribution |  | Users dropped uniformly in entire cell |
| 6 | Base station transmit power per 200 KHz (at the antenna connector) |  | 43 dBm |
| 7 | NB-IoT Tx power (at the antenna connector) |  | Max. 23 dBm per uplink physical channel with open loop power control |
| 8 | Path loss model |  | L=I + 37.6log10(R), R in kilometers I=120.9 for the 900 MHz band |
| 9 | Shadowing standard deviation |  | 8 dB |
| 10 | Correlation distance of Shadowing |  | 110 m |
| 11 | Shadowing correlation | Between cell sites | 0.5 |
|  |  | Between sectors of the same cell site | 1.0 |
| 12 | Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns) |  | See table 5-7, 3GPP TR 45.914, 65° H-plane. |
| 13 | BS antenna gain |  | 18 dBi |
| 14 | NB-IoT device Antenna gain |  | -4 dBi |
| 15 | BS cable loss |  | 3 dB |
| 16 | Building Penetration Loss |  | Based on distributions derived from adapted COST 231 NLOS model. See Annex D.1 of [4]. |